IALA S-247

Product Specification

Data Classification and Encoding Guide

Edition 1.0.0 – September 2021

IALA Differential eLoran Reference Station Almanac Product Specification

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |
| Oct. 2019 | Various | Draft the product specification |
| Sep. 2021 | Various | Updated various sections and figures |

# Overview

## Preface

The “Data Classification and Encoding Guide” has been developed to provide consistent, standardized instructions for encoding S-247 Differential eLoran Reference Station Almanac data. The purpose of the Data Classification and Encoding Guide is to facilitate S-247 encoding to meet IALA standards.

## S-247 Data Classification and Encoding Guide - Metadata

Note:  This information uniquely identifies this Annex to the Product Specification and provides information about its creation and maintenance.

Title:	The IALA Differential eLoran Reference Station Almanac Product Specification, Appendix A – Data Classification and Encoding Guide

Version:	1.0.0

Date:		September 2021

Language:	English

Classification:	Unclassified

Contact:	IALA-AISM

10, rue des Gaudines

78100 Saint Germain en Laye, France

Telephone: +33 1 34 51 70 01    Fax: +33 1 34 51 82 05

URL:		www. iala-aism.org

Identifier:	S-247

Maintenance:	The product specification is maintained by IALA-AISM and amendments are performed on a needs base, up to maximum one new release per calendar year.

## Terms, definitions and abbreviations

### Terms and definitions

aggregation

special form of association that specifies a whole-part relationship between the aggregate (whole) and a component (see composition)

association

semantic relationship between two or more classifiers that specifies connections among their instances

NOTE:  A binary association is an association among exactly two classifiers (including the possibility of an association from a classifier to itself)

attribute

named property of an entity

NOTE:  Describes the geometrical, topological, thematic, or other characteristic of an entity

composition

a strong  aggregation; if a container object is deleted than all of its containee objects are deleted as well (that is, containee objects cannot exist without the container object)

enumeration

A fixed list of valid identifiers of named literal values. Attributes of an enumerated type may only take values from this list (source: ISO 19136:XX, Geographic information — Geography Markup Language (GML))

feature

Abstraction of real world phenomena

NOTE:  A feature may occur as a type or an instance.  The terms “feature type” or “feature instance” should be used when only one is meant

EXAMPLE:  The feature instance named “Eiffel Tower” may be classified with other phenomena into a feature type “tower”

geometric primitive

geometric object representing a single, connected, homogeneous element of geometry

NOTE:  Geometric primitives are non-decomposed objects that present information about geometric configuration.  They include points, curves, surfaces

point

0-dimensional geometric primitive, representing a position

NOTE:  The boundary of a point is the empty set

### Abbreviations

ENC	Electronic Navigational Chart

GML	Geography Markup Language

GNSS	Global Navigation Satellite System

IHO	International Hydrographic Organization

IMO	International Maritime Organization

ISO	International Organization for Standardization

## Use of language

Within this document:

“Must” indicates a mandatory requirement;

“Should” indicates an optional requirement, that is the recommended process to be followed, but is not mandatory;

“May” means “allowed to” or “could possibly”, and is not mandatory.

## Maintenance

Changes to the Data Classification and Encoding Guide must occur in accordance with the S-247 Differential eLoran Reference Station Almanac Product Specification.

# General

The S-247 Data Classification and Encoding Guide describes how data describing the real world should be captured using the types defined in the S-247 Feature Catalogue. It provides the encoding rules and guidance required to create S-247 datasets. This standard is specifically concerned with those entities in the real world that are of relevance to eLoran. The data model defines real world entities as a combination of descriptive and spatial characteristics.  Within the model these sets of characteristics are defined in terms of feature, spatial and information types.  A type is defined as a stereotype of class that is used to specify a domain of instances (objects) together with the operations applicable to the objects. A type may have attributes and may be related to other types.

## Feature types

Feature types contain descriptive attributes and do not contain any geometry (that is, information about the shape and position of a real world entity).

Features have two aspects – feature type and feature instance.  A feature type is a class and is defined in a Feature Catalogue.  A feature instance is a single occurrence of the feature type and represented as an object in a dataset.  A feature instance is located by a relationship to one or more spatial instances.  A feature instance may exist without referencing a spatial instance.

## Geometric primitives

The allowable geometric primitive for each feature type is defined in the Feature Catalogue.  Within this document, allowable primitives are included in the tables containing a description of each feature type.  Allowable geometric primitives are point. Each spatial value must be referenced by at least one feature instance. Within this document, allowable primitives are included in the description of each feature type.

## Information types

An information type is an identifiable object that can be associated with features in order to carry information particular to the associated features.  An example of the use of an information type may be the requirement to include a note about overhead cables.  Information types can also be associated with other information types.  This may be done where there is further supplementary information that is relevant to the information type. Information types carry attributes but not geometry.

## Attributes

Attributes may be simple type or complex type.  Complex (C) attributes are aggregates of other attributes that can be simple type or complex type.  Simple attributes are assigned to one of 8 types.

The binding of attributes to feature types, the binding of attributes to attributes to construct complex attributes, and attribute multiplicity is defined in the Feature Catalogue.  Within this document, the allowable attributes are included in the description of each feature type, as well as the allowable values for enumeration type attributes.

### Multiplicity

In order to control the number of allowed attribute values or sub-attribute instances within a complex attribute, S-100 uses the concept of multiplicity.  This defines lower and upper limits for the number of values, whether the order of the instances has meaning and if an attribute is mandatory or not.  Common examples are shown in the table below:

Format : MinOccurs, MaxOccurs (if * Infinite) (ordered) – sequential

| Multiplicity | Explanation |
| --- | --- |
| 0,1 | An instance is not required; there can be only one instance. |
| 1,1 | An instance is required and there must only be one instance. |
| 0,* | An instance is not required and there can be an infinite number of instances. |
| 1,* | An instance is required and there can be an infinite number of instances. |
| 1,* (ordered) | An instance is required and there can be an infinite number of instances, the order of which has a specific meaning. |
| 2,2 | Two instances are required and no more than two. |

Note:  The function of the S-57 attribute type “List” has been replaced by Enumeration (EN) with an upper limit of multiplicity greater than 1.  This means that when more than one value is needed for an enumerated attribute, the attribute code is populated multiple times with the required values.

Example:  A red and white tower is encoded with attribute colour = 3 (red) and colour = 1 (white).  Within this document, this example would be indicated as “colour = 3,1”.

### Simple attribute types

Each simple attribute is assigned to one of 9 types:

EN 	Enumeration:  A fixed list of valid identifiers of named literal values.  Attributes of an enumerated type may only take values from this list.

CL	Codelist:  An open enumeration, or the identifier of a vocabulary (mapping between codes, labels and definitions).

BO	Boolean:  A value representing binary logic.  The value can be either True or False.  The default state for Boolean type attributes (that is, where the attribute is not populated for the feature) is False.

RE	Real:  A signed Real (floating point) number consisting of a mantissa and an exponent.  The representation of a real is encapsulation and usage dependent.

Examples:  23.501, -0.0001234, -23.0, 3.141296

IN	Integer:  A signed integer number.  The representation of an integer is encapsulation and usage dependent.

Examples:  29, -65547

TE	Free text:  A CharacterString, that is an arbitrary-length sequence of characters including accents and special characters from a repertoire of one of the adopted character sets.

TD	Truncated Date:  A truncated date allows a partial date to be encoded as an extension to the ISO 8601 compliant date attribute type values for year, month and day according to the Gregorian Calendar.  Character encoding of a date is a string which follows the calendar date format (complete representation, basic format) for date specified by ISO 8601:2004.  See clause X.X.

Example:  19980918 (YYYYMMDD)

TI	Time:  A time is given by an hour, minute and second. Character encoding of a time is a string that follows the local time (complete representation, basic format) format defined in ISO 8601:2004.

Time zone according to UTC is optional.

Example:  183059 or 183059+0100 or 183059Z

The complete representation of the time of 27 minutes and 46 seconds past 15 hours locally in Geneva (in winter one hour ahead of UTC), and in New York (in winter five hours behind UTC), together with the indication of the difference between the time scale of local time and UTC, are used as examples.

Geneva: 152746+0100

New York: 152746-0500

DT	Date and Time:  A DateTime is a combination of a date and a time type. Character encoding of a DateTime shall follow ISO 8601:2004 (see TD and TI above and clause X.X).

Example:  19850412T101530

Real or integer attribute values must not be padded by non-significant zeroes.  For example, for a signal period of 2.5 seconds, the value populated for the attribute signal period must be 2.5 and not 02.50.

### Mandatory and conditional attributes

Some attributes are mandatory and must be populated for a given feature type.  There are some reasons why attribute values may be considered mandatory:

- They are required to support correct portrayal;

- Certain features make no logical sense without specific attributes;

- Some attributes are required for safety of navigation.

Within this document, mandatory attributes (multiplicity 1,1; 1,n (n>1); or 1,*) are identified in the description of each feature type.  For easy reference, Table 2.2 below summarises the mandatory attributes for each feature type (note that mandatory sub-attributes of complex attributes are not included in this table):

### Missing attribute values

Where a value of a mandatory attribute is not known, the attribute must be populated with an empty (null) value.

Where the value of a non-mandatory attribute is not known, the attribute should not be included in the dataset.

In a base dataset, when an attribute code is present but the attribute value is missing, it means that the producer wishes to indicate that this attribute value is unknown.

In an Update dataset, when an attribute code is present but the attribute value is missing it means:

	that the value of this attribute is to be replaced by an empty (null) value if it was present in the original dataset, or

	that an empty (null) value is to be inserted if the attribute was not present in the original dataset.

3 Geo Features

3.1 dLoran Station

| IHO Definition: A place equipped to transmit radio waves. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| S-10x Geo Feature: dLoran Station |  |  |  |  |  |  |
| Super Type: |  |  |  |  |  |  |
| Primitives: point |  |  |  |  |  |  |
| Real World | Paper Chart Symbol |  |  | ECDIS Symbol |  |  |
| S-10x Attribute |  | S-57 Acronym | Allowable Encoding Value |  | Type | Multiplicity |
| ID Code |  |  |  |  | IN | 1, 1 |
| Status |  |  | 1 : permanent 2 : occasional 3 : recommended 4 : notinuse 5 : periodic/intermittent 6 : reserved 7 : temporary 8 : private 9 : mandatory 19 : planned 35 : manual observation 36 : unknown status |  | EN | 0, * |
| Scale Minimum |  |  |  |  | IN | 0, 1 |
| Feature Name |  |  |  |  | C | 0, * |
| Display Name |  |  |  |  | (S) BO | 0, 1 |
| Language |  |  |  |  | (S) TE | 0, 1 |
| Name |  |  |  |  | (S) TE | 1, 1 |
| INT 1 Reference: Remarks: Distinction: |  |  |  |  |  |  |

| Feature/Information associations |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type | Association Name | Association Ends |  |  |  |  |  |
|  |  | Class | Role | Mult | Class | Role | Mult |
| association | Additional Information |  |  |  | ELoranStationAlmanac | referencedBy | 0, 1 |

4 Information types

4.1 dLoran Station Almanac

| IHO Definition: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| S-10x Information Type: dLoran Station Almanac |  |  |  |  |  |  |
| Super Type: |  |  |  |  |  |  |
| Primitives: None |  |  |  |  |  |  |
| Real World | Paper Chart Symbol |  |  | ECDIS Symbol |  |  |
| S-10x Attribute |  | S-57 Acronym | Allowable Encoding Value |  | Type | Multiplicity |
| Name of eLoran Region |  |  |  |  | TE | 0, 1 |
| Issue Number of Data |  |  |  |  | TE | 0, 1 |
| Date Start |  |  |  |  | DA | 0, 1 |
| Number of ASF Maps |  |  |  |  | IN | 1, 1 |
| ID Numbers of ASF Maps |  |  |  |  | TE | 0, * |
| Number of Transmitters |  |  |  |  | IN | 0, 1 |
| Reference Document |  |  |  |  | C | 0, 1 |
| Title |  |  |  |  | (S) TE | 1, 1 |
| Version |  |  |  |  | (S) TE | 1, 1 |
| Name of Issuing Organization |  |  |  |  | C | 0, 1 |
| Language |  |  |  |  | (S) TE | 0, 1 |
| Name |  |  |  |  | (S) TE | 1, 1 |
| Graphic |  |  |  |  | C | 0, 1 |
| Pictorial Representation |  |  |  |  | (S) TE | 1, 1 |
| Source Date |  |  |  |  | (S) DA | 0, 1 |
| Picture Caption |  |  |  |  | (S) TE | 0, 1 |
| Picture Information |  |  |  |  | (S) TE | 0, 1 |
| Information |  |  |  |  | C | 0, 1 |
| File Locator |  |  |  |  | (S) TE | 0, 1 |
| File Reference |  |  |  |  | (S) TE | 0, 1 |
| Headline |  |  |  |  | (S) TE | 0, 1 |
| Language |  |  |  |  | (S) TE | 0, 1 |
| Text |  |  |  |  | (S) TE | 0, 1 |
| dLoran Related Transmitters |  |  |  |  | C | 1, * |
| Designator of Transmitter |  |  |  |  | (S) TE | 0, 1 |
| Norminal ASF Value |  |  |  |  | (S) RE | 0, 1 |
| Norminal ECD Value |  |  |  |  | (S) RE | 0, 1 |
| INT 1 Reference: Remarks: Distinction: |  |  |  |  |  |  |

5 Association Names

5.1 Additional Information

| IHO Definition: an information association for the binding between at least one instance of a geo feature and an instance of an information type. Remarks: •No remarks. |  |  |  |
| --- | --- | --- | --- |
| Role Type | Role | Associated With | Multiplicity |
| Association | Referenced By |  | 0, 1 |
|  | Reference | dLoranv Station |  |

6 Association Roles

6.1 Reference

IHO Definition:

6.2 Referenced By

IHO Definition:

7 Attribute and Enumerate Descriptions

7.1 Name of eLoran Region

IHO Definition: Remarks:

7.2 Issue Number of Data

IHO Definition: Remarks:

7.3 Date Start

IHO Definition: Remarks:

7.4 Number of ASF Maps

IHO Definition: Remarks:

7.5 ID Numbers of ASF Maps

IHO Definition: Remarks:

7.6 Number of Transmitters

IHO Definition: Remarks:

7.7 ID Code

IHO Definition: Remarks:

7.8 Scale Minimum

IHO Definition: Remarks:

7.9 File Locator

IHO Definition: Remarks:

7.10 File Reference

IHO Definition: Remarks:

7.11 Headline

IHO Definition: Remarks:

7.12 Language

IHO Definition: The method of human communication, either spoken or written, consisting of the use of words in a structured and conventional way. Remarks: The language is encoded by a 3 character code following ISO 639-2/T.

7.13 Text

IHO Definition: Remarks:

7.14 Prictorial Representation

IHO Definition: Remarks:

7.15 Source Date

IHO Definition: Remarks:

7.16 Picture Caption

IHO Definition: Remarks:

7.17 Picture Information

IHO Definition: Remarks:

7.18 Name

IHO Definition: Remarks:

7.19 Norminal ASF Value

IHO Definition: Remarks:

7.20 Norminal ECD Value

IHO Definition: Remarks:

7.21 Pictorial Representation

IHO Definition: Remarks:

7.22 Designator of Transmitter

IHO Definition: Remarks:

7.23 Display Name

IHO Definition: Remarks:

7.24 Title

IHO Definition: Remarks:

7.25 Version

IHO Definition: Remarks:

7.26 Status

IHO Definition: 1) permanent IHO Definition: 2) occasional IHO Definition: 3) recommended IHO Definition: 4) notinuse IHO Definition: 5) periodic/intermittent IHO Definition: 6) reserved IHO Definition: 7) temporary IHO Definition: 8) private IHO Definition: 9) mandatory IHO Definition: 19) planned IHO Definition: 35) manual observation IHO Definition: 36) unknown status IHO Definition: Remarks:

7.27 Differential Correction Flag

IHO Definition: 1) relative IHO Definition: 2) absolute IHO Definition: Remarks:

7.28 Category of Radio Station

IHO Definition: 21) dGNSS Station IHO Definition: 22) dLoran Station IHO Definition: 23) eLoran Station IHO Definition: Remarks:

8 Complex Attributes

8.1 Information

IHO Definition: Textual information about the feature. The information may be provided as a string of text or as a file name of a single external text file that contains the text. Sub-attributes: File Locator (see clause 7.8) File Reference (see clause 7.9) Headline (see clause 7.10) Language (see clause 7.11) Text (see clause 7.12) Remarks: At least one of the sub-attributes file reference or text must be populated.The sub-attribute file reference is generally used for long text strings or those that require formatting, however, there is no restriction on the type of text (except for lexical level) that can be held in files referenced by sub-attribute file reference.

8.2 Graphic

IHO Definition: Pictorial information such as a photograph, sketch or other graphic, optionally accompanied by descriptive information about the graphic and the location relative to its subject from which it was made. Sub-attributes: Pictorial Representation (see clause 7.20) Source Date (see clause 7.14) Picture Caption (see clause 7.15) Picture Information (see clause 7.16) Remarks: •No remarks.

8.3 Name of Issuing Organization

IHO Definition: Sub-attributes: Language (see clause 7.11) Name (see clause 7.17) Remarks: •No remarks.

8.4 dLoran Related Transmitters

IHO Definition: Sub-attributes: Designator of Transmitter (see clause 7.21) Norminal ASF Value (see clause 7.18) Norminal ECD Value (see clause 7.19) Remarks: •No remarks.

8.5 Reference Document

IHO Definition: Sub-attributes: Title (see clause 7.23) Version (see clause 7.24) Remarks: •No remarks.

8.6 Feature Name

IHO Definition: Provides the name of an entity, defines the national language of the name, and provides the option to display the name at various system display settings. Sub-attributes: Display Name (see clause 7.22) Language (see clause 7.11) Name (see clause 7.17) Remarks: •No remarks.