IALA S-201

Product Specification

Edition 1.1.0 – October 2022

IALA AtoN Product Specification

ANNEX A

Data Classification Encoding Guide (DCEG)

# Overview

## Preface

The “Data Classification and Encoding Guide” has been developed to provide consistent, standardized instructions for encoding S-100 compliant AtoN data. The purpose of the Data Classification and Encoding Guide is to facilitate S-201 encoding. The document describes how to encode information that the AtoN service provider considers relevant to an AtoN. A “AtoN service provider” is a AtoN authority or an organization authorized by a government or other relevant government institution.

## S-201 Appendix A; Data Classification and Encoding Guide - Metadata

Note:  This information uniquely identifies this Appendix to the Product Specification and provides information about its creation and maintenance.

Title:	The International Association of Lighthouse Authorities Aids To Navigation Product Specification, Appendix A – Data Classification and Encoding Guide

Version:	1.1.0

Date:		October 2022

Language:	English

Classification:	Unclassified

Contact:	International Association of Lighthouse Authorities

10 rue des Gaudines,

78100

St Germain en Laye.

Paris, France

Telephone: +33 134 51 70 01

URL:		http://www.iala-aism.org/

Identifier:	S-201 Appendix A

Maintenance:	Changes to S-201 Appendix A; Data Classification and Encoding Guide are coordinated by the IALA e-Nav Committee WG1

## Terms, definitions and abbreviations

### Terms and definitions

aggregation

special form of association that specifies a whole-part relationship between the aggregate (whole) and a component (see composition)

attribute

named property of an entity

NOTE:  Describes the geometrical, topological, thematic, or other characteristic of an entity

feature

Abstraction of real world phenomena

NOTE:  A feature may occur as a type or an instance.  The terms “feature type” or “feature instance” should be used when only one is meant

EXAMPLE:  The feature instance named “Eiffel Tower” may be classified with other phenomena into a feature type “tower”

enumeration

A fixed list of valid identifiers of named literal values. Attributes of an enumerated type may only take values from this list (source: ISO 19136:XX, Geographic information — Geography Markup Language (GML))

geometric primitive

geometric object representing a single, connected, homogeneous element of geometry

NOTE:  Geometric primitives are non-decomposed objects that present information about geometric configuration.  They include points, curves, surfaces and solids

### Abbreviations

AtoN	Aids To Navigation

IALA	International Association of Lighthouse Authorities

IHO	International Hydrographic Organization

## Use of language

Within this document:

“Must” indicates a mandatory requirement;

“Should” indicates and optional requirement, that is the recommended process to be followed, but is not mandatory;

“May” means “allowed to” or “could possibly”, and is not mandatory.

## Maintenance

Changes to the Data Classification and Encoding Guide must occur in accordance with the S-201 AtoN Product Specification clause 1.5.1.

# General

The S-201 Data Classification and Encoding Guide describes how data describing the real world should be captured using the types defined in the S-201 Feature Catalogue. It provides the encoding rules and guidance required to create S-201 AtoNs.  This standard is specifically concerned with those entities in the real world that are of relevance to Aids to Navigation. This concept is considered to be geo-spatial.  As a result, the model defines real world entities as a combination of descriptive and spatial characteristics.  Within the model these sets of characteristics are defined in terms of feature, spatial. A type is defined as a stereotype of class that is used to specify a domain of instances (objects) together with the operations applicable to the objects. A type may have attributes and may be related to other types.

## Feature types

Feature types contain descriptive attributes and do not contain any geometry (i.e. information about the shape and position of a real world entity).

Features have two aspects – feature type and feature instance.  A feature type is a class and is defined in a Feature Catalogue.  A feature instance is a single occurrence of the feature type and represented as an object in a data set.  A feature instance is located by a relationship to one or more spatial instances.  A feature instance may exist without referencing a spatial instance.

## Spatial primitives

The allowable spatial primitive for each feature type is defined in the Feature Catalogue.  Within this document, allowable primitives are included in the tables containing a description of each feature type.  Allowable spatial primitives are point, curve and surface.

Each spatial value must be referenced by a feature instance.

## Information types

An information type is an identifiable object that can be associated with features in order to carry information particular to the associated features. Information types can also be associated with other information types.  This can be done where there is further supplementary information that is relevant to the information type. Information types carry attributes but not geometry.

## Attributes

Attributes may be simple type or complex type.  Complex (C) attributes are aggregates of other attributes that can be simple type or complex type.  Simple attribute types consist of Text (TE), Date (DA), Time (TI), DateTime (DT), Enumeration (EN), Boolean (BO), Integer (IN) and Real (RE) (see clause X.X).

The binding of attributes to feature types, the binding of attributes to attributes to construct complex attributes, and attribute multiplicity is defined in the Feature Catalogue.  Within this document, the allowable attributes is included in the description of each feature type, as well as the allowable values for enumeration type attributes.

Real or integer attribute values must not be padded by non-significant zeroes.  For example, for a signal period of 2.5 seconds, the value populated for the attribute signal period must be 2.5 and not 02.50.

### Multiplicity

In order to control the number of allowed attribute values or sub attribute instances within a complex attribute, S-100 uses the concept of multiplicity.  This defines lower and upper limits for the number of values, whether the order of the instances has meaning and if an attribute is mandatory or not.  Common examples are shown in the table below:

Format : MinOccurs, MaxOccurs (if * Infinite) ordered – sequential

| Multiplicity | Explanation |
| --- | --- |
| 0,1 | An instance is not required; there can be only one instance. |
| 1,1 | An instance is required and there must only be one instance. |
| 0,* | An instance is not required and there can be an infinite number of instances. |
| 1,* | An instance is required and there can be an infinite number of instances. |
| 1,* ordered | An instance is required and there can be an infinite number of instances, the order of which has a specific meaning. |
| 2,2 | Two instances are required and no more than two. |

Example:  A red and white tower is encoded with attribute colour = 3 (red) and colour = 1 (white).

### Simple attribute types

Each simple attribute is assigned to one of 8 types:

EN 	Enumeration:  A fixed list of valid identifiers of named literal values.  Attributes of an enumerated type may only take values from this list.

BO	Boolean:  A value representing binary logic.  The value can be either True or False.  The default state for Boolean type attributes (i.e. where the attribute is not populated for the feature) is False.

RE	Real:  A signed Real (floating point) number consisting of a mantissa and an exponent.  The representation of a real is encapsulation and usage dependent.

Examples:  23.501, -0.0001234, -23.0, 3.141296

IN	Integer:  A signed integer number.  The representation of an integer is encapsulation and usage dependent.

Examples:  29, -65547

TE	Free text:  A CharacterString, that is an arbitrary-length sequence of characters including accents and special characters from a repertoire of one of the adopted character sets.

DA	Date:  A date provides values for year, month and day according to the Gregorian Calendar.  Character encoding of a date is a string which must follow the calendar date format (complete representation, basic format) for date specified by ISO 8601:1988.  See clause X.X.

Example:  19980918 (YYYYMMDD)

TI	Time:  A time is given by an hour, minute and second. Character encoding of a time is a string that follows the local time (complete representation, basic format) format defined in ISO 8601:1988.

Time zone according to UTC is optional.

Example:  183059 or 183059+0100 or 183059Z

The complete representation of the time of 27 minutes and 46 seconds past 15 hours locally in Geneva (in winter one hour ahead of UTC), and in New York (in winter five hours behind UTC), together with the indication of the difference between the time scale of local time and UTC, are used as examples.

Geneva: 1527460100

New York: 1527460500

DT	Date and Time:  A DateTime is a combination of a date and a time type. Character encoding of a DateTime shall follow ISO 8601:1988 (see DA and TI above and clause X.X).

Example:  19850412T101530

CL(OE)	Codelist (open enumeration): A fixed list of valid identifiers of named literal values.  Attributes of an enumerated type may take values from this list or the pattern “other: ...” i.e., the word ‘other’ followed by a colon and a single space character, followed by one or more alphanumeric strings separated by single spaces.

Example:  other: loxodromic

### Mandatory  and conditional attributes

Some attributes are mandatory and must be populated for a given feature type.  There are some reasons why attribute values may be considered mandatory:

- They are required to support correct portrayal;

- Certain features make no logical sense without specific attributes;

- Some attributes are required for safety of navigation.

Where a value of a mandatory attribute is not known, the attribute should be populated with an empty (null) value. Where the value of a non-mandatory attribute is not known, the attribute should not be populated (i.e. not included in the dataset). Within this document, mandatory attributes (multiplicity 1,1; 1,n (n>1); or 1,*) are identified in the description of each feature type. Missing attribute values

Where a value of a mandatory attribute is not known, the attribute should be populated with an empty (null) value.

Where the value of a non-mandatory attribute is not known, the attribute should not be included in the dataset.

In a base dataset (EN application profile), when an attribute code is present but the attribute value is missing, it means that the producer wishes to indicate that this attribute value is unknown.

In an Update dataset (ER application profile), when an attribute code is present but the attribute value is missing it means:

	that the value of this attribute is to be replaced by an empty (null) value if it was present in the original dataset, or

	that an empty (null) value is to be inserted if the attribute was not present in the original dataset.

### Spatial attribute types

Spatial attribute types must contain a referenced geometry and may be associated with spatial quality attributes.  Each spatial attribute instance must be referenced by a feature instance or another spatial attribute instance.

Spatial quality attributes are carried in an information class called Spatial quality. Only points, multipoints and curves can be associated with Spatial quality.  Currently no use case for associating surfaces with spatial quality attributes is known, therefore this is prohibited.  Vertical uncertainty is prohibited for curves as this dimension is not supported by curves.

### Dates

When encoding dates using the attributes dredged date, fixed date range, periodic date range, reported date, survey date range and swept date, and no specific year, month or day is required, the following values must apply in conformance to ISO 8601:1988.

- No specific year required, same day each year:	--MMDD

- No specific year required, same month each year:	--MM

- No specific day required:	CCYYMM

- No specific month required:	CCYY

Notes:	CCYY = calendar year; MM = month; DD = day.

In the first two values, the dashes (--) must be included.

#### Seasonal features

If it is required to show seasonality of features, it must be done using the attribute Status = 5 (periodic/intermittent).  If it is required to encode the start and/or end dates of the season, this must be done using the complex attribute periodic date range.

### Times

If it is required to show the beginning and end of the active period of a feature, it must be encoded using the complex attribute time range, sub-attributes time end and time start.  When using these sub-attributes, all times must be encoded as Coordinated Universal Time (UTC).  The attribute descriptions for time end and time start (see clauses X.X) states that the mandatory format is CCYYMMDDThhmmss, where T is the separator, and this format must be used.

### Colours and colour patterns

If it is required to encode multiple colours on a feature, they must be encoded using the attributes colour pattern and colour as follows:

- For horizontal stripes (colour pattern = 1), the first colour in the list must be the top-most, and subsequent colours follow sequentially from top to bottom.  For example, colour = 3,1 to encode a red stripe above a white stripe.

- For vertical stripes (colour pattern = 2), the first colour in the list must be the left-most, and subsequent colours follow sequentially from left to right.  For example, colour = 3,1,3 to encode red, white, red vertical stripes

- For diagonal stripes (colour pattern = 3), the first colour in the list must be the top-left-most, and subsequent colours follow sequentially from top left to bottom right. For example, colour = 1,3,1,3,1 to encode white, red, white, red, white diagonal stripes.

- For squares (colour pattern = 4), the first colour in the list must be the top-left-most square.  Subsequent colours follow sequentially from left to right along the top row then repeated for subsequent rows until the bottom right-most square is reached.  For example, colour = 1,3,3,1 to encode white, red squares on the top row and red, white squares on the bottom row.

- For border stripes (colour pattern = 6), the first colour in the list must be the border stripe, the second colour must be that of the background. For example, colour = 3,1 to encode a  red border stripe on a white background.  Where a border stripe is combined with other patterns, the border stripe colour must be the first colour in the list, and subsequent colours must be interpreted in accordance with the rules defined for the additional patterns.  Therefore, if a pattern contains a border stripe as well as other patterns, the border stripe must be the first value in the list of colour pattern.

Note that the attribute colour pattern is mandatory for any feature (except Light) that has more than one colour.

### Radar conspicuous features (see S-4 – B-485.2)

The attribute radar conspicuous is used to encode whether or not a feature is radar conspicuous.

Remarks:

- If it is required to encode a feature which has no radar reflector, but is radar conspicuous, it must be indicated using attribute radar conspicuous.

- If it is required to encode a surface or point feature which is radar conspicuous because it is fitted with a radar reflector, it must be indicated using radar conspicuous on the feature.

- If it is required to encode radar reflectors on curve features (e.g. overhead cables), this must be done using the feature Radar Reflector (see clause X.X).

## Datasets

A Dataset is a grouping of features, attributes, geometry and metadata which comprises a specific coverage.

## Description of table format for S-201 meta and geo features

X.X	Clause heading

| IHO Definition: FEATURE: Definition. (Authority for definition). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Feature S-201 feature |  |  |  |  |  |
| Primitives: Point, Curve, Surface Allowable geometric primitive(s) |  |  |  |  |  |
| Real World Example if real world instance(s) of the Feature. | Paper Chart Symbol Example(s) of paper chart equivalent symbology for the Feature. |  | ECDIS Symbol Example(s) of ECDIS symbology for the Feature. |  |  |
| S-201 Attribute |  | Allowable Encoding Value * |  | Type | Multiplicity |
| Category of beer |  | 1 : ale 2 : lager 3 : porter 4 : stout 5 : pilsener |  | EN | 1,1 |
| This section lists the full list of allowable attributes for the S-201 feature. Attributes are listed in alphabetical order. Sub-attributes (Type prefix (S)) of complex (Type C) attributes are listed in alphabetical order and indented directly under the entry for the complex attribute (see below for example). |  | This section lists the allowable encoding values for S-201 (for enumerate (E) Type attributes only). Further information about the attribute is available in Section XX. |  | Attribute type (see clause X.X). | Multiplicity describes the “cardinality” of the attribute in regard to the feature. See clause X.X. |
| Fixed date range |  |  |  | C | 0,1 |
| Date end |  |  |  | (S) DA | 0,1 |
| Date start |  |  |  | (S) DA | 0,1 |
| X Sub-clause heading(s) Introductory remarks. Includes information regarding the real world entity/situation requiring the encoding of the Feature, and where required nautical cartographic principles relevant to the Feature to aid the compiler in determining encoding requirements. Specific instructions to encode the feature. Remarks: - Additional encoding guidance relevant to the feature. X.X Sub-sub-clause heading(s) Clauses related to specific encoding scenarios for the Feature. (Not required for all Features). Remarks: - Additional encoding guidance relevant to the scenario (only if required). Distinction: List of features in the Product Specification distinct from the Feature. |  |  |  |  |  |

*  For (EN) type attributes, the enumerates listed are only those allowable for the particular occurrence of the attribute relevant to the feature.  Allowable values may vary for the attribute depending on the feature to which the attribute is bound.  Such bindings are defined in the S-201 Feature Catalogue.  The full list of enumerates that may be assigned to an attribute in S-201 can be found in Section XX – Attribute and Enumerate Descriptions – of this document.

# Aids to Navigation - Overview

## Geo features forming parts of navigational aids

Aids to navigation are composed of fixed or floating structure features carrying equipment features.

The most common structure features are:  Beacon Cardinal,  Beacon Isolated Danger, Beacon Lateral, Beacon Safe Water, Beacon Special Purpose/General,  Buoy Cardinal, Buoy Installation, Buoy Isolated Danger, Buoy Lateral, Buoy Safe Water, Buoy Special Purpose/General, Daymark, Light Float, Light Vessel, Landmark, Offshore Platform, Pile, SiloTank

Equipment features consist of:  Daymark, Fog Signal, Light, Environment Observation Equipment, Retroreflector,  Radar Transponder Beacon, Topmark.

Where the source indicates the existence of a navigation aid equipment feature, but there is no indication as to the type or shape of the structure feature, a Beacon Special Purpose feature, with the attribute beacon shape populated with an empty (null) value, must be encoded as the master feature (see clause X.X).

Radar reflectors must not be encoded as separate features when attached to navigational aids.  If it is required to encode their existence, it must be done using the attribute radar conspicuous.

Navigational Aid

Structure object (Parent)

Equipment objects (Child)

Marked objects

Light

Fog Signal

Association

Relationships

A parent to child relationship must be created in order to relate the different features comprising a navigational aid.  Where a parent to child relationship is created, there must be only one parent (structure) feature related to one or more child (equipment) features.  A child feature must not be related to more than one parent  feature, and a feature must not be both a parent and a child feature.

Navigational Aids – Parent / Child Relationship

When the navigational aid contains a structure feature, this feature must be the parent feature, and the equipment features must be the child.  Note that Daymark may be a parent feature or a child feature; where a navigational aid contains a Daymark and there is no other base structure (which can serve as the parent  feature) indicated on the source, the Daymark feature should be encoded as the parent feature.

When the nature of the base structure on land is unknown or there is no structure feature, one of the equipment features may be chosen as the parent feature, giving priority to a Light feature, if one exists. Alternatively, a Pile feature of type point or a Beacon Special Purpose/General feature may be encoded as the structure feature at the same position as the equipment features.  When the nature of the base structure in the water is unknown, an ECDIS Base Display feature (see S-52, Annex A, clause 13.2), e.g. Pile feature of type point or a Beacon Special Purpose/General feature, must be encoded as the structure feature at the same position as the equipment features.

Navigational Aids – Parent / Child Relationship:  Real World Examples

In the above real world examples, the parent and child features that make up the navigational aids are point spatial types, and they must share the same geographic point spatial type.

If it is required to encode the name of the navigational aid, it must be done using the complex attribute feature name on the parent feature.  The name must not be repeated for the child features.  If the name is painted on the structure, it must be encoded with the same spelling in the complex attribute feature name (sub-attribute name, no value populated for sub-attribute language) if it is based on the Latin alphabet.  If the name is not based on the Latin alphabet, it must be encoded using feature name (name), with an appropriate value populated for the sub-attribute language, and transliterated for encoding on an iteration of feature name (name) with no value populated for language.

All point features comprising a navigational aid must share the same geographic point spatial instance.

The navigational aid may be associated with the features which it marks using the collection feature Association (see clause XX).  Several navigational aids and several marked features may be associated in the same relationship.

## Buoyage systems and direction of buoyage

Systems of buoyage are described as lateral, cardinal, or a combination of lateral and cardinal.  Lateral systems depend on a direction of buoyage being defined.  The cardinal system depends solely on the main points of the compass.  Special purpose buoys often mark the limits or centre of an area (e.g. an exercise area, a dumping ground) and do not necessarily have lateral or cardinal system characteristics.

The IALA Maritime Buoyage System details, including the extent of Regions A and B, are given in other publications (e.g. UK’s booklet NP 735 “IALA Maritime Buoyage System”).  Although it is called a buoyage system, it applies to all fixed and floating marks except lighthouses, some sector lights, leading lights and marks, major floating lights and lights on offshore structures.  Five types of marks are provided by the system: Lateral, Cardinal, Isolated danger, Safe water and Special marks, which may be used in any combination.  Emergency Wreck Marking Buoys were added on a trial basis in 2006.

### Buoyage systems and direction of buoyage

The buoyage system of the dataset and, where necessary, the direction of buoyage, must be encoded using the meta feature Navigational System of Marks:

All parts of the dataset containing data must be covered by Navigational System of Marks features, with the attribute marks navigational – system of indicating the buoyage system in operation.  Navigational System of Marks features with a value encoded for marks navigational – system of must not overlap.

Within a dataset, there may be some areas where the direction of buoyage is defined by local rules and must, therefore, be specified.  These areas should be encoded as separate Navigational System of Marks surface features, with the complex attribute orientation indicating the direction of buoyage (marks navigational – system of must not be encoded for these features).  Navigational System of Marks features with a value encoded for orientation (orientation value) must not overlap, but in areas where local buoyage directions apply, Navigational System of Marks with a value encoded for orientation (orientation value) may overlap Navigational System of Marks with a value encoded for marks navigational – system of (see Figure below).

Area (b) overlaps area (a)

Navigational System of Marks (surface) (b)

orientation (mandatory)

Navigational System of Marks (surface) (a)

marks navigational –  system of (mandatory)

Buoyage system and direction

Individual buoys and beacons may not be part of the general buoyage system.  This should be encoded using marks navigational – system of on these buoy and beacon features.

Remarks:

- At least one of the attributes marks navigational – system of or orientation must be populated for Navigational System of Marks features.

#### Encoding IALA marks within IALA A or B

In the following tables the symbol ‘/’ indicates that this attribute is not relevant for that particular feature.  The tables contain the most common examples of encoding; other encoding combinations are possible.  For encoding of buoys, substitute Buoy for Beacon in the Feature column.

| Real World Feature | Feature | category of cardinal mark | colour | colour pattern | marks navigational – system of |
| --- | --- | --- | --- | --- | --- |
| North cardinal beacon | Beacon, Cardinal | 1 | 2,6 | 1 | 1 and 2 (IALA A and B) |
| East cardinal beacon | Beacon, Cardinal | 2 | 2,6,2 | 1 | 1 and 2 (IALA A and B) |
| South cardinal beacon | Beacon, Cardinal | 3 | 6,2 | 1 | 1 and 2 (IALA A and B) |
| West cardinal beacon | Beacon, Cardinal | 4 | 6,2,6 | 1 | 1 and 2 (IALA A and B) |
| Real World Feature | Feature | category of lateral mark | colour | colour pattern | marks navigational – system of |
| Port lateral beacon | Beacon, Lateral | 1 | 3 | / | 1 (IALA A) |
| Starboard lateral beacon | Beacon, Lateral | 2 | 4 | / | 1 (IALA A) |
| Preferred channel to starboard lateral beacon | Beacon, Lateral | 3 | 3,4,3 | 1 | 1 (IALA A) |
| Preferred channel to port lateral beacon | Beacon, Lateral | 4 | 4,3,4 | 1 | 1 (IALA A) |
| Port lateral beacon | Beacon, Lateral | 1 | 4 | / | 2 (IALA B) |
| Starboard lateral beacon | Beacon, Lateral | 2 | 3 | / | 2 (IALA B) |
| Preferred channel to starboard lateral beacon | Beacon, Lateral | 3 | 4,3,4 | 1 | 2 (IALA B) |
| Preferred channel to port lateral beacon | Beacon, Lateral | 4 | 3,4,3 | 1 | 2 (IALA B) |

| Real World Feature | Feature | colour | colour pattern | marks navigational – system of |
| --- | --- | --- | --- | --- |
| Isolated danger beacon | Beacon, Isolated Danger | 2,3,2 | 1 | 1 and 2 (IALA A and B) |
| Safe water beacon | Beacon, Safe Water | 3,1 or 1,3 | 2 | 1 and 2 (IALA A and B) |
| Special purpose beacon | Beacon, Special Purpose | 6 | / | 1 and 2 (IALA A and B) |
| Emergency wreck marking buoy | Buoy, Emergency Wreck Marking | 5,6 | 2 | 1 or 2 (IALA A or B) |

| Real World Feature | Feature | topmark / daymark shape | colour | colour pattern | marks navigational – system of |
| --- | --- | --- | --- | --- | --- |
| North cardinal topmark | Topmark | 13 | 2 | / | 1 and 2 (IALA A and B) |
| East cardinal topmark | Topmark | 11 | 2 | / | 1 and 2 (IALA A and B) |
| South cardinal topmark | Topmark | 14 | 2 | / | 1 and 2 (IALA A and B) |
| West cardinal topmark | Topmark | 10 | 2 | / | 1 and 2 (IALA A and B) |
| Isolated danger topmark | Topmark | 4 | 2 | / | 1 and 2 (IALA A and B) |
| Port lateral topmark | Topmark | 5 | 3 | / | 1 (IALA A) |
| Starboard lateral topmark | Topmark | 1 | 4 | / | 1 (IALA A) |
| Port lateral topmark | Topmark | 5 | 4 | / | 2 (IALA B) |
| Starboard lateral topmark | Topmark | 1 | 3 | / | 2 (IALA B) |
| Safe water topmark | Topmark | 3 | 3 | 2 | 1 and 2 (IALA A and B) |
| Special purpose topmark | Topmark | 7 | 6 | / | 1 and 2 (IALA A and B) |
| Emergency wreck marking topmark | Topmark | 8 | 6 | / | 1 or 2 (IALA A or B) |

# AtoN Fixed structures

If it is required to encode a beacon, it must be done using the object classes Beacon cardinal, Beacon Isolated danger, Beacon lateral, Beacon safe water or Beacon special purpose.

Remarks:
 If it is required to encode the altitude of the ground level above the vertical datum at the position of
a beacon, it must be done using the attribute Elevation, but only for beacons built on land.

 If it is required to encode the total altitude of a beacon, including any equipment objects (e.g.
topmark, light), above the vertical datum, it must be done using the attribute Height.

 If it is required to encode the total vertical length of a beacon, including any equipment objects
(e.g. topmark, light), above the seabed or ground, it must be done using the attribute Vertical length.

 The attribute value Beacon shape = 1 (stake, pole, perch, post) implies that
stakes, poles, perches and posts must be embedded in the water. Encoders should note that
stakes, poles, perches or posts may also exist on land, and where this is the case, they should be
encoded using the relevant beacon object class.

 If it is required to encode a cairn that bears the colour(s) specified by a navigational system of
marks, it must be done using a beacon object.

 If it is required to encode a beacon that has more than one colour, the attributes Colour and
Colour pattern must be encoded, according to the rules

In the following table, the symbol ‘/’ indicates that this attribute does not exist for that particular object
class. A blank indicates that the encoder may choose a relevant value for the attribute. The table
contains the most common examples of coding; other coding combinations are possible.

| Real World Feature |  | Feature |  | Beacon shape |  | Category of special purpose mark |  | Other attributes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Minor not permanent mark |  | Beacon *** |  | 1 |  |  |  |  |
|  | Beacon Lateral |  | 1 |  | / |  |  |  |
|  | Beacon Lateral |  | 2 |  | / |  |  |  |
| Cairn |  | Beacon *** |  | 6 |  |  |  |  |
| Coloured or white mark |  | Daymark |  | / |  |  |  | Nature of construction = 9 |
| Coloured topmark with function of beacon |  | Daymark |  | / |  |  |  | Nature of construction = 9 |
| Painted board with function of leading beacon |  | Daymark |  | / |  | 16 |  | Nature of construction = 9, Topmark/daymark shape = 6 |
| Beacon tower |  | Beacon *** |  | 3 |  |  |  |  |
| Lattice beacon |  | Beacon *** |  | 4 |  |  |  |  |
| Leading beacon |  | Beacon Special Purpose |  |  |  | 16 |  |  |
| Beacon marking a clearing line |  | Beacon Special Purpose |  |  |  | 41 |  |  |
| Beacon marking measured distance |  | Beacon Special Purpose |  |  |  | 17 |  |  |
| Cable landing beacon |  | Beacon Special Purpose |  |  |  | 6 |  |  |
| Outfall landing beacon |  | Beacon Special Purpose |  |  |  | 8 |  |  |
| Pipeline landing beacon |  | Beacon Special Purpose |  |  |  | 39 |  |  |
| Refuge beacon |  | Beacon Special Purpose |  |  |  | 44 |  |  |
| Firing practice area beacon |  | Beacon Special Purpose |  |  |  | 1 |  |  |
| Notice board |  | Beacon Special Purpose |  |  |  | 18 |  |  |
| Buoyant beacon |  | Beacon *** |  | 7 |  |  |  |  |
|  |  |  |  |  |  |  |  | table 12.2 |

## Beacon, Special Purpose/General (BeaconSpecialPurposeGeneral)

| Definition: BEACON SPECIAL PURPOSE/GENERAL. A beacon is a prominent specially constructed object forming a conspicuous mark as a fixed aid to navigation or for use in hydrographic survey. (IHO Dictionary – S-32). A special purpose beacon is primarily used to indicate an area or feature, the nature of which is apparent from reference to a chart, Sailing Directions or Notices to Mariners. (UKHO NP 735, 5th Edition). Beacon in general: A beacon whose appearance or purpose is not adequately known. (S-57 Edition 3.1, Appendix A – Chapter 1, Page 1.12, November 2000). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Beacon Special Purpose/General |  |  |  |  |  |
| Primitives: Point |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Beacon shape |  | 1 : stake, pole, perch, post 2 : withy 3 : beacon tower 5 : pile beacon 6 : cairn 7 : buoyant beacon |  | EN | 1,1 |
| Category of special purpose mark |  | 1 : firing danger area mark 2 : target mark 3 : marker ship mark 4 : degaussing range mark 5 : barge mark 6 : cable mark 7 : spoil ground mark 8 : outfall mark 10 : recording mark 11 : seaplane anchorage mark 12 : recreation zone mark 14 : mooring mark 15 : LANBY (Large Automatic Navigational Buoy) 16 : leading mark 17 : measured distance mark 18 : notice mark 19 : TSS mark (Traffic Separation Scheme) 20 : anchoring prohibited mark 21 : berthing prohibited mark 22 : overtaking prohibited mark 23 : two-way traffic prohibited mark 24 : reduced wake mark 25 : speed limit mark 26 : stop mark 27 : general warning mark 28 : sound ship’s siren mark 29 : restricted vertical clearance mark 30 : maximum vessel’s draught mark 31 : restricted horizontal clearance mark 32 : strong current warning mark 33 : berthing permitted mark 34 : overhead power cable mark 35 : channel edge gradient’ mark 36 : telephone mark 37 : ferry crossing mark 39 : pipeline mark 40 : anchorage mark 41 : clearing mark 42 : control mark 43 : diving mark 44 : refuge beacon 45 : foul ground mark 46 : yachting mark 47 : heliport mark 48 : GNSS mark 49 : seaplane landing mark 50 : control mark 51 : work in progress mark 52 : mark with unknown purpose 53 : wellhead mark 54 : channel separation mark 55 : marine farm mark 56 : artificial reef mark 57 : ice mark 58 : nature reserve mark |  | EN | 1,* |
| Colour |  | 1 : white 2 : black 3 : red 4 : green 5 : blue 6 : yellow 7 : grey 8 : brown 9 : amber 10 : violet 11 : orange 12 : magenta 13 : pink |  | EN | 1,* |
| Colour pattern |  | 1 : horizontal stripes 2 : vertical stripes 3 : diagonal stripes 4 : squared 5 : stripes (direction unknown) 6 : border stripe |  | EN | 0,1 |
| Condition |  | 1 : under construction 2 : ruined 5 : planned construction |  | EN | 0,1 |
| Radar conspicuous |  |  |  | BO | 0,1 |
| Visually conspicuous |  |  |  | BO | 0,1 |
| Elevation |  |  |  | RE | 0,1 |
| Height |  |  |  | RE | 0,1 |
| Marks navigational – system of |  | 1 : IALA A 2 : IALA B 9 : no system 10 : other system 11 : CEVNI |  | EN | 0,1 |
| Nature of construction |  | 1 : masonry 2 : concreted 6 : wooden 7 : metal 8 : glass reinforced plastic (GRP) 9 : painted 10 : latticed |  | EN | 0,* |
| Vertical length |  |  |  | RE | 0,1 |
| Vertical accuracy |  |  |  | RE | 0,1 |
| Object name in national language |  |  |  | (S) TE | 0,1 |
| Object name |  |  |  | (S) TE | 0,1 |
| Status |  | 1 : permanent 2 : occasional 4 : not in use 5 : periodic/intermittent 7 : temporary 8 : private 12 : illuminated 18 : existence doubtful |  | EN | 0,* |
| Aid availability category |  | 1 : Category 1 2 : Category 2 3 : Category 3 |  | EN | 1,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Textual description in national language |  |  |  | (S) TE | 0,1 |
| Textual description |  |  |  | (S) TE | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  |  | TE | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| ### Special purpose/general beacons Special beacons are used to indicate to the mariner a special area or feature, the nature of which is usually apparent from the chart or associated publication. To conform to the IALA Maritime Buoyage System (see clause X.X), the body of the beacon is yellow. The topmark (if fitted) is a yellow diagonal ‘X’ (St Andrew’s cross). Lights (if fitted) are yellow and of any rhythm except those used for cardinal, isolated danger and safe water marks. If it is required to encode a beacon having the function of a special purpose mark, or a beacon whose appearance or purpose is inadequately known, it must be done using the feature Beacon Special Purpose/General. In the following table, a blank indicates that the encoder may choose a relevant value for the attribute. The table contains the most common examples of coding; other coding combinations are possible. Feature; INT1; Feature; beacon shape; category of special purpose mark; Other attributes. Minor not permanent mark; Q90; Beacon ***; 1. Cairn; Q100; Beacon ***; 6. Beacon tower; Q110; Beacon ***; 3. Lattice beacon; Q111; Beacon ***; 4. Leading beacon; Q120; Beacon Special Purpose/General; 16. Beacon marking a clearing line; Q121; Beacon Special Purpose/General; 41. Beacon marking measured distance; Q122; Beacon Special Purpose/General; 17. Cable landing beacon; Q123; Beacon Special Purpose/General; 6. Outfall landing beacon; Q123; Beacon Special Purpose/General; 8. Pipeline landing beacon; Q123; Beacon Special Purpose/General; 39. Refuge beacon; Q124; Beacon Special Purpose/General; 44. Firing practice area beacon; Q125; Beacon Special Purpose/General; 1. Notice board; Q126; Beacon Special Purpose/General; 18. Buoyant beacon; P5; Beacon Special Purpose/General; 7. Remarks: - The attribute colour pattern must be populated if more than one value is populated for the mandatory attribute colour. - If it is required to encode the altitude of the ground level above the vertical datum at the position of a beacon, it must be done using the attribute elevation, but only for beacons built on land. - If it is required to encode the total altitude of a beacon, including the topmark and any equipment features (e.g. light), above the vertical datum, it must be done using the attribute height. - If it is required to encode the total vertical length of a beacon, including the topmark and any equipment features (e.g. light), above the seabed or ground, it must be done using the attribute vertical length. - If it is required to encode a cairn that bears the colour(s) specified by a navigational system of marks, it must be done using a beacon feature. ### Signs and notice boards If it is required to encode a fixed or floating sign or notice board, it must be done using a Beacon Special Purpose/General feature or Buoy Special Purpose/General feature (see clause X.X), with attribute category of special purpose mark = 18 (notice mark), or using the feature Daymark (see clause X.X). Remarks: - If it is required to encode a sign or notice board that has more than one colour, the attributes colour and colour pattern must be used, according to the rules laid out in clause X.X. - If it is required to encode any text shown on a notice board or sign, it must be done using the complex attribute information. - If it is required to encode the shape and colour of a notice board, it must be done by encoding the board as a Daymark feature. Distinction: Beacon, cardinal; beacon, isolated danger; beacon, lateral; beacon, safe water; daymark. |

## Lateral beacons (BeaconLateral)

| Definition: BEACON LATERAL. A beacon is a prominent specially constructed object forming a conspicuous mark as a fixed aid to navigation or for use in hydrographic survey. (IHO Dictionary – S-32). A lateral beacon is used to indicate the port or starboard hand side of the route to be followed. They are generally used for well defined channels and are used in conjunction with a conventional direction of buoyage. (UKHO NP 735, 5th Edition). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Beacon lateral |  |  |  |  |  |
| Primitives: Point |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Beacon shape |  | 1 : stake, pole, perch, post 2 : withy 3 : beacon tower 5 : pile beacon 6 : cairn 7 : buoyant beacon |  | EN | 1,1 |
| Category of lateral mark |  | 1 : port-hand lateral mark 2 : starboard-hand lateral mark 3 : preferred channel to starboard lateral mark 4 : preferred channel to port lateral mark |  | EN | 1,1 |
| Colour |  | 1 : white 2 : black 3 : red 4 : green 5 : blue 6 : yellow 7 : grey 8 : brown 9 : amber 10 : violet 11 : orange 12 : magenta 13 : pink |  | EN | 1,* |
| Colour pattern |  | 1 : horizontal stripes 2 : vertical stripes 3 : diagonal stripes 4 : squared 5 : stripes (direction unknown) 6 : border stripe |  | EN | 0,1 |
| Condition |  | 1 : under construction 2 : ruined 5 : planned construction |  | EN | 0,1 |
| Radar conspicuous |  |  |  | BO | 0,1 |
| Visually conspicuous |  |  |  | BO | 0,1 |
| Elevation |  |  |  | RE | 0,1 |
| Height |  |  |  | RE | 0,1 |
| Marks navigational – system of |  | 1 : IALA A 2 : IALA B 9 : no system 10 : other system 11 : CEVNI |  | EN | 0,1 |
| Nature of construction |  | 1 : masonry 2 : concreted 6 : wooden 7 : metal 8 : glass reinforced plastic (GRP) 9 : painted 10 : latticed |  | EN | 0,* |
| Vertical length |  |  |  | RE | 0,1 |
| Vertical accuracy |  |  |  | RE | 0,1 |
| Object name in national language |  |  |  | (S) TE | 0,1 |
| Object name |  |  |  | (S) TE | 0,1 |
| Status |  | 1 : permanent 2 : occasional 4 : not in use 5 : periodic/intermittent 7 : temporary 8 : private 12 : illuminated 18 : existence doubtful |  | EN | 0,* |
| Aid availability category |  | 1 : Category 1 2 : Category 2 3 : Category 3 |  | EN | 1,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Textual description in national language |  |  |  | (S) TE | 0,1 |
| Textual description |  |  |  | (S) TE | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  |  | TE | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| ### Lateral Beacons Lateral beacons are generally used for well defined channels, in conjunction with a direction of buoyage. They indicate the port and starboard sides of the route to be followed. To conform to the IALA Maritime Buoyage System (see clause X.X), port hand beacons have a can shaped topmark. The colour of port hand beacons, topmarks and lights (if fitted) will be red in IALA region A and green in IALA region B. To conform to the IALA Maritime Buoyage System, starboard hand beacons have a conical shaped topmark. The colour of starboard hand beacons, topmarks and lights (if fitted) will be green in IALA region A and red in IALA region B. A preferred channel beacon is a modified lateral beacon, with horizontal colour bands. The predominant colour indicates which side is the preferred channel, the other colour indicates the secondary channel. If fitted, the light is Fl(2+1), the colour indicating the preferred channel. If it is required to encode a beacon having the function of a lateral mark, it must be done using the feature Beacon Lateral. Remarks: - The attribute colour pattern must be populated if more than one value is populated for the mandatory attribute colour. - If it is required to encode the altitude of the ground level above the vertical datum at the position of a beacon, it must be done using the attribute elevation, but only for beacons built on land. - If it is required to encode the total altitude of a beacon, including the topmark and any equipment features (e.g. light), above the vertical datum, it must be done using the attribute height. - If it is required to encode the total vertical length of a beacon, including the topmark and any equipment features (e.g. light), above the seabed or ground, it must be done using the attribute vertical length. - If it is required to encode a cairn that bears the colour(s) specified by a navigational system of marks, it must be done using a beacon feature. Distinction: Beacon, cardinal; beacon, isolated danger; beacon, safe water; beacon, special purpose/general; daymark. |  |  |  |  |  |

## Cardinal beacons (BeaconCardinal)

| Definition: BEACON, CARDINAL. A beacon is a prominent specially constructed object forming a conspicuous mark as a fixed aid to navigation or for use in hydrographic survey. (IHO Dictionary – S-32). A cardinal beacon is used in conjunction with the compass to indicate where the mariner may find the best navigable water. It is placed in one of the four quadrants (North, East, South and West), bounded by inter-cardinal bearings from the point marked. (UKHO NP 735, 5th Edition). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Beacon cardinal |  |  |  |  |  |
| Primitives: Point |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Beacon shape |  | 1 : stake, pole, perch, post 2 : withy 3 : beacon tower 5 : pile beacon 6 : cairn 7 : buoyant beacon |  | EN | 1,1 |
| Category of cardinal mark |  | 1 : north cardinal mark 2 : east cardinal mark 3 : south cardinal mark 4 : west cardinal mark |  | EN | 1,1 |
| Colour |  | 1 : white 2 : black 3 : red 4 : green 5 : blue 6 : yellow 7 : grey 8 : brown 9 : amber 10 : violet 11 : orange 12 : magenta 13 : pink |  | EN | 1,* |
| Colour pattern |  | 1 : horizontal stripes 2 : vertical stripes 3 : diagonal stripes 4 : squared 5 : stripes (direction unknown) 6 : border stripe |  | EN | 0,1 |
| Condition |  | 1 : under construction 2 : ruined 5 : planned construction |  | EN | 0,1 |
| Radar conspicuous |  |  |  | BO | 0,1 |
| Visually conspicuous |  |  |  | BO | 0,1 |
| Elevation |  |  |  | RE | 0,1 |
| Height |  |  |  | RE | 0,1 |
| Marks navigational – system of |  | 1 : IALA A 2 : IALA B 9 : no system 10 : other system 11 : CEVNI |  | EN | 0,1 |
| Nature of construction |  | 1 : masonry 2 : concreted 6 : wooden 7 : metal 8 : glass reinforced plastic (GRP) 9 : painted 10 : latticed |  | EN | 0,* |
| Vertical length |  |  |  | RE | 0,1 |
| Vertical accuracy |  |  |  | RE | 0,1 |
| Object name in national language |  |  |  | (S) TE | 0,1 |
| Object name |  |  |  | (S) TE | 0,1 |
| Status |  | 1 : permanent 2 : occasional 4 : not in use 5 : periodic/intermittent 7 : temporary 8 : private 12 : illuminated 18 : existence doubtful |  | EN | 0,* |
| Aid availability category |  | 1 : Category 1 2 : Category 2 3 : Category 3 |  | EN | 1,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Textual description in national language |  |  |  | (S) TE | 0,1 |
| Textual description |  |  |  | (S) TE | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  |  | TE | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| ### Cardinal beacons Cardinal marks are used in conjunction with the compass to indicate where a mariner may find best navigable water, taking their name from the quadrant in which they are placed in relation to the point marked. The mariner should pass N of a North mark, E of an East mark, etc. To conform to the IALA Maritime Buoyage System (see clause X.X), the body of the beacon has black and yellow bands, configured with black reflecting the points of the topmark cones (e.g. black above yellow for north). Black double-cone topmarks are an important feature of cardinal marks and are carried wherever practicable. The points are up for a north mark, down for a south mark, apart for an east mark and together for a west mark. Lights (if fitted) are white Q or VQ, uninterrupted for the north, 3 flashes for east, 6 flashes + LFl for south and 9 flashes for west (resembling an analogue clock). If it is required to encode a beacon having the function of a cardinal mark, it must be done using the feature Beacon Cardinal. Remarks: - The attribute colour pattern must be populated if more than one value is populated for the mandatory attribute colour. - If it is required to encode the altitude of the ground level above the vertical datum at the position of a beacon, it must be done using the attribute elevation, but only for beacons built on land. - If it is required to encode the total altitude of a beacon, including the topmark and any equipment features (e.g. light), above the vertical datum, it must be done using the attribute height. - If it is required to encode the total vertical length of a beacon, including the topmark and any equipment features (e.g. light), above the seabed or ground, it must be done using the attribute vertical length. - If it is required to encode a cairn that bears the colour(s) specified by a navigational system of marks, it must be done using a beacon feature. Distinction: Beacon, isolated danger; beacon, lateral; beacon, safe water; beacon, special purpose/general; daymark. |  |  |  |  |  |

## Isolated danger beacons (BeaconIsolatedDanger)

| Definition: BEACON, ISOLATED DANGER. A beacon is a prominent, specially constructed object forming a conspicuous mark as a fixed aid to navigation or for use in hydrographic survey. (IHO Dictionary – S-32). An isolated danger beacon is a beacon erected on an isolated danger of limited extent, which has navigable water all around it. (UKHO NP735, 5th Edition). |  |  |  |  |
| --- | --- | --- | --- | --- |
| S-201 Geo Feature: Beacon isolated danger |  |  |  |  |
| Primitives: Point |  |  |  |  |
| Real World | Paper Chart Symbol | ECDIS Symbol |  |  |
| S-201 Attribute | Allowable Encoding Value |  | Type | Multiplicity |
| Beacon shape | 1 : stake, pole, perch, post 2 : withy 3 : beacon tower 5 : pile beacon 6 : cairn 7 : buoyant beacon |  | EN | 1,1 |
| Colour | 1 : white 2 : black 3 : red 4 : green 5 : blue 6 : yellow 7 : grey 8 : brown 9 : amber 10 : violet 11 : orange 12 : magenta 13 : pink |  | EN | 1,* |
| Colour pattern | 1 : horizontal stripes 2 : vertical stripes 3 : diagonal stripes 4 : squared 5 : stripes (direction unknown) 6 : border stripe |  | EN | 0,1 |
| Condition | 1 : under construction 2 : ruined 5 : planned construction |  | EN | 0,1 |
| Radar conspicuous |  |  | BO | 0,1 |
| Visually conspicuous |  |  | BO | 0,1 |
| Elevation |  |  | RE | 0,1 |
| Height |  |  | RE | 0,1 |
| Marks navigational – system of | 1 : IALA A 2 : IALA B 9 : no system 10 : other system 11 : CEVNI |  | EN | 0,1 |
| Nature of construction | 1 : masonry 2 : concreted 6 : wooden 7 : metal 8 : glass reinforced plastic (GRP) 9 : painted 10 : latticed |  | EN | 0,* |
| Vertical length |  |  | RE | 0,1 |
| Vertical accuracy |  |  | RE | 0,1 |
| Object name in national language |  |  | (S) TE | 0,1 |
| Object name |  |  | (S) TE | 0,1 |
| Status | 1 : permanent 2 : occasional 4 : not in use 5 : periodic/intermittent 7 : temporary 8 : private 12 : illuminated 18 : existence doubtful |  | EN | 0,* |
| Aid availability category | 1 : Category 1 2 : Category 2 3 : Category 3 |  | EN | 1,1 |
| ID Code |  |  | (S) TE | 1,1 |
| Date end | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  | (S) TE | 0,* |
| Information in national language |  |  | (S) TE | 0,* |
| Textual description in national language |  |  | (S) TE | 0,1 |
| Textual description |  |  | (S) TE | 0,1 |
| Scale minimum | See clause X.X |  | IN | 0,1 |
| Source date |  |  | (S) TE | 0,1 |
| Source indication |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  | TE | 0,1 |
| Inspection frequency |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  | (S) TE | 0,1 |
| Aton maintenance record |  |  | URI | 0,1 |
| Installation date |  |  | (S) DA | 0,1 |
| ### Isolated danger beacons Isolated danger beacons are placed on isolated dangers of limited extent with navigable water all around them. To conform to the IALA Maritime Buoyage System (see clause X.X), the body of an isolated danger beacon is black, with one or more red bands. Black double-sphere topmarks are an important feature of isolated danger beacons and carried wherever practicable. The light (if fitted) is white Fl(2). If it is required to encode a beacon having the function of an isolated danger mark, it must be done using the feature Beacon Isolated Danger. Remarks: - The attribute colour pattern must be populated if more than one value is populated for the mandatory attribute colour. - If it is required to encode the altitude of the ground level above the vertical datum at the position of a beacon, it must be done using the attribute elevation, but only for beacons built on land. - If it is required to encode the total altitude of a beacon, including the topmark and any equipment features (e.g. light), above the vertical datum, it must be done using the attribute height. - If it is required to encode the total vertical length of a beacon, including the topmark and any equipment features (e.g. light), above the seabed or ground, it must be done using the attribute vertical length. - If it is required to encode a cairn that bears the colour(s) specified by a navigational system of marks, it must be done using a beacon feature. Distinction: Beacon, cardinal; beacon, lateral; beacon, safe water; beacon, special purpose/general; daymark. |  |  |  |  |

## Safe water beacons (BeaconSafeWater)

| Definition: BEACON, SAFE WATER. A safe water beacon is a prominent specially constructed object forming a conspicuous mark as a fixed aid to navigation or for use in hydrographic survey. (IHO Dictionary – S-32, Edition 5). A safe water beacon is used to indicate that there is navigable water around the mark. (UKHO NP735, 5th Edition). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Beacon safe water |  |  |  |  |  |
| Primitives: Point |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Beacon shape |  | 1 : stake, pole, perch, post 2 : withy 3 : beacon tower 5 : pile beacon 6 : cairn 7 : buoyant beacon |  | EN | 1,1 |
| Category of cardinal mark |  | 1 : north cardinal mark 2 : east cardinal mark 3 : south cardinal mark 4 : west cardinal mark |  | EN | 1,1 |
| Colour |  | 1 : white 2 : black 3 : red 4 : green 5 : blue 6 : yellow 7 : grey 8 : brown 9 : amber 10 : violet 11 : orange 12 : magenta 13 : pink |  | EN | 1,* |
| Colour pattern |  | 1 : horizontal stripes 2 : vertical stripes 3 : diagonal stripes 4 : squared 5 : stripes (direction unknown) 6 : border stripe |  | EN | 0,1 |
| Condition |  | 1 : under construction 2 : ruined 5 : planned construction |  | EN | 0,1 |
| Radar conspicuous |  |  |  | BO | 0,1 |
| Visually conspicuous |  |  |  | BO | 0,1 |
| Elevation |  |  |  | RE | 0,1 |
| Height |  |  |  | RE | 0,1 |
| Marks navigational – system of |  | 1 : IALA A 2 : IALA B 9 : no system 10 : other system 11 : CEVNI |  | EN | 0,1 |
| Nature of construction |  | 1 : masonry 2 : concreted 6 : wooden 7 : metal 8 : glass reinforced plastic (GRP) 9 : painted 10 : latticed |  | EN | 0,* |
| Vertical length |  |  |  | RE | 0,1 |
| Vertical accuracy |  |  |  | RE | 0,1 |
| Object name in national language |  |  |  | (S) TE | 0,1 |
| Object name |  |  |  | (S) TE | 0,1 |
| Status |  | 1 : permanent 2 : occasional 4 : not in use 5 : periodic/intermittent 7 : temporary 8 : private 12 : illuminated 18 : existence doubtful |  | EN | 0,* |
| Aid availability category |  | 1 : Category 1 2 : Category 2 3 : Category 3 |  | EN | 1,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Textual description in national language |  |  |  | (S) TE | 0,1 |
| Textual description |  |  |  | (S) TE | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  |  | TE | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| ### Safe water beacons Safe water marks are used to indicate there is safe water all around the mark. It may be used as a centre-line, mid-channel or landfall beacon, or to mark the best point of passage under a bridge. To conform to the IALA Maritime Buoyage System (see clause X.X), the body of the mark has red and white vertical stripes. A red spherical topmark is an important feature if the beacon is not spherical and carried wherever practicable. The light (if fitted) is white Oc, Iso, LFl or Mo(A) with a period of 10s. If it is required to encode a beacon having the function of a safe water mark, it must be done using the feature Beacon Safe Water. Remarks: - The attribute colour pattern must be populated if more than one value is populated for the mandatory attribute colour. - If it is required to encode the altitude of the ground level above the vertical datum at the position of a beacon, it must be done using the attribute elevation, but only for beacons built on land. - If it is required to encode the total altitude of a beacon, including the topmark and any equipment features (e.g. light), above the vertical datum, it must be done using the attribute height. - If it is required to encode the total vertical length of a beacon, including the topmark and any equipment features (e.g. light), above the seabed or ground, it must be done using the attribute vertical length. - If it is required to encode a cairn that bears the colour(s) specified by a navigational system of marks, it must be done using a beacon feature. Distinction: Beacon, cardinal; beacon, isolated danger; beacon, lateral; beacon, special purpose/general; daymark. |  |  |  |  |  |

# AtoN floating structures

If it is required to encode a buoy, it must be done using the object classes Buoy cardinal, Buoy installation,  Buoy isolated danger, Buoy lateral, Buoy safe water or Buoy special purpose.

Remarks:

- If it is required to encode the total vertical length, including any equipment objects (e.g. Topmark, Light), of the buoy above the water level, it must be done using the attribute Vertical length.

- If it is required to encode a buoy that has more than one colour, the attributes Colour and Colour pattern must be encoded, according to the rules laid out in clause 2.4.

- The optional attribute IALA_typeOfBuoy on buoy feature types is used to encode the type of buoy equipment used, e.g. LANBY-100, LS-35, LL-30, LL-28, LL-26, LL-26(M), LL-24, LS-24, LSP-24, LT-10, U-17C(P), U-17S(P), U-17C(S), U-17S(S), UR-17C(P), UR-17S(P), UR-17C(S), UR-17S(S)

In the following table, the symbol ‘/’ indicates that this attribute does not exist for that particular object class.  A blank indicates that the encoder may choose a relevant value for the attribute.  The table contains the most common examples of coding; other coding combinations are possible.

| Real Feature Type | Feature | Buoy Shape | Category of special purpose mark | Other attributes |
| --- | --- | --- | --- | --- |
| Oil or gas installation buoy | Buoy installation | 7 | / | Product = 1 or 2, Category of installation buoy = 1 or 2 |
| Firing danger area buoy | Buoy special purpose/general |  | 1 |  |
| Target | Buoy special purpose/general |  | 2 |  |
| Marker ship | Buoy special purpose/general |  | 3 |  |
| Barge | Buoy special purpose/general |  | 5 |  |
| Degaussing range buoy | Buoy special purpose/general |  | 4 |  |
| Buoy marking cable | Buoy special purpose/general |  | 6 |  |
| Spoil ground buoy | Buoy special purpose/general |  | 7 |  |
| Buoy marking outfall | Buoy special purpose/general |  | 8 |  |
| Buoy marking pipeline | Buoy special purpose/general |  | 39 |  |
| Superbuoy | Buoy *** | 7 |  |  |
| Large automatic navigational buoy | Buoy special purpose/general | 7 | 15 |  |
| Data-collecting buoy of superbuoy size | Buoy special purpose/general | 7 | 9 |  |
| Buoy marking wave recorder (or current meter) | Buoy special purpose/general |  | 10 | INFORM = wave recorder (e.g.) |
| Seaplane anchorage buoy | Buoy special purpose/general |  | 11 |  |
| Buoy marking traffic separation scheme | Buoy special purpose/general |  | 19 |  |
| Buoy marking recreation zone | Buoy special purpose/general |  | 12 |  |
|  |  |  |  | table 12.3 |

## Installation buoy (BuoyInstallation)

| Definition: BUOY, INSTALLATION. A buoy is a floating object moored to the bottom in a particular place, as an aid to navigation or for other specific purposes. (IHO Dictionary – S-32). An installation buoy is a buoy used for loading tankers with gas or oil. (S-57 Edition 3.1, Appendix A – Chapter 1, Page 1.20, November 2000). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Buoy installation |  |  |  |  |  |
| Primitives: Point |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Buoy shape |  | 1 : conical (nun, ogival) 2 : can (cylindrical) 3 : spherical 4 : pillar 5 : spar (spindle) 6 : barrel (tun) 7 : superbuoy 8 : ice buoy |  | EN | 1,1 |
| Category of installation buoy |  | 1 : catenary anchor leg mooring (CALM) 2 : single buoy mooring (SBM or SPM) |  | EN | 0,1 |
| Colour |  | 1 : white 2 : black 3 : red 4 : green 5 : blue 6 : yellow 7 : grey 8 : brown 9 : amber 10 : violet 11 : orange 12 : magenta 13 : pink |  | EN | 1,* |
| Colour pattern |  | 1 : horizontal stripes 2 : vertical stripes 3 : diagonal stripes 4 : squared 5 : stripes (direction unknown) 6 : border stripe |  | EN | 0,1 |
| Radar conspicuous |  |  |  | BO | 0,1 |
| Navigational system of marks |  | 1 : IALA A 2 : IALA B 3 : no system 4 : other system |  | EN | 0,1 |
| Nature of construction |  | 6 : wooden 7 : metal 8 : glass reinforced plastic (GRP) 9 : painted 10 : latticed |  | EN | 0,* |
| Object name in national language |  |  |  | (S) TE | 0,1 |
| Object name |  |  |  | (S) TE | 0,1 |
| Status |  | 1 : permanent 2 : occasional 4 : not in use 5 : periodic/intermittent 7 : temporary 8 : private 18 : existence doubtful |  | EN | 0,* |
| Vertical length |  |  |  | RE | 0,1 |
| Vertical accuracy |  |  |  | RE | 0,1 |
| Type of buoy |  |  |  | (S) TE | 0,1 |
| Aid availability category |  | 1 : Category 1 2 : Category 2 3 : Category 3 |  | EN | 1,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Textual description in national language |  |  |  | (S) TE | 0,1 |
| Textual description |  |  |  | (S) TE | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  |  | TE | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| INT 1 Reference: L 16 ### Installation buoys (see S-4 – B-445.4) Although the oil and gas from some fields are sent ashore by submarine pipeline, a variety of mooring systems have been developed for use in deep water and in the vicinity of certain ports, to allow the loading of large vessels and the permanent mooring of floating storage vessels or units. These offshore systems include large mooring buoys, designed for mooring vessels up to 500,000 tonnes, and platforms on structures fixed at their lower ends to the sea floor. They allow a vessel to moor forward or aft to them, and to swing to the wind or stream, and are termed installation buoys. If it is required to encode an installation buoy, it must be done using the feature Buoy Installation. Remarks: - The attribute colour pattern must be populated if more than one value is populated for the mandatory attribute colour. - If it is required to encode the total vertical length, including any equipment features (e.g. light), of the buoy above the water level, it must be done using the attribute vertical length. Distinction: Buoy, special purpose/general; mooring/warping facility; offshore platform. |  |  |  |  |  |

## Lateral buoys (BuoyLateral)

| Definition: BUOY LATERAL MARKS. A buoy is a floating object moored to the bottom in a particular place, as an aid to navigation or for other specific purposes. (IHO Dictionary – S-32). A lateral buoy is used to indicate the port or starboard hand side of the route to be followed. They are generally used for well defined channels and are used in conjunction with a conventional direction of buoyage. (UKHO NP 735, 5th Edition). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Buoy lateral |  |  |  |  |  |
| Primitives: Point |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Buoy shape |  | 1 : conical (nun, ogival) 2 : can (cylindrical) 3 : spherical 4 : pillar 5 : spar (spindle) 6 : barrel (tun) 7 : superbuoy 8 : ice buoy |  | EN | 1,1 |
| Category of lateral mark |  | 1 : port-hand lateral mark 2 : starboard-hand lateral mark 3 : preferred channel to starboard lateral mark 4 : preferred channel to port lateral mark |  | EN | 1,1 |
| Colour |  | 1 : white 2 : black 3 : red 4 : green 5 : blue 6 : yellow 7 : grey 8 : brown 9 : amber 10 : violet 11 : orange 12 : magenta 13 : pink |  | EN | 1,* |
| Colour pattern |  | 1 : horizontal stripes 2 : vertical stripes 3 : diagonal stripes 4 : squared 5 : stripes (direction unknown) 6 : border stripe |  | EN | 0,1 |
| Radar conspicuous |  |  |  | BO | 0,1 |
| Navigational system of marks |  | 1 : IALA A 2 : IALA B 3 : no system 4 : other system |  | EN | 0,1 |
| Nature of construction |  | 6 : wooden 7 : metal 8 : glass reinforced plastic (GRP) 9 : painted 10 : latticed |  | EN | 0,* |
| Object name in national language |  |  |  | (S) TE | 0,1 |
| Object name |  |  |  | (S) TE | 0,1 |
| Status |  | 1 : permanent 2 : occasional 4 : not in use 5 : periodic/intermittent 7 : temporary 8 : private 18 : existence doubtful |  | EN | 0,* |
| Vertical length |  |  |  | RE | 0,1 |
| Vertical accuracy |  |  |  | RE | 0,1 |
| Type of buoy |  |  |  | (S) TE | 0,1 |
| Aid availability category |  | 1 : Category 1 2 : Category 2 3 : Category 3 |  | EN | 1,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Textual description in national language |  |  |  | (S) TE | 0,1 |
| Textual description |  |  |  | (S) TE | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  |  | TE | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| ### Lateral buoys Lateral buoys are generally used for well defined channels, in conjunction with a direction of buoyage. They indicate the port and starboard sides of the route to be followed. To conform to the IALA Maritime Buoyage System (see clause X.X), port hand buoys are usually can shaped, but may be another shape (except conical or spherical). Other shaped buoys have a can topmark. The colour of port hand buoys, topmarks and lights (if fitted) will be red in IALA region A and green in IALA region B. To conform to the IALA Maritime Buoyage System, starboard hand buoys are usually conical shaped, but may be another shape (except can or spherical). Other shaped buoys have a conical topmark. The colour of starboard hand buoys, topmarks and lights (if fitted) will be green in IALA region A and red in IALA region B. A preferred channel mark is a modified lateral mark, with horizontal colour bands. The shape and predominant colour indicates which side is the preferred channel, the other colour indicates the secondary channel. If fitted, the light is Fl(2+1), the colour indicating the preferred channel. If it is required to encode a buoy having the function of a lateral mark, it must be done using the feature Buoy Lateral. Remarks: - The attribute colour pattern must be populated if more than one value is populated for the mandatory attribute colour. - If it is required to encode the total vertical length, including the topmark and any equipment features (e.g. light), of the buoy above the water level, it must be done using the attribute vertical length. Distinction: Buoy, cardinal; buoy, emergency wreck marking; buoy, installation; buoy, isolated danger; buoy, safe water; buoy, special purpose/general; mooring/warping facility. |  |  |  |  |  |

## Cardinal buoy (BuoyCardinal)

| Definition: BUOY, CARDINAL. A buoy is a floating object moored to the bottom in a particular place, as an aid to navigation or for other specific purposes. (IHO Dictionary – S-32). A cardinal buoy is used in conjunction with the compass to indicate where the mariner may find the best navigable water. It is placed in one of the four quadrants (North, East, South and West), bounded by inter-cardinal bearings from the point marked. (UKHO NP 735, 5th Edition). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Buoy cardinal |  |  |  |  |  |
| Primitives: Point |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Buoy shape |  | 1 : conical (nun, ogival) 2 : can (cylindrical) 3 : spherical 4 : pillar 5 : spar (spindle) 6 : barrel (tun) 7 : superbuoy 8 : ice buoy |  | EN | 1,1 |
| Category of cardinal mark |  | 1 : north cardinal mark 2 : east cardinal mark 3 : south cardinal mark 4 : west cardinal mark |  | EN | 1,1 |
| Colour |  | 1 : white 2 : black 3 : red 4 : green 5 : blue 6 : yellow 7 : grey 8 : brown 9 : amber 10 : violet 11 : orange 12 : magenta 13 : pink |  | EN | 1,* |
| Colour pattern |  | 1 : horizontal stripes 2 : vertical stripes 3 : diagonal stripes 4 : squared 5 : stripes (direction unknown) 6 : border stripe |  | EN | 0,1 |
| Radar conspicuous |  |  |  | BO | 0,1 |
| Navigational system of marks |  | 1 : IALA A 2 : IALA B 3 : no system 4 : other system |  | EN | 0,1 |
| Nature of construction |  | 6 : wooden 7 : metal 8 : glass reinforced plastic (GRP) 9 : painted 10 : latticed |  | EN | 0,* |
| Object name in national language |  |  |  | (S) TE | 0,1 |
| Object name |  |  |  | (S) TE | 0,1 |
| Status |  | 1 : permanent 2 : occasional 4 : not in use 5 : periodic/intermittent 7 : temporary 8 : private 18 : existence doubtful |  | EN | 0,* |
| Vertical length |  |  |  | RE | 0,1 |
| Vertical accuracy |  |  |  | RE | 0,1 |
| Type of buoy |  |  |  | (S) TE | 0,1 |
| Aid availability category |  | 1 : Category 1 2 : Category 2 3 : Category 3 |  | EN | 1,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Textual description in national language |  |  |  | (S) TE | 0,1 |
| Textual description |  |  |  | (S) TE | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  |  | TE | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| ### Cardinal buoys Cardinal marks are used in conjunction with the compass to indicate where a mariner may find best navigable water, taking their name from the quadrant in which they are placed in relation to the point marked. The mariner should pass N of a North mark, E of an East mark, etc. The shape of cardinal buoys is not significant (although they are usually pillar or spar). To conform to the IALA Maritime Buoyage System (see clause X.X), the body has black and yellow bands, configured with black reflecting the points of the topmark cones (e.g. black above yellow for north). Black double-cone topmarks are an important feature of cardinal marks and are carried wherever practicable. The points are up for a north mark, down for a south mark, apart for an east mark and together for a west mark. Lights (if fitted) are white Q or VQ, uninterrupted for the north, 3 flashes for east, 6 flashes + LFl for south and 9 flashes for west (resembling an analogue clock). If it is required to encode a buoy having the function of a cardinal mark, it must be done using the feature Buoy Cardinal. Remarks: - The attribute colour pattern must be populated if more than one value is populated for the mandatory attribute colour. - If it is required to encode the total vertical length, including the topmark and any equipment features (e.g. light), of the buoy above the water level, it must be done using the attribute vertical length. Distinction: Buoy, emergency wreck marking; buoy, installation; buoy, isolated danger; buoy, lateral; buoy, safe water; buoy, special purpose/general; mooring/warping facility. |  |  |  |  |  |

## Safet water buoys (BuoySafeWater)

| Definition: BUOY, SAFE WATER. A buoy is a floating object moored to the bottom in a particular place, as an aid to navigation or for other specific purposes. (IHO Dictionary – S-32). A safe water buoy is used to indicate that there is navigable water around the mark. (UKHO NP 735, 5th Edition). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Buoy safe water |  |  |  |  |  |
| Primitives: Point |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Buoy shape |  | 1 : conical (nun, ogival) 2 : can (cylindrical) 3 : spherical 4 : pillar 5 : spar (spindle) 6 : barrel (tun) 7 : superbuoy 8 : ice buoy |  | EN | 1,1 |
| Colour |  | 1 : white 2 : black 3 : red 4 : green 5 : blue 6 : yellow 7 : grey 8 : brown 9 : amber 10 : violet 11 : orange 12 : magenta 13 : pink |  | EN | 1,* |
| Colour pattern |  | 1 : horizontal stripes 2 : vertical stripes 3 : diagonal stripes 4 : squared 5 : stripes (direction unknown) 6 : border stripe |  | EN | 0,1 |
| Radar conspicuous |  |  |  | BO | 0,1 |
| Navigational system of marks |  | 1 : IALA A 2 : IALA B 3 : no system 4 : other system |  | EN | 0,1 |
| Nature of construction |  | 6 : wooden 7 : metal 8 : glass reinforced plastic (GRP) 9 : painted 10 : latticed |  | EN | 0,* |
| Object name in national language |  |  |  | (S) TE | 0,1 |
| Object name |  |  |  | (S) TE | 0,1 |
| Status |  | 1 : permanent 2 : occasional 4 : not in use 5 : periodic/intermittent 7 : temporary 8 : private 18 : existence doubtful |  | EN | 0,* |
| Vertical length |  |  |  | RE | 0,1 |
| Vertical accuracy |  |  |  | RE | 0,1 |
| Type of buoy |  |  |  | (S) TE | 0,1 |
| Aid availability category |  | 1 : Category 1 2 : Category 2 3 : Category 3 |  | EN | 1,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Textual description in national language |  |  |  | (S) TE | 0,1 |
| Textual description |  |  |  | (S) TE | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  |  | TE | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| ### Safe water buoys Safe water marks are used to indicate there is safe water all around the mark. It may be used as a centre-line, mid-channel or landfall buoy, or to mark the best point of passage under a bridge. To conform to the IALA Maritime Buoyage System (see clause X.X), the shape of a safe water buoy is spherical, pillar or spar. The body of the mark has red and white vertical stripes. If the shape of the buoy is not spherical a red spherical topmark is carried wherever practicable. The light (if fitted) is white Oc, Iso, LFl or Mo(A) with a period of 10s. If it is required to encode a buoy having the function of a safe water mark, it must be done using the feature Buoy Safe Water. Remarks: - The attribute colour pattern must be populated if more than one value is populated for the mandatory attribute colour. - If it is required to encode the total vertical length, including the topmark and any equipment features (e.g. light), of the buoy above the water level, it must be done using the attribute vertical length. Distinction: Buoy, cardinal; buoy, emergency wreck marking; buoy, installation; buoy, isolated danger; buoy, lateral; buoy, special purpose/general; mooring/warping facility. |  |  |  |  |  |

## Isolated danger buoys (BuoyisolatedDanger)

| Definition: BUOY, ISOLATED DANGER. A buoy is a floating object moored to the bottom in a particular place, as an aid to navigation or for other specific purposes. (IHO Dictionary – S-32). An isolated danger buoy is a buoy moored on or above an isolated danger of limited extent, which has navigable water all around it. (UKHO NP 735, 5th Edition). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Buoy isolated danger |  |  |  |  |  |
| Primitives: Point |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Buoy shape |  | 1 : conical (nun, ogival) 2 : can (cylindrical) 3 : spherical 4 : pillar 5 : spar (spindle) 6 : barrel (tun) 7 : superbuoy 8 : ice buoy |  | EN | 1,1 |
| Colour |  | 1 : white 2 : black 3 : red 4 : green 5 : blue 6 : yellow 7 : grey 8 : brown 9 : amber 10 : violet 11 : orange 12 : magenta 13 : pink |  | EN | 1,* |
| Colour pattern |  | 1 : horizontal stripes 2 : vertical stripes 3 : diagonal stripes 4 : squared 5 : stripes (direction unknown) 6 : border stripe |  | EN | 0,1 |
| Radar conspicuous |  |  |  | BO | 0,1 |
| Navigational system of marks |  | 1 : IALA A 2 : IALA B 3 : no system 4 : other system |  | EN | 0,1 |
| Nature of construction |  | 6 : wooden 7 : metal 8 : glass reinforced plastic (GRP) 9 : painted 10 : latticed |  | EN | 0,* |
| Object name in national language |  |  |  | (S) TE | 0,1 |
| Object name |  |  |  | (S) TE | 0,1 |
| Status |  | 1 : permanent 2 : occasional 4 : not in use 5 : periodic/intermittent 7 : temporary 8 : private 18 : existence doubtful |  | EN | 0,* |
| Vertical length |  |  |  | RE | 0,1 |
| Vertical accuracy |  |  |  | RE | 0,1 |
| Type of buoy |  |  |  | (S) TE | 0,1 |
| Aid availability category |  | 1 : Category 1 2 : Category 2 3 : Category 3 |  | EN | 1,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Textual description in national language |  |  |  | (S) TE | 0,1 |
| Textual description |  |  |  | (S) TE | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  |  | TE | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| ### Isolated danger buoys Isolated danger buoys are moored above isolated dangers of limited extent with navigable water all around them. The shape of isolated danger buoys is not significant (although they are usually pillar or spar shaped). To conform to the IALA Maritime Buoyage System (see clause X.X), the body is black, with one or more red bands. Black double-sphere topmarks are an important feature of isolated danger buoys and carried wherever practicable. The light (if fitted) is white Fl(2). If it is required to encode a buoy having the function of an isolated danger mark, it must be done using the feature Buoy Isolated Danger. Remarks: - The attribute colour pattern must be populated if more than one value is populated for the mandatory attribute colour. - If it is required to encode the total vertical length, including the topmark and any equipment features (e.g. light), of the buoy above the water level, it must be done using the attribute vertical length. Distinction: Buoy, cardinal; buoy, emergency wreck marking; buoy, installation; buoy, lateral; buoy, safe water; buoy, special purpose/general; mooring/warping facility. |  |  |  |  |  |

## Special purpose/general buoys (BuoySpecialPurposeGeneral)

| Definition: BUOY, SPECIAL PURPOSE/GENERAL. A buoy is a floating object moored to the bottom in a particular place, as an aid to navigation or for other specific purposes. (IHO Dictionary – S-32). A special purpose buoy is primarily used to indicate an area or feature, the nature of which is apparent from reference to a chart, Sailing Directions or Notices to Mariners. (UKHO NP 735, 5th Edition). Buoy in general: A buoy whose appearance or purpose is not adequately known. (S-57 Edition 3.1, Appendix A – Chapter 1, Page 1.24, November 2000). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Buoy special purpose/general |  |  |  |  |  |
| Primitives: Point |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Buoy shape |  | 1 : conical (nun, ogival) 2 : can (cylindrical) 3 : spherical 4 : pillar 5 : spar (spindle) 6 : barrel (tun) 7 : superbuoy 8 : ice buoy |  | EN | 1,1 |
| Category of special purpose mark |  | 1 : firing danger area mark 2 : target mark 3 : marker ship mark 4 : degaussing range mark 5 : barge mark 6 : cable mark 7 : spoil ground mark 8 : outfall mark 9 : ODAS (Ocean-Data Acquisition-System) buoy 10 : recording mark 11 : seaplane anchorage mark 12 : recreation zone mark 14 : mooring mark 15 : LANBY (Large Automatic Navigational Buoy) 16 : leading mark 17 : measured distance mark 18 : notice mark 19 : TSS mark (Traffic Separation Scheme) 20 : anchoring prohibited mark 21 : berthing prohibited mark 22 : overtaking prohibited mark 23 : two-way traffic prohibited mark 24 : reduced wake mark 25 : speed limit mark 26 : stop mark 27 : general warning mark 28 : sound ship’s siren mark 29 : restricted vertical clearance mark 30 : maximum vessel’s draught mark 31 : restricted horizontal clearance mark 32 : strong current warning mark 33 : berthing permitted mark 34 : overhead power cable mark 35 : channel edge gradient’ mark 36 : telephone mark 37 : ferry crossing mark 39 : pipeline mark 40 : anchorage mark 41 : clearing mark 42 : control mark 43 : diving mark 44 : refuge beacon 45 : foul ground mark 46 : yachting mark 47 : heliport mark 48 : GNSS mark 49 : seaplane landing mark 50 : control mark 51 : work in progress mark 52 : mark with unknown purpose 53 : wellhead mark 54 : channel separation mark 55 : marine farm mark 56 : artificial reef mark 57 : ice mark 58 : nature reserve mark 59 : floating fish aggregating device (FAD) |  | EN | 1,* |
| Colour |  | 1 : white 2 : black 3 : red 4 : green 5 : blue 6 : yellow 7 : grey 8 : brown 9 : amber 10 : violet 11 : orange 12 : magenta 13 : pink |  | EN | 1,* |
| Colour pattern |  | 1 : horizontal stripes 2 : vertical stripes 3 : diagonal stripes 4 : squared 5 : stripes (direction unknown) 6 : border stripe |  | EN | 0,1 |
| Radar conspicuous |  |  |  | BO | 0,1 |
| Navigational system of marks |  | 1 : IALA A 2 : IALA B 3 : no system 4 : other system |  | EN | 0,1 |
| Nature of construction |  | 6 : wooden 7 : metal 8 : glass reinforced plastic (GRP) 9 : painted 10 : latticed |  | EN | 0,* |
| Object name in national language |  |  |  | (S) TE | 0,1 |
| Object name |  |  |  | (S) TE | 0,1 |
| Status |  | 1 : permanent 2 : occasional 4 : not in use 5 : periodic/intermittent 7 : temporary 8 : private 18 : existence doubtful |  | EN | 0,* |
| Vertical length |  |  |  | RE | 0,1 |
| Vertical accuracy |  |  |  | RE | 0,1 |
| Type of buoy |  |  |  | (S) TE | 0,1 |
| Aid availability category |  | 1 : Category 1 2 : Category 2 3 : Category 3 |  | EN | 1,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Textual description in national language |  |  |  | (S) TE | 0,1 |
| Textual description |  |  |  | (S) TE | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  |  | TE | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| ### Special purpose/general buoys Special marks are used to indicate to the mariner a special area or feature, the nature of which is usually apparent from the ENC, paper chart or associated publication. Special marks may also be used to mark a channel within a channel (e.g. a Deep Water route), using yellow buoys of the appropriate lateral shape, or yellow spherical buoys to mark the centreline. A special buoy may be any shape but must not conflict with lateral or safe water marks (e.g. an outfall buoy on the port-side of a channel could be can but should not be conical). To conform to the IALA Maritime Buoyage System (see clause X.X), the body of the buoy is yellow. The topmark (if fitted) is a yellow diagonal ‘X’ (St Andrew’s cross). Lights (if fitted) are yellow and of any rhythm except those used for cardinal, isolated danger and safe water marks. If it is required to encode a buoy having the function of a special purpose mark, or a buoy whose appearance or purpose is inadequately known, it must be done using the feature Buoy Special Purpose/General. In the following table, a blank indicates that the encoder may choose a relevant value for the attribute. The table contains the most common examples of coding; other coding combinations are possible for Buoy Special Purpose/General features. Feature; INT1; Feature; buoy shape; category of special purpose mark; Other attributes. Firing danger area buoy; Q50; Bouy Special Purpose/General; 1. Target; Q51; Bouy Special Purpose/General; 2. Marker ship; Q52; Bouy Special Purpose/General; 3. Barge; Q53; Bouy Special Purpose/General; 5. Degaussing range buoy; Q54; Bouy Special Purpose/General; 4. Buoy marking cable; Q55; Bouy Special Purpose/General; 6. Spoil ground buoy; Q56; Bouy Special Purpose/General; 7. Buoy marking outfall; Q57; Bouy Special Purpose/General; 8. Buoy marking pipeline; Bouy Special Purpose/General; 39. Emergency wreck marking buoy; Bouy Special Purpose/General; 4 or 5; 27; colour = 5,6 colour pattern = 2. Superbuoy; Q26; Buoy ***; 7. Large automatic navigational buoy; P6; Bouy Special Purpose/General; 7; 15. Ocean data acquisition system (ODAS) buoy; Q58; Bouy Special Purpose/General; 7; 9; Subsurface ODAS encoded as Obstruction (see clause X.X). Buoy marking wave recorder (or current meter); Q59; Bouy Special Purpose/General; 10; information = wave recorder (e.g.). Seaplane anchorage buoy; Q60; Bouy Special Purpose/General; 11. Buoy marking traffic separation scheme; Q61; Bouy Special Purpose/General; 19. Buoy marking recreation zone; Q62; Bouy Special Purpose/General; 12. Floating waste bin; Bouy Special Purpose/General; Empty (null) value; information = waste bin (e.g.). Floating Fish Aggregating Device (FAD); Bouy Special Purpose/General; /; 59; Subsurface FAD encoded as Obstruction (see clause X.X). Remarks: - The attribute colour pattern must be populated if more than one value is populated for the mandatory attribute colour. - If it is required to encode the total vertical length, including the topmark and any equipment features (e.g. light), of the buoy above the water level, it must be done using the attribute vertical length. - Subsurface Fish Aggregating Devices (FAD) and Ocean Data Acquisition System (ODAS) equipment must be encoded, where required, using an Obstruction feature (see clause X.X). Distinction: Buoy, cardinal; buoy, emergency wreck marking; buoy, installation; buoy, isolated danger; buoy, lateral; buoy, safe water; mooring/warping facility. |

## Buoy emergency  wreck marking (BuoyEmergencyWreckMarking)

| Definition: BUOY, EMERGENCY WRECK MARKING An emergency wreck marking buoy is a buoy moored on or above a new wreck, designed to provide a prominent (both visual and radio) and easily identifiable temporary (24-72 hours) first response. (IHO Registry) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Buoy emergency wreck marking |  |  |  |  |  |
| Primitives: Point |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Buoy shape |  | 1 : conical (nun, ogival) 2 : can (cylindrical) 3 : spherical 4 : pillar 5 : spar (spindle) 6 : barrel (tun) 7 : superbuoy 8 : ice buoy |  | EN | 1,1 |
| Colour |  | 1 : white 2 : black 3 : red 4 : green 5 : blue 6 : yellow 7 : grey 8 : brown 9 : amber 10 : violet 11 : orange 12 : magenta 13 : pink |  | EN | 1,* |
| Colour pattern |  | 1 : horizontal stripes 2 : vertical stripes 3 : diagonal stripes 4 : squared 5 : stripes (direction unknown) 6 : border stripe |  | EN | 0,1 |
| Radar conspicuous |  |  |  | BO | 0,1 |
| Navigational system of marks |  | 1 : IALA A 2 : IALA B 3 : no system 4 : other system |  | EN | 0,1 |
| Nature of construction |  | 6 : wooden 7 : metal 8 : glass reinforced plastic (GRP) 9 : painted 10 : latticed |  | EN | 0,* |
| Object name in national language |  |  |  | (S) TE | 0,1 |
| Object name |  |  |  | (S) TE | 0,1 |
| Status |  | 1 : permanent 2 : occasional 4 : not in use 5 : periodic/intermittent 7 : temporary 8 : private 18 : existence doubtful |  | EN | 0,* |
| Vertical length |  |  |  | RE | 0,1 |
| Vertical accuracy |  |  |  | RE | 0,1 |
| Type of buoy |  |  |  | (S) TE | 0,1 |
| Aid availability category |  | 1 : Category 1 2 : Category 2 3 : Category 3 |  | EN | 1,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Textual description in national language |  |  |  | (S) TE | 0,1 |
| Textual description |  |  |  | (S) TE | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  |  | TE | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| TBD |  |  |  |  |  |

# AtoN relevant structures

## Offshore platform (OffshorePlatform)

| Definition: OFFSHORE PLATFORM. A permanent offshore structure, either fixed or floating. (Adapted from IHO Dictionary – S-32). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Offshore platform |  |  |  |  |  |
| Primitives: Point, Surface |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Category of offshore platform |  | 1 : oil derrick/rig 2 : production platform 3 : observation/research platform 4 : articulated loading platform (ALP) 5 : single anchor leg mooring (SALM) 6 : mooring tower 7 : artificial island 8 : floating production, storage and off-loading vessel (FPSO) 9 : accommodation platform 10 : navigation, communication and control buoy (NCCB) 11 : windmotor |  | EN | 0,1 |
| Colour |  | 1 : white 2 : black 3 : red 4 : green 5 : blue 6 : yellow 7 : grey 8 : brown 9 : amber 10 : violet 11 : orange 12 : magenta 13 : pink |  | EN | 0,* |
| Colour pattern |  | 1 : horizontal stripes 2 : vertical stripes 3 : diagonal stripes 4 : squared 5 : stripes (direction unknown) 6 : border stripe |  | EN | 0,1 |
| Condition |  | 1 : under construction 2 : ruined 4 : wingless 5 : planned construction |  | EN | 0,1 |
| Radar conspicuous |  |  |  | BO | 0,1 |
| Visually conspicuous |  |  |  | BO | 0,1 |
| Height |  |  |  | RE | 0,1 |
| Nature of construction |  | 1 : masonry 2 : concreted 3 : loose boulders 4 : hard surfaced 5 : unsurfaced 6 : wooden 7 : metal 8 : glass reinforced plastic (GRP) 9 : painted |  | EN | 0,* |
| Object name in national language |  |  |  | (S) TE | 0,1 |
| Object name |  |  |  | (S) TE | 0,1 |
| Product |  | 1 : oil 2 : gas 3 : water 8 : drinking water 23 : electricity |  | EN | 0,* |
| Status |  | 1 : permanent 2 : occasional 4 : not in use 7 : temporary 8 : private 12 : illuminated 16 : watched 17 : un-watched 19 : buoyed |  | EN | 0,* |
| Vertical accuracy |  |  |  | RE | 0,1 |
| Vertical datum |  | 3 : Mean sea level 16 : Mean high water 17 : Mean high water springs 18 : High water 19 : Approximate mean sea level 20 : High water springs 21 : Mean higher high water 24 : Local datum 25 : International great lakes datum 1985 26 : Mean water level 28 : Higher high water large tide 29 : Nearly highest high water 30 : Highest astronomical tide (HAT) |  | EN | 0,1 |
| Vertical length |  |  |  | RE | 0,1 |
| Manned structure |  |  |  | BO | 0,1 |
| Aid availability category |  | 1 : Category 1 2 : Category 2 3 : Category 3 |  | EN | 1,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Textual description in national language |  |  |  | (S) TE | 0,1 |
| Textual description |  |  |  | (S) TE | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  |  | TE | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| ### Offshore platforms Several different types of platforms are in use. They are normally piled steel or concrete structures, the latter held in position on the sea floor by gravity. Tension Leg Platforms (TLP) consist of semi-submersible platforms secured to flooded caissons on the sea floor vertically below them by wires kept in tension by the buoyancy of the platform. Platforms may serve a number of purposes. They may carry any of the following equipment: drilling and production equipment; oil and gas separation and treatment plants; pump-line stations; and electricity generators. They may be fitted with cranes, a helicopter landing deck, and accommodation for up to 350 people. Platforms may stand singly or in groups connected by pipelines. Some stand close together in a complex, with bridges and underwater cables connecting them. Unwanted gas or oil is sometimes burnt from a flaring boom extending from the platform or from a nearby flare stack. If it is required to encode a permanent offshore platform, it must be done using the feature Offshore Platform. Remarks: - The attribute height is only relevant for fixed platforms, and is referred to the vertical datum (see clause X.X). - The attribute vertical length is only relevant for floating platforms, and is referred to the seal level. - If it is required to encode sites of dismantled platforms, this must be done using Foul Ground features (see clause X.X). - Platforms may carry lights (see clause X.X), fog signals (see clause X.X) and helicopter platforms (see clause X.X). Where fitted, lights should be encoded as described in clause X.X, with the Offshore Platform being used as the structure feature for the Light equipment feature(s). - The extent and nature of any restricted area related to an offshore platform should be encoded using a Restricted Area feature (see clause X.X). ### Wellheads In the course of developing an oil or gas field, numerous wells may be drilled. Some, which will not be required again, may be sealed at or below the sea floor and abandoned; such wells must not be encoded, as they have no relevance to navigation. A submerged wellhead is a submarine structure projecting some distance above the sea floor and capping a temporarily abandoned (or “suspended”) oil or gas well. Their associated pipes and other equipment usually project some 2 - 6 metres, but in some cases as much as 15 metres, above the sea floor. Some may be covered by steel cages to avoid snagging trawling gear. In certain instances, a wellhead may project above the sea surface. Wellheads must be encoded on at least the largest maximum display scale ENC data, together with associated buoys, as a hazard to fishing and, dependant on depth, as a hazard to deep-draught vessels and towed structures. If it is required to encode wellheads, this must be done using Obstruction features of type point (see clause X.X), with attributes: category of obstruction - 2 - wellhead height status - 4 - not in use (disused) value of sounding vertical length - vertical length of the wellhead above the seabed water level effect - 2 - always dry (for wellheads that protrude at high water) 3 - always under water/submerged ### Offshore safety zones Under UNCLOS, a coastal State may establish safety zones around artificial islands, installations and structures in their EEZ and on their continental shelf. These installations include drilling rigs, production platforms, wellheads, moorings and other associated structures. Safety zones normally extend 500 metres from the outermost points of the installations. Within these zones, appropriate measures can be taken to ensure the safety of navigation and of the installations. If it is required to encode an offshore safety zone, it must be done using a Restricted Area feature (see clause X.X), with attribute category of restricted area = 1 (offshore safety zone). ### Flare stacks As with refineries on land, offshore terminals may burn off gas from production platforms or from “flare stacks” set up as separate structures a short distance from the production platforms. If it is required to indicate the presence of a flare stack on an offshore platform, it must be done through population of the attribute flare stack. Distinction: Buoy, installation; hulk; landmark; offshore production area. |  |  |  |  |  |

## Light vessels (LightVessel)

| Definition: LIGHT VESSEL. A distinctively marked vessel anchored or moored at a charted point, to serve as an aid to navigation. By night, it displays a characteristic light(s) and is usually equipped with other devices, such as fog signal, submarine sound signal, and radio-beacon, to assist navigation. Also called light ship. (IHO Dictionary – S-32). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Light vessel |  |  |  |  |  |
| Primitives: Point |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Colour |  | 1 : white 2 : black 3 : red 4 : green 5 : blue 6 : yellow 7 : grey 8 : brown 9 : amber 10 : violet 11 : orange 12 : magenta 13 : pink |  | EN | 1,* |
| Colour pattern |  | 1 : horizontal stripes 2 : vertical stripes 3 : diagonal stripes 4 : squared 5 : stripes (direction unknown) 6 : border stripe |  | EN | 0,1 |
| Radar conspicuous |  |  |  | BO | 0,1 |
| Visually conspicuous |  |  |  | BO | 0,1 |
| Horizontal accuracy |  |  |  | RE | 0,1 |
| Horizontal length |  |  |  | RE | 0,1 |
| Horizontal width |  |  |  | RE | 0,1 |
| Nature of construction |  | 6 : wooden 7 : metal 9 : painted |  | EN | 0,* |
| Object name in national language |  |  |  | (S) TE | 0,1 |
| Object name |  |  |  | (S) TE | 0,1 |
| Status |  | 1 : permanent 2 : occasional 4 : not in use 5 : periodic/intermittent 7 : temporary 8 : private 14 : public 16 : watched 17 : un-watched |  | EN | 0,* |
| Vertical accuracy |  |  |  | RE | 0,1 |
| Vertical length |  |  |  | RE | 0,1 |
| Manned structure |  |  |  | BO | 0,1 |
| Aid availability category |  | 1 : Category 1 2 : Category 2 3 : Category 3 |  | EN | 1,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Textual description in national language |  |  |  | (S) TE | 0,1 |
| Textual description |  |  |  | (S) TE | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  |  | TE | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| ### Lights vessels Major floating lights are generally classed as those with a nominal range in excess of 10 nautical miles. Special circumstances, e.g. an isolated location, may mean that a floating light of lower range is given this status. The structure on which the light is fixed will be a light vessel, a major light float or a LANBY (Large Automatic Navigational Buoy, which is a type of superbuoy). If it is required to encode a light vessel, it must be done using the feature Light Vessel. Remarks: - The attribute colour pattern must be populated if more than one value is populated for the mandatory attribute colour. Distinction: Beacon, cardinal; beacon, isolated danger; beacon, lateral; beacon, safe water; beacon special purpose/general; buoy, cardinal; buoy, emergency wreck marking; buoy, installation; buoy, isolated danger; buoy, lateral; buoy, safe water; buoy, special purpose/general; light float. |  |  |  |  |  |

## Piles (Pile)

| Definition: PILE. A long heavy timber or section of steel, wood, concrete, etc., forced into the earth or seabed which may serve as a support, as for a pier, or a free standing pole within a marine environment. (IHO Dictionary – S-32). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Pile |  |  |  |  |  |
| Primitives: Point, Curve, Surface |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Category of pile |  | 1 : stake 3 : post 4 : tripodal 5 : piling 6 : area of piles 7 : pipe |  | EN | 0,1 |
| Colour |  | 1 : white 2 : black 3 : red 4 : green 5 : blue 6 : yellow 7 : grey 8 : brown 9 : amber 10 : violet 11 : orange 12 : magenta 13 : pink |  | EN | 0,* |
| Colour pattern |  | 1 : horizontal stripes 2 : vertical stripes 3 : diagonal stripes 4 : squared 5 : stripes (direction unknown) 6 : border stripe |  | EN | 0,1 |
| Condition |  | 1 : under construction 2 : ruined 5 : planned construction |  | EN | 0,1 |
| Visually conspicuous |  |  |  | BO | 0,1 |
| Height |  |  |  | RE | 0,1 |
| Vertical accuracy |  |  |  | RE | 0,1 |
| Vertical length |  |  |  | RE | 0,1 |
| Vertical datum |  | 3 : Mean sea level 16 : Mean high water 17 : Mean high water springs 18 : High water 19 : Approximate mean sea level 20 : High water springs 21 : Mean higher high water 24 : Local datum 25 : International great lakes datum 1985 26 : Mean water level 28 : Higher high water large tide 29 : Nearly highest high water 30 : Highest astronomical tide (HAT) |  | EN | 0,1 |
| Aid availability category |  | 1 : Category 1 2 : Category 2 3 : Category 3 |  | EN | 1,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Textual description in national language |  |  |  | (S) TE | 0,1 |
| Textual description |  |  |  | (S) TE | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  |  | TE | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| ### Piles If it is required to encode a pile or post that is not used as a mooring/warping facility or an aid to navigation, it must be done using the feature Pile. Remarks: - Stumps of piles or posts that are dangerous to navigation must be encoded, where required, using Obstruction features (see clause X.X), with attribute category of obstruction = 1 (snag/stump), and must not be encoded using Pile. - The value category of pile = 5 (piling), sometimes termed “row of piles” or “sheet piling”, should only be used for Pile of type curve. Point primitive may be used for smaller maximum display scale ENC data. - Stakes and posts that are identified on the source to serve the purpose of aids to navigation must be encoded, where required, using the appropriate beacon feature (e.g. Beacon Special Purpose/General), with attribute beacon shape = 1 (stake, pole, perch, post). - See clause X.X for details of how to encode a pile or post that is used as a mooring/warping facility. Distinction: Beacon, cardinal; beacon, isolated danger; beacon, lateral; beacon, safe water; beacon special purpose/general; mooring/warping facility. |  |  |  |  |  |

## Silo tank (SiloTank)

| Definition: SILO/TANK. A silo is a large storage structure used for storing loose materials. (Defence Geospatial Information Working Group; Feature Data Dictionary Register, 2012). A tank is a container used for the storage of liquids and/or gases. (Adapted from Defence Geospatial Information Working Group; Feature Data Dictionary Register, 2012). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Silo/tank |  |  |  |  |  |
| Primitives: Point, Surface |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Building shape |  | 5 : high-rise building 6 : pyramid 7 : cylindrical 8 : spherical 9 : cubic |  | EN | 0,1 |
| Category of silo/tank |  | 1: silo in general 2: tank in general 3: grain elevator 4: water tower |  | EN | 0,1 |
| Colour |  | 1 : white 2 : black 3 : red 4 : green 5 : blue 6 : yellow 7 : grey 8 : brown 9 : amber 10 : violet 11 : orange 12 : magenta 13 : pink |  | EN | 0,* |
| Colour pattern |  | 1 : horizontal stripes 2 : vertical stripes 3 : diagonal stripes 4 : squared 5 : stripes (direction unknown) 6 : border stripe |  | EN | 0,1 |
| Condition |  | 1 : under construction 2 : ruined 5 : planned construction |  | EN | 0,1 |
| Radar conspicuous |  |  |  | BO | 0,1 |
| Visually conspicuous |  |  |  | BO | 0,1 |
| Elevation |  |  |  | RE | 0,1 |
| Height |  |  |  | RE | 0,1 |
| Nature of construction |  | 1 : masonry 2 : concreted 6 : wooden 7 : metal 8 : glass reinforced plastic (GRP) 9 : painted |  | EN | 0,* |
| Object name in national language |  |  |  | (S) TE | 0,1 |
| Object name |  |  |  | (S) TE | 0,1 |
| Product |  | 1 : oil 2 : gas 3 : water 7 : chemicals 8 : drinking water 9 : milk 14 : sand 18 : liquefied natural gas (LNG) 19 : liquefied petroleum gas (LPG) 20 : wine 21 : cement 22 : grain 23 : ice |  | EN | 0,* |
| Status |  | 1 : permanent 4 : not in use 12 : illuminated 13 : historic |  | EN | 0,* |
| Vertical accuracy |  |  |  | RE | 0,1 |
| Vertical length |  |  |  | RE | 0,1 |
| Vertical datum |  | 3 : Mean sea level 16 : Mean high water 17 : Mean high water springs 18 : High water 19 : Approximate mean sea level 20 : High water springs 21 : Mean higher high water 24 : Local datum 25 : International great lakes datum 1985 26 : Mean water level 28 : Higher high water large tide 29 : Nearly highest high water 30 : Highest astronomical tide (HAT) |  | EN | 0,1 |
| Aid availability category |  | 1 : Category 1 2 : Category 2 3 : Category 3 |  | EN | 1,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Textual description in national language |  |  |  | (S) TE | 0,1 |
| Textual description |  |  |  | (S) TE | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  |  | TE | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement Aton maintenance record |  |  |  | (S) TE | 0,1 |
|  |  |  |  | URI | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| ### Tanks, silos Isolated tanks or gasholders may be good landmarks and should be represented true to scale (i.e. as surface) where possible, to enable them to be used as fixing marks. Groups of tanks, as at a refinery, may be useful for general identification of position but cannot usually be used for precise position-fixing because of uncertainty of the location of individual tanks. If it is required to encode a tank or silo, it must be done using the feature Silo/Tank. Remarks: - For buildings, see clause X.X; for landmarks, see clause X.X. For common encoding combinations, see clause X.X. - Groups of tanks in close proximity (tank farm) must be encoded, where required, using the feature Production/Storage Area (see cluse X.X). Individual, visually conspicuous tanks within a tank farm may be encoded as Silo/Tank within the Production/Storage Area. Distinction: Building, single; control point; landmark; production/storage area. |  |  |  |  |  |

## Landmarks (Landmark)

| Definition: LANDMARK. Any prominent object on land which can be used in determining a location or a direction. (IHO Dictionary – S-32). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Landmark |  |  |  |  |  |
| Primitives: Point, Curve, Surface |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Category of landmark |  | 1 : cairn 2 : cemetery 3 : chimney 4 : dish aerial 5 : flagstaff (flagpole) 6 : flare stack 7 : mast 8 : windsock 9 : monument 10 : column (pillar) 11 : memorial plaque 12 : obelisk 13 : statue 14 : cross 15 : dome 16 : radar scanner 17 : tower 18 : windmill 19 : windmotor 20 : spire/minaret 21 : large rock (or boulder) on land 22 : triangulation mark 23 : boundary mark 24 : observation wheel 25 : Torii |  | EN | 1,* |
| Colour |  | 1 : white 2 : black 3 : red 4 : green 5 : blue 6 : yellow 7 : grey 8 : brown 9 : amber 10 : violet 11 : orange 12 : magenta 13 : pink |  | EN | 0,* |
| Colour pattern |  | 1 : horizontal stripes 2 : vertical stripes 3 : diagonal stripes 4 : squared 5 : stripes (direction unknown) 6 : border stripe |  | EN | 0,1 |
| Condition |  | 1 : under construction 2 : ruined 4 : wingless 5 : planned construction |  | EN | 0,1 |
| Radar conspicuous |  |  |  | BO | 0,1 |
| Visually conspicuous |  |  |  | BO | 0,1 |
| Elevation |  |  |  | RE | 0,1 |
| Function |  | 2 : harbour-master’s office 3 : custom office 4 : health office 5 : hospital 6 : post office 7 : hotel 8 : railway station 9 : police station 10 : water-police station 11 : pilot office 12 : pilot lookout 13 : bank office 14 : headquarters for district control 15 : transit shed/warehouse 16 : factory 17 : power station 18 : administrative 19 : educational facility 20 : church 21 : chapel 22 : temple 23 : pagoda 24 : Shinto shrine 25 : Buddhist temple 26 : mosque 27 : marabout 28 : lookout 29 : communication 30 : television 31 : radio 32 : radar 33 : light support 34 : microwave 35 : cooling 36 : observation 37 : time ball 38 : clock 39 : control 40 : airship mooring 41 : stadium 42 : bus station 43 : passenger terminal building 44 : sea rescue control 45 : observatory 46 : ore crusher 47 : boathouse 48 : pumping station |  | EN | 0,* |
| Height |  |  |  | RE | 0,1 |
| Nature of construction |  | 1 : masonry 2 : concreted 3 : loose boulders 6 : wooden 7 : metal 8 : glass reinforced plastic (GRP) 9 : painted 10 : latticed |  | EN | 0,* |
| Object name in national language |  |  |  | (S) TE | 0,1 |
| Object name |  |  |  | (S) TE | 0,1 |
| Status |  | 1 : permanent 2 : occasional 4 : not in use 5 : periodic/intermittent 7 : temporary 8 : private 12 : illuminated 13 : historic 14 : public 16 : watched 17 : un-watched |  | EN | 0,* |
| Vertical accuracy |  |  |  | RE | 0,1 |
| Vertical length |  |  |  | RE | 0,1 |
| Vertical datum |  | 3 : Mean sea level 16 : Mean high water 17 : Mean high water springs 18 : High water 19 : Approximate mean sea level 20 : High water springs 21 : Mean higher high water 24 : Local datum 25 : International great lakes datum 1985 26 : Mean water level 28 : Higher high water large tide 29 : Nearly highest high water 30 : Highest astronomical tide (HAT) |  | EN | 0,1 |
| Manned structure |  |  |  | BO | 0,1 |
| Aid availability category |  | 1 : Category 1 2 : Category 2 3 : Category 3 |  | EN | 1,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Textual description in national language |  |  |  | (S) TE | 0,1 |
| Textual description |  |  |  | (S) TE | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  |  | TE | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| ### Buildings, landmarks, tanks, silos Depending on height and the topographic relief, structures considered to be landmarks should be encoded up to several miles/kilometres inland. Waterfront, landmark and some public buildings should be encoded precisely and individually on the larger maximum display scale ENC data. When representing buildings generally, forming urban and suburban areas, villages, and other built-up areas, the aim of the compiler must be to create the correct impression of the extent of the built-up area and the density of the buildings. If it is required to encode a landmark (other than a tank or silo), it must be done using the feature Landmark. Remarks: - For buildings, see clause X.X; for silos, tanks and water towers, see clause X.X. For common encoding combinations, see clause X.X. For structures such as offshore wind turbines, see clause X.X. - A water tower must be encoded, where required, using the feature Silo/Tank (see clause X.X). - A ruined landmark should be encoded in the same way as the feature in good condition, but with attribute condition = 2 (ruined). - Radio and television masts and towers are likely to be visible over long distances and should be encoded as landmarks, even when well inland. They will usually carry air obstruction lights. - To aid identification of landmarks by the mariner it may be useful to add the height of the top of the structure above ground level (vertical length) or above the general height datum (height). - Buildings constructed as places of worship often form significant landmarks; their size and structure incorporating towers, spires, cupolas, etc often render them conspicuous. These buildings when known to be prominent or conspicuous should be encoded up to several miles inland (see Figure below, examples (a) and (b). - The attribute category of special purpose mark should only be used if the Landmark is used as the front or rear lead for a transit, clearing line or measured distance, or for a leading line. Values for category of special purpose mark such as 16 (leading mark), 17 (measured distance mark) or 41 (clearing mark) in particular should be used for these purposes. See also clause X.X. - When a building is shown as a surface, indicating its true shape, and it is required to encode a prominent feature such as a tower or spire that is part of the structure, two features must be created (see Figure below): - a Building feature of type surface for the main building, - a Landmark feature of type point for the prominent feature. a) b) c) Landmark (point) function = 20 category of landmark = 17 Building (surface) function = 20 Landmark (point) function = 20 category of landmark = 17 Landmark (point) category of landmark = 17 Tr Tr - Not all landmarks are visually conspicuous. If a feature is visually conspicuous (i.e. it is distinctly and noticeably visible from seaward), the attribute visually conspicuous must be encoded (see S-4 – B-340). Distinction: Beacon, special purpose/general; building single; control point; daymark; pylon/bridge support; silo/tank ; topmark. |  |  |  |  |  |

## Light floats (LightFloat)

| Definition: LIGHT FLOAT. A boat-like structure used instead of a light buoy in waters where strong streams or currents are experienced, or when a greater elevation than that of a light buoy is necessary. (IHO Dictionary – S-32). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Light float |  |  |  |  |  |
| Primitives: Point |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Colour |  | 1 : white 2 : black 3 : red 4 : green 5 : blue 6 : yellow 7 : grey 8 : brown 9 : amber 10 : violet 11 : orange 12 : magenta 13 : pink |  | EN | 1,* |
| Colour pattern |  | 1 : horizontal stripes 2 : vertical stripes 3 : diagonal stripes 4 : squared 5 : stripes (direction unknown) 6 : border stripe |  | EN | 0,1 |
| Radar conspicuous |  |  |  | BO | 0,1 |
| Visually conspicuous |  |  |  | BO | 0,1 |
| Horizontal length |  |  |  | RE | 0,1 |
| Horizontal width |  |  |  | RE | 0,1 |
| Horizontal accuracy |  |  |  | RE | 0,1 |
| Nature of construction |  | 6 : wooden 7 : metal 9 : painted 10 : latticed |  | EN | 0,* |
| Object name in national language |  |  |  | (S) TE | 0,1 |
| Object name |  |  |  | (S) TE | 0,1 |
| Status |  | 1 : permanent 2 : occasional 4 : not in use 5 : periodic/intermittent 7 : temporary 8 : private 14 : public 16 : watched 17 : un-watched |  | EN | 0,* |
| Vertical length |  |  |  | RE | 0,1 |
| Vertical accuracy |  |  |  | RE | 0,1 |
| Manned structure |  |  |  | BO | 0,1 |
| Aid availability category |  | 1 : Category 1 2 : Category 2 3 : Category 3 |  | EN | 1,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Textual description in national language |  |  |  | (S) TE | 0,1 |
| Textual description |  |  |  | (S) TE | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  |  | TE | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| ### Lights floats If it is required to encode a light float, it must be done using the feature Light Float. Remarks: - The light on a light float is a separate feature, handled as with buoys, beacons, etc. Distinction: Buoy, cardinal; buoy, emergency wreck marking; buoy, installation; buoy, isolated danger; buoy, lateral; buoy, safe water; buoy, special purpose/general; light vessel. |  |  |  |  |  |

# AtoN Equipment

## Topmarks	(Topmark)

| Definition: TOPMARK. A characteristic shape secured at the top of a BUOY or BEACON to aid in its identification. (IHO Dictionary – S-32). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Tomark |  |  |  |  |  |
| Primitives: Point |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Colour |  | 1 : white 2 : black 3 : red 4 : green 5 : blue 6 : yellow 7 : grey 8 : brown 9 : amber 10 : violet 11 : orange 12 : magenta 13 : pink |  | EN | 1,* |
| Colour pattern |  | 1 : horizontal stripes 2 : vertical stripes 3 : diagonal stripes 4 : squared 5 : stripes (direction unknown) 6 : border stripe |  | EN | 0,1 |
| Height |  |  |  | RE | 0,1 |
| Navigational system of marks |  | 1 : IALA A 2 : IALA B 3 : no system 4 : other system |  | EN | 0,1 |
| Status |  | 1 : permanent 2 : occasional 4 : not in use 5 : periodic/intermittent 7 : temporary 8 : private 14 : public 16 : watched 17 : un-watched |  | EN | 0,* |
| Topmark/daymark shape |  | 1 : cone, point up 2 : cone, point down 3 : sphere 4 : 2 spheres 5 : cylinder (can) 6 : board 7 : x-shape (St. Andrew’s cross) 8 : upright cross (St George’s cross) 9 : cube, point up 10 : 2 cones, point to point 11 : 2 cones, base to base 12 : rhombus (diamond) 13 : 2 cones (points upward) 14 : 2 cones (points downward) 15 : besom, point up (broom or perch) 16 : besom, point down (broom or perch) 17 : flag 18 : sphere over rhombus 19 : square 20 : rectangle, horizontal 21 : rectangle, vertical 22 : trapezium, up 23 : trapezium, down 24 : triangle, point up 25 : triangle, point down 26 : circle 27 : two upright crosses (one over the other) 28 : T-shape 29 : triangle pointing up over a circle 30 : upright cross over a circle 31 : rhombus over a circle 32 : circle over a triangle pointing up 33 : other shape (see information) |  | EN | 1,1 |
| Vertical accuracy |  |  |  | RE | 0,1 |
| Vertical length |  |  |  | RE | 0,1 |
| Vertical datum |  | 3 : Mean sea level 16 : Mean high water 17 : Mean high water springs 18 : High water 19 : Approximate mean sea level 20 : High water springs 21 : Mean higher high water 24 : Local datum 25 : International great lakes datum 1985 26 : Mean water level 28 : Higher high water large tide 29 : Nearly highest high water 30 : Highest astronomical tide (HAT) |  | EN | 0,1 |
| Remote monitoring system |  |  |  | (S) TE | 0,* |
| Remotely Monitored |  |  |  | BO | 0,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Textual description in national language |  |  |  | (S) TE | 0,1 |
| Textual description |  |  |  | (S) TE | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  |  | TE | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |

## Lights (Light)

| Definition: LIGHT. A luminous or lighted AID TO NAVIGATION. For internationally agreed abbreviations of light characteristics see the International Chart Specifications. (IHO Dictionary – S-32). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Light |  |  |  |  |  |
| Primitives: Point |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Category of light |  | 4 : leading light 5 : aero light 8 : flood light 9 : strip light 10 : subsidiary light 11 : spotlight 12 : front 13 : rear 14 : lower 15 : upper 17 : emergency 18 : bearing light 19 : horizontally disposed 20 : vertically disposed |  | EN | 0,* |
| Colour |  | 1 : white 2 : black 3 : red 4 : green 5 : blue 6 : yellow 7 : grey 8 : brown 9 : amber 10 : violet 11 : orange 12 : magenta 13 : pink |  | EN | 1,* |
| Exhibition condition of light |  | 1 : light shown without change of character 2 : daytime light 3 : fog light 4 : night light |  | EN | 0,1 |
| Height |  |  |  | RE | 0,1 |
| Light characteristic |  | 1 : fixed 2 : flashing 3 : long-flashing 4 : quick-flashing 5 : very quick-flashing 6 : ultra quick-flashing 7 : isophased 8 : occulting 9 : interrupted quick-flashing 10 : interrupted very quick flashing 11 : interrupted ultra quick flashing 12 : morse 13 : fixed and flash 14 : flash and long-flash 15 : occulting and flash 16 : fixed and long-flash 17 : occulting alternating 18 : long-flash alternating 19 : flash alternating 25 : quick-flash plus long-flash 26 : very quick-flash plus long flash 27 : ultra quick-flash plus long flash 28 : alternating 29 : fixed and alternating flashing |  | (S) EN | 1,1 |
| Light visibility |  | 1 : high intensity 2 : low intensity 3 : faint 4 : intensified 5 : unintensified 6 : visibility deliberately restricted 7 : obscured 8 : partially obscured 9 : visible in line of range |  | EN | 0,* |
| Navigational system of marks |  | 1 : IALA A 2 : IALA B 3 : no system 4 : other system |  | EN | 0,1 |
| Multiplicity of lights |  |  |  | IN | 0,1 |
| Object name in national language |  |  |  | (S) TE | 0,1 |
| Object name |  |  |  | (S) TE | 0,1 |
| Orientation |  |  |  | RE | 0,1 |
| Sector limit one |  |  |  | IN | 0,1 |
| Sector limit two |  |  |  | IN | 0,1 |
| Signal group |  |  |  | (S) TE | 0,1 |
| Signal period |  |  |  | (S) RE | 0,1 |
| Signal sequence |  |  |  | (S) TE | 0,1 |
| Status |  | 1 : permanent 2 : occasional 4 : not in use 5 : periodic/intermittent 7 : temporary 8 : private 14 : public 16 : watched 17 : un-watched |  | EN | 0,* |
| Vertical accuracy |  |  |  | RE | 0,1 |
| Vertical length |  |  |  | RE | 0,1 |
| Vertical datum |  | 3 : Mean sea level 16 : Mean high water 17 : Mean high water springs 18 : High water 19 : Approximate mean sea level 20 : High water springs 21 : Mean higher high water 24 : Local datum 25 : International great lakes datum 1985 26 : Mean water level 28 : Higher high water large tide 29 : Nearly highest high water 30 : Highest astronomical tide (HAT) |  | EN | 0,1 |
| Type of light |  |  |  | (S) TE | 0,1 |
| Type of battery |  |  |  | (S) TE | 0,1 |
| Value of geographical range |  |  |  | RE | 0,1 |
| Value of luminous range |  |  |  | RE | 0,1 |
| Remote monitoring system |  |  |  | (S) TE | 0,* |
| Remotely Monitored |  |  |  | BO | 0,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Textual description in national language |  |  |  | (S) TE | 0,1 |
| Textual description |  |  |  | (S) TE | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  |  | TE | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| If the actual structure that supports a light in a water area is unknown, a feature type Pile with no attributes should also be encoded at the same position as the light. 7.2.1 Description of lights If it is required to encode a light and its sectors, each sector of the light must be encoded using one separate feature type Light. These features must be child feature of the same parent feature, which is either the structure object or one of the Light, so that the relationship between them is indicated using the parent/child relationship mechanism Remarks: - If it is required to encode details of the lighting technology (e.g. neon), it must be done using the attribute Information - If it is required to encode the purpose of a marine spotlight, it must be done using Information - For lights, the allowable values for the attribute Colour are 1 (white); 3 (red); 4 (green); 5 (blue); 6 (yellow); 9 (amber); 10 (violet); 11 (orange); and empty (null) value. - The attribute value Light characteristic = 28 (alternating) is not consistent with the existing IALA definition and should be read as follows: “a signal light that shows continuously, in any given direction, two or more colours in a regularly repeated sequence with a regular periodicity”. - For purposes of consistency with the IHO and IALA dictionaries, the meaning for attribute values Light characteristic = 13, 14, 15, and 16 should be read as follows: Light characteristic = 13 (fixed/flash) Fixed and flashing; Light characteristic = 14 (fixed/long flash) Fixed and long flashing; Light characteristic = 15 (occulting/flash) Occulting and flashing; Light characteristic = 16 (occulting/long flash) Occulting and long flashing. - AtoN offices focus the maintenance of AtoN system like equipment and structure. The types and kinds information of AtoN can be important to AtoN offices. The optional attribute IALA_typeOfLight (text) on the Lights feature type is used to encode the type of light bulb, e.g. Bulb type(250mm, 300mm, 400mm), LED type(Integral type, 200, 200HI, 250, 300, 350) - The optional attribute IALA_typeOfBattery (text) on the Lights feature type is used to encode the type of battery e.g. types of storage battery for light : Hi-Ca 100, PS-250E, LDA-400, VGS-350 - 7.2.2 Types and functions of lights - If it is required to encode types and functions of lights, this must be done using the attribute Category of light - 7.2.3 Rhythms of lights - If it is required to encode the rhythms of lights, this must be done using the attributes Light characteristic and Signal group - . The use of these attributes is defined in the following table; it contains the most common examples of coding; other coding combinations are possible: Rhythms of lights; F; Oc; Oc(2); Oc(2+3); Iso; Fl; Fl(3); LFl. Light characteristic; 1; 8; 8; 8; 7; 2; 2; 3. Signal group; prohibited; (1); (2); (2+3); (1); (1); (3); (1). Rhythms of lights; Q; Q(3); IQ; VQ; VQ(3); IVQ; UQ; IUQ. Light characteristic; 4; 4; 9; 5; 5; 10; 6; 11. Signal group; (1); (3); ( ); (1); (3); ( ); (1); ( ). Rhythms of lights; Mo(K); FFl; Q(6)+LFl; VQ(6)+LFl; Al.WR; Al.Fl.WR; Al.Fl(2W+1R); Al.Oc(4)WR. Light characteristic; 12; 13; 25; 26; 28; 19; 19; 17. Signal group; (K); ()(1); (6)(1); (6)(1); ( ); (1); (2+1); (4). table 12.4. Some lights recently constructed may appear to the mariner as "fixed and flashing - FFL" by night, while the real world object actually comprises two separate lights vertically disposed, one fixed and the other flashing (F&Fl). When it is known that two separate features actually exist, they must be encoded as separate objects, in this case two Light objects, one with attribute Light characteristic = 1 (fixed) and the other with Light characteristic = 2 (flashing), and not as one Light with Light characteristic = 13 (fixed and flashing). ### Elevations of lights If it is required to encode the elevation of a light on a fixed structure, it must be done using the attribute Height. If it is required to encode the height above the water surface of a light on a floating structure, it must be done using the attribute Information on the Light object. ### Times of exhibition and exhibition conditions ### 7.2.5.1 Night lights If it is required to encode a night light, it must be done using a Light object, with attribute Exhibition condition of light = 4 (night light). ### 7.2.5.2 Unwatched lights This information should not be encoded, but unwatched (unmanned) lights, with no standby or emergency arrangements, may be encoded using attribute Status = 17 (unwatched). ### 7.2.5.3 Occasional lights ### If it is required to encode an occasional light, it must be done using attribute Status = 2 (occasional). ### 7.2.5.4 Private lights Some AtoNs are established by non-government organization for research, academic purpose. These AtoNs are considered private and are encoded with Status=8 (private). If it is required to encode a private light that is not regularly exhibited, it must be done using Status = 2,8 (occasional, private). ### Daytime lights If it is required to encode a daytime light, it must be done using attribute Exhibition condition of light = 1 (light shown without change of character). If it is required to encode a light having characteristics shown by day different to those shown at night, it must be done by encoding two Light objects sharing the same point spatial object: - one Light object with Exhibition condition of light = 2 (daytime light), - one Light object with Exhibition condition of light = 4 (night light). ### Fog lights - If it is required to encode a fog light , it must be done using a Light object, with attributes Exhibition condition of light = 3 (fog light) and Status = 2 (occasional). If it is required to encode a light having characteristics shown in fog that are different to those shown in conditions of normal visibility, it must be done by encoding two Light objects sharing the same point spatial object: - one Light object with attributes Exhibition condition of light = 3 (fog light) and Status = 2 (occasional) - one Light object with attributes Exhibition condition of light = 2 (daytime light) or 4 (night light) and attribute Information = Character of the light changes in fog. ### Sector lights and lights not visible all round ### 7.2.8.1 Sector lights ###### Each sector in which the light is visible from seaward must be encoded as one LIGHTS object. ###### There must be no object created to encode a sector where no light is exhibited. ###### Limits of sectors must be encoded using attributes SECTR1 and SECTR2. ###### Where it is required to encode an oscillating light sector (see S-4 – B-475.7), it should be done as follows: ###### For lights in the IALA A system that are alternating and oscillate increasingly from white to green (to starboard) and red (to port) with increasing deviation from the track defined by the directional light: ###### Light: Light characteristic = 28 (alternating); Colour = 1,3 (white, red); Sector limit one; Sector limit two; Signal period; Signal period; Information = White phase decreases as bearing to light increases Light: Light characteristic = 28 (alternating); Colour = 1,4 (white, green); Sector limit one; Sector limit two; Signal period; Signal group; Information = White phase increases as bearing to light increases ###### For lights in the IALA B system that are alternating and oscillate increasingly from white to red (to starboard) and green (to port) with increasing deviation from the track defined by the directional light; transpose the colours red and green in the above encoding. ###### For lights in the IALA A system that are occulting green (to starboard) and red (to port) which oscillate with increasing period of eclipse to isophased or flashing with increasing deviation from the track defined by the directional light:12.8.6.2 Lights obscured by obstructions Light: Light characteristic = 8 (occulting); Colour = 3 (red); Sector limit one; Sector limit two; Signal period; Signal group; Information = Light phase decreases as bearing to light increases Light: Light characteristic= 8 (occulting); Colour = 4 (green); Sector limit one; Sector limit two; Signal period; Signal group; Information = Light phase increases as bearing to light increases For lights in the IALA B system that are occulting red (to starboard) and green (to port) which oscillate with increasing period of eclipse to isophased or flashing with increasing deviation from the track defined by the directional light; transpose the colours red and green in the above encoding. Oscillating lights which are not IALA should be encoded similar to the above. For instance, where a light contains white sectors that are occulting and oscillate with increasing period of eclipse to isophased or flashing with increasing deviation from the track defined by the directional light: For the sector to port of the track defined by the directional light: Light: Light characteristic = 8 (occulting); Colour = 1 (white); Sector limit one; Sector limit two; Signal period; Signal group; Information = Light phase decreases as bearing to light increases For the sector to starboard of the track defined by the directional light: Light: Light characteristic= 8 (occulting); Colour = 1 (white); Sector limit one; Sector limit two; Signal period; Signal group; Information = Light phase increases as bearing to light increases Figure 22 – Obscured light sectors If an encoded light is obscured in a part of the navigable area of a sector beyond an offshore obstruction, it must be encoded as several Light objects. The partially obscured sector of (b), seaward of the island, must be encoded as a Light object, with attributes Light visibility = 8 (partially obscured) and Information = Sector obscured only beyond .…. The sectors in which the light is visible from seaward ((a) and (c)) must be encoded as separate Light objects. If there is no navigable water between the light and the obstacle, the masked sector (e) must be encoded as a Light object, with Light visibility = 3 (faint) or 7 (obscured). #### White fairway sectors The light sectors must be encoded as separate Light objects. The fairway defined by the succession of navigable areas in the white sectors may be encoded using the object class Fairway #### Leading lights - If it is required to encode a leading light, it must be done using a Light object, with attribute: Category of light = 4,12 - front leading light 4,13 - rear leading light 4,14 - lower leading light 4,15 - upper leading light Remarks: - The attribute Orientation must not be used for leading lights, except for directional lights. - Even if, on the source, the leading lights are merged into a single symbol, at least one Light object must be created for each light. - The leading line must be encoded. #### Directional lights If it is required to encode a directional light, it must be done using a Light object, with attribute Category of light = 1 (directional function) and, if the light is intensified in this sector, with attribute Light visibility = 4(intensified). The mandatory attribute Orientation must only be encoded to indicate the orientation, measured from seaward, of the leading line of the directional light when the attributes Sector limit one and Sector limit two are not populated, and there is no Recommended track or Navigation line object associated with the directional light. Where the Light has attributes Sector limit one and Sector limit two populated, and/or there is an associated Recommended track and/or Navigation line encoded, Orientation for the Light must be populated with and empty (null) value. If it is required to encode the recommended track and/or navigation line associated with a directional light, it must be done using the method described in clause 10.1. #### 7.2.8.5 Moiré effect lights If it is required to encode a moiré effect light, it must be done using a Light object, with attribute Category of light = 16 (moiré effect). The mandatory attribute Orientation must only be encoded to indicate the orientation, measured from seaward, of the leading line of the moiré effect light when the attributes Sector limit one and Sector limit two are not populated, and there is no Recommended track or Navigation line object associated with the moiré effect light. Where the Light has attributes Sector limit one and Sector limit two populated, and/or there is an associated Recommended track and/or Navigation line, Orientation for the Light must be populated with and empty (null) value. If it is required to encode the recommended track and/or navigation line associated with a moiré effect light, it must be done using the method described in clause 10.1. ### Various special types of lights Type; Category of light; Remarks. Subsidiary light; 10. Aero light; 5. Air obstruction light; 6. Fog detector light; 7. Bearing light; 18. Flood light; 8; Only to encode flood lights that are visible from seaward. The illuminated structure should be encoded using appropriate object classes, with attribute Status = 12 (illuminated). Synchronised lights; Status =15. Synchronised lights may be associated using the collection object Assocation. Strip light; 9. Spot light; 11; Only to encode spot lights that are visible from seaward. The illuminated feature should be encoded using appropriate object classes, with attribute Status = 12 (illuminated). Emergency light; 17; Must be encoded as a separate object to the main Light object. Horizontally disposed lights; 19; The number of lights must be encoded using attribute Multiplicity of lights. Vertically disposed lights; 20; The number of lights must be encoded using attribute Multiplicity of lights. table 12.5. For ECDIS display reasons, where an encoded strip light serves the purpose of an aid to navigation, the attribute Category of light = 9 (strip light) for the Light object should not be populated. To identify that the aid to navigation is a strip light, the attribute Information should be populated with Strip light or equivalent for the Light. ### Light structures If a light that is on a fixed structure is located in a water area and the structure that supports it is unknown, a Pile object with no attributes should also be encoded at the same position as the light. This will ensure that a symbol will be shown on ECDIS systems when the Light objects are not displayed during daytime navigation. ### 12.8.9 Weather observation equipment AtoN section of local offices have a role to observe real time marine weather and established marine weather observation equipment on the AtoN structure like Land mark, buoy and beacon. The feature type IALA_weatherObservation is used encode the presence of marine weather observation equipment. ### 12.8.10 Remote monitoring There are remote monitoring system of AtoN using AIS network. The local offices established the integration management system based on AIS and is monitoring the status of AtoN equipments remotely. It needs to conisder which AtoN has this equipment. The optional attribute IALA_AISMonitoring (Boolean) structure feature types is used to indicate if an AtoN is remotely monitored via AIS. ### 12.8.11 AtoN inspection routines There are legal regulation for AtoN management. In terms of AtoN management, it needs to introduce the last inspection date and inspection results on the inspection. E.g. << AoN inspection period >> - Unmanned lighthouse : more than once two months - Light beacon : more than once one month - Light buoy : more than once one month << Inspection item >> - Position of AtoN - Operation status of AtoN equipment - Operation status of Power supply The optional attributes IALA_InspectionFrequency (text) and IALA_InspectionRequirements (text) are used to encode details on AtoN inspection routines. 12.8.12 Historic AtoN or Tourist attraction AtoN can be a historical relics and tourist attractions. For the purpose, the date of first installation or operation needs to consider. Indeed, AtoN section of local office manages and provides the informtion. The optional attribute IALA_installationDate (S100_TruncatedDate) on structure feature types is used to encode the date of first operation as to indicate if the AtoN is a historic relic. 12.8.13 Address of AtoN If it is required to encode an address of an AtoN the optional attribute contactAddress (complexAttributeType from NIPWG wiki) on any structure feature type is be used. This is in addition to the spatial attributes. |

## Fog signals (FogSignal)

| Definition: FOG SIGNALS. A warning signal transmitted by a vessel, or aid to navigation, during periods of low visibility. Also, the device producing such a signal. (IHO Dictionary – S-32). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Fog signal |  |  |  |  |  |
| Primitives: Point |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Category of fog signal |  | 1 : explosive 2 : diaphone 3 : siren 4 : nautophone 5 : reed 6 : tyfon 7 : bell 8 : whistle 9 : gong 10 : horn |  | EN | 1,1 |
| Signal frequency |  |  |  | IN | 0,1 |
| Signal generation |  | 1 : automatically 2 : by wave action 3 : by hand 4 : by wind |  | EN | 0,1 |
| Signal group |  |  |  | TE | 0,1 |
| Signal period |  |  |  | RE | 0,1 |
| Signal sequence |  |  |  | TE | 0,1 |
| Status |  | 1 : permanent 2 : occasional 4 : not in use 5 : periodic/intermittent 7 : temporary 8 : private 15 : synchronized |  | EN | 0,* |
| Value of maximum range |  |  |  | RE | 0,1 |
| Signal output |  |  |  | RE | 0,1 |
| Type of battery |  |  |  | TE | 0,1 |
| Remote monitoring system |  |  |  | (S) TE | 0,* |
| Remotely Monitored |  |  |  | BO | 0,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Textual description in national language |  |  |  | (S) TE | 0,1 |
| Textual description |  |  |  | (S) TE | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  |  | TE | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| ### Fog signals The term “fog signal” refers to the sound emitted, not the apparatus. Fog signals are short range aids to navigation, principally used as hazard warnings. For various reasons they are unreliable as indicators of position. Their importance relative to other aids to navigation has declined but they are still considered useful for the safe navigation of vessels with very limited (or non-functioning) electronic equipment. A fog signal should be shown on ENCs at an maximum display scale on which vessels may navigate within range. The position from which a fog signal is emitted is usually on a buoy, or close enough to a light to be treated as sounded from the same position as the light. If it is required to encode a fog signal, it must be done using the feature Fog Signal. Remarks: - The characteristic rhythm of fog signals (other than those actuated by waves, which are irregular) may be more important than their type when mariners are attempting to identify them. The number of sound emissions (e.g. blasts, strokes) and the period must therefore be encoded, where known, using the attributes signal group, signal period and signal sequence. - Where required, the attribute signal frequency must be quoted in Hertz, e.g. a signal frequency of 950 MHz must be encoded as 950000000. Distinction: Signal station, warning. |  |  |  |  |  |

## Retroreflectors (RetroReflector)

| Definition: RETROREFLECTOR. A means of distinguishing unlighted marks at night. Retroreflective material is secured to the mark in a particular pattern to reflect back light. (UKHO NP 735, 5th Edition). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Retroreflector |  |  |  |  |  |
| Primitives: Point |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Colour |  | 1 : white 2 : black 3 : red 4 : green 5 : blue 6 : yellow 7 : grey 8 : brown 9 : amber 10 : violet 11 : orange 12 : magenta 13 : pink |  | EN | 0,* |
| Colour pattern |  | 1 : horizontal stripes 2 : vertical stripes 3 : diagonal stripes 4 : squared 5 : stripes (direction unknown) 6 : border stripe |  | EN | 0,1 |
| Height |  |  |  | RE | 0,1 |
| Navigational system of marks |  | 1 : IALA A 2 : IALA B 3 : no system 4 : other system |  | EN | 0,1 |
| Status |  | 1 : permanent 4 : not in use 8 : private |  | EN | 0,* |
| Vertical accuracy |  |  |  | RE | 0,1 |
| Vertical datum |  | 3 : Mean sea level 16 : Mean high water 17 : Mean high water springs 18 : High water 19 : Approximate mean sea level 20 : High water springs 21 : Mean higher high water 24 : Local datum 25 : International great lakes datum 1985 26 : Mean water level 28 : Higher high water large tide 29 : Nearly highest high water 30 : Highest astronomical tide (HAT) |  | EN | 0,1 |
| Remote monitoring system |  |  |  | (S) TE | 0,* |
| Remotely Monitored |  |  |  | BO | 0,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Textual description in national language |  |  |  | (S) TE | 0,1 |
| Textual description |  |  |  | (S) TE | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  |  | TE | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| ### Retroreflectors Retroreflective material may be secured to unlit marks to aid their identification at night. The material is coloured according to one of two recognized IALA codes (“Standard” and “Comprehensive”). In any specified area only one of the codes will be used and this may be given in nautical publications. If it is required to encode a retroreflector, it must be done using the feature Retroreflector. Remarks: - The complex attribute information is used to describe letters, patterns or numerals shown on the retroreflector. - The body carrying the retroreflector is a separate feature. Distinction: Beacon, cardinal; beacon, isolated danger; beacon, lateral; beacon, safe water; beacon, special purpose/general; buoy, cardinal; buoy, emergency wreck marking; buoy, installation; buoy, isolated danger; buoy, lateral; buoy, safe water; buoy, special purpose/general; radar reflector. |  |  |  |  |  |

## Radar reflectors (RadarReflector)

| Definition: RADAR REFLECTOR. A device capable of, or intended for, reflecting radar signals. (IHO Dictionary – S-32). A radar reflector is usually a “tetrahedron or pentagonal corner reflector (…) to facilitate reflection towards the sender”. (International Maritime Dictionary, 2nd Ed.). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Radar reflector |  |  |  |  |  |
| Primitives: Point |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Height |  |  |  | RE | 0,1 |
| Status |  | 1 : permanent 4 : not in use 8 : private |  | EN | 0,* |
| Vertical accuracy |  |  |  | RE | 0,1 |
| Vertical datum |  | 3 : Mean sea level 16 : Mean high water 17 : Mean high water springs 18 : High water 19 : Approximate mean sea level 20 : High water springs 21 : Mean higher high water 24 : Local datum 25 : International great lakes datum 1985 26 : Mean water level 28 : Higher high water large tide 29 : Nearly highest high water 30 : Highest astronomical tide (HAT) |  | EN | 0,1 |
| Remote monitoring system |  |  |  | (S) TE | 0,* |
| Remotely Monitored |  |  |  | BO | 0,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Textual description in national language |  |  |  | (S) TE | 0,1 |
| Textual description |  |  |  | (S) TE | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  |  | TE | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| ### Radar reflectors If it is required to encode radar reflectors on curve features (e.g. overhead cables), this must be done using the feature Radar Reflector. Remarks: - If it is required to encode a feature which has no radar reflector, but is radar conspicuous, it must be indicated using attribute radar conspicuous on the feature. - If it is required to encode an area or point feature which is radar conspicuous because it is fitted with a radar reflector, it must be indicated using radar conspicuous on the feature. A Radar Reflector feature must not be encoded in this case. Distinction: Retro-reflector. |  |  |  |  |  |

## Environment observation equipment (EnvironmentObservationEquipment)

| Definition: RADAR REFLECTOR. A device capable of, or intended for, reflecting radar signals. (IHO Dictionary – S-32). A radar reflector is usually a “tetrahedron or pentagonal corner reflector (…) to facilitate reflection towards the sender”. (International Maritime Dictionary, 2nd Ed.). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Radar reflector |  |  |  |  |  |
| Primitives: Point |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Height |  |  |  | RE | 0,1 |
| Status |  | 1 : permanent 4 : not in use 8 : private |  | EN | 0,* |
| Type of environment observation equipment |  |  |  | TE | 1,* |
| Type of battery |  |  |  | TE | 0,1 |
| Remote monitoring system |  |  |  | (S) TE | 0,* |
| Remotely Monitored |  |  |  | BO | 0,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Textual description in national language |  |  |  | (S) TE | 0,1 |
| Textual description |  |  |  | (S) TE | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  |  | TE | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| ### Radar reflectors If it is required to encode radar reflectors on curve features (e.g. overhead cables), this must be done using the feature Radar Reflector. Remarks: - If it is required to encode a feature which has no radar reflector, but is radar conspicuous, it must be indicated using attribute radar conspicuous on the feature. - If it is required to encode an area or point feature which is radar conspicuous because it is fitted with a radar reflector, it must be indicated using radar conspicuous on the feature. A Radar Reflector feature must not be encoded in this case. Distinction: Retro-reflector. |  |  |  |  |  |

## Daymark (Daymark)

| Definition: DAYMARK. The identifying characteristics of an aid to navigation which serve to facilitate its recognition against a daylight viewing background. On those structures that do not by themselves present an adequate viewing area to be seen at the required distance, the aid is made more visible by affixing a daymark to the structure. A daymark so affixed has a distinctive colour and shape depending on the purpose of the aid. (IHO Dictionary – S-32, Edition 5). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Daymark |  |  |  |  |  |
| Primitives: Point |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Category of special purpose mark |  | 1 : firing danger area mark 2 : target mark 3 : marker ship mark 4 : degaussing range mark 5 : barge mark 6 : cable mark 7 : spoil ground mark 8 : outfall mark 10 : recording mark 11 : seaplane anchorage mark 12 : recreation zone mark 14 : mooring mark 15 : LANBY (Large Automatic Navigational Buoy) 16 : leading mark 17 : measured distance mark 18 : notice mark 19 : TSS mark (Traffic Separation Scheme) 20 : anchoring prohibited mark 21 : berthing prohibited mark 22 : overtaking prohibited mark 23 : two-way traffic prohibited mark 24 : reduced wake mark 25 : speed limit mark 26 : stop mark 27 : general warning mark 28 : sound ship’s siren mark 29 : restricted vertical clearance mark 30 : maximum vessel’s draught mark 31 : restricted horizontal clearance mark 32 : strong current warning mark 33 : berthing permitted mark 34 : overhead power cable mark 35 : channel edge gradient’ mark 36 : telephone mark 37 : ferry crossing mark 39 : pipeline mark 40 : anchorage mark 41 : clearing mark 42 : control mark 43 : diving mark 44 : refuge beacon 45 : foul ground mark 46 : yachting mark 47 : heliport mark 48 : GNSS mark 49 : seaplane landing mark 50 : control mark 51 : work in progress mark 52 : mark with unknown purpose 53 : wellhead mark 54 : channel separation mark 55 : marine farm mark 56 : artificial reef mark 57 : ice mark 58 : nature reserve mark |  | EN | 0,* |
| Colour |  | 1 : white 2 : black 3 : red 4 : green 5 : blue 6 : yellow 7 : grey 8 : brown 9 : amber 10 : violet 11 : orange 12 : magenta 13 : pink |  | EN | 1,* |
| Colour pattern |  | 1 : horizontal stripes 2 : vertical stripes 3 : diagonal stripes 4 : squared 5 : stripes (direction unknown) 6 : border stripe |  | EN | 0,1 |
| Elevation |  |  |  | RE | 0,1 |
| Height |  |  |  | RE | 0,1 |
| Nature of construction |  | 1 : masonry 2 : concreted 4 : hard surfaced 6 : wooden 7 : metal 8 : glass reinforced plastic (GRP) 9 : painted 10 : latticed |  | EN | 0,* |
| Object name in national language |  |  |  | (S) TE | 0,1 |
| Object name |  |  |  | (S) TE | 0,1 |
| Status |  | 1 : permanent 4 : not in use 5 : periodic/intermittent 7 : temporary 8 : private 12: illuminated |  | EN | 0,* |
| Topmark/daymark shape |  | 1 : cone, point up 2 : cone, point down 3 : sphere 4 : 2 spheres 5 : cylinder (can) 6 : board 7 : x-shape (St. Andrew’s cross) 8 : upright cross (St George’s cross) 9 : cube, point up 10 : 2 cones, point to point 11 : 2 cones, base to base 12 : rhombus (diamond) 13 : 2 cones (points upward) 14 : 2 cones (points downward) 15 : besom, point up (broom or perch) 16 : besom, point down (broom or perch) 17 : flag 18 : sphere over rhombus 19 : square 20 : rectangle, horizontal 21 : rectangle, vertical 22 : trapezium, up 23 : trapezium, down 24 : triangle, point up 25 : triangle, point down 26 : circle 27 : two upright crosses (one over the other) 28 : T-shape 29 : triangle pointing up over a circle 30 : upright cross over a circle 31 : rhombus over a circle 32 : circle over a triangle pointing up 33 : other shape (see information) |  | EN | 1,1 |
| Vertical accuracy |  |  |  | RE | 0,1 |
| Vertical datum |  | 3 : Mean sea level 16 : Mean high water 17 : Mean high water springs 18 : High water 19 : Approximate mean sea level 20 : High water springs 21 : Mean higher high water 24 : Local datum 25 : International great lakes datum 1985 26 : Mean water level 28 : Higher high water large tide 29 : Nearly highest high water 30 : Highest astronomical tide (HAT) |  | EN | 0,1 |
| Vertical length |  |  |  | RE | 0,1 |
| Remote monitoring system |  |  |  | (S) TE | 0,* |
| Remotely Monitored |  |  |  | BO | 0,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Textual description in national language |  |  |  | (S) TE | 0,1 |
| Textual description |  |  |  | (S) TE | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  |  | TE | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| ### Daymarks If it is required to encode a daymark, it must be done using the feature Daymark. The term “daymark” may also simply refer to any unlighted aid to navigation, and encoders may choose to encode Daymark instead of Beacon Special Purpose/General, particularly for leading marks (see clause X.X). In North America, the term “daybeacon” is used for an unlit beacon. In the following table, a blank indicates that the encoder may choose a relevant value for the attribute. The table contains the most common examples of coding; other coding combinations are possible. Feature; INT1; Feature; category of special purpose mark; Other attributes. Coloured or white mark; Q101; Daymark; nature of construction = 9. Coloured topmark with function of beacon; Q102.1; Daymark; nature of construction = 9. Painted board with function of leading beacon; Q102.2; Daymark; 16; nature of construction = 9, topmark shape = 6. Remarks: - The attribute colour pattern must be populated if more than one value is populated for the mandatory attribute colour. - If it is required to encode the altitude of the ground level above the vertical datum at the position of a daymark, it must be done using the attribute elevation, but only for beacons built on land. - If it is required to encode the total altitude of a daymark, including any equipment features (e.g. light), above the vertical datum, it must be done using the attribute height. - If it is required to encode the total vertical length of a daymark, including any equipment features (e.g. light), above the seabed or ground, it must be done using the attribute vertical length. - If it is required to encode a cairn that bears the colour(s) specified by a navigational system of marks, it must be done using a beacon feature. - If it is required to encode a daymark that has more than one colour, the attributes colour and colour pattern must be encoded, according to the rules laid out in clause X.X. Distinction: Beacon, cardinal; beacon, isolated danger; beacon, lateral; beacon, safe water; beacon, special purpose/general; topmark. |

## Radar transponder beacon (RadarTransponderBeacon)

| Definition: RADAR TRANSPONDER BEACON. A transponder beacon transmitting a coded signal on radar frequency, permitting an interrogating craft to determine the bearing and range of the transponder. Also called racon. (IHO Dictionary – S-32). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Radar transponder beacon |  |  |  |  |  |
| Primitives: Point |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Category of radar transponder beacon |  | 1 : ramark, radar beacon transmitting continuously 2 : racon, radar transponder beacon 3 : leading racon/radar transponder beacon |  | EN | 1,1 |
| Object name in national language |  |  |  | (S) TE | 0,1 |
| Object name |  |  |  | (S) TE | 0,1 |
| Radar wave length |  |  |  | (S) TE | 0,1 |
| Sector limit one |  |  |  | IN | 0,1 |
| Sector limit two |  |  |  | IN | 0,1 |
| Signal group |  |  |  | TE | 0,1 |
| Signal sequence |  |  |  | TE | 0,1 |
| Status |  | 1 : permanent 2 : occasional 4 : not in use 5 : periodic/intermittent 7 : temporary 8 : private |  | EN | 0,* |
| Value of nominal range |  |  |  | RE | 0,1 |
| Type of battery |  |  |  | TE | 0,1 |
| Remote monitoring system |  |  |  | (S) TE | 0,* |
| Remotely Monitored |  |  |  | BO | 0,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Textual description in national language |  |  |  | (S) TE | 0,1 |
| Textual description |  |  |  | (S) TE | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  |  | TE | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| ### Radar beacons Radar beacons are transmitters operating in the marine radar frequency band. The signals produce a characteristic line on a vessel’s radar display enabling the mariner to determine their position with greater certainty than would be possible by means of a normal radar display alone. If it is required to encode a radar beacon, it must be done using the feature Radar Transponder Beacon. Remarks: - The Radar Transponder Beacon must only be used to encode the technical equipment itself, independent of the building or structure in which it is installed. If it is required to encode the building or structure (e.g. mast, tower, radar dome), it must be done using an appropriate feature (e.g. Building, Landmark). - The attribute signal group is used to encode Morse identification letter(s) for the radar beacon, where known. - Leading racons are established such that, when their bearing lines are coincident on a vessel’s radar display, the bearing serves to indicate the track to be followed. If it is required to encode the bearing line and the recommended track for leading racons, it must be done as described in clause X.X. Where the bearing line coincides with a leading line defined by lights or other visual features making up a range system, navigation lines and recommended tracks must not be duplicated. The features making up the range system must be aggregated using the collection feature Aggregation (see clause X.X). - If, for some reason, the radar transponder beacon signal is obscured between certain bearings, this information should be encoded using the complex attribute sector limit to encode the “visible” sector, as for lights (see clause X.X). - The sweep period may be encoded using the complex attribute information. Distinction: Radar line; radar range; radar station. |  |  |  |  |  |

## Power source (PowerSource)

| Definition: POWER SOURCE A |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Power source |  |  |  |  |  |
| Primitives: Point |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Category of power source |  | 1: battery 2: generator 3: solar panel 4: electrical service |  | EN | 1,1 |
| Manufacturer |  |  |  | (S) TE | 0,1 |
| Status |  | 1 : permanent 4 : not in use 5 : periodic/intermittent 7 : temporary 8 : private 12: illuminated |  | EN | 0,* |
| Remote monitoring system |  |  |  | (S) TE | 0,* |
| Remotely Monitored |  |  |  | BO | 0,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Textual description in national language |  |  |  | (S) TE | 0,1 |
| Textual description |  |  |  | (S) TE | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  |  | TE | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| TBD |  |  |  |  |  |

# Moorings

## Bridle (Bridle)

| Definition: BRIDLE. A Two lengths of chain connected by a central ring and used for lifting wide loads. (IALA Dictionary,8-3-195) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Bridle |  |  |  |  |  |
| Primitives: NoGeometry |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Aton number |  |  |  | URI | 1,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Seasonal action required |  |  |  | IN | 0,* |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Bridle link type |  |  |  | (S) TE | 0,1 |
| Legs details |  |  |  | (S) TE | 0,1 |
| TBD |  |  |  |  |  |

## Cable submarine (CableSubmarine)

| Definition: CABLE SUBMARINE. A |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Cable Submarine |  |  |  |  |  |
| Primitives: NoGeometry |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Aton number |  |  |  | URI | 1,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Seasonal action required |  |  |  | IN | 0,* |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Cable dimensions |  |  |  | EN | 0,1 |
| Category of cable |  | 7: mooring cable |  | EN | 0,1 |
| Status |  | 1 : permanent 4 : not in use 5 : periodic/intermittent 7 : temporary 8 : private 12: illuminated |  | EN | 0,* |

## Mooring Shackle (MooringShackle)

| Definition: MOORING SHACKLE. A shackle at the lower end of a mooring chain, for attachment to an anchor or sinker. (IALA Dictionary, 8-5-150) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Mooring shackle |  |  |  |  |  |
| Primitives: NoGeometry |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Aton number |  |  |  | URI | 1,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Seasonal action required |  |  |  | IN | 0,* |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Nature of construction |  | 1 : masonry 2 : concreted 6 : wooden 7 : metal 8 : glass reinforced plastic (GRP) 9 : painted 10 : latticed |  | EN | 0,* |
| Shackle type |  | 1 : forelock shackles 2: clenching shackle 3: bolt shackle 4: screw pin shackle 5: kenter shackle 6: quick release link |  | EN | 0,1 |
| Weight |  |  |  | RE | 0,1 |

## Sinker Anchor (SinkerAnchor)

| Definition: SINKER ANCHOR A heavy weight (of concrete, cast-iron, etc..) that rests on the sea bed and to which a mooring line can be attached. (IALA Dictionary, 8-5-025) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Sinker Anchor |  |  |  |  |  |
| Primitives: NoGeometry |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Aton number |  |  |  | URI | 1,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Seasonal action required |  |  |  | IN | 0,* |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Nature of construction |  | 1 : masonry 2 : concreted 6 : wooden 7 : metal 8 : glass reinforced plastic (GRP) 9 : painted 10 : latticed |  | EN | 0,* |
| Sinker dimensions |  |  |  | EN | 0,1 |
| Sinker type |  |  |  | (S) TE | 0,1 |
| Weight |  |  |  | RE | 0,1 |

## Swivel (Swivel)

| Definition: SWIVEL. A chain link that provides for rotary motion between the lengths of chain that it connects. (IALA Dictionary, 8-5-165) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Swivel |  |  |  |  |  |
| Primitives: NoGeometry |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Aton number |  |  |  | URI | 1,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Seasonal action required |  |  |  | IN | 0,* |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Nature of construction |  | 1 : masonry 2 : concreted 6 : wooden 7 : metal 8 : glass reinforced plastic (GRP) 9 : painted 10 : latticed |  | EN | 0,* |
| Swivel type |  |  |  | (S) TE | 0,1 |
| Weight |  |  |  | RE | 0,1 |

# Recommended Track and Line

## Recommended track (RecommendedTrack)

| Definition: RECOMMENDED TRACK. A track recommended to all or only certain vessels. (IHO Dictionary – S-32). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Recommended track |  |  |  |  |  |
| Primitives: Curve, Surface |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Category of recommended track |  | 1 : based on a system of fixed marks 2 : not based on a system of fixed marks |  | EN | 1,1 |
| Depth range minimum value |  |  |  | RE | 0,1 |
| Depth range maximum value |  |  |  | RE | 0,1 |
| Object name in national language |  |  |  | (S) TE | 0,1 |
| Object name |  |  |  | (S) TE | 0,1 |
| Orientation |  |  |  | RE | 1,1 |
| Quality of sounding measurement |  | 1 : depth known 2 : depth or least depth unknown 6 : least depth known |  | EN | 0,* |
| Sounding Accuracy |  |  |  | RE | 0,1 |
| Status |  | 1 : permanent 2 : occasional 5 : periodic/intermittent 7 : temporary 9 : mandatory 12 : illuminated 16 : watched 17 : un-watched |  | EN | 0,* |
| Technique of sounding measurement |  | 1 : found by echo-sounder 2 : found by side scan sonar 3 : found by multi-beam 6 : swept by wire-drag 7 : found by laser 8 : swept by vertical acoustic system 9 : found by electromagnetic sensor 13 : swept by side-scan sonar |  | EN | 0,* |
| Traffic flow |  | 1 : inbound 2 : outbound 3 : one-way 4 : two-way |  | EN | 0,1 |
| Vertical datum |  | 3 : Mean sea level 16 : Mean high water 17 : Mean high water springs 18 : High water 19 : Approximate mean sea level 20 : High water springs 21 : Mean higher high water 24 : Local datum 25 : International great lakes datum 1985 26 : Mean water level 28 : Higher high water large tide 29 : Nearly highest high water 30 : Highest astronomical tide (HAT) |  | EN | 0,1 |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Textual description in national language |  |  |  | (S) TE | 0,1 |
| Textual description |  |  |  | (S) TE | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  |  | TE | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| ### Recommended tracks Recommended tracks and fairways usually comprise a number of sections (sometimes termed “legs”) which lead between dangers lying close on both sides of the track or fairway. Tracks commonly include some sections which are leading lines (see clause X.X). The distinction between tracks and fairways, in this context, is that tracks have no specified outer limits and fairways do have specified outer limits. It is important to recognise that it is not the role of cartographers to create “recommended” tracks and other “recommended” routeing measures; such recommendations are made by other authorities. The word “Recommended”, used in connection with recommended tracks and other recommended routeing measures usually implies that it has been recommended by a competent authority (such as a port authority within its port limits or a maritime safety authority) and may be adopted by IMO. Occasionally, the recommendation may be based on advice directly from a competent surveyor or established by precedent. Recommended tracks include all channels recommended for hydrographic reasons to lead safely between shoal depths. The use of such tracks is generally left to the discretion of the mariner and will depend on the vessel’s draught, the state of the tide, adequacy of navigational aids and so on. If it is required to encode a recommended track, it must be done using the feature Recommended Track. The use of Navigation Line and Recommended Track is defined in more detail in the following Table, and in the Figure below. Figure; Navigation Line; Recommended Track; Navigational Aids. 1; Recommended track on a leading line; category of navigation line = 3; category of recommended track = 1; at least 2. 2; Clearing line on marks in line; category of navigation line = 1; none; at least 2. 3; Transit line on marks in line; category of navigation line = 2; none; at least 2. 4; Recommended track on a bearing; category of navigation line = 3; category of recommended track = 1; 1. 5; Clearing line on a bearing; category of navigation line = 1; none; 1. 6; Transit line on a bearing; category of navigation line = 2; none; 1. 7; Recommended track not based on fixed marks; none; category of recommended track = 2; none. Two navigational aids Two navigational aids One navigational aid One navigational aid No navigational aids 1 2 & 3 5 & 6 4 7 Recommended Track Recommended Track Navigation Line Navigation Line Navigation Line Recommended Track Navigational System of Marks (surface) (b) orientation (mandatory) Remarks: - The attribute depth range minimum value is used to encode the shallowest depth along the track, where required. - The attribute maximum permitted draught is used to encode the maximum draught permitted on the track, where required. - The recommended track is that portion of a navigation line (see clause X.X) that a ship should use for navigation (see Figure above). - In the case of a two-way recommended track, only one value of orientation is encoded (in the complex attribute orientation); the other value can be deduced (i.e. the value in orientation (orientation value) + 180 degrees). The value of orientation encoded on orientation (orientation value) should be the value of the bearing from seaward. If it is not possible to define a seaward direction, the value that is less than 180° should be used. - When the traffic flow along a recommended track is one way (attribute traffic = 1, 2 or 3), the resultant direction of the line (accounting for the direction of digitising and any subsequent reversal of the line) associated with the Recommended Track must be the same as the direction of the traffic flow, in order to ensure the correct representation in the ECDIS of the direction to be followed. Distinction: Fairway; navigation line; recommended route centreline; recommended traffic lane part. |

## Navigation Line (NavigationLine)

| Definition: NAVIGATION LINE. A straight line extending towards an area of navigational interest and generally generated by two navigational aids or one navigational aid and a bearing. (Service Hydrographique et Oceanograhique de la Marine, France). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-201 Geo Feature: Navigation line |  |  |  |  |  |
| Primitives: Curve |  |  |  |  |  |
| Real World | Paper Chart Symbol |  | ECDIS Symbol |  |  |
| S-201 Attribute |  | Allowable Encoding Value |  | Type | Multiplicity |
| Category of navigation line |  | 1 : clearing line 2 : transit line 3 : leading line bearing a recommended track |  | EN | 1,1 |
| Orientation |  |  |  | RE | 1,1 |
| Status |  | 1 : permanent 2 : occasional 5 : periodic/intermittent 7 : temporary 8 : private 14 : public |  | EN | 0,* |
| ID Code |  |  |  | (S) TE | 1,1 |
| Date end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Date start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period end |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Period start |  | ISO 8601:1988 |  | (S) DA | 0,1 |
| Information |  |  |  | (S) TE | 0,* |
| Information in national language |  |  |  | (S) TE | 0,* |
| Textual description in national language |  |  |  | (S) TE | 0,1 |
| Textual description |  |  |  | (S) TE | 0,1 |
| Scale minimum |  | See clause X.X |  | IN | 0,1 |
| Source date |  |  |  | (S) TE | 0,1 |
| Source indication |  |  |  | (S) TE | 0,1 |
| Pictorial representation |  |  |  | TE | 0,1 |
| Inspection frequency |  |  |  | (S) TE | 0,1 |
| Inspection requirement |  |  |  | (S) TE | 0,1 |
| Aton maintenance record |  |  |  | URI | 0,1 |
| Installation date |  |  |  | (S) DA | 0,1 |
| ### Navigation lines Clearing Lines are important in rocky areas where dangers are not guarded by buoys and where sailing vessels (which are not always able to keep to a direct track) and other small craft may navigate close inshore. Transits marking isolated dangers are based on beacons or other marks which are erected on shore to indicate (approximately, unless there are two pairs of beacons) the position of an isolated danger. Leading lines based on beacons or lights must be encoded where the maximum display scale for the ENC data permits. Leading lines based on natural features should be encoded on the largest maximum display scale ENC data where they appear to be useful, particularly if other navigational aids seem inadequate. If it is required to encode a navigation line, it must be done using the feature Navigation Line. The use of Navigation Line and Recommended Track (see clause X.X) is defined in more detail in the following Table, and in the Figure below: Figure; Navigation Line; Recommended Track; Navigational Aids. 1; Recommended track on a leading line; category of navigation line = 3; category of recommended track = 1; at least 2. 2; Clearing line on marks in line; category of navigation line = 1; none; at least 2. 3; Transit line on marks in line; category of navigation line = 2; none; at least 2. 4; Recommended track on a bearing; category of navigation line = 3; category of recommended track = 1; 1. 5; Clearing line on a bearing; category of navigation line = 1; none; 1. 6; Transit line on a bearing; category of navigation line = 2; none; 1. 7; Recommended track not based on fixed marks; none; category of recommended track = 2; none. Two navigational Area (b) overlaps area (a) aids Two navigational aids One navigational aid One navigational aid No navigational aids 1 2 & 3 5 & 6 4 7 Recommended Track Recommended Track Navigation Line Navigation Line Navigation Line Recommended Track Navigation Line Remarks: - The value populated for the mandatory complex attribute orientation must be the value of the bearing from seaward. - The extent of the navigation line depends on the visibility of the navigational aids. - The recommended track is that portion of a navigation line that a ship should use for navigation. ### Measured distances (see S-4 – B-458) If the track to be followed is on a leading line or a bearing, it must be encoded in the way described in the Table and Figure above (cases 1 or 4). If the track is not on a leading line or bearing, it must be encoded only as a Navigation Line feature with the attribute category of navigation line being set to an empty (null) value. In either case, if it is required to encode the measured distance, it must be done using the attribute information (e.g. Measured distance = 1450 metres). If it is required to encode the transit lines, they must be done using Navigation Line features, with category of navigation line = 2 (transit line). If it is required to encode the beacons, they must be done using Beacon Special Purpose/General features, with attribute category of special purpose mark = 17 (measured distance mark). On occasions, one or more of the transits used for the measured distance may incorporate an existing landmark as the front or rear mark. In this case, if Landmark is encoded, category of special purpose mark = 17 must also be populated. Where the entire measured distance system exists within a single dataset, each transit line with its beacons must be aggregated into a collection feature Aggregation (see clause X.X). These two aggregation features and the track to be followed must be aggregated into another Aggregation feature. Remarks: - All features comprising a measured distance must have the same value populated for the attribute SCAMIN (see clause X.X). Distinction: Recommended route; recommended track. |

# Collection Objects

## Association

| Definition: ASSOCIATION. Used to identify an association between two or more objects. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-201 Collection Feature: Association |  |  |  |  |  |  |  |  |
| Primitives: None |  |  |  |  |  |  |  |  |
| Real World |  | Paper Chart Symbol |  |  |  | ECDIS Symbol |  |  |
| S-201 Attribute |  |  |  | Allowable Encoding Value |  |  | Type | Multiplicity |
| Category of Association |  |  |  | 1 : channel markings 2 : danger markings |  |  | CL (OE) | 1,1 |
| Feature associations |  |  |  |  |  |  |  |  |
| Role Type | Association Name |  | Role |  | Features |  |  | Multiplicity |
| Association | Associations |  | peer |  | Any Aids to Navigation Geographic feature |  |  | 1,* |
| Channel markings For channel markings, see 8.2.2. Danger marks The association object is used to show the relationship between the navigation lines and tracks and the associated dangers. Other associations If it is necessary to code Association objects other than channel markings or danger markings, they must be encoded as “other: …” where the ellipses are replaced by a short label conforming to S-100 clause 3-6.7 indicating the type of association. Remarks: No remarks Distinction: Aggregation. |  |  |  |  |  |  |  |  |

## Aggregation

| Definition: AGGREGATION. Used to identify an aggregation of two or more objects. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-201 Collection Feature: Aggregation |  |  |  |  |  |  |  |  |
| Primitives: None |  |  |  |  |  |  |  |  |
| Real World |  | Paper Chart Symbol |  |  |  | ECDIS Symbol |  |  |
| S-201 Attribute |  |  |  | Allowable Encoding Value |  |  | Type | Multiplicity |
| Category of Aggregation |  |  |  | 1 : leading line 2 : range system 3: measured distance |  |  | CL (OE) | 1,1 |
| Feature associations |  |  |  |  |  |  |  |  |
| Role Type | Association Name |  | Role |  | Features |  |  | Multiplicity |
| Association | Aggregations |  | peer |  | Any Aids to Navigation Geographic feature |  |  | 1,* |
| Range system To encode a range system the features Navigation Line, Recommended Track and the lights or other navigational aids making up the range system must be aggregated using the collection feature Aggregation. (See also clause 8.2.1) Leading line (clearing line or transit line) A leading line is distinct from a range system in that it does not have a section of the navigation line designated a recommended track. (See also clause 8.2.1) Measured distance To encode a measured distance the features Navigation Line and fixed marks (BeaconSpecialPurposeGeneral or Landmark) must be aggregated using the collection feature Aggregation. (See also clause 8.2.2) Other aggregations If it is necessary to code other types of Aggregation objects, they must be encoded as “other: …” where the ellipses are replaced by a short label conforming to S-100 clause 3-6.7 indicating the type of association. Remarks: No remarks Distinction: Association. |  |  |  |  |  |  |  |  |