Input paper: ARM10-3.10

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

X ARM	□ENG		□PAP					□  Input

□ENAV	□VTS							X  Information

Agenda item			3

Technical Domain / Task Number2

Working Group

Author(s) / Submitter(s)		               China MSA

Providing standardized real-time hydrological and meteorological

Information in port waters

# Summary

China Aton management authorities attach great importance to the needs of navigation users,. Every year, we collect user’s opinions and suggestions through seminars, questionnaires, visits and other ways. In recent years, some voyagers have paid close attention to hydrometeorological information such as wind, waves, currents and tides in port waters. To this end, China MSA have carried out research and development of relevant service systems.

This document describes one of the service systems, Harbor Environment Monitoring System, which has been deployed in several ports in the North China Sea. It is used for real-time monitoring of port hydrological and meteorological elements related to ship navigation safety (such as water flow velocity, flow direction, wave height, wave energy, tide level, wind speed, wind direction, visibility, water temperature, water level, air temperature and humidity, etc.). At the same time, these environmental elements are aggregated, analyzed, and mined, and provided to maritime parties through various means.

## Purpose of the document

This paper is intended to provide the ARM committee an overview of an information system of providing real-time hydrometeorological services in port waters.

## Related documents

None

# Background

With the continuous expansion of the scale of international trade, the density of ships in the waters of major ports increases rapidly. As a result, higher requirements have been put forward for the navigation aid service of coastal countries, the traditional navigation aids system gradually evolves to a comprehensive and integrated system. The introduction and development of e-navigation has accelerated the transformation process. E-Navigation is the harmonised collection, integration, exchange, presentation and analysis of maritime information onboard and ashore by electronic means to enhance berth to berth navigation and related services, for safety and security at sea and protection of the marine environment. The system architecture mainly includes ship-based system, shore-based system and navigation and communication infrastructure.

During the construction of e-navigation System, IALA pointed out in GUIDELINES “Design and Implementation Principles for harmonized System Architecturesof shore-based Infrastructure 1113” that the implementation of all systems should be based on meeting user requirements. This coincides with the concept of the China competent authority. Considering the needs of navigation users for hydrological and meteorological information,, the Qinhuangdao AtoN Division of China Maritime Safety Administration has developed a harbour environmental monitoring system as part of the E-Navigation related service system.

At present, the system has been applied in Qinhuangdao Port, Tangshan Port, Jingtang Port Area, Tianjin Port and other ports. It has achieved remarkable economic and social benefits in ship steering, port production, channel design and environmental monitoring, and has been widely praised by users.

# Discussion

The monitoring and issuance of hydrological and meteorological information involves a variety of disciplines and technologies, such as Sensor technology, materials science, communication technology, network technology and so on. These technologies have made great progress worldwide in recent years. First, sensors are developing in the direction of small size, multi-function, high accuracy and maintenance-free. Firstly, the sensor develops towards the direction of small size, multiple functions, high accuracy and maintenance-free. Secondly, the new material is widely used in the collection platform. Thirdly, the development of communication technology, especially satellite communication technology has greatly improved the stability of data transmission. Fourthly, the means of distribution is more diversified and more conducive to user acceptance. In short, the development of technology has made it possible to monitor and release hydrological and meteorological information in real time.

## Current situation of hydrological and meteorological information services in China

It is emphasized in the Chapter V of the SOLAS Convention that the State party undertakes to provide meteorological and hydrological information services to ships. In China, this responsibility is shared by the China Meteorological Administration(CMA), the State Oceanic Administration and the China Maritime Administration(SOA).

The CMA is mainly responsible for marine meteorological forecasting, and the SOA is responsible for marine hydrological forecasting. Generally speaking, the meteorological department is responsible for the above water level and the ocean bureau is responsible for the water level and below. In 2007, the Typhoon and Marine Meteorological Forecasting Center of China Meteorological Administration was established. As a special organization, it is engaged in marine meteorological forecasting. Now, it has initially established four levels of national, regional, provincial and municipal meteorological stations, including nearly 1000 observatories, meteorological stations, radar stations and meteorological satellites. In 1965, the National Marine Environment Forecasting Center was established. The forecasting area covers the China Sea and the world's major ocean areas. The main forecasting operational items include waves, storm surges, tsunamis, currents, tides, sea level and ocean weather. As the competent authority, the Navigation Guarantee Centre of China Maritime Safty Administration is mainly responsible for hydrological survey and provides navigational publications such as charts, tide tables and navigation guides to meet the needs of navigation safety.

## Introduction of Harbor Environment Monitoring System

Harbor Environment Monitoring System is a monitoring service network built in port, which is used to monitor the factors related to ship navigation safety (such as flow speed, flow direction, tide level, wave, wind speed, wind direction, visibility, etc.) and distribute to navigation users to provide safety guarantee for ships entering and leaving ports and berthing.

3.2.1               System Structure Framework

The system consists of data acquisition module, data communication module, data processing module and data release module. As shown in the following figure:

Figure 1  System Structure Diagram

Fig. 1: System Structure Diagram

3.2.2               Data Acquisition Module

The data acquisition module is composed of hydrological equipment, meteorological equipment, buoy, power supply system and control unit. Its physical diagram is as follows:

Hydrological information monitoring equipment: Acoustic wave and current profiler and Aquadopp current profiler

Meteorological information monitoring equipment: HMP45C temperature and humidity Probe, 05106 wind Wind Monitors, SVS1 Visibility Sensor.

Buoy: buoy is the carrier of collecting equipment. It is made of polyurea material. The floating body is filled with elastic closed cell foam, and metal material is used as reinforcing skeleton. It has strong impact resistance, good corrosion resistance, high reserve buoyancy, and the stability of the buoy meets the 4 person superscript maintenance operation. It can well adapt to the northern winter wave and ice condition, and the body presets the instrument mounting hole and the fixed support which is used to install hydrological and meteorological information acquisition equipment.

Power supply system: composed by solar photovoltaic battery pack, battery controller and storage battery. The system can ensure the normal operation of all kinds of equipment in 7 consecutive days of overcast and rainy weather.

Control Unit: The CR1000 produced by CAMPBELL Company of the United States is selected to pre-process the collected data, convert it into digital signal, and transmit it to the data transmission system. The device has the characteristics of small size, good sealing, programmable and storage. On the one hand, it can support sensor expansion, on the other hand, it can temporarily store and process data in case of communication failure to protect data continuity.

Figure 2  Entity map of acquisition platform

3.2.3             Data Transfer Module

There are two ways of data communication:

- Mobile wireless network. Generally speaking, it is the mobile phone communication network. The advantages are low cost, large capacity, high reliability. Obvious shortcomings: It cannot communicate in areas without signal coverage.

- Satellite communication, using Beidou short message communication, the advantage is that there is no coverage blind zone, the disadvantage is high cost and low communication capacity.

- Considering the advantages and disadvantages of the above methods, the system developed a function module with both GPRS and Beidou short message communication. GPRS mode was used in the mobile phone signal coverage area, and the non-signal area was automatically switched to Beidou short message communication. At the same time, the advantages and disadvantages of both methods were taken into account, which effectively guaranteed the stability of data transmission.

Figure 3   Dual-mode communication unit

3.2.4             Data Processing Module

System data processing center is the core of the whole system. It mainly includes hardware environment, network environment, wireless transmission channel, data receiving service, data storage service, data analysis service, system application service and so on. These service are integrated organically by platform software. The software platform adopts the mode of multi-channel concurrency, receives data from various measuring devices through virtual serial port, decodes and preprocesses them, and stores them in the database and presents them to users.

Figure 4  Data Processing Platform Software

3.2.5             Data Publishing Module

The ultimate goal of monitoring and processing Hydrometeorological Information is to release it to users. The system uses visual display technology to display data to managers and users. At the same time, according to the different needs, different means of information publishing are formulated, mainly including: intranet client access, Internet website publishing, shore-based LED display, mobile client and so on.

All of the above methods are published on the basis of network or wireless network. In the area where the mobile phone signal cannot be covered, the ship users cannot receive it. At present, research is being carried out to transmit data by AIS message.

Figure 5  Visual user interface

## Consideration of providing standardized real-time Hydrometeorological Information in port waters

Several ports in China have developed and implemented real-time Hydrometeorological Information Acquisition and dissemination systems, which can be seen that the technical bottleneck restricting such systems has been solved. However, we believe that there are still some problems in popularizing and applying such systems nationwide and worldwide.

- The legality of hydrological and meteorological information provided by the navigation marks administration. In terms of responsibilities, it should be broadcast by the meteorological department, which is the primary problem that should plague the promotion of this type of system in the field of AtoN management.

- Data accuracy. Although multiple sets of front-end acquisition equipment are put in the same place to check each other, the accuracy is not verified by other means. Therefore, when using, we provide free to the users, and the statement in advance is only for reference.

- The investment of system construction, maintenance and maintenance is large, especially in the open environment, the physical safety of equipment is difficult to guarantee.

- There are many kinds of equipment, different data formats, various means of publishing, and lack of uniform technical standards.

# References

None

# Action requested of the CommitteE

- The Committee is requested to study and develop recommendations or guidelines on the dissemination of hydrological and meteorological information by the AtoN competent authority.

- The Commission is requested to explore the feasibility of cooperative research with the World Meteorological Organization in relevant areas.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain