Input paper:   ARM10-3.18

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

X  ARM	□  ENG		□  PAP					□ Input

□  ENAV	□  VTS							X  Information

Agenda item 			3

Technical Domain / Task Number 2 	Working Group 2 / Task 5.1.11

Author(s) / Submitter(s)			Jin Hyoung PARK (KRISO), Chi-Kyoung LEE (MOF/ROK)

# Introduction to SMART-Navigation as a Maritime Service Portfolio 
in Korean Waters: its status and future plans

# Summary

## Purpose of the document

This document is intended to provide ARM committee members with information on the progress and future plans of SMART-Navigation project which is aiming to deploy maritime services in the context of e-Navigation services in Korean waters from 2021.

## Related documents

- MSC.467(101) on Guidance on the definition and harmonization of the format and structure of Maritime Services in the context of e-navigation

- MSC.1/Circ.1610 INITIAL DESCRIPTIONS OF MARITIME SERVICES IN THE CONTEXT OF E-NAVIGATION

- HGDM 1-5-5 - Comments on documents HGDM 151 and HGDM 152Application example of the proposed MSPs guideline structures by SMART-Navigation project

- IALA G1128 THE SPECIFICATION OF E-NAVIGATION TECHNICAL SERVICES

- ENAV22-3.1.5 Conceptual Description on the Maritime Messaging Service (MMS)

- ENAV23-11.1 Work Programme for Committees 2018-2022

# Background

IMO and IHO organized a joint working group on e-Navigation, the IMO-IHO Harmonization Group on Data Modeling (HGDM), to provide member states with guidance on e-Navigation service description and data modelling. Reflecting the output activities, IMO MSC101 approved a resolution to guide how to define and harmonize the format and structure of Maritime Services (MS) in the context of e-Navigation. According to the resolution, MS is to be described with combinations of Operational Services (OS) and Technical Services (TS).

In parallel with such progress in IMO, IALA council decided to move its task for development of guidance on Maritime Service from ENAV committee to ARM committee. As guidance on technical service specification is to be still in ENAV committee’s task list, the decision by the council could be interpreted as separation between OS description and TS description. This means information sharing on OS and TS needs to be done in more official ways although TS and OS are closely linked to each other as we could see in the MSC resolution.

This situation requires more active contribution from the members who have focused on ENAV committee activities. The authors of this document, who submitted various information documents on SMART-Navigation project to ENAV committee during the last work period, 2013-2017, would like to contribute on further development of the initial description on Maritime Service in the context of e-Navigation by submitting the information document, which contains the latest update on the project, to the committee.

# Discussion

## SMART-Navigation Project

The SMART-Navigation project is organized and funded by the Ministry of Oceans and Fisheries of the Republic of Korea to implement e-navigation services in its waters based on IMO's e-Navigation concept adding special features: (1) services for non-SOLAS ships and (2) broadband communication using navigation-dedicated LTE networks (LTE-Maritime) for better connectivity between ship and shore.

Concept of SMART-Navigation Service

The project is aiming to implement parts of MSs which are included in the MSC circular on Initial descriptions of maritime services in the context of e-navigation: MS1 to MS7, MS 10, MS11, MS12, MS14 and MS15. The project is implementing six maritime services in the context of e-Navigation: (1) Navigation Monitoring & Assistance Service (NAMAS), (2) Ship-borne System Monitoring Service (SBSMS), (3) Safe & Optimal Route Planning Service (SORPS), (4) Real-time Electronic Navigational Chart Distribution & Streaming Service (REDSS), (5) Pilot & Tug Assistance Service (PITAS) and (6) Maritime Environment and Safety Information Service (MESIS). Most of the services are designed for ships that pose a higher risk of having an accident or with a potential of having an enormous social, environmental or economic impact after an accident.

NAMAS monitors navigation of vulnerable ships and raises an alarm as part of the navigation assistance to prevent collisions and groundings. The service provides ships with safety information when requested or necessary. It uses positional information of ships and their route information when available. LTE-Maritime is used for the physical communication links for NAMAS.

SBSMS monitors on-board systems of passenger ships flying the Korean flag and other ships requesting the service to detect hazardous events within the ships, such as flooding, fire and engine failure. The service provides ships with guidance and procedures which will have been prepared for such events in terms of safety management procedure of the ship. LTE-Maritime and VDES are being considered as physical communication links for SBSMS but are not limited to these. Other communication links available on board for requesting this service can be used as well.

SORPS provides safe and optimal routes plan when requested. It can be used for voyage planning of merchant ships or for emergency route guidance for small vessels which are not equipped with navigation-aid systems such as RADAR and AIS. LTE-Maritime and VDES are being considered as physical communication links for SORPS but are not limited to these. Other communication links available on board for requesting this service can be used as well.

REDSS provides S-101 ENC of Korean waters for SOLAS and non-SOLAS ships when requested. It supports streaming as well for small ships which do not have on- board electronic chart system (ECS). Only LTE-Maritime is being considered as physical communication links for REDSS.

PITAS supports pilotage by providing pilots and tugs with information needed for pilotage. Pilotage request and ETA of ships are shared with pilots before ship’s arrival. Traffic density, tide height and surface current on the pilotage route are provided to pilots. VDES, LTE-Maritime are being considered as physical communication links for PITAS but other communication links can be used when requested by users (ships, pilot, etc.). PITAS could be used for a linkage between e-Navigation service and port operation optimization as ETA and RTA are used for optimizing navigation and port operation. Busan Port Authority (BPA) is evaluating how much PITAS could contribute on optimizing port operation from port side and optimizing port call from shipping company side.

MESIS provides maritime safety and navigation condition information (MSI) including navigational warning, weather information, hydrographic information and maritime environment information which are in S-10X format. Although MESIS provides services in terms of “data product” which is normally accepted by the community of the IHO, the service can deliver selected information only as well. with LTE-Maritime and VDES are being considered as physical communication links for MESIS but are not limited to these. Other communication links available on board for requesting this service can be used as well.

## SMART-Navigation services as a Maritime Service Portfolio in Korean waters

During the discussion at HGDM meeting, Maritime Service Portfolio (MSP) was redefined as list of services which are provided in certain port or waters. The original meaning of the MSP is to be used by Maritime Service in the context of e-Navigation.

Following the result of the discussions, SMART-Navigation services can be regarded as the MSP in Korean waters as SMART-Navigation aims to provide shore-based services applying all available standards and guidance on e-navigation concept.

In addition, machine to machine interface (M2M) is the main interaction scheme of SMART-Navigation services. This makes it possible for each of the six services of the SMART-Navigation regarded as an associated technical service of MSs.  As there is no completed description on an MS yet, it is necessary to use OS and TS alternatively.

The table below shows the relationship among MSs as defined in the IMO e-navigation Strategy Implementation Plan (SIP), together with the technical services as implemented by the SMART-Navigation project and S-100 product specifications.

| Maritime Services (MSs) |  | SMART-Navigation Technical Services | Related S-100 Product Specifications (interim) |
| --- | --- | --- | --- |
| MS1 | VTS Information Service (IS) | NAMAS | S-124, S-125, |
| MS2 | Navigational Assistance Service (NAS) |  | S-125 |
| MS3 | Traffic Organization Service (TOS) |  | S-127 |
| MS4 | Local Port Service (LPS) | NAMAS | S-xxx (marine services, harbor infrastructure) |
|  |  | MESIS | S-412 |
| MS5 | Maritime Safety Information Service (MSI) | MESIS | S-124, S-201, S-412 |
| MS6 | Pilotage Service | PITAS | S-xxx (marine services, harbor infrastructure) |
| MS7 | Tugs Service |  |  |
| MS8 | Vessel Shore Reporting | N/A | N/A |
| MS9 | Telemedical Assistance Service (TMAS) | N/A | N/A |
| MS10 | Maritime Assistance | NAMAS | S-xxx (marine services) |
|  |  | SBSMS |  |
| MS11 | Nautical Chart Service | REDSS | S-101, S-102, |
| MS12 | Nautical Publication Service | REDSS | S-101, S-201 |
| MS13 | Ice Navigation Service | N/A | N/A |
| MS14 | Meteorological Information Service | MESIS | S-412 |
| MS15 | Real-time Hydrographic and Environmental Information Service |  | S-104, S-111, S-112, S-201 |
| MS16 | Search and Rescue Service | N/A |  |

## Current status and future plans

After three years of service concept design, system design and implementation, SMART-Navigation project is on its alpha-test phase to verify service implementation from developers’ point of view. The first sea trial was carried out Gunsan region on 23rd of July 2019. The project team is planning to have six sea trial at least in this year before entering closed-beta test which will focus on validation of services from users’ point of view.

SMART-Navigation project has divided its testbed into four sections: East Bed, South Bed, West Bed and Virtual Bed. Each section has its own characteristics in terms of traffic density, type of ships, LTE-M communication quality, etc. Tests have been designed to reflect these characteristics to achieve objectives of the test with the best effectiveness and efficiency at the same time. SMART-Navigation services should be validated by showing that they resolve such difficulties imposed in each testbed area through providing ships with information when necessary and needed.

The East Bed is located in the east coast of the Korean peninsular facing the East Sea. It has least traffic density and tide difference compared to other regions while it has very good coverage of commercial LTE network already which indicates that the region will be the best LTE-M service area of Korean waters. Especially, port of Busan, the largest container port in Korea, is located in the East-bed. The East-bed is best testbed area for PITAS and e-Navigation services for SOLAS ships.

The South Bed has most complex coastal line consisting of archipelagos leading to limitation in watch-keeping. It is very hard to identify all the ships behind the island during navigating the region. As there are many feeders between Korea and China passing the area, there are many chances of interaction between SOLAS ships and Non-SOLAS ships. As archipelagos could create many blind spots of LTE-M network, stability of service subscription supported by MMS could be tested. High density fish farms and sea weeds farm give another challenge to navigators for their safe navigation in the area.

Testbed composition of SMART-Navigation project

The West Bed is suffering from the biggest tide difference up to 9 meters between full tide and low tide. Military trainings are often carried out in the area while providing ships with the shortest LTE-M coverage. S-10X testbed by Korea Hydrographic and Oceanographic Agency (KHOA) is located in the area.

The Virtual Bed is a cybernetics testbed located at Korea Research Institute of Ships and Ocean Engineering (KRISO). Live traffic data from Korean waters are streamed into the Virtual Bed for verification and validation of SMART-Navigation services from shore side point of view. Simulated traffics can be added to the live traffic as well during tests which could create more risk-specific situation without endangering ships and people in reality. The Virtual Bed is connected to other maritime simulators using Asia-Pacific Maritime Simulator Network (APMSN) which is a sister simulator network of European Maritime Simulator Network (EMSN). Engineering works to connect APMSN to EMSN is under progress in collaboration with Fraunhofer CML.

Sea trials have been carried out mainly in West Bed and South Bed as there are more chances to interact with small fishery ships in the regions. Through the sea trials so far, specific functions of each service have been tested where they work properly in real sea environments as they did in simulated environments. Core elements of SMART-Navigation service architecture such as MMS and MCP Connector have been tested and revised following the results. Although this is what should be done in alpha-test phase, the project team found some technical items that should be enhanced for the maturity of the services including: (1) position information fusion, (2) quality of dead reckoning, (3) acquisition of real sea information and (4) information delivery scheme. All these items will be enhanced and tested throughout alpha and (closed/open) beta test period until the end of 2020.

Electronic Chart System (ECS) will be used for service terminal onboard for SMART-Navigation service subscription. Ministry of Oceans and Fisheries is promoting fishing vessels and domestic liners to install ECS. For (closed or open) beta test, it is expected 2,300 ship-borne navigational aid devices could be engaged by the first half of 2020. In addition to real sea tests, simulation tests using APMSN will be carried out as well to validate services with more flexible scenarios in parallel with beta test.

Screenshot of SORPS service on ECS

# Action requested of the Committee

The Committee is requested to:

- Note this paper;

- Join simulated tests via APMSN/EMSN and real sea tests in collaboration with SMART-Navigation project.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain