IALA Guideline

1147

the USE of enhanced RADAR POSITIONING SYSTEMS

Edition 1.0

Document date

Revisions to this IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |
| month/year approved by Council | aaaaa | aaaaaa |

1	INTRODUCTION	2

1.1	(Example Heading level 2)	2

1.2	(Example Heading level 2)	2

2	AIMS AND OBJECTIVES (Example Heading level 1)	2

3	INTRODUCTION (Example Heading level 1)	2

3.1	(Example Heading level 2)	2

3.1.1	(Example heading level 3)	2

4	OVERVIEW (Example Heading level 1)	2

4.1	TABLES	2

5	FIGURES	2

6	DEFINITIONS	2

7	ACRONYMS	2

8	REFERENCES	2

ANNEX A	EXAMPLE OF AN ANNEX - LANDSCAPE	2

APPENDIX 1	EXAMPLE OF AN APPENDIX TITLE	2

ANNEX B	(EXAMPLE ANNEX TITLE)	2

ANNEX C	PERMITTED COLOUR PALETTE	2

List of Tables

Table 1	Example of a table with the significant information in the first column	2

Table 2	Example of a table with the significant information in the first row	2

Table 3	Example of a table with coloured rows	2

Table 4	Example table	2

List of Figures

Figure 1	Example figure caption	2

Figure 2	Another example figure caption	2

List of Equations

Equation 1	Geographical range	2

Equation 2	Theory of Special Relativity	2

# INTRODUCTION

Enhanced Radar Positioning System (ERPS) generates position information utilising enhanced Radar (eRadar) and enhanced Racons (eRacons). The system is independent from Global Navigation Satellite System (GNSS).

Add figure,,,,

## RESILIENT PNT AND BACKUP SYSTEMS

GNSS has been widely recognised as the primary PNT data source. However, its vulnerabilities have been identified, which could lead to GNSS outage or providing erroneous position information. This makes GNSS-dependent navigation systems unable to provide the expected performance. IMO, in its e-Navigation Strategy Implementation Plan (MSC.1/Circ.1595) identifies and captured the risk as one of the Risk Control Options (RCO 5) “Improved reliability and resilience of on-board PNT systems”. In order to achieve resilience in PNT service provision, it is necessary to put in place a back-up or fall-back arrangements utilising the techniques that do not share the failure modes of GNSS. In the case of GNSS outage or malfunction, the backup system can provide PNT services.

## INTRODUCTION TO ENHANCED RADAR POSITIONING

ERPS is a simple concept in which the eRacon provides absolute position information encoded in its response signal to the eRadar. The concept is similar to what navigators would do by hand, using radar target azimuth and distance to triangulate a vessel’s position. eRacons are essentially normal racons modified to encode their identification and position into the signal response to the radars that interrogate them. Data is encoded using modulation in the leading dash of the racon Morse code response. eRacon position (latitude, longitude and elevation) is surveyed and entered as static parameters in the eRacon configuration. eRacons must be located at fixed sites and not placed on buoys.

eRadars receive responses from eRacons, detect the modulated signals and extract the identification and position data from them. In order to calculate the position, it is necessary either to have:

A single eRacon signal together with own ship heading; or

Signals from two or more eRacons.

The calculated position would be transmitted to the connected navigation systems, such as ECDIS, through a standard NMEA GGA sentence.

Knowing the azimuth, range and location of the eRacon targets, the speed and heading of their own vessels and their own antenna elevation and rotation characteristics, eRadars calculate and report positions for their own vessels. If available, multiple eRacons are used simultaneously to improve position accuracy.

ERPS utilises e-Racons and e-Radar to calculate the position. E-Racon is the modified Racon that, when received radar interrogation signal, responds to the e-Radar with encoded the information about its ID and position. E-radar, in its turn, decodes the received information and calculate range and bearing from own ship to responding e-Racon and calculate the position.

# AIMS AND OBJECTIVES

Developing eRacon technology seeks to add a layer of resiliency to ports and waterways by diversifying position information inputs to the navigation system with accurate, reliable, and real-time positioning systems independent of GNSS. Establishing guidelines for eRacon and eRadar interoperability provides a framework for utilisation of ERPS.

ERPS, as a backup system, adds resilience to position fixing for harbour entrance, harbour approach and coastal waters navigation areas.

Talk about expected accuracy for 1 and 2 or more.

Defer availability continuity limitations.

# TECHNOLOGY

Task: Describe integrity; what it means in context of ERPS

# LIMITATIONS

Task: Describe accuracy, availability, continuity goals

Accuracy (complex, 1, 2, 3 or more racons)

Availability (complex, 1, 2, 3 or more racons)

Continuity (complex, 1, 2, 3 or more racons)

Annex A How it Works

Describe how concept works

Describe benefits

Describe difficulties (busy harbors, blocking, geometry)

Note that the concept is not standardized.

Describe technical characteristics of eRacons

Refer to other IALA documents for use and installation of racons in general

table below:

Table 2-1: IMO Resolution A.1046 operational Requirements

|  | Ocean waters | Harbour entrance, harbour approach and coastal waters |
| --- | --- | --- |
| Accuracy (95% Horizontal Navigation System Error (HNSE)) | 100 m | 10 m |
| System Integrity* | As soon as practicable by Maritime Safety Information | Within 10s |
| Signal Availability | 99.8% | 99.8% |
| Continuity | N/A | 99.97% (over 15 min) |

*Integrity warning of system malfunction, non-availability or discontinuity should be provided to users within 10s.

- For ocean waters: the system should provide positional information with an error not greater than 100 m with a probability of 95%. Signal availability should exceed 99.8%. An integrity warning of system malfunction, non-availability or discontinuity should be provided to users as soon as practicable by Maritime Safety Information (MSI) systems.

- Navigation in harbour entrances, harbour approaches and coastal waters: positional information with an error not greater than 10 m with a probability of 95%. Signal availability should exceed 99.8%. When the system is available, the service continuity should be ≥99.97% over a period of 15 minutes. An integrity warning of system malfunction, non-availability or discontinuity should be provided to users within 10s.

During the ERPS trial in Singapore in 2015 [3], the first trial in a busy harbour, the use of the eRacon in frequency agile mode (the normal racon mode, compared to fixed frequency mode) failed. This problem seemed to be similar to anecdotally reported problems of racons performing poorly in busy harbours. The reported effects range from limited response to no response, which can change over time due to vessel movement. To quantify the problem an experiment was designed to survey radar traffic. A test racon was made available to capture the survey data. It was observed that radar frequencies seem to be clumped around a small number of frequencies and that there may be many radars at the same frequency. This can lead to various effects that also apply to normal racons [5].

The authors acknowledge the limits of magnetron production capabilities. However, the authors also anticipate the industry movement towards solid state radars. Two suggestions for manufacturers of solid state radars are to: a) use more of the frequencies within the band, and b) avoid repeatedly using the same frequency on a large number of units [5].

Modulation of the eRacon signal can be visible on radar displays (e.g. “fuzzy” traces). Characteristics such as modulation frequency, modulation depth and data bit rate affect the display. Further study is needed to minimize display effects due to modulation.

Also observed was a geometric error similar to the Horizontal Dilution of Precision (HDOP) for GNSS. This is caused by overlapping azimuthal measurement errors. In many waterways, there will be a need to carefully plan the deployment (number and placement) of eRacons in order to minimize errors of this type.

As vessels approach eRacons, the elevations of the eRacon and eRadars antennas can be included in the solution in order to calculate a more accurate position. For instance, with a relative elevation difference of sixty meters, at one nautical mile separation, position error would be about one meter. Further study will determine if antenna elevation correction is needed.

Encouraged by the successful trials, the MPA has announced that it will be establishing a network of eRacons along the Singapore Strait as a test bed site to promote further development and use of ERPS. The aim is to promote the development of a standardized independent complementary electronic positioning system to the GNSS in order to enhance safety of navigation in confined waters

Describe eRacon/eRadar concept.

Describe why we need it. Compare to GNSS. IMO Resilient PNT. Backup or Contingent? Find definitions.

Alternative means of navigation may be provided at various levels; fully redundant, backup and
contingency1.
A redundant system provides the same functionality as the primary system, allowing a
seamless transition with no change in procedures.
A backup system ensures continuation of the navigation application, but not necessarily
with the full functionality of the primary system and may necessitate some change in
procedures by the user.
A contingency system allows safe completion of a manoeuvre, but may not be adequate
for long-term use

Backup systems may include existing or planned terrestrial systems such as:
Loran C;
Enhanced Loran (eLoran);
Radar and radar aids to navigation;
Cellular telephone based systems;
Ranging from DGNSS/AIS (R-Mode).

Radar can be used for position-fixing, but it does not generally provide a compatible input to an
electronic charting system; therefore different procedures are necessary for its use. However,
radar is a required fit on SOLAS vessels and radar map-matching (echo referencing) techniques
are well developed and used in waters such as the archipelagos between Sweden and Finland.
The most serious limitation of radar as a backup is in areas with low-lying, featureless coastlines,
for example in Europe those of Northern France, Belgium and the Netherlands or the East coast of
England. In order to make radar a universal backup, such areas would need to be marked with
sufficient radar aids to navigation.
This would not provide the same level of positioning service as Loran, nor would it give an
alternative timing reference. However, this option could justify further investigation in regions
where Loran is not a realistic option. Radar and radar beacons also have vulnerabilities to failure
of equipment or power supplies, multipath, rain and sea clutter and masking effects.
An

Radar has always been used as a resilient relative positioning system. When used with fixed, known absolute position targets, a fair absolute position solution for a vessel can be calculated. But identifying targets can be difficult and published (navigation chart) positions may be approximate. A system known as Enhanced Radar Positioning System (ERPS) uses specially modified racons (radar beacons, or eRacons) with specially modified radars (eRadars) to allow radars to automatically calculate absolute position. In this system, eRacons provide their absolute position encoded on their response signals to eRadars, which use these signals to calculate their own vessels’ positions. Recent trials in Singapore using three eRacons demonstrated dynamic absolute position differences of better than 27 meters compared to Real Time Kinetic position solutions, and accuracies of 2.5 meters with the vessel berthed. ERPS can be very cost effective in that radars (with digital receivers) and racons (with digital transmitters) can be fitted with needed software at very low per unit cost. This paper discusses the need for resilient, redundant positioning independent of Global Navigation Satellite Systems, the history and results to-date of ERPS test beds and practical issues and next steps in adopting ERPS, including standardization.

Many feel that there is an over-reliance on GNSS. There are many well documented cases of GNSS interference which significantly affect maritime navigation. In some situations, the interference resulted in the complete failure of the mariner’s GNSS receiver or mariners were presented with misleading navigational information. At present there is a lack of an electronic precision navigation systems if GNSS ceases to function; vessels could risk going aground or going off course which could result in collision with other vessels.

The many GNSS are based on similar technology and are therefore susceptible to the same interference and jamming: “However, GNSS is vulnerable to space-vehicle failures, solar disturbances, unintentional radio interference and, increasingly, deliberate jamming.” [6]. Reliance on positioning system technologies with known vulnerability can be considered to be an unacceptable navigational risk, especially in restricted waters where marine traffic is heavy.

In view of the vulnerabilities and over reliance on GNSS positioning, there is need to establish and develop accurate, reliable and real-time positioning systems independent of GNSS. The International Maritime Organization (IMO) is addressing these problems within its e-Navigation plan. “The IMO Maritime Safety Committee has stated that ‘e-Navigation systems should be resilient and take into account issues of data validity, plausibility and integrity for the systems to be robust, reliable and dependable. Requirements for redundancy, particularly in relation to position fixing systems should be considered.’ (IMO MSC 85/26 Add.1)” [6

Radar systems are already used as alternative positioning systems that are independent of GNSS. It is mandatory for all SOLAS vessels above 300 GT to have X band radar equipment installed on board to identify fixed targets and objects, provide bearing and distance and for collision avoidance purposes, and that the radar trigger and display racons. Further, “Radar systems are not susceptible to solar activity or jamming at GNSS frequencies…” [6]. Standard radar systems are best at producing relative positioning and are less useful for calculating absolute position.

A system known as Enhanced Radar Positioning System (ERPS) uses specially modified racons (radar beacons, or eRacons) with specially modified radars (eRadars) to allow the radars to calculate absolute position. In this system, eRacons provide their absolute position encoded on their response signals to eRadars, which use these signals to calculate their own vessel’s position. ERPS can be a viable and attractive back up option to GNSS, because it is independent and potentially uses equipment that is already provided on-board vessels and on land.

ERPS is a simple concept. In this system, eRacons provide their absolute position encoded on their response signals to eRadars, which use these signals to calculate their own vessels’ positions. This is similar to what navigators would do by hand, using radar target azimuth and distance to triangulate vessels’ positions.

eRacons are essentially normal racons, modified to encode their identification and position into the signal response to the radars that interrogated them. Data is encoded using modulation in the leading dash of the racon Morse code response. eRacon position (latitude, longitude and elevation) is surveyed and entered as static parameters in eRacon configuration. eRacons are located at fixed sites and not placed on buoys.

eRadars receive responses from eRacons, detect the modulated signals from eRacons and extract the identification and position data from them. Knowing the azimuth, range and location of the eRacon targets, the speed and heading of their own vessels and their own antenna elevation and rotation characteristics, eRadars calculate and report positions for their own vessels. If available, multiple eRacons are used simultaneously to improve position solutions.

There is no dependency on GNSS. The vessels’ own position can be calculated with no previous knowledge of the positions of the vessels. No other external data is needed.

Body text (To assist in the use of this guideline, the following acronyms and definitions have been used: GNSS IMO PNT)

Body text (left justified)

Body text (left justified)

List 1.

List1.

List 1 text (for subsequent text at the same level).

List 1:

List a.

List a.

List a text (for subsequent text at the same level).

List a:

List i.

List I text (for subsequent text at the same level).

Bullet 1;

Bullet 1 text (for subsequent text for the same bullet).

Bullet 1:

Bullet 2;

Bullet 2 text (for subsequent text for the same level).

Bullet 2:

Bullet 3;

Bullet 3 text (for subsequent text for the same level)

Bullet 3.

# Example Heading level 1

Body text

## (Example Heading level 2)

Body text

Rg=2.03h0+Hm

Geographical range

Where:

Rg is the geographical range (nautical miles) (alternatively NM)

ho is the elevation of observer’s eye (metres) (alternatively m)

Hm is the elevation of the mark (metres) (alternatively m)

### (Example heading level 3)

Body text.

E=mc2

Theory of Special Relativity

Where:

E is the kinetic energy (Joules) (alternatively J)

m is the mass (kilograms) (alternatively Kg)

c is the speed of light (metres/second) (alternatively m/s)

#### (Example heading level 4)

Body text.

# Example Heading level 1

Body text.  Bullets have only one sentence.  Anything further needs to appear in the relevant 'bullet text' style.

Bullet 1:

Bullet 1:

Bullet 1.

## TABLES

Body text

Example of a table caption; table with the significant information in the first column

| Table heading | Table text |
| --- | --- |
| Table heading | Table text |
| Table heading | Table text |
| Table heading | Table text |
| Table heading | Table text |
| Table heading | Table text |

Example of a table caption; table with the significant information in the first row

| Table heading | Table heading | Table heading |
| --- | --- | --- |
| Table text | Table List 11 - Table list a Table list i | Table text |
| Table text | Table text | Table text |
| Table text | Table text | Table text |
| Table text | Table text | Table text |

Example of ‘normal text’ following a Table

Example of a table caption; table with coloured rows

| Table heading | Table heading | Table heading |
| --- | --- | --- |
| Table text | Table text | Table text |
| Table text | Table text | Table text |
| Table text | Table text | Table text |
| Table text | Table text | Table text |
| Table text | Table text | Table text |
| Table text | Table text | Table text |

Note:  Colours for text and cell shading need to be selected from the permitted palette (see ANNEX C)

# FIGURES

Example figure caption

Another example figure caption

# DEFINITIONS

Suggested text: The definitions of terms used in this IALA Guideline can be found in the International Dictionary of Marine Aids to Navigation (IALA Dictionary) at http://www.iala-aism.org/wiki/dictionary and were checked as correct at the time of going to print.  Where conflict arises, the IALA Dictionary should be considered as the authoritative source of definitions used in IALA documents.

# ACRONYMS

IMO	International Maritime Organization (Acronym style)

# REFERENCES

Abcd

Efgh

EXAMPLE OF AN ANNEX - LANDSCAPE

Body text

example of ANNEX heading level 1

Body text

example of annex heading level 2

Body text

Example of annex heading level 3

Body text

Example of Annex heading level 4

Body text

Example table caption

| No | Title/Topic | IMO References | Requirements | Possible Audit Questions | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | Table text | Table text | Table text | Table text | Table text |
|  |  | Table text | Table text |  |  |
|  |  | Table text | Table text |  |  |

EXAMPLE OF AN APPENDIX TITLE

APPENDIX HEADING 1

Body text

APPENDIX HEADING 2

Body text

APPENDIX HEADING 3

Body text

Appendix Heading 4

Body text

(EXAMPLE ANNEX TITLE)

Introduction (Example Annex Heading 1)

Body text.

Example of ANNEX HEADING Level 2

Body text

Example of annex heading level 3

Body text

Example of Annex heading level 4

Body text

PERMITTED COLOUR PALETTE

The IALA colour palette is divided in 3 palettes of different level of hierarchy that has to be respected.

Corporate colours (Not shown)

IALA’s corporate colour palette is directly inspired from the colours in our logotype:

- dark blue

- white

- yellow

- gradient blue

Primary & secondary colours

The primary colours are to be applied in complement with the corporate colours.

This second level of colours gives rhythm and helps to segment our publications.

The secondary colours are used to highlight information, titles in a minor proportion only.

These colours can’t be replaced by other tints.

PANTONE PROCESS CYAN C CMYK : C 100

RGB : R 0 - G 159 - B 223

CMYK : 50 % OF THE TONE RGB : R 131 - G 208 - B 245

CMYK : 50 % OF THE TONE RGB : R 148 - G 217 - B 213

CMYK : 50 % OF THE TONE RGB : R171 - G 219 - B 233

CMYK : 50 % OF THE TONE RGB : R 178 - G 193 - B 237

PANTONE 326C CMYK : C 81 - Y 39

RGB : R 0 - G 175 - B 170

PANTONE 7703 C

CMYK : C 79 - M 2 - Y 10 - K 11

RGB : R 0 - G 181 - B 208

PANTONE 660 C CMYK : C 88 - M 50

RGB : R 64 - G 126 - B 201

CMYK : 20 % OF THE TONE RGB : R 212 - G 237 - B 252

CMYK : 20 % OF THE TONE RGB : R 213 - G 240 - B 237

CMYK : 20 % OF THE TONE RGB : R 216 - G238 - B 245

CMYK : 20 % OF THE TONE RGB : R 218 - G 223 - B 246

PANTONE 258 C CMYK : C 51 - M 79

RGB : R 153 - G 80 - B 159

CMYK : 50 % OF THE TONE RGB : R 201 - G 169 - B 208

CMYK : 50 % OF THE TONE RGB : R 183 - G214 - B 155

CMYK : 50 % OF THE TONE RGB : R 246 - G 174- B 135

CMYK : 50 % OF THE TONE RGB : R 157 - G 157 - B 156

PANTONE 739 C

CMYK : C 78- Y 95- K 5

RGB : R82 - G 174 - B 50

PANTONE 2347 C

CMYK :M 88 - Y 100

RGB : R 230 - G 56 - B 17

PANTONE COOL GRAY 11 C CMYK : K 100

RGB : R 87 - G 87 - B 86

CMYK : 20 % OF THE TONE RGB : R 232 - G 221 - B 288

CMYK : 20 % OF THE TONE RGB : R226 - G 238 - B 217

CMYK : 20 % OF THE TONE RGB : R 253 - G 224- B 208

CMYK : 20 % OF THE TONE RGB : R218 - G 218 - B 218

CMYK : 10 % OF THE TONE RGB : R 237 - G 237 - B 237

Guideline

Recommendation

Model Course

PRIMARY COLOURS

SECONDARY COLOURS

The vulnerabilities are captured in IALA Recommendation R-129 paragraph 3.

Example footnote

Example of footnote