Input paper:   ENG10-3.1.10

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	X  ENG		□  PAP					X  Input

□  ENAV	□  VTS							□  Information

Agenda item 			n.n

Technical Domain / Task Number 2	TD#1 - Light and vision physics, Visual Signalling /

Author(s) / Submitter(s)			Pärtel Keskküla

# Proposed amendments to the NAVGUIDE related to Threshold of Illuminance and abbreviations

# Summary

NAVGUIDE says at the bottom of the page 31 that “it is recommended that to observe the relative position of the lights easily and to derive the maximum possible accuracy from leading and sector lights, it is generally necessary to have a minimum illuminance of 1 μlux at the eye of the observer”. As I have understood it is supposed to be 0.2 lx for sector lights.

An editorial observation besides that: While all other units are abbreviated, lux is generally not abbreviated in NAVGUIDE (with two exceptions) (and in some other documents), despite its abbreviation lx being available.

# Action requested of the Committee

- Consider correcting the NAVGUIDE.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain