Input paper:   ENG10-3.1.12

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	X  ENG		□  PAP					□  Input

□  ENAV	□  VTS							X  Information

Agenda item 			3

Technical Domain / Task Number 2	…………………………………

Author(s) / Submitter(s)			Ginés Moreno, Ana Cezón, Gema Cueto-Felgueroso / GMV

Manuel López / European GNSS Agency (GSA)

Thiago Tavares / VVA

# Use of Galileo Timing to support implementation of R-MODE

# Summary

This paper proposes different Galileo timing use cases to support the R-mode (Ranging Mode) infrastructure synchronization.

## Purpose of the document

IALA ENG Committee WG3 Radionavigation Services is invited to consider and if found feasible, implement any of the proposed use cases into R-mode system.

## Related documents

No related documents.

# Background

GNSS is already widely used in different fields for timing and synchronization purposes, including critical infrastructures in which a freely, stable and accessible timing source is required.

R-mode (Ranging mode) technology is being developed in the frame of maritime community as a terrestrial backup positioning system for maritime and inland waterways navigation in case any of GNSS vulnerabilities takes place. Jamming of GNSS signals can be achieved quite easily using relatively low-cost equipment in any single or multiple frequencies. Spoofing is more difficult to achieve, therefore the most common spoofing techniques only spoof GPS L1 signal but there are already some techniques that are able to spoof with a low-cost equipment a dual-frequency (L1&L2) receiver.

R-mode allows calculating user receiver position in relation to fixed-shore and synchronized radio transmitters. Up to date, the most relevant initiatives for R-mode deployment make assumptions regarding infrastructure synchronization; in particular ACCSEAS project assumed perfect synchronisation between nodes. The most recent project, R-mode Baltic, is currently implementing GPS-based timing solutions, but the next steps of the project may consider other technologies (atomic clocks, Galileo PRS, eLoran). At this point, Galileo represents an interesting and cost-effective timing solution which could be considered in the frame of R-mode.

According to the outcomes from R-mode Baltic project, 10 ns of timing accuracy during 2h is necessary to provide coastal and inland waterways navigation using R-mode to fulfil the accuracy requirement of 3 meters. This poses stringent requirements for timing, taking into account that the level of performance required to be ensured in critical infrastructures is generally larger than 10 ns. To mitigate the synchronization error on the reference station side, a rubidium oscillator (at least) should be utilized. This conclusion is equal to the one obtained in ACCSEAS project.

This paper analyses the adoption of new Galileo and EGNOS Timing services for R-Mode system.

# Discussion

The addition of Galileo satellites will contribute to improve the accuracy performance of GNSS users, including the time and synchronization community. Galileo satellites may also improve integrity and ensure availability of the timing service in case other GNSS constellation experiences outages. Galileo presents an additional advantage with respect other GNSS constellations thanks to the new OS-NMA (Open Service Navigation Message Authentication) capability, which adds an extra security layer against spoofing attacks.

The EC (European Commission) is also working on the definition of an EGNOS and Galileo Timing service with committed end-to-end performances in the frame of the EGALITE project.

Considering this landscape, three different use cases have been identified in which Galileo could provide added value to timing users (and in particular for R-mode synchronization) which are summarized below:

| Description | Benefits | Drawbacks |
| --- | --- | --- |
| Use Case 1: Galileo OS Timing Determination Capability | - Simplicity - Already available - Redundancy | - No end-to-end performance commitment. - Vulnerability to interferences |
| Use Case 2: Galileo Timing service with committed end-to-end performances | - Guaranteed performances - Integrity at system level | - Not currently available - Requires appropriate user equipment |
| Use Case 3: Galileo OS-NMA Complementary to Galileo OS and/or guaranteed service | - Robustness of the solution - Does not affect Galileo timing performances | - Requires appropriate user equipment |

None of the solutions commit the stringent timing requirements (10 ns) for a long period of time (2h) and which are even larger in comparison to other applications. However, typical Galileo timing performances may fulfil such R-mode needs since in the latest available Galileo OS Performance Report (Q1 2019), the average accuracy is around 8.5ns.

In conclusion, Galileo is a promising technology to be considered as a cost-effective alternative to the use of atomic clocks for R-mode infrastructure synchronization. Let us remark that the nature of R-mode is to be a backup solution for local GNSS vulnerabilities aforementioned. Despite GNSS is still needed for R-mode synchronisation, the limited range of intentional attacks makes difficult that a single spoofer affects a vessel and a significant number of R-Mode stations. Therefore, situations described allow for the use of GNSS timing at a station level for timing purposes.

# References

Maritime Safety Committee, MSC 85/26, Annex 20. Strategy for the development and implementation of e-navigation, 19 December 2008

Multisystem - Radionavigation Receiver Performance Standard MSC.401(95), 8 June 2015

Worldwide Radionavigation System. IMO Res. A.1046 (27)	, 30 November 2011

“Guidelines for shipborne position, navigation and timing (PNT) data processing” (MSC.1/circ.1575), 16 June 2017

IALA Recommendation GNSS vulnerability and mitigation measures R-129, 07 December 2012

Guidelines for shipborne position, navigation and timing (PNT) data processing. MSC.1/Circ.1575, 16 June 2017

Galileo Open Service – Service Definition Document (OS-SDD) Issue 1.1, May 2019

“Time and Synchronization User Requirements” GSA-MKD-TS-URED-233690 v1.0, 08/09/2017

“GSA User Technology Report”, Issue 2.0, 2018

“GSA Market Report” Issue 5, 2017

ACCSEAS Final Report Review of ACCSEAS Solutions through tests and demonstrations, 13/05/2015

Feasibility Study of R-Mode using AIS Transmissions. Investigation of possible methods to implement a precise GNSS independent timing signal for AIS transmissions, 29/08/2014

Feasibility Study of R-Mode using MF DGPS Transmissions	Report	11/03/2014

Hoppe, M & Johnson, Gregory & Swaszek, Peter & Alberding, J & Oltmann, J.-H. (2014). Feasibility of DGNSS R-mode as a Component of the Multi-Source Positioning Service, 04/2014

Hoppe, M & Strenge, Rainer. Developments in radio navigation systems. PIANC-World Congress Panama City, Panama 2018, 05/2018

R-Mode Baltic - Baseline and Priorities, 11/03/2019

# Action requested of the Committee

The Committee is requested to:

- Note the information within this paper.

Discuss the matter at an appropriate time and

Include the proposed into the IALA documentation if so considered by the discussion

- Annex Galileo timing capabilities for R-MODE synchronization

Time and synchronization community is experiencing an increased need for reliability and security, fostered by new regulation impositions and more stringent requirements in terms of accuracy, stability and reliability. The timing capabilities have the particularity to be exploited by sectors which are strategic and critical for the functioning of the modern society, such as Telecommunications, Energy and Finance.

Galileo features which could be valuable for timing users are presented in section 1 of this annex. Then, section 2 proposes different use cases in which Galileo could be used for timing purposes based on the outcomes of the previous section. Finally, the applicability of Galileo for R-mode infrastructure synchronization based on previous analysis is presented in section 3.

- Galileo timing capabilities

For timing users, the addition of Galileo satellites will contribute to ensuring that the most challenging requirements in terms of accuracy are met, even for 5G requirements. Moreover, Galileo satellites may also improve integrity and ensure availability of the timing service in case other GNSS systems experience outages.

Critical infrastructures can be subject of cybersecurity attacks which can also affect GNSS. At this point, Galileo presents an advantage with respect other GNSS constellations thanks to the new OS-NMA capability, which adds an extra security layer against spoofing attacks. In the frame of services with protection against attacks, EC launched a project to define an EGNOS and Galileo Timing service, which is called EGALITE.

This section presents in greater detail the level of timing performance that could be achieved thanks to the use of Galileo OS (see section 1.1), together with the introduction to EGALITE project (section 1.2).

  - Galileo Open Service

Ranging allows users to continuously estimate their distance to Galileo satellites, and in this way user’s position and time can be estimated using ranging measurements from at least 4 different satellites. GNSS satellites broadcast additional data which allow GNSS receivers to relate the GNSS time scale to international reference timescales. In the case of Galileo, the precise atomic clocks in the Galileo system give direct and accurate access to UTC reference. In this way, UTC Time determination is one of the services supported by Galileo OS SIS, together with the ranging and positioning capabilities since it provides integer offset between the Galileo System Time (GST) and UTC and fractional GST-UTC difference at nanosecond level

This allows users to time-stamp their data in UTC and to precisely synchronize to UTC their clocks, which can be situated at very different locations in the world.

The Galileo UTC Time Determination Accuracy is the accuracy in the UTC estimation and depends on the instantaneous GST determination error and in the Galileo SUS UTC Time dissemination Accuracy (error in the broadcast GST-UTC conversion parameters) and characterised by:

- Galileo SIS UTC Time Dissemination Accuracy: driven by the accuracy of the broadcast GST-UTC parameters

- Galileo SIS UTC Frequency Dissemination Accuracy: major contributor to the UTC frequency transfer accuracy

The Minimum Performance Levels defined to timing services in Galileo OS SDD are:

Table 5-1: MPL for Galileo timing services

| Parameter | Value | Conditions and constraints |
| --- | --- | --- |
| Galileo SIS UTC Time Dissemination Accuracy | For each SF and DF combination, less than 30 ns (95%) | - For any healthy OS SIS - At any user location - Normalised annually - Propagation and user contributions excluded |
| Galileo SIS UTC Frequency Dissemination Accuracy | For each SF and DF combination, less than 3E-13 (95%) |  |
| Availability of Galileo OS UTC Time Dissemination Service | For each SF, greater than 87% of time a user is provided at least one healthy SF SIS For each DF, greater than 87% of time a user is provided at least one healthy DF SIS | - Calculated over a period of 30 days - Above a minimum elevation angle of 5 degrees - From any point in the service coverage - Including planned and unplanned outages |
| Availability of the Galileo UTC Time Determination Service | For each SF, greater than 87% For each DF, greater than 87% | - With UTC Time determination accuracy better than 31 ns (95%) - At least 1 satellite in view above a minimum elevation of 5 degrees - From users at a known location - From any point in the service coverage - Calculated over a period of 30 days - Including Broadcast Group Delay errors and neglecting single frequency ionospheric delay model errors (only for SF) - Propagation and user contributions excluded |

Apart from Galileo OS current features, let us remark the OS-NMA authentication capability which is being developed and foreseen to be available for 2020. The main point to take into account is that Galileo OS-NMA allows users to check the authenticity of the navigation data which are they receiving, so this adds a security layer and protection against spoofing.

  - EGALITE project

EGALITE (EGNOS and Galileo Timing Service Extension and Consolidation) project EGALITE project is funded by the EC (European Commission) as part of the Horizon 2020 framework programme and aimed at studying the possibility of developing an EGNSS Timing Service based on Galileo and/or EGNOS (European Geostationary Navigation Overlay Service) with committed end-to-end performances. The main activities performed in this project are:

- Extension and Consolidation of European GNSS Timing Services early defined by the EC in previous activities, including the definition of Integrity capability for Timing in the context of EGNOS and Galileo and the analysis on the usage of High-Accuracy techniques for Timing.

- Analysis of GNSS use in Legal Time across Europe.

- Awareness and Dissemination activities in order to interact and gather feedback from Timing and Synchronization community, including end users and receiver manufacturers.

The results of this activities will lead the project to propose EGNSS Timing Services based on both EGNOS and Galileo with committed end-to-end performances provided EGNSS Timing Receivers are certified.

Implementing a proper Service implies to put all the necessary elements in place to be able to meet a certain level of performances and to ensure that it is maintained over the system lifetime. Declaring the minimum level of performance means that the users can rely on that minimum level of service to build their applications. As such, one of the conclusions obtained from this project is the definition of three different levels of performances for which timing accuracy has to be ensured. These three levels are based on different application requirements and the most stringent of them aims to provide 10ns of timing synchronization.

In other words, the service being proposed in the frame of EGALITE project is based on the definition of a guaranteed Galileo timing service, in which there is a commitment with the users in the sense of providing certain level of performance, as long as users are equipped with the appropriate certified receiver. This is equivalent to the use of EGNOS SoL service in maritime.

In addition, the service is based on some specific flags that will be sent to the user to alert him whether Galileo may be used for timing purposes or not. The baseline system under definition is based on a SIS dissemination channel, but it may be feasible that in the future these flags could also be sent via internet (e.g. EDAS). This may be interesting for R-mode users, but let us remark that in such case, Galileo timing service compatible receivers would also be needed in each node of the network which requires synchronization.

- Galileo timing use cases

Based on the Galileo capabilities for timing presented in section 1, the following use cases for the use of Galileo for timing can be identified:

- Use Case 1: Use of Galileo OS Timing Determination Capability

- Use Case 2: Use of Galileo Timing service with committed end-to-end performances

- Use Case 3: Use of Galileo OS-NMA

Each use case is analyzed separately in the following subsections, together with the identification of the benefits and drawbacks for each implementation. Let us remark that these use cases are applicable for all the timing users, but it has been particularized in some cases for R-mode synchronization purposes.

  - Use Case 1: Galileo OS Timing Determination Capability

This use case is based on the use of Galileo OS for timing purposes, taking advantage of the Galileo OS Timing Determination capability provided by the freely accessible Galileo OS signal. This could be done either alone or in combination with other GNSS constellations.

Let us remark that even though Galileo OS-SDD indicates a MPL for timing around 30 ns, the typical performances are much better (below 10 ns). As an example the first 2019 Q1 Galileo performance report estimated a long term 95th percentile of UTC (Time) Dissemination Accuracy around 8.4 [ns] and the GGTO Determination Accuracy consistently around to 6.5 [ns]

Benefits:

- Simplicity and not expensive solution: it does not require a specific receiver. On the other hand, it is recommended to use a GNSS timing receiver optimized with an internal clock which maintains time stability in case of GNSS outage.

- Already available technology: ready to be used.

- Redundancy in the case more GNSS solutions are used. Better performance with respect non-Galileo solutions.

Drawbacks:

- There is no commitment on the end-to-end performances achieved at user level, SDD only provides reference values which do not confirm that R-mode requirement could be fulfilled.

- Vulnerability to intentional or natural interferences, in the same level to other GNSS constellations.

  - Use Case 2: Galileo Timing service with committed end-to-end performances

This is related to the use of a service equivalent to the one proposed in the frame of EGALITE project. The main difference with respect Galileo OS is the use of a specific service for timing users, and with committed end-to-end performances, that is, a service with performances at user level guaranteed to certain level of performance.

Benefits:

- Integrity at system level: alarms/warnings will we raised in case of system malfunction.

- Guaranteed service: performances are guaranteed to certain values (under certain assumptions such as not system malfunction and/or the use of specific receivers)

Drawbacks:

- Service under deployment, not totally defined nor available yet.

- Requires a certified or appropriate equipment to get access to such service, which it is likely to be more expensive and complex that a not-compliant receiver.

  - Galileo OS-NMA

Finally, Galileo OS-NMA is proposed as a complement to the two previous use cases as a mechanism to protect against spoofing attacks. Let us remark that the first one mainly refers to the use of current Galileo OS, so modifications are minimum to allow use of Galileo OS-NMA. In this way, Galileo OS-NMA could service as means to validate the use of Galileo OS or other Galileo timing services for timing and synchronization purposes.

Benefits:

- Robustness against spoofing

- It does not improve nor degrade Galileo timing performances (either from Use case 1 or 2): timing performances are not affected.

Drawbacks:

- Requires appropriate equipment to process the Galileo OS-NMA information, which may not be just a simple SW upgrade at existing Galileo receivers.

  - Summary

Please find below the table that summarizes the use cases proposed for Galileo timing purposes:

Table 5-2: Summary of Galileo timing use cases

| Description | Benefits | Drawbacks |
| --- | --- | --- |
| Use Case 1: Galileo OS Timing Determination Capability | - Simplicity - Already available - Redundancy | - No end-to-end performance commitment. - Vulnerability to interferences |
| Use Case 2: Galileo Timing service with committed end-to-end performances | - Guaranteed performances - Integrity at system level | - Not currently available - Requires appropriate user equipment |
| Use Case 3: Galileo OS-NMA Complementary to Galileo OS and/or guaranteed service | - Robustness of the solution - Does not affect Galileo timing performances | - Requires appropriate user equipment |

- Galileo timing as an input for R-mode

The previous sections have analysed the needs from R-mode in terms of timing and synchronization requirements and also, the capabilities of Galileo for timing users.

This section aims to analyse the feasibility from a technical perspective that Galileo could be considered as a timing source to synchronise R-mode infrastructure. Let us remark that according to the outcomes from R-Mode Baltic project, 10ns of timing accuracy is necessary to provide coastal and Inland Waterways navigation using R-mode. This poses stringent requirements for timing, taking into account that the level of performance required in critical infrastructures is generally larger than 10 ns.

At this point, Galileo OS MPL value is 30 ns, but typical Galileo performances tend to be lower. Indeed, in the latest available Galileo OS Performance Report (Q1 2019), the average accuracy is around 8.5ns. In this way, Galileo OS is technically feasible to meet the requirements from R-mode. However, the main point is that this level of performance is not guaranteed.

On the other hand, EGALITE project is analysing a potential Galileo service with a committed performance in the range of 30 ns considering integrity. In this way, it shall be explored whether the committed values from this service could reach the level of performance demanded by R-mode. The key is that, if the service is guaranteed, it can be considered as a trustable service for timing purposes since certain level of performance would be ensured during certain periods of time.

To both options, the use of Galileo OS-NMA represents a promising technology as long as R-mode users could be warned somehow in case they are experiencing spoofing and not being using Galileo under such conditions.

In addition, it is expected that a spoof attack does not affect simultaneously the objective vessel and several R-Mode stations, since its effect is only observed locally (around tens of kilometres around the spoofer). Therefore, even if the R-Mode stations is attacked and synchronization is lost, it may raise a warning and the other stations would provide the backup service to surrounding vessels. Furthermore, timing synchronization service that use multi-frequency signals may be used for R-Mode since L2 and L5 attacks are less common and the usage of more than one frequency allows the receiver may switch to using measurements from other frequencies and ignores the spoofed frequency.

In conclusion, Galileo is a promising technology to be considered as a cost-effective alternative to the use of atomic clocks for R-mode infrastructure synchronization. Let us remark that the nature of R-mode is to be a backup solution for GNSS vulnerabilities aforementioned. Despite GNSS is still needed for R-mode synchronisation, the limited range of intentional attacks makes difficult that a single spoofer affects a vessel and a significant number of R-Mode stations. Therefore, situations described allow for the use of GNSS timing at a station level for timing purposes.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain