Input paper:   ENG10-1.3.1.15 Rev1

Input paper for the following Committee(s): 	check as appropriate	Purpose of paper:

□  ARM	  ENG		□  PAP				  Input

□  ENAV	□  VTS						 Information

Agenda item 			3

Technical Domain / Task Number 2	…………………………………

Author(s) / Submitter(s)			CHINA MARITIME SAFETY ADMINISTRATION

# THE DEVELOPMENT STATUS AND PLAN OF BEIDOU AND BEIDOU SATELLITE-BASED AUGMENTATION SYSTEM

# Summary

This paper describes the composition, construction, service planning, future test plans of BeiDou Satellite-Based Augmentation System (BDSBAS), and updates The BeiDou Navigation Satellite System (BDS) information.

As an important milestone of the development of the he BeiDou Navigation Satellite System (BDS), it was announced that the development of the BDS-3 preliminary system is completed, to provide global services starting on December 27, 2018. BDS officially began to provide RNSS services to the world and planned to fully deploy BDS-3 by around 2020.

The construction goal of BDSBAS is to implement wide-area differential processing and integrity monitoring for BDS/GNSS satellites in China and its surrounding areas, broadcast augmentation information to users through BeiDou GEO satellites, to improve the accuracy, integrity, continuity and availability of BDS/GNSS in the service area.

## Purpose of the document

The purpose of the document is providing technical description for the Committee to review in order to update some information of BDS and BDSBAS in some related documents and obtain feedback and comments to be considered in the development of the associated Guidelines

## Related documents

[1]	Development of the BeiDou Navigation Satellite System （Version 3.0）, December 2018

[2]	BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1I (Version 3.0) , February 2019

[3]	BeiDou Navigation Satellite System  Open Service Performance Standard （Version 2.0）, December 2018

[4]	BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B3I (Version 1.0), February 2018

[5]	BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B2a (Version 1.0), December, 2017

[6]	Applications of the BeiDou Navigation Satellite System, December2018

[7]	BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1C (Version 1.0), December, 2017

[8]	Draft Guidelines for Manufacturers for the Implementation of SBAS in Shipborne Receivers Including Methods of Testing and Required Test Results, 31 July 2017

[9]	ICAO Standards and Recommended Practices (SARPS) Annex10 Volume I (Radio Navigation Aids)

[10]	RTCA The Minimum Operational Performance Standard, MOPS DO 229 Revision E

[11]	IMO Resolution MSC.379(93),” Adoption of the performance standards for shipshore BDS receiver equipment, July 2014

[12]	IMO Resolution MSC.401(95) performance standards for multi-system shipborne radionavigation receivers, August 2015

# Background

BDSBAS is an important part of BDS. China will carry out the design, test and construction of BDSBAS in accordance with the minimum ICAO Standards and Practices (SARPs) performance specifications, and provide service to the civil aviation, maritime, railways and other high integrity needs of users in China and its surrounding areas.

Currently, implementation plan of the system has been demonstrated which solidifies the technical status of the system in the standards of next-generation Dual Frequency Multiple Constellation (DFMC) SBAS.

# Discussion

## BDS Development Status and Plans

BeiDou Navigation Satellite System (BDS) has been formally providing regional-services on December 27, 2012.

BDS possess initial operational capability to provide open service for global users, especially for those in the countries around the Silk Road Economic Belt and the 21st-Century Maritime Silk Road before 2018. BDS will be completed by around 2020 provide open services for the users world-wide.

    - BDS Construction

The BDS-3 preliminary system is completed to provide global services on December 27, 2018. The 46th BDS satellite was successfully launched onboard a Long March 3B rocket from Xichang Satellite Launch Center at 02:09 AM on June 25th, 2019. As the 21st constellation satellite of and the second inclined Geo-stationary (IGSO) satellite added to the BDS-3 constellation, this satellite, upon completion of a series of on-orbit tests, will join the 20 previously launched BDS-3 satellites, to provide timely services and further enhance the BDS coverage capacity and service performance.

    - The overall performance of BDS (B1C and B2a signals)

BDS B1C and B2a signals (of BeiDou-3 basic system) prove good enough to provide global services. When PDOP ≤6, the availability is over 87%, the measured mean positioning precision is as follows: horizontally positioning accuracy is about 2.4m, vertical positioning accuracy is about 4.3m, velocity accuracy is about 0.06m/s and timing accuracy is 19.9ns (95%) . (the confidence level is 95%) as shown in Fig. 1

Availability of BDS-3 (PDOP≤6， el=5l )

    - Five types of services provided by BDS-3

BDS-3 will provide global basic navigation services, short-message communications services, international search & rescue services, satellite-based augmentation services and precise point positioning services to users. As shown in Fig. 2 and 3.

BDS Basic Navigation Services

BDS Services Area of Different Types

    - Short-message communication services (regional)

As for the regional short-message communications in China and surrounding areas, the system capacity will be increased by 10 times, per message length is 14000 bits, the user equipment transmit power will be reduced to 3W.

Service Planning:

3 GEO satellites

Serve China and the surrounding regions

System capacity increased by 10 times

Construction Development:

1 GEO satellite has been launched and the in-orbit technology experiments are currently being conducted

BDS  Short-message Communication Services Area(Regional)

    - Short-message communication services (global)

Global short-message communications service uses 14 MEO satellites to provide service to global users and the communications ability is about 560 bits a time.

Service Planning:

14 MEO satellites

Global encounter access

560 bits per message

Construction Development:

8 MEO satellites with short-message communication function have been launched and the in-orbit technology experiments are currently being conducted

    - International search and rescue services

International search & rescue service can serve global users by following IMO and COSPAS-SARSAT standards. The system, together with other satellite navigation system, constitute a global MEO search system and provide return link function, which can greatly increase search & rescue efficiency and ability.

Service Planning:

Follow IMO and COSPAS-SARSAT standards

6 MEO satellites with SAR payload

Return-links

Construction Development:

2 MEO satellites with SAR payload have been launched and the in-orbit technology experiments are currently being conducted

    - Satellite-based augmentation services

SBAS services will serve users in China and surrounding areas, support single-frequency and double-frequency multi-constellation augmentation services by following ICAO standards and strive to satisfy requirements set by ICAO.

Service Planning:

3 GEO satellites

Follow ICAO standards

Serve China and surrounding areas

Construction Development:

1 GEO satellite has been launched and the broadcast experiment is carried forwad

SBAS Services Area

    - Precise Point Positioning Service

PPP services use 3 GEO satellites to broadcast PPP signals and can provide dynamic decimeter-level and static centimeter-level high-precision service.

Service Planning:

3 GEO satellites

Serve China and surrounding areas

Positioning precision: dynamic decimeter-level，static centimeter-level

Construction Development:

1 GEO satellite has been launched and the in-orbit experiments are currently being conducted

## BDSBAS Introduction and development

    - System architecture

BDSBAS is mainly comprised of space segment, ground segment and user segment.

The space segment includes 3 Geostationary Earth Orbit (GEO) satellites which are the information channel of differential and integrity.   The differential and integrity information is injected by GUS by RF up-link. GEO satellite receive differential and integrity information  and transfer to ground user. The GEO-1 satellite (SBAS PRN 130) has been launched successfully on November 1 2018,  and the other two will be launched in 2019 and 2020, respectively. BDSBAS L1/B1C signal will compliant to the ICAO SBAS L1 standards, while BDSBAS L5/B2a will compliant to DFMC SBAS ICD which is under developing.

Frequencies and Signal of BDSBAS GEO Satellites

| FRE | Signal component | Central FRE（MHz） | MODULATION | INFO rate (bps) | POWER allocation |
| --- | --- | --- | --- | --- | --- |
| B1 | SBAS-B1C | 1575.42 | BPSK(1) | 250 | 1/3 |
| B2 | SBAS-B2a_data | 1176.45 | QPSK（10） | 250 | 1/3 |
|  | SBAS-B2a_pilot | 1176.45 |  | 0 | 1/3 |

System Architecture of BDSBAS

Signal Coverage of BDSBAS

The ground segment consists of Operation Control Centres (OCC), Data Processing Centres (DPC), Uplink Stations (US) and Monitoring Stations (MS) in China and overseas.

The Ground Segment of BDSBAS

| Station Type | Number of Stations |
| --- | --- |
| OCC | 3 |
| DPC | 3 |
| US | 3 |
| MS | 50 |

The user segment refers to SBAS terminals used in civil aviation, maritime and railway applications.

    - Data processing flow

Data Acquisition

The MS carries out continuous monitoring of all visible navigation satellites, forming pseudo-range and carrier observation data, and collecting meteorological data through meteorological equipment, and sending the data to the DPC.

Data Processing

Based on the received data, the DPC will process satellite orbit and clock , generate ionospheric delay differential information, satellite differential information (ephemeris correction and clock correction) and integrity information (UDRE, GIVE, DFRE etc. ), and send these information to the OCC.

Message Arrangement

Based on the received information, the OCC generates service messages according to the corresponding protocol, and then sends the messages to the US.

Message Injection and Broadcast

The US injects the messages to the BDSBAS GEO. The GEO broadcasts the messages to the users in China and surrounding areas.

User Calculation

User equipped with BDSBAS receiver receives messages form the GEO, uses the differential information from the messages to improve the accuracy, and uses the integrity information from the messages to calculate the protection level which is used to determine whether the service can be used or not. If the protection level is larger than the alarm threshold, the receiver will immediately send alert to the user.

    - System Performance

Service Coverage: 10°N~55°N；75°E~135°E;

Augmentation Object: BDS and GPS will be augmented. Galileo E1C/E5a and GLONASS L1/L3 will be considered if recommended by ICAO SARPs;

Service mode: Single Frequency (SF) mode using GNSS L1 signals only with augmentation on SBAS L1 as identified by DO-229 (or equivalent); Dual-Frequency Multi-Constellation (DFMC) mode using GNSS L1 and GNSS L5 signals with augmentation on SBAS L5;

Service Level: APV-I in SF mode and CAT-I in DFMC mode.

The Designed Performance of BDSBAS

| Performance |  | BDSBAS |  |
| --- | --- | --- | --- |
|  |  | SF | DFMC |
| Service Coverage |  | 10°N~55°N；75°E~135°E | 10°N~55°N；75°E~135°E |
| Augmentation Constellation |  | BDS B1C GPS L1C/A | BDS B1C/B2a GPS L1C/A/L5 GALILEO E5a/E1 GLONASS L1/L2 or L3 |
| Signal | Frequency | 1575.42MHz | 1176.45MHz |
|  | Signal Structure | SBAS L1 C/A | DFMC SBAS L5 |
| Accuracy（95%） |  | H：2.5m V：4m | H：1.5m V：2m |
| Integrity | Time To Alarm | 10s | 6s |
|  | Integrity Risk | 2×10-7/150s | 2×10-7/150s |
|  | Alarm Threshold | HAL：40m VAL：50m | HAL：40m VAL：10m |
| Continuity |  | 1-8×10-6/15s | 1-8×10-6/15s |
| Availability |  | Better than 99% | Better than 99.9% |
| Compatibility |  | SF SBAS ICD | DFMC SBAS ICD |
| User capacity |  | Unlimited |  |

    - BDSBAS Development Schedule

According to the BDSBAS construction plan, the development schedule is as follows:

Start system design and initial SF SBAS trials since 2012, and actively participate in the revision work of relevant international standards;

Start ground station construction and DFCM SBAS trials since 2015;

Launch the first GEO satellite in 2018;

Finish launching 3 GEO satellite no later than 2020, attain initial operating capability and start to provide SBAS service to users in China and its surrounding areas;

Start upgrade and approval of BDSBAS after 2020;

Reach full operational capability after 2022, and start to provide service to aviation equipment compliant to DO-229 (or equivalent).

Development Schedule of BDSBAS

## BDSBAS STATUS

    - BDS B1C on-orbit Test

The results of on-orbit test show the level of the received BDSBAS B1C signal can achieve -157dBW seen as in Fig.9.

BDSBAS B1C spectrum 1575.42MHz, BPSK(1)

### BDS B2a on-orbit Test

The results of on-orbit test show the level of the received  BDSBAS B2a signal can achieve -157dBW seen as in Fig.10.

BDSBAS B2a spectrum 1176.45MHz，QPSK(10)

### BDSBAS RF on-orbit test

The results of on-orbit test of BDSBAS RF signals show that it meets the technical specifications specified in the ICAO standards shown as Table. 4.

The results of on-orbit test of BDSBAS RF signals

| NO | item | signal | ICAO requirement | Test results |
| --- | --- | --- | --- | --- |
| 1 | Carrier FRE | B1C | 1575.42MHz | 1575.42MHz |
| 2 |  | B2a | 1176.45MHz | 1176.45MHz |
| 3 | spurious | B1C | ≤-40dBc | -56.66 |
| 4 |  | B2a | ≤-40dBc | -62.34 |
| 5 | modulation | B1C | BPSK（1），symbol rate 500sps，code length 1023，Chip rate 1.023Mchip/s | BPSK（1），symbol rate 500sps，code length 1023，Chip rate1.023Mchip/s |
| 6 |  | B2a | BPSK（10），symbol rate 500sps，code length 10230，Chip rate 10.23Mchip/s | BPSK（10），symbol rate 500sps，code length 10230，Chip rate 10.23Mchip/s |
| 7 | Phase noise | B1C/B2a | PLL of 10 Hz one-sided noise bandwidth is able to track the carrier to an accuracy of 0.1 radian | 0.00578 rad |
| 8 | spectrum | B1C | At least 95% of the broadcast power will be contained within this 3 dB bandwidth | Band wide：±18.414MHz |
| 9 |  | B2a | At least 95% of the broadcast power will be contained within this 3 dB bandwidth | Band wide：±35.805MHz |
| 10 | FRE stability | B1C | <5e-11(1s~10s) | 1.5e-12/1s 5.2e-13/10s |
| 11 |  | B2a | <6.7e-11(1s~10s) |  |
| 12 | Coherence of code & carrier | B1C | short-term：< 0.15m long-term：< 0.19m | 0.12m 0.133m |
| 13 |  | B2a | short：<0.2m long-term：<0.255m | 0.1m 0.16m |
| 14 | Coherent of L1 & L5 | B1C/B2a | short-term：<0.2m long-term：<0.255m | short：0.13m long-term：0.178m |

# Action requested of the Committee

The Committee is invited to consider the information provided in the paper and provide feedback and comments so that can update some information of BDS and BDSBAS in some related documents and be included as input to the work item on GNSS and SBAS Guidelines or Recommendations.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain