Input paper:   ENG10-3.1.16

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	X  ENG		□  PAP					X  Input

□  ENAV	□  VTS							X  Information

Agenda item 			3

Technical Domain / Task Number 2	…………………………………

Author(s) / Submitter(s)			CHINA MSA….….

# Composite Solution: Wind Driven and Reflective Bird Deterrent

# Summary

Considering the update of Guideline G1091 Bird Deterrents and Bird Fouling Solutions, for the continuously enrich and improve the guidelines’ solutions, CHINA MSA had made a series experiments on bird deterrents. This document will introduce these experiments and their effects, which may be a reference for bird deterrent solutions.

## Purpose of the document

The document aims to giving a composite solution to disperse the birds and share an experience of Bird Deterrent to enrich and improve the guideline G1091.

## Related documents

Guideline G1091 Bird Deterrents and Bird Fouling Solutions

ENG9-2.1.9 Bird deterrents and bird fouling alternative solutions

# Background

As the existence of buoys, many birds naturally choose to stay on them to rest or nest, or even leave bird fouling. Birds that stay on the buoys may block the lights of the lantern, while nests and fouling that cover the hull of the buoys and solar panels may affect the colour and energy supply of the buoys. It becomes a problem for ATONs personnel to disperse the birds, at the same time to ensure the safety of birds and protect the environment.

Yantian port, Shenzhen, China, one of the world's busiest container port, faces a similar dilemma. The bird fouling, decreases the effectiveness of ATONs, even affects the safety and efficiency of navigation. Therefore, the CHINA MSA chose Yantian port to conduct the research.

Considering, the IALA guideline G1091 Bird Deterrents and Bird Fouling Solutions, and the proposal, which had been accepted by ENG 9, ENG9-2.1.9 Bird deterrents and bird fouling alternative solutions made a good supplement for the guideline G1091. However, wind driven and reflective bird deterrent is different from the solutions mentioned in theses relevant documents, so we share these experience and effort in this proposal.

# Discussion

CHINA MSA conducted a series of trials on bird deterrents in Yantian port, specific information will be shown as follow:

## Birds impacts on ATONs in Yantian port

Shenzhen, Yantian port is a top ten container port in the world, with large ships sailing in the busy channel. Safety and efficiency have significant meanings for this world-class port. At the same time, Yantian port is rich in species and quantity of birds due to its excellent natural environment by mountains and sea. Therefore, the ATONs in the port is often disturbed by birds, which has a great negative impact on the navigation safety and efficiency.

Abundant birds in Yantian port

Birds and Bird Fouling

Bird fouling is difficult to clean and corrode easily

## A variety of solutions

In order to solve the negative impact of birds, a variety of methods had been tried but the results are not ideal.

- Bird spikes

Bird spikes

Spring and steel spikes are used to make birds unable to stand, but the protection range is small. And it may interfere the maintain work of ATONs.

Ultrasonic Bird Deterrent

Ultrasonic Bird Deterrent

- Release ultrasound to stimulate the nervous system of birds, destroy the living environment of birds, large size, high energy consumption, inconvenient installation.

Laser Bird Deterrent

Laser Bird Deterrent

It emits a dazzling green laser, which scares the birds away. The price is high and the range is small.

These methods have disadvantages such as small protection range or harm to birds. Due to these facts, wind driven and reflective bird deterrent was taken into consideration.

## Wind Driven and Reflective Bird Deterrent

Wind Driven and Reflective Bird Deterrent combine both physical and electronic approaches.

The physical way relies on wind to drive the rotation. The steel rod can interfere bird standing. As the habit of birds being afraid of light, retroreflector can scare birds away.

The electronic way uses an audible bird deterrent with solar integrated electronic generator to scare away birds, even without wind.

This device is self-sufficient in energy consumption, it does not need extra energy and very environmentally friendly.

- Wind Driven and Reflective Bird Deterrent Structure

Basic Structure

Steel Rod and Audible bird deterrent

Audible bird deterrent

Application

- Wind Driven and Reflective Bird Deterrent began to be widely installed and applied in the buoys of Yantian port, and achieved good results.

Effect of Wind Driven and Reflective Bird Deterrent

## Results

Through experiments, the following characteristics of Wind Driven and Reflective Bird Deterrent are summarized:

- Material: the material is anti-corrosion and wear-resistant, which can well adapt to the working environment at sea.

- Energy: it can be driven by the breeze, and rotate speed could be 15-20 r/min. It does not need extra energy and very environmentally friendly.

- Maintenance: the equipment can be easily installed, no maintenance is needed and the service life can reach 3 years

- Effects: after installing retroreflector, steel rod and audible bird deterrent, can reach a 100% coverage for lantern and solar panel, and 80% coverage for the hull of a buoy.

# References

Guideline G1091 Bird Deterrents and Bird Fouling Solutions

ENG9-2.1.9 Bird deterrents and bird fouling alternative solutions

# Action requested of the Committee

The committee is invited to note the solution of Wind Driven and Reflective Bird Deterrent, and take action as appropriate.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain