Input paper:   ENG10-3.1.17

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	X  ENG		□  PAP					□  Input

□  ENAV	□  VTS							X  Information

Agenda item 			3

Technical Domain / Task Number 2	…………………………………

Author(s) / Submitter(s)			CHINA MSA…..….

# Transformation of PRB-21 Lantern

# Summary

According to the guideline G1049 Use of Modern Light Sources in Traditional Lighthouses Optics, China MSA made an experiment on Mulantou Lighthouse, a traditional lighthouse for more 30 years old which located on the south part of China, in order to adapt the internal working environment and optical device requirements of the lighthouse, the PRB-21 lantern was partially modified and optimized for a better performance. This documents will give  a briefly introduction about the work has been done by China MSA.

## Purpose of the document

The committee is invited to note the information.

## Related documents

G1049 Use of Modern Light Sources in Traditional Lighthouses Optics

# Background

PRB-21 large-scale rotary lantern is one kind of excellent lantern. Its light has a long rang which could be more than 26 nautical miles, moreover it works very stably. So it was installed in some of the large-scale lighthouse of China in the 1990s. It played an important role in navigational safety, It is also an important historical and cultural composition of the navigation mark.

After the duty of near 30 years, some part of these lanterns are broken and cannot be fixed, most of them are facing the problem of replacement. In order to solve these problems, also protect the appearance and the efficiency of traditional lighthouses, some transformation has been made for PRB-21 Lantern, including using LED light array which consume less energy to replace the vacuum spotlights array, developing a smart controller for PRB-21 lantern, using a conductive slip ring instead of mercury to conduct electricity etc.

# Discussion

## The problems of the aged PRB-21 lantern

High energy consumption

- The light array of PRB-21 lantern uses 48 vacuum spotlights of 200W30V, of which 8 vacuum spotlights are connected in series and 6 series are connected in parallel. The emergency light array uses 16 vacuum spotlights of 28.5W6V, of which 4 vacuum spotlights are connected in series and 4 series are connected in parallel. The overall power reaches 10KW, which consume so much energy. When the municipal electricity cannot be provided, diesel generator power generation was used to generate power, but the use of diesel may bring some pollution to the environment.

Rotating platform is conducted by mercury, which are very dangerous

- The PRB-21 lantern uses a mercury tank to conduct power for rotating. But mercury needs to be removed out and then put back after cleaning. The cleaning is very difficult and dangerous. Because mercury is toxic and volatile, the safety of the stuff and environment may be affected.

Cannot be remote controlled

- The PRB- 21 lantern cannot meet the need of remote controlled, so the staff needs to be on duty 24 hours, which is very expensive. Moreover efficiency won’t be high.

The structure is complex, which is hard to maintain

- PRB-21 lantern is composed with a control box (size of 650 mmX460mmX230mm), a transformer box (size of 500 mmX500mmX400mm), a distribution box and an anti-surge protector box (size of 300mmX300mmXx200mm), which makes the whole system and the wiring very complicated. Therefore the maintenance needs to be done by specialist.

## Transformation of PRB-21 lantern PRB-21

- Use LED light array which consume less energy to replace the vacuum spotlights array

- We developed LED light array and the matching flat-panel Fresnel lenses. The newly developed LED light array has a vertical divergence angle of 1.4 degrees and has a range of more than 25 nautical miles, which meets requirements and can completely replace the original vacuum spotlights array. Moreover the power consumption is only 460W, which is 4.6% of the original one. LED light array can use DC 24 to 36V power supply and be powered by solar cells, which is conducive to energy saving.

Figure 1   LED light array

Figure 2   LED light array

- Develop a smart controller for PRB-21 lantern

- We developed a smart controller for PRB-21 lantern, which enable the PRB-21 lantern device to be controlled remotely. The smart controller can control the rotation of the lantern, the switching state of the light, as well as the detection of the operating voltage, current, etc. Moreover the smart controller can use GPS, GPRS, Beidou and other means to communicate with the remote server. Such made management and maintenance done remotely and intensively.

- Rotating platform is conducted by a conductive slip ring instead of mercury

- Rotating platform is conducted by a conductive slip ring instead of mercury. The conductive slide ring uses the sliding contact of the conductive ring to conduct electricity between the rotating part of the fixed seat frame and the sliding part. This way to conduct electricity is much more safe to lighthouse keepers and environment.

## Effect

- In 2017, we had transformed two PRB-21 lanterns in Mulantou lighthouse and Jinmujiao lighthouse. They work reliably and they are easy to maintain. And the range can be more than 25 nautical miles, which prove that the performance of them excellent.

- Figure 3   Effect after the transformation

# References

G1049 Use of Modern Light Sources in Traditional Lighthouses Optics

# Action requested of the Committee

The Committee is requested to note the information and take action as appropriate

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain