Input paper: ENG10-3.1.19

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM		X  ENG		□  PAP						X  Input

□  ENAV	□  VTS								□   Information

Agenda item 					3

Workplan Task Number / Technical Domain 2	2.3.3

Working Group					WG2

Author(s) / Submitter(s)				Eng. Mariano Luis Marpegan / Hidrovia S.A.

# Input Paper for Draft Guideline on Tidal Current Signal System

# Summary

## Purpose of the document

This document has been made with the aim of adding historical antecedents with reference to the communication systems implemented in port placed in different regions to provide the sailor with tidal information.

## Related documents

ENG8-10.5 Develop Guideline on Tidal Flow Data Capture and Display and its development in ENG9-9.7 WP WG2 Draft Guideline on tidal flow/current signal system_V0.5 (ENG8-12.2.15).

# BACKGROUND

## Introduction to Tidal Current/Flow

This paper was created in accordance with the development of the related documents mentioned before. It reflects the antecedents, evolution and how significant it is for the sailor to know the port tidal condition and height, considered as determinant variables that guarantee safety during the navigation.

In the past, radio or internet systems were not available to send information about the tidal current and height to the sailor, the so-called maritime or fluvial signals were used instead. These signals were formed by means of a structure with circles and flags hanging from it and flashing lights which served to indicate the sailors the tidal condition and height in their approximation to the port.

Thus, sailors could take the necessary precautions to navigate safely knowing beforehand the tidal current and height to avoid accidents such as strandings, collisions, among others (of course, with the resulting economic loss).

## Antecedents 1 - Argentina, South America

Indications were performed with black flags and circles during the day, while during the night, the task was accomplished with red, green and white flashing lights.

Night signaling to indicate the tidal condition:

- Rising: by a long white flash

- Falling: by two short white flashes

- Red flashes were needed to indicate that the river has risen over ‘zero,’: a long red flash was emitted every each meter and a short flash, every ten centimeters.

- Green flashes were needed to indicate that the water height was lower than ‘zero:’ a long green flash was emitted every two meters and a short Green flash every ten centimeters;

- Red and green short flashes were used to indicate that the river was in its stationary phase (levelled);

Long flashes lasted 2.5 seconds whereas the short ones lasted 0.8 seconds.

*UL stands for Upper Level and LL stands for lower level.

Figure 1 Example of tidal condition signaling, Argentina.

## Antecedents 2 – France, Europe

Currrently, some European ports are still providing sailors with information about the tidal condition and height as follows:

On the left part of the figure the tidal condition is indicated with a flag code during the day and a code based in flashes of light during nighttime:

- Falling tide indication suggests a green flag during daytime or two green lights in horizontal position during nighttime

- Rising tide indication suggests a black cone with its apex upwards during daytime and with a green light over a white light during nighttime

- During the high tide stationary phase, indication suggests a blue cross over a white base in daytime and two white lights in horizontal position at nighttime.

- Finally, during the low tide stationary phase, indication suggests a black cone with its apex downwards in daytime and a white light over a green base at nighttime.

As shown on the right part of the figure, tidal height is expressed by means of a code of symbols similar to Roman numbers:

- A black cone is used to show 20 centimeters, a black rectangle to show 1 meter and a black circle to show 5 meters. In case they are repeated, there should be an addition between the values of each signal.

- Signals are used during the day but during the night they are all replaced: the cone is replaced by a green light, the rectangle by a red light and the circle, by a white one.

Figure 2 Example of tidal condition signaling, France.

Considering the development of certain ports and the service they offer in relation to the safety in navigation, several systems have been updated during the last decades. Nowadays there exist several ports which have in their entrances a tidal current data capture and display showing the tidal condition and height at every moment.

# Actions requested from the Committee

The Committee is requested to:

- Review this document;

- Consider it to be included as a background in the Draft Guideline on Tidal Current Signal System.

Input document number to be assigned by the Committee Secretary.

Input papers should be assigned to a work task as listed in the Committee work plan which is available in input papers. Leave open if uncertain but consider how the paper is to be processed if not relevant to a work task.