Input paper:   ENG10-3.1.23

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM		  ENG		□  PAP						  Input

□  ENAV	□  VTS								□  Information

Agenda item 	(from agenda)			3

Workplan Task Number / Technical Domain 2	Floating AtoN

Working Group					WG 1 AtoN Design and Delivery

Author(s) / Submitter(s)	              Prof. Dr. Seung-Gi, GUG of Korea Maritime and Ocean

University

# The Development of Auto Releasing New Danger Mark Buoy

# Summary

According to the IALA NAVGUIDE, the term “New Danger” is defined as newly discovered hazards which are not yet shown in nautical documents and includes naturally occurring obstructions such as sandbanks or rocks or man-made dangers such as wrecks. ‘New Dangers’ should be appropriately marked using Lateral, Cardinal, Isolated Danger marks or by using the Emergency Wreck Marking Buoy. However, the Emergency Wreck Marking Buoy has difficulties in implementation, especially with the accuracy and time required for the installation. In case of sudden accidents and sinking, it is difficult to install the emergency wreck marking buoy immediately, due to unpredictable weather condition, surrounding marine environment and the position of accident. This paper summarizes an auto releasing new danger mark buoy that will be installed on deck and can be released automatically from the deck of a vessel to float on water and quickly mark the position/angle of accident in case of sudden accidents or sinkages. The buoy will be used to reduce the risk of navigation and to prevent secondary collisions.

## Purpose of the document

This document aims to develop a ‘New Danger Marks’ based on Maritime Buoyage System (MBS). It is not only expected to prevent a secondary accident and assist a prompt Search and Rescue(SAR) but this system give a chance to saving cost, time and labour by skipping some indispensable procedures like installing a new buoy on the site where accident occurred. Furthermore it would be applicable to the related SAR sector such as sinking vessel or aviation on the ocean.

## Related documents

- IALA Guideline No. 1046 Response Plan for the Marking of New Wrecks (2005)

- IALA Recommendation O-133 Emergency Wreck Marking Buoy (2005)

- R1015 Marking of Hazardous Wrecks (2017)

- IALA Maritime Buoyage System (2018)

- IALA NAVGUIDE (2018)

# Background

According to the dominant and international operation situation concerning Wreck Marking Buoy it is difficult to deal with prompt response recognizing the accurate position especially an accident occurred at specific sea area. In establishing and demolishing cases it is difficult to install the Wreck Marking Buoy right after the port authorities have received wreck position in that it requires majority of work force and specific equipment(ex: crane) which’s gross tonnage is more than 500kg.

# Discussion

The Auto Release Danger Mark Buoy is consist of ① Auto release, ② Auto Reel Mooring Rope and ③ Auto Lighting Lantern technique. This is located on upper deck and released automatically under a certain hydraulic condition so that it could exactly indicate the position where accident happened (Fig. 1).

Fig. 1 Principle of Auto Releasing New Danger Mark Buoy

# 3.1 aUTO RELEASE EQUIPMENT FOR AUTO BUOYANCE

The Auto Releasing New Danger Mark Buoy also designed to prevent secondary accidents in the area and enable immediate search and rescue (SAR) by applying a technology that indicates the location of the accident by Auto Buoyance after a small new danger mark buoy attached to the vessel in the event of a missing, capsized, or sinking etc. is automatically released by certain water pressure.

New Danger Mark Buoy is always located on the upper deck fastened with a belt. When an accident occurred, the Buoy automatically released on 1.5m~4m water pressure. The built-in electronic control system that called solenoid escape device is designed to automatically open the lock on the buoy to release. Finally it float automatically (Fig. 2).

Fig. 2 Solenoid Escape Device in Auto Release Unit

# 3.2 Auto reel mooring Rope

When installing an Auto Reel mooring Rope some key information such as a size of own ship and main course should be considered. The end of an each mooring rope is connected to a buoy and a vessel. Then mooring rope is secured in a box so that it works up to a depth. This mooring rope is automatically fixed when a buoy reaches on the sea surface. At the bottom of the buoy's device, the space for mooring rope to connect the buoy with the hull was secured up to 200 meters, and for ships that sailing more than 200m depth, a separate mooring rope storage space was secured in the space next to the buoy unit, which could be used for any depth of water.

Auto reel adjust the length of mooring rope. After a vessel is sunk a wrapped mooring rope is released by the gravity applied on weight furnished in the unit.  By reaching under the sea surface, the force is made upward when the gravity and buoyancy on a weight becomes equivalent. Then an axis hanged on a pulley has fastened the mooring rope (Fig. 3).

Fig. 3 Auto Reel Mooring Rope

# 3.3 auto lighting lantern

The installed wreck marking buoy should indicate a position of the buoy and accident so that it could be easily recognized by bare eyes whether in day time or night. Auto Lighting Lantern is structured as watertight and it automatically ignites when a buoy escapes. In terms of supplying an energy to the lantern by solar heat saved on the solar panel, it would be used during the installing and operational period (Fig. 4).

Considering Auto Lighting Lantern should be sustainably escaped from a vessel and should work, it has to maintain a stable temperature through a radiator by emitting a heat. It needs an additional consideration on a requirement such as corrosion resistance test, temperature rebound test, strength test etc.

Fig. 4 Auto Lighting Lantern (Yellow & Blue)

# 3.4 information transmission system for sinking ship using ais sart and wcdma

AIS SART was automatically required to transmit distress signals to nearby ships and coastal stations as the buoy appears on the water surface. To display and alarm the distress location on the ECDIS, MMSI and location information of the ship were sent at least once a minute. (Fig. 5)

Fig. 5 MMSI & Location Information of the ship were Displayed on AIS Receiver and ECDIS

Since WCDMA has 100km distance range, it is possible for ships to send location, voice message and distress signals at a distance of about 100km from the coast. Install the WCDMA on the lantern so that when a ship is sunk, the buoy can automatically send the distress signals, location and ship’s name to Coast Guard at least once a minute. (Fig. 6)

Fig. 6 Location, Ship's name and time Information of the ship were displayed on cell phone

# 3.5 Result of Field Test

The development of Auto Releasing New Danger Mark Buoy have been completed (Fig. 7). And field test have been conducted at Busan Port (35-04.86N, 129-05.1483E) on 10:00 ~ 13:00 (Thu) 27th Dec. 2018 (Fig. 8). The depth was 11.7m. The weather was 2 ℃, wind direction NE, wind speed 1.6 m/s, and 1024.2 hpa. Upon entering the water, buoy and mooring rope was automatically released. After the auto reel mooring rope box touched the sea floor, the mooring rope was fixed. Field test was successful (Fig. 9 and Fig. 10).

Fig. 7 Auto Releasing New Danger Mark Buoy

Fig. 8 Location of Field Test

Fig. 9 Field Test (1)

Fig. 10 Field Test (2)

# 3.6 Case of installation

An Auto Releasing New Danger Mark Buoy has been installed on the radioactive waste carrier, the cheongjeong-Nuri (fig. 11). The specifications of Cheongjeong-Nuri are as follows. LOA 78.6m, Beam 15.8m, Draft 4.0m, GT 2,600 and Nav. Full Speed 12.0kts. (fig. 12)

Fig. 11 Case of Installation on the cheongjeong-Nuri

Fig. 12 The radioactive waste carrier, Cheongjeong-Nuri

# References

International Association of Marine Aids to Navigation and Lighthouse Authorities (2018), IALA NAVGUIDE 2018

International Association of Marine Aids to Navigation and Lighthouse Authorities (2018), IALA NAVGUIDE 2018, MARITIME BUOYAGE SYSTEM

International Association of Marine Aids to Navigation and Lighthouse Authorities (2015), IALA Guideline 1046 A Response Plan for the Marking of New Wreck

International Association of Marine Aids to Navigation and Lighthouse Authorities (2015), IALA Recommendation O-133 Emergency Wreck Marking Buoy

Ministry of Oceans and Fisheries (2013), A study on the basic and working design for the mooring and special buoy.

Ministry of Oceans and Fisheries (2015), Aids to Navigation Act

# Action requested of the Committee

The Committee is requested to:

- Note the information within this paper.

Discuss the matter at an appropriate time and

Include the proposed into the IALA documentation if so considered by the discussion

Input document number, to be assigned by the Committee Secretary

Input papers should be assigned to a work task as listed in the Committee work plan which is available in  input papers. Leave open if uncertain but consider how the paper is to be processed if not relevant to a work task