IALA Guideline

G1133

Determination of Required Luminous Intensity For Marine Signal Lights

Edition 1.0

Document date

Revisions to this IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |
| month/year approved by Council | aaaaa | aaaaaa |

1	INTRODUCTION	5

2	Approach	5

2.1	Replacing Existing Lights	5

3	Determining AtoN Light Intensity Requirement	6

3.1	Maximum Required Intensity	7

3.1.1	Minimum Distance of Observation	7

3.1.2	Maximum Meteorological Visibility	7

3.1.3	Maximum Illuminance for Glare	7

3.1.4	Calculating the Maximum Required Intensity	7

3.2	Minimum Required Intensity	8

3.2.1	Maximum Distance of Observation	8

3.2.2	Minimum Meteorological Visibility	8

3.2.3	Minimum Illuminance at the Observer	8

3.2.4	Calculating the Minimum Required Intensity	11

3.3	Selecting a Design Intensity	11

3.3.1	Nominal Range for Publishing	11

4	Implementing AtoN Light Intensity Requirement	12

4.1	De-rating Factors due to Implementation	12

4.1.1	Service Condition Factors	12

4.1.2	Astragal and Glazing Loss Factor	15

4.1.3	Filter Material Factor	17

4.2	Specification Effective Intensity	17

4.3	Specification Peak Intensity	18

5	Examples	18

5.1	Example 1	18

5.1.1	Minimum Distance of Observation	19

5.1.2	Maximum Meteorological Visibility	19

5.1.3	Maximum Illuminance for Glare	19

5.1.4	Calculating the Maximum Required Intensity	19

5.1.5	Maximum Distance of Observation	19

5.1.6	Minimum Meteorological Visibility	19

5.1.7	Minimum Illuminance at the Observer	19

5.1.8	Calculating the Minimum Required Intensity	19

5.1.9	Selecting a Design Intensity	19

List of Tables

Table 1	Maximum illuminance at the eye of the observer	7

Table 2	Minimum illuminance to use with different background luminance	9

Table 3	Required illuminance under different daytime conditions	9

Table 4	Typical rival lights intensities	10

Table 5	Typical filter material factors for LED and Tungsten Halogen lamps	17

List of Figures

Figure 1	Process of determining the required luminous intensity from the AtoN and light-emitting apparatus.	6

Figure 2	Example of background luminance and rival lights during AtoN light design	8

Figure 3	Example of a lumen degradation of a light source with hours of operation.	13

Figure 4	Demonstration of astragal obscuration on the total light output from a fixed optic.	15

List of Equations

Equation 1	Application of Allard’s Law for calculating the maximum required intensity	7

Equation 2	Minimum illuminance at the eye of the observer from background luminance	9

Equation 3	Illuminance at the eye of the observer caused by rival lights	10

Equation 4	Application of Allard’s Law for calculating the minimum required intensity	11

Equation 5	Application of Allard’s Law for calculating the nominal range	11

Equation 6	Calculation of lumen degradation due to failure within the light source	13

Equation 7	Calculation of lumen degradation due to failure within the light source	14

Equation 8	Calculation of lumen degradation due to failure within the light source using intensity	14

Equation 9	Calculating the Service Condition Factor	14

Equation 10	Obscured area of a fixed optic light due to astragals	16

Equation 11	Estimation of the loss due to astragal obscuration	16

Equation 12	Glazing and Astragal Loss Factor	16

Equation 13	Calculation of the Specification Effective Intensity	17

Equation 14	Specification Peak Intensity	18

Equation 15	Application of Blondel-Rey equation for rectangular flash profiles	18

# INTRODUCTION

A suitable luminous intensity is one of the basic features of an AtoN light. The intensity should be sufficient to ensure that the light is visible for navigation as required. It should not be too intense in order to avoid glare.

The purchasing and operating costs are strongly influenced by the luminous intensity of the AtoN light. A 10 cd light can be realised by a low cost and small lantern immediately available from stock of a manufacturer. A sector light of 1 000 000 cd will be high cost, require a special design and will be produced for a single application.

This Guideline should enable the reader to determine the light intensity to provide a good service to the user in a given area, whilst maintaining a balance between performance and cost.

# Approach

There may be several approaches to designing an AtoN light that is suitable to take into account the local conditions around the AtoN. The approach taken in this Guideline attempts to capture most, if not all, the issues surround the design of the light.

The approach is broken into two major sections: AtoN light intensity requirement and AtoN light intensity implementation. These two sections deal with two different aspects of ensuring that the AtoN light performs as the user expects.

The AtoN light intensity requirement determines what light intensity is required from the AtoN, given local conditions surrounding the light and the user. This section will result in the light intensity that is officially published in the List of Lights and nautical charts.

However, there is often additional factors that need to be taken into account that deal with the implementation of the required intensity. These are largely due to design decisions at the AtoN itself that will ultimately impact the intensity produced by the AtoN. This second section will help ensure that whatever solution is placed at the AtoN, it will produce the required light intensity, and therefore meet the navigational requirements.

The overall process is shown in Figure 1. This figures shows how the different aspects of design come together, and they will be dealt with separately in the sections below. It requires the designer to have determined what the “useable range or area” of the light should be, i.e. over what distances is the light meant to be observed. This will be based on risk analysis of traffic and hazards in the area of interest, and what signal would be required to help the user to navigate the area.

## Replacing Existing Lights

The approach assumes that a new light is being installed. It can also be used to confirm that an existing light is suitable for the area concerned.

In the event that an existing AtoN light is being renewed, it is possible to determine the design intensity from the published nominal range of the light. XXX TBC XXX

Process of determining the required luminous intensity from the AtoN and light-emitting apparatus.

# Determining AtoN Light Intensity Requirement

In this section, we will determine the light intensity required from the AtoN that will provide the level of service that is expected in a given area. The light intensity is chosen from a range of intensities between a minimum and a maximum intensity. The process determines what these minimum and maximum intensities are, and then the user selects an intensity that meets those requirements.

We shall consider the minimum and maximum intensity selection separately.

## Maximum Required Intensity

The maximum required intensity, Imax, is the maximum intensity that the AtoN light can be. It is determined by three factors that are considered below.

### Minimum Distance of Observation

The minimum distance of observation, dmin, is the distance from the AtoN to the closest point of the useable range or area given in metres. This can be determined easily from a map or chart of the area concerned.

### Maximum Meteorological Visibility

The maximum meteorological visibility, Vmax, is the best possible visibility in the local area given in metres. This value, along with the minimum meteorological visibility detailed below, may be difficult to determine without some data to provide an indication of what value it may be. A good source of such information is a local meteorological organisation that may be able to provide visibility data covering the area of interest. Should such data be available, it may be possible to analyse it to determine the level of visibility to a confidence level. For example, the maximum visibility in a given area is 30 km for 95% of the time – in other words, the maximum visibility is higher than that for 5% of the time, but it is considered rare enough for these calculations.

In the absence of any meteorological data, use Vmax=20 M = 37040 m.

### Maximum Illuminance for Glare

If the light intensity is too high and/or the observer is very close, then it can cause glare, resulting in uncomfortable viewing for the user. Therefore, it is necessary to determine what the maximum illuminance at the eye of the observer, Emax, that is acceptable given the conditions for the user given in lux. Two values are given for this, depending on whether there is background luminance, as shown in Table 1.

Maximum illuminance at the eye of the observer

| Background luminance | Emax (lux) |
| --- | --- |
| None (dark) | 0.01 |
| present | 0.1 |

### Calculating the Maximum Required Intensity

In order to calculate the maximum required intensity, all the factors above are used in Allard’s Law as described in Recommendation R0202 [1]. Thus,

Imax=Emax∙dmin2∙0.05-dminVmax

Application of Allard’s Law for calculating the maximum required intensity

Where:

Imax is the maximum required intensity (cd)

Emax is the maximum allowed illuminance at the eye of the observer (lux) (see Section 3.1.3)

dmin is the distance of closest point of the useable range to the AtoN light (m) (see Section 3.1.1)

Vmax is the maximum meteorological visibility in the area of interest (m) (see Section 3.1.2)

## Minimum Required Intensity

The minimum required intensity, Imin, is the intensity that the AtoN light must provide to ensure that it can be seen throughout the useable range or area in less optimal conditions. It is determined by a number of factors which are described in detail below.

### Maximum Distance of Observation

The maximum distance of observation, dmax, is the distance from the AtoN to the farthest point of the useable range or area given in metres. This can be determined easily from a map or chart of the area concerned.

### Minimum Meteorological Visibility

The minimum meteorological visibility, Vmin, is the worst possible visibility to be considered in the local area given in metres. As discussed in Section 3.1.2, it may be difficult to determine without a source of data to confirm typical visibility in poor weather in the area. If such information is available, then as for the maximum meteorological visibility, a statistical approach can be taken so that the visibility guaranteed for, say, 75% of the time is used.

For example, the area of interest may have a visibility of 8 km for 90% of the time. So, for 10% of the time, the visibility is worse than 8 km. However, in deciding the level of performance expected from the AtoN light, the designer may decide that this is acceptable, and therefore use 8 km for the value of Vmin.

That said, care must be taken with this approach since it requires a balance between ensuring a good performance from the light in poor weather conditions and the intensity of the light. As the value of Vmin lowers, the minimum required intensity increases exponentially, and may end up with an intensity greater than the maximum required intensity. This would result in a light that is unnecessarily intense and difficult to observe at close range due to glare.

If no meteorological visibility data is available, then the designer must choose a value that is deemed to be acceptable to the area under consideration. This may require engaging with local users and stakeholders to determine what level of service is needed. Note that a higher value of Vmin could result in an AtoN light that cannot be seen at the furthest point of the useable range or area under worsening weather conditions.

### Minimum Illuminance at the Observer

The minimum illuminance at the observer, Emin, is dependent on the amount of light that is seen behind the AtoN light from the direction of the useable range or area. This background luminance, if sufficiently high, can cause the AtoN light to blend into the background and not be conspicuous. Therefore, the AtoN light must have sufficient intensity to contrast against such a background. An example of a scenario is shown in Figure 2.

background luminance

rival

lights

AtoN being designed

Buoys (potential rival lights)

Example of background luminance and rival lights during AtoN light design

Table 2 shows a list of minimum illuminance at the observer required under different circumstances. The designer should, if no measurements are available, to select the most appropriate situation for the AtoN light being designed. If the background luminance has been measured, then Equation 2 should be used to calculate Emin directly.

Minimum illuminance to use with different background luminance

| Background Condition | Lights | Emin (μlux) |
| --- | --- | --- |
| Lights for night-time use |  |  |
| - no background luminance | all lights except leading lights | 0.2 |
| - no background luminance | leading lights | 1 |
| - minor background luminance | all lights | 2 |
| - substantial background luminance | all lights | 20 |
| Lights for daytime use | all lights | 1000 |

#### Known Background Luminance

With a known, possibly measured, background luminance, Lb, it is possible to calculate the minimum illuminance at the eye of the observer by applying Equation 2.

Emin=0.242×10-6×1+0.4*Lb2

Minimum illuminance at the eye of the observer from background luminance

Where:

Emin is the minimum illuminance at the eye of the observer (lux)

Lb is the measured background luminance (cd/m2)

Equation 2 does not give the same value for Emin when Lb=0 as Table 2, and therefore, under completely dark background luminance conditions, the value of 0.2 μlux should be used (or 1 μlux for leading lines).

This equation is useful for determining the daytime performance of the light under different meteorological conditions, as shown in Table 3. If typical meteorological conditions can be cited for the location of the AtoN, then this table can be used to ensure that the light intensity is sufficient to be seen in the useable area of range during those conditions.

Required illuminance under different daytime conditions

| Meteorological Condition | Luminance, Lb (cd/m2) | Required Illuminance, Emin (μlux) |
| --- | --- | --- |
| Very dark overcast sky | 100 | 13 |
| Dark overcast sky | 200 | 24 |
| Ordinary overcast sky | 1000 | 107 |
| Bright overcast sky or clear sky away from the direction of the sun | 5000 | 506 |
| Bright cloud or clear sky close to the direction of the sun | 10000 | 1000 |
| Very bright cloud | 20000 | 1980 |
| Glaring cloud | 50000 | 4910 |

#### Rival Lights

Rival lights are sources of point source lights in the area of observation around the AtoN light. If an excessive number of lights exist, or they are of higher intensity than the AtoN light, then it can make the AtoN light difficult to discern and must less conspicuous. Such an example is shown in Figure 2. Therefore, the minimum illuminance created by the AtoN light should be equal to or greater than the illuminance created by the rival light.

As a general rule, it is possible to apply Allard’s Law to establish the illuminance created by a rival light. Thus,

Erival=Irivaldrival2∙0.05-drivalVmin

Illuminance at the eye of the observer caused by rival lights

Where:

Erival is the illuminance at the eye of the observer caused by rival light (lux)

Irival is the intensity of the rival light (cd)

drival is the distance of farthest point of the useable range to the rival light (m)

Vmin is the minimum meteorological visibility in the area of interest (m) (see Section 3.2.2)

Equation 3 must be applied to all major rival light sources in the vicinity of the AtoN light to ensure that the most prominent light source is captured. It will be necessary to determine the distance of the rival light to the furthest point of the useable range or area, and this should be possible using a map of the local area. The intensity of the rival light is more difficult to establish. However, as a guideline to possible intensity values, Table 4 contains some typical intensities for a number of potential rival light sources. The field-of-view to consider rival lights should be 2 degrees when viewing the AtoN from the furthest point in useable range of the AtoN.

Once Erival has been calculated for all rival lights, the highest value is selected and compared to Emin determined above. If Erival is greater than Emin, then Emin should be revised to be equal to or greater than Erival. The greater the value of Emin above Erival, the more conspicuous the AtoN light will be in comparison to the rival light.

Typical rival lights intensities

| Purpose | Type | Luminous intensity (cd) | Characteristics |
| --- | --- | --- | --- |
| Aeronautical obstacle lights | Low-intensity, Type A, red, fixed | 10 | omnidirectional |
|  | Low-intensity, Type B, red, fixed | 30 | omnidirectional |
|  | Medium intensity, Type A, white, flashing | 2000 | omnidirectional |
|  | Medium intensity, Type B, red, flashing | 2000 | omnidirectional |
| Road traffic lights | Green, Red, Yellow | 25 to 200 | pencil beam |
| Vessel Navigation Lights (Length <12 m) | White | 4.3 | Sectored |
|  | Red, Green | 0.9 | Sectored |
| Vessel Navigation Lights (Length between 12 and 50 m) | White | 12 to 52 | Sectored |
|  | Red, Green | 4.3 | Sectored |
| Vessel Navigation Lights (Length >50 m) | White | 94 | Sectored |
|  | Red, Green | 12 | Sectored |

### Calculating the Minimum Required Intensity

In order to calculate the minimum required intensity, all the factors above are used in Allard’s Law as described in Recommendation R0202 [1]. Thus,

Imin=Emin∙dmax2∙0.05-dmaxVmin

Application of Allard’s Law for calculating the minimum required intensity

Where:

Imin is the minimum required intensity (cd)

Emin is the minimum allowed illuminance at the eye of the observer (lux) (see Section 3.2.3)

dmax is the distance of farthest point of the useable range to the AtoN light (m) (see Section 3.2.1)

Vmin is the minimum meteorological visibility in the area of interest (m) (see Section 3.2.2)

## Selecting a Design Intensity

The design intensity, Idsg, is an intensity chosen by the designer to sit between the minimum and maximum required intensities calculated above. Thus,

Imin≤Idsg≤Imax

In many cases, the design intensity will be made equal to the minimum required intensity. However, there may be operational reasons that a designer will want to choose a higher design intensity (e.g. a desire to be more conspicuous than usual in a given area), in which case the designer can chose a value higher than the minimum (but not more than the maximum required intensity).

For the rest of this Guideline, the design intensity will be used for subsequent calculations.

### Nominal Range for Publishing

The design intensity is the intensity of light exhibited from the AtoN. As such, it is this value on which the published nominal range should be based upon. The calculation of the nominal range should be based on the Allard’s Law using a number of pre-defined conditions.

Idsg=Enominal∙dnominal2∙0.05-dnominal18520

Application of Allard’s Law for calculating the nominal range

Where:

Idsg is the design intensity (cd)

Enominal is the illuminance at the eye of the observer under nominal conditions (lux).

For daytime, Enominal=1000 μlux.

For night-time, Enominal=0.2 μlux.

dnominal is the nominal range of the AtoN light (m)

The nominal range is given in metres, and will need to be converted to nautical miles by dividing the result by 1852 (e.g. 5000 m = 2.7 M). The nominal range also assumes a meteorological visibility of 10 M (= 18520 m). Note that the values for the illuminance of the eye only has two values: one for night-time and one for daytime. These are the only values that should be used, regardless of what values were used in calculating the design intensity.

Unfortunately, it is not possible to calculate dnominal directly by rearranging Equation 5. It must be determined by other means. The most popular method is to use a reverse lookup table where the intensity for different ranges is calculated. It is also possible to use an iterative process where the range is updated between each calculation to get the value of the intensity closer to the one required. Each method have their pros and cons, and it is up to the reader to determine the best solution for their application. A quick Excel function can be used, as shown below, to calculate the nominal range using the design intensity in cell B1 (in cd) and the value for Enominal in cell C1 (in lux):

=INDEX(ROW($A$1:$A$4000)*0.01,MATCH($B$1,$C$1*((ROW($A$1:$A$4000)*18.52)^2)*0.05^(-ROW($A$1:$A$4000)*18.52/18520),1))

This formula will give the nominal range with a resolution of 0.01 M up to 40 M, but it must be entered as an array formula, i.e. press CTRL-SHIFT-ENTER instead of the usual ENTER to enter the formula into the cell. Although the range $A$1:$A$4000 is referred to in the function, the contents of those cells are not used.

# Implementing AtoN Light Intensity Requirement

In the previous Chapter, the process of determining the design intensity was discussed. This intensity is the light output from the AtoN, and therefore is the value that the published nominal range is based on.

However, there are a number of issues that affect the light output of the light-emitting apparatus and what is output from the AtoN. These issues need to be taken into account so that the equipment of the correct specification can be determined. Failure to do this will cause the AtoN light to fail to meet its published performance, and a degraded service.

## De-rating Factors due to Implementation

In this section, we will apply several de-rating factors to the design intensity to determine what the light-emitting apparatus should be performing at. This will be the specification intensity, so called because it should be the intensity specified by authorities when procuring light-emitting apparatus for the AtoN light under consideration.

The designer should consider each of the issues given below, and determine what value should apply for each de-rating factor. These de-rating factors will then be applied to the design intensity to determine the specification intensities.

### Service Condition Factors

The service condition factor is a means of allowing for the light output from an AtoN to degrade and continue to perform as published. It should capture all effects that are under the control of the operating authority through maintenance. The effects of dirt, dust, optical degradation of materials and light source degradation should all be captured in this factor.

It should be recognised that the value for the service condition factor is highly dependent on the maintenance regime employed at the AtoN. For example, if a lighthouse is cleaned on a weekly basis, the service condition factor may by much less than a lighthouse that is cleaned on an annual basis because the build-up of dirt and dust will not be as great.

Refer to ISO/CIE TS 22012?

Need a steer on this

#### Light Source Degradation

All light sources degrade with time, but some light sources degrade more quickly than others. Manufacturers normally provide some form of lumen degradation information in the datasheets of their lamps. Such information is invaluable to identify the limits of the light sources used in the AtoN.

An example of the lumen depreciation of a light source with hours of operation is shown in Figure 3. It shows that, for this particularly light source, there is a quick drop in lumen output early on in its lifetime, followed by a more gradual drop in output as time goes on. If the light source is going to be operating normally before being changed, then this drop in lumen depreciation must be accounted for. If the light source is changed after 4000 hours of operation, then around 22% of lumen depreciation must be accounted for. If the light source is changed after 10000 hours of operation, then 34% of lumen depreciation must be accounted for.

Example of a lumen degradation of a light source with hours of operation.

Using such information from manufacturers, it is possible to determine the value for the light source degradation, ll. This would be the amount of lumen depreciation exhibited by the light source at the end of its service life. In the example above, if the light source is changed after 4000 hours, then ll= 0.22 (= 22 %).

As a general rule, LED technology tends to exhibit much less lumen degradation than other types of light sources for the same hours of operation. However, since LEDs have a longer service life than other light sources, the value of ll for LEDs is not 0, and can be as high as 0.2 for 25000 hours of service. It is best to confirm the lumen depreciation information with the manufacturer of the LED to identify a suitable value for ll. Values of 0.05 to 0.1 (equal to 5 % to 10 % loss) may be typical.

In general terms,

ll=ΦoΦn

Calculation of lumen degradation due to failure within the light source

Where	ll is the loss due to lumen depreciation during the service life of the light source.

Φn is the total luminous flux of the light source in new condition (lm).

Φo is the total luminous flux of the light source after its service life (lm).

#### Light Source Failure

Some light sources are formed of several individual smaller light sources operating together to form a single light source. A typical example of this are integrated lanterns where several LEDs arranged in a ring operate together to effectively form a single light source.

During the service life of a multi-source light source, there will be a certain probability of one of the individual light sources (or driver) to fail. This loss factor accounts for the drop of total lumen output of the light source due to the individual failure. Thus,

lf=Φn-ΦfΦn

Calculation of lumen degradation due to failure within the light source

Where	lf is the loss due to failure within the light source.

Φn is the total luminous flux of the light source in normal operating condition (lm).

Φf is the total luminous flux of the light source in failure operating condition (lm).

Since lf is a dimensionless ratio, it may be easier to determine its value based on the 10th-percentile azimuth value of the multi-source light source, such that:

lf=In-IfIn

Calculation of lumen degradation due to failure within the light source using intensity

Where	lf is the loss due to failure within the light source.

In is the 10th-percentile azimuth intensity of the light source in normal operating condition (cd).

If is the 10th-percentile azimuth intensity of the light source in failure operating condition (cd).

If a failure of a light source is automatically fixed by the AtoN control system replacing it with an identical light source, then lf= 0. Examples of such systems are the use of lampchangers or having an identical standby integrated lantern operating on detection of a failure of the main integrated lantern.

#### Optical Path Degradation

This, perhaps, is the most difficult type of degradation to quantify since it is rare for any form of relevant data to exist for marine aids-to-navigation. There will be variation from one AtoN to another, and quite often variation between different directions on the same AtoN due to prevailing weather conditions or a tendency for wildlife to gather at a certain location on the AtoN.

Some attempts have been made to quantify the optical path degradation in the UK and Ireland, and the results have been surprising. Using a measurement process where the intensity of the bean created by lighthouses are measured in the field, it has been possible to determine the impact of cleaning on the glazing and optics of the lighthouse.

Measurements were carried out at three different lighthouses, and the losses due to dust accumulating on the traditional optics were measured to be 45 %, 0 % and 32 %. Similarly, the losses due to dirt on the glazing at two of the lighthouses were 12 % and 30 %. Unfortunately, it has not been possible to determine over what time period the accumulation of dirt and dust occurred in these cases.

In the vast majority of cases, the best that can be achieved in quantifying the optical path degradation is to estimate it based on the experience and knowledge of the environment that the AtoN is in and how regular the maintenance teams can visit to clean the AtoN. Using this information, the value of ld is set to the ratio between the amount of light lost due to dirt and dust and the amount of light in clean conditions.

In lieu of any information whatsoever, it may be useful to set ld= 0.25, which is equal to a maximum of 25 % loss of light due to dirt and dust.

#### Determining the Service Condition Factor

The service condition factor brings together all the degradation elements from above into a single factor to be applied to the design intensity. It is calculated by:

kscf=1-ll1-lf1-ld

Calculating the Service Condition Factor

Where	kscf is the Service Condition Factor.

ll is the lumen depreciation of the light source at the end of its service life (see Section 4.1.1.1).

lf is the lumen depreciation due to a failure during the light source service life (see Section 4.1.1.2).

ll is the lumen depreciation due to build-up of dirt and dust between maintenance visits (see Section 4.1.1.3).

### Astragal and Glazing Loss Factor

When the light-emitting apparatus is installed inside a lantern room or some structure, losses can be experienced due to:

- light absorption, reflection and scattering in the glazing material, and

- obscuration caused by solid structures (e.g. astragals).

The losses experienced by these means are fixed – they are inherent in the design of the AtoN. Different designs and materials will experience different losses, and therefore must be considered on a per design basis.

#### Glazing Material Loss

The type of glazing used in an AtoN can have an impact on the amount of light transmitting through it. The amount of loss is purely subject to the type of material that is used – the issue of cleanliness is dealt with under service condition factor in Section 4.1.1. The loss due to glazing is given as a dimensionless factor, ka.

A typical value of loss through glass is approximately 10 %, i.e. lg=0.1. However, acrylic glazing substitute can have a loss as low as 2 % (lg=0.02). A value for the transmittance (and therefore the loss) can be found from datasheets, consulting with the manufacturer or by measurement. In the event that no such information is available, assume 10 % as the loss through glazing.

If no glazing material is used between the light projection system and outside of the AtoN, then lg=0.

#### Astragal Obscuration Loss

Astragals, or any other solid structure, between the light emitting apparatus and the user will cause obscuration, and reduce the light intensity in certain directions. In the most extreme cases, it is possible to completely obscure the light (e.g. using vertical astragals with fixed optics). The amount of obscuration depends entirely on the size of light projected outwards and the nature of the structure surrounding it. For this reason it is possible to change the impact of astragals by modifying the design.

It is possible to calculate the effects of astragals on the light intensity if the size of the projected light source and the structures surrounding it are known. However, it is accepted that it is a difficult calculation.  It may be possible to estimate the impact of astragals by simplifying the system to a single situation, and assume that the level of obscuration is constant in the direction of the useable range or area.

Demonstration of astragal obscuration on the total light output from a fixed optic.

Take the example shown in Figure 2. This figure shows the situation of a fixed optic without astragal obscuration and with astragal obscuration. The loss, la, represents the difference in the observed lit area between these two situations. The lit area obscured by the astragal can be determined using Equation 6.

Ao=ncwawlsinθ

Obscured area of a fixed optic light due to astragals

Where:

Ao is the area of obscuration in m2.

nc is the number of astragals crossing the light source from a given direction.

wa is the width of the astragal in m.

wl is the width of the light source in m.

Θ is the angle of the astragal from vertical.

The obscuration loss due to astragals is

la=AoAu

Estimation of the loss due to astragal obscuration

Where:

la is the loss due to astragals.

Ao is the total area of obscuration due to astragals in m2.

Au is the total unobscured area of the lit area in m2.

If the light from the light emitting apparatus is unobstructed by astragals (or other structures) in the direction of the useable range or area, then la=0.

In the event where information on the astragals is not available or too complex to model, then it may be necessary to estimate the value of la. In the past, a value of la= 0.06 was used combined with a glazing factor of lg= 0.1 to produce a glazing/astragal loss factor of 0.15. This was determined with the traditional use of large optics relative to the size of the astragals. The modern practice of using smaller optics may have an effect of increasing the value of la, but it is dependent on the size of the light source projected from the AtoN and the location of the astragals relative to the projected light.

#### Determining the Glazing and Astragal Loss Factor

The glazing and astragal factor, kga brings the two losses described above into a single factor that determines the overall reduction in the light intensity due to these effects. Thus,

kga=1-lg∙1-la

Glazing and Astragal Loss Factor

Where:

kga is the glazing and astragal loss factor.

lg is the glazing material loss (see Section 4.1.2.1)

la is the astragal obscuration loss (see Section 4.1.2.2)

### Filter Material Factor

Many lighthouses have sectors created by the use of colour filters in order for a light colour to be different in a given direction. By their very nature, filters attenuate the total light as it absorbs light that does not fit into its passband. For example, a red filter will absorb blue, green and yellow. This means that any light energy in those wavelengths will be lost, causing the intensity to drop. If such an approach is taken, then the drop in intensity due to the use of filter material needs to be taken into account.

Typical filter material factors for LED and Tungsten Halogen lamps

|  | Value of kf |  |
| --- | --- | --- |
|  | Red Filter | Green Filter |
| White LED | 0.05 – 0.15 | 0.30 – 0.45 |
| Tungsten Halogen | 0.15 – 0.25 | 0.30 – 0.40 |

The factor depends on the spectrum of the light and the wavelength dependant transmittance of the filter material. Both quantities can either be measured to derive an overall transmittance, or the overall transmittance can be measured directly with a photometer. Typical values for red and green filters with different light sources are shown in Table 3. These values are only for guidance, and the transmittance for the filters used should be determined using the light source to be used in the AtoN.

Note that if no additional colour filters to those inside the light-emitting apparatus are used, then this factor can be ignored (i.e. kf=1) because it will be captured in the specification intensities below.

## Specification Effective Intensity

The Specification Effective Intensity, Ise, is the minimum effective intensity of the light that will meet the Design Intensity after taking the de-rating factors into account. Note that this is the effective intensity – the intensity of a flashing light that is perceived. Recall that the design intensity is the intensity that the user observes regardless of the flash duration and profile. As such, the design intensity is also the effective intensity.

Thus,

Ise=Idsgkscf∙kga∙kf

Calculation of the Specification Effective Intensity

Where:

Ise is the Specification Effective Intensity in cd.

Idsg is the Design Intensity in cd (see Section 3.3)

kscf is the service condition factor (see Section 4.1.1)

kga is the glazing and astragal loss (see Section 4.1.2)

kf is the filter material factor (see Section 4.1.3)

The Specification Effective Intensity is fixed only by the parameters given above – it does not take into account the flash character. This could be advantageous since knowledge of the flash shape is not required at the point of procurement – only the Specification Effective Intensity and the shortest flash duration is needed. This would ensure that all possible solutions can be considered.

It should be noted that as discussed in Section 4.1.2.2, the amount of obscuration is dependent on the nature of the light projection out of the light emitting apparatus. There may be room to adjust kga if a favourable light distribution is obtained from a particular design approach, thus allowing some adjustment in the Specification Effective Intensity. Care should be taken to make such adjustments since a too favourable value would result in a light emitting apparatus not providing the Design Intensity required.

## Specification Peak Intensity

If the flash profile of the light emitting apparatus is known, then it is possible to determine a factor to convert the Specification Effective Intensity to the Specification Peak Intensity. This is done by applying the Modified Allard Method as described in Recommendation R0204 and Guideline G1135 to the flash profile to

Isp=IsekMAM

Specification Peak Intensity

Where:

Isp is the Specification Peak Intensity in cd

Ise is the Specification Effective Intensity in cd (see Section 4.2)

kMAM is the factor taking into account the flash profile.

For a perfectly rectangular flash profile, it is possible to apply the Blondel-Rey equation to calculate kMAM. Thus,

kMAM=TT+a

Application of Blondel-Rey equation for rectangular flash profiles

Where:

kMAM is the factor taking into account the flash profile.

T is the shortest flash duration in a flash character in seconds.

a is the visual constant in seconds. It is equal to 0.1 s, except for blue signal light at night where a=0.2 s.

For non-rectangular flashes, the full Modified Allard Method will have to be applied in order to determine  kMAM. See Guideline G1135 for more details.

For steady-burning lights, kMAM=1, meaning that the Specification Peak Intensity is the same as the Specification Effective Intensity.

# Examples

## Example 1

A lighthouse is being refitted with a light to serve an area from 1 M to 8 M from the lighthouse. It is necessary for the light to be seen for at least 50 % of the time in prevailing weather conditions. The flash character is Fl W (2) 10 s, with a flash duration of 0.5 seconds. The light will only be exhibited during the night.

### Minimum Distance of Observation

In this example, the minimum distance of observation is 1 M. So, dmin=1852 m.

### Maximum Meteorological Visibility

Using the local meteorological office data, the best meteorological is considered to be 20 M. So, Vmax=37040 m.

### Maximum Illuminance for Glare

On inspection of the background of the lighthouse from the direction of the area of utilisation, it was found that there is no background luminance. Therefore, Emax=0.01 lux.

### Calculating the Maximum Required Intensity

By applying Equation 1, we find that the maximum intensity is:

Imax=Emax∙dmin2∙0.05-dminVmax
Imax=0.01∙18522∙0.05-185237040
Imax=39841

Therefore, the maximum intensity the light emitted from the lighthouse is 39841 cd.

### Maximum Distance of Observation

The maximum distance of observation is 8 M. So, dmax=14816 m.

### Minimum Meteorological Visibility

There is a requirement for the light to be see in the area of utilisation for 50 % of the time. Using data from the local metrological office, the visibility is at least 5 M for 50 % of the time. So, Vmin=9260 m.

### Minimum Illuminance at the Observer

Since the light will only be exhibited during the night, and there is no background luminance. Therefore, according to Table 2, Emin=0.2 μlux.

Also, there are no rival lights in the area, and therefore, no adjustment to Emin is required.

### Calculating the Minimum Required Intensity

By applying Equation 4, we find that the minimum intensity is:

Imin=Emin∙dmax2∙0.05-dmaxVmin
Imax=0.0000002∙148162∙0.05-148169260
Imax=5298

Therefore, the minimum required intensity for the light to be seen at the furthest point in the area of utilisation under the adverse weather conditions prescribed in the requirements is 5298 cd.

### Selecting a Design Intensity

We wish to save as much power as possible, and since there is no need to be extra conspicuous, it is decided that the design intensity, Idsg=Imin=5298 cd.

#### Nominal Range for Publishing

Now that the design intensity has been selected, the nominal range for publishing can be calculated. Using Equation 5 or the table in Recommendation R0202, we find that 5298 cd has a nominal range of 13 M to the nearest whole nautical mile.