Input paper: ARM12-7.3.8

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

XARM	□ENG		□PAP					X  Input

□ENAV	□VTS							□  Information

Agenda item 		7

Technical Domain / Task Number 2	………1.5.10………………

Author(s) / Submitter(s)		………CHINA MSA……………….

Feature specification of the Physical Mobile Aids to Navigation

# INTRODUCTION

The danger area caused by a sunken ship should be indicated by isolated danger marks or Emergency Wreck Marking Buoy. As moving wreckage cannot be accurately identified at in the sea, it may pose a safety hazard to the ship's navigation. To make the situation worse, there are many different type of drift buoys for different purpose (oil spill monitoring, marine data collection and personnel search and rescue), but there is no standard to follow in term of marking. Without proper marking, there is a potential risk for accident and fishermen might accidentally destroy the floating buoys without notice and causing a lost to the research team if the application is meant for research purpose. To increase the safety of marine navigation and mark the area of sunken ship, the paper provides an overview of the standard research that can regulate the Physical Mobile Aids to Navigation (MAtoN), which is helpful for the development standard of the MAtoN.

AtoN Authority will require a level of effort to implement the MAtoN. Most of the information regarding MAtoN required to complete the description of MAtoN through the recognised marine publications.

Recognizing the dangers of moving objects for seafarers, the IALA has developed guidelines for the use of Mobile Aids to Navigation (MAtoN). This paper is intended to provide the standard that can regulate the Physical Mobile Aids to Navigation (MAtoN), which is supplementary of the guidelines.

## Purpose of the document

This paper is intended to provide the ARM committee an overview of the standard research that can regulate the Physical Mobile Aids to Navigation (Physical MAtoN).

## Related documents

ARM11-8.4 WP Draft IALA Guideline on Mobile AtoN 16 Oct clean copy (ARM10-13.2.10)

# Background

Once a sunken ship accident has occurred in the Qingdao area. Originally, the AtoN Management Administration was preparing to install an Emergency Wreck Marking Buoy at the shipwreck area, unfortunately, the wreckages were flooding on the sea, causing some threat to the vessel, then as the AtoN Authority, we should think of a solution to mark the drift wreckage for vessel crews’ safety.

Now the existing Mobile Aids to Navigation (MAtoN) in China area are mostly used for oil pollution monitoring, marine data collection as well as search & rescue applications research, the size of the buoy is not standard and it is very hard for the vessel navigator to detect them as almost all of them without blinking light at the night.

- The oil spill tracking position buoy, obtaining position information and transmitting it to the integrated command centre platform for easy recovery of oil spillage and oil spill emergency equipment; no flash light at night.

Figure1. Oil spill tracking position buoy

- Profiling drifting buoy can drift along with the current in the ocean and automatically control the rise and fall motion, thus completing the measurement of the vertical profile environmental parameters. In order to prevent capture, it also does not flash at night.

Figure2. Profiling drifting buoy

- The floating buoy for sea search and rescue with adjustable attitude. The buoy body has a humanoid appearance and is used for research on the search & rescue applications.

Figure3. Floating buoy for sea search and rescue

# Discussion

IALA gives the guide of using the Mobile AtoN, and based on the current situation, we proposed the standard that can regulate the Physical MAtoN according to the current situation.

## Use of MAtoN in IALA

A Mobile AtoN (MAtoN) is defined as a non-fixed or un-moored AtoN; but does not include a fixed or moored buoy that is adrift from station. Types of MAtoN which could be used are physical or virtual. Special signs may be used to mark the MAtoN, the size and shape of the MAtoN can vary depending on its application. The Marks of physical MAtoN show in the table 1.

Table 1- Physical MAtoN characteristics described in IALA Use Guide (draft) in ARM-11

| Type of AtoN, colour, and shape | Special Mark. Colour yellow. Shape optional, but not in conflict with lateral marks. |
| --- | --- |
| Top mark | Single yellow “X” shape. To increase conspicuity, and if practical, a top mark of a special mark should be used. |
| Additional marks | For hazards with low profile/partly submerged and where visual observations are primarily undertaken from the air, an additional mark, like a circular symbol, can be added, if practicable. |
| Position sensor | A physical AIS AtoN |
| Light (when fitted) |  |
| Colour | Yellow |
| Rhythm | Flicker 1s Eclipse 0.7s Repeat Flicker 1s Eclipse 0.5s Normal Fl 1s Eclipse 0.5s Normal Fl 1s Eclipse 3s. (Flicker 5hz) Minimum range 3NM |

## China proposed the standard of the Physical MAtoN

- Physical mobile aids to navigation

Physical MAtoN should, where possible, be designed to meet the following specifications and light characteristics:

Table 2- Physical MAtoN characteristics of China proposed

| Buoy / mark characteristics |  |
| --- | --- |
| Colour | Yellow |
| Shape | Optional, it can be determined based on the usage scenario. If it is used to mark a drifting shipwreck, choosing the buoyant shape with a top mark; when used for the oil spill monitoring or ocean data collection, a spherical shape can be selected, and no top mark is required. |
| Top mark | blue single “X” shape |
| Day mark |  |
| Light Characteristics |  |
| Colour | Yellow |
| Rhythm | Morse code ”M” (--) The period is 12 seconds |
| Racon | If fitted, Morse “M” = (Keep clear of me) should be used |
| AIS (when fitted) |  |
| AIS Naming Convention | “Mobile + name”, and should reflect the nature of the hazard. For example, the name of the MAtoN is set to “mobile+... wrack " can indicate the drifted shipwreck. |

The Mobile AtoN should be set to yellow colour and marked with a black dual wavy line graphic; in addition, a mark indicating its purpose should be set up at the obvious place of the buoy using the black, and it can be observed from any horizontal direction on the water. If requited, the top mark adopts a blue "X" shape; as shown in Figure 4.

Figure 4. Schematic diagram of mobile AtoN

During AIS usage, the name field of AtoN should include the word “mobile”, and reflect the nature of the hazard. AIS message should also be able to provide approximate direction and speed of a mobile AtoN (MAtoN). In parallel, the type/feature of AtoN reflected in the message shall include the term “MOBILE”. The height of an antenna for an AIS device on a Mobile AtoN will vary based on the scenario, it should not be limited to 1 metre.

- Deployment of physical mobile aids to navigation

For drifts such as the wreckage of a sunken ship, it is impossible for the Mobile AtoN to be placed at sea immediately because of weather conditions, the marine environment and the location of the accident. Therefore, we recommend installing a buoy on the deck of the hull, which will automatically released when the ship sinks, and the buoy will drift along with the sunken ship, thus reducing the probability of a secondary accident in the set-up process of the buoy.

Figure5. Buoy placed on the deck

The Mobile AtoN should avoid entering the ports and navigation channels. If the visual effect is unsatisfactory, it should be able to send radar signals or AIS signals to avoid collision. National Authorities or the appropriate jurisdiction should promulgate the particulars of the MAtoN by use of all appropriate means. This can be done through the appropriate recognised marine publications to notify the mariner the official situation that a MAtoN is deployed and therefore in use, as well as other appropriate methods of promulgating Maritime Safety Information (MSI). The details of the MAtoN announcement include: the type of buoy provided, drift range, size and the flashing features (colour, frequency, etc.).

# References

- R1016-Ed1-Mobile-Marine-Aid-to-Navigation-MAtoN_Dec2017

- ARM11-8.4 WP Draft IALA Guideline on Mobile AtoN 16 Oct clean copy (ARM10-13.2.10)

# Action requested of the Committee

The Committee is requested to:

- 1    Note the relevant feature specifications of Mobile Marine Aids to Navigation (MAtoN) in this paper, such as colour, shape, top mark, day mark, light characteristics, AIS naming and deployment of Physical MAtoN.

- 2      consider incorporating them into the use of Mobile Marine Aids to Navigation(MAtoN) of the IALA Guideline：5.1 physical MATON characteristics and 6.1deploment of the physical MATON.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain