Input paper:   ENG12-3.1.14.2

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

☐  ARM	☒  ENG		☐  PAP						☐  Input

☐  ENAV	☐  VTS								☒  Information

Agenda item 	(from agenda)			3.1

Workplan Task Number / Technical Domain 2	3.4.1

Working Group					WG 3

Author(s) / Submitter(s)	Rodrigo González and Elisabet Lacarra (ESSP), Manuel López (GSA) and Kaisu Heikonen (Väylä).

# EGNOS performance along Finnish coast

# Introduction

EGNOS is the Satellite Based Augmentation System (SBAS) over Europe, currently designed to augment the GPS navigation system constellation by broadcasting additional signals from geostationary (GEO) satellites.

Figure 1-1 EGNOS architecture

EGNOS is the satellite based positioning service over Europe with better accuracy with respect to GPS standalone position. In addition, EGNOS provides Integrity, which can be suitable for safety critical applications in the maritime sector. Because of that, the European Commission, EC (EGNOS owner), the GNSS Agency, GSA (EGNOS Services Programme Manager), the European Space Agency, ESA (EGNOS design agency) and the European Satellite Services Provider, ESSP (EGNOS service provider) are working in close collaboration to support the standardisation of SBAS in maritime receivers. In addition, two of officially recognised European Standardisation Organisations, the European Committee for Standardisation, CEN and the European Committee for Electrotechnical Standardisation, CENELEC, are working on a proposal to develop a new test standard on SBAS for shipborne receivers in International Electrotechnical Commission, IEC (IEC 61108-x).

# SBAS Guidelines for shipborne receivers

IMO MSC.401 [1] and IEC 61108-4 [2] (Shipborne DGPS and DGLONASS maritime radio beacon receiver equipment) allow for the use of SBAS in shipborne receivers but there is no standard for its implementation. Most of maritime GNSS receiver models are SBAS compatible but they could present important differences in their performance since there are no common guidelines. This is why ESSP, GSA and ESA have been working in a guidelines document for manufacturers for the development of SBAS receivers for the maritime domain [3], which has been coordinated in the frame of Radio Technical Commission for Maritime Services, RTCM Special Committee 104.

These SBAS guidelines are in draft mode and planned to be published by end of 2020. This document will present the receiver algorithm recommendations and the set of SBAS messages to be processed by an SBAS receiver for the maritime sector to be compliant with the operational requirements stated in International Maritime Organization (IMO) Resolution A.1046 (27) [4] for “Harbour entrances, Harbour approaches and Coastal waters” and for “Ocean Waters”. It will also include the tests specifications to be passed in order to meet the expected performance.

In a nutshell, a maritime SBAS receiver shall at least apply satellite and ionospheric corrections (MT0, MT 2-5, MT 18, MT 24, MT 25, MT 26), taking into account the warnings broadcast by the system and time-out intervals for the validity of the message Information. Besides, the receiver should use smoothed pseudo-ranges and monitor the quality of the satellite signal tracking for undetected measurements faults (i.e. cycle slips, step detector...).

Finally, the following checks are mandatory to be done by the SBAS receiver, providing the same failure warnings and status indicators as the maritime GPS receiver equipment [2]:

- HDOP check: The SBAS receiver unit shall provide an indication if the specified HDOP has been exceeded (HDOP > 4).

- Integrity using RAIM: The SBAS receiver equipment shall incorporate integrity monitoring using fault detection, for example receiver autonomous integrity monitoring (RAIM), or similar means to determine if accuracy is within the performance standards and provide an integrity indication. The integrity indication for different position accuracy levels (10 m  and 100 m) shall be expressed in three states as requested by IEC 61108-1[2]:

  - Safe. The integrity calculation can be performed with a confidence level above 95% for the selected accuracy level and RAIM calculates the probable position error to be within the selected accuracy level. This generally requires at least 5 "healthy" satellites available and in a robust geometry, whereas 4 satellites are at least required for navigation.

  - Caution. Insufficient information (i.e. Low number of satellites (i.e. <5) or computed probabilities greater than expected).

  - Unsafe. RAIM calculates the probable position error exceeding the selected accuracy level.

GSA launched the MAREC project with Kongsberg in 2018, which contributed to the SBAS standardisation for Safety of Life at Sea (SOLAS) and non- Safety of Life at Sea (non-SOLAS) applications both for navigation equipment and for Automatic Identification System (AIS). Under the MAREC project, Kongsberg developed a maritime receiver in line with these SBAS guidelines [3], whose algorithms were tested in a laboratory and in a real environment.

These SBAS guidelines [3] aim at supporting the IEC standardisation process for an SBAS shipborne receiver standard to ensure a safe use of SBAS by the maritime community (type-approved receivers).

# EGNOS L1 Martime Service

EC and GSA are working for the potential provision of an EGNOS L1 maritime service for “Harbour entrances, Harbour approaches and Coastal waters” and for “Ocean Waters” by 2022.

With the support of ESSP, EGNOS L1 performance (accuracy, availability, continuity, integrity) were analysed concluding that EGNOS L1 meet the operational requirements stated in International Maritime Organization (IMO) Resolution A.1046 (27) [4] for “Harbour entrances, Harbour approaches and Coastal waters” and for “Ocean Waters” over Europe. Assessment is ongoing to define the potential servicer area for the EGNOS L1 maritime service, which plans to cover most of European coast waters. Moreover, GNSS campaigns on board vessels (such as the one presented in section 5 of this paper) along European coasts have been carried out to demonstrate the EGNOS benefits in real environmental conditions and potential and common vessels.

The service provision scheme required to guarantee the required service level is under definition, which was presented in EMRF (European Maritime Radionavigation Forum) to maritime authorities. This service plans to include an EGNOS Maritime Safety Information (MSI) to mariners and a potential establishment of specific working agreements between the EGNOS Service Provider and any national competent authority.

In addition, vessels should be equipped with type approved receivers for SBAS L1 in order to ensure the required operational performance for maritime community. For that, EC requested to CEN, CENELEC and TSI develop standardisation regarding SBAS L1 receivers for maritime applications, which will be covered in a new standard in IEC 61108 series, [2]. The IEC standardisation process plans to be started by Q3 2020, whose process may be completed by 2022.

# Maritime requirements based on IMO Resolution A.1046 (27)

The list of service performance parameters for a complete characterisation of EGNOS maritime service is derived from the list in IMO Res. A.1046 (27) (Table 4-1) and includes Signal Availability, Horizontal Accuracy 95%, Position update rate, Service Coverage,  Service Continuity and Time To Alarm for “Harbour entrances, Harbour approaches and Coastal waters” and for “Ocean Waters”.

| Parameter | Maritime requirements based on IMO A.1046 (27) |  |
| --- | --- | --- |
|  | Ocean Waters | Harbour entrances, harbour approaches and coastal waters |
| Horizontal Accuracy 95% | 100m | 10m |
| Signal Availability | 99.8% | 99.8% |
| Service continuity (over 15min) | - | 99.97% |
| Position update rate | 2s | 2s |
| Time to Alarm1 | Maritime Safety Information as soon as practicable | 10s |
| System coverage | Adequate2 | Adequate2 |
| 1Generation of integrity warnings in cases of system malfunctions, non-availability or discontinuities. |  |  |
| 2Taking into account the radio frequency environment, the coverage of the system should be adequate to provide position-fixing throughout this phase of navigation. |  |  |

Table 4-1World-Wide Radionavigation System IMO requirements

It is noted that “Ocean Waters” are understood as the waters not belonging to the territorial waters (up to 12 NM from the coast line).

# GNSS Performance Assessment along Finnish coast on board Mastera oil tanker

The aim of this GNSS data campaign is to assess EGNOS performance at user level in the maritime domain at high latitudes in Europe along Gulf of Finland. GPS standalone performance is also assessed.

## Characteristics of GNSS data campaign

The characteristics of this GNSS data campaign in maritime domain are:

- Route: the vessel, Mastera, departed from Porvoo (Finland) to Primorsk (Russia) then until Naantali (Finland). Afterwards, the vessel went from Naantali (Finland) to Primorsk (Russia) being two days stopped close to Uusimaa waiting for orders (adrift) and finally arrived at Porvoo (Finland).

- Time framework: the vessel departure was on 01.11.2019 and arrived on 14.11.2019 at Porvoo.

- Vessel: Mastera. This is a Finnish Aframax crude oil tanker operated by Neste Shipping. This icebreaking tanker transports crude oil year-round from the Russian oil terminal in Primorsk to Neste Oil refineries in Porvoo and Naantali. https://www.neste.com/

Figure 5-1 Mastera oil tanker

Figure 5-2 GNSS data collection path

The following equipment was used during the data campaign:

- GNSS MF antenna.

- GPS splitter. It is used for GPS signal distribution with high performance to the GNSS SiS recorder and to the GNSS receivers.

- GNSS SiS recorder. It records and replays RF signal in L1 band.

- GNSS MFMC receiver. This High-end GNSS receiver is used for two goals:

  - Monitor GNSS performance.

  - Compute real position of the reference path for obtaining a posterior SBAS position error during the time frame of the data campaign.

  - Compute GPS position solution.

It is noted that a second high-end receiver is used as back-up equipment for the other one in this installation.

- SBAS maritime receiver. It is used to obtain directly SBAS position solution.

## Methodology of computing

The activity for the GNSS performance assessment consists of three general lines:

- Real position. It is obtained in post-processing using high precision techniques (PPP, Precise Point Positioning) with the GNSS data obtained by the receiver. Figure 5-3 describes the flow of data through different tools to obtain the precise path of the Mastera vessel in order to compute GNSS performance.

It is stressed that both high-end receivers are with the same configuration. One is the back-up of the other one.

- SBAS Navigation solution. This is computed directly with the GNSS maritime receiver. The computation of the performance analysis is done using internal Analysis Tools developed by ESSP. Figure 5-3 summarises the process of the methodology followed.

- GPS-only solution. RF L1 signal recorded during the maritime dynamic data campaign is replayed in the maritime receiver to obtain GPS standalone solution. It has to be stressed that this recorded scenario only lasts five days (from November 1st to 6th 2020).

Figure 5-3 Process of GNSS PVT solution

## GNSS performance analysis

GNSS performance analysis is based on the position solution obtained with some GNSS receivers installed in the vessel:

- a high-end receiver;

- a maritime receiver.

Besides, the RF L1 signal in space recorded during almost six days on-board is replayed in the following receivers in order to analyse their GNSS performance:

- a maritime receiver in line with these SBAS guidelines [3];

- a maritime receiver (to analyse GPS position solution);

- a high-end receiver (to analyse GPS position solution).

### EGNOS signal in space

EGNOS signal availability refers to the percentage of time during which reliable information is provided by the system within the specified area of service. Thus, EGNOS 1046 Signal Availability assesses the percentage of time the EGNOS signal in space is provided by the GEOs according to messages that can be processed by an SBAS receiver aligned with the receiver guidelines, [3].

EGNOS broadcasts through two operational GEO satellites.  This redundancy benefits EGNOS receivers which can instantaneously GEO switch and therefore, EGNOS signal availability is calculated as the combined signal availability of both operational EGNOS GEO satellites.

EGNOS signal unavailability happens when there is a simultaneous signal in space outage in both EGNOS GEO satellites.

During the whole period of Mastera vessel trip (November 2019) the operational EGNOS GEO satellites were PRN 123 and PRN 136.

Figure 5-4 EGNOS operational satellites (November 2019)

The EGNOS monitoring information shows that the EGNOS signal in space availability was 100% during the whole data campaign period, which meets the 99.8% EGNOS signal in space availability requirement [4].

### EGNOS / GPS standalone availability

EGNOS as a radio-navigation system has a particularity which is EGNOS GEO satellites broadcast messages over the GEOs footprint (Figure 5-4) but EGNOS performance is not the same in the whole EGNOS GEO footprint. The reason of that is because EGNOS messages include ionospheric corrections only for a region around Europe and, for a subset of GPS satellites.

It is noted that a GPS satellite is monitored if EGNOS differential corrections and integrity information is available for that GPS satellite. Then, this GPS satellite can be used for the EGNOS navigation solution. As a result, users in Europe and the surrounding area can use EGNOS differential corrections and integrity information to improve their positioning.

Therefore, EGNOS availability is the percentage of time an user is able to compute a position based on EGNOS.

Minimum PDOP availability for GPS L1 is the percentage of time that PDOP is lower or equal than 6 per day.

Table 4-1 shows the percentage of time the receiver is computing the position solution using EGNOS and GPS standalone.

|  | EGNOS Availability [%] |  | GPS standalone Availability / min PDOP [%] |
| --- | --- | --- | --- |
| DOY | High-end receiver | Maritime receiver | Maritime receiver |
| 305 | 100 | 100 | 100 / 100 |
| 306 | 100 | 100 | 100 / 100 |
| 307 | 100 | 100 | 100 / 100 |
| 308 | 100 | 100 | 100 / 100 |
| 309 | 100 | 100 | 100 / 100 |
| 310 | 100 | 100 | 100 / 100 |
| 311 | 100 | 100 | -- |
| 312 | 100 | 100 | -- |
| 313 | 100 | 100 | -- |
| 314 | 100 | 100 | -- |
| 315 | 100 | 100 | -- |
| 316 | 100 | 100 | -- |
| 317 | 100 | 100 | -- |
| 318 | 100 | 100 | -- |

Table 5-1 Availability obtained using EGNOS and using GPS standalone

N. B. GPS standalone performance assessment is only between 01.11.2019 16:15 UTC and 06.11.2019 19:43 UTC (5 days).

Analysing these results, it can be concluded that:

- EGNOS position availability was 100% for both receivers.

- GPS L1 standalone availability was 100% which fulfils the 99% position service availability performance for an average location according to the GPS SPS PS, [5].

- Minimum PDOP availability is also in agreement with the GPS commitment in the GPS SPS PS, [5].

No EGNOS continuity events were detected during the data campaign period.

### EGNOS / GPS accuracy

On one side, Horizontal Accuracy is the 95% percentile of the Horizontal Position Error (HPE) distribution. HPE is the 2D radial error of the instantaneous measured position by the GNSS receiver) respect to the true instantaneous position.

On the other side, the compliance to update rate of displayed position shall be demonstrated by the receiver/equipment manufacturers. Nonetheless, it is expected to be capable an update rate of 1Hz.

Table 5-2 compares the horizontal accuracy between the user performance obtained using EGNOS and GPS standalone.

|  | EGNOS HNSE 95% percentile [m] |  | GPS standalone HNSE 95% percentile [m] |
| --- | --- | --- | --- |
| DOY | High-end receiver | Maritime receiver | Maritime receiver |
| 305 | 1.179 | 0.869 | 1.444 |
| 306 | 0.935 | 0.791 | 1.809 |
| 307 | 0.970 | 0.792 | 1.222 |
| 308 | 0.970 | 0.747 | 1.638 |
| 309 | 1.023 | 0.835 | 1.307 |
| 310 | 0.967 | 0.841 | 2.000 |
| 311 | 0.950 | 0.907 | -- |
| 312 | 0.956 | 0.718 | -- |
| 313 | 0.873 | 0.762 | -- |
| 314 | 1.000 | 0.786 | -- |
| 315 | 1.057 | 0.808 |  |
| 316 | 0.911 | 0.784 |  |
| 317 | 0.870 | 0.792 |  |
| 318 | 1.049 | 0.872 |  |
| TOTAL | 0.971 | 0.815 | 1.551 |

Table 5-2 HNSE (95%) obtained using EGNOS and using GPS standalone

Table 5-2 presents daily HNSE (95%) quite close to 1m when using EGNOS solution against GPS standalone, in particular, there is a global HNSE (95%) for the 13 days using the maritime receiver, of 0.815m whereas the global value for the same receiver reproducing the recorded RF L1 signal to compute GPS standalone solution during 5 days is 1.551m.

To be more precise, it can be compared EGNOS HNSE (95%) against GPS standalone HNSE (95%) exactly for the same period (since 01.11.2019 at 16:15 UTC to 06.11.2019 at 19:43:21 UTC) using the same Maritime receiver and replaying the same RF L1 signal-in-space.

|  | EGNOS solution |  | GPS standalone |  |
| --- | --- | --- | --- | --- |
| DOY | HNSE (95%) [m] | Max. HNSE [m] | HNSE (95%) [m] | Max. HNSE [m] |
| 305 | 0.869 | 1.219 | 1.444 | 1.721 |
| 306 | 0.791 | 0.910 | 1.809 | 2.184 |
| 307 | 0.792 | 1.088 | 1.222 | 1.552 |
| 308 | 0.747 | 0.970 | 1.638 | 2.132 |
| 309 | 0.835 | 1.353 | 1.307 | 1.804 |
| 310 | 0.798 | 1.092 | 2.000 | 2.544 |
| TOTAL | 0.800 | 1.353 | 1.551 | 2.544 |

Table 5-3 HNSE (95%) obtained using EGNOS and using GPS standalone with maritime receiver since 01.11.2019 at 16:15 UTC until 19:43:21 UTC

Table 5-3 shows the clear improvement of EGNOS horizontal accuracy between 54% and 151% for the case of the maritime receiver against GPS standalone.

Figure 5-5 EGNOS and GPS standalone HNSE values with the Maritime receiver

Figure 5-6 Global plots of the EGNOS HNSE distribution (left) and GPS standalone HNSE distribution (right) with the Maritime receiver

Analysing these results, it can be concluded that:

- Percentiles at 95% of Horizontal position errors are lower than 1.2 meters for both high-end and maritime receivers in all days and usually lower than 1 meter.

- EGNOS Horizontal accuracy (95%) was lower than 1.2 meters for both receivers, which is compliant with IMO Res. A.1046 (27), [4].

- All OS EGNOS receivers satisfied the commitment accuracy performances defined in EGNOS OS (HPE < 3m, VPE< 4m), [6].

- The GPS Single Frequency receiver showed nominal accuracy results (considered when HPE < 9m, VPE< 15m satisfying the commitments in GPS SPS PS, [5].

- EGNOS provides a clear improvement of horizontal accuracy between 54% and 151% for the case of the maritime receiver with regards to the use of GPS standalone.

- EGNOS provides a clear improvement of vertical accuracy between 149% and 289% for the case of the maritime receiver with regards to the use of GPS standalone

# General Conclusions

Results from two receivers (one maritime and other high-end receiver) on board the vessel that navigated during thirteen days along the Gulf of Finland show that:

- The EGNOS signal in space availability was 100% during the data campaign period meeting the 99.8% requirement of IMO Res. A.1046 (27), [4]. In fact, both GNSS receivers were able to track EGNOS messages from both operational GEO satellites (PRN123 and PRN 136) the 100% of time.

- EGNOS position availability was 100% for both GNSS receivers during the whole period.

- EGNOS position continuity was 100% for both receivers during the whole period since no EGNOS continuity events were detected during the data campaign period. This value is compliant with IMO Res. A. 1046 (27), [4], for “harbour entrances, harbour approaches and coastal waters”.

- 95th percentile of the Horizontal Position Error was 0.81m during the data campaign period with a maritime receiver and 0.971 m for the high-end receiver., meeting the 10 meter requirement for “harbour entrances, harbour approaches and coastal waters” and the 100-meter requirement of “Ocean waters” established in IMO Res. A.1046 (27), [4].

- EGNOS horizontal position accuracy is enhanced between 54% and 151% with respect to GPS standalone solution for the case of the maritime receiver. As an example, on 02.11.2019, the percentile at 95% of the horizontal position accuracy was of 0.791 m for EGNOS, whereas of 1.809 m for GPS.

In consequence, the EGNOS performance observed on board the oil tanker, Mastera, indicates that EGNOS can support “Harbour entrances/approaches and coastal/ocean waters” along the Gulf of Finland according to IMO Res A.1046 (27) [4], meeting the 10 meters confidence level at 95%, the signal-in-space availability requirement of 99.8% and the service continuity of 99.97%.

# ACKNOWLEDGMENTS

We would like to express our gratitude to OSM Group AS (Norwegian Oil Transport provider) to allow ESSP to install the receiver on board this oil tanker, Mastera, as well as to Väylä (Finnish Transport Infrastructure Agency) to invite ESSP to perform this GNSS data campaign in the Gulf of Finland and coordinate it with OSM Group AS.

Finally, the authors would like to acknowledge the efforts done by EC and the GSA to work at programme level for the future provision of EGNOS L1 maritime service.

# References

IMO Resolution MSC.401 (95) performance standards for multi-system shipborne radio-navigation receivers.

IEC 61108 - Maritime navigation and radio-communication equipment and systems – Global navigation satellite systems (GNSS) http://www.iec.ch.

Draft Guidelines for Manufacturers for the Implementation of SBAS in Shipborne Receivers Including Methods of Testing and Required Test Results, 31 July 2017.

IMO Resolution A.1046 (27) Worldwide Radio-navigation System, 20 December 2011.

GPS Standard Positioning Service Performance Standard 30.09.2008, 4th Edition.

EGNOS Open Service (OS) - Service Definition Document v2.3
http://egnos-portal.gsa.europa.eu/library/technical-documents

Input document number, to be assigned by the Committee Secretary

Input papers should be assigned to a work task as listed in the Committee work plan which is available in  input papers. Leave open if uncertain but consider how the paper is to be processed if not relevant to a work task