Input paper:   ENG12-3.1.4

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM		x  ENG		□  PAP					□  Input

□  ENAV	□  VTS							x  Information

Agenda item 				Action item 50 from ENG10

Technical Domain / Task Number 2	WG1 Visual & Physical AtoN

Author(s) / Submitter(s)			Alwyn Williams and Peter Dobson

# Practical Methods of Measuring Light Source Degradation

# Summary

This paper outlines a method for the measuring of light source degradation in the field, including the results of trials to establish the effectiveness of the technique. This approach is intended for light sources within fixed and rotating optics.

# Background

Over the years, the provision of visual aids to navigation (AtoN) has been provided by a number of different forms of light source. More recently, this is being achieved with light emitting diodes (LED). However, in older installations, this may still be being provided by tungsten halogen lamp filament lamps or metal halide lamps.

Needle Lighthouse light source using tungsten halogen lamps.

As part of a periodic check to confirm effective performance of replacement lamps and driver arrangement, checks of the voltage and current can be captured for comparison against original commissioning performance. This, however, can be both invasive and sometimes difficult to achieve.

In recent years, we have found this approach has become unreliable. As the lamp driver equipment ages, evidence of lamps not being powered effectively have be seen through blackening of the envelope and a shortened life, although it was not known what impact this would have on the optical performance.

Blackened 35W Metal Halide lamps from Eddystone lighthouse.

Currently, the only way to confirm the intensity of an aid to navigation light is by a field measurement, which involves comparing the light output with a calibrated reference lamp. It takes a considerable effort to carry out a field measurement, and for most maintenance visits, it is not a practical proposition.

Consideration was then given to a manner in which a simple periodic optical light source checks could be achieved as part of maintenance routine, without the need for a complete field measurement procedure. If a method of capturing reference measurement during commissioning can be tied to a field measurement, then an effective simplified validation of visual AtoN performance based only on the output of the light source can be obtained.

# Concept

## Requirements

The method of measurement needs to be relatively simple, quick to perform and should not depend on an extensive range of equipment. As such, the method proposed below will not directly measure the performance of the AtoN, but rather determine the consistency of performance of the light source. It does not take into account optical effects or degradation of the light path beyond the light source due to dirt or dust accumulation.

## Potential Measurement Devices

The correct method of determining the total output of a light source is to mount it inside an integrating sphere that captures all the light emitted by the light source. This is then measured to determine the luminous flux, in lumens, generated by the light source. It is not practical to transport and fit an integrating sphere in a lighthouse, so this has been discounted as an option.

Another method, as used in the GRAD (General Lighthouse Authorities Research and Development) light measurements system, requires a luminance meter that measures the luminance of the light over a known angle. However, the angle is normally very shallow – typically 2 degrees or less. Inside the lighthouse optic, there may not be enough distance between the measurement location and the light source to get the whole light source inside the measurement aperture, causing errors in the measurement. However, it should be noted that the process described below would work in principle with a luminance meter.

An example of an illuminance meter with a separate sensor head. Source: Konica Minolta

The method described below relies on an illuminance meter, also known as a luxmeter. This device measures the amount of light falling on the sensor, and displays the result in lux (Figure 3). The disadvantage of an illuminance meter in this application is the sensitivity to light from a wide angle, and so it is more sensitive to fluctuations in ambient light levels than a spot luminance meter. Therefore, this must be taken into account in the method.

It is critical that good record keeping is in place to ensure consistency in the measurement locations and of the measured values. This is to minimise uncertainty in the results and maximum usefulness to the maintenance crews.

## Method

The proposed method is as follows:

- Mark locations inside the optic to use as the measurement points for the sensor. These should be at the same horizontal plane as the centre of the light source. It is recommended that at least three locations around the light source are used to get a good overview of the light source performance.

At each location:

Arrange the illuminance meter sensor so that it is pointing directly towards the light source. In the case of flashing light sources, it may be necessary to configure the illuminance meter to integrate the readings over the length of a flash to ensure consistency of results. Record the configuration used as it will be necessary to use the same settings for subsequent measurements.

With a black card behind the light source (to minimise the effect of reflections from glazing on the opposite side), take several readings of the light source while it is off. This is the reference level which takes into account the ambient light level. See Figure 4.

Measuring the ambient light level

With the black card still in place, take several readings of the illuminance with the light source on. See Figure 5.

Measuring the light source

Subtract the average illuminance value while the light source is off from the average illuminance value while the light source is on. This difference in value represents the amount of light emitted by the light source. Some advanced meters may have this referencing facility built into the device, in which case, the difference can be read directly from the screen.

- Do not use the same reference illuminance for all locations – each location requires the reference illuminance to be measured individually.

Record the illuminance difference value for all locations.

## Limitations

This is a highly simplified means of measuring the performance of the light source. Due to the nature of the instrument, it will be sensitive to changes in the ambient light. Whilst the “off” reference reading will help to mitigate most of these issues, some amount of uncertainty remains. This, to some extent, can be reduced by increasing the number of measurement locations around the light source or by fitting the sensor inside a black-coated cylindrical shield. Where practicable, measurements should be taken at night to provide further consistency.

## Link to AtoN Performance

It is not immediately possible to determine whether an AtoN is failing to perform as published using the method described in this document. In order to achieve this, the AtoN must be coupled with details of a GRAD field measurement to understand the link between the output of the light source and the intensity of the AtoN. Therefore, by carrying out the method detailed in this document during a field measurement (with cleaned optics and glazing, ideally), it is possible to create this link. The link can then be used with subsequent light source measurement to indicate the performance of the AtoN without the need for the GRAD field measurement.

It should be noted that subsequent light source measurement would only capture degradation in the light source, and not any effects of accumulation of dust and dirt on the optics and glazing. The only method currently available to measure those effects is to carry out a full light measurement by GRAD.

## Equipment

To test the concept discussed above, a suitable illuminance meter needed to be selected that has the feature necessary to accommodate the measurement process. A Konica Minolta T-10MA Body & Head illuminance meter was chosen.

Illuminance equipment used.

In this paper, we will be discussing two different methods of mounting the sensor around the light in order to obtain consistent results. The first method involves a method of mounting the sensor at some distance away from the light source, but still within the optic. The second method is specifically for use with LED lights used by Trinity House. We shall consider each method separately.

# Validation of Method 1

The first method places the sensing head of the illuminance meter inside a plastic box. This plastic box was also fitted with a laser pointer and power supply. The purpose of the laser pointer is to help with alignment to the light source. A suitable flexible tripod was employed for the mounting of the box within an optic.

Following its construction, this arrangement was trialled within a test room environment to ensure familiarity and awareness of the goal (see Figure 7). The results were sufficiently promising to conduct tests at real lighthouses.

Test setup.

## Field trials

### Flamborough Lighthouse

The first location to trial the measurement process was at Flamborough lighthouse on a Venture 1000 W Metal Halide lamp in a first-order rotating lens (see Figure 8).

Flamborough Lighthouse light source

There were a number of physical challenges of using the equipment and approach on this lens. These were:

Trying to find a number of measuring location for the tripod, avoiding obstacles mounted around the lamp pedestal;

Keeping the light measurement box clear of the lamps, should the lamp change operate;

manoeuvring the tripod and working within the rotating optic;

Safety of the test operator.

Overcoming these challenges, a series of result were obtained once the light source had been operational for about 10 minutes and achieved full brightness.

Laser focused on the lamp envelope (note the red dot on the arc tube)

1000 W MBI lamp measurements

| Lamp 1 with 1361 operational hours |  |  |  |
| --- | --- | --- | --- |
| Position | Ambient illuminance (lx) | Lamp Illuminance (lx) | Variation about the mean |
| 1 | 45 | 183000 | +2 % |
| 2 | 48 | 168000 | -6 % |
| 3 | 49 | 192000 | +7 % |
| 4 | 47 | 175100 | -2 % |
| Mean | 47 | 179525 |  |
| Lamp 2 with 113 operational hours |  |  |  |
| 1 | 53 | 212000 | +12 % |
| 2 | 76 | 173100 | -9 % |
| 3 | 65 | 192000 | +1 % |
| 4 | 58 | 182000 | -4 % |
| Mean | 63 | 189775 |  |

The results of the test with lamp 1 and lamp 2 is shown in Table 1. The first feature to notice is that the ambient illuminance is much lower than the illuminance caused by the lamp. This means that, in this case, the uncertainty due to ambient light level is going to be low. The variation between the different positions is telling too. It is understood that the readings were, in effect, spot readings once the readings had settled down. Therefore, the variations could be due to uncertainty in the measurement, but also due to natural variations in the light output with location (e.g. some shadowing caused by wires in the lamp). It could also be due to the measurement positions being at different distances from the lamp too.

It is also evident that the lamp with the longest operational hours has just over 5 % lower mean light output than the newer lamp. For the hours they have been operational, this is expected amount of degradation in the light output, and so goes some way to validate the process.

### Whitby Lighthouse

The next location to try was at Whitby lighthouse on a cluster of three 250 W Tungsten Halogen lamps (see Figure 10) that are flashed for 1 second every 5 seconds within a drum lens.

Whitby Lighthouse light source

On this occasion some similar physical challenges were encountered as with the first trial, that is:

Trying to find a number of measuring location for the tripod, avoiding obstacles mounted around the lamp pedestal;

Keeping the light measurement box clear of the lamps, should the lamp change operate;

Safety of the test operator.

On this occasion, the sensor head was not focussed at any one lamp, but within the centre of the cluster, see photos in Figure 11.

Illuminance sensor head

Measurement arrangement

A set of result were only obtained from one of the lamp positions (main 1) as shown in Table 2, based on the similar performance between lamp arrangements shown at Flamborough.

3 x 250 W Tungsten Halogen lamp measurements

| Position | Ambient illuminance (lx) | Lamp Illuminance (lx) | Variation about the mean |
| --- | --- | --- | --- |
| 1 | 41 | 28730 | +4 % |
| 2 | 85 | 27900 | +1 % |
| 3 | 74 | 26011 | -6 % |
| Mean | 67 | 27547 |  |

Once again, the illuminance caused by the lamps under test dominated the ambient light level. We also find some variation in the illuminance with location. Again, this could be due to measurement uncertainty, variation in light output with angle or variation in distance between the light and the illuminance meter sensor head.

In capturing the data for Whitby, it was quickly identified that to make the testing easier, a flashing lamp should ideally be set to steady burning.

# Validation of Method 2

The second method of measuring the light output was specifically designed for GRAD’s LED light source. A metallic tube was created that sits over the upper heatsink of the light, and is sized as such to prevent as little lateral movement as possible. The top of the tube rests on top of the heatsink to provide a stable vertical datum for the sensor. The illuminance sensor is mounted on the side of the tube, directly in front of the LEDs (see Figure 12. The inside of the tube is mostly painted black to reduce the amount of reflections internally, and thus reducing the uncertainty in the measurement.

Illuminance sensor head

Measurement approach for LED light source

During the measurement, the LEDs can be tested individually and the tube is rotated until the reading has peaked for that particular LED. The illuminance is then recorded for that LED. Before trying the technique in the field, it was tested on the bench to ensure that consistent readings could be achieved. It was found that it does, and so the method was tried at Caldey Lighthouse.

## Field Trial at Caldey Lighthouse

This measurement approach was tried at Caldey (see Figure 13) because it has recently been fitted with a brand new LED light source. Being a new light source, the design was able to incorporate some solutions that would overcome the challenges identified at Whitby and Flamborough. For example, the ability to provide a light test mode that sets the light to a steady burning condition.

Caldey Lighthouse optic (left) and the GRAD LED light source fitted there (right)

During these trials a number of challenges were still present such as:

Variation in performance from different measurement location;

Safety of the test operator.

The results of the tests are shown in Table 3, and shows that there is consistency in light output between all the LEDs around the light source. It is noted that one reading is unusually high, but that could be due to a small misalignment of the sensor that was not picked up at the time of measurement.

Results from an 8-sided RLS 10-24 at Caldey Lighthouse

| Position | Lamp Illuminance (lx) | Variation about the mean |
| --- | --- | --- |
| 1 | 16690 | -2 % |
| 2 | 16830 | -1 % |
| 3 | 16420 | -3 % |
| 4 | 17150 | 1 % |
| 5 | 16980 | 0 % |
| 6 | 16750 | -1 % |
| 7 | 18210 | 7 % |
| 8 | 16890 | -1 % |
| Mean | 16990 |  |

This technique seem to be a good way of measuring the light output from the GRAD light source with consistent results. However, it also shows that, due to its closeness to the light source, any misalignment can cause a larger shift in the measurement that would otherwise occur with the first method described earlier.

# Summary.

The methods described in this paper provide means by which the light output can be measured with consistent results. However, the true effectiveness of the methods will only be determined with subsequent measurements using the techniques. This will take some time to determine, especially for LED light sources that degrade relatively slowly. The authors will provide an update on the technique as more data becomes available over the next 2-3 years.

It should be recognised that the trial in Flamborough Head with the old and new lamps does already show promise for the technique as, on average, it shows that the older lamp has degraded by an amount in line with the manufacturer’s data sheets. If the measurement process is sufficiently accurate, it should be possible to determine when the lamps should be changed, based on its luminous output, rather than simply based on operational hours. Given the increased scarcity of traditional lamps, it may be necessary to do this in order to conserve the stock of lamps until the lighthouse can be re-engineered for modern light sources.

It is, in theory, possible to use the technique for integrated lanterns too. However, due to the narrow vertical divergence, it is likely that much larger repeatable errors will occur since a small variation in height can result in larger variations in illuminance. This application needs to be further considered in the future.

The ENG Committee is kindly requested to consider adding methods such as those highlighted in this paper to Guidelines on light maintenance to ensure that the performance of visual signals are sufficiently  maintained by lighthouse authorities.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain

Basic luxmeters do not have this function.