Input paper:   ENG12-3.1.9

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM		X  ENG		□  PAP					X  Input

□  ENAV	□  VTS							□  Information

Agenda item 			3.1

Technical Domain / Task Number 2	2.1.3 - Develop guidance for AtoN equipment with extreme environmental conditions

Author(s) / Submitter(s)			Alison Raby / Peter Dobson

# Wave loading on rock lighthouses

# Summary

Most rock lighthouses were built a century or more ago and their survivability has been cast into doubt by anecdotal accounts of severe vibrations reported by maintenance staff. Predicted increases in sea level leading to wave impacts higher up the towers, and possible changes in the patterns of storminess also need to be taken into account. This paper describes the STORMLAMP project which is a comprehensive investigation of the rock lighthouses in the UK and Ireland. Initial modal tests on seven towers are described, and the selection of the two most exposed for long-term monitoring is justified. Then the structural and hydrodynamic modelling investigations culminate in detailed observations for the Wolf Rock and the Fastnet Rock lighthouses. A step-by-step procedure for investigating rock lighthouse responses to wave loading is provided.

## Purpose of the document

The methodologies of determining wave loading and structural response of rock lighthouses developed in the STORMLAMP project can make an important contribution to a guideline on extreme weather events.

## Related documents

‘Monitoring and modelling of rock lighthouses’ paper presented to ENG3 in 2016.

# Background

## Background to the STORMLAMP project

In 2011 Trinity House began working with the University of Plymouth to assess the structural stability of their rock lighthouses under extreme wave loading. A literature review unearthed archival sources, but identified a lack of modern data concerning loading on rock lighthouses, the structures being of novel shape compared with the more common cylindrical structures ordinarily found in the marine environment. A pilot project on the iconic Eddystone lighthouse, 20 km offshore of Plymouth, was therefore instigated.

The instrumentation solution would have to be low-power as only 30 W on average was available, but also robust because of the harsh environment, and ideally remote-controlled, due to access issues, by helicopter. So, in the summer of 2013, geophones for structural vibration modelling, and a 4 camera video system were deployed. The timing was fortuitous as the 2013/14 winter storms which created havoc in the southern UK, resulted in a rich set of vibration data, though interestingly, the maximum displacements were just a fraction of a millimetre. Data from the vibrations revealed the response frequencies of the lighthouse, which were critically important in the validation of a structural analysis model used to determine the stability of the lighthouse under wave loading.

Subsequently, Plymouth put together a more comprehensive investigation called STORMLAMP (STructural behaviour Of Rock Mounted Lighthouses At the Mercy of imPulsive waves, https://stormlamp.org.uk/), funded by the UK EPSRC. Plymouth focused on the wave hydrodynamics and long-term wave/water level statistics, University of Exeter considered the field vibration testing, and UCL undertook the structural analyses. Trinity House, Irish Lights and the Northern Lighthouse Board all contributed to this project. The scope also extended to engineering consultants interested in applying the same methods to more generic coastal structures.

## Modal testing of lighthouses

Beginning in the summer of 2016, Exeter led the modal testing of seven rock lighthouses: Les Hanois, Wolf Rock, Longships, Bishop Rock, Fastnet Rock, Dubh Artach, and back to Eddystone (Figure 1) [1].

Locations of lighthouses subjected to modal tests

This procedure revealed the structural characteristics of the towers. Over a three-year period all of the towers were fully characterised, either by forced vibration testing (FVT) or ambient vibration survey (AVS). FVT used an electrodynamic shaker to generate frequency response functions and AVS used sophisticated ‘operational modal analysis’. Figure 2 shows an example from the Wolf Rock lighthouse.

Ambient response from the Wolf Rock lighthouse

These investigations provided complementary information on the modal properties of mass, frequency, damping ratio and shape. Techniques are currently being developed to use the parameters in combination with monitored storm vibrations to infer wave loads from monitored response.

## Long-term monitoring

The Wolf Rock lighthouse, just off the southwest tip of England was one of the towers of most concern to Trinity House, based on accounts from personnel on station during storms. One of the reasons that the structural vibrations are thought to be so significant is the type of waves that can be encountered at that location. The Wolf Rock is a very constrained pinnacle of rock in a water depth of around 60 m which means that breaking north and northwesterly waves can hit the tower. This is an unusual situation as most rock lighthouses receive impacts from already-broken waves which have been affected by a more extensive surrounding reef. Also, the lowest level of Wolf Rock lighthouse construction is at approximately +2.0m CD, whereas the design water level of a storm with 1 year average return interval is +6.5 m CD, which leaves about 7 courses of the lighthouse underwater. Finally, the Wolf Rock lighthouse was the first one to be fitted with a helideck structure, and this is coming to the end of its design life. The second contender for monitoring was the Fastnet Rock lighthouse. At the time of initial discussions this had a generous diesel power supply, meaning that instrumentation constraints would not be so severe. Also, due to its location, it is fully exposed to waves from the Atlantic Ocean. However, it sits far higher above the water level than the Wolf Rock lighthouse: the first full course of blocks is about 11 m above chart datum. Both lighthouses are shown in Figure 3.

The monitoring systems comprise a tri-axial accelerometer (JA-70SA), and on Wolf Rock a stereo imaging system has also recently been deployed. This system has been designed with valuable guidance provided by Dr Alvise Benetazzo from ISMAR – CNR, Venice, who was involved in a similar system for La Jument lighthouse in Brittany, France.

Monitored lighthouses: (left) Wolf Rock owned by Trinity House, note the helideck structure on top of the tower (photo courtesy of Peter Ganderton, University of Plymouth) and (right) Fastnet Rock owned by Irish Lights (photo courtesy of James Bassitt, University of Exeter)

The video system comprises two 5-megapixel synchronised cameras, a Linux computer with a microprocessor with GPS and GSM connection for control, saving data to a Network Attached Storage drive. It has bespoke software, designed such that the system ‘sleeps’ until woken for storm measurements to conserve power usage. The wiped and heated camera housings are rated to IP66 and computing hardware housed in waterproof/crushproof case. There is also a wireless bridge network connection which links the system to the Cape Cornwall Coastwatch station, and then on to the University of Plymouth.

## Wave hydrodynamics

Wave hydrodynamics modelling features three main aspects: (i) wave forecasting, (ii) physical modelling, and (iii) computational fluid dynamics (CFD) modelling.

Extreme Bayesian analysis was undertaken to quantify waves of particular return periods to apply to the structural models ([2],[3]) and for the physical models. Some of the key findings are that the Fastnet Rock extreme wave climate shows no statistically significant effects due to the climate change. This contrasts with the Wolf Rock location which does show a small increasing trend in wave height, though there are uncertainties due to atmospheric cycles. However, even modest changes in wave loading have implications for structural response.

The physical modelling focused on obtaining data to understand wave loading on the unique geometries, and to compare with existing empirical equations. Principally though, the data would be used to validate the CFD models which could explore a much wider parameter space. The tests were undertaken in Plymouth’s COAST Laboratory which comprises an Ocean basin of up to 3 m depth, a Coastal basin of up to 0.5 m depth, and several wave flumes [4]. The Ocean basin tests featured a 1:40 scale model of the actual Wolf Rock lighthouse (Figure 4). Accurate modelling of the surrounding bathymetry and topography required stitching together of data obtained from a photogrammetry survey by Mr Peter Ganderton and a single beam echo sounder survey undertaken by Trinity House (Figure 4 (left)).

The CFD investigation used a fully nonlinear Navier-Stokes solver with the open-source code OpenFOAM. Models have been produced in both 2D and 3D numerical wave tanks. Figure 5 shows how the simulated wave front wraps around the shoal (represented by a conical frustum) and lighthouse structure (represented by a vertical cylinder).

Currently we are working on a parametric study using CFD, with a ‘surrogate model’ for wave loading on a cylinder positioned on top of an elliptical frustum. Input parameters and the range of the parameter space are being designed, with the selection of test cases following “Design of Experiment” principles. Outputs will be used to predict the forces and runup on a full range of lighthouses.

Testing of 1:40 scale Wolf Rock lighthouse in the University of Plymouth

CFD model output

## Structural modelling

The structural analysis of the lighthouse response to wave loading comprised: limit analysis (Figure 6 (left)), finite element modelling (FEM) (Figure 6 (centre)) and discrete element modelling (DEM) (Figure 6 (right)). For the limit analysis, the critical overturning and sliding loads are calculated, providing a useful tool for preliminary assessment and prioritisation of detailed analysis and interventions [5]. The FEM, using Abaqus 6.14 (Dassault Systèmes) tested a number of different approaches, determining that using a homogeneous continuous material was unsuitable for strong impacts, but using a discontinuous structure would model opening of the joints and reveal rocking behaviour. However, the discontinuous structure came at a high computational cost. The DEM which used 3DEC 5.0 (Itasca Inc.), allowed for detachment and separation of the blocks, essential for the non-continuous blocks of the lighthouses. This allows for the accurate representation of the actual inter-locking granite blocks.

Structural modelling: limit analysis, FEM and DEM

A particularly important aspect of the structural analysis was the modelling of the 1970s helideck structure, used for accessing the lighthouses. Information about these more contemporary structures was obtained directly from the original designers, with further anecdotal information from some of the lighthouse keepers who were stationed at the lighthouse during that period of time. The structural analysis combined DEM of the granite tower with FEM of the steel helideck [3]. Analysis of these structures produced a number of important findings: Fastnet lighthouse can withstand the worse Atlantic storms due to its massive structure and its position on the rock, well above the mean sea level [2]. The 1:250 year wave impact on Wolf Rock causes intense rocking and opening of the horizontal joints in the model but it is not expected to cause collapse. However, it would not be able to survive the impact of that wave without the presence of vertical keying. Finally, the ageing Wolf Rock helideck exhibits measurable displacements from wave loading, with associated high tension forces and plastic strain. There are large axial forces on vertical gallery posts too, enough to damage bolts at the base of the helideck and the encastrated bars [3].

# Discussion

The STORMLAMP project has brought together a number of engineering disciplines to shed light on the complexities of the wave loading and subsequent response of rock lighthouses. The same methodologies may be applied to other similar structures in the marine environment. To facilitate this, the project has produced a step-by-step guide to how the procedures might be implemented elsewhere, as shown in Figure 7 overleaf.

# References

Brownjohn, J., Raby, A., Bassitt, J., Antonini, A., Hudson, E., & Dobson, P. (2018). Experimental modal analysis of British rock lighthouses. Marine Structures, 62, 1–22.

Antonini, A., Raby, A., Brownjohn, J. M. W., Pappas, A., & D’Ayala, D. (2019). Survivability assessment of Fastnet lighthouse. Coastal Engineering. https://doi.org/10.1016/j.coastaleng.2019.03.007

Raby, A.C., Antonini, A., Pappas, A., Dassanayake, D.T., Brownjohn, J.M.W., D’Ayala, D. (2019). Wolf Rock lighthouse: past developments and future survivability under wave loading. Philosophical Transactions of the Royal Society A doi.org/10.1098/rsta.2019.0027.

Dassanayake, D.T., Raby, A., Antonini, A. (2019). Physical Modelling of the Effect of Shoal Geometry on Wave Loading and Runup on a Cylinder, In: Proc. ASCE Coastal Structures Conference, Hannover.

Pappas, A., D’Ayala, D., Antonini, A., & Raby, A. (2018). Rock mounted iconic lighthouses under extreme wave impacts: Limit Analysis and Discrete Element Method. ICCM2018, Rome.

# Action requested of the Committee

The Committee is requested to:

- Review this document

- Consider it to be input as a background in the Draft Guideline the extreme environmental conditions on Aids to navigation - Task 2.3.1

- Alternatively consider the input paper as input for a new guideline with focus on this topic of structural assessment- New task

Step-by-step procedure for investigating wave loading on rock lighthouses

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain