Input paper: ENG12-n.n.n

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM		X  ENG		□  PAP						□   Input

□  ENAV	□  VTS								X  Information

Agenda item 	(from agenda)

Workplan Task Number / Technical Domain 2	…………………………………

Working Group					WG 2

Author(s) / Submitter(s)			China MSA

Use of Modern Equipment on Heritage Lighthouses

- Summary

Through rational utilization of human resources and equipment resources of heritage   lighthouses, the modernization and upgrading of heritage lighthouses are carried out, which provides ideas for the realization of modern management and multi-function construction of heritage lighthouses.

  - Purpose of document

The proposal combines the current situation of heritage lighthouses in China and proposes the application of modern equipment for heritage lighthouses, which will provide some reference for IALA to Draft Guideline on Use of Modern Equipment on Heritage Lighthouses. (ENG9-9.3).

  - Related documents

ENG9-9.3 WP WG2 Draft Guideline on Use of Modern Equipment on Heritage Lighthouses

- background

Because of their unique architectural style and historical background, heritage lighthouses have many functions such as navigation and lighting, geographical coordinates, historical heritage, cultural education, military defense, and declaration of sovereignty, etc. Their construction and protection have always been the focus of attention of all countries in the world, and also an important issue in the construction of E-NAV. With the rapid development of radio navigation facilities, the application of modern integrated navigation systems in ports and ships is becoming more and more common, and the navigation value of heritage lighthouses is gradually weakened. With the development of science and technology, modern equipment is rapidly developed and widely used in various fields. Compared to visual beacons, radio beacons usually have a larger coverage area, higher accuracy, and wider temporal and spatial continuity. Therefore, it is necessary to explore how to combine the development of science and technology, make full use of the advantages of the lighthouse itself, the application of modern equipment, and build a lighthouse with multi-function and adapt to the characteristics of the times.

- Discussion

This proposal mainly discusses what kinds of modern equipment should be used to upgrade heritage lighthouses and what functions that these modern equipment can achieve. It can be considered from two aspects: one is to improve management efficiency internally. Through the use of modern information management methods, it lays the foundation for the realization of unmanned energy-saving and environmental protection operation and maintenance mode of lighthouses. Secondly, it can improve the service level. Through the installation of modern equipment, the multi-function construction of the lighthouse can realize the transformation from the heritage single navigation and lighting function to the modern comprehensive navigation aid system.

  - Modern management of Heritage lighthouse

    - Modulator monitoring

In order to reduce the frequency of technical maintenance by staff directly on the lighthouse, a telemetry remote control terminal can be installed on the lighthouse to monitor the quality of the lighthouse, lighting status, current and voltage, and auxiliary equipment, and establish a corresponding communication network to transmit the working parameters collected by the telemetry remote control terminal to the beacon management center remotely, and analyze and process the data through the supporting software to achieve real-time monitoring of the lighthouse and fault alarm, playback, storage and other functions, and send instructions for remote maintenance of the lighthouse to improve the emergency efficiency and lay the foundation for unattended lighthouse.

    - Energy Management

For lighthouses with access to utility power, utility power is preferred as the energy source and is operated in strict accordance with the electrical system installation standards.... For remote lighthouse in accordance with the lighthouse location of the power supply, comprehensive consideration of its fuel and equipment transportation and maintenance and other factors, can be solar power supply system. Solar energy belongs to the clean energy, its power generation device has the advantages of fuel saving, no noise, no emissions, etc., to achieve the lighthouse energy saving and environmental protection mode of operation. At the same time, the power generated by the solar panels can be stored in the battery for unified management of the battery, and once the voltage and current abnormalities occur, the alarm can be realized through the detection system, and the management personnel can carry out part of the maintenance work remotely.

    - Work environment monitoring

In order to protect the lighthouse from destruction or theft, a radar/infrared electronic detector or high-definition video monitoring system can be installed around the lighthouse to monitor the destruction of the lighthouse by collecting video image information from the lighthouse and its surroundings. At the same time, through the intelligent video module embedded in the front-end processing equipment, continuous analysis of monitoring images, identify suspicious activities, so as to achieve the purpose of early warning. Lighthouse management personnel can work through the mobile terminal, without the limitations of the network and office space, real-time grasp of the operation of the lighthouse, improve the emergency and management efficiency of the lighthouse.

  - Multi-function construction of Heritage lighthouse

    - Construction OF MULTIPLE  AIDS  TO  Navigation  Function

By making full use of lighthouse equipment resources and human resources, installing radar transponders, physical AIS beacons, AIS base stations, RBN-DGNSS stations, VHF base stations, radar stations and other radio aids to navigation facilities, improving the accuracy and reliability of lighthouse aids to navigation functions, further expanding the types and scope of beacon aids to navigation services and building a modern integrated aids to navigation system, as shown in Figure 1.

Figure 1 Sketch map of modern integrated navigation system

    - Social service functions：

Similarly, using the tall tower body of a heritage lighthouse and the open and unobstructed surrounding environment to install a public network communication base station, which can provide communication signals to and from ships. At the same time, taking advantage of the important geographic location of lighthouses, the hydrological and meteorological information near the lighthouse can be monitored by meteorological and hydrological sensors, which is not only conducive to grasping severe meteorological information such as storms in advance and protecting the lighthouse, but also to issuing meteorological warnings to ships, giving full play to the social public welfare attributes of lighthouses and providing diversified comprehensive maritime security services for more seafaring users.

    - Cultural function：

The lighthouse museum and lighthouse park can be set up according to the cultural landscape, scenic spots and traffic conditions in the region where the lighthouse is located, and the main buildings or lenses components with historical cultural value can be displayed. The humanistic value and historical cultural value of the heritage lighthouse can be fully developed, and the navigation culture and beacon culture can be publicized to the public.

  - Key issues in the installation of modern equipment for Heritage lighthouse

The key issues in modernizing equipment for heritage lighthouses consist in space and energy. Depending on the geographical location of the heritage lighthouse, lighthouses can be divided into three categories: land lighthouse, island lighthouse and underwater lighthouse.

- Usually，the land lighthouse can connected to the utility power, and it has enough extension space to realize the installation and use of all the necessary modern equipment.

- Island lighthouse is generally located in remote islands with limited energy and space conditions. Appropriate modern equipment should be selected for installation and use. Small radio installations, such as radar transponders, can be powered by solar power, while large radio installations can be powered by a combination of solar and diesel generators or solar and wind power. For space conditions, the tower body and surrounding buildings should be reasonably planned, and the necessary modern equipment should be selected for installation.

- Underwater lighthouse is similar to the Island lighthouse. It mainly relies on the tower space to accommodate modern equipment. Both energy and space need to be reasonably planned and equipped with the necessary modern equipment.

- references

[1] IALA Guideline 1007-Lighthouse-Maintenance.

- Action requested of the committee

The committee is requested to review the proposal and take action as appropriate.

ANNEX1…  Telemetry and telecontrol transformation scheme and technical parameters

ANNEX 2…Power transformation scheme

ANNEX 3…Heritage lighthouse HD video surveillance system installation solutions

ANNEX 4…Modern equipment configuration for heritage lighthouses

ANNEX 5…Modern Equipment Configuration for DaguLighthouse

ANNEX1...  TELEMETRY AND TELECONTROL TRANSFORMATION SCHEME AND TECHNICAL PARAMETERS

- Telemetry and telecontrol transformation scheme and technical parameters

To realize the telemetry and telecontrol of heritage lighthouses, a corresponding communication network needs to be established. Considering the complex lighting quality, new light source and the protection of heritage lighthouse resources, the main scheme of Telemetry and telecontrol transformation is to add external telemetry and remote control equipment.

- classified  Modulator

The proposal divides lighthouses into two types according to the power of their modulator: low power (less than 200W) and high power (more than 200W).

- Telemetry and telecontrol transformation scheme of low power lighthouse

For the low power lighthouse, an external telemetry and telecontrol device S6901 can be used. This equipment is a split-type and consists of telemetry and telecontrol terminal communication control box and communication antenna, as shown in Figure 2 and Figure 3. The communication control box package includes communication unit, control unit, detection unit and metal shell. This type of terminal adopts advanced control system, which can automatically detect the properties of voltage, current, light status, temperature and humidity, ambient light, rotation period, etc. Through corresponding communication mode, it can realize the remote monitoring of lighthouse terminal.

- TELEMETRY AND TELECONTROL TRANSFORMATION SCHEME OF high POWER LIGHTHOUSE

The high power lighthouse needs to be equipped with external telemetry and telecontrol equipment such as S8102. The communication, control and detection units contained in the communication control box are high-power units, and components such as mutual inductors and ACDC need to be installed. The communication control box and communication antenna are the same as S6901. External telemetry and telecontrol equipment has strong adaptability to different lighthouse modulator, easy to install and maintain. All components are resistant to corrosion and aging, and the protection grade is IP65. The detailed parameters are shown in Tables 1 and 2.

Figure 2 S6901/S8102 Communication control box      Figure 3  S6901/S8102 Communication antenna

Tables 1 The technical parameters of S6901

| Input Voltage | DC9~36V |
| --- | --- |
| Quiescent Dissipation | 50mA@12V |
| Maximum Load | 240W@12V |
| Remote Measurement Mode | SMS/GPRS/CDMA/Beidou navigation system/NB-IoT |
| Location Mode | / |
| Level Of Protection | IP67 |
| Operating Temperature | -35℃~ +65℃ |
| Weight | 5kg |
| Installation Dimension | 440*300*220mm |

Tables 2 The technical parameters of S8102

| Input voltage | AC220V±20V |
| --- | --- |
| Input Voltage | 70mA@12V |
| Quiescent Dissipation | 800W@220V |
| Maximum Load | SMS/GPRS/CDMA/Beidou navigation system/NB-IoT |
| Remote Measurement Mode | / |
| Location Mode | Undervoltage, fault, location deviation |
| Level Of Protection | IP67 |
| Operating Temperature | -35℃~ +65℃ |
| Weight | 5kg |
| Installation Dimension | 440*300*220mm |

ANNEX 2…POWER TRANSFORMATION SCHEME

- utility power

If stable and good quality utility power can be connected, utility power is preferred as the primary energy source, and diesel generators can be used as emergency backup power and implemented in accordance with electrical installation standards.

- Solar power system

If the lighthouse cannot be connected to the utility and the safety level requirements of the electrical system are not strict, the electrical facilities use less electricity, a solar power system can be used. If the lighthouse is installed with high-powered modern facilities, space permitting, the original solar power system can be expanded or a new system can be built on the basis of the original solar power system.

- Solar power system＋Diesel generator

If the lighthouse adopts solar energy system to generate electricity, the diesel engine can be used as backup power source, which can form a complementary power supply system combining solar power and diesel engine. In order to eliminate the influence of the radio power supply system on the original power supply system of the lighthouse, the power supply system of the large radio facilities should be built separately.

- Solar power system＋wind energy

In order to achieve the purpose of environmental protection, wind energy and solar energy can be combined to connect the same battery pack at the same time, so that both can play a role at the same time. When there is no sunlight at night or rainy days, wind power can be generated. Both of them play a role at the same time to further improve the stability and reliability of the power supply system, realize the all-weather generating function, and one set of damage or extreme weather, keep the power supply system to continue to work.

ANNEX 3…INSTALLATION SCHEME OF VIDEO MONITORING SYSTEM

- system composition

General video surveillance system consists of front-end monitoring system, wireless transmission system and back-end management center. The front-end monitoring system consists of video acquisition equipment such as network cameras and network ball machines, which collects video information and then transmits the signal to the corresponding communication base station via 3G/4G signal or microwave system to achieve signal transmission with the back-end management center. The system energy can be powered using the energy system of the lighthouse.

- Construction scheme of front-end monitoring system

The front-end monitoring system can adopt video acquisition equipment such as ball machine and fixed camera, which should have anti-corrosion and lens cleaning functions, and be equipped with protective measures such as fixed equipment, lightning protection and power voltage regulation.

In the construction position, the ball machine can be installed outside the lighthouse platform, monitoring the lighthouse's surrounding environment and navigation channel by using it’s rotating radius. The fixed camera can be installed according to the actual situation, such as the arrangement inside the modulator, the light house or AIS base station lighthouse, monitoring the light operation of the lamp, the operation of light house and AIS base station equipment. As multiple camera equipment will cause the superposition of power consumption, the number of camera equipment should be controlled as much as possible.

- Construction scheme of wireless transmission system

The wireless transmission system can be realized by 5.8G microwave or 3G/4G networks. The microwave wireless transmission and 3G/4G network can be selected to cooperate with the appropriate operator after site survey and signal testing. At the same time, select a network DVR with 3G/4G wireless transmission, and installed in the interior of the lighthouse, to achieve video image processing, video image transmission, video recording and preservation, alarm information collection and other functions.

- Construction scheme of back-end management center

The back-end management center can use the supporting software of the monitoring system to obtain real-time video and historical video of the front-end equipment. If the unit has already built the monitoring system and platform, it can be connected to the original monitoring system and platform for unified management.

ANNEX 4…MODERN EQUIPMENT CONFIGURATION SCHEME

- land lighthouse

Tables 3 Application scheme of modern equipment for land Lighthouse

| Space and energy conditions of heritage lighthouse |  |  |  |
| --- | --- | --- | --- |
| Space |  |  | There is enough extension space for the installation of large modern equipment and auxiliary equipment. |
| Energy |  |  | If there is a utility power and the quality is good, adopt it. |
|  |  |  | No utility power, solar and diesel generators. |
|  |  |  | If space is enough, use solar and wind combined. |
| Name of modern equipment |  |  | Installation conditions and feasibility. |
| Modern lighthouse management | Telemetry And Telecontrol System |  | Suitable for installation. |
|  | Video Monitoring System |  | Suitable for installation. In particular, if lighthouses are close to densely populated areas or have tourist value, real-time monitoring shall be carried out to prevent people, vehicles from illegally entering or damaging the facilities around lighthouses. |
| Multi-function construction of lighthouse | Navigation Services | AIS Base Station | Suitable for installation. Send and receive AIS data to provide high-precision positioning service for AIS slipway. |
|  |  | AIS Beacon | Suitable for installation. |
|  |  | RBN-DGNSS | If space and equipment conditions allow, suitable for installation and can make full use of the manpower and equipment resources of the lighthouse. |
|  |  | Radar Station | Suitable for installation. |
|  |  | VHF Base Station | Suitable for installation. |
|  |  | Radar Transponder | Suitable for installation. |
|  | Social Services | Communication Base Station | Suitable for installation. Sufficient energy is available to transmit public network signals that can provide communication signals for ships and lighthouse equipment. |

- Island lighthouse

Tables 4  Application scheme of modern equipment for Island Lighthouse

| Space and energy conditions of heritage lighthouse |  |  |  |
| --- | --- | --- | --- |
| Space |  |  | Building space is relatively narrow, reasonable planning and utilization should be made on the premise that the heritage body of the lighthouse will not be affected. |
| Energy |  |  | Similar to land lighthouse |
| Name of modern equipment |  |  | Installation Conditions And Feasibility |
| Modern lighthouse management | Telemetry And telecontrol System |  | Suitable for installation, just use solar panels. |
|  | Video Monitoring System |  | Unattended island, suitable for installation. At the same time, it can observe the channel and navigation information around the lighthouse. |
| Multi-function construction of lighthouse | Navigation-aid Services | AIS Base Station | Space and energy is limited，unsuitable for installation. |
|  |  | AIS Beacon | Suitable for installation. |
|  |  | RBN-DGNSS | Space and energy is limited，unsuitable for installation. |
|  |  | Radar Station | Suitable for installation. |
|  |  | VHF Base Station | Space and energy is limited，unsuitable for installation. |
|  |  | Radar Transponder | Suitable for installation. |
|  | Social Services | Communication Base Station | Suitable for installation if necessary. |
|  |  | Hydro-Sensor | Suitable for installation. Collect hydrometeorological information near the lighthouse. |

- Underwater lighthouse

Tables 5 Application scheme of modern equipment for Underwater Lighthouse

| Space and energy conditions of heritage lighthouse |  |  |  |
| --- | --- | --- | --- |
| Space |  |  | Mainly rely on the lighthouse tower space installation equipment. |
| Energy |  |  | There is not enough space to install large solar and wind power generation equipment, which can be combined with solar energy and diesel generators. |
| Name of modern equipment |  |  | Installation conditions and feasibility. |
| Modern lighthouse management | Telemetry And Telecontrol System |  | Suitable for installation, Solar panels can be used for power supply. |
|  | Video Monitoring System |  | Suitable for installation if space conditions permit. It can effectively monitor the collision of ships with lighthouses. |
| Multi-function construction of lighthouse | Navigation-aid Services | Large Navigational AIDS | Space and energy is limited, unsuitable for installation. |
|  |  | Small Navigational AIDS | Installation is optional according to the space and energy conditions of the tower. |
|  | Social Services | Communication Base Station | Installation is optional according to the space and energy conditions of the tower. |
|  |  | Hydro-Sensor | Suitable for installation. Collect hydrometeorological information near the lighthouse. |

ANNEX 5…TAKE DAGU LIGHTHOUSE AS AN EXAMPLE

Dagu lighthouse is the first offshore lighthouse designed and built by China and located on the sea of Dagu estuary in Bohai Bay. The tower is a reinforced concrete cylindrical structure with a height of 38.3m and a total of 11 floors. The model of the modulator is TYH-Polaris22, with integrated main and backup lights. The control system is equipped with the functions of rotation, luminous, and telemetry and telecontrol. It supports various communication modes such as Beidou and NB-IOT. The main lights have a nominal range of 22 nautical miles. The backup light operates independently, with a range of 10 nautical miles. A radar transponder is mounted on the top to provide visual and radar AIDS to ships porting and departing Tianjin port.

Dagu Lighthouse adopts the 3G/4G dual-mode network of China Telecom, which broadcasts and sends out 360° signals through three sets of directional antennas. 3G signals are CDMA-based, with an effective coverage radius of 70km, while 4G signals are FDD-LTE-based, with an effective coverage radius of 50km, and the signal coverage basically includes key areas such as the main channel of Tianjin Port, north-south anchorages, bulk chemical anchorages, and 100,000-ton anchorages. This effectively improves the long-term poor public network signal coverage in the waters of Tianjin Port. In the future, it will provide efficient communication network support for the use of ship informationization equipment, the daily work and life of crew members, various maritime navigation facilities and information collection equipment. The system equipment adopts automatic unattended mode design and is powered by solar battery and diesel engine, which can ensure the continuous operation of the base station for 24 hours.

A special digital line has been opened from Dagu lighthouse to its subordinate administrative unit, which is used for data transmission of the lighthouse and its affiliated facilities and equipment network data transmission, committed to serve the lighthouse hydrological and meteorological information transmission, ship monitoring, beacon status monitoring around the lighthouse, etc., and effectively play a hub role.

By establishing a lighthouse exhibition hall and developing creative tourism resources based on lighthouse elements, Dagu Lighthouse will attract public participation and further promote it as a window to promote nautical knowledge and beacon culture.

Figure4  Installation effect drawing of new lighthouse

Figure5  Dagu lighthouse communication base station equipment and Video monitoring system equipment

Figure6 Video monitoring system is installed in Dagu lighthouse to observe its surroundings

3 Input document number, to be assigned by the Committee Secretary

? Input papers should be assigned to a work task as listed in the Committee work plan which is available in  input papers. Leave open if uncertain but consider how the paper is to be processed if not relevant to a work task