Input paper:   ENG13-3.1.2.7

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□ARM	RENG		□PAP					□   Input

□ENAV	□VTS							R  Information

Agenda item 			3.1

Technical Domain / Task Number 2	…………………………

Author(s) / Submitter(s)			China MSA…………………………

The application of Beidou remote control and monitoring system in China

# Summary

In order to fully achieve remote control and monitoring(RCM) of AtoN and provide users with high quality navigation services, Beidou control and monitoring system( RCMS) has been used in China to solve the problem that the telecommunication base station in the traditional RCMS can 't receive the signal from the distant AtoN.

## Purpose of the document

This proposal introduces the application of Beidou RCMS in China and provides a useful reference for IALA.

## Related documents

Nil.

# Background

China's oceans and rivers are widely distributed and the shipping needs are huge, but the navigation channels are complex and the number of AtoN is huge. Therefore, the management of AtoN has been committed to improving the effectiveness of navigation assistance. This is inseparable from the intelligent RCMS, and the existing RCMS cannot satisfy modern navigation. With the development of Beidou technology, it has successfully realized all-weather and all-regional navigation RCM of AtoN.

# Discussion

This proposal mainly studies the practical application of Beidou RCMS, and mainly studies:

- The composition of Beidou RCMS;

- The operating principle of Beidou RCMS;

- The application method of Beidou RCMS;

- The application effect of Beidou RCMS.

## The composition of Beidou RCMS

The composition structure of the Beidou RCMS includes: Beidou remote control and monitoring equipment, Beidou communication link, background server, and data display platform. Beidou remote control and monitoring equipment is divided into Beidou terminal or Beidou lamp. Beidou lamp can be built-in voltage and current sensing device, light quality acquisition device, Beidou satellite navigation and positioning device, acceleration sensing device, temperature sensing device, humidity sensing device, environmental brightness sensing device, Beidou communication device, central control device and power supply device, etc. The Beidou communication link includes the Beidou communication device, the Beidou satellite, and the ground control terminal (the Beidou commander and the communication slave) in the remote control terminal equipment. The background server includes a data receiving module, a data analysis module, a data analysis module, a data storage module, and a Beidou command sending module. The data display platform includes a telemetry and remote control management platform, an electronic chart visualization platform and a mobile client. As shown below:

Figure 1: The composition of Beidou remote control and monitoring system

## The operating principle of Beidou RCMS

The operating principle of the Beidou RCMS is: use Beidou remote control and monitoring equipment to transfer the telemetry data of AtoN (including voltage and current, light quality, position coordinates, acceleration, temperature, humidity, ambient brightness, power supply, etc.) to generate Beidou short messages; the Beidou communication module in the remote control and monitoring device transmits the Beidou short messages from the Beidou communication satellite to the ground Beidou control terminal (receiving and transmitting messages); then the information is transmitted to the designated real-time communication server through the serial port. The real-time communication server parses the Beidou short message and stores the parsed data in the database; finally, the data is displayed on the management platform, electronic chart visualization platform and mobile client with different business rules .

## The application method of Beidou RCMS

At present, Beidou communications in the field of AtoN mainly has two application methods, as follows:

- Beidou RCM lights. The Beidou communication unit is built into the light to form a navigation light with built-in Beidou communication. The advantage of this method is that the manufacturer can monitor the voltage, current, and position information of the navigation equipment according to the needs, and increase the transmission of information, such as light. Status, battery power, light quality information, signal strength, charging voltage, charging current, lamp temperature, battery temperature, environmental brightness and other information; the disadvantage is that it is highly integrated and cannot be used with other devices.

- BeidouRCM terminal. Design an independent Beidou terminal to realize data collection by connecting to the equipment to be monitored. After the terminal collects the data, it transmits data remotely through the Beidou communication unit. The advantage of this method is that for different types of equipment, an external Beidou terminal can be added to realize remote monitoring of the equipment, and it has strong versatility.

## The application effect of Beidou RCMS

Traditional GPRS/CDMA/SMS RCM cannot fully monitor AtoN, and the telecom base station cannot receive signal from AtoN far offshore (usually more than 20 nautical miles offshore). In the past, only the inspection of patrol boats, feedback from passing ships and pilots can be  relied on. The misinformation is more, and the transmission of information is relatively lagging. Since AtoN which is usually far offshore cannot be monitored, , and ship inspection costs are too large, which is not conducive to navigation aid supervision. Therefore, the Chinese navigation aid management department makes good use of Beidou technology to replace traditional GPRS/CDMA/SMS communication according to actual needs. In this way, communication can be carried out anytime and anywhere without being restricted by the surrounding infrastructure, which completely solves the problem that the signal of the telecommunication base station cannot cover the long distance AtoN, and realizes the 100% RCM.

Beidou RCM uses short message communication, which can transmit about 628 bits at a time. The transmission information completely includes the light-emitting state, battery power, light quality information, signal strength, charging voltage, charging current, lamp temperature, battery temperature and various information such as environmental brightness; Beidou RCM with strong communication and anti-interference, while adopting S/L band satellite transmission, can penetrate the stratosphere and troposphere, and the service success rate exceeds 95%. Normal communication can also be realized under extreme weather conditions; Communication delay is about 0.5s, and the point-to-point communication delay is about 1~5s. It can quickly report the working status of AtoN to the server, thus ensuring the timeliness of the AtoN failure alarm; Beidou RCM with cost-effective require low equipment requirements, and at the same time, The networking is easy, with the command-type user machine as the core, it can quickly form a one-to-many communication network, which can be applied to AtoN on a large scale.

The Beidou RCM technology has been used in China's seas since 2017. As of January 2021, China has a total of more than 7,500 AtoN using Beidou RCM technology. The current Beidou communication method uses the Beidou second-generation communication technology, which is limited by the communication bandwidth and transmits a small amount of data. It can transmit 78 bytes of data at a time, which can basically meet the requirements for AtoN monitoring. After the application of Beidou third-generation communication technology, the communication bandwidth has increased, and the data content can be increased according to demand.

In the future, the Beidou RCMS and the AtoN will continue to be deeply integrated, which will better solve the problem of traditional AtoN isolation and data isolation. Through the advanced Beidou communication technology, AtoN and the monitoring center can be connected into a network, which can realize the navigation data issued to passing ships in real time, so that ships can share AtoN data to ensure ship safety.

In summary, the Beidou RCMS with high reliability and high accuracy has successfully realized all-weather and all-regional navigation RCM of AtoN.  It can reduce the number of ship inspections to save the maintenance costs, and completely solve the problem that the telecommunication base station in the traditional RCMS can 't receive the signal from the distant AtoN.

Figure 2: Beidou remote control and monitoring system display

# References

IALA Guideline 1008 - remote control and monitoring of aids to navigation

# Request for action by the committee

The committee is invited to note the information and take action as appropriate.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain