Input paper: 1ENG13-3.1.2.6

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□ARM	ENG		□PAP					□   Input

□ENAV	□VTS							  Information

Agenda item 			3.1

Technical Domain / Task Number 2	…………………………

Author(s) / Submitter(s)			China MSA…………………………

# Four-Season Universal light Buoy in Frozen Ports

# summery

This proposal mainly introduces the technical index, application experience, effect and economic benefit of the four-season universal light buoy in frozen ports.

## Purpose

This proposal is an information file which intended to provide some references for the design of navigation buoysand application  for frozen ports and the arctic routes.

## Relevant documents

- Version 1-Navigation services for polar regions-2013.12-003

# Context

The construction and maintenance of navigation AIDS in frozen ports in winter is a worldwide problem. In China, most of the light buoys are removedor replacedwith ice buoys in winter, and the navigation AIDS efficiency of the waterway is greatly reduced.In order to ensure theoperation of the port, it is an urgent task to develop a four-season-all-purpose light buoy, which has sufficient ice resistance and can continuously play its navigational performance in the long term.

Over the years, China has been committed to the research of this topic, has developed and applied  several buoys such as the bar, ice buoys, hinged type lamp pile and the new ice buoy,et cetera. These buoys, meet the minimum navigational aid requirements in winter, possess the maximum ability of ice-resisting, reduce the loss during the freezing period, and gradually enhance the winter navigational assistance of frozen ports efficiency.In 2015, China successfully developed and promoteda four-season, general-purpose light buoy in frozen ports, solving the problem of inconsistent navigation performance of light buoys in China's frozen ports throughout the year.

In view of the temperature, battery and other key points in Item 2.3.1 of ENG Committee Work Plan 2018-2022, this proposal introduces the application experience of four-season universal light buoy in frozen ports of the china north sea,which intended to provide some references for the design of navigation buoysand application  forfrozen ports and the arctic routes.

# brief

Four-seasonuniversal light buoys are mainly designed in terms of the appearance, structure, material, light, energy and other aspects. The technical indexes are shown in Table 1.

## 3.1          Ice resistance

The four-season universal light buoy  adopts a streamlined three-section structure with a cylinder in the middle and a cone at both ends (Figure 1 and Figure 2), which can resist abrasion and absorbing impact, and effectively resist the impact and collision of floating ice.this buoy also can float  under the ice surfacepassively when it come across the large ice chunks ,  to ensure that the location will not shift and the battery inside the body will not be damaged  (the simulation is shown in Figure 3).

## 3.2           structural stability

In order to resist the impact and collisionof iceeffectively,the four-season universal light buoyadopts a fishbone stainless steel skeleton(Figure4), which enhances the structural stability and impact resistance, Using high polymer polyethylene material, and 3D rotational moulding technology to produce floating body, which filled with polyethylene foam.

## 3.3            replacement period

Under the impact and friction of the floating ice，the conventional steel buoy is easy to wear and to rust, resulting in color fading or even color disappearance. The four-season universal light buoy adopts a  polymer polyethylene floating body, the surface color of the buoy does not fade or fall off even after a long-term use under harsh conditions such as high salinity, humidity, low temperature, flowing ice or ultraviolet radiation.  So the replacement period will be longer.

Using the solid-state lithium ion battery as the energy source, can provide the light with long period and stable voltage energy, which extend the energy replacement cycle of the buoy to more than 2 years.

The light can be configured with SMS/GPRS/CDMA/ BEIDOU and other modules, so that the data transmission is not limited by the offshore distance, and the working status of the beacon can be remotely measured and remote controlled, which can prolong the inspection and maintenance cycle.

## 3.4        LIGHT

The TYH-100 universal LED buoy light is composed of LED array light source, annular Fresnel lens and anticorrosive copper shell. This light is firm, small, strong anti-aging performance and low lens height, which can effectively resist the impact of ice flow. The horizontal beam emitted from the light has uniform focusing characteristics, and the uniformity of horizontal light distribution is greater than 85% (Figure 5).

## 3.5            ENERGY SOURCE

At present, the 254 four-season universal light buoy are divided into two power supply modes: one is powered by 800Ah lithium battery (94), which can provide energy for at least 2 years. The other is the powered by 125Ah lithium battery and 20W solar power supply system (160), which can use the  solar energy to charge the battery. The lithium battery can provide at least 3 months of energy for the lamp in the absence of sun light.

# Visual effect and radar scanning effect

The visible area of the four-season universal light buoy is ≥ 3.8m2, the light range is ≥3nmiles, and the focal plane height is ≥3m, the color is consistent with conventional light buoys. visual effect is fine (Figure 6).After long-term use under the conditions of high salinity, low temperature, flowing ice and ultraviolet radiation, the surface of the polymer polyethylene buoy does not fade or fall off, which ensure the stability of the visual effect.

The radar scanning effect of the four-season universal light buoy is between the conventional light buoy and ordinary ice buoy (figure 7 and figure 8).

# Application in China

Since 2015, 254 four-season universal light buoy have been used in Chinese frozen ports, of which only two have abnormal performance. After the test, the visualization area, visual effect and radar scanning effect are close to the Φ2.4m conventional steel buoy, and obviously better than the Φ1.4m steel ice buoy. The four-season universal light buoy significantly improve the navigation efficiency of frozen ports in winter, ensure thetraffic capacityof the frozenports, guarantee the safety of navigation. four-season universal light buoy have been recognized by the pilot, navigator andmany other users.

The bodies of those 2 abnormal buoys were broken, which was suspected to be damaged by the ship's propeller rather than impacted by flowing ice. We will continue to observe whether the four-season universal light buoy can withstand the most severe ice conditions in China in the future. Meanwhile, the    impact strength  enhancement is listed as the key problem to be solved in the follow-up research.

# Economic and social benefits

Compared with ordinary steel buoy and ice buoy, the application of 254  four-season universal light buoy  can save  acquisition cost about 4.2672 million yuan,  and save maintenance cost about 5.461 million yuan per year at the same time. Meanwhile, the storage area can be reduced by 75%.It can produce great benefits within the extended maintenance cycle, and increase the efficiency of navigation performance (attachment 2).

# 7    Freezing conditions in china northern seas and the differences with the polar regions

The china northern sea is  in the mid-latitudes of the north temperate zone, between 35 degrees north latitude to 41 degrees north latitude, near the east coast of Eurasia, there are varying degrees of icing phenomenon every winter, it is the lowest latitude frozen sea area in the northern hemisphere, the sea ice are all first year ice (generated after the winter, melted after the spring of next year), most of the ice age was in December to next  march. The ice is about 5-20 centimetres thick.

The most serious ice situation is in Yingkou Port, the ice period is from late November to late March of the next year. Generally, it starts to freeze up in mid-December and then thaw in next mid-March. The ice is generally 40-50 centimetres thick, and the biggest thickness can be more than 1 meter, it is difficult to break the ice because the hardness of the ice is great, and the flow ice can extend to the sea for 40-60 nautical miles.

The ice is thicker in the polar regions than it is in the northern seas of China, and can move or damage navigation marks more severely.

The winter in China northern sea is about 4-5 months long, the buoy only shines at night and is supplemented by the solar system during the day. On the other hand, The polar regions are at high latitudes, they are dark for most of the time in half the year. In this case, all equipment of the buoy need to run continuously for several months under the rated 24 hours condition, and during this period, there is no automatic  energy supplement, which requires the configuration with a larger energy  capacity for the buoy.

# 8   discussion

NONE

# 9   reference

NONE

# 10   request for the committee

The committee is invited to note the information and take action as appropriate.

Attachment 1: Technical index and figures

Attachment 2: Economic and social benefits

Attachment 1: Technical index and figures

Table 1 technical index

|  | technical index |
| --- | --- |
| Visual effect | visible area≥3.8m2, light rang≥3nmiles,focal plane height ≥3m. |
| Ice resistance | The body does not shift or damage under the condition that the thickness of floating ice is less than 20cm and the area of ice is less than 100m2. |
| environment | water depth range: -10m ~ -20m,working temperature: -35C° ~ +40C°. |
| maintenance | Maintenance cycle≥2 years, easy for small beacon vessel to inspect and maintain |

Figure 1 Appearance                                                                      Figure 2 Structure

Figure 3 Simulation of  Ice resistance

Figure 4 Skeleton

Figure 5 TYH-100 universal LED buoy light

Figure 6 port hand buoy(red can) and starboard hand buoy(green cone)

Figure 7  Visual effect comparison

Figure 8 radar scanning effect comparison

Attachment 2: Economic and social benefits

- Economic benefits

1.1 Direct  economic benefits

Table 2 expenses statement

|  |  | Ordinary buoy |  | Four-season universal light buoy | difference |
| --- | --- | --- | --- | --- | --- |
| Acquisition cost |  | Ordinary steel buoy | 84.8K | 168K | 16.8K |
|  |  | Ice buoy | 100K |  |  |
| Maintenance operation cost | operation expenses | 16.4K |  | 4.1K | 21.5K |
|  | Transportation expenses | 1.06666K |  | 0.26666K |  |
|  | Maintenance expenses | 8.388K |  | 0 |  |

The application of 254  four-season universal light buoy  can save  acquisition cost about 4.2672 million yuan,  and save maintenance cost about 5.461 million yuan per year at the same time.

1.2 Indirect  economic benefits

1.2.1   Storage area

Each buoy use  60m2of storage area.

Ordinary buoy: 60m2×254×(1.5+0.5）=30480m2

Four-season universal light buoy: 60m2×254×0.5=7620m2

The storage area can be reduced by 75%.

1.2.2 Navigation security

The application ofFour-season universal light buoyimplements navigation performance consistent throughout the year in the frozen ports. There is no need for the change in the spring and winter, its ice resistance performance can guarantee the normal navigation performance, and reduce the operation frequency, which effectively improve the navigation environment, ensure the high efficient production, and bring immeasurable economic benefitsfor the port logistics economy .

2  social benefits

The four-season universal light buoy is made ofUV resistant and environmentally friendly highpolymer polyethylene, which effectively restrains the Marine biological pollution and eliminates the  maintenance of steel buoy(clean up and rust removal).Thepower system and lamp can be remote control and maintenance-free for two consecutive years, greatlyreduce the frequency of inspection and the labor intensity ofmanagement personnel.

The four-season universal light buoy has a good ice resistance performance, stable structure, and strong impact resistance, the color of the surface do not fade, or fall off. All these features guarantee its visual effect, radar scanningeffect and the ability to resist ice, which implements the consistency through four seasons.Moreover, with a variety of function integration, The four-season universal light buoyimprove the whole guided navigation buoy system efficiency.

Leave open if uncertain