# Working towards a Draft New IALA Recommendation

On the Creation of IALA Stakeholder’s Harmonized User Requirements (ISHR)

# Introduction

Core idea:

The IALA Stakeholder’s Harmonized User Requirements (ISHR) is part of the “IALA e-Navigation Stack”.

The ISHR is not meant to be “an IALA wish list to Santa Klaus”, but rather as a well structured, traceable, consistent, both persistent and “living” collection of individually identifiable user requirements, as derived from IMO user needs documentation, IALA committee work, business case studies, and the like. The ISHR intends to be conformity with well established international Requirement Management methods and standards, such as those developed at ISO. This implies that individual Harmonized User Requirements will be identified by a unique identifier as described in this document.

The ISHR also includes the [complete] list of “operational services”, and how they can be traced to the appropriate user requirements.

- This document is intended to provide the framework description and set the rule base for the creation of IALA Stakeholder’s Harmonized User Requirements (ISHR)

- It is not meant to create or reflect individual ISHRs as such! This needs to be done in different documents (compare draft IALA documentation plan).

# IMO’s user needs and how they inform the ISHR

--- Explain, what the relationship between IMO’s stated user needs and the ISHR are

--- They are not identical, but the ISHR are informed by IMO stated user needs. It should be noted, that user needs for the shipboard side may require in ISHR of the shore side, too.

--- Besides the IMO defined user needs there are additional ISHR stemming from IALA’s present mandate (valid also without e-Navigation)

# User requirements architecture and the concept of an Operational e-Navigation Service

Certain important key terms are introduced here. These key terms comprise:

- “architecture” for user requirements, tasks, information needs, and information items, abbreviated for the purpose of this document “user requirements architecture”

- the concept of an Operational e-Navigation Service

- the distinction between “information” and “data”, and the role of Human-Machine-Interfaces therein.

## Existence of a user requirements architecture

- there is such a thing like an architecture for user requirements, tasks, information needs, and information items, which is called – for short – “user requirements architecture”

## Re-use of terms in different contexts

It has been observed that identical terms are re-used in different contexts. This impedes the mutual understanding and needs to be overcome when the e-Navigation intended harmonization is to be achieved.

Considering the context in which a specific key word is used in may be the key to a correct mutual understanding. Three different contexts have been identified:

- “User requirement domain”, including in particular mariners themselves;

- “Technical system architecture domain”, including system engineers;

- “Equipment manufacturers”.

The following Table correlates the meaning of the identical key words “user requirements”, “architecture”, and “implementation” used in the different contexts. The correlation can be seen, when reading the lines of the Table. The respective context is given by the columns.

| User requirement domain (mariners) | Technical system architecture domain (system engineering) | Equipment manufacturers |
| --- | --- | --- |
| “user needs/requirements” and their “architecture” | “user requirements” | “(user) requirements” |
| “implementation” | “architecture”, i.e. “e-Navigation system architecture” |  |
|  | “implementation” | “architecture” |
|  |  | “implementation” |

Table: key words of the e-Navigation and their different meaning in different contexts

Example: When system engineers of Aids-to-Navigation administrations, who are assembled at the Architecture group of the IALA e-Navigation committee talk about “architecture” or “e-Navigation system architecture” they operate within the field, which would be called “implementation” by mariners only, but which would be considered “requirements” by the equipment manufacturers the Aids-to-Navigation administration would commission to actually build the systems they devised. Equipment manufacturers would themselves devise a even more detailed “architecture” and would themselves call for “implementation”, namely e.g. for software programmers.

Conclusion: By observing the context a key word is used in, confusion could be avoided, when the same key words are used in the discussion on e-Navigation. This contributes to mutual understanding and to harmonization.

## Existing proposals for user requirements architectures

--- Maybe this section will not be needed eventually

There are some existing proposals for user requirements architectures. They are introduced in the following sections.

### 5-key-word methodology as derived from e-Navigation definition

- 5-eNAV-key word methodology: compare IMO NAV 55 / 11 / 4 (UK input => David Patraiko)

- This appears the most likely candidate to structure the ISHR as well, because it has been approved by IMO

### Vessel Traffic Management as a functional framework

The definition of VTM concept is as follows:

“Vessel Traffic Management is the functional framework of harmonized measures and services to enhance the safety, security, efficiency of shipping and the protection of the marine environment in all  navigable waters” (compare VTS29-output9=eNAV6-8-22).

The very definition of the VTM identifies it as a “framework”, i.e. architecture, of operational services, i.e. of services within the user domain. Therefore, the VTM concept is a point in case.

## Harmonization of user requirements using a commonly agreed user requirement architecture

### Introduction

- That should be the goal! Refer to IMO e-Navigation strategy

### The “task - information need - information item” approach

- explain that approach,

- compare WG1’s work on that; include that description

- conclude with the derivation chain:

  - high level user needs

  - tasks => organized in a tree

  - information needs associated with tasks

  - information items as subsets to the information needs

- to the e-Navigation system engineers, it is the last step that is the most important to their work.

### The distinction between information and data

From an e-Navigation system engineering point of view the bottom line of any such commonly agreed user requirement architecture is, that eventually machines will need to deliver data to and fro between human users and internal or external machine-based entities. This is called “implementation” in the context of the discussion of the user requirements (see above).

There appears to be common understanding regarding the derivation of information items from “high level user needs”, as given in the following figure:

--- use updated figure as introduced by IMO e-Navigation correspondence group

  - User requirements in general (top level) 		(information domain)

  - Tasks of the user 					(information domain)

  - Stated information needs				(information domain)

  - Individual information objects 			(information domain)

  - Interaction by Human-Machine-Interface (i.e. with Operational Presentation Surface)

  - Maritime data objects  				(data domain)

  - Maritime data encoding-for-data exchange 	(data domain)

Note: Such an internationally agreed common architecture for the user needs or the user domain could be achieved by extensively employing the above concept of information objects with the following concept of the “operational service”. The “operational services” identified as such could be arranged in a tree structure, as well.

## The concept of an “Operational Service”

The definition of an Operational e-Navigation Service, or Operational Service for short, is as follows:

An Operational e-Navigation Service, or Operational Service for short, is an activity or a combination of activities, provided by one or more stakeholders within the maritime domain, for the benefit of maritime stakeholders to support their mission at all levels, which has been harmonized and brought into accordance with the goals of IMO’s e-Navigation concept.

An Operational Service is characterized by providing information to users. It is such distinguished from a technical e-Navigation service, which is providing data (see below).

One well established precedence for this concept of operational e-Navigation services in use by IMO are “Vessel Traffic Services (VTS)”. By definition, “VTS” is a combination of three Operational Services already identified by IMO as such, namely “Information Service (INS)”, “Navigation Assistance Service (NAS)”, and “Traffic Organization Service (TOS)” [SOLAS-V, Rule 12, and IMO A.857(20), Annex 1, §9]. Another example is the so-called “Allied Services” [IMO A.857(20), Annex 1, §10].

## Link between ISHR and the technical layers of the “IALA e-Navigation stack”

For considering the technical layers of the IALA e-Navigation stack only the following items would be required, using the above terminology, i.e.

- “Stated information needs” and “stated information items” in the user domain, for what machines are supposed to deliver to the users.

- Identification of the “Operational Services” to be supported by the Initial technical e-Navigation architecture as a justification.

with the following definitions

- “stated” means “defined or described in an internationally recognized document”, such as IMO resolutions and  performance standards

- “information needs”: summary statements regarding a field of information, for instance: “dynamic information of a vessel”

- “information items”: individual information objects, such as “position of a vessel” in the above example; the information items for a subset to the information needs.

The “Maritime Information Objects (MIO) tree” is a good example – once internationally recognized – of “stated information needs/items”.

Example for the link between user needs and system architecture: The development of the AIS was justified by the “operational services” “collision avoidance”, “VTS”, and “Ship reporting”. The stated information items (compare IMO Performance Standards for AIS) were for instance ship’s ID, position, speed, course, etc., which were subdivided into three categories of stated information items, namely “dynamic information”, “static information” and “voyage-related information”.

# --- Rule Base for ISHR etc. ---

Note: This phrase was generalized using IALA’s VTM concept; VTS29-output-8=eNAV6-8-21.