AISM Association Internationale de Signalisation Maritime     IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

IALA Recommendation

[e-NAV-???]

On

the IALA Common Shore-based System Architecture (CSSA)

[working towards Edition 1.0]

[December 2013]

Initial Release

10, rue des Gaudines

78100 Saint Germain en Laye, France

Telephone: +33 1 34 51 70 01    Fax: +33 1 34 51 82 05

e-mail: contact@iala-aism.org       Internet:  www.iala-aism.org

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

IALA Recommendation on the IALA Common Shore-based System Architecture (CSSA)

(Recommendation e-NAV-???)

THE COUNCIL:

RECALLING the function of IALA with respect to Safety of Navigation, the efficiency of maritime transport and the protection of the environment;

RECOGNISING that the e-Navigation architecture will assist in the development and maintenance of application interactions from ship to ship, ship to shore, shore to ship and from shore to shore, in the following areas

- shipboard equipment including Human-Machine-Interfaces to the shipboard users;

- Maritime Service Portfolios (MSPs) including service spectra and associated service quality;

- Communications;

- Resilient Position, Navigation and Timing (PNT);

- Shore-based infrastructure including shore-based technical services and their Human-Machine-Interfaces to the shore-based users and

- The Common Marine Data Structure (CMDS);

RECOGNISING ALSO that the responsibilities of IALA National Members regarding international and/or regional e-Navigation applications:

- that the shore-based systems of National Members are embedded into local, national, regional and global topologies of systems;

- that there are data/information flow chains within these topologies of systems which support e-Navigation applications;

- that each shore-based system contributes in a specific way to those data/information flow chains;

- that the disruption of this contribution would have an effect on the e-Navigation applications supported;

- that the shore-based system in itself consists of components, which support the data/information flow chain through that system;

- that therefore certain requirements regarding quality parameters, such as accuracy, reliability, continuity, confidentiality, and others, will need to be taken into account when designing the Common Shore-based System Architecture (CSSA);

RECOGNISING FURTHER that the CSSA laid out in this Recommendation will assist National Members in the efficient deployment of their services to the mariner, to other shore-based authorities and to the maritime community at large by

- allowing the shore-based system of a National Member to participate in the harmonised data exchange ship-shore, shore-ship, and shore-shore;

- enabling the integration of standardised technical services and components;

- creating a global market for those standardised technical services and components;

- ensuring maintenance- and development-friendly architecture of the technical services and components;

- improving the efficiency in organisational terms by exploiting commonalities;

- improving the quality of services and systems;

- improving the life cycle management;

- harmonising data exchange to other shore-based stakeholders’ systems (local, national, regional, global);

- complying with the anticipated performance requirements from IMO, i.e. rendering the shore-based systems “fit for e-Navigation” or  “e-Navigation compliant”;

- obtaining cost benefits from the above;

- taking into account the sensitivity of data;

NOTING the

- IALA Recommendation e-NAV 140 on e-Navigation Architecture [1];

- IMO’s strategic plan regarding e-Navigation [2];

- IMO has expressed an interest in the contribution of IALA to the work on e-Navigation [3];

- IMO has agreed on an overarching e-Navigation architecture [4];

RECOMMENDS that National Members and other appropriate Authorities providing marine aids to navigation services adopt the design and implementation principles set out in the annex to this recommendation.

Table of Contents

Document Revisions	1

Table of Contents	5

Index of Tables	5

Index of Figures	6

Annex	7

1	Introduction to the Recommendation Annex	7

2	The place of the Common Shore-based System (CSS) within the overarching e-Navigation architecture	7

3	Derivation of system requirements from user requirements	11

3.1	Information/data flow, application interactions, and user interfaces	11

3.2	The engineering approach for the Common Shore-based System	12

4	Goals for the Common Shore-based System Architecture	14

4.1	Quality benefits	14

4.2	Cost related benefits	15

4.3	Organisation and staff related benefits	15

4.4	Public relations and societal benefits	16

5	Fundamental system architecture design principles	16

6	Common Shore-based System Architecture general layout	17

7	CSSA detail layout	21

8	Consequences and Options	27

8.1	The usefulness of the CSSA as a reference framework	27

8.2	General requirements for the CSS and its components	27

8.3	The CSSA’s support of Maritime Service Portfolios (MSPs) definitions	28

8.4	Interactions between the CSS of an IALA national member and global topologies of systems	29

8.5	Options for different national IALA members	31

9	Migration and Life-Cycle Management Issues	31

9.1	Migration and Integration of legacy systems to the CSSA	31

9.2	Life-cycle management considerations	32

10	References	33

11	Abbreviations	33

Index of Tables

Table 1	Derivation of information needs, information objects and data objects for technical services from user requirements	11

Index of Figures

Figure 1	The overarching e-Navigation architecture and the Common Shore-based System (CSS)	8

Figure 2	Common Shore-based System Architecture (CSSA) as embedded in the overarching e-Navigation architecture and in relation to stated information needs and formats	10

Figure 3	Engineering analysis of requirements for the Common Shore-based System	14

Figure 4	Structural overview on Common Shore-based System Architecture (CSSA)	18

Figure 5	Structural Overview with Additional Detail	20

Figure 6	Generic Functional Setup of the Common Shore Based System (CSS)	26

Figure 7	Topology of different kinds of shore-based systems	30

Figure 8	Example of life-cycle management modules for CSSA as informed by ITIL V3 (ISO 20000 based) [Source: Wikipedia]	33

Annex

IALA Common Shore-based System Architecture (CSSA)

# Introduction to the Recommendation Annex

The first section introduces the CSSA in the context of the overarching e-Navigation Architecture and explains the meaning of the symbols used in the diagrams. It also lists the relationships and interactions between the various applications, equipment, users and services.

The next section discusses the development of the shore-based system requirements from the user needs and user requirements.

The following sections focus on the design and implementation principles of the Common Shore-based System (CSS) deployed by an IALA National Member.

The development of the CSSA as a model comprises the main part, with a discussion of how legacy systems (i.e. present system architecture(s)) can be migrated into this model in the final section. The topics are presented as follows:

- Goals

- Fundamental design considerations

- General layout

- More detailed layout

- Consequences and options

- Migration issues.

# The place of the Common Shore-based System (CSS) within the overarching e-Navigation architecture

The Recommendation e-NAV 140 on the e-Navigation Architecture [1] presented the general shore-based perspective on the e-Navigation concept, on IALA’s mandate, and on the shore systems in general.

This Recommendation focuses the attention on the shore side of the overarching e-Navigation architecture in accordance with the mandate of IALA. This is illustrated by the shaded area on the shore side in Figure 1 on the following page (source: [4], Figure 1, with highlight added).

- The overarching e-Navigation architecture and the Common Shore-based System (CSS)

Figure 2 on the next page shows the resulting place of the CSS of an IALA National Member within the overarching e-Navigation architecture.

In Figure 2 the following graphical symbols are used:

- Interfaces between entities involved are symbolised by a little circle and a line leading to the system which provides the interface.  The interface as such belongs to the system which provides it;

- Technical systems are symbolised by cubes, which expresses the encapsulation principle (black boxes);

- Human-Machine-Interfaces are indicated by the acronym HMI, and Machine-Machine-Interfaces, i.e. mainly IT or computer interfaces, are indicated by the acronym M2M.

- The arrows with the dotted lines indicate requirements, which are put forward by the entity at which the dotted line starts;

- Requirement arrow(s) always point to interfaces (circles).  This is to indicate, that the interface(s) fulfil the requirement(s).

Figure 2 also shows the

- interactions of shore-based applications with shipboard applications, as operational services which are facilitated by the interaction of the shipboard technical equipment with the shore-based technical system;

- potentially user-independent, automated interaction of the shore-based technical system with the shipboard technical equipment;

- relationship between the shore-based system and its shore-based users, indicating that information needs must be satisfied by an appropriate technical service at its appropriate Human Machine Interface;

- interaction with other shore-based systems, named “third party” systems from the point of view of the National Member’s system, both for sending and receiving data;

- existence of technical services as one building block of the CSS as a forward reference to following sections.

- Common Shore-based System Architecture (CSSA) as embedded in the overarching e-Navigation architecture and in relation to stated information needs and formats

# Derivation of system requirements from user requirements

This chapter discusses the derivation of shore-based system requirements from user needs and user requirements.

The e-Navigation concept is mainly about information flow and therefore the shore-based system architecture under consideration uses Information Technology (IT) concepts and terms.

The Common Shore-based System Architecture (CSSA) can support a variety of Human Machine Interfaces (HMI) for different shore-based users.

## Information/data flow, application interactions, and user interfaces

When considering the overarching e-Navigation architecture, one should think in terms of information/data flow, application interactions, and user interfaces, as follows.

User requirements can be described in terms of their respective information needs for their tasks to perform.  The required information items are delivered at the Human Machine Interfaces of the user applications by the shore-based system to fulfil the user requirements.  These information items are transmitted, stored, and processed as data objects by the shore-based system.  They are exchanged using the functional links between applications, both shipboard and ashore.

From the above it follows, that when there is an information flow between users and applications, there is always an associated data flow.

Error: Reference source not found summarizes the above relationships.  For the precise definitions of information vs. data refer to the glossary.

- Derivation of information needs, information objects and data objects for technical services from user requirements

- User requirements in general (top level) 		(information domain)

- Tasks of the user 					(information domain)

- Stated information needs				(information domain)

- Individual information objects 			(information domain)

__interaction by Human-Machine-Interface (Operational Presentation Surface)__

- Maritime data objects  				(data domain)

- Maritime data encoding-for-data exchange 	(data domain)

The following conclusions can be drawn from Figures 1 and 2:

  - When designing and describing e-Navigation applications, they should also be described as interactions between applications using the information/data flow concept:

- (Ultimate) sources of data should be identified together with the data objects stemming from those sources, i.e. the source data;

- Likewise, (ultimate) sinks for data should be identified, together with the data objects required by a particular sink in the information flow, i.e. recipient data;

- When there is intermediate processing of data objects involved, the appropriate algorithms should be stated;

- All data objects should be described by their relevant attributes or properties, including their constraints (such as permissible min/max values).

  - Figures 1 and 2 also represent a statement regarding the distribution of responsibilities between operational stake-holders and engineers:

- Operational stake-holders state their information and portrayal requirements (symbolised in Figure 2 by dotted arrows) at the user interfaces of the shore-based system. The operational stake-holders should be continuously involved in the design and implementation process to ensure that their information and portrayal requirements are met by the engineering process;

- Engineers analyse these information requirements and take into account the management goals, in particular considering the life-cycle management aspects of any system or component.  The result of their analysis is an engineering representation (e.g. “specification”);

- This engineering representation is described in one or more stand-alone documents.  Thus, it can and should be verified and validated by operational stake-holders and management;

- Engineers will eventually provide the appropriate technical Human-Machine-Interface (HMI), which fulfils the stated information and portrayal requirements;

- A Human-Machine-Interface (HMI) can be any kind of appropriate combination of displays, keyboards, voice interfaces (microphone / loudspeaker), and other human interaction devices.  The suite of those devices at one operational working position is called the Operational Presentation Surface (OPS).  This comprises the concepts of modern IT human interfacing technologies, such as windows techniques;

- The distribution of responsibilities is reflected in an appropriate documentation framework.  As far as IALA documentation is concerned, this is further developed in the last section.

## The engineering approach for the Common Shore-based System

The stated information requirements as described above can then be analysed using an appropriate engineering methodology to provide comprehensive documentation of the data objects and their inter-relationship, technical services functions and their interaction, and component specifications.

The shore-based technical services supporting the information requirements of the shore-based users in turn require certain physical links (signal-in-air) and certain shipboard devices in most cases.  Similarly, a requirement chain exists for the shipboard user.  (Note: These statements address the requirement chains as opposed to information flow chain.)

During this process management goals such as life-cycle management requirements, need to be taken into account.

There are state-of-the-art engineering methodologies available to facilitate that work, such as the Object-oriented Engineering Process (OEP) and the use case methodology.

The most important part of the paradigm of the CSS is the OEP (see Figure 3 below).  It describes the methodology applied to the engineering task at hand in the following step-by-step process.

- List and specify all stated information requirements and the associated information items for the CSS as a whole.

- List and specify the internal requirements for the CSS as a whole from a management and technical point of view, taking into account life-cycle management and resource usage requirements.

- Use engineering methodologies to do an engineering analysis of requirements for the CSS:

The well attested use case approach provides such an engineering methodology.  It is a point in case that IMO defines e-Navigation in accordance with the use case concept: e-Navigation is the harmonised collection, integration, exchange, presentation and analysis of maritime information onboard and ashore (…):

- The five keywords highlighted above describe what tasks are to be performed with maritime information and are indeed use case designations.

- It should further be noted, that IMO’s definition explicitly discerns the use cases for the shipboard side and the shore side (also highlighted above).

- Hence, it can be concluded that IMO’s definition of e-Navigation is fully compatible with many state-of-the-art engineering methodologies for requirement analysis, and vice versa.

- Derive the essential system requirements by exploiting the similarities between stated information requirements of different users and of internal information requirements.

- Design a CSS layout using the concept of a technical service as building blocks.

- For each and every essential system requirement identify the interactions of the relevant individual technical services needed to fulfil that specific essential system requirement.

- Draft a precise functional description of an individual technology as a technical service in accordance with a generic service model.

- Derive all component requirements for components of an individual technical service.

- Capture all the above descriptions in a documentation methodology and prepare for the submission to quality management audits.

- Engineering analysis of requirements for the Common Shore-based System

# Goals for the Common Shore-based System Architecture

By implementing and adopting the CSSA the following possible benefits can be achieved. Many of them have also been specifically identified in the IMO e-Navigation strategy [2].

## Quality benefits

- Provision of user information, as stated and portrayed in a defined way, including accuracy, integrity, reliability, continuity, and latency;

- demonstration of “e-Navigation compliancy” to stakeholders by means of appropriate certification for service level achievements;

- application of objective criteria for each technical service provided;

- same level of service for different users with similar requirements;

- extensive usage of applicable international technical standards;

- improved provision, in architectural terms, to incorporate new functionalities at planning phase, based on revised information requirements and portrayal;

- increased dynamic adaptability to incorporate new operational functionalities at run-time, based on new information requirements and portrayal;

- improved responsiveness to new or amended information requirements and portrayal;

- Promote Innovation, expand component offerings and improve their quality by creating an “eco-system” for technical services and their vendors

## Cost related benefits

- Full life-cycle cost evaluation facilitated by the early identification of cost items, both for investment and running costs;

- Cost reductions for planning and development by utilising applicable international standards and re-use of engineering concepts;

- Cost reductions for updating existing services and functionalities due to (rapid) change of technology;

- Cost reductions for implementing further or changed services and/or functionalities;

- Investment cost reduction: a) reduction in cost per unit due to standardization; b) reduction in numbers of units needed due to better utilization of the same kind of unit (down side: potentially increased single point of failure, hence effective mitigation strategies needed);

- Running cost reduction over full life-cycle, taking also into account cut in staffing level: a) remote access/control/maintenance as opposed to only-localized access/control/maintenance resulting in consequential savings of working time and travel; b) automation; c) easier replacement of individual components;

- Avoidance of vendor-lock-in: IALA National Members would not be locked-into purchasing the majority of their CSS technical services from a single vendor. Instead it would allow them to assemble a CSS using “best-in-class” technical services. They would be able to mix-n-match technical services that best fit their needs without facing inter-vendor compatibility issues. They would also be in a better stronger position to negotiate provision of technical service support and upgrades with their vendors.

## Organisation and staff related benefits

- Maintenance and widening of the expertise of technical personnel;

- Achievement of common level of understanding between different, interacting authorities, who both employ the CSSA;

- Optimization of the business processes and organizational procedures involved based on the use of the CSSA;

- potential optimization of organizational structure;

- improvement of training efficiency for technical personnel due to the transfer of knowledge from the general (i.e. the understanding of the generic model) to the specific technology under consideration (i.e. individual technical service);

## Public relations and societal benefits

- Contribution to an improved public relation image of the administration by use of an internationally accepted, advanced system architecture;

- Ease of public access to information by e.g. publicly accessible information portals provided by the administration, while the administration simultaneously has the benefit of a standardized method of presenting that information to the public;

- Success in achieving the IMO stated goals for improving the safety and efficiency of navigation, protection of the environment and security [2].

Most of the above benefits are critical to the success in achieving the IMO stated goals for the e-Navigation concept.

# Fundamental system architecture design principles

To achieve the above goals the following fundamental principles for the CSSA are introduced:

- User-requirement driven system design: Only clearly and consistently stated user requirements result in technical service provided.

- Shift from technology-orientation to information-orientation while designing the system layout: All technical solutions based on data modelling as a rule.

- Introduction of the OEP for all technical aspects.

- Employment of the principles of modularity and encapsulation, while preserving a holistic view of the system’s intended functionality.

- Application of a uniform model for all technical services provided by the system, regardless of technology, thus exploiting commonality.

- Implementation of functional specifications, based on international standards to the largest extent possible: Procurement of technical solutions based on functional specifications as a rule.

- Adherence to open system architecture and focus on open and standardized interfaces between components and services: No proprietary interfacing as a rule.

- Employment of remote access techniques where feasible in order to allow for minimal number of technical operation and maintenance centres: Components without remote access capabilities as a rule not permissible (the access capabilities must allow for open interfacing, too).

- Implementation of life-cycle management in order to prevent “quick-fix-solutions” with associated long-term costs: Full life-cycle coverage of technical proposals required for their approval.

- Documentation of each and every functional aspect in a uniform, overall documentation system: This is a pre-requisite for any quality management system.

- Separation of technical operation and maintenance tasks and personnel on one hand (no authorisation to change functionality and/or structure of component) and system development and optimization tasks and personnel on the other hand (authorisation to change functionality and/or structure).

- Take into account regulatory constraints when creating the system architecture, but there may be consequential requirements to amend existing or create new regulations based on the development of that system architecture.

- Support concepts such as certification in general, IT security certification in accordance with ISO 27000 series, and also the IMO Member State Audit Scheme (IMSAS).

# Common Shore-based System Architecture general layout

The CSSA describes the technical set-up of the shore-based system of an IALA National Member. The main building block of the CSSA is the technical service. Its qualities are as follows: A technical service encapsulates all primary functionalities dealing with a specific technology or with a specific user, depending on the kind of technical service. To reap the maximum benefit, all technical services of the CSSA should adhere to the same object-oriented engineering model. The generic, object-oriented service model can be found in another IALA Recommendation. All technical services are self-contained and provide all capabilities needed for their tasks, including their own service management.

Figure 4 provides a structural overview representation of the CSSA.

It should be noted, that the CSSA is modelled in a client-server-fashion; the individual technical services can regularly assume either roles, i.e. be “clients” or “servers”, depending on their present role in a given interaction chain to support a given application-to-application data exchange within the overarching e-Navigation architecture.

- Structural overview on Common Shore-based System Architecture (CSSA)

Figure 4 shows some individual technical services (i.e. the User Interaction Service and the Gateway Service) and some groups of individual technical services (i.e. Data Collection and Data Transfer Services, Value Added Data Processing Services).

Basically all technical services shown in Figure 4 do what is implied by their name:

- The “Data Collection and Data Transfer Services” are a group of technical services interfacing the shore-based system via the physical links to the vessels’ electronic systems (including their Integrated Navigation Systems), to traffic objects other than vessels, to the waterways and to the natural environment. For example, the AIS Service, the Radar Service and the Visual Aids-to-Navigation Services (fixed, floating) belong to this group. Some operate bi-directionally (e.g. the AIS Service), while others operate uni-laterally on their appropriate physical links.

- The “Value Added Data Processing Services” also are a group of individual technical services. Their main task is to add value to (raw) data by processing, combination, comparison etc., store data and information and provide it upon request to other technical services.

- The “User Interaction Service” – a single individual technical service – is specialised to provide the Human-Machine-Interface to the primary users of the CSS, i.e. such users who are supported directly by the system via dedicated displays, keyboards and other human interaction devices.

- The “Gateway Service” – another individual technical service – specialises in data exchange shore-to-shore. It interfaces mainly to external systems of “third parties”, which provide data needed within their own system or which request relevant data to be forwarded to them. The “Gateway Service” can also interface different shore-based systems locally, regionally, and globally.

One further important conclusion can be derived from Figure 4: It is the combination of the technical services in their system-wide interaction, which provides the required functionality of the whole of the system to its users.

Figure 5 on the following page provides a structural overview and adds a bit more detail, as will be explained below.

It should be noted, that data exchange between different technical services is exclusively done by Service Couplers and only in Node sites, e.g. using local area network technologies at the Nodes. The topology options in regard to the Node architecture are explained in the IALA Recommendation on the generic service model.

LEGEND:

HMIHuman-Machine-Interface

M2MMachine-Machine-Interface

OP/DPTechnical Operating Personnel

(operates and maintains technical equipment;

no licence to change structure and/or functionality) /

Technical Development Personnel

(plans, deploys and further develops technical equipment;

licence to change structure and/or functionality)

NOTE: By default, external users can request a HMI (Configuration Type to be provided by the User Interaction Service) and/or a dedicated Gateway (Machine-to-Machine-Interface; configuration type to be provided by Gateway Service), depending on individual operational requirements.

01-Nov 2007

OP/

DP

OP/

DP

OP/

DP

-Radio Transmission -

Atmosphere

Waterways

Acoustical

Transmission

General Shipping

Commercial Shipping

Yachts

Inland Waterway Barges

Ships operated by an administration

Buoy Tender

Law Enforcement Vessels

Navy Ships

Search and Rescue Vessels

Pilot Boats

Etc.

Passenger

Crew

Applications

Equipment

Ships

Visual

Transmission

Data Collection

and

Data

Transfer

Services

HMI

Value-added Data Proces-sing

Services

User

Interaction

Service

Service Couplers

Service Couplers

Service Couplers within Nodes

Shore-based external systems (of third parties)

Gateway Service

Satellite

Radio Navigation / GNSS

Physical

Measurements

M2M

M2M

M2M

M2M

M2M

HMI

HMI

HMI

HMI

OP/

DP

M2M

- Structural Overview with Additional Detail

# CSSA detail layout

Figure 6 on a following page shows CSSA in almost full detail. It particularly indicates the variety and plurality of different individual technical services and interactions with the shipping and the environment (left hand side), with shore-based primary users (right hand side), and to other shore authorities, shore-based users and shore-based stakeholders (lower side of Figure 6).

The names of the technical services given in Figure 6 are descriptions of what they provide to the CSS. Each of the individual technical services has been assigned a three-letter mnemonic, which is uniquely assigned within the CSSA, and which will be used for various purposes when detailing the architecture and when encoding data items in shore-based data exchange.

Additional technical services and/or additional functionalities of existing technical services may be defined in the future as prompted by new technologies and/or new functionality requirements.

Figure 6 is a recommendation in regard to the operational services supported and technical services implemented for an IALA national member. Each IALA national member can provide the services as needed and can provide services not included. The following list gives a short explanation of each of the technical services illustrated in Figure 6:

- Data Collection and Data Transfer Services:

- The AIS Service (AIS) allows the CSS to access the AIS VHF Data Link, supporting the various receiving and transmitting functions of the AIS intended for a shore-based competent authority.

- The Radar Service (RAD) delivers to the CSS all radar-derived data, i.e. radar images, radar plots, and radar tracks, each of which with radar-specific relevant quality parameters.

- The Direction Finding Service (DFS) determines the current position of a mobile VHF voice/data transmission device operating on one of the several frequency channels monitored. Thus, a DF track together with relevant quality parameters (e.g. accuracy estimation) is provided to the CSS.

- The DGNSS Augmentation Service (DGN) delivers to the CSS all relevant measurements regarding the current “on-air” status of selected components of World Wide Radio Navigation System (WWRNS), such as particular GNSS, terrestrial augmentation and terrestrial backup-systems of other authorities. Such measurements could be forwarded to e. g. the User Interaction Service for appropriate display to users e.g. in VTS Centres, thus fulfilling user requirements accordingly, and/or to the surveying community and/or the general public via the Gateway Service for their added value. Also, based on these run-time measurements, the DGNSS Augmentation Service provides own DGNSS augmentation data to the own CSS for transmission to vessels e.g. via the AIS Service or via the Medium Wave Broadcast Service. Hence, the DGNSS Augmentation Service does not transmit DGNSS augmentation data to shipping on its own but relies on some other technical service to fulfil that specific task.

- The Medium Wave Broadcast Service (MWB) is for transmitting DGNSS augmentation data that have been provided by the DGNSS Augmentation Service using medium wave bands around 300 kHz (radio beacon band).

- The VHF Communication Service (VHF) allows for shore-based bi-directional voice communication with any mobile radio station operating in the VHF maritime mobile service band.

- The Short Wave Communication Service (SWC) provides the shore-based bi-directional voice and data communication with any mobile station operating on the frequencies of the short wave bands assigned to maritime use.

- The GMDSS VHF DSC Service (DSC) allows the CSS to participate in the digital GMDSS DSC-Channel 70 in the VHF maritime mobile service band.

- The Short Wave GMDSS Service (SWG) allows for the bi-directional communication in the short wave channels dedicated to the GMDSS.

- The Navtex Service (NTX) is a component of the IMO/IHO Worldwide Navigation Warning Service and of the GMDSS. The service provides navigational and meteorological warnings and forecasts as well as urgent safety information over medium frequency radio to automated Navtex receivers/printers on-board ships.

- The Aviation Communication Service (AVC) allows for shore-based bi-directional voice and/or possibly data communication with any aircraft or helicopter, thus supporting operational applications such as Search and Rescue and pollution combat.

- The Fixed Visual Aids Service (FXA) provides fixed visual or classical Aids-to-Navigation to the mariner. It should be noted, that “visual” may be a misnomer to the extent that a fixed Aid-to-Navigation could also include on site racons and / or on site AtoN AIS stations, which conceptually would be a part of the FXA.

- The Floating Visual Aids Service (FLA) provides floating visual or classical Aids-to-Navigation to the mariner. The same cautionary note regarding non-visual technologies as with the FXA applies.

- The CCTV Video Service (VID) provides the CSS with the capability to visually observe the areas of waterways covered by video camera observation. The output of the CCTV Video Service would a stream of images of the waterway, potentially including some video content analysis data, to be further processed in some other appropriate service of the CSS.

- The Local Public Address Service (LPA) allows for acoustically addressing areas of a waterway, for acoustic voice communications and for the dissemination of other acoustic signals to mariners. Hence, the acoustic fog signals would belong to that service.

- The Environmental Sensor Service (ENS) provides the CSS with measurements of relevant parameters of the maritime environment. Such relevant sensor data could be hydrological, meteorological or of other physical nature.

- Value Added Data Processing Services:

- The Position Determination Service (POS) processes all real-time data regarding traffic objects (vessels, aircrafts, floating objects etc.) from all available and/or relevant sensory sources (e.g. AIS, radar, direction finding). Also, fusion algorithms dealing with the different sensory information may be processed here, thus providing the CSS with a multi-sensor-validated track of every traffic object under surveillance by the CSS, together with relevant quality parameters such as accuracy estimations, tracking status and reliability etc. For every traffic object surveyed not only the “last known” position is stored, but also their past tracks derived from the validated position reports.

- The Ship Data Consistency Analysis Service (SDA) collects, evaluates, stores and provides on demand non-real-time data regarding traffic objects, in particular their static and voyage related data. To that end this technical service draws on a variety of available sources of information for that kind of data, such as AIS reports, historical data from past voyages of the same traffic object, manually acquired data (e.g. from User Interaction Service), and third party data bases (accessed via Gateway Service). This service also provides pseudo tracking identifiers for the Position Determination Service (POS), see above, for correlation of real-time tracking data with static and voyage related data of the same traffic objects. Amongst other things, this service may handle data as defined by IMO FAL (Convention).

- The Environmental Data Evaluation Service (ENE) collects, evaluates, stores and provides on demand all data of the physical environment of the waterways relevant to safety and efficiency of shipping and to the protection of the environment. To that end this technical service draws on a variety of available sources of information for that kind of data, such as the CSS’s own Environmental Sensor Service (ESS), own historical data, manually acquired data, and in particular from third party data bases (accessed via Gateway Service). The Environmental Data Evaluation Service also creates added-value information on the physical environment by evaluating the available raw data from the various sources by appropriate algorithms. For instance, tidal windows and under-keel clearances may be calculated here.

- The Vector Chart Service (VEC) collects, stores, and provides on demand all data pertaining to vectorized ENCs, both drawing on official sources for ENCs but also drawing on the system’s own sources, i.e. on manual input via the User Interaction Service. The Vector Chart Service maintains the ENCs from external sources as they are, but provides overlay layers which contain the additional information from the system’s own sources, such as coverage areas of operational and technical services or route topology modelling data.

- The Shipping Industry Database Service (SID) collects, stores, and provides on demand all data pertaining to the “shipping industry”, i.e. data provided by or learned about relevant stakeholders (compare IMO list of ship-board and shore-based stakeholders in [2]).

- The Data Mining Service (DMS) allows for complex queries on present and historical data available from all other technical services of the system. It creates stores and provides reports to the requesting service, e.g. the User Interaction Service or the Gateway Service.

For the Gateway Service (GWY) and the User Interaction Service (UIA) see above.

Each of the above technical services except for the User Interaction Service provides a functional (machine-to-machine) interface to any other of the technical services of their own CSS, i.e. on their “domestic” side. This interface provides the respective functionalities of a technical service as a set of Basic Services specific to that technical service.

For example, the AIS Service delivers the so-called Basic AIS Services, which is a set of well defined, AIS specific functionality to access the complex features of the AIS VHF data link in an abstract representation. This encapsulates the subtleties of the AIS and its complexity and thereby avoids proliferation of AIS VHF data link specifics throughout the CSS.

The User Interaction Service provides its functional interface as a Human Machine Interface to its human users. This functional interface, which may be called Operational Presentation Surface (OPS) generally is subdivided into individual functional windows each with their own specified user interaction.

It should be noted, that data exchange between different technical services is exclusively done by so-called Service Couplers and only in so-called Node sites, e.g. using local area network technologies at the Nodes. This has at least two important benefits: The wide-area networking topologies of the different technical services, which may be different from each other, are encapsulated within the service. Also, local area networks allow for maximum and highly cost-efficient performance of data exchange at the Node sites.

It should also be noted, that the Value Added Data Processing Services are only represented at Node sites.

The Shore-Based Wide Area Network Service (SBN) is a utility service, which support any other requesting service of the CSS. It provides the feeder link and the backbone connectivity for the individual technical services of the CSS. Because of its supporting position in regard to its requesting services, it is encapsulated by its requesting service.

The On-Site Infrastructure (INF) provides resource building blocks needed to support the service components on their sites of installation, hence On-Site Infrastructure. Main topics include housing and other structures, traditional utility provision such as power, water, sewer and roads, precise timing, local data networking (LAN), independent fault detection and alert management, HMIs for Technical Operation Personnel and Technical Development Personnel. It is important to note that this is where there is usually the largest part of the cost for the National member to support and maintain its CSS.

For further details on the above and additional features of the technical services refer to another IALA Recommendation on the generic service model of a technical service.

The following chapters explain in more detail the options for partial implementation and/or configuration by different national members.

The Operational Presentation Surface, the “User Front End”, was introduced in section 3.1.  The architecture can exploit commonalities in functions. For instance, “traffic image display” will be required by several operational users, although the configuration settings for each of the operational users may be different (e.g. tactical traffic display vs. strategic traffic display). The concept of Configuration Type encapsulates the ability to customize presentation to different users. This concept is shown in Figure 6 within the User Interaction Service.

Position Determination Service

(POS)

HMI

Ship Data Consistency Analysis

Service (SDA)

HMI

Environmental Data Evaluation

Service (ENE)

HMI

Vector Chart Service

(VEC)

HMI

Data Mining Service

(DMS)

HMI

Shipping Industry Database

Service

(SID)

HMI

OP/

DP

OP/

DP

OP/

DP

OP/

DP

OP/

DP

OP/

DP

Notices To Mariners Service

Hydrological Forecast Models

ENC Distribution

Waterway Depth Information Provider

Dredging Operations

Tidal Information

Shipping Statistics and Shipping Fees

Shore-based Pilot Applications

Future external information source/sink

Other allied services

Ship Agents

Ship Register(s)

Regional Maritime Cooperation

(such as HELCOM, SafeSeaNet)

External Communication Provider

External Meteorological Services

und Wetter

Port State Control

Different Security Services

Customs

Inland Waterway Authoroties

Port Authorities

Land Surveyors

Visual Transmission

(mainly)

OP/

DP

Configuration type for Traffic Information

Traffic Support and Traffic Control

HMI

Configuration Type for

Security Applications

HMI

Configuration Type for

Maritime Rescue Coordination (MRCC)

HMI

Configuration Type for Accident

Management and Pollution Combat

bekämpfung und Unfallmanagement

HMI

Configuration Type for Waterways

Maintenance Operations (e.g. Dredging)

HMI

Configuration Type for Editing of

Notices to Mariners

HMI

Configuration Type for

Lock Operation

HMI

Configuration Type for

Ice Reports / Ice Breaker Operations

HMI

Configuration Type for

Ship Data Consistency Analysis

HMI

Configuration Type for

Data Evaluation and Data Mining

HMI

Configuration Type for

Pilot Dispatching Center

Betrieb Lotsenwachstation

HMI

DGNSS Augmentation Service (DGN)

HMI

OP/DP

Aviation Communication Service (AVI)

HMI

OP/DP

GMDSS VHF DSC Service (DSC)

HMI

OP/DP

CCTV Video Service (VID)

HMI

OP/DP

Local Public Address Service (LPA)

HMI

OP/DP

Gateway Service   (GWY) – individual configuration types (Gateway) for each external user (sources / sinks)

HMI

Environmental Sensor Service (ENS)

HMI

OP/DP

- Radio Transmission -

Atmosphere

Waterway

Local Area Network(s)  within the Nodes

Local Area Network(s)  within the Nodes

Satellite

Radio Navigation GNSS

On-site

Infrastructure

(Energy, Housing,..)

(INF)

HMI

OP/

DP

Shore-based

Wide-Area

Network Service

(SBN)

HMI

OP/

DP

LRIT Data Center

SAR

General Shipping

-Merchant Marine

-  Fishing Vessels

-Leisure Crafts

-InlandWaterways Barges on Sea Fairways

Administration ships

-  multi purpose vessels

-buoy tenders

-ice breaker

-  SAR vessels

-   pilot vessels

-survey vessels

-  customs vessels

-  police vessels

-  Navy vessels

Floating Objects

-Lost Cargo

-Life boats / rafts

-  floating pollutants (e. g. oil)

Shipping

Passenger

Crew/Pilot

Application(s)

Radio link transceiver

PPU

Generic Functional Setup of the Common Shore Based System (CSS)

Application Services

Value-added Data Processing Services

Data Collection and Data Transfer Services

User Interaction Service   (UIA)

LEGEND:

HMI	Human-Machine-Interface

M2M	Machine-Machine-Interface

OP/DP		Technical Operating Personnel (operates and maintains technical equipment; no licence to change structure and/or functionality) / 
Technical Development Personnel (plans, deploys and further develops technical equipment; licence to change structure and/or functionality)

NOTE: 	By default, external users can request a HMI (Configuration Type to be provided by the User Interaction Service) and/or a dedicated Gateway (Machine-to-Machine-Interface; configuration type to be provided by Gateway Service), depending on individual operational requirements. / Due to editorial constraints, the plurality of the “Data Collection and Data Transfer Services” in particular in the domain of communications is only indicated by examples.

Shore-based
Operators / Users

HMI

Acoustic

Transmission

Service Couplers in Local Area Network(s) within the Nodes

Physical status

Indicators

External/other network users

requested by every service 
(request + access encapsulated by service)

requested by every service 
(request + access encapsulated by service)

OP/

DP

Co-located third party services

All M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

Other / Future Configuration Types

may be added as required

Information Providers / Information Sinks /
Allied Services

Off-Shore Structures

Radar Service (RAD)

HMI

OP/DP

AIS Service (AIS)

HMI

OP/DP

Direction Finding Service (DFS)

HMI

OP/DP

Medium Wave Broadcast Service (MWB)

HMI

OP/DP

Short Wave GMDSS Service (SWG)

HMI

OP/DP

Fixed Visual Aids Service (FXA)

HMI

OP/DP

VHF Communication Service (VHF)

HMI

OP/DP

NAVTEX Service (NTX)

HMI

OP/DP

Floating Visual Aids Service (FLA)

HMI

OP/DP

# Consequences and Options

The above configuration is typical for an IALA national member which exhibit’s a broad scope of activities in the fields of Aids-to-Navigation and VTS. This Chapter now introduces in more detail the configuration options for different IALA national members when setting up their CSS using the above reference framework.

## The usefulness of the CSSA as a reference framework

The CSSA has been presented as a reference framework for the design of shore-based systems. As has been shown in Chapter 3, the concept of deriving the essential system requirements from user requirements is the starting point for system design.

With defined system requirements, the concept of technical services can be applied. Technical services are used as building blocks to describe a CSS.

All of the necessary services for information and data flow, application interactions and user interfaces can be derived from the essential system requirements.

Services can be described using a generic service model. The technical services introduced in Chapter 7 each have their own IALA recommendation.

The reference framework created by the CSSA would help in creating an “innovation friendly eco-system” for technical services and their vendors:

- It would expand the market for individual technical services. This would greatly increase the number of solutions that would be offered to IALA National Members for each technical service reduce their cost and simultaneously improve their quality.

- Technical service developers would be able to rely on well-proven, reliable and already available other technical services (i.e. Data Collection Services, Data Transfer Services, Value Added Data Processing Services and Application Services) and thus avoid having to developing their own (proprietary) ones. This could significantly reduce the cost of developing technical services and improve their quality.

## General requirements for the CSS and its components

There are requirements are almost always used and are described here as a reference. Some of these may not be applicable to all systems. However, all of these must be considered and either applied or rejected with reason as a part of the design process.

These terms are in common use and they are not defined here.

- Scalability

- Interoperability

- Flexibility

- Modularity

- Reliability / continuity / availability

- Latency

- Maintainability / Life-cycle-friendliness / future-proofing

- Security

- Integrity

- Survivability / robustness

- Safety

- Seamlessness

- Verifiability / validability

- Usability

- Extensibility

- Inclusivity

- Consistency

## The CSSA’s support of Maritime Service Portfolios (MSPs) definitions

As the CSSA is a service-oriented architecture, it supports directly the IMO developed concept of the Maritime Service Portfolios and, thereby, those concepts correlated with MSPs, such as the concept of a “Maritime Cloud”.

A Maritime Service Portfolio (MSP) is defined as “a set of operational and/or technical services” which are provided to shipping (in most cases from a-shore) and which – one could add immediately – are bundled together for a very specific purpose. The MSP concept is further described in an IALA Recommendation on the Structure of Maritime Service Portfolios.

The CSSA includes many of the shore-based technical services which support operational services delivered by shore-based operators in e.g. VTS centres maintained and operated by many IALA national members. For example, the radar-derived data flow through the Radar Service (RAD) via the Position Determination Service (POS) and via the User Interaction Service (UIA) to the VTS operator is supporting his/her task to provide the operational services, which the VTS-MSP comprises, namely Navigational Information Service (NIS), Navigational Assistance Service (NAS), and Traffic Organisation Service (TOS). In addition, these operational services would require also the support of communication-oriented services of the CSSA, e.g. and most prominently the VHF Communication Service (VHF).

Hence, the CSSA provides a means for a requirements derivation chain from operational services to technical services as all relevant technical services in such a data flow chain are identified. Thus, the CSSA directly supports not only the traceability of system requirements within the CSS itself, but also the traceability of requirements within the MSPs, i.e. a means to identify operational services calling technical services which in turn call other technical services.

Although each and every service which interacts with shipping (left hand side of Figures above) is – by definition – part of a Maritime Service Portfolio, if called, it should be noted that not every service of the CSSA is part of a MSP. (Well-known) examples for services which are providing technical services within a MSP are the Visual Aids services (fixed, floating), the DGNSS Augmentation Service, the VHF Communication Service, but also the AIS Service in its transmitting mode. Those services which provide CSS-domestic functionality are not part of the MSPs, for example the Radar Service (RAD), the AIS Service in its reception mode, the Value-Added Services in total, the Gateway Service, and the User Interaction Service.

## Interactions between the CSS of an IALA national member and global topologies of systems

In principle, as a consequence of the development of the CSSA, every such system should be able to interact with any other system on whatever scale, i.e. the connectivity of any such system should not be limited by architectural considerations.

The following Figure 7 discusses topologies of systems. Each of the “connections” discussed is provided by (at least) one service.

Each “system” shown which interacts with shipping (lower margin of Figure) encapsulates sets of services into MSPs, i.e. services provided to shipping.

Each of the systems deployed, maintained and operated by an IALA national member (“National” shown in Figure 7), should be planned and built in accordance with the CSSA principles set out in this Recommendation, or be migrated towards in the course of usual replacement(s) over time (see following Chapter).

It should be noted that the present CCSA was developed with an IALA national member’s system in mind. However, the regional or even global systems shown in the Figure 8 may also benefit from adopting the principles of the CSSA, as tailored to their services and requirements.

- Topology of different kinds of shore-based systems

## Options for different national IALA members

Not all components identified in the CSSA need to be provided/ deployed by every single CSS. Hence, the CSSA is scalable. Also, there may be additional individual technical services not mentioned in the above system layout that an administration may wish to provide. Hence, the CSSA is extensible, as additional technical services may be added (while services which are no longer existent may be removed). It should be noted, that scalability and extensibility need to comply with the principles of the CSSA. This scalability and extensibility can be applied both for the range of individual technical services provided as well as the functionality level of an individual technical service.

More than one IALA national member might share responsibility for providing a service, or there might be overlapping service. The IALA national member participating must describe the shared or overlapped service in their CSS. The shared and overlapped services are bundled into MSPs that can also be used as tools for mutual understanding.

An IALA national member may wish to contract some CSS services. Contracted services must be included in the CSS. Contracted services must be fully described by the CSS, and all requirements defined. IALA national members should apply quality management requirements on the contractor that are at least as strong as their own requirements. Contracted services might need to meet a higher level of requirements such as abiding by Non Disclosure Agreements for added security.

Some IALA national members may be responsible for the provision of operational services delivered from (e.g. VTS) centres they maintain and operate themselves; in this case these IALA national members need to set up the User Interaction Service (UIA) in the required configuration to present data on appropriate Human-Machine-Interfaces (HMI) to their operators in their centres. However, other IALA national members may only be responsible for providing data flow capabilities through technical services and not operate any centre for any operator in their responsibility. In this case the Gateway Service would provide the appropriate Machine-to-Machine interfaces, and no User Interaction Service would be required for such an administration. Generalizing, these examples illustrate that the CSSA provides the option to “swap” – on the planning level - between CSS configurations which exhibit a User Interaction Service with one, several, or many configuration types shown above and CSS configurations which have no such capabilities but a much more elaborate “landscape” of configuration types at the Gateway Service.

Finally, it should be noted that the CSSA would also be helpful in the extreme case for an IALA national member to condense the whole of its CSS on one spot only (i.e. in a One-Spot-Node-Configuration) or even within the fewest number of machines on that one spot possible. While there are pros and cons of such a configuration, the services of the CSSA would be transformed to software modules running on one or few computers at that one spot, and the CSSA could still be supportive for software development in those cases.

# Migration and Life-Cycle Management Issues

This Chapter discusses aspects of development of shore-based systems over a (longer) period of time and how change is accommodated.

## Migration and Integration of legacy systems to the CSSA

Legacy systems were not developed with a-priori knowledge of the e-Navigation architecture.  During a transition period both services and systems developed in accordance with the CSSA as described in this IALA recommendation and legacy services and systems will co-exist.

The ultimate goal of IALA National members should be to arrive at the CSSA, by deploying their own specific configuration of it.  Therefore, some migration of those legacy systems towards the CSSA may be required. Also, legacy components and/or systems may be integrated into an IALA national member’s (emerging) CSS by appropriate interfacing (Service Couplers).

During the period of co-existence of the emerging CSS and the legacy systems there must not be a threat to the established level of safety and of efficiency and of the protection of the environment.

## Life-cycle management considerations

As stated above, the CSS will comprise of many technical services and components contributing to the overall objectives.  IALA recommends applying a life-cycle management process to all of those technical services and components to reduce the total cost of ownership and ensure appropriate compatibility with the CSSA. Life-cycle management for an IALA national member’s CSS should be based on (an) appropriate, internationally recognized, service-oriented standard(s) for life-cycle management.

One internationally recognized, service-oriented, life-cycle management best practice collection is contained in the ISO standard 20000, as further refined by the ITIL V3 documentation. The ITIL V3 (ISO 20000 based) should be considered a fundamental system architecture design guidance for the CSSA. The following Figure 8 shows pictorially the concepts presented in the ITIL V3 documentation. Figure 8 hints at the wealth of guidance for implementing life cycle management that is available in ITIL V3.

There could be other internationally recognized standard ways to introduce life-cycle management which need to be further investigated.

For more details on recommended life-cycle management best practices related to the technical service refer to the future IALA document Life-Cycle Considerations.

- Example of life-cycle management modules for CSSA as informed by ITIL V3 (ISO 20000 based) [Source: Wikipedia]

# References

[1] 	IALA Recommendation e-NAV 140 on the e-Navigation Architecture. Ed. 1.0, Dec. 2009.

[2]	IMO/MSC-85/26, Add.1, Annex 20 entitled “Draft Strategy for the Development and Implementation of e-Navigation” and Annex 21 “Draft Framework for the Implementation Process for the e-Navigation Strategy“.

[3]	MSC86/23/4 as adopted by MSC86.

[4]	IMO NAV57/15, paragraphs 6.31ff, explicitly referencing NAV57-WP.6, paragraphs on the overarching e-Navigation architecture and in particular the Figure.

# Abbreviations

The following abbreviations are used throughout this recommendation. The mnemonics used to designate technical services of the CSS are in italics.

AIS		Automatic Identification System

AIS		AIS Service of the CSS

AVC		Aviation Communication Service of the CSS

VID		CCTV Video Service of the CSS

CMDS		Common Maritime Data Structure

CSS 		Common Shore-based System

CSSA 		Common Shore-based System Architecture

DFS		Direction Finding Service of the CSS

DGN		DGNSS Augmentation Service of the CSS

DGNSS	Differential Global Navigation Satellite Service

DMS 		Data Mining Service of the CSS

DP		Technical Development Personnel

DSC		Digital Selective Calling

DSC 		GMDSS VHF DSC Service of the CSS

ENE		Environmental Data Evaluation Service of the CSS

ENS		Environmental Sensor Service of the CSS

FAL		Facilitation Committee of IMO

FLA		Floating Visual Aids Service of the CSS

FXA		Fixed Visual Aids Service of the CSS

GNSS		Global Navigation Satellite System

GWY		Gateway Service of the CSS

HMI		Human-Machine-Interface

IHO		International Hydrographic Organization

IMO		International Maritime Organisation

IMSAS		IMO Member States Audit Scheme

INF		On-Site Infrastructure of the CSS

INS		Integrated Navigation System, as defined by IMO Performance Standards

INS 		Traffic Information Service, as part of the Vessel Traffic Services

ISO		International Standardization Organisation

IT		Information Technology

ITIL V3	IT Infrastructure Library (V3 = Version 3, service-oriented)

LPA		Local Public Address Service of the CSS

M2M		Machine-to-Machine Interface

MRCC		Maritime Rescue Coordination Centre

MSI		Maritime Safety Information

MSP(s)	Maritime Service Portfolio(s)

MWB		Medium Wave Broadcast Service of the CSS

NAS		Navigational Assistance Service

NDT		NAVDAT Service of the CSS

NTX		Navtex Service of the CSS

OEP		Object-oriented Engineering Process

OP		Technical Operating Personnel

OPS		Operational Presentation Surface

PNT		Positioning, Navigation and Timing

POS		Position Determination Service of the CSS

RAD		Radar Service of the CSS

SBN		Shore-Based Wide Area Network Service of the CSS

SDA		Ship Data Consistency Analysis Service of the CSS

SID		Shipping Industry Database Service of the CSS

SWC		Short Wave Communication Service of the CSS

SWG		Short Wave GMDSS Service of the CSS

TOS		Traffic Organisation Service

UIA		User Interaction Service of the CSS

VEC		Vector Chart Service of the CSS

VHF		Very High Frequency

VHF		VHF Communication Service of the CSS

VID		CCTV Video Service (Closed Circuit Television) of the CSS

VTMIS		Vessel Traffic Management and Information Services

VTS		Vessel Traffic Services

WWRNS	World Wide Radionavigation System

“Regional” in the context of this Recommendation is seen from a global perspective, i.e. a region could be a continent or a substantial part of a continent. E. g. the Baltic Sea as a whole is a region in this regard.

According to the IALA Constitution a National Member is an authority of any country, or any part of that country, which is legally responsible for the provision, maintenance or operation of marine aids to navigation within that country, or any part of that country (hereinafter referred to as National Member).

It should also be noted, that Figure 2 returns the usage of the term VTS completely to the operational realm. ”Vessel Traffic Services” (VTS) is an operational term and comprises the three operational services: Traffic Information Service, Traffic Organisation Service, and Navigational Assistance Service (IALA VTS Manual).  Over time, the term VTS came to be used as a synonym for the technical system, which supports the above operational services.  Common usage was to consider a “VTS” as a well defined set of certain technologies, in particular radar and VHF voice communication. When referring to the technical equipment needed to support the Vessel Traffic Services, the technical term VTS System should be used.

This implies that information needs and formats are captured and stated in appropriate documentation.

Data objects are defined by the Common Maritime Data Structure (CDMS).

It should be noted that voice is also construed as being data in this context; in modern IT systems voice is encoded into a digital data format.

Essential system requirements are defined as being both a set of minimum requirements and as a means to exploit synergies due to commonalities or similarities detected. Thus the essence is established, hence essential system requirements.

While there is presently no direct definition of “e-Navigation compliancy” or of an “e-Navigation compliant” operational or technical service or device provided by IMO, the working and therefore tentative definition can be inferred from the IMO e-Navigation strategy. “e-Navigation compliant” would mean that an operational or technical service or device has been proven, tested, or checked by a competent body to be in conformity with relevant IMO performance standards, which were expressively created or revised as part of the implementation of IMO’s e-Navigation strategy.

It should be noted, that by convention the individual technical services (i.e. the instances in the object-oriented terminology) are capitalised (“Service”) as opposed to technical services (using small letters) in a generic sense (i.e. the class descriptions in object-oriented terminology).

It should also be noted, that there is no such thing as a “super-service”, which would manage or control all services. Such a “super-service” is unnecessary when adhering properly to the object-oriented paradigm and would also constitute single point of failure of the CSStowever hore-based e-Navigation gineering methodology tp provide comprehensive documentation of the00000000000000000000000000000CSSC. (However, this does not contradict a centralised control of the technical operation of the CSS or its centralised maintenance.)

In the future, this service may also be used for ranging or timing signals.

Today, this communication is done mainly using analogue voice communication technology, but there is an opportunity for innovation to eventually replace the present analogue technology by high-capacity digital voice/data communication means in the VHF maritime mobile service band.

This service has started its legacy period, as administrations, e.g. the US; have relinquished their watch on these frequencies.

Presently, there are plans for a future NAVDAT service intended to extend the service provided by Navtex.

The introduction of advanced Maritime Safety Information (MSI) is being considered, which may affect the ENE and the VEC Services.

The Maritime Service Portfolios (MSPs) and the Maritime Identity Registry parts of the future MSP Almanac may be located here, too, since both have a strong relationship to the maritime stakeholders.

An introduction to the concept of Nodes and the data distribution model can be found in another IALA Recommendation on the generic service model of a technical service.

While this definition may appear simplistic, it introduces far reaching consequences considering the many existing and envisaged operational and technical services in the maritime domain and the aspiration of the e-Navigation strategy of IMO to harmonize those services internationally.

A full introduction and discussion is given in IALA recommendation e-NAV-140 “The e-Navigation Architecture - the Shore-based Perspective” (Ed. 2), Section 7. The figure is reproduced here for convenience.

Compare IALA recommendation on the generic service layout.

ITIL = IT Infrastructure Library. The Version 3 (V3) has incorporated the service-oriented system layout paradigm.