ANNEX 1

IALA Product Specification 1xxx

AtoN Information

Draft  0.0.9 – September 2013

Contents	Page

1 Overview	1

1.1    Introduction	1

1.2   References	1

1.3   Terms, definitions and abbreviations	1

1.4   IALA-1xxx  General Data Product Description	2

1.5   Data specification metadata	3

2 Specification Scopes	3

3 Data Product Identification	3

4 Data Content and structure	5

4.1   Application Schema 	5

4.2   Feature Catalogue 	6

4.3   Feature Types	7

4.3.1   Meta Feature Types	7

4.3.2   Geographic Feature Types 	7

4.4   Attributes	7

4.4.1   Numeric Attribute Values 	7

4.4.2   Text Attribute Values 	7

4.4.3   Mandatory Attribute Values 	7

4.5   Geometry	7

5 Coordinate Reference Systems (CRS)	7

5.1   Introduction 	7

5.2   Horizontal Geodetic Datum 	7

5.3   Vertical Datum 	8

5.4   Units of Measure	8

6 Data Quality	9

6.1   Quality, Reliability and Accuracy of Data	9

7 Data Capture and Classification	10

8 Data Maintenance	12

8.1   Introduction	12

8.2   IALA-1xxx Product Specification Maintenance	12

8.2.1   Maintenance Procedures	12

8.3   Data Maintenance	12

9 Portrayal	13

10 Data Product format (encoding)	14

11 Data Product Delivery 	15

11.1   Feature and Portrayal Catalogue Delivery	15

12 Metadata	16

13 Language 	17

# Overview

## Introduction

The Aids to Navigation (AtoN) Information Product Specification provides a common structure for the exchange of information about AtoNs. This includes lights, buoys, beacons, racons, AIS AtoN and audible signals. The product contains the positions, properties, operational status and general comments related to an AtoN.

The Product Specification can be used to exchange AtoN information in a consistent form between Lighthouse Authorities, Hydrographic Offices and other organizations (to include commercial and professional agencies). This information can be expanded and used to update Electronic Navigational Charts and other nautical products and publications. It may also be used to convey Maritime Safety Information related to AtoN.

### References

### Normative references

IHO S.100 IHO Universal Hydrographic Data Model, January 2010

### Informative references

IMO, 2008. Safety of Navigation Circular SN/Circ.243

IHO S.52

IALA, 2012. Guideline 1088 on an Introduction to Preparing S-100 Product Specifications.

IALA 2010.IALA Navguide.

### Terms, Definitions & Abbreviations

Terms and definitions in addition to those in Annex A of S-100 are as given in the IALA Navguide.

### Abbreviations

IALA-AISM International Association marine aids to navigation and Lighthouse Authorities

CRS		Coordinate Reference System

ECDIS		Electronic Chart Display Information System

ENC		Electronic Navigational Chart

IHO		International Hydrographic Organization

IMO		International Maritime Organization

ISO		International Organization for Standardization

## Data product specification metadata

Title:		Aids to Navigation Information Product Specification

IALA-1xxx Version: 	0.0.9

Identifier:		<Unique IALA identifier>

S-100 Version:		1.0.0

Date:		27 Sep 2013

Language:		English

Classification:		001-unclassifed

Contact:		IALA-AISM

10, rue des Gaudines

78100 Saint Germain en Laye, France

Telephone +33 1 34 51 70 01  Fax+33 1 34 51 82 05

URL: 			http://www.iala-aism.org/IALA-1xxx

Maintenance:				  This Product Specification is maintained by WG6 of the IALA e-NAV 			  Committee on a 4 yearly cycle.

# Specification Scope

Level:			General Scope

# DATA PRODUCT IDENTIFICATION

Title:	Aids to Navigation Information (IALA – AtoN)

Alternate Title:	IALA - AtoN

Abstract: 	Positions and signals provided by Aids to Navigation. Additional general information can also be included.

Content: 	This Product Specification is a complete description of all the appropriate features, attributes and their relationships necessary to define an AtoN Information product. The detailed content is documented within the Feature Catalogue and the relationships defined in the Application Schema. Details of how these features should be symbolised are contained in the associated Portrayal Catalogue.

Geographic Description: Areas specific to marine navigation and inland waterways.

Spatial Extent:

Description: Global, marine, inland waterways

East Bounding Longitude: 180

West Bounding Longitude: -180

North Bounding Latitude:  90

South Bounding Latitude:   -90

Temporal Extent:      Not Applicable

Spatial Resolution:

Purpose: 	The purpose of an AtoN Information dataset is to exchange positions and signals provided by Aids to Navigation. Additional general information can also be included.

Language: 	English

Spatial Resolution Type:	Vector

Point of Contact: 		Producer

Use Limitation: 					 None

# Data Content and structure

Introduction

An AtoN Information dataset is a feature-based product.  The following sub-sections contain the product application schema expressed in UML and an associated feature catalogue.  The feature catalogue provides a full description of each feature type including its attributes and attribute values in the data product.

## Application Schema

Figure 1 AtoN Information Application Schema

Figure 2 Enumeration 1

Figure 3 Enumeration 2

## Feature Catalogue

(To be produced once application schema is stable)

A Feature Catalogue in XML format shall be available from IALA with an accompanying XSL Stylesheet.

Name: AtoN Information Feature Catalogue

Scope:

Field of application:

Version Number: 0.1

Version Date:

Producer: IALA

Functional Language: English

Feature Types

Meta Feature Types:  No Meta Feature Types are included in this product.

Geographic Feature Types:  Geographic feature types form the principal content of the data product and are fully defined in the Feature Catalogue.

Attributes

Numeric Attribute Values:  Floating point or integer attribute values must not be padded by non-significant zeroes.

### Text Attribute Values:  Character strings must be encoded using the character set specified in Unicode Transformation Format-8 (UTF-8).

### Mandatory Attribute Values:  Where attributes determine the display of a Feature, or essential to the overall feature type construction they may be mandatory. All mandatory attributes are identified in the feature catalogue.

# Coordinate Reference Systems (CRS)

## Introduction

The coordinate reference systems are separated into the horizontal and vertical components.

## Horizontal Geodetic Datum

WGS84 (World Geodetic System of 1984) shall be used for the horizontal reference system for spatial data (EPSG code 4326).

## Vertical Datum

A vertical datum attribute will be required on all relevant features and should be specified in the data set metadata (for example Highest Astronomical Tide).

# Data Quality

## Quality, Reliability and Accuracy of Data

Attributes at the feature level indicate method of positioning and date of observation or confirmation of operation. These attributes are defined in the Feature Catalogue.

In dataset metadata the overall positional uncertainty and date to which the dataset has been validated must be provided.

# Data Maintenance

Datasets defined in this specification are maintained by the producer on an as required basis.  Updating is done incrementally (change notes), including meta data.

# Portrayal

A Portrayal Catalogue defines the display rules and symbology which apply to the Features defined in this specification. The Portrayal Catalogue is not provided at present, but may be developed, using S-52 ENC portrayal as an input and reference to IMO SN/Circ. 243.

# Data Product format (encoding)

| formatName | S-100 GML |
| --- | --- |
| version | 1.0 |
| characterSet | UTF-8 |
| specification | S-100 GML Profile 1.0 |

The S-100 GML Profile 1.0 is provided with this data specification as it is not yet included in S-100. A GML application schema .XSD document is also provided to support data validation. This data specification is constrained to level 1 of this GML profile as specified in the GML application schema document.

Example encoding;

<?xml version="1.0" encoding="UTF-8" ?>

<?Sample data created by UKHO using version 2.1 of the draft AtoN Application Schema?>

- <gml:FeatureCollection gml:id="_1" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns:xs="http://www.w3.org/2001/XMLSchema" xmlns:a201="http://www.iho.int/A-201" xmlns:gml="http://www.opengis.net/gml/3.2" xmlns:xlink="http://www.w3.org/1999/xlink" xmlns:smil20="http://www.w3.org/2001/SMIL20/" xmlns:smil20lang="http://www.w3.org/2001/SMIL20/Language" xsi:schemaLocation="http://www.iho.int/A-201 a201_AtoN_GML.xsd">

- <gml:featureMember>

- 	<a201:FloatingAtoN gml:id="_2">

- 		<a201:geometry>

- 			<gml:Point gml:id="_3">

<gml:pos>-0.68483 50.64309</gml:pos>

</gml:Point>

</a201:geometry>

<a201:atonID />

<a201:atonOwnerAuthority>Trinity House</a201:atonOwnerAuthority>

<a201:atonType>lit AtoN</a201:atonType>

<a201:commonName>Owers</a201:commonName>

<a201:horizontalAccuracy>20</a201:horizontalAccuracy>

<a201:ialaCategory>category 2</a201:ialaCategory>

<a201:ialaRegion>A</a201:ialaRegion>

<a201:internationalID>A4411</a201:internationalID>

<a201:locationID>1234</a201:locationID>

<a201:locationPortName>Channel</a201:locationPortName>

- 		<a201:daymarkTopmark>

- 			<a201:daymarkTopmark>

<a201:topmarkColour>black</a201:topmarkColour>

<a201:topmarkShape>2 cones (points downward)</a201:topmarkShape>

</a201:daymarkTopmark>

</a201:daymarkTopmark>

<a201:markColour>yellow</a201:markColour>

<a201:markColour>black</a201:markColour>

<a201:shape>pillar</a201:shape>

- 		<a201:equipmentRacon>

- 			<a201:Racon gml:id="_4">

<a201:geometry />

<a201:atonID />

<a201:atonOwnerAuthority>Trinity House</a201:atonOwnerAuthority>

<a201:atonType>racon</a201:atonType>

<a201:horizontalAccuracy />

<a201:ialaCategory>category 2</a201:ialaCategory>

<a201:ialaRegion>A</a201:ialaRegion>

<a201:locationID>1234</a201:locationID>

<a201:locationPortName>Channel</a201:locationPortName>

<a201:raconCharacter>O</a201:raconCharacter>

<a201:raconType>UNKNOWN</a201:raconType>

<a201:raconRange>UNKNOWN</a201:raconRange>

</a201:Racon>

</a201:equipmentRacon>

<a201:equipmentLight>

-			 <a201:Light gml:id="_5">

<a201:geometry xlink:href="_3" />

<a201:atonID />

<a201:atonOwnerAuthority>Trinity House</a201:atonOwnerAuthority>

<a201:atonType>lit AtoN</a201:atonType>

<a201:horizontalAccuracy>20</a201:horizontalAccuracy>

<a201:ialaCategory>category 2</a201:ialaCategory>

<a201:ialaRegion>A</a201:ialaRegion>

<a201:locationID>1234</a201:locationID>

<a201:locationPortName>Channel</a201:locationPortName>

<a201:lightColour>white</a201:lightColour>

<a201:lightDescription>Q(6)+LFl W 15s</a201:lightDescription>

<a201:nominalLightRange>UNKNOWN</a201:nominalLightRange>

- 				<a201:rhythmOfLight>

- 					<a201:rhythmOfLight>

<a201:lightCharacteristic>quick-flash plus long flash</a201:lightCharacteristic>

<a201:signalGroup>(6)(1)</a201:signalGroup>

<a201:signalPeriod>15</a201:signalPeriod>

</a201:rhythmOfLight>

</a201:rhythmOfLight>

</a201:Light>

</a201:equipmentLight>

# Data Product Delivery

| Item Name | Description |
| --- | --- |
| unitsOfDelivery | Datasets |
| transferSize | Undefined |
| mediumName | Undefined |
| otherDeliveryInformation |  |

## Feature and Portrayal Catalogue Delivery

This specification does not currently include a Portrayal Catalogue. Feature Catalogues are provided on the IALA website and users informed by a change notice on that site when any changes occur.

# Metadata

The data product specification metadata shall include the following items in Table 2 [extension to ISO 19131]:

| Item Name | Description | Multiplicity | Type |
| --- | --- | --- | --- |
| title | Title of the data product specification | 1 | CharacterString |
| version | Version of the data product specification | 1 | CharacterString |
| date | Date the product specification was created / last updated | 1 | Date |
| language | Language(s) of the data product specification, e.g. translations | 1..* | CharacterString |
| classification | Security classification code on the data product specification | 0..1 | MD_ClassificationCode (ISO 19115) |
| contact | Party responsible for the data product specification | 1 | CI_ResponsibleParty (ISO 19115) |
| URL | Online-address where the resource is downloadable | 0..1 | URL |
| identifier | Persistent unique identifier for a published version of the product specification1. | 1 | CharacterString |
| maintenance | Description of the maintenance regime for the product specification. | 1 | MD_MaintenanceInformation (ISO 19115) |

Table 11-2— Data product specification metadata