e-NAV14	Input paper

Agenda item	8.2

Task Number	2

Submitter	Natacha Riendeau, Canadian Coast Guard

Concept of Operations for e-Navigation in Canada

# Summary

This paper provides a summary of the concept of operations for the implementation of e-Navigation services in Canadian waters.  Based on Canada’s Vision for e-Navigation, it provides additional details on how e-Navigation information may be accessed and exchanged in Canada. This concept of operations is still under development, and will continue to be refined based on stakeholder input.

## Purpose of the document

Committee members are encouraged to PROVIDE comments on the concept of operations for e-Navigation that is being followed in Canada.

# Background

In the early 2000’s, the Canadian Coast Guard (CCG) began looking at better ways of communicating and integrating navigational information for the benefit of both mariners and shore-based authorities.  Since then, testbed projects have enabled Canada to identify and to address many important issues such as the way information can be collected, exchanged, transmitted and displayed. The success of these projects was due in great part to the close collaboration among mariners, pilots and the governmental organizations responsible for collecting and providing data and information. Marine pilots have played a key role as well in test beds, either in supplying computerized-devices onboard (Portable Pilot Units) which allow displaying information and traffic, or by collectively working for the transmission of environmental data (e.g. wave height, currents, etc.) from ‘smart buoys’, web portals, and the AIS base stations.

In August 2012, a national Vision for implementing e-Navigation in Canada was officially approved by all contributing governmental organizations. Based on comprehensive users needs, this national document describes the Canadian Vision for e-Navigation, its guiding principles, objectives and the implementation strategy that Canada has been following, and will continue to follow over the coming years. In concrete terms, this Vision is guiding stakeholders in sharing and communicating the required data/information services through the use of a national portal and other communication means such as the AIS base stations.  These services have been identified and are continually validated through comprehensive interactions with users and reflected in a Required Services Matrix.

# Discussion

The overall concept of operations for e-Navigation in Canada is that shore authorities will make data accessible to users via a single, Canadian national e-Navigation web Portal, called the Maritime Information Portal.  The CCG’s role will be to maintain and support the Common Maritime Information Infrastructure in collaboration with other Government partners who maintain responsibility for the data.  CCG will work to ensure information is provided in the right format while avoiding duplication of information whenever possible.  The overall concept is that necessary data and information will be made available to users.  However, it will be their responsibility of the industry to develop the technology and tools to facilitate its access and integration, subject to international regulations.  Some information updates or notification of new information will also be provided through the AIS Network and other electronic means, such as Very High Frequency Data Exchange (VDE).

- Canada e-Navigation Draft Concept

- Phase I, Voyage Planning

This is the beginning of the journey where mariners will prepare for their upcoming voyage by gathering the information relevant to the upcoming trip from the e-Navigation portal, and then creating a voyage plan.

Regulation 34 of Chapter V of the Convention for the Safety of Life at Sea (SOLAS) and the Charts and Nautical Publications Regulations, 1995 require masters to ensure the intended voyage has been adequately planned.  The degree of voyage planning depends upon the size of the vessel, its crew and the length of the voyage.

Voyage planning, as set out in IMO's Guidelines for Voyage Planning, involves four distinct stages:

- Appraisal – Gathering the information relevant to the coming trip;

- Planning – Preparing a detailed plan of the expected trip and alternate plans; this includes establishing waypoints, identifying hazards, setting the times for passing certain landmarks and decision points where the mariner must decide to proceed or engage alternate plans;

- Execution – Implementing the plan and making the necessary decisions and, if necessary, implementing alternate plans;

- Monitoring – Keeping an eye on your progress and the effectiveness of the plan's execution.

Information that users may gather from the Maritime Information Portal in order to develop their voyage plan include:

- Meteorological Information (e.g., wind speed, visibility, weather conditions, wave height and direction, air and water temperature, atmospheric pressure, and related weather warnings);

- Ice Information (e.g., ice charts, routing advice, forecasts, advisories and bulletins);

- Hydrographical Information (e.g., forecast tides, real-time water levels, predicted water levels, forecast current);

- Aids to Navigation Information (e.g., status of existing aids to navigation and related buoy tending information);

- Navigational Charts (and updates);

- Restrictions to Navigation (e.g., construction work, temporary restricted areas, harbour closures, etc.);

- Vessel Traffic Services (e.g. notices to shipping)

A voyage or sail plan includes the travel route and basic details about the vessel.

Once the user prepares a voyage plan, using information downloaded from the Maritime Information Portal, mariners may share the plan with a responsible person on shore, or one of the Canadian Coast Guard Marine Communications and Traffic Services (MCTS) Centres.

- Phase II, Voyage

The second phase is the voyage itself where most of the data gathered from the Maritime Information Portal during the first phase (Voyage Planning) is now used by navigators.

Navigators will have planned their voyage based on updated nautical charts, forecast weather conditions and warnings, ice information and routing advice, predicted water levels, forecasted currents, etc., taking care to review any applicable notices to shipping or relevant status updates for aids to navigation.  However, once at sea, conditions may change and navigators are required to make decisions and implement alternate voyage plans based on current or real-time information.

Implementation of e-Navigation in Canada will allow users to receive updated information or real-time information via telecommunication means, such as AIS, Very High Frequency Data Exchange (VDE) or the internet, when available.

Providing updated and real-time information to users will have clear benefits for safety, security and efficiency, as decisions will be based on better, more reliable information.  For example, real-time navigational buoy status may be transmitted to users via AIS.  This means that navigators will be able to confirm the exact location of the buoy and status of its light, allowing navigators to rely upon the navigational mark and avoid marked hazards.  Similarly, available water depths (real-time and forecasted) may be transmitted to users.  This will provide mariners with up-to-date information on the actual availability of the water column to ensure they optimize their cargo to a safe level and to avoid potential groundings.

- Phase III, Post-Voyage

The third phase is the post-voyage phase where most of the data is analyzed by navigators and shore-based authorities.  Analysis of the information provides validation that the information that was provided during Phase I (voyage planning) or transmitted during Phase II (voyage) was precise, timely, and fit for purpose.  Following analysis, reports can be produced and shared between users and shore-based authorities.  This phase in the cycle allows the creation of a consolidated maritime picture, providing benefits and opportunities to shore-based authorities and mariners to improve processes, programs, operations, updates, as well as providing training opportunities for navigators.

- Next steps

As part of the concept of operations, the Canadian Coast Guard is currently working on how the data and information will be provided to the mariners. This involves data formatting, services standards (reliability, availability) and the IALA Common Shore-based System Architecture.

e-Navigation will impact shore-based information systems, as they will require harmonization with international direction.  Most shore-based systems will require changes or eventual replacement in order to support e-Navigation services, and to achieve full e-Navigation compliance. Although not completed, the impact on international shore-based system architecture may be different for each shore-based system, and will also depend on the services provided.

Moving forward, the development of e-Navigation in Canada will continue to be user-driven, taking into consideration user requirements, as well as national and international directions, guidelines, recommendations and regulations.

# References

- This document was written by Natacha Riendeau and André Châteauvert in collaboration with Jean-François Coutu and Stéphane Lessard.  Any questions or comments related to this paper can be sent to: natacha.riendeau@dfo-mpo.gc.ca

- Draft Concept of Operations for e-Navigation in Canada document by CCG (January 2012 version1)

- e-Navigation in Canada –Vision and Strategy (August 2012)

- Canadian Coast Guard Internet Website for e-Navigation

- http://www.ccg-gcc.gc.ca/e-Navigation

# Action requested of the Committee

Committee members are encouraged to PROVIDE comments on the concept of operations for e-Navigation that is being followed in Canada.

- ************

http://www.ccg-gcc.gc.ca/e-Navigation