This paper addresses the task of the NAVGUIDE update (Task 29; compare e-NAV14-1.2Rev1). This task is supposed to be complete BY THE END of the session week. Hence, I took the liberty to review the relevant documents e-NAV14-8.8.1, 8.8.2, 8.8.3, and in particular 8.8.4 "Draft NAVGUIDE", chapter 4, in regards to WG5 areas of interest.

I produced a revised version (track changes), attached, which I politely ask WG5 to review and indicate any desired amendments.

Upon meeting in break-out session during committee week I will then call the document, as amended, and do hope that we will have a brief approval of our contribution to that document.

Best regards,

Jan-Hendrik Oltmann

Dezernat Maritime Verkehrstechnik

Telefon +49 (0)431 3394 5701

Telefax +49 (0)431 3394 6399

Mobil +49 (0)172 15 26 928

Jan-Hendrik.Oltmann@wsv.bund.de

Generaldirektion Wasserstraßen und Schifffahrt

Außenstelle Nord

Hindenburgufer 247

24106 Kiel

www.ast-nord.gdws.wsv.de

www.wsv.de

- e-NAVIGATION

## Introduction

e-Navigation is an IMO-led broad strategic vision for the harmonisation of marine navigation systems and supporting shore services, underpinned by user needs.

The concept involves the utilization and integration of all available navigational tools to secure a greater level of safety and accident prevention.  Implementation of e-Navigation will, at the same time, deliver substantial operating efficiencies with resulting commercial benefits.

e-Navigation will incorporate the use of new technologies in a structured way and ensure that their use is compliant with the various electronic navigational and communication technologies and services that are already available.

It should be noted that without e-Navigation, the multiplicity of systems and equipment will continue to evolve, with varying degrees of effectiveness.  e-Navigation is an opportunity to optimise these developments and ensure the focus of future developments is based on a holistic approach to safe berth-to-berth navigation.

Research indicates that around 60% of collisions and groundings are caused by direct

human error. Despite advances in bridge resource management training, it seems that the majority of watch-keeping officers make critical decisions for navigation and collision avoidance in isolation, due to a general reduction in manning.

In human reliability analysis terms, the presence of someone checking the decision-making process improves reliability by a factor of 10. If e-navigation could assist in improving this aspect, both by well-designed onboard systems and closer cooperation with vessel traffic management instruments and systems, risk of collisions and grounding and their inherent liabilities could be dramatically reduced.

However, although e-navigation may be able to improve the situations described above, there is also a need to recognize the role of the practice of good seamanship, the provision of suitable training and the use of procedures.

In 2006, seven IMO Member States made a joint submission to the Maritime Safety Committee to “develop a strategic vision for the utilization of existing and new navigational tools, in particular electronic tools, in a holistic and systematic manner.”

IMO, supported by other international organisations such as IALA and IHO and the maritime industry, has made substantial progress in translating the concept of e-navigation to an operational reality.  It is expected that by 2014, IMO will approve an e-Navigation Strategy Implementation Plan (SIP).

## Definition of e-Navigation

The definition of e-Navigation adopted by IMO is:

“e-Navigation is the harmonised collection, integration, exchange, presentation and analysis of maritime information onboard and ashore by electronic means to enhance berth to berth navigation and related services, for safety and security at sea and protection of the marine environment”

WHAT DOES THE “e” IN e-NAVIGATION STAND FOR?

It is generally accepted that the IMO concept of “e-Navigation” can be thought of as a brand, without the need for “e” to be specifically defined.

The concept of e-Navigation was proposed by IMO Member States in 2006 as a process for the harmonisation, collection, integration, exchange and presentation of maritime information. As such, the “e” could have stood for “enhanced” or “electronic”, but this would unnecessarily limit what can be done within e-Navigation. It must be noted that generic electronic marine navigation already exists in many forms and should not be confused with this specific IMO initiative.

## A vision for e-Navigation

A vision of e-navigation is embedded in the following general expectations for the

onboard, ashore and communications elements:

On board

Navigation systems that benefit from the integration of own ship sensors,

supporting information, a standard user interface, and a comprehensive system for

managing guard zones and alerts. Core elements of such a system will include,

actively engaging the mariner in the process of navigation to carry out his/her

duties in a most efficient manner, while preventing distraction and overburdening.

Ashore

The management of vessel traffic and related services from ashore enhanced

through better provision, coordination, and exchange of comprehensive data in

formats that will be more easily understood and utilized by shore-based operators

in support of vessel safety and efficiency.

Communications

An infrastructure providing authorized seamless information transfer on board

ship, between ships, between ship and shore and between shore authorities and

other parties with many related benefits.

## Strategy and Implementation

In 2008, IMO approved the development of an e-Navigation Strategy Implementation Plan.  This includes the development of a technical architecture, gap analysis, cost benefit analysis and the creation of a detailed implementation plan.  A structured and phased approach is required to capture evolving user needs, making use of the existing agreed methodology.

The Strategy Implementation Plan includes priorities for deliverables, a schedule for implementation and provision for the continual assessment of user needs.   Implementation will be a phased, iterative process.  The architecture for e-navigation will encompass hardware, data, information, communications technology and software.  It will be based on a modular and scalable concept and will cater for continued development and enhancements.

The deployment of new technologies is to be based on a systematic assessment of how the technology can best meet defined and evolving user needs within the e-navigation concept. Development of usability standards and further development of the IMO Human Element Analyzing Process (HEAP) will also be undertaken

Implementation of e-Navigation will commence once IMO approves a Strategy Implementation Plan (expected to be in 2014), which is expected to include:

- Identification of responsibilities to appropriate organizations/parties;

- Transition arrangements;

- A phased implementation schedule along with possible roadmaps;

- Priorities for deliverables, resource management and a schedule for implementation and the continual assessment of user needs;

- Proposals for a systematic assessment of how new technology can best meet defined and evolving user needs;

- A plan for the development of any technology and institutional arrangements necessary to fulfil the requirements of e-navigation in the longer term;

- Proposals on public relations and promotion of the e-navigation concept to key stakeholder groups;

- Identification of potential sources of funding for development and implementation, particularly for developing regions and countries and of actions to secure that funding.

High Level User Needs

The IALA methodology was used to capture evolving user needs. It was based on the elements contained within the accepted definition of e-navigation and applied templates to define specific user needs based on the harmonized collection, integration, exchange, presentation, analysis and human element aspects for all users. Following extensive feedback from Member States, other maritime organizations, and interested parties, an analysis was conducted resulting in the identification of high-level generic user needs for both ship and shore users. Thus the needs of a typical SOLAS ship and a generic shore authority have been used as the basis for the identification of the high-level user needs described below. A more detailed user needs may have to be identified as a part of the implementation plan.

- Common Maritime Information/Data Structure

Mariners require information pertaining to the planning and execution of voyages,

the assessment of navigation risk and compliance with regulation. This

information should be accessible from a single integrated system. Shore users

require information pertaining to their maritime domain, including static and

dynamic information on vessels and their voyages. This information should be

provided in an internationally agreed common data structure. Such a data

structure is essential for the sharing of information amongst shore authorities on a

regional and international basis.

- Automated and Standardized Reporting Functions

E-navigation should provide automated and standardized reporting functions for

optimal communication of ship and voyage information. This includes

safety-related information that is transmitted ashore, sent from shore to ship borne

users and information pertaining to security and environmental protection to be

communicated amongst all users. Reporting requirements should be automated or

pre-prepared to the extent possible both in terms of content and communications

technology. Information exchange should be harmonized and simplified to reduce

reporting requirements. It is recognized that security, legal and commercial issues

will have to be considered in addressing communications needs.

- Effective and Robust communications

A clear need was expressed for there to be an effective and robust means of

communications for ship and shore users. Shore-based users require an effective

means of communicating with vessels to facilitate safety, security and

environmental protection and to provide operational information. To be effective,

communication with and between vessels should make best use of audio/visual

aids and standard phrases to minimize linguistic challenges and distractions to

operators.

- Human Centred Presentation Needs

Navigation displays should be designed to clearly indicate risk and to optimize

support for decision making. There is a need for an integrated “alert management

system” as contained in the revised recommendation on performance standards for Integrated Navigation Systems (INS) (resolution MSC.252(83)).

Consideration should be given to the use of decision support systems that offer suggested responses to certain alerts, and the integration of navigation alerts on board ships within a whole ship alert management system. Users require uniform and

consistent presentations and operation functionality to enhance the effectiveness of internationally standardized training, certification and familiarization.

The concept of S-Mode has been widely supported as an application on board ship

during the work of the Correspondence Group. Shore users require displays that

are fully flexible supporting both a Common Operating Picture (COP) and a User

Defined Operating Picture (UDOP) with layered and/or tabulated displays. All

displays should be designed to limit the possibility of confusion and

misinterpretation when sharing safety-related information. E-navigation systems

should be designed to engage and motivate the user while managing workload.

- Human Machine Interface

As electronic systems take on a greater role, facilities need to be developed for the

capture and presentation of information from visual observations, as well as user

knowledge and experience. The presentation of information for all users should

be designed to reduce “single person errors” and enhance team operations. There

is a clear need for the application of ergonomic principles, both in the physical

layout of equipment and in the use of light, colours, symbology and language.

- Data and System Integrity

E-navigation systems should be resilient and take into account issues of data

validity, plausibility and integrity for the systems to be robust, reliable and

dependable. Requirements for redundancy, particularly in relation to position

fixing systems, should be considered.

- Analysis

E-navigation systems should support good decision making, improve performance

and prevent single person error. To do so, shipboard systems should include

analysis functions that support the user in complying with regulations, voyage

planning, risk assessment, and avoiding collisions and groundings including the

calculation of Under Keel Clearance (UKC) and air draughts. Shore-based

systems should support environmental impact analysis, forward planning of vessel

movements, hazard/risk assessment, reporting indicators and incident prevention.

Consideration should also be given to the use of analysis for incident response and

recovery, risk assessment and response planning, environment protection

measures, incident detection and prevention, risk mitigation, preparedness,

resource (e.g., asset) management and communication.

- Implementation Issues

Best practices, training and familiarization relating to aspects of e-navigation for

all users should be effective and established in advance of technical

implementation. The use of simulation to establish training needs and assess its

effectiveness is endorsed. E-navigation should as far as practical be compatible

forwards and backwards and support integration with equipment and systems

made mandatory under international and national carriage requirements and

performance standards. The highest level of interoperability between e-navigation

and external systems should be sought where practicable.

IALA’s Role

IALA’s has contributed substantially to the formulation of the IMO Strategy for the implementation of e-Navigation and to the Strategy Implementation Plan. IALA is developing shore user requirements, Maritime Service Portfolios (MSPs), information systems and data structures, a World Wide Radio Navigation Plan, a Radio Communications Plan, enhanced AIS, VHF Data Exchange and a shore-side architecture, together with a documentation structure to encompass the whole of e-Navigation.

### The IMO adopted overarching architecture

IMO has defined and adopted the overarching architecture as given in Figure 4.5.1 (NAV57, WP.6, as adopted by MSC90, May 2012, refers).

Figure 4.5.1: IMO adopted overarching architecture

Figure 4.5.1 is composed of a number of elements, which are introduced – in a step-by-step fashion – in the following sections.

### User’s perspective of e-Navigation in architectural terms

One way of understanding the concept of e-Navigation is to look at it from a user’s perspective.  As visualized in Figure , the architecture can be divided into three parts; ship side, shore side and the interaction between ship and shore. This represents the overarching architecture from a users’ perspective.

The ship side represents the users on-board a ship, whilst the shore side typically represents users from communities like Vessel Traffic Services (VTS), Allied Services and even users from communities within the logistics domain.

Figure 4.5.2: e-Navigation Users’ Perspective at a Given Moment and at a Given Place

To enable both sides to communicate and to exchange information, e-Navigation uses the general term “service”. From a user’s perspective, the important services will be the “operational services”. However, there also needs to be “technical services” to be able to provide these operational services (Figure ).  Altogether, these services are referred to as the Maritime Service Portfolio (MSP).

A MSP defines and describes the whole set of operational and technical services and the level of these services, as provided by a stakeholder in a given sea area, on a waterway, or in a port, as appropriate. The MSP concept was conceived to achieve harmonisation, simplification, and tighter interaction of services and systems on board and ashore. It will be further developed based upon the IMO concept of a “Common Maritime Data Structure” (CMDS). Both concepts are explained in some more detail below.

Figure 4.5.3: The e-Navigation Services Concept

### Maritime Service Portfolios (MSPs)

A Maritime Service Portfolio (MSP) defines and describes a set of operational and technical services and their level of service provided by a stakeholder in a given sea area, waterway, or port, as appropriate. MSPs should be developed to achieve harmonization, modernization, integration and simplification on board and ashore, taking into account the use of the IHO's S-100 standard.

The objective of the MSP concept is to align global maritime service with the need for information and communication services in a defined operational area. To achieve such, the first step should be to identify the need for information services and communication infrastructure in the different areas.  A set of services will require a certain communication infrastructure capacity, varying from area to area.

The international work on Maritime Service Portfolios (MSPs) has only now started, and several interested parties such as IALA members or regional projects, are creating first contributions. Thus, any definition regarding the details of MSPs at this stage should be considered tentative and subject to change.

IALA has started work to explore the structure of the MSPs, which eventually is expected to be a contribution to IMO when defining MSPs. The following are initial findings from IALA’s work in this regard.

The starting point for understanding the concept and the potential of the MSPs is the individual service. A service is the fundamental building block of any MSP. The generically defined operational and technical services can be arranged in a linear list, which is only subdivided by the distinction of “operational” and “technical”.

Examples of operational services are the well-established services “Navigational Information Service (NIS)”, “Navigational Assistance Service (NAS)”, “Traffic Organisation Service (TOS)” or “Search and Rescue Service (SAR)”, to name a few. Technical services are for example any and all radio communication services. There may be some services which exhibit two faces, depending on how they are approached. Examples for these services are the visual Aids-to-Navigation services, which could be subdivided further into floating and fixed.

It should be noted, that there are interdependency relationships between the services, i.e. an operational service would require one or several technical service(s) to perform its desired functionality or a technical service would require one or several technical service(s) to perform its desired functionality. Thus, there would be interdependency cascades between the services which will become apparent as soon as one would look into the service description. For example, any of the above mentioned examples for operational services would require at least one technical service for radio communication.

The point of the MSP is to purposefully bundle together a selection of those individual operational and technical services into a set and designate a specific name to that set.

Both aspects are equally important:

- The purpose of the intended set determines the selection of the specific operational and technical services to be bundled together in a MSP. The variety of different purposes to compose MSPs introduces a degree of liberty and flexibility.

Possible dimensions for purposes may be e.g.:

  - functional: the purpose would be to provide a certain functionality to the maritime domain (examples are given below).

  - geographical: the purpose would be to designate which services would be available in a defined geographical area.

  - Stakeholder oriented: A specific stakeholder may wish to define his own MSP for describing his portfolio offered to the maritime community.

  - Regulatory oriented: A regulatory body may use a MSP to achieve regulatory goals in simultaneously simple and precise terms, such as prescribing a minimum level of service provision in its area of competency.

- The name of a MSP must be carefully chosen to reflect the purpose of the MSP on one hand, being practical for usage on the other hand. While thus already conveying purpose and providing a means to handle, the name also provides also a means to reduce complexity by encapsulation, i.e. the name of the MSP encapsulates all services bundled  by this MSP.

- Illustration: The relationship between services and MSPs there could be illustrated by analogy given by chemistry. The atomic elements (in chemistry) would by analogy be the services, while the molecules (in chemistry) would be the MSPs. Thus as a limited number of elements (in chemistry) allows for a large number of molecules to be composed (in chemistry), a limited number of well defined services eventually would allow to defined a number of MSPs. This analogy would also hold true in some other important regards: “Water” (in chemistry) is a simple name for the molecule consisting of two hydrogen and one oxygen atoms, and this name meaningful conveys the features of water and also effectively encapsulates all the details of the chemistry of water. The encapsulation of the MSPs in regard to the services could be understood similarly.

Different MSPs will reference eventually the very same services. Thus a synergy is created, because the very same service definition (as contained in appropriate international documentation or standards) will be “re-used”.

The concept of the MSP thus introduces a certain degree of liberty to create MSP definitions, thus providing the required flexibility. To prevent loss of common understanding it would be required to thoroughly standardize the relevant services internationally.

There could exist cascades of MSPs, too, as one MSP may use another existing MSP for ease of reference.

The above description is in the generic domain; i.e. it does not yet consider the several instances of one specific generic service provided by different individual competent authorities or stakeholders. The requirement for international standardization of the services follows from the desire to harmonize the service quality delivered by different individual providers of this service on the instance level. It will be the challenge for any such international standardization to strike an appropriate balance between international harmonization on one hand and necessary flexibility to adapt a service provision to local needs.

A relevant part of the international MSP definitions are, should and will be done at and by IMO.  On the other hand, there are regional, national, and local competent authorities who legitimately would use the MSP concept for their purposes, e. g. by defining their specific MSPs. In addition, there may be stakeholders beyond the competent authorities domain, which also may wish define their specific MSPs for their purposes. The goal of e-Navigation, namely to achieve a maximum harmonization in the maritime domain, would be achieved once there is a common, agreed, standardized list of both services and fundamental MSP definitions, to which the more specific MSPs could point or reference.

Thereby, there is margin to reduce complexity (namely by encapsulation of individual services in MSP, and less comprehensive MSPs in more comprehensive MSPs) while maintaining consistency by using the common building blocks, i.e. the individual services.

MSPs are only valid together with at statement to where they are applied. There have been proposed several equally valid methods of applications of MSPs:

- the MSPs applied to geographical areas, i.e. the coverage concept: IMO already seems to have given an area definition regarding MSPs in this regard: Port/Harbour, Coastal, Ocean, Off-Shore, Remote/Polar. This concept introduces the notion of “blanket coverage”, and there are certain operational and technical services which – due to their nature - are provided in “blanket coverage” fashion.

- the MSPs applied to a shipping route topology based on the analysis of the traffic pattern in a given sea area. Thus, different MPSs may be applied to different routes, according to need of the traffic pattern in a route. This concept would require a Route Topology Model and a traffic pattern analysis of the sea area under consideration.

- the MSPs applied to phases of vessel voyage: Different phases of a vessel’s voyage have different needs for service provisions, i.e. different MSPs to be provided.

It is expected, that there will be identified margin, for all three concepts to co-exist eventually, as it is clearly recognized that not just one concept will satisfy all requirements or fit all relevant circumstances.

Obviously, more work has to be done on this internationally.

### The Common Maritime Data Structure

The purpose of the IMO defined Common Maritime Data Structure (CMDS, see Figure ) is to harmonise data exchange in the maritime domain by providing a common, authoritative reference. The CMDS is an abstract representation of entities within the maritime domain. It should be accessible by any stakeholder or implementer and should be the reference for the development of maritime services, applications and databases.

Figure 4.5.4: The Harmonised Common Maritime Data Structure

Considering the extent of the maritime domain, with all of its stakeholders, the responsibility for the CMDS is subdivided into smaller units, each of which is governed by a recognised authority.  However, these authorities cooperate to harmonise the CMDS as a whole (e.g., to avoid duplication of entries).  This is one of the main tasks of the IMO established IMO/IHO Harmonisation Group on Data Modeling (HGDM).

IHO developed the IHO GI Registry, based on its S-100 standard, as a tool for data modeling for the specification and production of Electronic Navigational Charts (ENC) and Digital Nautical Publications (DNP). The GI Registry is generic in setup and has been adopted by IMO as the tool to develop the CMDS.

Figure 4 describes the simplified generic structure of the GI Registry. The major features of the registry include registers for:

- Product Specifications – includes everything needed to fully describe and specify a product such as data exchange protocols and references to HMI and CMDS entities from the GI Registry.

- Human-Machine Interface (HMI) – HMI definitions/ specifications can also include references to CMDS entities from the GI Registry. (This register is named Portrayal by IHO.)

Figure 1.5.5 Simplified View of the IHO GI Registry

### The shore-based set-up for facilitating e-Navigation

IALA members use shore-based systems (which are composed of products) to provide commonly recognised operational services, for example, in the realms of Aids-to-Navigation and VTS.  Many of these shore-based systems have a similar architecture. This architecture has been captured in the generic Common Shore-based System Architecture (CSSA), and IALA is working on a new IALA recommendation for the CSSA layout.

In Figure 4.5.6, below, the responsibility of the shore-based service provider is within the oval.

Figure 4.6.6 also shows a more detailed representation of the technical services already introduced by Figure 4.6.3. The common elements of the technical services are:

- Data Collection (e.g., radar, AIS, etc.)

- Data Exchange (ship/shore and shore/ship; e.g., VHF Data Exchange, AIS, etc.)

- Data Processing

- Gateway (shore/ shore; e.g., LRIT Service Center, inter-VTS services, etc.)

- Human-Machine Interface (HMI)

Note that not all technical services require all of these elements.

The arrows indicate data flows between these common elements. The data that is exchanged is modeled within the CMDS.

IALA is a Domain Owner within the IHO Registry for the domains of AtoNs, VTS and AIS and is accepted by IHO as a Submitting Organisation.

Figure 4.5.6: Simplified View of the Common Shore-based System Architecture (CSSA)

In addition, e-Navigation will impact the shore-based systems by requiring harmonization. The concept of the CSSA was developed to accommodate the requirements imposed by provision of e-Navigation services.

Existing legacy systems do have varying degrees of compliance to e-Navigation. Most of these systems will require amendment or eventual replacement in order to support e-Navigation services and achieve full e-Navigation compliance.

## Technology for e-Navigation

Many sub-systems or components will have to be developed, enhanced or provided to realise the full integrated concept of e-Navigation. Crucial among these are Positioning, Navigation and Timing (PNT), communications and information systems.  It should be noted that not all of the following technologies or systems will be needed within the e-Navigation concept.

## IALA World Wide Radio Navigation Plan

IALA has developed a World Wide Radio Navigation Plan (WWRNP) aimed at providing the WWRNS to support e-Navigation. One key concept in this plan is the separation of the generation of correction data from the means of transmission, to facilitate broadcasting by a variety of methods. This could lead to the integration of terrestrial systems (DGNSS beacons, e-Loran, AIS) to provide shared data channels and common correction sources, as well as additional ranging signals, contributing to a redundant position-fixing solution, complementary to, but independent of GNSS.

Future standards for position-fixing systems should be considered in the context of position-fixing requirements for e-Navigation. This WWRNP could be the basis for a submission to IMO as a contribution to the WWRNS. The plan provides guidance to IALA members regarding potential future developments, which will enable members to identify areas requiring resource allocation and research activity.

## Electronic Position Fixing Systems

The table below summarises the characteristics of the systems discussed in the following sections:

| System | Type | Coverage/date | Accuracy (95%) | Integrity |
| --- | --- | --- | --- | --- |
| GPS | Satellite | global/now | 9 m | No |
| GLONASS | Satellite | global/now | 12 m | No |
| Galileo | Satellite | global/2020 | tbc | Some services |
| Beidou | Satellite | regional/2015 global/2020 | 10 m | Some services |
| IRNSS | Satellite | regional/? global/? | tbc | tbc |
| QZSS | Satellite | regional/? | tbc | tbc |
| IALA DGNSS | GBAS | Local/now | 2 m | yes |
| WAAS | SBAS | Regional/now | 2 m | yes |
| EGNOS | SBAS | Regional/now | 2 m | yes |
| MSAS | SBAS | Regional/now | tbc | yes |
| GAGAN | SBAS | Regional/now | tbc | yes |
| SDCM | SBAS | Regional/now | tbc | yes |
| Loran-C | Terrestrial | Regional/now | 50-100 m | yes |
| eLoran | Terrestrial | Regional/now* | 8-20 m | yes |
| R mode | Terrestrial | Local/ in development | tbc | potentially |

*Limited areas of the UK at present, being deployed in Republic of Korea

## Global Navigation Satellite Systems (GNSS)

Global Navigation Satellite System (GNSS) is a generic term for a satellite system that provides a world-wide position determination, time and velocity capability, for multi-modal use.

GNSS is based on a constellation of active satellites, which continuously transmit coded signals in one or more frequency bands. These signals can be received by users anywhere on the earth’s surface to determine their position and velocity in real time, based on ranging measurements.

If a GNSS conforms to IMO Resolution A.1046(27) for a World-Wide Radionavigation System (WWRNS), the receivers of that GNSS will satisfy the IMO carriage requirements for position fixing equipment referred to in Chapter V of the SOLAS Convention.

GNSS receivers, in combination with other equipment, are able to provide PNT information such as:

- absolute positioning

- relative positioning (this can be further processed to derive speed over ground (SOG) , course over ground (COG), etc.)

- timing

This information may refer to a stationary observer (static positioning) or to a moving observer (kinematic positioning).

Several Global Navigation Satellite Systems (GNSS) have been deployed, fully or partially, or are under development.

Since 1996, the US “Navstar” Global Positioning System (GPS) and the Russian Global Navigation Satellite System (GLONASS) have been recognised as components of the WWRNS.  In the future, GNSS will include other systems such as the “GALILEO”, a system currently under construction by the European Union, and “BeiDou”, currently under construction by China. Furthermore, regional GNSS components like “QZSS” from Japan and IRNSS from India are planned to become operational in the next few years.

### GPS

The Global Positioning System, Standard Positioning Service (GPS SPS) is a three-dimensional positioning, three-dimensional velocity and time system that became fully operational in 1995.  The system is operated by the United States Air Force on behalf of the United States Government.

The U.S. Government provides two levels of GPS service. The Precise Positioning Service (PPS) provides full system accuracy to designated users. The Standard Positioning Service (SPS) provides accurate positioning to all users.

The GPS has three major segments: space, control, and user. The GPS Space Segment consists of a nominal constellation of 24 satellites in six orbital planes. The satellites operate in circular 20,200 km (10,900 nm) orbits at an inclination angle of 55 degrees and with an approximate 12-hour period.

The GPS SPS is available on a non-discriminatory basis and free of direct user fees to all users with an appropriate receiver.  The service satisfies the requirements for general navigation and harbour approach with a horizontal position accuracy of 9 metres (95% probability)2.

A modernization program aims to improve the accuracy and availability for all users and involves new ground stations, new satellites, and four additional navigation signals: three new civilian signals known as L2C, L5 and L1C and a new military code called M-Code. Full Operational Capability of the L2C code is not expected before 2016 and for L5 before 2020.

Further information on GPS may be found on the USCG NAVCEN website http://www.navcen.uscg.gov.  The website also has a link to the latest United States Federal Radionavigation Plan that provides a comprehensive account of current and future developments for GPS.

### GLONASS

The Global Navigation Satellite System (GLONASS) is a three-dimensional positioning, velocity and time system managed by the Russian Space Agency for the Russian Federation.

It is available on a non-discriminatory basis and free of direct user fees to all users with an appropriate receiver.  With a full complement of 24 satellites, the service satisfies the requirements for general navigation and gives a horizontal position accuracy in the region of 12.4m (95%) over any 24-hour interval, given a position dilution of precision (PDOP) of 2..

Recent launches have included the improved GLONASS M satellites with a second civil signal. Since 2011, the constellation is being replenished with GLONASS-K satellites that provide a third civil signal on L3.

GLONASS satellites use Frequency Division Multiple Access (FDMA), however new satellites will provide additional signals using code division multiple access (CDMA) to become interoperable with other GNSS.

Further information on GLONASS and future developments may be found on the Russian Space Agency, Information Analytical Centre website http://www.glonass-ianc.rsa.ru

### Galileo

Galileo is the European GNSS designed to be interoperable with other GNSS, managed and operated under civil control. It is currently under development by the European Space Agency with funding by European Union.  Galileo is aiming at providing an Open Service (OS), providing positioning and synchronisation information intended for high-volume satellite radio-navigation applications, a Commercial Service (CS) for added value professional or commercial applications, and a Public Regulated Service (PRS) restricted to government-authorised users requiring high-level of service continuity. Galileo will in addition provide a Search and Rescue Service (SAR), in support of the COSPAS-SARSAT.

Deployment started in October 2011 with the launch of the first two In Orbit Validation (IOV) satellites, with a further two due to be launched in October 2012. Target is to reach Initial Operational Capability (IOC) by 2014-2015 starting with a constellation of 18 first generation satellites, which will support the provision of continuous but limited service provision starting in the 2014-2016 timeframe. Complete Galileo Service Provision is due to start in the period 2018-2020 based on the Galileo Full Operational Capability (FOC) configuration and the fully deployed 30 satellites constellation. The first generation of Galileo will incorporate a number of features which were not part of previous GNSS systems. These include a signal in the E1 band with the advanced Multiplex Binary Offset Carrier modulation with improved performances, while ensuring both compatibility and interoperability with GPS and with other systems

At the time of writing, further information on Galileo may be found on the following internet page: http://ec.europa.eu/enterprise/policies/satnav/galileo/index_en.htm

### BeiDou

BeiDou (formally known as COMPASS) is a GNSS being developed by China to provide positioning, navigation and timing services to worldwide users. It can also provide wide area differential services with the accuracy of 1m and short messages services with the capacity of 120 Chinese characters each time. In 1994, China started the feasibility study of the BeiDou System. In 2000, two BeiDou experimental satellites were launched to begin a regional system. In 2004, China started to construct the full/final BeiDou Navigation Satellite System which is expected to be completed by 2020.

The BeiDou Navigation Satellite System space constellation will consist of five GEO satellites and 30 non-GEO satellites (27 in medium earth orbit and 3 in inclined geostationary orbit).There will be two levels of service provided; free service to civilians and licensed service to Chinese government and military users. The free service will have 10 meter position accuracy and can synchronize clocks with an accuracy of 10 ns, and measure speeds within 0.2 m/s.

At the time of writing, further information on BeiDou may be found on the following internet page: http://www.en.beidou.gov.cn/csnclist.html

### QZSS

Japan is developing a Quasi-Zenith Satellite System (QZSS).   QZSS is based on 3 satellites in highly elliptical, inclined orbits guaranteeing one satellite always in visibility of Japan with a minimum elevation angle of 60 degrees. Each satellite will transmit 6 signals in the L-band: 3 in L1, one in E6, one in L2 and one in L5.

One of the signals aims to provide sub-metre accuracy and integrity while maintaining compatibility with SBAS. The signal in E6 aims to support a commercial service with high data rate (2 kbps). The other signals are GPS-like signals, including L2C and L1C standards.

### IRNSS

The Indian Regional Navigational Satellite System (IRNSS) will be an independent navigation system covering the Indian region through a space segment of 3 GEO satellites and 4 IGSO satellites. The inclination of the orbital plane of the IGSO satellites is low, so that all the satellites can be seen simultaneously over India. It is planned to launch the first IRNSS satellite in 2013, with system completion by 2014.

Three IRNSS services are anticipated:

Open Service using signals in the L5 and S bands.

Precise Positioning Service using signals in the L5 and S bands.

Restricted Access Service using signals in the L5 band only.

The Open and Precise services target dual frequency users but it is also intended to compute and broadcast ionosphere-corrections to support single frequency users. Owing to the limited coverage of the IRNSS network of reference stations the satellites will, apart from the navigation payload, also include a dedicated C-band uplink/down-link ranging payload to support precise satellite orbit determination.

## Differential GNSS

The aim of GNSS augmentation services such as Differential GNSS (DGNSS) is the improvement of GNSS based positioning within a given area. In this context various methods can be applied to increase the accuracy of GNSS based positioning and to verify the integrity of applied components (systems, services, sensors) and provided data. An essential basis for DGNSS service provision are own GNSS measurements gathered in real time at single reference stations or a network of them.

In the simplest case the DGNSS service provides 3-dimensional positional correction parameters derived from differences between surveyed and known position at the reference station. This correction method is applied to satellite signals received at the user site which are common to those used at the reference station.

An increase on position accuracy can be expected, if the DGNSS service provides correction terms of ranging errors per satellite in view. This principle is applied by IALA Beacon DGNSS providing range and range rate corrections derived at reference station site from differences between surveyed and known distances to satellites in view. Satellite Based Augmentation Systems (SBAS) provides area correction parameter, whose application enables the determination of range and range correction for users in large-scale regions such as Europe. In this case a network of ranging and integrity monitoring stations (RIMS) is used to measure and model the spatial varying error behaviour.

Accuracies in the centimetre level can be achieved by DGNSS services employing the application of real time kinematic techniques (RTK) for positioning. The gain of accuracy is achieved by the common processing of range and phase measurements collected at reference station and user site to apply single and double difference methods during position determination.

In safety-critical application DGNSS services should be enriched with integrity functions realising the monitoring of data and system integrity in real time. The monitoring can be realised by plausibility and consistency tests as well as methods estimating error behaviour and budgets. Integrity data as results of the integrity monitoring informs the user about the current usability of applied components and provided output data.

The DGNSS service provision is realized by radio signals carrying augmentation, correction, and integrity data. Users operating in service areas and equipped with appropriate receivers can use these augmentation data

- to enhance accuracy of GNSS based positioning;

- to notify faulty satellites or GNSS failure;

- to detect satellite signals with increased propagation errors;

- to exclude disturbed signals from positioning;

- and to be informed about the usability of services or other information.

Currently, DGNSS services are provided for operational satellite navigation systems such as GPS and GLONASS. In principle, similar DGNSS services can be provided for future GNSS such as GALILEO, BeiDou and QZSS.

Each DGNSS service can be separated into a part generating and a part distributing the augmentation data. The generation of DGNSS augmentation data requires own GNSS measurements gathered at a single reference station or a network of them. Different DGNSS messages and services may use different generation methods and means of dissemination. At present certain communication channels used for the provision of DGNSS augmentation data are assigned to specific DGNSS services. For example, the provision of DGNSS augmentation data is realized by terrestrial radio transmitters (IALA Beacon Transmitter, AIS) or via satellite transponders (SBAS).

### 4.10.1	IALA Beacon DGNSS

Aim of IALA Beacon DGNSS is the provision of non-encrypted differential corrections as well as integrity information to maritime users to improve accuracy and integrity of GNSS based determination of position, velocity and time data (PVT). The method of differential positioning was developed in the nineties, is internationally accepted and supported in most coastal waters, especially in areas of high traffic density. The differential corrections are determined at known positions of reference stations or a network of them. For this purpose the difference between expected and measured ranges is used to derive actual range and range rate corrections. Additionally, integrity monitoring functionalities are implemented to assess the usability of GNSS signals and provided DGNSS service.

The radio link used for the provision of DGNSS correction and integrity data is internationally defined at ITU (Recommendation ITU-R M.823-3). At present the DGNSS signal transmission is realized in the maritime radionavigation band (283.5 to 325 kHz).  At user site type-approved DGNSS radio beacon receivers meeting IEC 61108-4 test and performance standards are necessary to enable the ship-side use of DGNSS services for an improved PVT data determination.

The recapitalisation of DGNSS infrastructure is an ongoing process.  Of the options available, some service providers have opted to replace existing hardware with similar dedicated Reference Stations and Integrity Monitors (RSIM); some have invested in software RSIM; while others have adopted a network of reference stations to create virtual RSIMs .  Other solutions, such as integration with SBAS may evolve within the enhancement of the maritime PNT system.

The full list of about 400 maritime radio beacon based DGNSS stations (as notified to IALA by authorities) can be accessed via the IALA website http://www.iala-aism.org.

### 4.10.2	AIS for DGNSS transmissions

Automatic Identification System (AIS) is a ship to ship and ship to shore data broadcast system, operating in the VHF maritime band, and is described in more detail in section 4.19.

AIS has the capability of providing DGNSS corrections to onboard equipment using standardized transmissions (Message No 17) as described in IALA Recommendation A-124. The use of AIS Message No 17 increases the number of vessels which benefit from DGNSS transmissions, with respect to better accuracy and integrity.

### 4.10.3	Maritime phase-based GBAS (MGBAS)

In the last decades the development of phase-based techniques was driven by surveying to achieve position accuracies with GNSS in the centimetre level. In IALA R-135 the RTK technique was mentioned as an approach to meet maritime requirements on high-precision positioning in port areas and for automatic docking. Several manufacturers of maritime GNSS/DGNSS equipment provide solutions supporting RTK based positioning.

It is noted that RTK is a short-range system and that there is a need to introduce monitoring and assessment of the integrity of RTK services and RTK based positioning in the context of safety-critical applications.

Precise point positioning (PPP) is a measurement method that performs precise position determination using a single GNSS receiver, with additional information.

The major error sources in conventional GNSS-based positioning are precise satellite orbit and clock errors. A world-wide network of continuously operating reference stations (CORS) can calculate the orbit and clock errors to a level sufficient to enable improved positioning.

A further component of the system is the communication channel used to provide (multiple frequency) receivers with this updated orbit and clock error information. Typically a satellite link or any kind of TCP/IP based communication (e.g. mobile internet) is used to provide this data. The user’s receiver then applies the new precise information within its position algorithms when calculating its position.

### 4.10.4	Precise Point Positioning

PPP is a phase based method but differs from real-time kinematic (RTK) positioning which requires access to observations from one, or a network of base stations, with known coordinates. The word “precise” is also used to distinguish it from the conventional point positioning technique that primarily uses code or phase-smoothed code for position determination.

The main advantage of this method is that it can provide global and absolute positioning, independent of local GNSS infrastructure; however it is reliant on available communications and may require a few minutes to initialise.

Assuming the communications link is available, PPP could be used as a reliable positioning method in case of disaster recovery where local infrastructure has been affected by outages or major geographical dislocation.

### 4.10.5	RTK over AIS

In survey applications the RTK correction information is usually distributed to users via VHF/UHF radio modems or via commercial broadband internet. However when used in hydrographical measurements further away from the shoreline these communication options might not be available all the time. The communication options in these areas would be via satellite or via AIS (the latter is also available only inside coastal VHF coverage usually less than 70km from shoreline).

RTK over AIS is in operational use for selected user groups in some countries and it has been reported to function without major problems and deliver the required positioning accuracy level. The correction data are broadcast to the mobile unit which initiates the transmissions.

When the mobile user requests RTK corrections, the AIS base station will start to reserve time slots (by FATDMA) for transmissions. For example, two 5 slot sequences every second can be reserved for both AIS channels. This will result in approximately 20% loading of the VDL. Using a lower message transmission frequency would not guarantee the expected RTK accuracy, especially on a moving platform.  However, a higher transmission rate would cause too much loading on the VDL. The correction delay of about 1-1.5 seconds caused by the transmission over the AIS data link has been reported to be acceptable. When not required, the mobile user stops the request for RTK corrections and the base station stops reserving the time slots and releases them for other use.

The limitations of this technique are that only one mobile user can be served by one AIS base-station at a time, reduced understanding of accuracy due to rapid atmospheric fluctuations and that it may not be applicable in areas of high VDL loading.  The channel loading problem may be addressed in the future by using the additional channels allocated for VDES.

### 4.10.6	SBAS

Satellite Based Augmentation Systems support wide-area or regional augmentation through the use of additional satellite-broadcast messages. Such systems are commonly composed of multiple ground stations, located at accurately-surveyed points. The ground stations take measurements of the GNSS satellite signals and environmental factors which may impact the signal received by the users. Using these measurements, information messages are created and sent to one or more GEO satellites for broadcast to the end users.

#### 4.10.6.1	WAAS

The Wide Area Augmentation System (WAAS) has been implemented by the US FAA to support the use of GPS for general and commercial aviation over continental United States. It was recently extended to cover parts of Mexico and Canada. At present, the WAAS architecture includes 38 reference stations, 3 master stations, 4 up-link stations, 2 geostationary satellite links and 2 operational control centres.

Further information on WAAS can be found on the USCG Navigation Centre website http://www.navcen.uscg.gov

#### 4.10.6.2	EGNOS

The European Geo-stationary Navigation Overlay Service (EGNOS) is a joint project of the European Space Agency (ESA), the European Commission (EC) and Eurocontrol.  It consists of three GEOs and a network of ground stations and transmits correction and integrity information for GPS and was designed for aviation safety-of-life applications. The system provides correction information in the L1 band through three GEO satellites. EGNOS is presently fully interoperable with the current generation of WAAS and MSAS. Like these systems, EGNOS is planning an evolution towards a dual-frequency capability using the L1/L5 bands.

Further information on EGNOS can be found via website: http://ec.europa.eu/enterprise/policies/satnav/egnos/index_en.htm

#### 4.10.6.3	MSAS

In Japan, the Multi-Satellite Augmentation System (MSAS) is an SBAS similar to EGNOS and WAAS. MSAS has been commissioned for aviation use, with two GEO-links using the L1 band via dedicated satellites shared with communications and meteorological missions.

The system has been operational since 2007 and there are plans to add additional services on L5 in the future.

#### 4.10.6.4	GAGAN

India is developing a GPS-Aided Geo Augmented Navigation system (GAGAN), which is an SBAS similar to WAAS and EGNOS. GAGAN includes 8 reference stations, 1 mission control centre, 1 up-link station and 1 Geo link through the L1/L5 transponder on the INMARSAT 4-F1 satellite.

At the time of writing, further information on GAGAN may be found on the following internet page: http://www.isro.org/

#### 4.10.6.5	SDCM

Russia is also developing an augmentation to provide corrections for GLONASS and GPS called SDCM (System for Differential Corrections and Monitoring).  This system will consist of 3 geo-stationary satellites, assigned PRN codes 125,140 and 141.  Two satellites have been launched and are in operation with the third due to be launched in 2014.

## Receiver Autonomous Integrity Monitoring (RAIM)

RAIM is a technology developed to assess the integrity of GNSS signals and therefore the integrity of GNSS based positioning. This kind of integrity monitoring is autonomously realized within the user’s receiver with special importance for safety-critical applications, such as aviation and maritime.

Range measurements are required from at least 4 GNSS satellites to enable the determination of position, velocity and time data. However the application of RAIM in a navigation receiver requires redundancy in the range measurements. Safety-critical RAIM algorithm might use only “Fault Detection” (FD) or “Fault Detection and Exclusion” (FDE), which enables the continuation of operation in the presence of a single GNSS satellite and signal failures.  To detect a faulty satellite, at least five range measurements are required, whereas to isolate and exclude a faulty satellite, at least six range measurements are required. While RAIM can detect many failure modes, it cannot detect some failures affecting multiple satellites.

The upcoming availability of various GNSS will increase the usable number of navigation signals for RAIM based positioning. New and modernized GNSS supports the provision of GNSS signals in 2 or more frequency bands and improves therefore the capability of GNSS based ranging.

Future advancement in RAIM algorithms should improve the availability and continuity of RAIM based positioning.  Such enhanced RAIM techniques – so called Advanced RAIM (ARAIM) – may become available to maritime users (see http://www.navipedia.net/index.php/ARAIM).

## Terrestrial Systems

### Loran-C

Loran–C was a hyperbolic radionavigation system developed during the 1960s to meet U.S. Department of Defense requirements. The Russian Federation operates a similar radionavigation system called CHAYKA. There are currently about 19 Loran–C and CHAYKA chains operating around the world. The principal coverage areas include Saudi Arabia, China Sea, Korea, North West Pacific, Russian Federation and North West Europe.

Loran–C chains comprise between three to five stations that have a spacing of 600 to 1000 nautical miles. The signal format is a structured sequence of specially designed radio pulses on a carrier wave frequency centred on 100kHz. One of the stations is designated as the ‘master’ and transmits groups of 9 pulses. The other stations are called ‘secondaries’ and these transmit groups of 8 pulses.

The spacing between groups of ‘master’ pulses from a single chain is a characteristic unique to that chain and is referred to as the Group Repetition Interval (GRI).

The 100kHz carrier wave frequency favours the propagation of a stable ground wave over long distances. Careful signal design allows Loran receivers to determine positions using the ground wave and reject the delayed sky wave that would potentially distort the received signal.

The transmissions from each chain are monitored and controlled continuously. System abnormality indicators are built into the signal format and can be identified by the receiver providing inherent integrity warnings.

### eLoran

Enhanced Loran (eLoran) is a terrestrial navigation system developed from Loran-C.  It is a Positioning, Velocity, and Timing (PVT) service for use by land, sea and air navigation as well as other applications reliant on timing data.

eLoran is independent to and has dissimilar failure modes to GNSS and therefore complements GNSS use. Although offering reduced accuracy, it will allow GNSS users to retain the safety, security, and economic benefits of GNSS, even when their satellite services are disrupted.

eLoran provides positional accuracy in the region of 8 - 20 metres and time and frequency performance (to stratum-1 level) similar to current GNSS.

eLoran differs from Loran-C as it uses an all-in-view method of operation, calculating the distance to all eLoran stations in view. eLoran stations are also synchronised with, but independently of, GNSS time. Synchronising to a common time source allows receivers to employ a mixture of eLoran and GNSS signals.

eLoran receivers calculate the distance from each station by firstly assuming that the entire earth’s surface is covered in sea-water. By knowing the speed of the signal over sea-water along with the times of transmission and reception a pseudorange can be calculated. This pseudorange is then adjusted to take into account the propagation delays due to the signal passing over land, these delays are called Additional Secondary Factor delays (ASFs).   ASFs are measured by the service provider and are supplied to users as a database built into their receivers.   ASFs may change slightly due to weather or seasonal effects, reducing the efficiency of the correction and affecting accuracy.  However, this is resolved by installing a differential-Loran reference station nearby, which is able to measure the difference and calculate a correction.  The correction information is then passed back to the eLoran station for dissemination to the user over the eLoran data channel.

The inclusion of a data channel as part of the main transmission is one of the inherent features of eLoran and can be used to provide other data services in addition to differential corrections.

For more information the reader is encouraged to seek the advice of Radio technical Commission for Maritime Services Special Committee 127 (SC-127) on eLoran Systems.

### Compatibility between eLoran and Loran-C

Legacy receivers are able to use both eLoran and Loran-C signals as eLoran stations form part of the presently organised chains. However, legacy Loran-C receivers will likely not provide the user with the best accuracy performance.

Users should ensure their receivers are able to decode the Loran Data Channel to receive integrity alerts, UTC time and differential-Loran correction data. They should also ensure their receiver is capable of storing and applying up to date ASF data.

### R-mode

Investigations are being conducted on the benefit of expanding the functionality of existing systems; by providing a timing signal from which the user may then calculate their position independently from GNSS and this is known as Ranging-mode (R-mode).

At present the IALA MF beacon system and AIS Services are being considered as candidates for modification to add R-mode functionality.  By providing timing information over their normal MF or VHF transmissions, a shipboard receiver may then calculate a distance (range) to the transmitter.  By calculating the range to several stations, the user is able to calculate the ship’s position.  Coverage, geometry and interference questions would need to be investigated.

The provision of R-Mode services would require the availability of an accurate non-GNSS timing source at the transmitter.  High stability clocks could be an expensive option and it is more likely that time would be sourced from a low frequency radio time clock or eLoran.

## Radar Aids to Navigation

Radar aids to navigation are devices that provide returns to a ship’s radar that help to locate and/or identify a navigation mark.

The IMO carriage requirements contained in (Chapter V, Regulation 19) of the SOLAS Convention 1974 (as amended), states:

- all ships of 300 gross tonnage and upwards to carry a 9 GHz radar, and;

- all ships of 3,000 gross tonnage and upwards to be fitted with a 3 GHz radar or, where considered appropriate by the Administration, a second 9 GHz radar.

Some administrations may impose other carriage requirements.

IMO Resolution MSC.192(79) Adoption of the Revised Performance Standards for Radar Equipment 06 December 2004 states that 9 GHz radars should be capable of detecting radar beacons and should be capable of detecting SARTs and radar target enhancers.  By omission, 3GHz radars are not required to detect radar beacons and SARTS. With the removal of the 3GHz radar racon detection requirement, ship-owners are free to use higher performing radars, often referred to as New Technology (NT) radars.

9 GHz radars are also extensively carried by vessels not covered by SOLAS or local regulation. Because of this high rate of carriage, radar aids to navigation in the 9 GHz band are especially useful.

### Radar Reflectors

A radar reflector is a passive device designed to return the incident radar pulses of electromagnetic energy back towards the source and thereby enhance the response on the radar display. By design, a radar reflector attempts to minimise the absorption and random scattering effects.

A radar reflector is generally installed as a supplementary device at sites that would also be marked with a light. The main objectives of its use are to enhance:

- target detection at long ranges (for example, for landfall navigation);

- target detection in areas of sea or rain clutter; and

- radar conspicuity of aids to navigation to reduce the risk of collision damage.

The performance of a radar reflector can be defined in terms of its effective radar cross section (RCS). This is a value determined by comparing the strength of radar signals returned by the radar reflector with the equivalent return from a radar reflective sphere of 1square metre reflecting area.

The range at which a radar reflector target can be detected is dependent on the heights of the radar antenna and the reflector and the output power of the radar. There are analogies to the geographical range of visual marks. The radar performance of corner cluster reflectors may vary considerably from one make to another.

Use of small radar reflectors can also be subject to multipath fading effects. Please see IALA Guideline No. 1010 on racon range performance for a discussion on multipath fading.

Most radar reflectors are designed for use by 9 GHz radars. The reflectors are also usable with 3 GHz radars; however, the effective radar cross section is about an order of magnitude less.

### Radar Target Enhancers

A Radar Target Enhancer (RTE) is a device that amplifies and returns the pulse from a ship’s radar to give an enhanced image on the radar screen. The returned signal from an RTE is not coded. The RTE was designed primarily for buoys and small vessels that might normally carry a passive radar reflector. RTE testing has shown RTEs to have provided an effective radar cross section (RCS) of about 100 square metres, compared with an RCS of 20 to 30 square metres for passive radar reflectors typically fitted to buoys.

There are commercially available RTEs operating in the both the 3 GHz and 9 GHz bands.

RTE use is subject to multipath fading effects. Please refer to IALA Guideline No. 1010 on racon range performance for a discussion on multipath fading.

### Radar Beacon (Racon)

Racons are receiver/transmitter devices operating in the maritime radar frequency bands (9 and 3 GHz) that enhance the detection and identification of certain radar targets.  Note IMO MSC.192(79) removed the requirement for 3GHz radar to detect racons.

A racon responds to the presence of a ship’s radar by sending a characteristic pulse train. The response appears as a coded mark (or “paint”) on the ship’s radar display (refer Figure 18) that highlights the range and bearing of the racon. The display paint can be fixed to a specified length or can be dependent on the radar range setting and uses a Morse character for identification.

Figure 18:  Example of a racon and a radar display with and without the racon character

Applications

A racon is generally considered to be a supplementary aid to navigation installed at sites that would also be marked with a light. The number of vessels capable of making use of a racon is effectively unlimited.

A racon can be used for:

- ranging and identification of positions in ice conditions or on inconspicuous coastlines;

- identification of aids to navigation, both seaborne and land based;

- landfall identification;

- indicating centre and turning point in precautionary areas or Traffic Separation Scheme (TSS)

- marking hazards;

- indicating navigable spans under bridges; and

- identifying leading lines.

### Frequency-Agile Racon

A frequency-agile racon responds on the frequency on which it is interrogated and the response can be re-painted on each radar sweep. The purpose of frequency agility is to provide a signal to the radar that is within the receiver bandwidth of the radar.

However, to avoid masking other features on the radar screen, the racon response is usually switched on and off on a preset cycle.

Signal Characteristics

Racons operate in the 9 GHz band with horizontal polarisation, and/or in the 3 GHz band with horizontal and optionally vertical polarisation.

Table 18 – Preferred terminology for the description of racon operating frequencies

| Preferred Terminology | Alternatives |  |  |
| --- | --- | --- | --- |
| 9 GHz | 9300 -9500 MHz | X - band | 3 cm |
| 3 GHz | 2900 -3100 MHz | S - band | 10 cm |

### Racon Performance Criteria

The availability of a racon is the principal measure of performance determined by IALA. In the absence of any specific considerations, IALA recommends that the availability of a racon should be at least 99.6%

Refer to IALA publications:

IALA Guideline 1010 on racon range performance; (note this paper assumes radar transmission powers associated with magnetron radars and the suggested racon detection ranges may be reduced when operating with New Technology radars incorporating solid state transmission and pulse compression)

IALA Recommendation R-101 on maritime radar beacons (Racons)

IALA Recommendation O-113 for the marking of fixed bridges over navigable

waters.

### Racon Technical Considerations

There are a number of technical considerations in the use of racons to assist the navigation of a ship:

- The angular accuracy of the bearing between the ship and racon depends entirely on the interrogating radar, while the accuracy of the range measurement depends on both the radar and racon;

- When racons are used in leading line applications, an alignment accuracy of about 0.3 degrees can be expected;

- When the ship is very close to the racon, side-lobes from the radar antenna can trigger the racon. The resulting multiple responses on the radar display may be a distraction and can mask other targets. Side-lobe suppression techniques are standard features of frequency agile racons.

### Use of Racons with New Technology Radars

All currently available and installed racons are designed for use with high power pulse radars. In comparison, NT radars use low power transmissions with long pulses. Because of the low received peak signal strength and long pulse at the racon, current racons may not detect NT radars and may not transmit a response usable by NT radars. Studies have shown that the range at which pulsed NT radars are able to reliably trigger Racons is shorter than would be achieved with a conventional magnetron pulsed radar. Note that FMCW radars (also grouped into the family of New Technology radars) are a special case, requiring individual analysis and measurement. The IMO regulations regarding X band radars and Racons remain unchanged and although detection and triggering range might be reduced, it remains the responsibility of manufacturers of X band NT radars to retain Racon functionality.

Despite changes to the IMO regulations relating to S band Racons, existing Racons with 3 GHz capability will continue to be useful to 3GHz pulse radars of both Magnetron and pulsed New Technology variants although advanced clutter reduction techniques optimised in the knowledge that display of Racon signals is no longer a functional requirement in this frequency band, may attenuate or even remove the Racon pulse train from the radar video and display. At some point there may be a requirement to replace all high power pulse radars, at which time present racons with 3 GHz pulse capability may need replacement.

### Radar Referenced Positioning

Algorithms may be developed to allow the radar display to be overlaid upon the electronic chart using detectable recognised navigational features (Racons, passive radar beacons or land edge patterns etc.). This technique, although unlikely to challenge the accuracy of a GNSS based position fix, might be adopted as part of a PNT integrity assessment and / or as a back up in the event of GNSS service or equipment failure or corruption.

### Non-radio positioning (Inertial)

Many studies have been carried out on the integration of GNSS with Inertial Measurement Units (IMU) for marine navigation. There exist various grades of IMU, covering from the very expensive navigation grade through tactical grade to low cost units based on MEMS (Micro Electro Mechanics System) technology.  The IMU grade characterizes the achievable performance of data provision covering velocities and orientations. A small IMU grade is associated with higher drift rates. Depending on the different drift rates an IMU can provide contingency functionality for various lengths of GNSS outages.

Additionally, a GNSS – IMU sensor fusion scheme enables an improved integrity monitoring for various output data like position, speed and rate of turn. In combination with a GNSS compass an IMU can provide accurate and stable heading data even for longer GNSS outages. None of the currently available inertial systems is capable of maintaining all levels of navigation accuracy during a lengthy outage of GNSS. For ocean areas, both navigation and tactical-grade IMUs will give protection for appreciable outages over 15 minutes and navigation grade IMUs up to over 1 hour. For the coastal areas the required accuracy of 10 meter could be obtained for 3.5 minutes with a navigation-grade IMU and 1.5 minutes with a tactical grade IMU.

The idea of an IMU on a chip with unit cost as low as a GPS module is anticipated in the near future. Such low cost MEMS based IMUs are a useful component of ship’s navigation system to fulfill the contingency requirement in cases of short-term GNSS outage.  Nearly all MEMS accelerometers work by measuring the motion of a proof mass against a fixed frame. The main sensing approaches are capacitive or piezoresistive. The power requirement of such sensors is in the range 1-10 mW. The drift of a MEMS based inertial system depends mostly on the gyro error behavior. MEMS based gyroscopes with tactical grade performance (0.1-100deg/h) are available with accuracies in the region of 1 deg/h and are expected to reach close to 0.1 deg/h high volume production in less than 5 years.  Increasingly MEMS based INS IMUs could make a useful contribution by providing short term contingency and by enabling improved integrity monitoring.

### 4.13.10		Non-radio positioning (ePelorus)

An electronic pelorus is a device for taking bearings of visual marks and converting them to an electronic format for input to an electronic chart system. Such a device would enable the integration of visual AtoNs with e-Navigation. The concept was promoted in a paper by the Nautical Institute in about 2008, as a backup system for navigation.

Only a costly military version appears to exist at present. However, the feasibility of constructing a low-cost ePelorus from off-the-shelf components is being investigated, to demonstrate its effectiveness as a backup and to evaluate the potential for integrating visual AtoNs with e-Navigation.

Communications

## Maritime Radio Communications Plan

IALA has prepared a Maritime Radio Communications Plan (MRCP) for the communications required to support e-Navigation. The MRCP is intended to meet the key strategy element of identifying communications technology and information systems to meet user needs. This may involve the enhancement of existing systems or the development of new systems. The IALA work starts by identifying existing and future systems, then drawing on the user requirements already identified to assess the information flows and the data channels needed.

### Long Range Identification and Tracking

- Introduction

Competent authorities with responsibility for aids to navigation, port security and other shoreside activities are often faced with the requirement to maintain surveillance of maritime approaches to their ports and port facilities for safety, security, and environmental protection.

These authorities are pursuing vessel tracking technologies to assist in the detection, classification, identification, and tracking of vessels. Among these technologies, Long Range Identification and Tracking (LRIT) has been implemented internationally for tracking ships globally.

- LRIT Concept

Long range identification and tracking (LRIT) is a cooperative surveillance capability. In the simplified LRIT concept (Figure 19), a ship carries radio communications equipment that reports identification, position and time to authorities tracking that ship.  However, the final implementation of LRIT is more complicated as explained below.

Figure 19:  Simplified LRIT Concept (courtesy of INMARSAT)

- LRIT Performance Standards and Functional Requirements

The approved performance standards and functional requirements for long-range identification and tracking lay out the LRIT system architecture (Figure 20) and describe how the long-range identification and tracking system works.

Figure 20 - LRIT system architecture

In this architecture, the Administration determines whether its ships will report to a national, regional/cooperative, or the international LRIT data centre. Each of these types of centres may use multiple communications service providers. The architecture is also designed to accommodate multiple application service providers.  Building on the basic concept noted above, a ship carries radio communications equipment that reports identification, position and time to the national, regional/cooperative, or international LRIT data centre tracking that ship. The Administration of the ship can access the LRIT information directly from the data centre.  Other Contracting Governments that are entitled to that information (i.e., port and coastal states) can request the information through their data centre and thence through the international LRIT Data Exchange.  The LRIT information is routed to the requesting data centre through the data exchange.

## Automatic Identification System

### Overview

Automatic Identification System (AIS) is a ship and shore-based data broadcast and interrogation technology, operating in the VHF maritime band, making it possible to monitor and track ships from suitably equipped ships and shore stations.

AIS’ characteristics and capability make it a powerful tool for enhancing situational awareness, thereby contributing to the safety of navigation and efficiency of shipping traffic management. Shipboard AIS enables the provision of fast, automatic and accurate information regarding risk of collision allowing the Closest Point of Approach (CPA) & Time to Closest Point of Approach (TCPA) to be calculated from the positional information transmitted by target vessels.  AIS increases the possibility of detecting other ships, even if they are behind a bend in a channel or river or behind an island in an archipelago.  AIS also solves the problem inherent with radars, by detecting smaller craft, fitted with AIS, in sea and rain clutter.

An AIS unit is a VHF radio transceiver capable of exchanging information such as station identity, position, course over ground, speed, length, ship type and cargo information etc., with other ships and suitable receivers ashore within VHF range.  Figure 21 gives an overview of the system.

Once set up correctly, information from an operational shipboard AIS unit is transmitted continuously and automatically, without any intervention by the ship’s staff. AIS transmissions consist of bursts of digital data ‘packets’ from individual stations, according to a pre-determined time sequence.

Therefore, AIS is an important supplement to existing systems, including radar.  In general, data received via AIS will enhance the information available to the Officer of the Watch and the Vessel Traffic Service Operator (VTSO).

The International Maritime Organization (IMO) has established carriage requirements for merchant ships. The International Telecommunication Union (ITU) has defined the technical characteristics and ratified the global frequencies. In addition, the International Electrotechnical Commission (IEC) has developed methods for testing AIS for global interoperability.

Note:  This section aims to provide a broad overview of AIS.  The reference list at the end of this section assists the reader seeking amplifying information on various aspects of AIS.

### Purpose & Function

The purpose of AIS is to positively identify vessels, provide additional information in order to assist in collision avoidance and assist in vessel tracking.  It also aims to simplify and promote the exchange of information automatically, thereby reducing the need for doing so verbally (e.g. mandatory ship reporting by radiotelephony).

AIS satisfies the following functional requirements, as laid down by IMO.

- in a ship-to-ship mode for collision avoidance;

- as a means for littoral States to obtain information about a ship and its cargo; and

- as a VTS tool, i.e. ship-to-shore (traffic management).

AIS automatically exchanges shipboard information (provided by shipboard sensors), between vessels and between a vessel and a shore station(s).

### System Characteristics

Frequencies and capacity

AIS operates on two dedicated VHF FM radio frequencies AIS1 (channel 87B – 161.975 MHz) and AIS2 (channel 88B– 162.025 MHz) in the maritime mobile band.

Transmissions consist of bursts of ‘data packets’ from individual stations, according to an automatically determined time-ordered sequence.  Stations organize themselves on the common frequencies (AIS 1 and AIS 2) based on the knowledge of their own transmissions and that of other stations.  This method of operation is known as Self Organizing Time Division Multiple Access (SOTDMA).  The time slots for AIS transmissions are all precisely aligned to Coordinated Universal Time (UTC), provided for by a Global Navigation Satellite System (GNSS) receiver.  This avoids the possibility of two stations transmitting at the same time, in the same slot.

There are 2250 time slots available on each frequency per minute, making the total number of slots equal to 4500.

Figure 21 – Overview of the AIS System

### Shipboard AIS

A shipboard AIS unit transmits its own data to other vessels and to AIS equipped stations continuously and autonomously.  It also receives AIS data of other stations (ship and shore) and can display this data textually and graphically, as required.

Each AIS station consists of a VHF transmitter, two VHF SOTDMA receivers, a VHF DSC receiver, a GNSS receiver (to provide timing for slot synchronisation), and a marine electronic communications link to shipboard display and sensor systems.

Positional information can be derived form the internal GNSS or an external electronic position fixing system.

The display panel with the unit is often the only means of showing received AIS data. Together with a keypad, this basic configuration is known as a Minimum Keyboard and Display (MKD).

The display part of a MKD, as a minimum, consists of three lines of data, each showing bearing, range and identity of the target.  In practice, most MKDs display more lines of data and may also have a simple graphical display, showing the relative location of targets, rather like the Plan Position Indicator of a radar.

Ideally, AIS information should to be displayed graphically on a radar, ECDIS or on its own dedicated display.

### Information available

The AIS information transmitted by a ship station includes four different data sets:

- fixed or static information is entered into the AIS unit on installation and need only be changed if the ship changes its name, call sign etc.  This information is broadcast every six minutes or on request by a shore authority.

- voyage related information (destination, ETA etc) is manually entered and updated during the voyage.  This information is also broadcast every six minutes.   In order that correct AIS information is broadcast to other vessels and shore authorities, mariners are reminded to enter current voyage related data such as draught, type of hazardous cargo, destination and ETA properly at the beginning of each voyage and whenever changes occur.

- dynamic information is automatically updated from the ship sensors connected to the AIS.  This includes COG, SOG, position (with accuracy and integrity flag), time and navigation status (e.g., underway).

- Broadcast or addressed short safety related messages, as required

Refer to IALA publications:

IALA Guideline 1028 on the Universal Automatic Identification (AIS) – Volume 1, Part 1 – Operational Issues;

IALA Guideline 1029 on the Universal Automatic Identification System (AIS) – Volume 1, Part 2 – Technical Issues.

IALA Guideline 1082 on an Overview of AIS

### Shore based AIS

SOLAS Chapter V, Regulation 19, 2.4 refers to the carriage requirements for AIS.  The regulation states that AIS shall provide and receive information from appropriately equipped shore stations. The provision of shore based AIS will be necessary to attain the full benefit of the 1974 SOLAS Convention (as amended).

Competent authorities should therefore consider the provision of an AIS shore infrastructure so that the full benefit of the system can be realised in terms of navigation safety and protection of the environment.

As AIS can be seen as a tool related to Vessel Traffic Services (VTS), Competent Authorities should consider implementing AIS into existing VTS Centres. Information on the use of AIS in VTS operations is contained in Sections 1015-1027 of the IALA VTS Manual

Refer to IALA publications:

IALA Recommendation A-123 on the provision of shore based automatic identification systems

IALA Recommendation A-124 on the Automatic Identification System (AIS) Shore Station and Networking Aspects relating to the AIS Service.

### Meteorological & Hydrological Information

IMO is responsible for the AIS Binary Broadcast Messages (AIS Message 8) and a message structure has been defined for meteorological and hydrological information.

A number of countries operate tide gauges and current meters to assist the prediction of tidal heights and streams or for the broadcast of real-time information to shipping.  The latter is generally used to overcome the sometimes considerable differences between actual tide heights and predicted values due to meteorological and mean sea level fluctuations. Providing real-time information of this type, for example dynamic under-keel clearance, wave heights or sea state can be seen as applications of e-Navigation, requiring integration between shore-side and ship-borne systems.

### AIS as an aid to navigation

A special type of AIS station fitted to an aid to navigation provides positive identification of the aid without the need for a special ship-borne display.  In addition, AIS as an AtoN can provide information and data that will:

- complement or replace an existing aid to navigation, providing identity, state of ‘health’ and other information such as real time tidal height and local weather to surrounding ships or back to a shore authority;

- provide the position of floating aids (mainly buoys) by transmitting an accurate position (corrected by DGNSS) to monitor if they are on station;

- provide real-time information for performance monitoring, with the connecting data link serving to remotely control changes in AtoN parameters or switching on back-up equipment;

- provide local hydrological (hydrographical) and meteorological information;

- possibly replace radar beacons (racons) in the future, providing longer range detection and identification in all weather conditions; and

- gather shipping traffic data on AIS fitted ships for future aid to navigation planning purposes.

- For practical or economic reasons it may not be appropriate to fit an AIS to an AtoN.  In this case the AIS AtoN may be broadcast by another station, as if it comes from the AtoN. This was formerly known as a synthetic AIS AtoN. The notion of “Synthetic” AIS AtoN is mainly of relevance for the service provider.

- There are two types of synthetic AIS AtoN, ‘Monitored’ and ‘Predicted’. A ‘Monitored Synthetic AIS AtoN’ is transmitted from an AIS Station that is located remotely. The AtoN physically exists and there is a communication link between the AIS Station and the AtoN. The communication between the AtoN and AIS shall confirm the location and status of the AtoN.

- A ‘Predicted Synthetic AIS AtoN’ is transmitted from an AIS Station that is located remotely.  The AtoN physically exists but the AtoN is not monitored to confirm its location or status.  It should not be used for floating aids to navigation.

### Virtual AtoNs

- A Virtual AIS AtoN is transmitted from an AIS station to establish an aid to navigation that can be seen on electronic displays onboard, but does not physically exist. A nearby base station or AtoN station can broadcast this message, allowing it to be deployed rapidly in case of a new hazard, loss of or damage to a physical AtoN.. The AIS message will clearly identify this as a Virtual AIS AtoN.

- The Virtual AtoN will not be visible on the displays of many ships and, if visible, the symbols may differ from one display to another. The consequences may be confusion, lack of information for the user and the undermining confidence in ECDIS, the chart and other systems. It is likely to take at least a decade to harmonise the provision of Virtual AtoN as a result of the’grand-fathering‘ clauses in the carriage requirement program for ECDIS and the likely schedule for the adoption of INS & e-Navigation.

- Depending on the medium, a Virtual AtoN service can be spoofed and jammed easily. Jamming can be unintentional or intentional while spoofing would typically be intentional.

- Refer to IAlA publications:

Refer to IALA publications:

IALA Recommendation A-126 on the use of the automatic identification system (AIS) in marine aids to navigation.

IALA Recommendation O-143 on Virtual Aids to Navigation

IALA Guideline 1050 on the Management and Monitoring of AIS Information.

IALA Guideline 1081 on Virtual Aids to Navigation

### Carriage Requirements

There are two ‘types’ of AIS units for ship stations.  These are termed Class A and Class B units.

Class A ship-borne mobile units must comply with ITU-R M. 1371, and are required to be carried on board those vessels to which Regulation 19 of SOLAS Chapter V applies and meet the IMO performance standard.  All these ships were to have AIS fitted by 31 December 2004.

Class B ship-borne mobile equipment, while also complying with ITU-R M.1371, is designed for vessels such as pleasure craft and fishing vessels.  These units provide  less functionality than Class A units, and do not necessarily meet all the IMO performance requirements.  They are designed to operate co-operatively with Class A units.

Administrations can require the carriage of Class B units as part of their domestic regulations.

### Cautions when using AIS

The Officer of the Watch (OOW) should always be aware that other ships, in particular pleasure craft, fishing vessels, warships and some shore stations including VTS centres, may not be fitted with AIS.

The OOW should always be aware that AIS fitted on other ships as a mandatory carriage requirement, might, under certain circumstances, be switched off, particularly where international agreements, rules or standards provide for the protection of navigational information. AIS can also provide incorrect information if the input data is wrong.

Navigators should be aware of the limitations of AIS.  In particular, government agencies and owners should ensure that watch-keeping officers are trained in the use of AIS. Because of these limitations navigators are advised that AIS should not be used as the primary means of collision avoidance.

### Strategic Applications of AIS

From a number of maritime perspectives (such as VTS and regulatory compliance), the availability of comprehensive ship information, offers a mechanism for:

- better monitoring of compliance with national and international regulations for mandatory routeing and reporting systems, Particularly Sensitive Sea Areas, discharging of oil, garbage disposal etc;

- maritime logistics applications such as fleet management, cargo tracking and port facilities (movement of pilot boats, tugs etc);

- better control, co-ordination and response in the event of marine incidents, such as SAR and pollution;

- shore -based navigational assistance;

- shipping information gathered from AIS can be channelled into a central repository of a local, national or regional network serving maritime administrations, port authorities, shipping agents, freight handlers, customs, immigration, etc.

Further information on AIS can be found within IMO, IALA, ITU and IEC documentation

Refer to IALA Publications:

IALA Guideline 1026 on AIS as a VTS Tool

IALA Guideline 1028 - Volume 1, Part I – Operational Issues,

IALA Guideline 1029 - Volume 1 Part II – Technical Issues, Edition 1.1

IALA Technical Clarifications on ITU Recommendation ITU-R M.1371-1 Edition 1.5

Refer to IMO Publications:

Recommendation on Performance Standards for an Universal Shipborne Automatic Identification System (AIS) (MSC 74(69) Annex 3).

Guidelines for the onboard operational use of shipborne Automatic Identification Systems (AIS) (Resolution A.917 (22), as amended by Resolution A.956 (23)).

Performance Standards for the presentation of navigation-related information on shipborne navigational displays (Resolution MSC. 191(79).

SN/Circ. 227 Guidelines for the installation of a shipborne Automatic Identification System (amended by SN/Circ 245).

SN/Circ. 236 Guidance on the application of AIS Binary Messages

SN/Circ. 243 Guidelines for the presentation of navigation-related symbols, terms and abbreviations  	SN/Circ. 244 Guidance on the use of UN/LOCODE in the destination field of AIS messages.

Refer to ITU Publications:

ITU-R M.1371-1Recommendation on the Technical Characteristics for a Ship-borne Universal Automatic Identification System (AIS) Using Time Division Multiple Access in the Maritime Mobile Band.

Radio Regulations, Appendix S18, Table of Transmitting Frequencies in the VHF Maritime Mobile Band.

ITU-R M.823-2 Recommendation on the technical characteristics of differential transmissions for global navigation satellite systems from maritime radio beacons in the frequency band 283.5-315 kHz in region 1 and 285-325 kHz in regions 2 and 3.

Refer to IEC Standards:

61993 Part 2:  Class A Ship-borne equipment of the Automatic Identification System (AIS) - Operational and Performance requirements, methods of testing and required test results.

61108-1 (2nd edition): navigation and radiocommunication equipment and systems – Global navigation satellite systems (GNSS).

61162-1 (2nd edition) Maritime navigation and radiocommunication equipment and systems – Digital interfaces – Part 1: Single talker and multiple listeners.

62320-1: Maritime Navigation and Radiocommunication equipment and systems – Automatic Identification System. AIS base stations - Minimum operational and performance requirements - methods of test and required test results

62320-2 Maritime Navigation and Radiocommunication equipment and systems – Automatic Identification System. AIS aids to navigation - Minimum operational and performance requirements - methods of test and required test result

62287-2 (Part A and B) Class B AIS (Part A – CSTDMA; Part B – SOTDMA)

61097-14 (pending) Global Maritime Distress and Safety System (GMDSS).  AIS search and rescue transmitter (AIS-SART) – Operational and performance requirements: methods of testing and required test results.

## Electronic Chart Display and Information System (ECDIS)

Although ECDIS, as ship borne equipment, is not an “aid to navigation” as defined by IALA, it deserves to be mentioned because it brings major changes to the manner in which vessels are navigated.  ECDIS uses digital vector chart data in a way that replaces the traditional paper charts with a more versatile electronic product that can draw on a variety of positioning and data inputs, such as GNSS, DGNSS, AIS, radar, echo sounder, compass, an electronic chart, navigational publications, the chart amendments and tidal and meteorological information to provide a real-time aid to decision making.

Performance Standards

The performance standards for ECDIS have been defined by the International Maritime Organization (IMO), in conjunction with the International Hydrographic Organization (IHO).

IMO Resolution A.817(19) as amended by Resolution MSC.64(67) and by Resolution MSC.86(70) enables maritime administrations to accept ECDIS as a legal alternative to navigation using paper charts and to meet the chart carriage requirements of SOLAS Chapter V/19.

Performance Elements

There are two key performance elements to ECDIS:

•	An approved processing system (or ‘box’) that has been certified as meeting the IEC 61174 and other relevant performance testing specifications, and;

•	Electronic Navigational Charts (ENCs) that have been issued by or on the authority of a government, hydrographic office or other relevant authority and meet the standards set down in the 3rd Edition of the IHO Publication 57 (S-57) and other related IHO standards governing electronic charts.

Raster Navigation Charts (RNC) that are effectively electronic copies of paper charts and have been issued by or on the authority of a government, hydrographic office or other relevant authority, may be used in an ECDIS to meet carriage requirements, but only in those cases where no ENC has been published covering the area in question.

While an ECDIS ‘box’ may be capable of reading other forms of electronic charts, it ceases to be a compliant system without the official ENCs.

Electronic charts that will not satisfy the SOLAS carriage requirements include:

•	All electronic charts that are not issued under the authority of a national authority.

•		All charts that do not conform to the relevant IHO standards for electronic charts.

Additional information on ECDIS is available on the IMO and IHO websites.

## Maritime Information

The timely provision and display of maritime information will be an essential component of e-Navigation. Generically called Marine Information Overlays (MIOs), this includes both static and dynamic information capable of being used ashore (e.g., at a VTS Centre) and onboard ships at sea.  Static information could pertain to marine protected areas, sea ice coverage, emergency management/response areas, and seafloor bathymetry.  Dynamic operational information would be broadcast via AIS binary messages as time-critical information regarding ship/voyage data, marine traffic signals, area notices, dangerous cargo, environmental, meteorological, hydrographic, and status of AtoN.  In particular, mariners require this type information pertaining to the planning and execution of voyages, the assessment of navigation risk and compliance with regulation. The provision and use of MIOs will depend on the current situation and task-at-hand.

At the 54th session of the IMO Safety of Navigation Subcommittee (July 2008) it was recommended that there be Common Maritime Information/Data Structure that would be accessible from a single integrated system.  Shore users require information pertaining to their maritime domain, including static and dynamic information on vessels and their voyages.  Ideally, this information should be provided in “an internationally agreed common data structure.  Such a data structure is essential for the sharing of information amongst shore authorities on a regional and international basis.”

At present, there is no specific guidance or standards related to the presentation/display of MIOs on shore-based equipment or systems.  However, there are a number of general and equipment-specific international standards that have been adopted by IMO, IHO, and IEC that contain “guidance” related to the presentation/display of various types of shipborne navigation-related information.  This is something that will need to be part of e-Navigation development and implementation.

## AtoN Attribute Information

The exchange of information about AtoN between any parties in a digital environment will require an internationally agreed standard so that information can be automatically compiled for sending and automatically understood by systems that receive it.

Such a standard has been developed in accordance with IHO S-100 and will be submitted to the IHO Registry by IALA. This will enable harmonisation of the management of information about AtoN, and in particular information that is relevant to mariners: “situation normal” data (position, colour, shapes, light etc) and also “situation abnormal” (lost top mark, light on reduced range, unlit etc).  In GIS terms this sort of information can be described as attribute data (information particular to a GIS object, such as an AtoN) and metadata (data about the attribute data).

## e-Navigation Testbeds

The term testbed is used across many disciplines to describe a platform that is used for research, development or testing.  Such a platform is can be protected from a live (or production) environment. However, in the maritime domain, it is often necessary to conduct live tests with appropriate safety precautions in place.

In the context of e-navigation, a testbed is used to demonstrate/evaluate a proof of concept of one or more of the e-navigation solutions, systems and services.  These testbeds may be established in a live or simulated test environment.

In order for e-navigation solutions to have global application, IALA will facilitate the collation and sharing the outcomes of testbeds,

In recent years, some of the more prominent testbeds that have been set up for e-navigation applications include:

•	EfficienSea (Baltic Sea)

•	ACCSEAS (North Sea)

•	Mona Lisa (Baltic Sea) and

•	The Marine Electronic Highway (MEH) (in the Straits of Malacca and Singapore).

The implementation of e-Navigation will be phased and iterative. IALA has taken on the role of coordinating the requirements and results of e-Navigation Test Beds.

A registry is simply a bookkeeping device where definitions/ specifications are kept in organised locations known as registers. The registry eases the tasks of development of new things, by providing a centralised source for finding definitions/ specifications.

Refer to in particular MSC85/26, Annex 20, paragraphs 9.9.1., 9.1.5, and 9.9.3.

2	 GPS SPS Performance Standard, 2008

United Nations Office for Outer Space Affairs, “Current and Planned Global and Regional Navigation Satellite

Systems and Satellite-based Augmentations Systems”, 2010

A 1kW transmitter will generally allow position fixing to better than 10 metres over a radius of about 200 nautical miles.

United Nation Office of Outer Space Affairs, “Current and Planned Global and Regional Navigation Satellite Systems and Satellite-based Augmentations Systems”, 2010

GLA R&RNAV 2010.  Second Racon Trials with NT Radar, Report No: RPT-29/NW/10

IMO Resolution MSC.74 (69) Annex 3 refers.

Section 12 of the IALA Guideline 1028 on AIS Volume 1 Part 1 – Operational Issues