Title: Requirement Engineering Process for IALA e-Navigation

Authors: Ing. Peter J. W. Hooijmans, Ministry of Transport, Public Works and Water management, The Netherlands (peter.hooijmans@rws.nl) and Dr Michele Fiorini, SELEX – Sistemi Integrati, Italy (mfiorini@selex-si.com)

Status: working document to Architecture Technical Working Group (WG5) e-Navigation of IALA

Version: 1.1 (03-May-2010)

During the last intersessional meeting in Cape Town it was recognized that the transition process of user needs to system requirements should be structured and traceable. This to mitigate the risk that a “Santa Claus” wish list turns into the requirement specification for the e-Nav architecture.

This is where requirement engineering is needed.

Requirements engineering is something very common for large and complex projects. To guarantee throughout the entire process that the (correct)user needs are realised a process is necessary to structure this.

In appendix A some systems engineering models are shown. In short it is possible to say that the V-model is a widely used model for requirements engineering and the constant verification and validation of the requirements throughout the entire process of a project. From user needs at the start to a completed system at the end while the Spiral model is more suitable for complex information technology projects.

Currently the spiral approach is used for the development of the e-NAV architecture.

The purpose is not to discuss the usage of the model but to have the best of both. In this case, to get a solid system requirement specification (V-model) or concept of requirements (spiral model).

Since the more familiarity (of the authors) with the V-model, it will be used to explain the process to come from user and stakeholder needs to a system requirement specification.

To obtain a clear system requirement specification a process of interaction with stakeholder and users is necessary to obtain the needs of the different parties.

The ISO/IEC 15288 is a Systems Engineering standard covering processes and life cycle stages. The described process for specification and requirement analyses are very useful to use in this stage of the development of the e-Navigation architecture.

After studying the relevant chapters on the specification process and requirement analyses process it is possible to come to the conclusion that this can be very helpful in our approach.

The goal of the stakeholder requirement specification process is tot formulate requirements for a system that can deliver the services as desired by users and stakeholders. The activities related with this process are described in the ISO/IEC 15288 but one is of great importance to our task. This process should guarantee that each requirement is traceable to its source and the need of the stakeholder or user. This requires that we should use a kind of format on which requirements are delivered. In table 1 a list of items is given which need to be provided in order to make requirements traceable and to make the requirement process transparent.

If all the user needs are transformed into requirements, a requirements analyses process is needed to translate these requirements into a system requirement specification which is more of a technical nature.

IALA should be responsible to provide a System requirement specification and a (global) shore based system Architecture. This is still high level engineering. Depending on the policy of the concerning national Authority this could be made more in detail, with or without providing a SubSystem Design (see figure hereafter).

In the forthcoming intersessional meeting it should be discussed what process is to be used to translate user needs into a system requirements specification. We recommend that the ISO/IEC 15288 is used but are biased because of the experience with this and a better suggestion is welcome also. Secondly the table 1 should be reviewed to see if certain items are missing or do need to be on the list.

Table 1

| Item | content (example) |
| --- | --- |
| Unique Requirement number | E-001010101 |
| Requirement initiator | D. Patraiko (user needs..) |
| Created by | P. Hooijmans |
| Author first version | P. Hooijmans |
| Short description | exchange of ship name |
| Description | the international name of a ship must be exchanged between shore-ship and shore-shore |
| Comment | the international name of a ship is the name which is registered at Lloyds… |
| Source | document, guideline, resolution etc etc. or example: IMO document on user need 1.x.x.x |
| Category | t.b.d. |
| Creation date | 01-01-10 |
| Date 1st version | 01-04-10 |
| Date and Time last change | 01-02-2010 12:00:00 |
| Date approval |  |
| Altered by | J. Doe |
| Approved by |  |
| under construction | yes/no |
| Class |  |
| requirement status | draft/concept/final |
| Priority | L/M/H |
| Project number |  |
| Revision | 1.0 |
| Related data object | "vessel as such" |
| data object document | 54-110.10 |
| Risk | L/M/H |

Appendix A

Introduction

In the next paragraph various model are analysed after some background definition.

Definition of system and systems engineering

According to the International  Council on Systems Engineering (INCOSE) a system is defined as “a combination of interacting elements organized to achieve one or more stated purposes” and systems engineering like “an interdisciplinary approach and means to enable the realization of successful systems. It focuses on defining customer needs and required functionality early in the development cycle, documenting requirements, then proceeding with design synthesis and system validation while considering the complete problem. Systems Engineering integrates all the disciplines and specialty groups into a team effort forming a structured development process that proceeds from concept to production to operation. Systems Engineering considers both the business and the technical needs of all customers with the goal of providing a quality product that meets the user needs”.

Some systems engineering principles

Start with Your Eye on the Finish Line

You should reach consensus at the very beginning of the project on what will constitute success at the end. This means that the stakeholders should start with an agreement of what the project should accomplish and the metrics that will be used to measure the success of the project

Stakeholder Involvement is Key

Successful projects involve the customer, users, operators, and other stakeholders in the project development. Systems engineering is a systematic process that includes reviews and decision points intended to provide visibility into the process and encourage stakeholder involvement

Define the Problem Before Implementing the Solution

Very often, you'll have a solution already in mind at the start of a project and may even find yourself "backing into" requirements to match your solution. Resist this temptation and instead use the systems engineering process to first define the problem. You'll find that there are actually multiple ways to solve the problem, and a good trade study will help you to determine the best solution on the basis of a clear understanding of the stakeholder needs

Delay Technology Choices

Technology is constantly changing. The choices available when a project is initially conceived may well be replaced by better technology by the time the project is implemented. Specifying technology too early will result in outdated technology or constant baseline changes as you try to keep up with technology advancements. It's best to follow the systems engineering process by defining the needs, requirements, and high-level design without specifying technology. You'll have a stable baseline, and you'll be able to make the most appropriate technology choices when it is time to implement.

Divide and Conquer

A key systems engineering strategy is the decomposition large and complex systems into smaller subsystems and then of the subsystems into more manageable hardware and software components.

Traceability

As you move from one step to the next in the systems engineering process, it is important to be able to relate the items in one step with those in another. The requirement will be related to a user need as well as to a test that will be used to verify the requirement.

Heritage

The origin of the Systems Engineering Standards could be dated back to more then 40 years ago when the United States Department of Defence (DoD) produce the Military Standard 499 (MIL.STD-499), Engineering Management, dated July 17, 1969 for application within the defence industry.

Various updated and significant rewritten has been followed as depicted hereafter

Cfr.

Martin, james N., Evolution of EIA 632 from an Interim Standard to a Full Standard and Evolution of EIA 632: Process for Engineering a System, Proc INCOSE, 1998

and even more

Cfr.

Sheard, Sara A., The Frameworks Quagmire: A Brief Look, Proc. INCOSE, 1997b.

Why a model for systems engineering?

Nearly two-thirds of software development projects in the US fail, due to either cancellation, overrunning of budgets, or delivery of software that is never put into production.

Reasons for these failures should be identified in lack of stakeholders (e.g. users, customers) involvement, no clear statement of requirements, no project ownership, no clear vision and objectives and lack of planning.

A good systems engineering development model should impose discipline on developers to produce a consistent set of requirements, functional arrangements and design solutions with the goal to improve productivity and provide deliverables that satisfy product’s end purpose.

Cfr.

Grapham, I., Object-Oriented Methods: Principles and Practice, 3rd Ed., Addison-Wesley, 2001.

Some systems engineering models

In the next pages few system engineering models widely used and well known are described trying to provide a short and schematic representation of them. There are many more models available in open literature but we will focus on 4 of them for suck of brevity.

1st model: Waterfall

In a simplified view, system life cycle development begins with the gathering of requirements and domain knowledge and ends with system deployment, maintenance and eventually retirement.

The Waterfall model was accepted as the primary SE process model through most of the 1980s. Each phase of sequential development is completed via formal review before the next phase begins. It is most useful when the problem and solution methods are well understood.

2nd model: Incremental (Iterative) Waterfall

Incremental development is defined as the development of a system in a series of versions or increments. At each, increment, a subset of functionality is selected, designed, developed and implemented. Often the rework of system functionality can proceed with a “reasonable” amount of certainty that the desired result will be achieved.

3rd model: Spiral

The spiral model corresponds to a sequence of waterfall models. This model corresponds to risk-oriented, iterative enhancement and it recognizes that implementation options are not always clear at the beginning of a project.

The radial direction corresponds to cumulative costs incurred and the angular direction corresponds to progress made in completing a spiral cycle. Each cycle of development has the following phases: Identification

A key characteristic of the Spiral model is the assessment of management risks at regular stages in the project and the initiation of actions to counter these risks. Before each cycle, risk analysis is initiated and, at the end of each cycle, a review procedure assesses whether or not to proceed to the next loop in the spiral.

Cfr.

Boehm, Barry, Information Technology in Action, Prentice Hall, 1993.

4th model: Vee

The Vee model depicts a top-down development and bottom-up implementation approach. On the left side, decomposition and definition descends as in traditional Waterfall model while on the right side integration and verification ascends culminating at the system level.

The Vee model is composed of three layers in increasing engineering details: User’s perspective (it is the stakeholders view; who is interested in presenting requirements and receiving a finished product), Systems Engineer’s perspective (it encompasses architectural details to address decomposition of the system-level specification into system design and subsystems’ specifications and designs; build and tested subsystem and finally the tested system) and Contractor perspective (it covers the implementation process –usually performed by contractors- and it is associated with component specifications and designs with fully tested components).

Advantages and limitations of the above models

Cfr.

Technical Memorandum no.1, Phase I – Systems Engineering Management Plan: A Process review and Appraisal of the Systems Engineering Capability for the Florida Department of Transportation (FDOT), Version 2, available on the Internet at www.floridaits.com/SEMP/Files/PDF_Report/030220-TMI-V2.pdf

Closing note

<< Technology applied to an efficient operation will magnify the efficiency.

Technology applied to an inefficient operation will magnify the inefficiency >>   Bill Gates.