This document needs to be put into the IALA template format for Recommendations

Appendix 5 – 	Interfacing model of a service

# Introduction

This Annex exhibits the following flow of thought, when addressing interfacing issues of the service:

- The context of the service as an e-Navigation service within the shore-based e-Navigation system is considered. Interfacing issues resulting from that are discussed. This is the bird’s eye view on the service from a shore-based e-Navigation system perspective.

- All of the interfaces of the service are identified, i.e. a complete list of all interfaces of the service is provided. There are both Machine-Machine-Interfaces (M2M) between components and Human-Machine-Interfaces (HMI) specifically supporting the Technical Operation Personnel. This complete list of interfaces takes into account in particular the contribution of the Distribution Model (Annex 3).

- In a next step, the Machine-Machine-Interfaces of the service are addressed in more detail. Issues like recommended data encoding, including in particular the relationship to the Maritime Data Exchange Format (MDEF), and recommended protocol stacks are discussed.

- Finally, the Human-Machine-Interfaces of the service are discussed in more detail. Information is provided for administrations as to what are best practices when setting up Human-Machine-Interfaces of the service towards their Technical Operation Personnel.

# The place of the service under consideration in the common e-Navigation system architecture

Figure A5.1 illustrates the place of the service under consideration in the shore-based e-Navigation system architecture as described by IALA Recommendation on the E-Navigation Architecture – the shore perspective (IALA Recommendation eNAV-101). Figure 1 also applies the general architectural statements of that Recommendation to the AIS technology.

In addition, Figure 1 clearly identifies the co-operative nature of the information exchange between the shore-based AIS Service and the applications connected to it on the shore side and the mobile AIS stations and the mobile applications connected to them on the mobile side. The shore based applications connected to the AIS Service will be further explored in the next section.

It should be noted, that the AIS and the AIS Service share the dependencies of the e-Navigation architecture, i.e. the dependency on GNSS for position fixing and/or timing and the dependency on infrastructure.

Figure 1: Place of the AIS Service within the e-Navigation architecture

IBS

Ship’s

sensors

Shipboard

Applications

AIS VHF

Data Link

(AIS VDL)

AIS Technology

= encapsulated

link technology

Shore-based e-Nav system

Ship technology 
environment

Appli-cation 1

Application 2

Application 3

Application 4

mobile AIS station

INS

AIS Service

Application-to-
application 
(peer-to-peer)

functional
connection

other

ships

other

ships

mariner

Shore-based
users

World Wide Radionavigation System (WWRS) of IMO, including GNSS, GNSS augmentation and terrestrial backup

## The AIS Service within the shore-based e-Navigation system architecture

IALA Recommendation on the common shore-based e-Navigation system architecture” (eNAV-201, Edition [2008]) defines the relative place of the AIS Service within that system architecture. It identifies the AIS Service as a shore based e-Navigation service within the so called “Data Collection and Data Transfer services” of the common shore-based e-Navigation system architecture (compare Figure 2).

Figure 2: Place of the AIS Service within the shore-based e-Navigation system architecture

In particular the need to provide a functional interface between the AIS Service and other requesting services within the shore-based e-Navigation system is identified.

Since the “Data Collection and Data Transfer services” of the shore-based e-Navigation system will be requested by other e-Navigation services within that system, there can be identified multiple client-server-relationships. Those can be combined to a hierarchy within the shore-based e-Navigation system as indicated by Figure 3 (following page).

Figure 3: Client-server relationships of the AIS Service with other shore-based e-Navigation services – resulting hierarchy of shore-based e-Navigation services and the place of the AIS Service

Figure 3 shows, that the AIS Service can / will be requested by

- the Value-added Data Processing Services: The AIS Service is requested as an information source on traffic objects (mainly ships) for data correlation and combination purposes (“data mining”);

- the User Interaction Service: The AIS Service is requested as an information source on traffic objects (mainly ships). The received data is then displayed to shore-based users by a Human Machine Interface. Also the AIS Service is requested for transparent communications via the AIS VHF Data Link.

- the Gateway Service: The AIS Service is requested as an information source on traffic objects (mainly ships); information is forwarded to shore-based external systems by appropriate Machine Machine Interfaces.

- other Data Collection and Data Transfer Services: The AIS Service is requested as an information source in some cases (e. g. for received Aids-to-Navigation data to be forwarded to the Visual Aids Service) but mainly for transparent communications via AIS (e.g. for setting up synthetic/virtual visual aids-to-navigation on behalf of the Visual Aids Service or for DGNSS correction data transmission via AIS).

Hence, in these cases the AIS Service will act as a requested service (server). The (other) requesting services act as clients to the AIS Service.

There are (few) cases, where the AIS Service will act as a requesting service (client) to other e-Navigation services within the shore based e-Navigation system, e.g. when initiating DSC channel management via the DSC Service.

Note, that the need for data transmission via e.g. a data transfer network due to the geographical distribution of components of the AIS Service is encapsulated within the AIS Service and therefore “not visible” at this level.

[System ID - issue

Note also, that the so called „System or Virtual MMSI“ (00MID9xxx) for addressed information exchange via AIS would allow to address specific services and/or users within the shore-based e-Navigation system (refer to Recommendation ITU-R M.585-3 “Assignment and use of maritime mobile service identities”).]

## The functional interface of the AIS Service to shore-based applications

### Introduction

The above figures indicated the functional interfaces between the AIS Service within the shore-based e-Navigation system by arrows. The primary concern is with the information exchanged between the AIS Service and other shore-based e-Navigation services. The functional interface thereby facilitates the integration of the AIS Service into shore-based applications.

Since information is abstract, the functional interfaces need first and foremost to be considered in functional terms of information delivery and only later from a technological point of view. The functional interface encapsulates the technical details of both the AIS technology and of the layout and local configuration of AIS Service.

### The AIS Service as a “requested service”

Due to the hierarchy shown in the previous section where the AIS Service has been identified as a requested service (compare Figure 3), it is the AIS Service which provides the functional interface description: the requesting services (clients of the AIS Service) need to comply with the interfaces provided by the AIS Service. They can do so e.g. by using an “AIS Service driver” which is common practise in IT. Thereby they encapsulate the details of their interfacing to the AIS Service and make their own functionality / software independent of smaller changes of the interface of the AIS Service.

### Use case description and Entity-relationship-description

There are at least two relevant angles of perspective regarding the functional interface of the AIS Service:

- the use case model, which allows to seamlessly derive use cases of the AIS Service from user requirements;

- the entity relationship model, which focuses on relevant associations between abstract and/or physical entities within the AIS Service and abstract and/or physical entities outside the AIS Service.

Both views complement each other:

- The use case model answers the question: “What is the AIS Service good for?” and thus directly supports application design once the service use cases of the AIS Service have been identified and described.

- The entity relationship model answers the question: “What aspects does a competent administration need to consider, when setting up and running its AIS Service?” It is particular important for life-cycle management.

### The functional interface of the AIS Service in terms of use cases

The functional interface of the AIS Service in terms of use cases can be determined when correlating the following aspects:

- What fundamental functionalities would be expected from the shore-based AIS Service. This is the top-down requirement derivation, eventually starting at user requirements and business processes. Figure 4 shows application of that the seamless derivation process regarding the AIS Service (compare IALA Recommendation on shore-based e-Navigation architecture – the shore perspective).

Figure 4: Seamless derivation of service use cases and component requirements from system level requirements

- What fundamental functionalities could be delivered by the shore-based AIS Service taking into account the engineering knowledge regarding the capabilities and limitations of the AIS technology? This is the bottom-up approach.

Both point of view meet in the definition of what has been called the external Basic AIS Services (BAS). Hence, the functional interface in terms of service use cases of the AIS Service is defined as a set of external BAS.

### Functional interfaces in terms of entity relationships

Figure 5 (which is Figure 1 of the Main Body) shows highlighted the associations or interactions of the AIS Service with external entities, both abstract and physical. Associations which cross the boundary of the AIS Service require external interfacing of the AIS Service in most cases.

In addition, there are internal interfaces between the functional components of the AIS Service, which eventually will materialize physically within an IT environment.

It should be noted, that the content of the Human Machine Interfaces (HMI) of the AIS Service to the Technical Operation Personnel (TOP) and to the Technical Development Personnel (TDP) is determined by the tasks the AIS Service requires over its full life-cycle from that personnel (compare Annex 13 – “Efficient operation and maintenance of the AIS Service”). Hence, this relationship is highlighted in the Figure 5. The actual physical implementation of the Human Machine Interfaces will be done by the appropriate functional components as shown below.

It should also be noted, that certain abstract models of the AIS Service which were developed previously have a strong bearing on the Interface Model, namely

- the Basic AIS Service definitions and the definitions of other use cases

- the Data Model of the AIS Service

- the Structure Model of the AIS Service

- the Interaction Model and the Data Storage Model of the AIS Service

Figure 5: Entities which require interfacing of the AIS Service (highlighted)

# Identification of the individual interfaces of the service

As a first step, it is helpful to re-consider the Figure 10 of the Main Body of this recommendation. In this Figure, the tasks of the functional components were identified within the service and in relationship to the external shore-based e-Navigation system or the Physical Link(s). Figure A5.1 is now used to highlight the tasks of many functional interfaces due to the tasks of the functional components they connect.

Figure A5.1: Identification of the tasks of the functional interfaces (highlighted/shaded) in relationship to the functional component’s tasks they connect

Note: The Human Machine Interfaces to the Technical Operation Personnel have not been included in Figure A5.1.

The tasks highlighted in the Figure A5.1 above have already been taken into account when developing the Structure Model of the service (compare Main Body of this Recommendation). Figure A5.2 on the following page now highlights the interfaces between the functional components of the Structure Model.

Figure A5.3 then goes on to specifically identify all interfaces of the service, both internal and external, both Machine-to-Machine and Human Machine interfaces (bold lines). Figure A5.3 asserts completeness in regard to the various interfaces of the service. Figure A5.3 constitutes the core of the Interfacing model of the service.

Figure A5.2: Structure model of the service with highlighted interfaces (bold lines / boxes)

Technical Operation Personnel (master control)

HMI

Logical Layer -

Logical Shore Station (LSS)

HMI

Net Data

Status

Service

Management Layer

HMI

Physical Shore Station (PSS)

HMI

Physical Link Terminal Equipment (PLTE) Layer

HMI

Physical Link Couplers

(PLC) Layer

Physical Link

( = Medium)

Technical Operation Personnel

(there are tasks for each layer in principle)

Configuration

Physical

Layer /

Remote

Shore

Station

* Note: This symbolic representation does not imply any inference as to the amount of required personnel for the technical operation of the service. This symbolic representation aims at indicating that human interaction with the largely automated service is required as the last resort - and in some cases possibly on a regular basis - on all layers of the service.

Boundary of shore-based e-Nav system

Figure A5.3: Complete identification of functional interfaces of the service (external/internal; not including interfaces of infrastructure  components)

# Application data encoding and protocol stacks for the Machine-to-Machine interfaces of the service

## Introduction

### Bundle of application data encoding and protocol stacks

This chapter recommends certain application data encoding techniques and protocol stacks (ISO/OSI layered model stack) for the Machine-to-Machine interfaces of the service, which were identified above. By the same token, certain other options for application data encoding are discouraged.

The recommendation always comes together as a bundle of at least one application data encoding technique and one IT protocol stack. The reason is that some application data encoding techniques are fit to certain IT protocol stacks, while this may not be true of different combinations.

### Data Model of the service and the impact of the UMDM

When designing the application data encoding, the Data Model of the service has a strong bearing: It is these data objects and data properties (also referred to as data types) which need to be encoded so that a data exchange may take place between machines. Since, the Data Model of the service is a subset of the (emerging) Universal Maritime Data Model, the application data encoding technique should provide for abstract data encoding, sufficient flexibility to cater for future changes and in particular for the Universal Data Object Identifier (U.DOI) as one fundamental ingredient.

### Open System Philosophy

Also, the application data encoding and IT protocol stacks recommendations should be in keeping with the open system philosophy of the common shore-based e-Navigation system architecture. Hence, IT protocol stacks compatible with the ISO/Open System Interconnection (OSI) protocol stack should be recommended.

### The eNAV-61162 encoding technique and its use of the UMDM/UDOI concepts

Combining these requirements with the nature of the data flow to and from the service, it appears that an application data encoding technique would be best suited, which would combine sentence-orientation with abstract and flexible data encoding capabilities. Also it should be based on well established maritime standards.

Already many years ago, the “traditional” IEC 61162 was created to fulfill these requirements. All data objects are represented as abstract data in ASCII characters with field delimiters. However, the “traditional” IEC 61162 sentence structure rigidly implied the meaning of certain fields by order of appearance. This made the exchange of single data objects cumbersome, because “empty” fields would be exchanged as well.

The notion of a binary encoded field in IEC 61162 (the so-called VDM-sentences) was introduced due to the specific interfacing needs of the AIS technology. This notion was initially considered alien to the original philosophy of the IEC 61162, and therefore rightly caused some concern. Today, a co-existence between the traditional abstract data encoding and the binary encoding within the framework of the IEC 61162 has been achieved. However, it was not created for the dissemination of AIS-derived or even any data in highly integrated and sophisticated shore-based IT systems, such as the shore-based e-Navigation system. This proliferation of the intricacies of the AIS technology would tie those systems to the development of the AIS radio specification for all time. From a system engineering point of view, this close connection carries many disadvantages. This gave rise to the notion to use XML extensively. This application data encoding technique provides maximum flexibility for abstract data, but introduces substantial overhead.

To combine the advantages of the traditional IEC 61162 data encoding technique, i.e. the abstract data encoding, the sentence-oriented structure and its wide recognition in the maritime field, with the current need for even more elaborate abstract data encoding and flexibility, the so-called “eNAV-61162” derivative of the IEC 61162 was developed, using the UMDM/UDOI concept extensively: The UDOI as a leading designator for the data fields of a sentence would provide the flexibility in terms of the concrete meaning of the individual data fields, thus overcoming the rigidity of the traditional approach.  This approach would be compatible with the philosophy of the traditional IEC 61162 while introducing one a little overhead both in terms of transmission bandwidth and parsing compared to XML. The details are developed in Annex 2.

### The XML encoding technique and its use of the UMDM/UDOI concept

--- to be described ---

### Encoding of Data Container content transparently forwarded by a service

--- maybe this paragraph needs to be moved /copied to a place where the different roles of different services of the shore-based e-Nav system are discussed.

It should be noted, that the Data Model also accommodates such data items (“data containers”), which are forwarded by a service transparently between the requesting service and its Physical Link(s) in either direction without changing or recognizing their content in any way. The encoding of these data containers is completely and exclusively determined by the requesting service, i.e. the encoding of the content of those data containers is described within the documentation of the requesting service, only. The own service does not need to know this encoding used.

Examples for transparent forwarding of these data containers and for encoding description of those data container’s content hosted exclusively at the requesting service are:

- transparent reception/transmission of application-specific (“binary”) data: any other e-Navigation service may hold the encoding description for its application-specific data.

- differential correction data: the encoding description for this data is maintained exclusively in the DGNSS Correction Service of the shore-based e-Navigation system architecture.

## Interfacing the service under consideration to other services

--- continue here ---

### Net data exchange interfacing to requesting services

Table 1 (overleaf) shows the recommended application encoding and IT protocol stacks for the Machine-to-Machine interface of the LSS to the requesting services of the shore-based e-Navigation system.

Some rationale and explanations to the Table are as follows:

- The rationale for the asymmetry regarding XML recommended only as an output data encoding technique from the LSS to the requesting e-Navigation service is as follows: While the XML may be useful for the requesting shore-based e-Navigation service and while it is relatively easy to compile XML files as output data, the AIS Service as such does not benefit from XML usage internally but would require a parsing process for incoming XML messages.

- All of the above interfacing techniques can be implemented concurrently at the LSS interface, thus providing a portfolio of data access options to the designer of the requesting service of the shore-based e-Navigation system and hence some flexibility over life-cycle.

- Alternatively, only some of the above interfacing techniques can be implemented at the discretion of the authority.

Table 1: Recommended application encoding and IT protocol stacks for Machine-to-Machine interface of the LSS to the requesting services of the shore-based e-Navigation system

| External Basic Service | eNav-61162 + SMTP | eNav-61162 + TELNET/SSH | XML + HTTP/FTP | eNav-61162+ SQL | Remarks Refer to Annex 2 regarding details on the eNav-IEC 61162 encoding method and to Annex 3 on the recommended protocol stacks. |
| --- | --- | --- | --- | --- | --- |
| External BS Cat. I | X | X | X | - | Push, dynamically: service under consideration to requesting service |
| External BS Cat. II | X | X | - | - | Push, dynamically: Requesting service to service under consideration |
| External BS Cat. III | X | X | - | - | Triggered once and autonomously performed by service under consideration for the requested period of time; Requesting service prompts service under consideration. |
| External BS Cat. IV | X | X | Q: - (1) A: X | X | Triggered once and autonomously performed by service under consideration for the requested period of time; service under consideration prompts requesting service. |
| Replay | X | X | Q: - (1) A: X | X | Replay of recorded data by service under consideration based on selection of the requesting service; requesting service prompts service under consideration; once initiated replay push by service under consideration. |

Legend:

Q	Query of requesting service of the shore-based e-Navigation system

A 	Answer/reply of the service under consideration (“own service”)

X 	Recommended

- 	Discouraged

(1) 	Use either eNav-61162 or SQL sentences for query.

### Management interface of AIS Service to requesting e-Navigation services

The Management Interface of the AIS Service to the requesting e-Navigation services serves to exchange configuration and status data between the AIS Service Management and the Service Management Layer of the requesting service.

This interface is by nature a query and answer interface. Queries are in particular the configurations or re-configurations regarding the Basic AIS Services by the requesting service and requests for specific status information from requesting service. Answers are in particular the acknowledgments or status alerts given by the AIS Service Management to the requesting service.

Hence, the following recommendation appears to be the best practice:

- IEC 61162-derived + SQL + TELNET/SSH and/or

- IEC 61162-derived + SMTP and/or

- IEC 61162-derived + TELNET/SSH

All of the above interfacing techniques can be implemented concurrently at the LSS interface, thus providing a portfolio of configuration and status-access options to the designer of the requesting service of the shore-based e-Navigation system and hence some flexibility over life-cycle.

Alternatively, only some of the above interfacing techniques can be implemented at the discretion of the authority. XML encoding is possible theoretically but is discouraged due to the lack of advantage for encoding configuration and status data on one hand and the massive overhead on the other hand when using it.

## Internal Interfaces of the AIS Service

The following Table 2 shows the recommended options for application encoding and IT protocol stack for the internal interfaces of the AIS Service. For descriptions of the application encoding methods and for the IT protocol stacks refer to the Attachments to this Annex.

Note: Only internationally fully described methods for application encoding and IT protocol stacks are recommended in keeping with the open system philosophy.

Table 2: Recommended application encoding and IT protocol stacks for Machine-to-Machine interfaces internal to the AIS Service

| Machine-Machine-Interface (M2M) | Recommended Option | Alternative for individual interface | Remarks |
| --- | --- | --- | --- |
| AIS net data interface PSS to Logical Layer (to IDS and to LSS) | - Application encoding: IEC 61162 derived - IT protocol stack: SMTP | Not recommended | - |
| Status/Configuration interface LSS to ASM-Node | - Application encoding: IEC 61162 derived - IT protocol stack: SMTP | Not recommended | - |
| Status/Configuration interface PSS to ASM-RS | - Application encoding: IEC 61162 derived - IT protocol stack: SMTP | Not recommended | - |
| Status/Configuration interface ASM-RS to ASM-Master | - Application encoding: IEC 61162 derived - IT protocol stack: SMTP | - Application encoding: like recommended option - IT protocol stack: TELNET or SSH | - |
| Status/Configuration interface ASM-Node to ASM-Master | - Application encoding: IEC 61162 derived - IT protocol stack: SMTP | - Application encoding: like recommended option - IT protocol stack: TELNET or SSH | - |
| Interface AIS Base Station to PSS | - Application encoding: IEC 61162 as amended by IEC 62320-1 - IT protocol stack: TCP-Socket | - Application encoding: like recommended option - IT protocol stack: RS-232 or equivalent | - |

# Human Machine Interface design considerations

## Introduction

In-depth consideration should be given to the Human Machine Interface design of the components of the AIS Service. Ergonomic requirements need to be fulfilled by default. Also, the HMIs determine to a large degree the efficiency of the Technical Operation Personnel over the full life-cycle of the AIS Service. Hence comfortable remote control functionalities and decision support should be considered.

## Options for delivering Human Machine Interfaces

Different classes of HMI can be identified as follows.

- HMI direct.

- Local HMI.

- HMI locally dispatched.

- Remote HMI.

The following explanations are given:

1. HMI direct

The „HMI direct” comprises all signalling, displays and input devices which are mounted on the machine and form an integral part of it, i.e. on-/off-switches, reset keys, LED indications, etc. The “HMI direct” is always at the site where the machine is located, i.e. at the Remote site and at the Node site. These interfaces regularly are intended for quick access to the most important information regarding the current status of a machine, but don’t serve for comfortable Technical Operation.

Figure 9: The „HMI direct“

2. Local HMI

Similar to „HMI direct“. The difference is: The „Local HMI“ comprises all separate signalling, displays and input devices which are connected to the machine but which are wholly dependent on that machine for their functionality. Examples are computer peripherals such as mouse, keyboard and display.

Figure 10: The „local HMI“

3. HMI locally dispatched

The „HMI locally dispatched“ is a human interaction device at the site of the machine, i.e. at Remote site or Node site, which is connected to the machine by a Local Area Network, i.e. it is on the same premise as the machine it connects to, and which runs independent of the machine as far as the basic human interaction functionalities are concerned, but depends on the machine regarding the information as such.

Figure 11: The „HMI locally dispatched“

4. Remote HMI

The „Remote HMI “ is a human interaction device at the Technical Operation Personnel site (either the fixed regular working place or mobile working place), which is connected to the machine by a Wide Area Network, i.e. it is not on the same premise as the machine it connects to, and which runs independent of the machine as far as the basic human interaction functionalities are concerned, but depends on the machine regarding the information as such. The provision of Remote HMI alone allows for centralized Technical Operation Personnel site(s); compare Distribution Model.

Figure 12: The „Remote HMI“

## Interfacing the AIS Service to Technical Operation Personnel with Human-Machine-Interfaces

### Recommended minimum HMIs for functional components

The following functional components of the AIS Service should provide a internet technology based machine-machine-interface, which would support the “HMI locally dispatched” and the “Remote HMI”:

- LSS

- PSS

- Fixed AIS Stations

- ASM Master

- ASM Node

- ASM RS

When the above functional components are materialized as integral components consisting of both hardware and firmware, i.e. as physical black boxes delivering the required functionality at their interfaces, there should be in addition appropriate “HMI direct” and/or “Local HMI”. The Fixed AIS Stations should have in addition an appropriate “HMI direct” and/or “Local HMI” in any case.

### Recommended IT protocols

For “HMI locally dispatched” and the “Remote HMI” the following interfacing setup is recommended:

Session of the Technical Operation Personnel with the machine under consideration:

- to initiate a session on a HMI:

A user enters the IP address of the machine into a standard internet browser. The standard browser connects to the machine via HTML (including use of internationally standardized script languages) and HTTP. The machine acts as a web server to the standard browser and presents a HTML screen and also functionality, which allows to automatically start a separate TELNET or SSH session window.

- during the session: The session is continued by using the TELNET or SSH session window. Note: During the session the data exchanged may be proprietarily encoded.

Requirements for the HTML usage:

The version of the HTML should be selected conservatively in order to allow a large variety of browsers to run the session. No browser specific HTML dialects or functionalities should be used. The HTML script should be checked by an appropriate testing tool for rigidly conforming to the international HTML specification chosen.

The current version of HTML, which fulfils these requirements, is HTML3, --- standard --- .

Requirements for the optional use of script languages:

--- similar statements apply

Annex 6 – 	Internal Time Latency model of a service

Annex 7 – 	Internal Reliability model of a service

Annex 8 – 	Test model of a service

# Annex 9 – 	generic functional description of the Logical Shore Station

The software process of a LSS can run on any appropriate computer at any appropriate place. The options for selecting an appropriate configuration for setting up the instances of the LSS, and its consequences, are discussed in the Distribution Model of the AIS Service (see Annex 3).

An individual software process of an LSS running in an individual computer is called instance of a LSS. Hence, an instance of a LSS is a software process, which transforms the AIS data flow(s) associated with one or more Remote Shore Station(s) into a different AIS-related data flow associated with one (1) requesting service within the shore-based e-Nav system. The run-time configuration of any instance of LSS is managed by the AIS Service Management.

For each requesting service, the AIS Service sets up at least one instance of an LSS. An instance of LSS of the AIS Service communicates with one instance of a logical interface of the requesting service in an 1:1 relationship.

It is required that there are reliable functional connections between any instance of LSS and all associated AIS Shore Stations.

This functionality description would allow for an eventual type approval applied to the PSS, the LSS, and the ASM software module, as far as the AIS-specific functionality is concerned.

# Annex 10.1 – 	generic functional description of the Physical Layer as a whole – the Remote Shore Station

# Introduction

The RSS is the most basic AIS-related entity, which can exist on its own in a real physical environment, as opposed to the Physical AIS Shore Station (PSS) or an AIS base station.

An RSS is physically fixed or is considered to be "fixed". An RSS may be theoretically mounted on a flying or floating platform, however. The latter cases are excluded from the scope of this document for simplicity's sake.

An RSS consists of at least the following components or functions:

- one Physical AIS Shore Station (PSS);

- one AIS base station (or one AIS repeater station);

- RF-domain equipment, at minimum simply a cable and a VHF antenna;

- an agent of the AIS Service Management at the Remote Site (ASM-RS) (generally another software process);

- service owned  infrastructure components on site, such as computer(s) to run the Physical AIS Shore Station and the software agents of the AIS Service Management, Local Area Networking components etc.;

- shared or framework on-site infrastructure, such as power supply, a means to protect the above component against environmental influence and damage, e.g. a shelter building or a housing case.

Thus, an RSS does not necessarily need to be considered large physically.

A RSS may generally have a UTC source of its own (there may be cases during which the AIS base station may be set up using only the synchronisation provided by the AIS VDL itself, i.e. UTC indirect or even slot synchronisation). This UTC source may be internal to the AIS base station such as a GNSS receiver, or external to the RSS (being a part of the on-site framework infrastructure), such as a solid-state (crystal oscillator) clock, also providing timing for the AIS base station(s) by an appropriate timing interface (such as IRIG / IEEE 1344).

Figure 11 on the following side illustrates these aspects. It zooms into Premise A given in Figure 9 above, while adding an overview of on-site framework infrastructure considerations (refer to Annexes 3 and 12 for further detail).

Figure 11: Typical setup of Remote Shore Station of the AIS Service (with on-site framework infrastructure and Technical Operation Personnel site; also typical)

Cautionary note: Figure 11 should not be construed as normative or the only recommended way to set up a Remote Shore Station of the AIS Service. Figure 11 does claim however to be correct and consistent with the normative statements in this Recommendation in regard to what it shows.

Conceptional co-location issues of RSS of the AIS Service with RSS of other shore-based e-Nav services at the same Remote Site, i.e. e.g. VHF Communications Service or DSC Service, are addressed in Annex 3 from the AIS Service’s point of view.

This annex on Remote Shore Stations (RSS) Considerations

- provides a detailed description of the RSS in total;

- provide guidance how to install all necessary components of a Remote Shore Station at the Remote Site in order to avoid interference;

- gives particular attention to the technical details of co-location with other e-Nav services operating at the same Remote Site.

# --- more chapters

# Annex 10.2	- generic functional description of the Physical Shore Station

This functionality description would allow for an eventual type approval applied to the PSS, the LSS, and the ASM software module, as far as the AIS-specific functionality is concerned.

# Annex 10.3 – generic functional description of the Physical Link Terminal Equipment

# Annex 10.4	- generic functional description of the Physical Link Couplers

# Annex 11 – 	generic functional description of the Service Management

This functionality description would allow for an eventual type approval applied to the PSS, the LSS, and the ASM software module, as far as the AIS-specific functionality is concerned.

Annex 12 – Site co-location issues and on-site infrastructure considerations for a service

# Co-location issues at Remote Sites

From a conceptual point of view, one shelter or housing case at a Remote Site may house components of different services of the common shore-based e-Navigation service.

In the context of an individual service under consideration, the term "Remote Shore Station (RSS)" indicates the presence of components of that service under consideration at a Remote Site, regardless of any co-located capabilities of any other service of the common shore-based e-Navigation system at the same Remote Site. Thus, each and every service of the common shore-based e-Navigation system uses the term Remote Shore Station within their respective context. Hence, from a shore-based e-Navigation system point of view, it is not only possible but also likely that there are Remote Shore Stations of different shore-based e-Navigation services at the same Remote Site.

The Figure A12.1 shows the resulting situation at the Remote Site, taking into account also the shared use of the on-site framework infrastructure and the common (local) Technical Operation Personnel for all services residing on-site.

# On-site infrastructure considerations

--- to be introduced later --

Annex 13 – 	Recommendation regarding efficient operation and maintenance of a technical e-Navigation service

It should be noted, that no service of the common shore-based e-Navigation system architecture except the User Interaction Service (UIS) provides a direct user interface; refer to main body of this Recommendation.

This is a state-of-the-art engineering principle. It protects application software from unnecessary changes due to configuration or technology improvements within the AIS Service.

Note: In Figure 5 the relationship to the AIS VHF Data Link is not highlighted. That is due to the fact, that the “interface” of the AIS VDL is provided externally to the individual AIS Service and even the individual shore-based e-Navigation system. The AIS VDL exists without them. Thus, the AIS VDL conception-wise provides the “interface” in the form of a “signal-in-space” description, and every individual AIS Service must adhere to that “interface”.

As is the case when mounted on an AtoN; the point here is, that the Physical AIS Shore Station has a fixed geographical position. If it is mounted on an AtoN the area in which the AtoN is allowed to swing is very small compared to the coverage area of this Physical AIS Shore Station.