- ANNEX 2

- IALA-XXXX Aids to Navigation Product Specification Feature Catalogue

- Version 0.1

- Date 2013-09-12

Dataset Attributes 
Simple Attributes 
Complex Attributes 
Association Roles 
Feature Associations 
Features

- Dataset Attributes

Top

- Simple Attributes

Top

- Attribute: AIS Aid to Navigation type

- Definition: The purpose of an AIS Aid to Navigation.

- Remarks:

- Code:

- Type: enumeration

- Values:

| Code | Value | Definition |
| --- | --- | --- |
| 1 | Default, Type of AtoN not specified |  |
| 2 | Reference point |  |
| 3 | RACON |  |
| 4 | Fixed structure off shore |  |
| 5 | Spare |  |
| 6 | Light without sectors |  |
| 7 | Light with sectors |  |
| 8 | Leading Light Front |  |
| 9 | Leading Light Rear |  |
| 10 | Beacon, Cardinal N |  |
| 11 | Beacon, Cardinal E |  |
| 12 | Beacon, Cardinal S |  |
| 13 | Beacon, Cardinal W |  |
| 14 | Beacon, Port Hand |  |
| 15 | Beacon, Starboard hand |  |
| 16 | Beacon, Preferred Channel port hand |  |
| 17 | Beacon, Preferred Channel starboard hand |  |
| 18 | Beacon, Isolated danger |  |
| 19 | Beacon, Safe water |  |
| 20 | Beacon, Special mark |  |
| 21 | Cardinal Mark N |  |
| 22 | Cardinal Mark E |  |
| 23 | Cardinal Mark S |  |
| 24 | Cardinal Mark W |  |
| 25 | Port hand Mark |  |
| 26 | starboard hand Mark |  |
| 27 | Preferred Channel Port Hand |  |
| 28 | Preferred Channel Starboard hand |  |
| 29 | Isolated danger |  |
| 30 | Safe water |  |
| 31 | Special mark |  |
| 32 | Light Vessel / LANBY / Rig |  |

- Attribute: AIS Tx Rx

- Definition:

- Remarks:

- Code:

- Type: enumeration

- Values:

| Code | Value | Definition |
| --- | --- | --- |
| 1 | Tr/Rx |  |
| 2 | Rx |  |
| 3 | Tx |  |

- Attribute: Category of fog signal

- Definition: The various means of generating the fog signal.

- Remarks: The classification "horn" is the generic term for fog signals "nautophone", "reed" and "tyfon".

- Code:

- Type: enumeration

- Values:

| Code | Value | Definition |
| --- | --- | --- |
| 1 | explosive | a signal produced by the firing of explosive charges. |
| 2 | diaphone | a diaphone uses compressed air and generally emits a powerful low-pitched sound, which often concludes with a brief sound of suddenly lowered pitch, termed the "grunt". |
| 3 | siren | a siren uses compressed air and exists in a variety of types which differ considerably in their sound and power. |
| 4 | nautophone | a horn having a diaphragm oscillated by electricity |
| 5 | reed | a reed uses compressed air and emits a weak, high pitched sound. |
| 6 | tyfon | a diaphragm horn which operates under the influence of compressed air or steam |
| 7 | bell | a ringing sound with a short range. |
| 8 | whistle | a distinctive sound made by a jet of air passing through an orifice. |
| 9 | gong | a sound produced by vibration of a disc when struck. |
| 10 | horn | a horn uses compressed air or electricity to vibrate a diaphragm and exists in a variety of types which differ greatly in their sound and power. |

- Attribute: Category of special purpose mark

- Definition:

- Remarks: A mark may be a beacon, a buoy, a signpost or may take another form.

- Code:

- Type: enumeration

- Values:

| Code | Value | Definition |
| --- | --- | --- |
| 1 | firing danger mark | a mark used to indicate a firing danger area, usually at sea. |
| 2 | target mark | any object toward which something is directed. The distinctive marking or instrumentation of a ground point to aid its identification on a photograph. |
| 3 | marker ship mark | a mark marking the position of a ship which is used as a target during some military exercise. |
| 4 | degaussing range mark | a mark used to indicate a degaussing range. |
| 5 | barge mark | a mark of relevance to barges. |
| 6 | cable mark | a mark used to indicate the position of submarine cables or the point at which they run on to the land. |
| 7 | spoil ground mark | a mark used to indicate the limit of a spoil ground |
| 8 | outfall mark | a mark used to indicate the position of an outfall or the point at which it leaves the land. |
| 9 | ODAS | Ocean Data Acquisition System |
| 10 | recording mark | a mark used to record data for scientific purposes. |
| 11 | seaplane anchorage mark | a mark used to indicate a seaplane anchorage. |
| 12 | recreation zone mark | a mark used to indicate a recreation zone. |
| 14 | mooring mark | a mark indicating a mooring or moorings. |
| 15 | LANBY | a large buoy designed to take the place of a lightship where construction of an offshore light station is not feasible. |
| 16 | leading mark | aids to navigation or other indicators so located as to indicate the path to be followed. Leading marks identify a leading line when they are in transit. |
| 17 | measured distance mark | a mark forming part of a transit indicating one end of a measured distance. |
| 18 | notice mark | a notice board or sign indicating information to the mariner. |
| 19 | TSS mark | a mark indicating a traffic separation scheme. |
| 20 | anchoring prohibited mark | a mark indicating an anchoring prohibited area. |
| 21 | berthing prohibited mark | a mark indicating that berthing is prohibited. |
| 22 | overtaking prohibited mark | a mark indicating that overtaking is prohibited. |
| 23 | two-way traffic prohibited mark | a mark indicating a one-way route. |
| 24 | "reduced wake" mark | a mark indicating that vessels must not generate excessive wake. |
| 25 | speed limit mark | a mark indicating that a speed limit applies. |
| 26 | stop mark | a mark indicating the place where the bow of a ship must stop when traffic lights show red. |
| 27 | general warning mark | a mark indicating that special caution must be exercised in the vicinity of the mark. |
| 28 | "sound ships siren" mark | a mark indicating that a ship should sound its siren or horn. |
| 29 | restricted vertical clearance mark | a mark indicating the minimum vertical space available for passage. |
| 30 | maximum vessels draught mark | a mark indicating the maximum draught of vessel permitted. |
| 31 | restricted horizontal clearance mark | a mark indicating the minimum horizontal space available for passage. |
| 32 | strong current warning mark | a mark warning of strong currents. |
| 34 | overhead power cable mark | a mark indicating an overhead power cable. |
| 35 | "channel edge gradient" mark | a mark indicating the gradient of the slope of a dredge channel edge. |
| 36 | telephone mark | a mark indicating the presence of a telephone. |
| 37 | ferry crossing mark | a mark indicating that a ferry route crosses the ship route often used with a "sound ships siren" mark. |
| 39 | pipeline mark | a mark used to indicate the position of submarine pipelines or the point at which they run on to the land. |
| 40 | anchorage mark | a mark indicating an anchorage area. |
| 41 | clearing mark | a mark used to indicate a clearing line. |
| 42 | control mark | a mark indicating the location at which a restriction or requirement exists. |
| 43 | diving mark | a mark indicating that diving may take place in the vicinity. |
| 44 | refuge beacon | a mark providing or indicating a place of safety. |
| 45 | foul ground mark | a mark indicating a foul ground. |
| 46 | yachting mark | a mark installed for use by yachtsmen. |
| 47 | heliport mark | a mark indicating an area where helicopters may land. |
| 48 | GPS mark | a mark indicating a location at which a GPS position has been accurately determined. |
| 49 | seaplane landing mark | a mark indicating an area where sea-planes land. |
| 50 | entry prohibited mark | a mark indicating that entry is prohibited. |
| 51 | work in progress mark | a mark indicating that work (generally construction) is in progress. |
| 52 | mark with unknown purpose | a mark whose detailed characteristics are unknown. |
| 53 | wellhead mark | a mark indicating a borehole that produces or is capable of producing oil or natural gas. |
| 54 | channel separation mark | a mark indicating the point at which a channel divides separately into two channels. |
| 55 | marine farm mark | a mark indicating the existence of a fish, mussel, oyster or pearl farm/ culture. |
| 56 | artificial reef mark | a mark indicating the existence or the extent of an artificial reef. |

- Attribute: Elevation

- Definition: The altitude of the ground level of an object, measured from a specified vertical datum.

- Remarks: The unit of measure is defined in the HUNI subfield of the DSPM record or in the HUNITS attribute of the M_UNIT meta object class, e.g., metre (m). The resolution is 0.1 m or 0.1 ft.

- Code:

- Type: real

- Units: Metre

- Attribute: Fixed mark type

- Definition:

- Remarks:

- Code:

- Type: enumeration

- Values:

| Code | Value | Definition |
| --- | --- | --- |
| 1 | coastal navigation general purpose light |  |
| 2 | east cardinal beacon |  |
| 3 | front lead |  |
| 4 | isolated danger beacon |  |
| 5 | north cardinal beacon |  |
| 6 | port lateral beacon |  |
| 7 | rear lead |  |
| 8 | safe water beacon |  |
| 9 | south cardinal beacon |  |
| 10 | special beacon |  |
| 11 | starboard lateral beacon |  |
| 12 | west cardinal beacon |  |
| 13 | beacon |  |
| 14 | beacon on land |  |
| 15 | buoyant beacon |  |
| 16 | sectored light |  |
| 17 | preferred channel to starboard beacon |  |
| 18 | preferred channel to port beacon |  |

- Attribute: Floating mark type

- Definition:

- Remarks:

- Code:

- Type: enumeration

- Values:

| Code | Value | Definition |
| --- | --- | --- |
| 1 | east cardinal |  |
| 2 | isolated danger |  |
| 3 | light float |  |
| 4 | light vessel |  |
| 5 | mooring |  |
| 6 | north cardinal |  |
| 7 | port lateral |  |
| 8 | safe water |  |
| 9 | south cardinal |  |
| 10 | special |  |
| 11 | starboard lateral |  |
| 12 | visitors mooring |  |
| 13 | west cardinal |  |
| 14 | buoyant beacon |  |
| 15 | emergency wreck marking |  |
| 16 | preferred channel to port |  |
| 17 | preferred channel to starboard |  |

- Attribute: Iala category

- Definition:

- Remarks:

- Code:

- Type: enumeration

- Values:

| Code | Value | Definition |
| --- | --- | --- |
| 1 | category 1 |  |
| 2 | category 2 |  |
| 3 | category 3 |  |

- Attribute: Iala region

- Definition:

- Remarks:

- Code:

- Type: enumeration

- Values:

| Code | Value | Definition |
| --- | --- | --- |
| 1 | A |  |
| 2 | B |  |

- Attribute: Light characteristic

- Definition:

- Remarks: A selection of the above characteristics is defined and illustrated diagrammatically in IHO Chart Specifications, M-4, 471.2.

- Code:

- Type: enumeration

- Values:

| Code | Value | Definition |
| --- | --- | --- |
| 1 | fixed | a signal light that shows continuously, in any given direction, with constant luminous intensity and colour. |
| 2 | flashing | a rhythmic light in which the total duration of light in a period is clearly shorter than the total duration of darkness and all the appearances of light are of equal duration. |
| 3 | long-flashing | a flashing light in which a single flash of not less than two seconds duration is regularly repeated. |
| 4 | quick-flashing | a light exhibiting without interruption very rapid regular alternations of light and darkness. |
| 5 | very quick flashing | a flashing light in which flashes are repeated at a rate of not less than 80 flashes per minute but less than 160 flashes per minute. |
| 6 | ultra quick flashing | a flashing light in which flashes are repeated at a rate of not less than 160 flashes per minute. |
| 7 | isophased | a light with all durations of light and darkness equal. |
| 8 | occulting | a rhythmic light in which the total duration of light in a period is clearly longer than the total duration of darkness and all the eclipses are of equal duration. |
| 9 | interrupted quick flashing | a quick light in which the sequence of flashes is interrupted by regularly repeated eclipses of constant and long duration. |
| 10 | interrupted very quick flashing | a light in which the very rapid alterations of light and darkness are interrupted at regular intervals by eclipses of long duration. |
| 11 | interrupted ultra quick flashing | a light in which the ultra quick flashes (160 or more per minute) are interrupted at regular intervals by eclipses of long duration. |
| 12 | morse | a rhythmic light in which appearances of light of two clearly different durations are grouped to represent a character or characters in the Morse code. |
| 13 | fixed/flash |  |
| 14 | flash/long-flash |  |
| 15 | occulting/flash |  |
| 16 | fixed/long-flash |  |
| 17 | occulting alternating |  |
| 18 | long-flash alternating |  |
| 19 | flash alternating |  |
| 20 | group alternating |  |
| 25 | quick-flash plus long-flash |  |
| 26 | very quick-flash plus long-flash |  |
| 27 | ultra quick-flash plus long-flash |  |
| 28 | alternating | a signal light that shows, in any given direction, two or more colours in a regularly repeated sequence with a regular periodicity. |
| 29 | fixed and alternating flashing |  |

- Attribute: Light colour

- Definition: The colour of a navigational light.

- Remarks:

- Code: lightColour

- Type: enumeration

- Values:

| Code | Value | Definition |
| --- | --- | --- |
| 1 | white |  |
| 2 | red |  |
| 3 | green |  |
| 4 | yellow |  |
| 5 | blue |  |

- Attribute: Light description

- Definition:

- Remarks:

- Code:

- Type: text

- Attribute: Light visibility

- Definition: The specific visibility of a light, with respect to the lights intensity and ease of recognition.

- Remarks:

- Code:

- Type: enumeration

- Values:

| Code | Value | Definition |
| --- | --- | --- |
| 1 | high intensity | non-marine lights with a higher power than marine lights and visible from well off shore (often "Aero" lights). |
| 2 | low intensity | non-marine lights with lower power than marine lights. |
| 3 | faint | a decrease in the apparent intensity of a light which may occur in the case of partial obstructions. |
| 4 | intensified | a light in a sector is intensified (i.e. has longer range than other sectors). |
| 5 | unintensified | a light in a sector is unintensified (i.e. has shorter range than other sectors). |

- Attribute: Mark colour

- Definition:

- Remarks:

- Code:

- Type: enumeration

- Values:

| Code | Value | Definition |
| --- | --- | --- |
| 1 | black |  |
| 2 | blue |  |
| 3 | green |  |
| 4 | orange |  |
| 5 | red |  |
| 6 | white |  |
| 7 | yellow |  |

- Attribute: MMSI number

- Definition:

- Remarks:

- Code:

- Type: integer

- Attribute: Nominal light range

- Definition: The nominal range at which an object can be seen or a signal detected.

- Remarks:

- Code:

- Type: real

- Attribute: Period of operation

- Definition: The times at which equipment is operational.

- Remarks:

- Code:

- Type: enumeration

- Values:

| Code | Value | Definition |
| --- | --- | --- |
| 1 | day |  |
| 2 | night |  |
| 3 | occasional |  |
| 4 | 24 hours |  |
| 5 | fog |  |

- Attribute: Shape

- Definition:

- Remarks:

- Code:

- Type: enumeration

- Values:

| Code | Value | Definition |
| --- | --- | --- |
| 1 | barrel |  |
| 2 | can |  |
| 3 | conical |  |
| 4 | major pile |  |
| 5 | minor pile |  |
| 6 | multiple |  |
| 7 | pillar |  |
| 8 | spar/spindle |  |
| 9 | spherical |  |
| 10 | super-buoy |  |
| 11 | tower |  |
| 12 | tripodal |  |

- Attribute: Surveyed by

- Definition:

- Remarks:

- Code:

- Type: text

- Attribute: Topmark colour

- Definition:

- Remarks:

- Code:

- Type: enumeration

- Values:

| Code | Value | Definition |
| --- | --- | --- |
| 1 | black |  |
| 2 | white |  |
| 3 | yellow |  |
| 4 | green |  |
| 5 | orange |  |
| 6 | red |  |

- Attribute: Topmark/daymark shape

- Definition:

- Remarks:

- Code:

- Type: enumeration

- Values:

| Code | Value | Definition |
| --- | --- | --- |
| 1 | cone, point up | is where the vertex points up. A cone is a solid figure generated by straight lines drawn from a fixed point (the vertex) to a circle in a plane not containing the vertex. Cones are commonly used as International Association of Lighthouse Authorities - IALA topmarks lateral. |
| 2 | cone, point down | is where the vertex points down. A cone is a solid figure generated by straight lines drawn from a fixed point (the vertex) to a circle in a plane not containing the vertex. Cones are commonly used as International Association of Lighthouse Authorities - IALA topmarks lateral. |
| 3 | sphere | a body the surface of which is at all points equidistant from the centre. Spheres are commonly used as International Association of Lighthouse Authorities - IALA topmarks safe water. |
| 4 | 2 spheres | two black spheres are commonly used as an International Association of Lighthouse Authorities - IALA topmark (isolated danger). |
| 5 | cylinder (can) | a solid geometrical figure generated by straight lines fixed in direction and describing with one of point a closed curve, especially a circle (in which case the figure is circular cylinder, its ends being parallel circles). Cylinders are commonly used as International Association of Lighthouse Authorities - IALA topmarks lateral. |
| 6 | board | usually of rectangular shape, made from timber or metal and used to provide a contrast with the natural background of a daymark. The actual daymark is often painted on to this board. |
| 7 | x-shape (St. Andrews cross) | having a shape or a cross-section like the capital letter X. An x-shape as an International Association of Lighthouse Authorities - IALA topmark should be 3 dimensional in shape. It is made of at least three crossed bars. |
| 8 | upright cross (St. Georges Cross) | a cross with one vertical member and one horizontal member, i.e. similar in shape to the character "+". |
| 9 | cube, point up | a cube standing on one of its vertexes. A cube is a solid contained by six equal squares a regular hexahedron (The New Shorter Oxford English Dictionary. 1993. vol 2) |
| 10 | 2 cones, point to point | 2 cones, one above the other, with their vertices together in the centre. |
| 11 | 2 cones, base to base | 2 cones, one above the other, with their bases together in the centre and their vertices pointing up and down. |
| 12 | rhombus | a plane figure having four equal sides and equal opposite angles (two acute and two obtuse) an oblique equilateral parallelogram. |
| 13 | 2 cones (points upward) |  |
| 14 | 2 cones (points downward) |  |
| 15 | besom, point up (broom or perch) | besom: a bundle of rods or twigs. perch: a staff placed on top of a buoy, rock or shoal as a mark for navigation. |
| 16 | besom, point down (broom or perch) | besom: a bundle of rods or twigs. perch: a staff placed on top of a buoy, rock or shoal as a mark for navigation. |
| 17 | flag | a flag mounted on a short pole. |
| 18 | sphere over rhombus | A sphere located above a rhombus. |
| 19 | square | a plane figure with four right angles and four equal straight sides |
| 20 | horizontal rectangle | where the two longer opposite sides are standing horizontally. A rectangle is a plane figure with four right angles and four straight sides, opposite sides being parallel and equal in length |
| 21 | vertical rectangle | where the two longer opposite sides are standing vertically. A rectangle is a plane figure with four right angles and four straight sides, opposite sides being parallel and equal in length |
| 22 | trapezium, up | which stands on its longer parallel side. A trapezium is a quadrilateral having one pair of opposite sides parallel. |
| 23 | trapezium, down | which stands on its shorter parallel side. A trapezium is a quadrilateral having one pair of opposite sides parallel. |
| 24 | triangle, point up | A triangle, point up. A triangle is a figure having three angles and three sides. |
| 25 | triangle, point down | A triangle, point down. A triangle is a figure having three angles and three sides. |
| 26 | circle | a perfectly round plane figure whose circumference is everywhere equidistant from its centre. |
| 27 | two upright crosses (one over the other) | two upright crosses, generally vertically disposed one above the other. |
| 28 | T-shape | having a shape like the capital letter T. |
| 29 | triangle pointing up over a circle | a triangle, vertex uppermost, located above a circle. |
| 30 | upright cross over a circle | an upright cross located above a circle. |
| 31 | rhombus over a circle | a rhombus located above a circle. |
| 32 | circle over a triangle pointing up | a circle located over a triangle, vertex uppermost. |

- Attribute: Type of structure

- Definition:

- Remarks:

- Code:

- Type: enumeration

- Values:

| Code | Value | Definition |
| --- | --- | --- |
| 1 | beacon |  |
| 2 | hut |  |
| 3 | lighthouse |  |
| 4 | tower - hex |  |
| 5 | tower - lattice |  |
| 6 | tower - masonry |  |
| 7 | tower with hut |  |
| 8 | offshore platform |  |
| 9 | wind turbine |  |

- Attribute: Vertical datum

- Definition: The datum to which both heights (vertical datum, see S-57 Part 3) and soundings (sounding datum, see S-57 Part 3) are referred.

- Remarks: When the vertical datum is unknown, such as water areas above locks, the value "local datum" is to be used, and further details may be encoded using "INFORM". The +0.3m approximation quoted in the "approximate" levels is somehow arbitrary and follows the British example of their definition for "approximate LAT".

- Code:

- Type: enumeration

- Values:

| Code | Value | Definition |
| --- | --- | --- |
| 1 | mean low water springs | (MLWS) - the average height of the low waters of spring tides. |
| 2 | mean lower low water springs | (MLLWS) - the average height of lower low water springs at a place. |
| 3 | mean sea level | (MSL) - the average height of the surface of the sea at a tide station for all stages of the tide over a 19-year period, usually determined from hourly height readings measured from a fixed predetermined reference level. |
| 4 | lowest low water | an arbitrary level conforming to the lowest tide observed at a place, or some what lower. |
| 5 | mean low water | (MLW) - the average height of all low waters at a place over a 19-year period. |
| 6 | lowest low water springs | an arbitrary level conforming to the lowest water level observed at a place at spring tides during a period of time shorter than 19 years. |
| 7 | approximate mean low water springs | an arbitrary level, usually within 0.3m from that of mean low water springs (MLWS). |
| 8 | Indian spring low water | (ISLW) - an arbitrary tidal datum approximating the level of the mean of the lower low water at spring tides. |
| 9 | low water springs | an arbitrary level, approximating that of mean low water springs (MLWS). |
| 10 | approximate lowest astronomical tide | an arbitrary level, usually within 0.3m from that of lowest astronomical tide (LAT). |
| 11 | nearly lowest low water | an arbitrary level approximating the lowest water level observed at a place, usually equivalent to the Indian spring low water (ISLW). |
| 12 | mean lower low water | (MLLW) - the average height of the lower low waters at a place over a 19-year period. |
| 13 | low water | an approximation of mean low water adopted as the reference level for a limited area, irrespective of better determinations at a later date. |
| 14 | approximate mean low water | an arbitrary level, usually within 0.3m from that of mean low water (MLW). |
| 15 | approximate mean lower low water | an arbitrary level, usually within 0.3m from that of mean lower low water (MLLW). |
| 16 | mean high water | (MHW) - the average height of all high waters at a place over a 19-year period. |
| 17 | mean high water springs | (MHWS) - the average height of the high waters of spring tides. |
| 18 | high water | the highest level reached at a place by the water surface in one tidal cycle. |
| 19 | approximate mean sea level | an arbitrary level, usually within 0.3m from that of mean sea level (MSL). |
| 20 | high water springs | an arbitrary level, approximating that of mean high water springs (MHWS). |
| 21 | mean higher high water | (MHHW) - the average height of higher high waters at a place over a 19-year period. |
| 22 | equinoctial spring low water | the level of low water springs near the time of an equinox. |
| 23 | lowest astronomical tide | (LAT) - the lowest tide level which can be predicted to occur under average meterological conditions and under any combination of astronomical conditions. |
| 24 | local datum | an arbitrary datum defined by a local harbour authority, from which levels and tidal heights are measured by this authority. |
| 25 | international great lakes datum 1985 | (IGLD 1985) - a vertical reference system with its zero based on the mean water level at Rimouski/Pointe-au-P??re, Quebec, over the period 1970 to 1988. |
| 26 | mean water level | the average of all hourly water levels over the available period of record. |
| 27 | lower low water large tide | (LLWLT) - the average of the lowest low waters, one from each of 19 years of observations. |
| 28 | higher high water large tide | (HHWLT) - the average of the highest high waters, one from each of 19 years of observations. |
| 29 | nearly highest high water | an arbitrary level approximating the highest water level observed at a place, usually equivalent to the high water springs. |
| 30 | highest astronomical tide (HAT) | the highest tidal level which can be predicted to occur under average meteorological conditions and under any combination of astronomical conditions. |

- Complex Attributes

Top

- Attribute: Daymark topmarkDaymark topmark

- Code:

- Defintion:

| Sub Attributes | Values | Multiplicity |
| --- | --- | --- |
|  |  | 1,1 |
|  |  | 1,0 |

- Association Roles

Top

- Feature Associations

Top

- Features

Top

- Feature: AIS Aid to Navigation

- Code: AISAtoN

- Defintion:

- Geometric Primitives:

| Attribute | Values | Multiplicity |
| --- | --- | --- |
|  |  | 1,1 |
|  |  | 1,1 |
|  |  | 0,1 |
|  |  | 1,1 |

- Feature: Directional light

- Code: DirectionalLight

- Defintion: A light illuminating a sector of a very narrow angle and intended to mark a direction to follow.

- Geometric Primitives:

| Attribute | Values | Multiplicity |
| --- | --- | --- |
|  |  | 0,1 |
| lightColour |  | 1,1 |
|  |  | 1,1 |
|  |  | 0,1 |
|  |  | 0,1 |
|  |  | 0,1 |
|  |  | 0,1 |
|  |  | 1,1 |

- Feature: Fixed AtoN

- Code: FixedAtoN

- Defintion:

- Geometric Primitives:

| Attribute | Values | Multiplicity |
| --- | --- | --- |
|  |  | 0,1 |
|  |  | 1,1 |
|  |  | 0,1 |
|  |  | 0,1 |

- Feature: Floating AtoN

- Code: FloatgingAtoN

- Defintion:

- Geometric Primitives:

| Attribute | Values | Multiplicity |
| --- | --- | --- |
|  |  | 1,1 |

- Feature: Fog detector light

- Code: FogDetectorLight

- Defintion: A light used to automatically determine the conditions of visibility which warrant the operation of a sound signal.

- Geometric Primitives:

| Attribute | Values | Multiplicity |
| --- | --- | --- |
|  |  | 0,1 |
| lightColour |  | 0,1 |
|  |  | 0,1 |
|  |  | 1,1 |

- Feature: Fog signal

- Code:

- Defintion: A warning signal transmitted by a vessel, or aid to navigation, during periods of low visibility. Also, the device producing such a signal.

- Geometric Primitives:

| Attribute | Values | Multiplicity |
| --- | --- | --- |
|  |  | 0,1 |
|  |  | 0,1 |
|  |  | 0,1 |

- Feature: Light

- Code:

- Defintion: A luminous or lighted aid to navigation.

- Geometric Primitives: point

| Attribute | Values | Multiplicity |
| --- | --- | --- |
| lightColour |  | 1, |
|  |  | 0,1 |
|  |  | 0,1 |
|  |  | 0,1 |
|  |  | 1,1 |
|  |  | 1,1 |

- Feature: Radar transponder beacon

- Code:

- Defintion: A transponder beacon transmitting a coded signal on radar frequency, permitting an interrogating craft to determine the bearing and range of the transponder.

- Geometric Primitives:

| Attribute | Values | Multiplicity |
| --- | --- | --- |
|  |  | 0,1 |
|  |  | 0,1 |

- Feature: Sectored light

- Code: SectoredLight

- Defintion: A sectored light is a light having one or more sectors, which have different characteristics acfross and sometimes within each sector.

- Geometric Primitives:

| Attribute | Values | Multiplicity |
| --- | --- | --- |
|  |  | 0,1 |
|  |  | 0,1 |
|  |  | 1,1 |