ANNEX 3

<?xml version="1.0" encoding="iso-8859-1" ?>

- <fc:Catalog xmlns:fc="http://www.iho.int/S100FC" xmlns:s100Base="http://www.iho.int/S100Base" xmlns:s100CI="http://www.iho.int/S100CI" xmlns:xlink="http://www.w3.org/1999/xlink" xmlns:s100FD="http://www.iho.int/S100FD" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xsi:schemaLocation="http://www.iho.int/S100FC S100FC.xsd">

- <fc:catalogInfo>

<fc:name>AtoN PS</fc:name>

<fc:scope>global</fc:scope>

<fc:fieldOfApplication>Aids to Navigation</fc:fieldOfApplication>

<fc:versionNumber>0.1</fc:versionNumber>

<fc:versionDate>2013-09-12</fc:versionDate>

- <fc:producer>

<s100CI:individualName>Tom Richardson</s100CI:individualName>

<s100CI:organisationName>IALA</s100CI:organisationName>

<s100CI:positionName />

<s100CI:contactInfo />

<s100CI:role>originator</s100CI:role>

</fc:producer>

</fc:catalogInfo>

- <fc:simpleAttributes>

- <fc:simpleAttribute>

<fc:name>AIS Aid to Navigation type</fc:name>

<fc:definition>The purpose of an AIS Aid to Navigation.</fc:definition>

<fc:code />

<fc:numericCode />

<fc:remarks />

<fc:dataType>enumeration</fc:dataType>

- <fc:listedValue>

<fc:label>Default, Type of AtoN not specified</fc:label>

<fc:definition />

<fc:code>1</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Reference point</fc:label>

<fc:definition />

<fc:code>2</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>RACON</fc:label>

<fc:definition />

<fc:code>3</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Fixed structure off shore</fc:label>

<fc:definition />

<fc:code>4</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Spare</fc:label>

<fc:definition />

<fc:code>5</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Light without sectors</fc:label>

<fc:definition />

<fc:code>6</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Light with sectors</fc:label>

<fc:definition />

<fc:code>7</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Leading Light Front</fc:label>

<fc:definition />

<fc:code>8</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Leading Light Rear</fc:label>

<fc:definition />

<fc:code>9</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Beacon, Cardinal N</fc:label>

<fc:definition />

<fc:code>10</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Beacon, Cardinal E</fc:label>

<fc:definition />

<fc:code>11</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Beacon, Cardinal S</fc:label>

<fc:definition />

<fc:code>12</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Beacon, Cardinal W</fc:label>

<fc:definition />

<fc:code>13</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Beacon, Port Hand</fc:label>

<fc:definition />

<fc:code>14</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Beacon, Starboard hand</fc:label>

<fc:definition />

<fc:code>15</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Beacon, Preferred Channel port hand</fc:label>

<fc:definition />

<fc:code>16</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Beacon, Preferred Channel starboard hand</fc:label>

<fc:definition />

<fc:code>17</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Beacon, Isolated danger</fc:label>

<fc:definition />

<fc:code>18</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Beacon, Safe water</fc:label>

<fc:definition />

<fc:code>19</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Beacon, Special mark</fc:label>

<fc:definition />

<fc:code>20</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Cardinal Mark N</fc:label>

<fc:definition />

<fc:code>21</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Cardinal Mark E</fc:label>

<fc:definition />

<fc:code>22</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Cardinal Mark S</fc:label>

<fc:definition />

<fc:code>23</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Cardinal Mark W</fc:label>

<fc:definition />

<fc:code>24</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Port hand Mark</fc:label>

<fc:definition />

<fc:code>25</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>starboard hand Mark</fc:label>

<fc:definition />

<fc:code>26</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Preferred Channel Port Hand</fc:label>

<fc:definition />

<fc:code>27</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Preferred Channel Starboard hand</fc:label>

<fc:definition />

<fc:code>28</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Isolated danger</fc:label>

<fc:definition />

<fc:code>29</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Safe water</fc:label>

<fc:definition />

<fc:code>30</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Special mark</fc:label>

<fc:definition />

<fc:code>31</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Light Vessel / LANBY / Rig</fc:label>

<fc:definition />

<fc:code>32</fc:code>

</fc:listedValue>

</fc:simpleAttribute>

- <fc:simpleAttribute>

<fc:name>AIS Tx Rx</fc:name>

<fc:definition />

<fc:code />

<fc:numericCode />

<fc:remarks />

<fc:dataType>enumeration</fc:dataType>

- <fc:listedValue>

<fc:label>Tr/Rx</fc:label>

<fc:definition />

<fc:code>1</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Rx</fc:label>

<fc:definition />

<fc:code>2</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Tx</fc:label>

<fc:definition />

<fc:code>3</fc:code>

</fc:listedValue>

</fc:simpleAttribute>

- <fc:simpleAttribute>

<fc:name>Category of fog signal</fc:name>

<fc:definition>The various means of generating the fog signal.</fc:definition>

<fc:code />

<fc:numericCode>27</fc:numericCode>

<fc:remarks>The classification "horn" is the generic term for fog signals "nautophone", "reed" and "tyfon".</fc:remarks>

<fc:dataType>enumeration</fc:dataType>

- <fc:listedValue>

<fc:label>explosive</fc:label>

<fc:definition>a signal produced by the firing of explosive charges.</fc:definition>

<fc:code>1</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>diaphone</fc:label>

<fc:definition>a diaphone uses compressed air and generally emits a powerful low-pitched sound, which often concludes with a brief sound of suddenly lowered pitch, termed the "grunt".</fc:definition>

<fc:code>2</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>siren</fc:label>

<fc:definition>a siren uses compressed air and exists in a variety of types which differ considerably in their sound and power.</fc:definition>

<fc:code>3</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>nautophone</fc:label>

<fc:definition>a horn having a diaphragm oscillated by electricity</fc:definition>

<fc:code>4</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>reed</fc:label>

<fc:definition>a reed uses compressed air and emits a weak, high pitched sound.</fc:definition>

<fc:code>5</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>tyfon</fc:label>

<fc:definition>a diaphragm horn which operates under the influence of compressed air or steam</fc:definition>

<fc:code>6</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>bell</fc:label>

<fc:definition>a ringing sound with a short range.</fc:definition>

<fc:code>7</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>whistle</fc:label>

<fc:definition>a distinctive sound made by a jet of air passing through an orifice.</fc:definition>

<fc:code>8</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>gong</fc:label>

<fc:definition>a sound produced by vibration of a disc when struck.</fc:definition>

<fc:code>9</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>horn</fc:label>

<fc:definition>a horn uses compressed air or electricity to vibrate a diaphragm and exists in a variety of types which differ greatly in their sound and power.</fc:definition>

<fc:code>10</fc:code>

</fc:listedValue>

</fc:simpleAttribute>

- <fc:simpleAttribute>

<fc:name>Category of special purpose mark</fc:name>

<fc:definition />

<fc:code />

<fc:numericCode>66</fc:numericCode>

<fc:remarks>A mark may be a beacon, a buoy, a signpost or may take another form.</fc:remarks>

<fc:dataType>enumeration</fc:dataType>

- <fc:listedValue>

<fc:label>firing danger mark</fc:label>

<fc:definition>a mark used to indicate a firing danger area, usually at sea.</fc:definition>

<fc:code>1</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>target mark</fc:label>

<fc:definition>any object toward which something is directed. The distinctive marking or instrumentation of a ground point to aid its identification on a photograph.</fc:definition>

<fc:code>2</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>marker ship mark</fc:label>

<fc:definition>a mark marking the position of a ship which is used as a target during some military exercise.</fc:definition>

<fc:code>3</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>degaussing range mark</fc:label>

<fc:definition>a mark used to indicate a degaussing range.</fc:definition>

<fc:code>4</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>barge mark</fc:label>

<fc:definition>a mark of relevance to barges.</fc:definition>

<fc:code>5</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>cable mark</fc:label>

<fc:definition>a mark used to indicate the position of submarine cables or the point at which they run on to the land.</fc:definition>

<fc:code>6</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>spoil ground mark</fc:label>

<fc:definition>a mark used to indicate the limit of a spoil ground</fc:definition>

<fc:code>7</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>outfall mark</fc:label>

<fc:definition>a mark used to indicate the position of an outfall or the point at which it leaves the land.</fc:definition>

<fc:code>8</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>ODAS</fc:label>

<fc:definition>Ocean Data Acquisition System</fc:definition>

<fc:code>9</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>recording mark</fc:label>

<fc:definition>a mark used to record data for scientific purposes.</fc:definition>

<fc:code>10</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>seaplane anchorage mark</fc:label>

<fc:definition>a mark used to indicate a seaplane anchorage.</fc:definition>

<fc:code>11</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>recreation zone mark</fc:label>

<fc:definition>a mark used to indicate a recreation zone.</fc:definition>

<fc:code>12</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>mooring mark</fc:label>

<fc:definition>a mark indicating a mooring or moorings.</fc:definition>

<fc:code>14</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>LANBY</fc:label>

<fc:definition>a large buoy designed to take the place of a lightship where construction of an offshore light station is not feasible.</fc:definition>

<fc:code>15</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>leading mark</fc:label>

<fc:definition>aids to navigation or other indicators so located as to indicate the path to be followed. Leading marks identify a leading line when they are in transit.</fc:definition>

<fc:code>16</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>measured distance mark</fc:label>

<fc:definition>a mark forming part of a transit indicating one end of a measured distance.</fc:definition>

<fc:code>17</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>notice mark</fc:label>

<fc:definition>a notice board or sign indicating information to the mariner.</fc:definition>

<fc:code>18</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>TSS mark</fc:label>

<fc:definition>a mark indicating a traffic separation scheme.</fc:definition>

<fc:code>19</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>anchoring prohibited mark</fc:label>

<fc:definition>a mark indicating an anchoring prohibited area.</fc:definition>

<fc:code>20</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>berthing prohibited mark</fc:label>

<fc:definition>a mark indicating that berthing is prohibited.</fc:definition>

<fc:code>21</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>overtaking prohibited mark</fc:label>

<fc:definition>a mark indicating that overtaking is prohibited.</fc:definition>

<fc:code>22</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>two-way traffic prohibited mark</fc:label>

<fc:definition>a mark indicating a one-way route.</fc:definition>

<fc:code>23</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>"reduced wake" mark</fc:label>

<fc:definition>a mark indicating that vessels must not generate excessive wake.</fc:definition>

<fc:code>24</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>speed limit mark</fc:label>

<fc:definition>a mark indicating that a speed limit applies.</fc:definition>

<fc:code>25</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>stop mark</fc:label>

<fc:definition>a mark indicating the place where the bow of a ship must stop when traffic lights show red.</fc:definition>

<fc:code>26</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>general warning mark</fc:label>

<fc:definition>a mark indicating that special caution must be exercised in the vicinity of the mark.</fc:definition>

<fc:code>27</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>"sound ships siren" mark</fc:label>

<fc:definition>a mark indicating that a ship should sound its siren or horn.</fc:definition>

<fc:code>28</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>restricted vertical clearance mark</fc:label>

<fc:definition>a mark indicating the minimum vertical space available for passage.</fc:definition>

<fc:code>29</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>maximum vessels draught mark</fc:label>

<fc:definition>a mark indicating the maximum draught of vessel permitted.</fc:definition>

<fc:code>30</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>restricted horizontal clearance mark</fc:label>

<fc:definition>a mark indicating the minimum horizontal space available for passage.</fc:definition>

<fc:code>31</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>strong current warning mark</fc:label>

<fc:definition>a mark warning of strong currents.</fc:definition>

<fc:code>32</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>overhead power cable mark</fc:label>

<fc:definition>a mark indicating an overhead power cable.</fc:definition>

<fc:code>34</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>"channel edge gradient" mark</fc:label>

<fc:definition>a mark indicating the gradient of the slope of a dredge channel edge.</fc:definition>

<fc:code>35</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>telephone mark</fc:label>

<fc:definition>a mark indicating the presence of a telephone.</fc:definition>

<fc:code>36</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>ferry crossing mark</fc:label>

<fc:definition>a mark indicating that a ferry route crosses the ship route often used with a "sound ships siren" mark.</fc:definition>

<fc:code>37</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>pipeline mark</fc:label>

<fc:definition>a mark used to indicate the position of submarine pipelines or the point at which they run on to the land.</fc:definition>

<fc:code>39</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>anchorage mark</fc:label>

<fc:definition>a mark indicating an anchorage area.</fc:definition>

<fc:code>40</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>clearing mark</fc:label>

<fc:definition>a mark used to indicate a clearing line.</fc:definition>

<fc:code>41</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>control mark</fc:label>

<fc:definition>a mark indicating the location at which a restriction or requirement exists.</fc:definition>

<fc:code>42</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>diving mark</fc:label>

<fc:definition>a mark indicating that diving may take place in the vicinity.</fc:definition>

<fc:code>43</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>refuge beacon</fc:label>

<fc:definition>a mark providing or indicating a place of safety.</fc:definition>

<fc:code>44</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>foul ground mark</fc:label>

<fc:definition>a mark indicating a foul ground.</fc:definition>

<fc:code>45</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>yachting mark</fc:label>

<fc:definition>a mark installed for use by yachtsmen.</fc:definition>

<fc:code>46</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>heliport mark</fc:label>

<fc:definition>a mark indicating an area where helicopters may land.</fc:definition>

<fc:code>47</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>GPS mark</fc:label>

<fc:definition>a mark indicating a location at which a GPS position has been accurately determined.</fc:definition>

<fc:code>48</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>seaplane landing mark</fc:label>

<fc:definition>a mark indicating an area where sea-planes land.</fc:definition>

<fc:code>49</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>entry prohibited mark</fc:label>

<fc:definition>a mark indicating that entry is prohibited.</fc:definition>

<fc:code>50</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>work in progress mark</fc:label>

<fc:definition>a mark indicating that work (generally construction) is in progress.</fc:definition>

<fc:code>51</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>mark with unknown purpose</fc:label>

<fc:definition>a mark whose detailed characteristics are unknown.</fc:definition>

<fc:code>52</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>wellhead mark</fc:label>

<fc:definition>a mark indicating a borehole that produces or is capable of producing oil or natural gas.</fc:definition>

<fc:code>53</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>channel separation mark</fc:label>

<fc:definition>a mark indicating the point at which a channel divides separately into two channels.</fc:definition>

<fc:code>54</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>marine farm mark</fc:label>

<fc:definition>a mark indicating the existence of a fish, mussel, oyster or pearl farm/ culture.</fc:definition>

<fc:code>55</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>artificial reef mark</fc:label>

<fc:definition>a mark indicating the existence or the extent of an artificial reef.</fc:definition>

<fc:code>56</fc:code>

</fc:listedValue>

</fc:simpleAttribute>

- <fc:simpleAttribute>

<fc:name>Elevation</fc:name>

<fc:definition>The altitude of the ground level of an object, measured from a specified vertical datum.</fc:definition>

<fc:code />

<fc:numericCode>90</fc:numericCode>

<fc:remarks>The unit of measure is defined in the HUNI subfield of the DSPM record or in the HUNITS attribute of the M_UNIT meta object class, e.g., metre (m). The resolution is 0.1 m or 0.1 ft.</fc:remarks>

<fc:dataType>real</fc:dataType>

- <fc:uom>

<s100Base:name>Metre</s100Base:name>

<s100Base:definition>The fundamental unit of length in the metric system.</s100Base:definition>

<s100Base:symbol />

</fc:uom>

</fc:simpleAttribute>

- <fc:simpleAttribute>

<fc:name>Fixed mark type</fc:name>

<fc:definition />

<fc:code />

<fc:numericCode />

<fc:remarks />

<fc:dataType>enumeration</fc:dataType>

- <fc:listedValue>

<fc:label>coastal navigation general purpose light</fc:label>

<fc:definition />

<fc:code>1</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>east cardinal beacon</fc:label>

<fc:definition />

<fc:code>2</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>front lead</fc:label>

<fc:definition />

<fc:code>3</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>isolated danger beacon</fc:label>

<fc:definition />

<fc:code>4</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>north cardinal beacon</fc:label>

<fc:definition />

<fc:code>5</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>port lateral beacon</fc:label>

<fc:definition />

<fc:code>6</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>rear lead</fc:label>

<fc:definition />

<fc:code>7</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>safe water beacon</fc:label>

<fc:definition />

<fc:code>8</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>south cardinal beacon</fc:label>

<fc:definition />

<fc:code>9</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>special beacon</fc:label>

<fc:definition />

<fc:code>10</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>starboard lateral beacon</fc:label>

<fc:definition />

<fc:code>11</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>west cardinal beacon</fc:label>

<fc:definition />

<fc:code>12</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>beacon</fc:label>

<fc:definition />

<fc:code>13</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>beacon on land</fc:label>

<fc:definition />

<fc:code>14</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>buoyant beacon</fc:label>

<fc:definition />

<fc:code>15</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>sectored light</fc:label>

<fc:definition />

<fc:code>16</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>preferred channel to starboard beacon</fc:label>

<fc:definition />

<fc:code>17</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>preferred channel to port beacon</fc:label>

<fc:definition />

<fc:code>18</fc:code>

</fc:listedValue>

</fc:simpleAttribute>

- <fc:simpleAttribute>

<fc:name>Floating mark type</fc:name>

<fc:definition />

<fc:code />

<fc:numericCode />

<fc:remarks />

<fc:dataType>enumeration</fc:dataType>

- <fc:listedValue>

<fc:label>east cardinal</fc:label>

<fc:definition />

<fc:code>1</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>isolated danger</fc:label>

<fc:definition />

<fc:code>2</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>light float</fc:label>

<fc:definition />

<fc:code>3</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>light vessel</fc:label>

<fc:definition />

<fc:code>4</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>mooring</fc:label>

<fc:definition />

<fc:code>5</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>north cardinal</fc:label>

<fc:definition />

<fc:code>6</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>port lateral</fc:label>

<fc:definition />

<fc:code>7</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>safe water</fc:label>

<fc:definition />

<fc:code>8</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>south cardinal</fc:label>

<fc:definition />

<fc:code>9</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>special</fc:label>

<fc:definition />

<fc:code>10</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>starboard lateral</fc:label>

<fc:definition />

<fc:code>11</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>visitors mooring</fc:label>

<fc:definition />

<fc:code>12</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>west cardinal</fc:label>

<fc:definition />

<fc:code>13</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>buoyant beacon</fc:label>

<fc:definition />

<fc:code>14</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>emergency wreck marking</fc:label>

<fc:definition />

<fc:code>15</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>preferred channel to port</fc:label>

<fc:definition />

<fc:code>16</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>preferred channel to starboard</fc:label>

<fc:definition />

<fc:code>17</fc:code>

</fc:listedValue>

</fc:simpleAttribute>

- <fc:simpleAttribute>

<fc:name>Iala category</fc:name>

<fc:definition />

<fc:code />

<fc:numericCode />

<fc:remarks />

<fc:dataType>enumeration</fc:dataType>

- <fc:listedValue>

<fc:label>category 1</fc:label>

<fc:definition />

<fc:code>1</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>category 2</fc:label>

<fc:definition />

<fc:code>2</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>category 3</fc:label>

<fc:definition />

<fc:code>3</fc:code>

</fc:listedValue>

</fc:simpleAttribute>

- <fc:simpleAttribute>

<fc:name>Iala region</fc:name>

<fc:definition />

<fc:code />

<fc:numericCode />

<fc:remarks />

<fc:dataType>enumeration</fc:dataType>

- <fc:listedValue>

<fc:label>A</fc:label>

<fc:definition />

<fc:code>1</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>B</fc:label>

<fc:definition />

<fc:code>2</fc:code>

</fc:listedValue>

</fc:simpleAttribute>

- <fc:simpleAttribute>

<fc:name>Light characteristic</fc:name>

<fc:definition />

<fc:code />

<fc:numericCode>107</fc:numericCode>

<fc:remarks>A selection of the above characteristics is defined and illustrated diagrammatically in IHO Chart Specifications, M-4, 471.2.</fc:remarks>

<fc:dataType>enumeration</fc:dataType>

- <fc:listedValue>

<fc:label>fixed</fc:label>

<fc:definition>a signal light that shows continuously, in any given direction, with constant luminous intensity and colour.</fc:definition>

<fc:code>1</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>flashing</fc:label>

<fc:definition>a rhythmic light in which the total duration of light in a period is clearly shorter than the total duration of darkness and all the appearances of light are of equal duration.</fc:definition>

<fc:code>2</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>long-flashing</fc:label>

<fc:definition>a flashing light in which a single flash of not less than two seconds duration is regularly repeated.</fc:definition>

<fc:code>3</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>quick-flashing</fc:label>

<fc:definition>a light exhibiting without interruption very rapid regular alternations of light and darkness.</fc:definition>

<fc:code>4</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>very quick flashing</fc:label>

<fc:definition>a flashing light in which flashes are repeated at a rate of not less than 80 flashes per minute but less than 160 flashes per minute.</fc:definition>

<fc:code>5</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>ultra quick flashing</fc:label>

<fc:definition>a flashing light in which flashes are repeated at a rate of not less than 160 flashes per minute.</fc:definition>

<fc:code>6</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>isophased</fc:label>

<fc:definition>a light with all durations of light and darkness equal.</fc:definition>

<fc:code>7</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>occulting</fc:label>

<fc:definition>a rhythmic light in which the total duration of light in a period is clearly longer than the total duration of darkness and all the eclipses are of equal duration.</fc:definition>

<fc:code>8</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>interrupted quick flashing</fc:label>

<fc:definition>a quick light in which the sequence of flashes is interrupted by regularly repeated eclipses of constant and long duration.</fc:definition>

<fc:code>9</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>interrupted very quick flashing</fc:label>

<fc:definition>a light in which the very rapid alterations of light and darkness are interrupted at regular intervals by eclipses of long duration.</fc:definition>

<fc:code>10</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>interrupted ultra quick flashing</fc:label>

<fc:definition>a light in which the ultra quick flashes (160 or more per minute) are interrupted at regular intervals by eclipses of long duration.</fc:definition>

<fc:code>11</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>morse</fc:label>

<fc:definition>a rhythmic light in which appearances of light of two clearly different durations are grouped to represent a character or characters in the Morse code.</fc:definition>

<fc:code>12</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>fixed/flash</fc:label>

<fc:definition />

<fc:code>13</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>flash/long-flash</fc:label>

<fc:definition />

<fc:code>14</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>occulting/flash</fc:label>

<fc:definition />

<fc:code>15</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>fixed/long-flash</fc:label>

<fc:definition />

<fc:code>16</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>occulting alternating</fc:label>

<fc:definition />

<fc:code>17</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>long-flash alternating</fc:label>

<fc:definition />

<fc:code>18</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>flash alternating</fc:label>

<fc:definition />

<fc:code>19</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>group alternating</fc:label>

<fc:definition />

<fc:code>20</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>quick-flash plus long-flash</fc:label>

<fc:definition />

<fc:code>25</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>very quick-flash plus long-flash</fc:label>

<fc:definition />

<fc:code>26</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>ultra quick-flash plus long-flash</fc:label>

<fc:definition />

<fc:code>27</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>alternating</fc:label>

<fc:definition>a signal light that shows, in any given direction, two or more colours in a regularly repeated sequence with a regular periodicity.</fc:definition>

<fc:code>28</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>fixed and alternating flashing</fc:label>

<fc:definition />

<fc:code>29</fc:code>

</fc:listedValue>

</fc:simpleAttribute>

- <fc:simpleAttribute>

<fc:name>Light colour</fc:name>

<fc:definition>The colour of a navigational light.</fc:definition>

<fc:code>lightColour</fc:code>

<fc:numericCode />

<fc:remarks />

<fc:dataType>enumeration</fc:dataType>

- <fc:listedValue>

<fc:label>white</fc:label>

<fc:definition />

<fc:code>1</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>red</fc:label>

<fc:definition />

<fc:code>2</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>green</fc:label>

<fc:definition />

<fc:code>3</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>yellow</fc:label>

<fc:definition />

<fc:code>4</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>blue</fc:label>

<fc:definition />

<fc:code>5</fc:code>

</fc:listedValue>

</fc:simpleAttribute>

- <fc:simpleAttribute>

<fc:name>Light description</fc:name>

<fc:definition />

<fc:code />

<fc:numericCode />

<fc:remarks />

<fc:dataType>text</fc:dataType>

</fc:simpleAttribute>

- <fc:simpleAttribute>

<fc:name>Light visibility</fc:name>

<fc:definition>The specific visibility of a light, with respect to the lights intensity and ease of recognition.</fc:definition>

<fc:code />

<fc:numericCode>108</fc:numericCode>

<fc:remarks />

<fc:dataType>enumeration</fc:dataType>

- <fc:listedValue>

<fc:label>high intensity</fc:label>

<fc:definition>non-marine lights with a higher power than marine lights and visible from well off shore (often "Aero" lights).</fc:definition>

<fc:code>1</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>low intensity</fc:label>

<fc:definition>non-marine lights with lower power than marine lights.</fc:definition>

<fc:code>2</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>faint</fc:label>

<fc:definition>a decrease in the apparent intensity of a light which may occur in the case of partial obstructions.</fc:definition>

<fc:code>3</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>intensified</fc:label>

<fc:definition>a light in a sector is intensified (i.e. has longer range than other sectors).</fc:definition>

<fc:code>4</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>unintensified</fc:label>

<fc:definition>a light in a sector is unintensified (i.e. has shorter range than other sectors).</fc:definition>

<fc:code>5</fc:code>

</fc:listedValue>

</fc:simpleAttribute>

- <fc:simpleAttribute>

<fc:name>Mark colour</fc:name>

<fc:definition />

<fc:code />

<fc:numericCode />

<fc:remarks />

<fc:dataType>enumeration</fc:dataType>

- <fc:listedValue>

<fc:label>black</fc:label>

<fc:definition />

<fc:code>1</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>blue</fc:label>

<fc:definition />

<fc:code>2</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>green</fc:label>

<fc:definition />

<fc:code>3</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>orange</fc:label>

<fc:definition />

<fc:code>4</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>red</fc:label>

<fc:definition />

<fc:code>5</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>white</fc:label>

<fc:definition />

<fc:code>6</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>yellow</fc:label>

<fc:definition />

<fc:code>7</fc:code>

</fc:listedValue>

</fc:simpleAttribute>

- <fc:simpleAttribute>

<fc:name>MMSI number</fc:name>

<fc:definition />

<fc:code />

<fc:numericCode />

<fc:remarks />

<fc:dataType>integer</fc:dataType>

</fc:simpleAttribute>

- <fc:simpleAttribute>

<fc:name>Nominal light range</fc:name>

<fc:definition>The nominal range at which an object can be seen or a signal detected.</fc:definition>

<fc:code />

<fc:numericCode />

<fc:remarks />

<fc:dataType>real</fc:dataType>

</fc:simpleAttribute>

- <fc:simpleAttribute>

<fc:name>Period of operation</fc:name>

<fc:definition>The times at which equipment is operational.</fc:definition>

<fc:code />

<fc:numericCode />

<fc:remarks />

<fc:dataType>enumeration</fc:dataType>

- <fc:listedValue>

<fc:label>day</fc:label>

<fc:definition />

<fc:code>1</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>night</fc:label>

<fc:definition />

<fc:code>2</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>occasional</fc:label>

<fc:definition />

<fc:code>3</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>24 hours</fc:label>

<fc:definition />

<fc:code>4</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>fog</fc:label>

<fc:definition />

<fc:code>5</fc:code>

</fc:listedValue>

</fc:simpleAttribute>

- <fc:simpleAttribute>

<fc:name>Shape</fc:name>

<fc:definition />

<fc:code />

<fc:numericCode />

<fc:remarks />

<fc:dataType>enumeration</fc:dataType>

- <fc:listedValue>

<fc:label>barrel</fc:label>

<fc:definition />

<fc:code>1</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>can</fc:label>

<fc:definition />

<fc:code>2</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>conical</fc:label>

<fc:definition />

<fc:code>3</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>major pile</fc:label>

<fc:definition />

<fc:code>4</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>minor pile</fc:label>

<fc:definition />

<fc:code>5</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>multiple</fc:label>

<fc:definition />

<fc:code>6</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>pillar</fc:label>

<fc:definition />

<fc:code>7</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>spar/spindle</fc:label>

<fc:definition />

<fc:code>8</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>spherical</fc:label>

<fc:definition />

<fc:code>9</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>super-buoy</fc:label>

<fc:definition />

<fc:code>10</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>tower</fc:label>

<fc:definition />

<fc:code>11</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>tripodal</fc:label>

<fc:definition />

<fc:code>12</fc:code>

</fc:listedValue>

</fc:simpleAttribute>

- <fc:simpleAttribute>

<fc:name>Surveyed by</fc:name>

<fc:definition />

<fc:code />

<fc:numericCode />

<fc:remarks />

<fc:dataType>text</fc:dataType>

</fc:simpleAttribute>

- <fc:simpleAttribute>

<fc:name>Topmark colour</fc:name>

<fc:definition />

<fc:code />

<fc:numericCode />

<fc:remarks />

<fc:dataType>enumeration</fc:dataType>

- <fc:listedValue>

<fc:label>black</fc:label>

<fc:definition />

<fc:code>1</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>white</fc:label>

<fc:definition />

<fc:code>2</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>yellow</fc:label>

<fc:definition />

<fc:code>3</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>green</fc:label>

<fc:definition />

<fc:code>4</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>orange</fc:label>

<fc:definition />

<fc:code>5</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>red</fc:label>

<fc:definition />

<fc:code>6</fc:code>

</fc:listedValue>

</fc:simpleAttribute>

- <fc:simpleAttribute>

<fc:name>Topmark/daymark shape</fc:name>

<fc:definition />

<fc:code />

<fc:numericCode>171</fc:numericCode>

<fc:remarks />

<fc:dataType>enumeration</fc:dataType>

- <fc:listedValue>

<fc:label>cone, point up</fc:label>

<fc:definition>is where the vertex points up. A cone is a solid figure generated by straight lines drawn from a fixed point (the vertex) to a circle in a plane not containing the vertex. Cones are commonly used as International Association of Lighthouse Authorities - IALA topmarks lateral.</fc:definition>

<fc:code>1</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>cone, point down</fc:label>

<fc:definition>is where the vertex points down. A cone is a solid figure generated by straight lines drawn from a fixed point (the vertex) to a circle in a plane not containing the vertex. Cones are commonly used as International Association of Lighthouse Authorities - IALA topmarks lateral.</fc:definition>

<fc:code>2</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>sphere</fc:label>

<fc:definition>a body the surface of which is at all points equidistant from the centre. Spheres are commonly used as International Association of Lighthouse Authorities - IALA topmarks safe water.</fc:definition>

<fc:code>3</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>2 spheres</fc:label>

<fc:definition>two black spheres are commonly used as an International Association of Lighthouse Authorities - IALA topmark (isolated danger).</fc:definition>

<fc:code>4</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>cylinder (can)</fc:label>

<fc:definition>a solid geometrical figure generated by straight lines fixed in direction and describing with one of point a closed curve, especially a circle (in which case the figure is circular cylinder, its ends being parallel circles). Cylinders are commonly used as International Association of Lighthouse Authorities - IALA topmarks lateral.</fc:definition>

<fc:code>5</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>board</fc:label>

<fc:definition>usually of rectangular shape, made from timber or metal and used to provide a contrast with the natural background of a daymark. The actual daymark is often painted on to this board.</fc:definition>

<fc:code>6</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>x-shape (St. Andrews cross)</fc:label>

<fc:definition>having a shape or a cross-section like the capital letter X. An x-shape as an International Association of Lighthouse Authorities - IALA topmark should be 3 dimensional in shape. It is made of at least three crossed bars.</fc:definition>

<fc:code>7</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>upright cross (St. Georges Cross)</fc:label>

<fc:definition>a cross with one vertical member and one horizontal member, i.e. similar in shape to the character "+".</fc:definition>

<fc:code>8</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>cube, point up</fc:label>

<fc:definition>a cube standing on one of its vertexes. A cube is a solid contained by six equal squares a regular hexahedron (The New Shorter Oxford English Dictionary. 1993. vol 2)</fc:definition>

<fc:code>9</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>2 cones, point to point</fc:label>

<fc:definition>2 cones, one above the other, with their vertices together in the centre.</fc:definition>

<fc:code>10</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>2 cones, base to base</fc:label>

<fc:definition>2 cones, one above the other, with their bases together in the centre and their vertices pointing up and down.</fc:definition>

<fc:code>11</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>rhombus</fc:label>

<fc:definition>a plane figure having four equal sides and equal opposite angles (two acute and two obtuse) an oblique equilateral parallelogram.</fc:definition>

<fc:code>12</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>2 cones (points upward)</fc:label>

<fc:definition />

<fc:code>13</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>2 cones (points downward)</fc:label>

<fc:definition />

<fc:code>14</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>besom, point up (broom or perch)</fc:label>

<fc:definition>besom: a bundle of rods or twigs. perch: a staff placed on top of a buoy, rock or shoal as a mark for navigation.</fc:definition>

<fc:code>15</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>besom, point down (broom or perch)</fc:label>

<fc:definition>besom: a bundle of rods or twigs. perch: a staff placed on top of a buoy, rock or shoal as a mark for navigation.</fc:definition>

<fc:code>16</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>flag</fc:label>

<fc:definition>a flag mounted on a short pole.</fc:definition>

<fc:code>17</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>sphere over rhombus</fc:label>

<fc:definition>A sphere located above a rhombus.</fc:definition>

<fc:code>18</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>square</fc:label>

<fc:definition>a plane figure with four right angles and four equal straight sides</fc:definition>

<fc:code>19</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>horizontal rectangle</fc:label>

<fc:definition>where the two longer opposite sides are standing horizontally. A rectangle is a plane figure with four right angles and four straight sides, opposite sides being parallel and equal in length</fc:definition>

<fc:code>20</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>vertical rectangle</fc:label>

<fc:definition>where the two longer opposite sides are standing vertically. A rectangle is a plane figure with four right angles and four straight sides, opposite sides being parallel and equal in length</fc:definition>

<fc:code>21</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>trapezium, up</fc:label>

<fc:definition>which stands on its longer parallel side. A trapezium is a quadrilateral having one pair of opposite sides parallel.</fc:definition>

<fc:code>22</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>trapezium, down</fc:label>

<fc:definition>which stands on its shorter parallel side. A trapezium is a quadrilateral having one pair of opposite sides parallel.</fc:definition>

<fc:code>23</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>triangle, point up</fc:label>

<fc:definition>A triangle, point up. A triangle is a figure having three angles and three sides.</fc:definition>

<fc:code>24</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>triangle, point down</fc:label>

<fc:definition>A triangle, point down. A triangle is a figure having three angles and three sides.</fc:definition>

<fc:code>25</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>circle</fc:label>

<fc:definition>a perfectly round plane figure whose circumference is everywhere equidistant from its centre.</fc:definition>

<fc:code>26</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>two upright crosses (one over the other)</fc:label>

<fc:definition>two upright crosses, generally vertically disposed one above the other.</fc:definition>

<fc:code>27</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>T-shape</fc:label>

<fc:definition>having a shape like the capital letter T.</fc:definition>

<fc:code>28</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>triangle pointing up over a circle</fc:label>

<fc:definition>a triangle, vertex uppermost, located above a circle.</fc:definition>

<fc:code>29</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>upright cross over a circle</fc:label>

<fc:definition>an upright cross located above a circle.</fc:definition>

<fc:code>30</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>rhombus over a circle</fc:label>

<fc:definition>a rhombus located above a circle.</fc:definition>

<fc:code>31</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>circle over a triangle pointing up</fc:label>

<fc:definition>a circle located over a triangle, vertex uppermost.</fc:definition>

<fc:code>32</fc:code>

</fc:listedValue>

</fc:simpleAttribute>

- <fc:simpleAttribute>

<fc:name>Type of structure</fc:name>

<fc:definition />

<fc:code />

<fc:numericCode />

<fc:remarks />

<fc:dataType>enumeration</fc:dataType>

- <fc:listedValue>

<fc:label>beacon</fc:label>

<fc:definition />

<fc:code>1</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>hut</fc:label>

<fc:definition />

<fc:code>2</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>lighthouse</fc:label>

<fc:definition />

<fc:code>3</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>tower - hex</fc:label>

<fc:definition />

<fc:code>4</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>tower - lattice</fc:label>

<fc:definition />

<fc:code>5</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>tower - masonry</fc:label>

<fc:definition />

<fc:code>6</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>tower with hut</fc:label>

<fc:definition />

<fc:code>7</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>offshore platform</fc:label>

<fc:definition />

<fc:code>8</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>wind turbine</fc:label>

<fc:definition />

<fc:code>9</fc:code>

</fc:listedValue>

</fc:simpleAttribute>

- <fc:simpleAttribute>

<fc:name>Vertical datum</fc:name>

<fc:definition>The datum to which both heights (vertical datum, see S-57 Part 3) and soundings (sounding datum, see S-57 Part 3) are referred.</fc:definition>

<fc:code />

<fc:numericCode>185</fc:numericCode>

<fc:remarks>When the vertical datum is unknown, such as water areas above locks, the value "local datum" is to be used, and further details may be encoded using "INFORM". The +0.3m approximation quoted in the "approximate" levels is somehow arbitrary and follows the British example of their definition for "approximate LAT".</fc:remarks>

<fc:dataType>enumeration</fc:dataType>

- <fc:listedValue>

<fc:label>mean low water springs</fc:label>

<fc:definition>(MLWS) - the average height of the low waters of spring tides.</fc:definition>

<fc:code>1</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>mean lower low water springs</fc:label>

<fc:definition>(MLLWS) - the average height of lower low water springs at a place.</fc:definition>

<fc:code>2</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>mean sea level</fc:label>

<fc:definition>(MSL) - the average height of the surface of the sea at a tide station for all stages of the tide over a 19-year period, usually determined from hourly height readings measured from a fixed predetermined reference level.</fc:definition>

<fc:code>3</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>lowest low water</fc:label>

<fc:definition>an arbitrary level conforming to the lowest tide observed at a place, or some what lower.</fc:definition>

<fc:code>4</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>mean low water</fc:label>

<fc:definition>(MLW) - the average height of all low waters at a place over a 19-year period.</fc:definition>

<fc:code>5</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>lowest low water springs</fc:label>

<fc:definition>an arbitrary level conforming to the lowest water level observed at a place at spring tides during a period of time shorter than 19 years.</fc:definition>

<fc:code>6</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>approximate mean low water springs</fc:label>

<fc:definition>an arbitrary level, usually within 0.3m from that of mean low water springs (MLWS).</fc:definition>

<fc:code>7</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>Indian spring low water</fc:label>

<fc:definition>(ISLW) - an arbitrary tidal datum approximating the level of the mean of the lower low water at spring tides.</fc:definition>

<fc:code>8</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>low water springs</fc:label>

<fc:definition>an arbitrary level, approximating that of mean low water springs (MLWS).</fc:definition>

<fc:code>9</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>approximate lowest astronomical tide</fc:label>

<fc:definition>an arbitrary level, usually within 0.3m from that of lowest astronomical tide (LAT).</fc:definition>

<fc:code>10</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>nearly lowest low water</fc:label>

<fc:definition>an arbitrary level approximating the lowest water level observed at a place, usually equivalent to the Indian spring low water (ISLW).</fc:definition>

<fc:code>11</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>mean lower low water</fc:label>

<fc:definition>(MLLW) - the average height of the lower low waters at a place over a 19-year period.</fc:definition>

<fc:code>12</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>low water</fc:label>

<fc:definition>an approximation of mean low water adopted as the reference level for a limited area, irrespective of better determinations at a later date.</fc:definition>

<fc:code>13</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>approximate mean low water</fc:label>

<fc:definition>an arbitrary level, usually within 0.3m from that of mean low water (MLW).</fc:definition>

<fc:code>14</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>approximate mean lower low water</fc:label>

<fc:definition>an arbitrary level, usually within 0.3m from that of mean lower low water (MLLW).</fc:definition>

<fc:code>15</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>mean high water</fc:label>

<fc:definition>(MHW) - the average height of all high waters at a place over a 19-year period.</fc:definition>

<fc:code>16</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>mean high water springs</fc:label>

<fc:definition>(MHWS) - the average height of the high waters of spring tides.</fc:definition>

<fc:code>17</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>high water</fc:label>

<fc:definition>the highest level reached at a place by the water surface in one tidal cycle.</fc:definition>

<fc:code>18</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>approximate mean sea level</fc:label>

<fc:definition>an arbitrary level, usually within 0.3m from that of mean sea level (MSL).</fc:definition>

<fc:code>19</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>high water springs</fc:label>

<fc:definition>an arbitrary level, approximating that of mean high water springs (MHWS).</fc:definition>

<fc:code>20</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>mean higher high water</fc:label>

<fc:definition>(MHHW) - the average height of higher high waters at a place over a 19-year period.</fc:definition>

<fc:code>21</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>equinoctial spring low water</fc:label>

<fc:definition>the level of low water springs near the time of an equinox.</fc:definition>

<fc:code>22</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>lowest astronomical tide</fc:label>

<fc:definition>(LAT) - the lowest tide level which can be predicted to occur under average meterological conditions and under any combination of astronomical conditions.</fc:definition>

<fc:code>23</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>local datum</fc:label>

<fc:definition>an arbitrary datum defined by a local harbour authority, from which levels and tidal heights are measured by this authority.</fc:definition>

<fc:code>24</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>international great lakes datum 1985</fc:label>

<fc:definition>(IGLD 1985) - a vertical reference system with its zero based on the mean water level at Rimouski/Pointe-au-P??re, Quebec, over the period 1970 to 1988.</fc:definition>

<fc:code>25</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>mean water level</fc:label>

<fc:definition>the average of all hourly water levels over the available period of record.</fc:definition>

<fc:code>26</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>lower low water large tide</fc:label>

<fc:definition>(LLWLT) - the average of the lowest low waters, one from each of 19 years of observations.</fc:definition>

<fc:code>27</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>higher high water large tide</fc:label>

<fc:definition>(HHWLT) - the average of the highest high waters, one from each of 19 years of observations.</fc:definition>

<fc:code>28</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>nearly highest high water</fc:label>

<fc:definition>an arbitrary level approximating the highest water level observed at a place, usually equivalent to the high water springs.</fc:definition>

<fc:code>29</fc:code>

</fc:listedValue>

- <fc:listedValue>

<fc:label>highest astronomical tide (HAT)</fc:label>

<fc:definition>the highest tidal level which can be predicted to occur under average meteorological conditions and under any combination of astronomical conditions.</fc:definition>

<fc:code>30</fc:code>

</fc:listedValue>

</fc:simpleAttribute>

</fc:simpleAttributes>

- <fc:complexAttributes>

- <fc:complexAttribute>

<fc:name>Daymark topmark</fc:name>

<fc:definition />

<fc:code />

<fc:numericCode />

- <fc:alias>

<fc:value />

<fc:context>S-57 Acronym</fc:context>

</fc:alias>

- <fc:subAttributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>1</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:subAttributeBinding>

- <fc:subAttributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>1</s100Base:lower>

<s100Base:upper infinite="true" xsi:nil="true">0</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:subAttributeBinding>

</fc:complexAttribute>

</fc:complexAttributes>

- <fc:informationTypes>

- <fc:informationType>

<fc:name>General comments</fc:name>

<fc:definition />

<fc:code>GeneralComments</fc:code>

<fc:numericCode>0</fc:numericCode>

- <fc:attributeBinding>

- <fc:multiplicity>

<s100Base:lower />

<s100Base:upper />

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

</fc:informationType>

- <fc:informationType>

<fc:name>Survey information</fc:name>

<fc:definition />

<fc:code>SurveyInformation</fc:code>

<fc:numericCode />

- <fc:attributeBinding>

- <fc:multiplicity>

<s100Base:lower>0</s100Base:lower>

<s100Base:upper>0</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

- <fc:attributeBinding>

- <fc:multiplicity>

<s100Base:lower>0</s100Base:lower>

<s100Base:upper>1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

</fc:informationType>

</fc:informationTypes>

- <fc:featureTypes>

- <fc:featureType>

<fc:name>AIS Aid to Navigation</fc:name>

<fc:definition />

<fc:code>AISAtoN</fc:code>

<fc:numericCode />

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>1</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>1</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>0</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>1</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

<fc:featureUseType>geographic</fc:featureUseType>

</fc:featureType>

- <fc:featureType>

<fc:name>Directional light</fc:name>

<fc:definition>A light illuminating a sector of a very narrow angle and intended to mark a direction to follow.</fc:definition>

<fc:code>DirectionalLight</fc:code>

<fc:numericCode />

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>0</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

- <fc:attributeBinding sequential="true">

- <fc:multiplicity>

<s100Base:lower>1</s100Base:lower>

<s100Base:upper infinite="true" xsi:nil="true">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="lightColour" />

</fc:attributeBinding>

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>1</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>0</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>0</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>0</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>0</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>1</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

<fc:featureUseType>geographic</fc:featureUseType>

</fc:featureType>

- <fc:featureType>

<fc:name>Fixed AtoN</fc:name>

<fc:definition />

<fc:code>FixedAtoN</fc:code>

<fc:numericCode />

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>0</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>1</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>0</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>0</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

<fc:featureUseType>geographic</fc:featureUseType>

</fc:featureType>

- <fc:featureType>

<fc:name>Floating AtoN</fc:name>

<fc:definition />

<fc:code>FloatgingAtoN</fc:code>

<fc:numericCode />

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>1</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

<fc:featureUseType>geographic</fc:featureUseType>

</fc:featureType>

- <fc:featureType>

<fc:name>Fog detector light</fc:name>

<fc:definition>A light used to automatically determine the conditions of visibility which warrant the operation of a sound signal.</fc:definition>

<fc:code>FogDetectorLight</fc:code>

<fc:numericCode />

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>0</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

- <fc:attributeBinding sequential="true">

- <fc:multiplicity>

<s100Base:lower>0</s100Base:lower>

<s100Base:upper infinite="true" xsi:nil="true">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="lightColour" />

</fc:attributeBinding>

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>0</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>1</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

<fc:featureUseType>geographic</fc:featureUseType>

</fc:featureType>

- <fc:featureType>

<fc:name>Fog signal</fc:name>

<fc:definition>A warning signal transmitted by a vessel, or aid to navigation, during periods of low visibility. Also, the device producing such a signal.</fc:definition>

<fc:code />

<fc:numericCode />

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>0</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>0</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>0</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

<fc:featureUseType>geographic</fc:featureUseType>

</fc:featureType>

- <fc:featureType>

<fc:name>Light</fc:name>

<fc:definition>A luminous or lighted aid to navigation.</fc:definition>

<fc:code />

<fc:numericCode />

- <fc:attributeBinding sequential="true">

- <fc:multiplicity>

<s100Base:lower>1</s100Base:lower>

<s100Base:upper infinite="true" xsi:nil="true" />

</fc:multiplicity>

<fc:attribute ref="lightColour" />

</fc:attributeBinding>

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>0</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>0</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>0</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>1</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>1</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

<fc:featureUseType>geographic</fc:featureUseType>

<fc:permittedPrimitives>point</fc:permittedPrimitives>

</fc:featureType>

- <fc:featureType>

<fc:name>Radar transponder beacon</fc:name>

<fc:definition>A transponder beacon transmitting a coded signal on radar frequency, permitting an interrogating craft to determine the bearing and range of the transponder.</fc:definition>

<fc:code />

<fc:numericCode />

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>0</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>0</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

<fc:featureUseType>geographic</fc:featureUseType>

</fc:featureType>

- <fc:featureType>

<fc:name>Sectored light</fc:name>

<fc:definition>A sectored light is a light having one or more sectors, which have different characteristics acfross and sometimes within each sector.</fc:definition>

<fc:code>SectoredLight</fc:code>

<fc:numericCode />

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>0</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>0</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

- <fc:attributeBinding sequential="false">

- <fc:multiplicity>

<s100Base:lower>1</s100Base:lower>

<s100Base:upper infinite="false" xsi:nil="false">1</s100Base:upper>

</fc:multiplicity>

<fc:attribute ref="" />

</fc:attributeBinding>

<fc:featureUseType>geographic</fc:featureUseType>

</fc:featureType>

</fc:featureTypes>

</fc:Catalog>