IALA Guideline

G1???

AtoN equipment AND STRUCTURES exposed to extreme environmental conditions

Edition 1.0

October 2022

Revisions to this IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

1	INTRODUCTION	6

2	SCOPE	6

3	EXTREME ENVIRONMENTAL CONDITIONS	6

3.1	MATRIX	6

3.2	WIND	8

3.2.1	Tropical cyclones	8

3.2.1.1	Criteria capable of being implemented	10

3.3	WAVES	11

3.3.1	Transmitted forces on floating aton	12

3.3.2	Transmitted forces on fixed aton	12

3.3.3	Tsunamis	13

3.3.4	Emergency and contingency plan	15

3.3.4.1	Suggested actions	15

3.3.4.2	Post-event evaluation	15

3.4	TIDES	16

3.4.1	Extreme Tidal	16

3.4.1.1	Extreme Lowest Astronomical Tide	17

3.4.1.2	Floodings	17

3.5	CURRENTS	18

3.5.1	Superficial currents	18

3.5.2	Tidal currents	18

3.6	GEOPHYSICAL CONDITIONS	18

3.6.1	Soil and Coastal morphology changes	18

3.6.1.1	Soil geology	19

3.6.1.2	Hydrography	19

3.7	CLIMATIC CONDITIONS	19

3.7.1	Temperature	19

3.7.1.1	Operating temperature	20

3.7.1.2	Storage temperature	21

3.7.2	High Ultra‐Violet Levels	21

3.7.3	Humidity and precipitation	22

3.7.4	Icing	23

3.7.4.1	Typical problems	24

3.8	ATMOSPHERIC CONDITIONS	26

3.8.1	Visibility	26

3.8.1.1	Difference between haze, fog and mist	26

3.8.2	Dust	26

3.8.3	Smoke	27

3.8.3.1	Incidence in navigation	28

3.8.3.2	Incidence in energy generation	29

3.9	WATER CONDITIONS	29

3.9.1	Corrosion	29

3.9.2	Abrasion	29

3.9.3	Impact damage	30

3.9.4	Marine Growth	30

3.10	Wildlife	31

3.10.1	Bird fouling	31

3.11	Incidence in people´s health	32

4	ACRONYMS & DEFINITIONS	33

4.1	Acronyms	33

4.2	Definitions	33

5	REFERENCES	33

List of Tables

Table 1	Matrix	8

Table 2	Wind scale - Saffir-Simpson	9

Table 3	Extreme tidal values	16

List of Figures

Figure 1	AtoNs affected by hurricanes	9

Figure 2	Front Leading Light with daymark lattice design	10

Figure 3	Examples of Structural modelling implemented	13

Figure 4	Expansive wave	14

Figure 5	Damages	14

Figure 6	Negative effect of floating vegetation	17

Figure 7	River coast flooded	17

Figure 8	Kirke Channel and Amadeo Ferry	18

Figure 9	Erosion process at the coast	19

Figure 10	Lead Acid Batteries ageing curve in relation to the temperature	21

Figure 11	High Ultra Violet effect on red buoy	22

Figure 12	Effect of ice on buoys	23

Figure 13	ICE incidence on AtoN structure	23

Figure 14	Port entrance mark covered by ice - Safety rail bent by ice on Tallinnamadal lighthouse	24

Figure 15	A buoy with ice protection over polycarbonate cupola - Reinforcement ribs on steel buoy - Damaged plastic ice buoy without foam to support it's structure	25

Figure 16	Appearance                              Skeleton	25

Figure 17	Presence of fog in a waterway	26

Figure 18	Presence of fog dust on AtoN device	27

Figure 19	Presence of smoke in a waterway	27

Figure 20	Fog bank	28

Figure 21	Abrasion chain	30

Figure 22	Marine growth	30

Figure 23	Bird fouling on buoys	31

Figure 24	Ant graffiti product application over the surface color	32

Figure 25	Risk on maintenance task buoy	33

List of Equations

No se encuentran elementos de tabla de ilustraciones.

# INTRODUCTION

Since its beginnings, the signalling system has been implemented to provide the mariners with the necessary information to travel safely. In this sense, the world-wide trade development and, as a matter of fact, the navigation routes extending across ports oriented to commerce, sports and leisure activities have been evolving in relation to their management, such as, load traffic vessels entrance and exit scheme, value of the load being shipped, required efficiency in navigation, among others. This efficiency scenario, operation windows or mooring planning and pier release directly affects the Competent Authorities who are responsible of granting the Services of Aids to Navigation in relation to the treatment of the environment variables, recording scope, extreme values, its early detection, measurement and distribution to the seafarer.

# SCOPE

This document aims to reinforce the most relevant information associated with these variables if they occur under extreme conditions including the current guidelines or other bibliography which are considered to have incidence in the Aids to Navigation.

Specific information from other guidelines was identified and then incorporated (with their reference) since there already exist several documents which address topics such as how extreme weather conditions (hot or cold extreme temperatures), among other variables, can affect the AtoNs performance.

The design of the aid to navigation and its equipment, or, the performance characteristics of it under extreme environmental conditions may not be available in the market at present, therefore will be develop the actions that these generate and their impact on Aids to Navigations and their equipment’s.

# EXTREME ENVIRONMENTAL CONDITIONS

The operation of an Aid to Navigation is affected by external forces, for example, extreme environmental conditions. If these conditions get worse, they can alter the normal operation of the AtoN, reduce the operating performance and its (useful) lifespan by damaging or deteriorating the structure and its equipment, as well as the conditioning associated with the maintenance plan, among others.

This document identifies and describes these negative effects as follows:

- Heeling, Shifting, and Vertical divergence;

- Reduced performance and (useful) lifespan;

- Stability of the AtoN and Damages on the structure;

- Damage of the AtoN and its equipment;

- Others.

Provided the knowledge, experiences and solutions developed for each problem addressed, in return, and, as an added value, certain situations which have been described were presented to the Organizations in charge of providing their service of aids to navigation in cases where extreme weather conditions, such as tsunamis or hurricanes, occur in their area of concern.

“Further information regarding the structural design of fixed AtoN is given in Guideline 1165 Sustainable Structural Design of Marine Aids to Navigation. This document provides information on the AtoN structural design process and helps the AtoN manager understand how to ensure environmental loads such as wind, wave and current are incorporated adequately into the design.”

## MATRIX

In this sense, a simple entry matrix has been developed allowing users to quickly identify the interrelation between the occurrence of extreme weather events (variables) and their impact on Aids to Navigation, and thus, the equipment installed in them.

| Extreme Environmental Conditions | Negative effect on floating AtoN | Negative effect on fix AtoN | Negative effect on Lanterns | Negative effect on Power supply |
| --- | --- | --- | --- | --- |
| Wind | 1) Stability,2) Position, swinging radius, 3) Size of Chain & mooring design, mooring length | 1) Stability of structure | 1) Vertical Divergence | 1) Wind generators and solar panels and cables |
| Waves | 1) Stability, 2) Position, 3) Bottom contact on shallow waters, 4) Size of Chain & Mooring Design, chain wear | 1) Transmitted forces on the structure. 2) Scour of the seabed causing instability | Effect on Vertical Divergence |  |
| Tides | 1) Size of Chain & Mooring Design, mooring length |  |  |  |
| Currents | 1) Stability, 2) Position, swinging radius, 3) Bottom contact on shallow waters, 4) Chain wear, mooring length, 5) Underwater lifespan of paint systems. | 1) Erosion of coastline 2) Underwater lifespan of paint systems. | 1) Effect on Vertical divergence |  |
| Ice and snow | 1) Position, swinging radius, 2) Structure 3) Visibility, occasional immersions, 4) loss of AtoN, 5) loss of retroreflecting film on AtoN | 2) Structure stability 3) loss of AtoN | 1) piling on lanterns 2) freezing | 1) reduction of capacity 2) influence on battery life 3) piling on solar panels |
| Coastal morphology changes |  | Stability – Position |  |  |
| High temperature | 1) Accelerate corrosion on the structure, thermal mechanical force (dilatation) | 1) Accelerate corrosion on the structure | Loss of useful life | Loss of useful life |
| High Ultra‐Violet Levels | Loss of color & degradation of material properties (plastic parts) | Loss of color & degradation of material properties | Loss of Led´s useful life | Loss of Solar Module useful life |
| Humidity | Corrosion |  | Loss of useful life by condensation cycle | Loss of useful life by condensation cycle |
| Icing | 1) Buoyancy, 2) Connectability with monitoring system, 3) Ice piling on AtoN, 4) Structural integrity | 1) Ice piling or around AtoN, 2) Structural integrity | 1) Visibility of light | 1) reduction of capacity 2) influence on battery life |
| Precipitation |  |  | 1) Visibility of light | Loss of useful life by humidity |
| Dust | Reduce the performance of day marks | Reduce the performance of day marks | 1) Visibility of light 2) Loss of useful life | Loss of useful life |
| Fog and Smoke |  |  | 1) Visibility of light | 1) Reduce the performance of solar panels |
| Marine Growth | 1) Floatability, 2) bio-Corrosion, 3) Paint scheme, recognition 4) Effect on buoy moorings and accelerate corrosion, drag force, overload weight for lifting | 1) Effect on foundations and accelerate corrosion |  |  |
| Salinity | 1) Corrosion, Loss of useful life | 1) Corrosion, Structural integrity | 1) Visibility of light | 1) Corrosion on contacts |
| Bird fouling | 1) Loss of color 2) Corrosion | 1) Loss of color 2) Corrosion | 1) Visibility of light, reduce the performance of lens | 1) Reduce the performance of solar panels charging |

Matrix

The effect of the interaction showed at the matrix will be addressed and explained throughout this Guideline.  Moreover, the incidence on navigation, access channel, staff health or port operations can also be identified by means of this matrix when an AtoN does not work correctly. Because of this, Organizations should recognize these errors and develop a new implementation program to mitigate the negatives effects.

## WIND

Wind has special incidence on the AtoN performance since, depending on its speed and direction, it generates the “swell effect” which modifies the behaviour of the hydrostatic buoys.

This effect has a great direct incidence on the vertical divergence of the lantern, unstable positions due to the generation of waves or the chain wear deterioration rate. Because all the issues mentioned before, it is necessary to consider the waves as another variable while designing the process of floating AtoNs.

### Tropical cyclones

A tropical cyclone is a generic term used for low atmospheric pressure systems formed in the oceans generally in homogeneous weather conditions and tropical zones. It is associated with a wide area of clouds, rain and electric storms, along with a superficial wind circulation, which is counterclockwise in the north hemisphere and clockwise in the south hemisphere. They are usually called typhoons in the Norwest region of the Pacific Ocean and hurricanes in the Northeast Pacific and North Atlantic Ocean regions.

There are at least three conditions for the existence of tropical cyclones:

•       A pre-existent disturbance: An extended rain clouds area with electric storms.

•       Warm ocean water temperature: 26.6 ˚C or higher, up to a 45 meters depth.

•       Winds at the superior atmosphere. Weak, with minor changes of speed and direction through the layers of the atmosphere.

Tropical cyclones are classified according their sustained maximum wind speed per minute, as follows:

•       Tropical depression: Maximum wind speed less than 63 km/h.

•       Tropical storm: Maximum wind speed between 63 y 117 km/h.

•       Hurricane: Maximum wind speed higher than a 117 km/h.

A hurricane is a tropical cyclone fully developed. They are classified in five categories according to the Saffir-Simpson scale:

| Category | Maximum sustained wind speed (km/h) | Damages |
| --- | --- | --- |
| 1 | 119 – 153 | Minimum |
| 2 | 154 – 177 | Moderated |
| 3 | 178 – 209 | Extended |
| 4 | 210 – 249 | Extreme |
| 5 | >250 | Catastrophic |

Wind scale - Saffir-Simpson

In a hurricane, the destructive effect of the wind is directly related to its speed. While tropical depression winds are only capable to damage weak structures, those of a 4 and 5 category hurricane can seriously affect concrete buildings, especially if they are high. Aids to Navigation sometimes lose their solar panels, top and day marks, and even marine lanterns and energy systems are also affected. Structures could also be damaged if their design is not suitable to withstand the effect of the winds or depending on their maintenance condition. Wind gusts are especially dangerous as they increase from 1.2 to 1.5 times the effect of the wind during 2 or 3 seconds.

Hurricanes and typhoons are meteorological events causing several damages to AtoN every year. Their effect could cause the closure of the waterways by the Competent Authority for several weeks or at least the delays in operations.

AtoNs affected by hurricanes

A short-term option to minimize the destructive effects of hurricanes or typhoons on Aids to Navigation could be the luminous and energetic system deactivation and the removal of other accessories for its preservation. This may avoid the loss of valuable equipment providing at the same time to the AtoN Authority the possibility to ready those waterways of mayor importance in a short period of time after the meteorological event has left the area. This must be done following an organized protocol including timely information to the maritime community in compliance with Regulation 13, Chapter V of SOLAS.

A long-term option should be manufacturing more resilient AtoNs to these events, including structural modifications to their design and considering for the mooring systems of buoys the extreme values of the meteorological factors such as current, waves and wind during the return period (gap between one event and the other of the same kind or category) evidenced in a specific geographic area and for different events on the Saffir-Simpson scale.

A practical solution to provide more resilient AtoNs against wind could be the use of lattice design for towers, daymarks and topmarks. Reference: Guideline 1094 Daymarks for AtoNs and Guideline 1165 Sustainable Structural Design of Marine Aids to Navigation.

Front Leading Light with daymark lattice design

#### Criteria capable of being implemented

Prior to the hurricane season, a detailed analysis regarding the priorities for the deactivation and further activation process must be conducted, preferably involving the criteria and expertise from the most relevant members of the maritime community such as pilots, maritime administrations, coast guards, navy, shipping companies, meteorological services, among other stakeholders. For this purpose, the IALA Risk Management Toolbox could be extremely valuable when focused on the risk related to maritime operations that could be present before, during and after the occurrence of such events. Modelling and simulation techniques should be an advantage when performing this analysis to show the possible or forecasted scenarios that must be taken under consideration in a given maritime area or waterway. This analysis should be run in a yearly basis as some elements related to the operational conditions of a waterway may vary, such as local maritime regulations, AtoN system modifications, among others.

The result of this analysis could be a plan to practically organize the deactivation and activation process, including:

- An Agreement and Cooperation Act to be signed by the process stakeholders, containing the list of AtoNs to be deactivated and activated according to the priorities determined in the analysis, and the means provided by each stakeholder to cooperate with the process.

- A table containing the number of AtoNs with real possibilities to be part of the process, sorted by type of AtoN.

- The composition of each Deactivation and Activation Brigade (DAB) to perform process, its mean of transportation, waterways to be covered, departure point, trajectory, communications, storage facilities for the preservation of the luminous and energetic system and other accessories, among other practical details. Proper training for the DAB personnel based on local AtoN System conditions should be provided.

This Plan should be submitted for the approval of the Competent Authorities to ensure its compliance with national, territorial and local regulations.

The criteria to establish the priorities for the deactivation process may include, among others:

- The importance of the AtoN for the maritime operations in a given waterway: those meant to be used for the conclusion of vessels manoeuvres prior to the arrival of the hurricane must be deactivated last.

- The meteorological conditions: the deactivation process should start well in advance to the deterioration of the sea conditions in order that service vessels could work safely, especially in buoys. The up-to date information on the current position, trajectory, speed displacement and strength of the hurricane provided by local meteorological services is essential.

- The distance from the departure point of DAB to the farthest AtoN to be deactivated: it would be unpractical trying to deactivate AtoNs located too far away.

- The average time spent in each deactivation, according to the complexity of the AtoN.

After a hurricane left an area or waterway, an evaluation of its AtoN system status should be made to assess the damages caused as soon as the meteorological conditions allows to do so, in order to determine the damages and to prepare all the necessary resources to repair or reactivate them.

Subsequently, the criteria to establish the priorities for the activation process may include, among others:

- The urgency to ready the most important waterways: those AtoNs meant to contribute to a safe navigation by these waterways must be activated first.

- The meteorological conditions: the activation process should start as soon as the weather condition allows the deployment of DABs.

- The use of most suitable and reliable equipment available in the AtoN maintenance station or preserved during the deactivation process to ready the most critical AtoN.

## WAVES

The wave effect is strictly linked with the performance of the aids to navigation, its influence in the inner navigational waterways, access channels and the port functionality.

The wave climate can be foreseen by means of simulations, weather forecast, or it can be measured by special buoys which measure the energy transmitted by the wave to interpret the vicinity wave climate nearby:

- Significant wave height (Hs)

- Mean direction

- Peak period (Tp)

### Transmitted forces on floating aton

The wave impact always has negative consequences on the aids to navigation which should be considered at the moment of being designed. These consequences depend, mainly, if the aids to navigation are fixed or floating aids.

Every time the mooring chain abruptly reaches its end, even in shallow waters, it has consequently high peaks of force.

In floating Aids to Navigation, the waves directly affect the stability of the signal and, as a matter of fact, the vertical deviation of the lightning system implemented.

Guideline 1066 “The Design of Floating Aid to Navigation Moorings” tackles the problem of the concordance of the location of the signals and the breaking of the wave. To establish this relationship, it is necessary to obtain the parameters of the wave height at break (Hb) and the depth of break (hb), obtaining these parameters reduces the uncertainty in the selection of sites for the installation of the aids to navigation.

There are many break criteria that allow estimating (in the case of regular waves) Hb based on the characteristics of the waves (Hs, Tp and mean direction) and the modal state of the beach. However, given that in nature the waves are to a greater or lesser extent, irregular, it is necessary to resort to the bibliography that best suits them to establish the breaking criterion that best suits the study environment. It is important to indicate that the breaking of the waves produces a great amount of energy to the buoy and the mooring. This event can lead to chain breakage or cause dead weight to be pulled elsewhere, presenting the opportunity to use spring anchors as a solution to these scenarios.

According to Sánchez-Arcilla and Lemos (1990), there are basically two types of breaking criteria (for waves at shallow and intermediate depths):

- Criteria that express the breaking conditions as a function of local parameters of the wave and bathymetric characteristics (or bottom slope).

- Criteria that specify the wave height at break as a function of bathymetric characteristics (beach slopes) and wave super elevation in the offshore zone (Ho / Lo).

Basically speaking, mooring of a buoy has, apparently, two contradictory functions. On the one hand, it keeps the buoy on its place, but on the other hand, the mooring has to follow the waves dynamics in order to  be capable of reducing the charges in the mooring and to absorb the energy created by the movements of the buoy and to compensate the differences between the levels of the water, the tides, the waves, etc. Reference: Guideline 1099-Hydrostatic Buoy Design.

### Transmitted forces on fixed aton

Wave impacts on fixed Aids to Navigation i.e. beacons and lighthouses placed on rock outcrops, cause deterioration of the structures particularly where the assets are ageing.

Trinity House, the Northern Lighthouse Board and the Irish Lights have supported the EPSRC-funded research project STORMLAMP, undertaken by a number of organisations (University of Plymouth, University College London and the University of Exeter University) to investigate the structural response of heritage rock lighthouses to wave loading during storms. They have estimated the wave loading on a number of vulnerable lighthouses using Bayesian-based statistical forecasts, empirical approaches, physical experiments and computational fluid dynamics. The resulting structural response has been monitored through lighthouse-based instrumentation, and modelled using detailed structural analysis, validated using data obtained in field modal testing. On the basis of the investigations they have been able to predict the structural vibrations and the survivability of the lighthouses into the future, providing crucial information for operational purposes and maintenance planning (Raby et al., 2019; Antonini et al., 2019).

Examples of Structural modelling implemented

Graphic reference: left - limit analysis; centre - fine element analysis; right - distinct element analysis

Tropical cyclones also generate very large waves called swells arriving to shore along with this event, affecting from 150 to 200 km of coastline and lasting a few hours. They are caused by the strong wind stress and the drop of atmospheric pressure applied on a shallow water area, reaching up to 6 meters height.

The combination of swells with astronomic tide when a hurricane is affecting a specific area produces the highest water level on high tide. This effect is increased by waves moving over this already elevated water level.

“Further information regarding the structural design of fixed AtoN is given in Guideline G1165 Sustainable Structural Design of Marine Aids to Navigation. This document provides information on the AtoN structural design process and helps the AtoN manager understand how to ensure environmental loads such as wind, wave and current are incorporated adequately into the design.”

### Tsunamis

Tsunamis are characterised by long return periods (tens to hundreds of years between events), long wavelengths (kms) and long wave periods (minutes to hours). In order importance, they are typically caused by undersea earthquakes, submarine/subaerial landslides including island flank collapses, underwater volcanic eruptions and meteorological conditions. There is also some evidence for generation by meteor strikes. Locations affected by tsunamis therefore tend to be close to plate boundaries e.g. the Pacific Ring of Fire, although the effects of the tsunamis may be experienced at large distances from the source.

The effects of tsunamis are generally not severe in deep water, where their height might only be of about 1 m. However, in shallow water where the wave height increases due to the shoaling effect, they may cause extensive damage due to two effects:

- Rising/falling water levels

- Large wave fronts, which can exceed 10 meters

AtoNs in deeper water will therefore probably be unaffected by tsunamis. However, damage is likely on structures such as rock lighthouses or operational facilities on the shoreline.

Examples of AtoNs damage due to tsunamis may be mentioned:

- Navigation buoys and the Great Basses Light and Little Basses lighthouses in Sri Lanka following the 2004 tsunami, where navigational buoys were lost around the coastline, and damage to the lighthouses included that to the fuel tanks and glazing.

- The five-storey high Scotch Cap lighthouse on Unimak Island, Alaska was completely destroyed in the 1946 Aleutian tsunami with the loss of 5 men (Patel and Patel, 2012).

A high magnitude and long lasting earthquake striking the areas close to the epicentre will undoubtedly cause a major disaster due to structural damage to existing infrastructure, basic services, the number of casualties and the victims who will see their daily lives collapsed by the extreme event experienced.

An earthquake may also lead to a tsunami that could damage floating and land-based navigational aids, port infrastructure and facilities. Although both events, separately, are particularly destructive, their effects can be combined or individual.

The waves of a tsunami can be extremely dangerous and devastating to low-lying coastal areas as they enter shallow water at high speed and hit the shoreline in a short period.  Tsunami impact time depends on the distance between the epicentre and the coast, ranging from tens of minutes to hours, and this propagation pattern will affect the countries near its origin.

Example of Chilean experience

A few minutes after the earthquake, the Pacific Tsunami Warning Center disseminates a tsunami warning to the Pacific Ocean, which will be issued to 53 countries across its basin, including Peru, Colombia, Panama, Costa Rica, Nicaragua, Antarctica, New Zealand, French Polynesia and the coasts of Hawaii.

Expansive wave

The height of the wave train will vary upon the area and the impact on the coastline, and it will depend on the type of coastline, slope and other features of the shoreline.

Damages

When such natural disasters occur, the personnel usually involved in navigational aid organizations, which in many countries are part of the Navy, is engaged in rescue tasks, community support and restoring basic services, as well as maintaining public order. This is therefore a critical scenario, where aids to navigation are not necessarily a priority.

Navigational aids located in the disaster area are also vulnerable to damage caused by an earthquake and subsequently a tsunami.  However, it is possible to identify or prioritize those affected aids based on parameters, such as type and size of the structure and average height above sea level.

### Emergency and contingency plan

In countries with a seismic culture, such as Chile, Japan or Cuba, where based on experience, special plans have been developed to respond to all types of disasters, aids to navigation are considered critical infrastructure for their contribution to safety.

The following is an example of the general guidelines to be addressed in the event of extreme environmental disasters such as earthquakes, tsunamis, typhoons or hurricanes.

#### Suggested actions

- Establish an emergency repairs unit made up by specialized personnel to inspect the affected area, in order to assess the condition of the navigational aids, according to the prioritization resulting from the variables of the structures and the average height above sea level.

- Once an initial assessment is completed, a basic corrective maintenance plan should be implemented using adequate equipment to restore damaged aids to normal.

- Provide a team of AtoN trained personnel equipped with the appropriate communications and personal protective equipment, and ready to operate in the event of an emergency.

- Keep written records with the following information:

- List of personnel and their contact telephone numbers and addresses.

- List of navigational aids installed at a certain maximum height above sea level, according to areas of action.

- List of large navigational aids, mainly concrete, according to areas of action.

- List of navigational aids powered by the public lighting network.

- Paper charts with flooding plan for the main ports of the country.

- Keep records of specific beacons.

- Provide operational VHF communications equipment and flashlights.

- Keep a lighthouse relighting kit, with minimum working tools to be shipped as quickly as possible.

- Stock minimum spares inventory and power supply for relighting of navigational aids.

- Implement consumables and spare parts warehouses to supply the necessary equipment.

- Provide a first aid kit

#### Post-event evaluation

- Communication systems may be suspended right after an earthquake, and internet, mobile and landline services will not be available.  VHF and HF should be considered as a reliable option.

- Radio and television may be the only means of communication available to learn about the status of the catastrophe in the early stages, which could be used as a starting point to respond to uncertainty among local people.

- In addition, both urban and interurban roads may be damaged partially or totally, and thus fuel supply may be affected.  Basic services such as electricity, drinking water and gas, may remain unavailable during the first days, even weeks, depending on the existing operational capacities, also for safety reasons.

- Depending on the materials used in fixed navigational aids such as fiberglass or polyethylene towers or beacons, they may not be damaged by the seismic activity but they will be potentially impacted by the tsunami, which may exceed ten meters upon sea front slope.

- Large old navigational aids made of concrete and iron, which have been manufactured prior to anti-seismic regulations and have been exposed to the effects of the marine environment, may result greatly damaged by the quake even resulting in destroyed structures.

- It should be noted that personnel responsible for the maintenance of aids to navigation might be needed to perform support tasks to assist local citizens, and the navigational aids would not be an immediate priority.

## TIDES

Usually not represent a critical situation for aids to navigation, due to it has been designed considering the tide amplitude, but in case of extreme values the floating aids to navigation can be affected, especially on their position when are demarking a narrow channel.

### Extreme Tidal

Extreme tidal phenomena can be manifested in the maritime area as well as well as in river areas. In both cases they can occur because of clearly identified, well-known and, to some extent, foreseen situations or events.

An extraordinary event of hydrometrical value is associated to a water rising or fall (event)of the sea created by the simultaneity of the astronomic and the meteorological tide.

The tide range or scope can be measured as the vertical difference between a high and a low tide. The most extreme extraordinary tide values occur according to the lunar phase (full or new moon) when the gravitational force of the sun and the moon are in phase, this kind of tide is known as Spring Tide.

| Location | Country | Tidal Range (feet) | Tidal Range (meters) |
| --- | --- | --- | --- |
| The Bay of Fundy | Canada | 38,4 | 11,7 |
| Ungava Bay, Quebec | Canada | 32 | 9,8 |
| Avonmouth / River Severn | England | 31,5 | 9,6 |
| Cook Inlet, Alaska | USA | 30,3 | 9,2 |
| Rio Gallegos | Argentina | 29 | 8,8 |
| Hudson Bay | Greenland | 28,5 | 8,7 |
| Granville | France | 28,2 | 8,6 |
| Magellan Strait | Chile | 28 | 8,5 |
| Cancale | France | 27,8 | 8,5 |
| Iles Chausey | France | 26,9 | 8,2 |

Extreme tidal values

Since the astronomic tide is the periodical variation of the hydrometrical level as a response of the gravitational interactions between the sun, the moon and the earth, it can be known with acceptable accuracy. Moreover, it is published by the responsible organizations for its establishment and the dissemination of the document globally known as Tidal Table.

Conversely, the meteorological tide has its origins in the daily or seasonal variations of the weather conditions which can occur periodically and can be foreseen.

In river flow regimes, an extraordinary event of hydrometrical values is associated with an increase or decrease of the rainfall cycle occurring in the inflow basin, which is strictly linked to climate change.

#### Extreme Lowest Astronomical Tide

The astronomical tide, a rare event which can be commonly visualized in coastal areas, occurs when the sea moves away from the shore in distances which can be measured by the human eye. This rare natural phenomenon together with the gravity force of one of the particular lunar phases (full moon) applies on the water masses and to the climate condition, named as Lowest Astronomical Tide (LAT), occurs monthly with a different intensity. However, there exist instances in which the simultaneity effects impact negatively in the position of buoys and can make it become crucial for the navigation in access channels, due to the real buoy position can move transversely of channel alienation. Another environmental conditions could generate the same impact negatively in the position of buoys would be the wind speed and direction.

#### Floodings

The sustained and gradual increase of the water volume in rivers in which a waterway develops can generate, when the hydrometrical height keeps extreme values, negative consequences in the signalling system due to the possible influx of aquatic vegetation or logs, and their action on the floating signals, since they can get entangled in the anchoring system resulting in the signal anchor dragging, the possible rupture of the anchoring or even though, the sinking or loss of it.

Regarding the fixed signalling system (beacon) it can occur than in the areas where these signals are installed, they can potentially be exposed to erosions or collapse which may affect the stability of these signals. Consequently, it is advisable to make an adequate selection of the model of the AtoN to be used, so that its design minimizes these effects.

Negative effect of floating vegetation

Likewise, ship sailing can be affected by the excessive growth of the hydrometrical level, the loss of manoeuvring due to a higher speed of the current, lack of reference of the coastline, distortion of the visual marks and the radar image which is interpreted by the mariner during sailing.

River coast flooded

## CURRENTS

They are given by the permanent movement of translation of a mass of water defined by its course and speed. Its origin is attributed to three main causes such as density variations within the water, winds and tides, generating wind or surface currents and tidal currents.

### Superficial currents

The breaking of the waves generates a current fundamentally parallel to the beach, which is a function of the angle with which the waves approach the coast (currents of oblique incidence) and of its wave height. These currents, called longitudinal currents, are of special importance in the equilibrium disposition of a beach and, more specifically, in their plan form, given their important capacity to transport sand.

AtoNs could also be affected by the increment of the superficial current, mainly on buoys. It affects them in different ways according to their design. They may be driven away as far as possible from the sinker or tilted to the extent of completely losing their verticality and hence affecting the AtoN published visual, luminous and radio electric range.

### Tidal currents

A tidal current is generated with the rise and fall of the tide in places near the coast. The current is created by the vertical movement of the tide near the shore, causing the water to move horizontally. When the current floods, the tide moves towards the land and away from the sea; when the current ebbs, the tide moves towards the sea and away from land.

In some specific places, the tidal currents can move at very high-speed creating swirls that can be seen on the water surface. In these cases, mariners must follow the recommendations and guidelines published by the local maritime authorities while navigating these waters.

Places like the Chacao Channel, which separates the Chiloe Island from the mainland, experience tidal currents with an average of 8 to 10 knots in the middle of the channel. In the Kirke Channel, near Puerto Natales, tidal currents have also been registered with an average of 10 knots during flood and ebb periods. In this place, in 2014, the MV “Amadeo I” wrecked after crashing the sea bottom rocks due to the fast currents, among other factors.

Kirke Channel and Amadeo Ferry

In 2019, the Chilean Maritime Authority installed a current meter which sent, through VHF Channel, meteorological and current information transmitted by voice to enhance navigation safety through the channel.

## GEOPHYSICAL CONDITIONS

### Soil and Coastal morphology changes

The changes that take place in the seabed and fluvial coasts require continuous bathymetry and topography work on behalf of the Competent Authorities in order to keep nautical charts updated.

Eventually, the generation of sandbanks or the collapse of the coast associated with important sedimentation and erosion processes, typical of fluvial behaviour, can affect the stability of the fixed aids to navigations (beacons) installed on the coast. For this matter, a plan for continuous relocation of floating and fixed aids to navigations due to morphological changes is required.

Specifically, in buoys, sedimentation processes linked to sediment transport produce significant deterioration in the anchoring system which leads to a higher rate of damage due to turbidity.

Erosion process at the coast

#### Soil geology

Is common that floating aids to navigation marking rivers be difficult to maintain or remove due to marine or river current carrying huge quantities of sediments generating sandbanks over the position of the sinker. In those cases, during maintenance tasks, when it is necessary to remove the floating aid to navigation, it can be difficult due to the sinker may be covered by a large amount of sediment or the river may be frozen in winter, and the ship's crane or other lifting means may not serve to remove it and the mooring chain must be cut, which would generate an environmental condition to consider.

#### Hydrography

The environmental variables measured in extreme values affect the hydrography, for example, the loosening of large masses of ice which may imply a risk to navigation, extreme value of low tides, high tides or flooding, which can substantially modify the morphology of the canal bed due to the high associated sedimentation rate,  the coast erosion with the possible loss of fixed signals installed on the coastline or the need to relocate them. Because of all these factors, it is necessary for the mariner to ensure himself the availability of the cartography and the information regarding the updated state of the signalling system.

Even though all mariners know the compulsory use of the cartography, sailing directions and tide tables for navigation, it is possible that extreme high values of wind, current, sea waves, tide, reduced visibility because of the snow or heavy rainfalls, dust storm, among other factors, can mislead the master of the ship or else lead him to make a wrong manoeuvre. Reference: 1108-Ed.1-Providing AtoN services in Polar Regions.

## CLIMATIC CONDITIONS

Extremely hot and humid environments can generate extreme weather events particular to that area. This may include but is not limited to tropical storms, fog, monsoons, cyclones, sand/dust storms, seasonal winds and an increased chance of lightning activity. These weather events present significant engineering and operational challenges and there may be design and installation requirements required in these areas that are not required elsewhere. Reference: 1136-Ed.1-Providing AtoN Services In Extremely Hot and Humid Climates

### Temperature

The effect of the temperature on the aids to navigation and its components are widely known, and its incidence depends on the range of magnitude. In order to assess the effects that the extreme temperature values cause on the aids to navigation, several specifications have been developed  and all the content included in Guidelines 1108 “Providing AtoN Services in Polar Regions”, 1067-0 “Selection of Energy Systems for Aids to Navigation and Related Equipment” and 1067-3 “Energy Storage for Aids to Navigation” has also been considered.

Under normal conditions, the selection and design of the energy system to be implemented in the aids to navigation (AtoN) will depend on the available options for the generation and storage of energy to be used in the AtoN. Reference: Guidelines 1067-0 “Selection of Energy Systems for Aids to Navigation and Related Equipment” and 1067-3 “Energy Storage for Aids to Navigation” establish that when the external provider of energy is difficult or impossible to be installed, the solar energy, the wind energy, or any other source of renewable energy should be considered as the succeeding best option and, in such situations when the renewable energy is not feasible to be used, primary batteries can be used instead.

Tables which serve as a guide for the election of the energy system and for the recommended storage system considering extreme temperatures as one of the variables for the analysis can be found in the guidelines mentioned before.

These alternatives of energy generation will be affected by the extreme temperature values which can be measured in the installation or functioning sites. Extremely low or high temperatures can affect the performance of the external equipment: it can shorten the expected useful life of the equipment and make certain materials become fragile, compromising the ingress protection index (IP##) provided by the manufacturer or even the structural integrity.

Moreover, the selection of lubricants to be used in movable parts should be considered. Examples can be mentioned as follows:

- They will give a poor performance in the component of the energy system, freezing of minor batteries. Hence, major batteries should be used;

- Ice generation cycles, freezing of the axis of the wind generators;

- The formation of ice in antennas and the concentration of snow can cause several problems in different systems;

- Low performance on the components of the energy system will be due to low temperatures;

- Low temperatures can also generate cycles of ice generation;

- High temperatures can affect the useful life of the battery.

#### Operating temperature

The range of temperature during the operation experienced by the battery will considerably affect its useful life and it is a significant factor in the election of the battery.

Batteries should operate at the specified temperatures stated by the manufacturer. Its operation outside these ranges will have a side effect in its capacity and useful life (life cycle) and it can also be hazardous.

Calculating battery life cycles depend on several factors such as maintenance, percentage of discharge, battery temperature, amount of times the battery is discharged, buzzing, etc.

A high temperature of the cell can be generated by the environment temperature or else by an excessive charge. In both cases the possible effects can be the following:

- Ageing acceleration,

- Spontaneous sulphation,

- Active matter dissolution.

Lead Acid Batteries ageing curve in relation to the temperature

#### Storage temperature

Regardless the technical considerations according to the preservation in stock, it is important to highlight that, at the moment of receiving the refreshment load, the storage temperature should be lower than 25°C.

When batteries are stored over long terms and at temperatures which are higher than 25 °C, together with large storage periods without a refreshment load, this results in a diminished capacity. Hence, the useful life of the battery will be shorter.

During storage the following instructions should be considered:

- Automatic discharge rate in relation to the environment temperature. The lower the temperature, the lower the discharge of the battery will be.

- Batteries should be stored in a dry, clean and ventilated place at an environment temperature between (0° C to 25° C). Reference: 1039-Ed.2-Designing Solar Power Systems for Aids to Navigation

### High Ultra‐Violet Levels

High UV light levels in prolonged periods of strong sunlight can cause degradation of material properties including colour retention, plastic lenses, steel and plastic buoys, structures, coating systems and electrical/electronic equipment and fixtures. UV exposure can also be a significant risk to workers and requires careful management and specific mitigation controls. Reference: 1136-Ed.1-Providing AtoN Services In Extremely Hot and Humid Climates

Careful design considerations should be made when using plastic buoys in very hot climates (equatorial regions) as some effects could severely impair the performance of the equipment in place. The following are examples of problems that may result from prolonged exposure of plastic material to high temperature. They should be considered to help determine if the design is suitable for the application (this list is not intended to be exhaustive).

- parts that bind or corrode from differential expansion of dissimilar materials;

- materials change in dimension, either totally or selectively;

- packing, gaskets, seals, etc. become distorted, bind, and fail causing mechanical or integrity failures;

- gaskets display permanent set;

- closure and sealing strips deteriorate;

- shortened operating lifetime;

- colour fading, cracking or crazing of plastic materials;

- outgassing of plasticizers.

The high rates of solar radiation (UV) in these regions can cause surface temperatures to increase by 15 to 30°C above ambient temperatures and surface temperatures can reach 80°C. As a result, it is important to consider studies made by various manufacturers comparing material degradation, reaction and resilience under these extreme conditions. Reference: 1006-Ed.-4-Plastic-Buoys

High Ultra Violet effect on red buoy

### Humidity and precipitation

High humidity environments present issues to both AtoN equipment and personnel. Water ingress and the effect of humidity and the buildup of condensation in AtoN equipment can cause electronic failure, thereby negatively impacting availability, reducing reliability and impacting service life.  Increased humidity can also have a negative effect on the worksite, worker comfort and output efficiency, thereby increasing the risk of incidents and impacting quality. Reference:1136-Ed.1-Providing AtoN Services In Extremely  Hot and Humid Climates.

In order to protect the integrity of the equipment installed in areas of extreme humidity, increase its lifespan and ensure its reliability, it is necessary to prevent the condensation cycle from starting during the installation process. In order to achieve this, an IP (Ingress Protection) level should be specified. Reference:1136-Ed.1 - Providing AtoN Services In Extremely Hot and Humid Climates - Annex A

Taking into account that highly-humid and saline environments produce corrosion in the structure, the installed equipment and its connectors, causing a high probability of malfunction, it is essential to guarantee the water tightness of AtoN devices to avoid the generation of condensation cycles, and so, ensure the reliability of the aid to navigation.

The UK & Irish GLAs produced a detailed report: Building Conditioning of Lighthouses, Accommodation, Outbuildings and Associated Structures (IGC5 TASK GROUP REPORT, 2009) that described these effects in some detail, with suggested monitoring approaches. Reference: 1007-Ed.1.1-Lighthouse Maintenance

### Icing

The navigation assistance in ice-covered ports during freeze-up faces great difficulties to the floating AtoN’s as well as fixed AtoN’s. For the buoys of common use fail to function effectively due to their movement, drifting away, damage, going under ice and even sinking in case of their encounter with sea ice through squeeze, clash, entrainment, frozen adhesion and fiction, and for the light range and visual effect of light beacons and other marks on land decrease due to heavy snow and fog.

In view of the fact that AtoNs may be damaged by the collision and extrusion of the floating ice, or be covered by the floating ice during freeze-up, the number of buoys instalment may be decreased during the winter season, restricted only at the key points of the fairway to reduce the probability of collision between vessels and buoys. Alternatively, special designs can be used which generally withstand ice forces.

Effect of ice on buoys

Ice sheets grow and shrink with the seasons and ice-charts may need to be updated frequently. Consideration should be given as to how this information can be managed in an efficient and cost-effective manner to ensure the mariner uses the most up to date information.

The growth and reduction of the ice sheet can also result in land-based systems having to be installed on permanently stable, high ground which may potentially be at sub-optimal locations, distant from the area of navigable waterway being served. Reference: 1108-Ed.1-Providing-AtoN-services-in-Polar-Regions

ICE incidence on AtoN structure

#### Typical problems

Fixed AtoN’s on land

- The mark could be covered by ice which deforms the visibility of the mark and light.

- Ice or snow accumulation on the glasses of lantern rooms may severely affects light visibility.

Additional problems with fixed AtoN’s at sea

- Fixtures and aerials or other construction elements may be damaged by ice if they are placed near sea level

- Moving ice, when breaking at an AtoN or hitting it may cause high acceleration figures which may destroy equipment

Port entrance mark covered by ice - Safety rail bent by ice on Tallinnamadal lighthouse

Floating AtoN’s

- To use buoys without producer’s approval on ice conditions is problematic due to stresses caused by ice.

- Submerged buoy under ice is very hard to be seen and may damage vessels hitting it. Considerations should be made to give notice to the mariners that availability targets aren’t met in ice conditions.

- Ice may damage buoy lanterns and therefore special lanterns or ice protection for lanterns is needed. They are usually a lot heavier and may create problems with buoy stability.

- Moving ice may tilt the buoy at such degree that light from the buoy can’t be seen.

- When ice is moving the buoy may be submerged so the buoy design should be conical or cylindrical without any barriers what ice could damage. Otherwise the ice could cause severe damages to the buoy. That’s why some countries have chosen not to use topmarks.

- If the buoy hasn’t got inside structure to support buoy hull the buoy may get severely damaged. In case of steel buoys generally reinforcement ribs are used and in case of plastic buoys the empty voids must be filled with closed cell foam.

- As coefficient of linear thermal expansion of PE (Polyethylene) is approximately ten times bigger than in case of steel there may be problems with PE buoys in case of large empty compartments. There may be damages to the buoy hull caused by environment temperature difference as inside the buoy may develop over pressure or vacuum.

- Heavier gravity sinkers are needed to hold the buoy on its position in moving ice condition.

- The depth of the water should allow the buoy to submerge under moving ice.

- When the buoy is submerged or the lantern is damaged the signals of the monitoring devices aren’t able to reach receivers.

- In case of shallow waters and thick ice the ice may touch the bottom of the sea and move objects there (including anchors).

- Ice often scrapes off retroreflecting films and bird spikes used on buoys.

- Retrieval of the off station floating AtoN is sometimes impossible because buoy tenders are not able to be operated in severe ice conditions.

- Sometimes it may be problematic to identify the colour scheme of a buoy in ice.

A buoy with ice protection over polycarbonate cupola - Reinforcement ribs on steel buoy - Damaged plastic ice buoy without foam to support it's structure

For example, in case of icing, could implement buoys built with fishbone stainless steel skeleton, and a streamlined three-section structure with a cylinder in the middle and a cone at both ends using high polymer polyethylene material. The surface color of the buoy does not fade or fall off even after a long-term use under harsh conditions, and it can effectively resist the impact and collision of floating ice, and float under the ice surface passively when it come across the large ice chunks.

Appearance                              Skeleton

## ATMOSPHERIC CONDITIONS

### Visibility

Fog is an atmospheric phenomenon in which the visibility of the sky is reduced due to the high presence of smoke, dust or any other particles suspended in the air. According to the World Meteorological Organization the darkening on the horizon can be distinguished between mist, ice mist, vapour mist (steam), fog, smoke, volcanic ash, dust, sand or snow.

However, the turbidity can generally be measured by means of the Pollutant Standard Index (PSI) which states the air quality or the pollutant level in it. This index can vary according to the country which consults it, since different indexes are used all over the world, such as the Air Quality Health

#### Difference between haze, fog and mist

These three phenomena entail a reduced visibility due to climate conditions (in relation to fog and mist) and in relation to the air pollution (in relation to the haze). This is the main difference between these three concepts, fog and mist are formed by suspended drops of water whereas haze can be formed by extremely small and tiny particles of air suspended.

The natural phenomenon of mist occurs when the hot air “crashes” against cold surfaces and the humidity “thickens” or condenses resulting in small amounts of water in the air.

Although the mist and the fog are similar phenomena, there is a difference between each other; the visibility distance, i.e. if the visibility distance is of one kilometre or shorter than a kilometre it is often named as mist, but if it is larger than a kilometre, it is called fog.

Presence of fog in a waterway

### Dust

Guideline 1136 “Providing AtoN Services in Extremely Hot and Humid Climates” considers the incidence that dust may have in the different physical properties, industrial as well as natural, which can block flashlights or day marks of aids to navigation and which may affect the luminous and visualization range, as well as the solar panels which, if covered in dust, can reduce their energy generation capacity. Furthermore, extreme dust also present risks related to health, such as respiratory problems. Dust is considered as a way of transporting pollutants and, as a matter of fact, it affects the safety of the personal in charge of assisting the AtoN.

In areas which affected by dust, the service provider or Competent Authority needs to re-consider the visit to the station where the effect of the dust will impact the general performance of the AtoN such as light, daymark, power generation (Solar Panel), and any other part which may be affected. The visit interval should be adjusted to be practically and economically feasible.

Generally, sandstorms start as a dark orange cloud followed by heavy winds which can make the Competent Authority to impose restrictions on the navigation in canals, manoeuvring areas, entry to ports or harbours, among others.

Presence of fog dust on AtoN device

### Smoke

In particular regions, agriculture producers’ resort to the controlled burning of wild vegetation in those areas placed near inner navigation areas aiming to regenerate the soil and make it productive. However, these fires can occur accidentally forming clouds made of smoke which causes the loss of visibility for the mariners, as well as health disorders for the living beings inhabiting the area (fauna and human beings).

The simultaneity of the environment variables in extreme conditions plus the human action help the propagation of the flames, and, at the same time, the propagation of the smoke, for example:

- Droughts,

- Winds,

- High temperatures.

Presence of smoke in a waterway

#### Incidence in navigation

The main impact in navigation, associated with the loss of visibility or the detection of limited visibility due to the mist, haze, dust, sand or smoke, is the closure of the maritime transport by the Competent Authority.

Depending on the port management, the closure may not affect the vessel traffic or it can impose a restriction in navigation, such as delays in operations during a period of hours and to reopen, again, after this period has finished, once the visibility conditions have improved.

The loss or reduction of visibility due to the presence of mist or fog do not affect the useful life of the aid to navigation. However, it reduces the visual range and affects the safety to navigation.

Guideline 1090 “The Use of Audible Signs” tackles the loss of visibility issue to the presence of mist issue and defines this phenomenon as follows: “establishment of the considerations under which an audible signal can be used and the disadvantages of its use due to the complexity to identify the direction or vicinity from where the sound comes from, as well as stating the patterns in relation to usual range and nominal use.”

At present, there exist electronic aids capable of warning the mariners in situations of low visibility. Hence, its implementation depends on the organization which grants the Service of Aids to Navigation which may not be the same as the service performed by the Maritime Authority. In this light, the mariner can be confused: on the one hand, it is possible that the aids for navigation placed on the bridge encourage him to navigate under low visibility conditions willing to finish his trip in the agreed time and, the Competent Authority or Port Authority restricts its navigation permissions until the visibility conditions change, being this the criterion to be established.

An example of this is how the affectation of the meteorological variable of fog when it occurs in an extreme situation conditions the operability of ports and the safety of navigation.

In the Port of San Antonio, one of the main ports of Chile, the occurrence of fog bank phenomena is very common during the morning until after noon, while in the afternoon an anticyclonic margin is formed with winds from the SW, which generates the removal of the fog although it causes adverse conditions in the state of the sea, then, the fog condition during the morning and the wind during the afternoon converge with another problem that is the narrowness of the entrance channel to the port.

Fog bank

In this context, the hours authorized for entering to the port are reduced to night hours, which is why the port must focus on finding a set of solutions oriented to new technologies, such as the signalling of the entrance channel with AIS AtoN Virtual with backup systems added to the efforts of work programming and entry of ships to the port.

#### Incidence in energy generation

The phenomenon related to the presence of haze, fog, dust or smoke affects the performance of the solar panels since the remaining insolation charging hours can be reduced in such places where the presence of haze and fog events occur during autumn or winter.

The concept of incidence of the visibility mentioned in Guideline 1108 “The Challenges of Providing AtoN Services in Polar Regions” takes into account the different geographical sceneries or navigational routes placed in high or low latitudes (Arctic or Antarctic navigation). Visibility on navigation depends on the adjustment of the human eye or on the contrast between the darkness during the day in winter and the brightness during summer months; both factors affect visibility in navigation.

## WATER CONDITIONS

The physical, chemical, and biological composition of marine waters can influence the rate of deleterious effects on both fixed and floating AtoN; such effects are discussed in the following paragraphs.

### Corrosion

Salinity, sulphate content, temperature, dissolved oxygen, and pH can all separately, and in combination, influence the corrosion of steel AtoN. Generally, higher temperatures as experienced in tropical waters, and higher levels of dissolved oxygen will combine to increase corrosive mechanisms.

The corrosion of steel in seawater is an electrochemical process, which is exacerbated by increased salinity.

pH of the water also has a variable eﬀect on corrosion. Typically, corrosion rates are high at lower pH due to acidic corrosion and at a higher pH, the corrosion can be caused due to a reaction known as caustic embrittlement. pH of waters can be locally affected by artificial influences such as discharges from oil refineries and chemical plants.

Accelerated low water corrosion (ALWC) of steel structures is found in certain regions. ALWC is an aggressive form of microbially influenced corrosion that may occur on steel in estuarine and marine structures. The rate of corrosion caused by ALWC can be ten times the normal rate of seawater corrosion. Sulphate reducing bacteria, which can thrive under marine growth, are a contributory element. ALWC is characterised by bright orange surface deposits underlain by black slime. The effects are most prevalent in tidal waters and ALWC is often observed around the low water level.

Marine organism can also be the cause of bio-corrosion of steel parts. This bio-corrosion affects both the submerged parts of the buoy and the mooring chains.

### Abrasion

Mobile sediments made up of abrasive sand such as basalt, can have an abrasive effect on all submerged materials including fixed, marine based AtoN foundations and floating AtoN mooring chains. Abrasive sediments can be moved by the mass of water in stormy weather or speedy current tide and can damage protective coatings and abrade the AtoN material. In steel, this can lead to an entry point for corrosion of bare metal. Steel mooring chains can be damaged by the grinding of granular sediments between the chain links and can undergo abrasion when in contact with the seabed they affect the lower slice of the mooring line. In quiet sea conditions, it is the movement of the ground mooring chain on the sea bottom that undergoes this abrasion while the buoy moves.

Abrasive particles in suspension in the water can damage mooring lines made of steel chains or synthetic ropes. This situation is generally encountered in rivers or estuaries, carrying these particles. Extremely turbid waters can also make inspection and maintenance of fixed, marine AtoN difficult and may require the use of specialist divers who are familiar with the structure.

Abrasion chain

### Impact damage

Macro-waste can be natural such as tree trunks or tree branches, or artificial such as drifting fishing nets or drifting steel cable. They are mainly carried by rivers. They can get caught in mooring lines made of chain and generate significant stress. They can also damage synthetic mooring ropes.

### Marine Growth

Very warm seawater, associated with extremely hot and humid climates, can cause the rapid growth of dense andsometimes destructive marine organisms on both buoy moorings and the foundations of fixed structures. This can increase the frequency and cost of maintenance, can adversely impact buoyancy and accelerate corrosion. Reference: 1136-Ed.1-Providing AtoN Services In Extremely Hot and Humid Climates

Fouling from marine growth may be so severe that the buoy will have to be lifted and cleaned at regular intervals. During servicing, fouling should not be removed with mechanical scrapers.  These will cause serious damage to the paint film and shorten the life of the paint system.  Remove fouling with water jetting using a suitable pressure that will not damage the paint system. Reference: 1077-Ed.1-Maintenance of Aids to Navigation

Marine growth

Guideline 1036 “Environmental Management in Aids to Navigation” develops the impact that the aids to navigation have in the environment and establish the general patterns for the management of waste pollution, acoustic pollution, as well as luminous pollution, and the protection of habitats, flora and fauna, among others.

## Wildlife

### Bird fouling

Bird fouling comes primarily from sea-birds landing and roosting, or attempting to land and roost, on an AtoN site. The discharge of faecal matter is what causes the bird fouling, but it can also be related to shedding of feathers, nesting debris and presence of rotting food. Bird fouling can have, among others, the following detrimental effects on AtoN sites:

• Excessive bird lime coverage on lanterns or optics, causing obstruction of the light source, resulting in reduced nautical range or in severe cases, total outage of the AtoN;

• Excessive bird lime coverage of solar panels, reducing the active area of the panel and severely limiting battery charging capacity, which can lead to negative effects on night-time signalling functions of the lantern and may eventually lead to total battery discharge and subsequent outages of the AtoN;

• Bird lime coverage on lighthouses or other daymarks can cause a change in the colour, severely affecting the ability of that AtoN to provide clear information to the user;

• Bird lime is highly caustic and can increase corrosion rates on AtoN structures, fittings and components, resulting in accelerated deterioration and reduced life span, higher maintenance costs and unsafe structures;

• Bird fouling on any site generally pollutes and contaminates, causing a number of associated issues.

Bird fouling on buoys

Guideline 1091 “Bird deterrent and bird fouling solution” develop the different methods to mitigate bird fouling on AtoN structures, such as implementing commercial products and deterrent systems, the application of engineering solutions, structural changes or revised installation methods where impossible to deter bird colonies, minimizing the negative effect of birds fouling. The exact method should be tailored to suit a particular site, situation and, in some cases, may need to be designed to suit the visitation habits of a particular species of bird. Reference: 1136-Ed.1 Providing AtoN Services In Extremely Hot and Humid Climates

Consideration with bird fouling in hot climates is that it can be baked on due to the extreme heat, making it very difficult and time consuming to remove. Intensive cleaning can create dust which poses inhalation related health and safety risks to workers. Bird deterrents should be used wherever possible to prevent this problem.

Some ways used to prevent this effect:

- Install the most appropriate bird deterrent for the location and bird species. ‐ This action prevent the bird fouling

- Paint over the surface colour with a special ant graffiti product. ‐ This action not prevent the bird fouling but ensure an easily remove with water pressure.

Ant graffiti product application over the surface color

## Incidence in people´s health

Dust and smoke have a harmful effect on people’s health. Besides eye irritation and nasal congestion, it can also affect lungs and cause severe headaches. Moreover, both phenomena can deteriorate allergic and asthmatic people’s life, as well as affect people’s sense of smell.

Some hot and humid climates promote high vegetation cover and growth. The need to constantly control and clear excessive vegetation so that does it not obstruct the operation of an AtoN can be an issue in these regions. The presence of dangerous fauna in hot and humid regions, such as marine stingers, snakes, scorpions, spiders, venomous insects, crocodiles and sharks can present serious risks to workers and the ability to safely access, operate and maintain AtoN. Reference: 1136-Ed.1-Providing-AtoN-Services-In-Extremely-Hot-and-Humid-Climates

Risk on maintenance task buoy

# ACRONYMS & DEFINITIONS

## Acronyms

IMO	International Maritime Organization (Acronym style)

MBS	IALA Maritime Buoyage System

AtoN	Aids to Navigation

PE	Polyethylene

## Definitions

The definition of terms used in this Guideline can be found in the International Dictionary of Marine Aids to

Navigation (IALA Dictionary) at (http://www.iala‐aism.org/wiki/dictionary).

# REFERENCES

Guideline 1006-Ed.4 “Plastic Buoys”

Guideline 1007-Ed.1.1 “Lighthouse Maintenance”

Guideline 1039-Ed.2-Designing Solar Power Systems for Aids to Navigation”

Guideline 1066 “The Design of Floating Aid to Navigation Moorings.”

Guideline 1067-0 “Selection of Energy Systems for Aids to Navigation and Related Equipment”

Guideline 1067-3 “Energy Storage for Aids to Navigation”

Guideline 1090 “The Use of Audible Signs”

Guideline 1091-Ed.2 “Bird deterrents and Bird Fouling Solutions”

Guideline 1094-Ed. 2 “Daymarks for Aids to Navigation”

Guideline 1098 “Application of AIS-AtoN on buoys”

Guideline 1099 “Hydrostatic Buoy Design”.

Guideline 1108 “The Challenges of Providing AtoN Services in Polar Regions”

Guideline 1136-Ed.1 “Providing AtoN Services In Extremely Hot and Humid Climates”

Antonini A., Raby A.C., Brownjohn J., Pappas A. & D'Ayala D. (2019) Survivability assessment of Fastnet lighthouse. Coastal Engineering 150: 18-38 doi.org/10.1016/j.coastaleng.2019.03.007.

Patel, P.S., and Patel, V.M. (2012) A Literature Review of Effect of Tsunami on Lighthouse Structures, National Conference on Advances in Engineering and Technology (NCAET-2012), Gujarat.

Raby, A.C., Antonini, A., Pappas, A., Dassanayake, D.T., Brownjohn, J.M.W., D’Ayala, D. (2019) Wolf Rock lighthouse: past developments and future survivability under wave loading Philosophical Transactions of the Royal Society A doi.org/10.1098/rsta.2019.0027