Input paper:   ENG16-3.1.3.3

Input paper for the following Committee(s): 					Purpose of paper:

□  ARM	X  ENG		□  PAP					x  Input

□  ENAV	□  VTS							□  Information

Agenda item 			3.1

Technical Domain / Task Number 2	…………………………………

Author(s) / Submitter(s)			Ana Senado, Emilio Gonzalez, Maria Mota, Manuel Lopez (EUSPA)

# Galileo Timing and Galileo Authentication Service

# Summary

This input paper addresses two Galileo main topics that can be relevant for the following IALA ENG documents:

- Implementation of R-Mode on MF and VHF frequencies

- IALA_ENG_WG3_Guideline_on_Timing_and_Synchronisation_v0.2

On the first hand, the Galileo UTC Dissemination Accuracy is presented. Galileo defines a Minimum Performance Level (MPL) for UTC Dissemination Accuracy of 30 ns. The present input paper will show that the actual accuracy is far beyond the MPL, being around 6ns.  This excellent performance allows to use Galileo as an accurate source for timing synchronization.

On the other side, Galileo Open Service Navigation Message Authentication (OSNMA) service is explained. This service authenticates Galileo navigation message and it allows users to assure that the Galileo message is received from original Galileo satellites and not from malicious source, contributing to the resiliency of the position computed. This service can be analysed as an complementary source to be incorporated in the R-mode position solution, since some spoofing attacks can be identified by using Galileo OSNMA.

# Background

Global Navigation Satellite Systems (GNSS) (today GPS, Galileo, GLONASS and BeiDou) have become the primary source of positioning, navigation and timing (PNT) for maritime operations. Furthermore GNSS-based positioning is used by many systems on vessels, like AIS (Automatic Identification System), ECDIS (Electronic Chart Display and Information System), ARPA (Automatic Radar Plotting Aid), some GMDSS related equipment (Global Maritime Distress and Safety System) and other navigation sensors. Safe navigation, the protection of the marine environment and the efficiency of access to ports are highly dependent on the availability, continuity, accuracy and integrity of GNSS based positioning and timing.

However, it is well known that low power satellite-based systems are vulnerable to jamming and natural interference [1]. When GNSS is corrupted or unavailable, PNT information are seriously affected resulting in an increase in risks to the safety of navigation. Unavailable PNT data, even for short periods, results in numerous alerts raised by multiple systems on the bridge systems. Hazardously misleading information may occur in position errors that are large enough to have a severe impact on navigation safety but may be small enough to remain undetected and raise no alerts.

Within the overall e-Navigation strategy the IMO has identified the user need on improved reliability, resilience and integrity of bridge equipment and navigation information as one of the five prioritized e-Navigation solutions.

A variety of technological solutions provide the potential to serve this backup requirement; for example, within the radio frequency (RF) domain “Signals of OPportunity” (SoOP) can deliver possible solutions. This term refers to the opportunistic use of RF signals, typically communications signals, which already exist in the geographical area of the user receiver. While these signals are not primarily intended for positioning, a SoOP navigation receiver attempts to exploit them as such. Specifically, if each SoOP can provide a (pseudo-)range to the receiver from a known location, a trilateration position solution is possible. The use of such ranging signals from existing DGNSS maritime radio infrastructure is known as “R-Mode” (ranging mode). Even if it were impossible to derive a complete position solution from R-Mode (e.g. due to insufficient, less than three signals being present), the available pseudorange information combined with measurements from existing positioning systems can be combined to provide a position solution.

The IMO opened up the usage of multiple position fixing systems by the inauguration of the Multisystem - Radionavigation Receiver Performance Standard (MSC.401(95)) [4] and the associated GUIDELINES FOR SHIPBORNE POSITION, NAVIGATION AND TIMING (PNT) DATA PROCESSING (MSC.1/circ.1575), [5].

For the first time this new performance standard allows the combination of any recognised IMO World-Wide Radionavigation System (WWRNS) with terrestrial position fixing systems as well as wide area augmentation systems. The rising numbers of available ranging signals from any source benefit the determination of position accuracy and associated integrity.

The time accuracy at R-Mode stations is fundamental for accurate distance estimation between a user receiver and an R-Mode transmitter station. An error of 10 ns equates to 3 m distance estimation error. To enable a potential R-Mode accuracy of better than 10 m the synchronisation error should not exceed a 10 ns deviation to UTC (or the R-Mode time reference frame). There are different options for the type of station clock and the way of time synchronisation with UTC. At the end each maritime administration has to consider which solution fulfils their requirements. A cost-benefit analysis is needed for this purpose.

On this sense Galileo offers a free and cost-effective solution to obtain an excellent timing performance below the 10ns. In addition, Galileo will offer an authentication service, that will assure that the Galileo message is received from Galileo satellite and not from any other malicious source.

# Discussion

This section presents

- The performance for the Galileo UTC Dissemination Service

- Galileo Open Service Authentication Service

## Galileo UTC Time Dissemination Service

GNSS satellites broadcast additional data which allow GNSS receivers to relate the GNSS time scale to international reference timescales. The precise atomic clocks in the Galileo system allow de delivery to users of very accurate timing data.

The OS UTC Time Determination Service provides Galileo users worldwide with direct and accurate access to the Universal Time Coordinated (UTC) reference. It provides both the integer offset between the Galileo System Time (GST) and UTC and the fractional GST-UTC difference at nanosecond level. This allows users to timestamp their data in UTC or local time and to precisely synchronise to UTC their clocks, which can be situated at very different locations in the world. [2]

The Galileo OS provides means for users to determine UTC by estimation of the GST-UTC difference using the parameters transmitted in the Galileo OS SIS. The accuracy in the UTC estimation, which is called the Galileo UTC Time Determination Accuracy, depends therefore on both the instantaneous GST determination error and on the error in the broadcast GST-UTC conversion parameters. This second component is defined as the Galileo SIS UTC Time Dissemination Accuracy.

Galileo Service Definition Document [2] sets the Minimum Performance Level (MPL) for the Galileo UTC Time Dissemination Accuracy to less than 30 ns.

Galileo SIS UTC Time Dissemination Accuracy MPL

| Minimum Performance Level | Conditions and Constraints |
| --- | --- |
| For each Single Frequency and Dual Frequency combination: < 30ns (95%) over all Age Of Data (AOD) | • For any healthy OS SIS • At any user location • Calculated over a period of 30 days • Propagation and user |

The performance of the Galileo UTC Time Dissemination Accuracy is excellent, down to the level of 6 ns, as it is presented in the Galileo OS Performance Reports [3]. It can be observed that for the whole reporting period, the accuracy has been around 6ns. This accuracy satisfies requirement for 10ns needed for R-mode.

Figure 1:Galileo UTC Time Dissemination Service Accuracy

## Galileo Open Service Authentication

Open Service Navigation Message Authentication (OSNMA) is a data authentication function for the Galileo Open Service worldwide users, freely accessible to all. OSNMA provides receivers with the assurance that the received Galileo navigation message is coming from the system itself and has not been modified.

OSNMA is authenticating data for geolocation information from the Open Service through the Navigation Message (I/NAV) broadcast on the E1-B signal component. This is realised by transmitting authentication-specific data in previously reserved fields of the E1 I/NAV message. By using these previously reserved fields, OSNMA does not introduce any overlay to the system, thus the OS navigation performance remains untouched.[4]

# References

R-Mode Baltic- Baseline and Priorities: https://www.r-mode-baltic.eu/wp-content/uploads/2020/06/R-Mode-Baltic-Baseline-and-Priorities.pdf

Galileo Open Service Service Definition Document: https://www.gsc-europa.eu/sites/default/files/sites/all/files/Galileo-OS-SDD_v1.2.pdf

Galileo Open Service Performance Report; Q1 2022: https://www.gsc-europa.eu/sites/default/files/Galileo-OS-Quarterly-Performance_Report-Q1-2022.pdf

Galileo Open Service Navigation Message Authentication Info Note: https://www.gsc-europa.eu/sites/default/files/sites/all/files/Galileo_OSNMA_Info_Note.pdf

# Action requested of the Committee

- The committee is invited to consider the information provided within this paper and within the presentation attached to support the following documentation:

- Implementation of R-Mode on MF and VHF frequencies

- IALA_ENG_WG3_Guideline_on_Timing_and_Synchronisation_v0.2

- Annex

Presentation attached

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain