Input paper: ENG16-3.1.2.6

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	X ENG		□  PAP					□  Input

□  ENAV	□  VTS							X  Information

Agenda item 			n.n

Technical Domain / Task Number 2

Author(s) / Submitter(s)			CHINA MSA

Use of Modern Equipment in Traditional Lighthouses

# Summary

In 2018, ENG WG2 decided to develop the Guideline on Use of Modern Equipment in Traditional Lighthouses, and incorporated this guideline into IALA Work Plan 2018-2022. At the ENG15 meeting, the preliminary daft outline of Guideline on Use of Modern Equipment in Traditional Lighthouses has been determined, and WG2 expected to receive more relevant cases and experiences of the use of modern equipment in traditional lighthouses at the ENG16 meeting for their consideration. This task has now been extended to IALA Work Plan 2023-2027. China MSA analyzed and summarized the modern equipment practice of traditional lighthouses all over China, and suggested that the experience could be applied to "3. Traditional lighthouses as platforms" and "4. Different types of modern equipment that may be installed on traditional lighthouses" in the above outline for consideration by Committee.

## Purpose of the document

Provide an information reference for the ENG Committee to develop "Guideline on Use of Modern Equipment in Traditional Lighthouses".

## Related documents

ENG15 WG2 TG-2.6.1 Draft Guideline on Use of Modern Equipment in Traditional Lighthouses.

# Background

At the ENG15 meeting, the preliminary daft outline of Guideline on Use of Modern Equipment in Traditional Lighthouses has been determined, including "Traditional lighthouses as platforms" "Different types of modern equipment that may be installed on traditional lighthouses" "Issues and concerns related to the use of traditional lighthouses" "Need of careful installation" "Refer to other IALA documents on use of modern optics in traditional lighthouses". To demonstrate the possibility and practicability of using traditional lighthouse as a modern technology platform, WG2 expected the participants to share more relevant cases and experiences for the preparation of the guideline.

# Discussion

## The significance of developing the Guideline on Use of Modern Equipment in Traditional Lighthouses

The lighthouse is a tower, or substantial building or structure, erected at a designated geographical location to carry a signal light and to assist marine navigation. The lighthouse can also provide a platform for other navigation facilities, emergency rescue and third-party cooperation, such as installing DGNSS station, radar or video surveillance, to assist in marine navigation and reduce the risk of marine navigation. The development of the guide can provide a reference for IALA members on the possibility of the traditional lighthouse as a modern technology platform, and provide guidance on modern equipment that can be installed on traditional lighthouses and the installation considerations of these modern equipment.

## China MSA provides the modern equipment practice of Chinese traditional lighthouses as a reference sample

On the basis of the analysis and summary of the modern equipment practice of Chinese traditional lighthouses, China MSA introduces some application cases of "traditional lighthouses as platforms for different types of services" and "the application status of modern equipment in traditional lighthouses" etc, provides experience reference for the content "3. Traditional lighthouses as platforms" and "4. Different types of modern equipment that may be installed on traditional lighthouses" in the outline of "Guideline on Use of Modern Equipment in Traditional Lighthouses".

# suggestion

It is recommended that the ENG committee use the information in the Annex as a reference in the preparation of the guideline.

# References

None.

# Action requested of the Committee

The committee is requested to note the information in annex and take action as appropriate.

- Use of Modern Equipment in Traditional Lighthouses

- Brief description

Efficiency, effectiveness and technological progress are the objective requirements of the lighthouse modernization process.

With the rapid development of radio navigation technology, the popularization of automatic control systems in the world, and the widespread application of new beacon lights and new energy sources, the performance of the lighthouse navigation services has been greatly improved. In addition, by installing different navigation aids equipment, the lighthouse becomes an integrated information platform for navigation service. In addition, by extending the service life of the lighthouse, the functions of the lighthouse other than navigation services, such as the social value, historical value, and popular science value of the lighthouse, should be developed. Through modern technological transformation of traditional lighthouses, the navigation ability and other benefits and efficiencies will be improved, which will ultimately benefit the public and inherit the lighthouse culture.

- traditional lighthouses as platforms

  - Mobile communication service

Lighthouses are generally built on coasts, harbours or watercourses because of their navigation service. Due to the curved surface of the earth, the tower should be high enough so that the light can be observed by passing vessels. In order to be easily distinguished by vessels, traditional lighthouses were usually built at the highest points of harbours, headlands and mountain peaks, occupying the best locations for transmitting "signals". Due to the characteristics of signal transmission, mobile communication base stations generally need to be built at the highest points to reduce the signal attenuation. Therefore, considering the factors such as geographical location, power supply, environmental protection and economic cost, mobile communication companies tend to cooperate with lighthouse authorities to build mobile communication facilities on lighthouses or use lighthouses for power supply. For example, in 2020, China Unicom built a 4G base station on Qianliyan Lighthouse, realizing a high-speed network series with Tuan Island, Dagong Island, Chaoliang Island, etc, achieving network coverage of 70 nautical miles in the main channel of Qingdao Port directly to the open sea. It not only solves the problem of signal transmission and power supply of the base station, but also lays a good foundation of network for the construction of intelligent maritime and intelligent AtoN, achieving a win-win situation.

- 4G base station on Qianliyan Lighthouse

  - Video surveillance system service

Some lighthouses are located on remote coasts, headlands and mountain peaks, and are unmanned. It is difficult for the staff to implement the regular inspection and maintenance work due to the rugged and winding mountain roads, sometimes the staff have to take boats to reach those  lighthouses. In 2012, a full coverage video surveillance system was installed in Nanshantou Lighthouse. Images can be transmitted back to the staff by installing a video surveillance system on the lighthouse, using microwave wireless transmission system or 4G wireless transmission system. This allows staff to keep track of the operation conditions of lighthouse equipment from thousands of kilometers away, and realize good fire safety management around the lighthouse. In addition, the video surveillance system installing on the lighthouse can also monitor the AtoN, vessels passing by and sea pollution conditions around the lighthouse, providing a good guarantee for navigation safety.

  - Cooperation between government authorities

In order to actively respond to the national initiative for departments and authorities of Hainan Province to work together to develop and construct the Hainan Free Trade Port, and support the work demands of the government, the AtoN authority and the local meteorological bureau signed a strategic cooperation agreement in the construction and operation of high-frequency ground wave radar network service, the local meteorological bureau has built a high-frequency ground wave radar automatic observation station next to Mulantou Lighthouse, the lighthouse provides power supply to support the construction of marine meteorology observation network.

- Application status of modern equipment in traditional lighthouses

In order to maximize the development and utilization of the value of traditional lighthouses, China has continuously improved the use efficiency of lighthouses by modernizing lighthouses and ancillary facilities to meet the requirements of modern intelligent shipping, and improve the social and cultural attributes of lighthouses.

  - Modernization of the lamp

The lamps used in many traditional lighthouses are historical lamps that have been in operation for decades or hundreds of years. They still perform the function of navigation, in the meanwhile symbolize the historical changes. But there are also some problems such as high failure rate, difficult in maintenance, difficult in management and high energy consumption. With the development of science and technology, the technology of navigation service has made a qualitative leap, higher requirements for the lamps such as intelligence and digitization have been put forward. We can replace the original light sources with LED light sources if they have problems of high energy consumption, short lifespan and difficult in maintenance. Due to various advantages of LED light sources, they are suitable to replace the thermal light source as lighthouse lamps. While reducing energy consumption, it facilitates the use of digital circuit control system, so as to realize online monitoring of the lamp parameters. In 2020, a large LED-rotating lamp with integrated structure of main and backup lights was installed on Nanshantou Lighthouse. It is equipped with functions of rotation, light control, telemetry and telecontrol, which could remotely set and query the relevant operating parameters of the lamp. Various communication modes were supported such as BDS (BeiDou Navigation Satellite System) and NB-IoT (Narrow Band Internet of Things).

.

- New lamp installed on Nanshantou Lighthouse

  - Modernization of the power supply equipment

Lighthouse energy source is mainly electric energy, which is used by the lighthouse main body, ancillary facilities and lighthouse keepers' daily life. At present, the power supply facilities used by the lighthouse mainly include municipal electricity supply, diesel generator power supply and renewable power supply. Renewable sources mainly include solar energy, wind energy and tidal energy. Solar energy has been widely used in lighthouses.

Shanbanzhou Lighthouse adopted "PV(photovoltaic)-Diesel-BESS(battery energy storage system) Micro-Grid System" for the comprehensive power system improvement project. PV, combined with diesel generator, realized intelligent power supply. Advanced BMS (battery management system) was adopted, which realized accurate measurement of the power level and electricity of BESS. The battery was well monitored and protected. With the development of science and technology, lithium battery has also begun to be used as lighthouse power supply in recent years,  occupying a place in the lighthouse power market with its advantages of environmental protection, lightweight and remote Bluetooth control, etc.

  - Traditional lighthouses as an important part of e-navigation services

With the development of technology, the traditional lighthouse that combined visual, acoustic, and radio AtoN in the past has developed into a modern lighthouse integrating lighthouse, racon, RBN-DGPS station and AIS base station. Hainan Island, for example, is located in southern China and is surrounded by a total of 25 lighthouses. The AIS base station, VHF system, differential station, radar beacon and other equipment systems carried by the lighthouse are combined with additional points to cover the coastline 360 degrees and minimize signal blind spots. As a key link for e-navigation services, lighthouses can connect the generated functional data to the e-navigation service platform built in China. Through this service platform, users in need can set up electronic fences based on lighthouses, so as to provide services such as risk avoidance, information broadcasting, forecasting, and warning for ships and users using the services. In addition,  it can also assist the Coast Guard in combating criminal activities such as smuggling at sea.

- AIS base station installed on lighthouse around the island

  - Other modern equipment that is installed on traditional lighthouses

    - Lightning protection system

The lighthouse should be equipped with lightning protection facilities, generally a lightning arrester is installed at the top of the lighthouse. In 2005, Kongtong Lighthouse upgraded the lightning protection system and installed AR-1000 type lightning protection terminals and long-term resistance reducing agents, and the lightning protection effect was significantly improved.

    - Bird repellent

Currently widely used electronic bird repellents are used to drive birds away by playing voice or releasing ultrasonic waves. For example, the Matijiao Lighthouse was installed with ultrasonic bird repellents in 2009, and since then, the perennial bird droppings coverage of solar panels has improved.

  - Other applications and services of traditional lighthouses

    - Lighthouse Helipad

Dongting Lighthouse cooperated with CITIC Offshore Helicopter Co., Ltd. to build a lighthouse helipad. This Helicopter is used by the company for daily helicopter training, while the company provides convenience in lighthouse helicopter inspections. The cooperation has achieved a win-win result.

    - Public education

Laotieshan Lighthouse AtoN theme park is one of the first "Navigation Knowledge Popularization Education Bases" in China. Laotieshan Lighthouse AtoN theme park holds open day activities on World Maritime Day every year to popularize the knowledge of beacon and spread marine culture, and receive an average of 5,000 public visits every year. In 2019, Laotieshan Lighthouse cooperated with College of International Collaboration, Dalian Maritime University to build a joint project practice base for "Maritime Safety and Environmental Management".

- Activities in Laotieshan Lighthouse

- ISSUES AND CONCERNS RELATED TO THE USE OF TRADITIONAL LIGHTHOUSES

  - The issues and concerns for traditional lighthouse services

When the traditional lighthouse provides services as a platform, it sometimes involves cooperation with third parties such as other government departments or enterprises. At this time, it is necessary to maintain friendly and smooth communication between the lighthouse and the third party, and on this basis, a good problem feedback mechanism should be formed, so that problems can be solved in time. For example, when conducting mobile communication services, when the signals of the 4G base station built by the operator and our AIS base station interfere with each other, different methods should be considered. If the interference is due to the CCI (Co-Channel Interference), the frequency needs to be adjusted; if the interference is due to the signal direction, the antenna orientation needs to be adjusted; if the interference is caused by low signal-to-noise ratio, the equipment transmission power should be increased. Lighthouse should pay attention to data transmission and storage, DC power supply and backup power supply, network security and other issues when providing video surveillance services. When signing a cooperation agreement with a government department, pay attention to clarifying the obligations and responsibilities of both parties, including equipment maintenance, power security, safety risk management, confidentiality agreements and other related content.

  - The issues and concerns for modern equipment installation on lighthouse

When upgrading traditional lighthouses with informatization, digitization and intelligence, relevant issues when installing modern equipment should also be considered. For example, the normal operation of the equipment should be ensured when the lamp is retrofitted. The energy-saving and environmental protection, moisture-proof and waterproof, safety performance and other elements of new energy source should be considered when transforming lighthouse energy source. Pay attention to the format, compatibility and network security issues when the lighthouse equipment data is connected to the third-party platform software. When it comes to equipment wiring and cabinet placement, pay attention to reasonable planning of storage space, not to damage the main structure of the lighthouse, fire and explosion protection and other related content.

- Other related documents of IALA

M1200 Complementary lighthouse use manual Ed2

1093 Ed.1 The Management of Surplus Lighthouse Property Dec2012

1049 Ed.2 Modern Light Sources in Traditional Optics Dec2007

1061 Ed.1 Light Applications Illumination of Structures Dec2008

1094 Ed2 Daymarks for Aids to Navigation June 2016

1036 Ed3 on Environmental Management in Aids to Navigation June 2017

Input document number, to be assigned by the Committee Secretary

? Leave open if uncertain