Input paper:   ENG16-3.1.2.9

Input paper for the following Committee (s):  	  check as appropriate                         Purpose of paper:

□  ARM	X  ENG		□  PAP					□  Input

□  ENAV	□  VTS							X Information

Agenda Item

Technical Domain / Task Number 2		      …………………………………

Author(s) / Submitter(s)		      …………CHINA MSA……

Application of 5G technology in tidal current data collection and display in Ningbo Zhoushan Port

# Summary

Multi-functional beacons are beacons loaded with various sensors on the basis of traditional beacons to achieve other functions in addition to the traditional navigational aids of beacons. Multi-functional beacons can monitor real-time hydro-meteorological information for ship pilotage, maritime supervision, maritime search and rescue, oil spill emergency response, port construction, channel dredging and other uses. With the increase of user demand, the sensors and equipment on the multi-functional beacon have increased, and a large amount of collected data needs to be transmitted back to the processing center in time. The current data transmission method can no longer meet the demand.This article mainly introduces the use of 700MHz 5G technology to transmit the data and image data collected by the Xiazhimen light vessel (position: 29°40′49.9″N, 122°33′00.4″E) in Ningbo-Zhoushan Port.

## Purpose of the document

China Maritime Safety Administration is willing to share with IALA members the application of 700MHz 5G technology in the field of E-navigation and maritime services, and hopes that this information can provide valuable reference for formulating or revising relevant guidelines.

## Related documents

- N/A

# background

The deep water trough outside Xiazhi Gate of Ningbo-Zhoushan Port is located in the waters between Xiazhi Island and Taohua Island，with excellent natural water depth, is the only 300,000-ton large-scale ship channel entering the central waters of Ningbo-Zhoushan Port, However, ships are often restricted in and out of the port due to complex hydrology, meteorology, sea conditions and poor visibility in this water area, In order to master the real-time hydro-meteorological data of this water area and improve the efficiency of ships entering and leaving the port, the Xiazhimen light vessel outside the mouth of this channel has been renovated since 2020 to realize various functions such as sensing hydro-meteorological information and CCTV video monitoring, and greatly improve the timeliness of ships entering and leaving the port.

# Discussion

## Multifunctional construction of Xiazhimen light vessel

The Ningbo Navigation Mark is equipped with visibility meter, weather meter, velocity meter, wave meter and other equipment on the Xiazhimen light vessel to sense the visibility, wind speed, flow rate, wave and other hydro-meteorology in real time in the water area. The CCTV video monitoring system has been built on the ship to realize dynamic monitoring of the waters around the ship.

## 5G technical advantages of 700MHZ frequency

As an advanced means of communication technology, 5G plays an important role in all walks of life with its advantages of large bandwidth, low latency, high reliability, high connectivity, and ubiquitous network. As the traditional radio and television system frequency band, 700MHz frequency band has the characteristics of low propagation loss, long transmission distance, wide coverage and strong penetration ability, through theoretical analysis, 700M 5G (N28) has the low-frequency advantage that other 5G mainstream frequency bands do not have, which is 14.8dB+ stronger than 5G 2.6G，20dB+ stronger than 3.5G，The coverage capacity of 5G is greatly improved. According to the field test of Zhoushan Radio and Television, 700M can achieve 118km effective coverage in the sea scene. The 700M 5G network fills the gap between 4G and 5G (2.6G, 3.5G, 4.9G) networks in other frequency bands, remote islands and uninhabited islands.

## The pilot application of 700MHZ 5G on the Xiazhimen light vessel

In July 2021, China Radio and Television has launched the maritime application of 5G technology with a frequency of 700MHz in aviation security. The test area is mainly in Zhoushan area. As of October, 6 base stations mainly covering anchorages have been built, which can realize high-definition video transmission with a base station coverage of 60KM. Ningbo Navigation Aid Office selected the Xiazhimen light vessel to carry out a pilot project to transform the light vessel. Based on the 700MHz 5G technology, the navigation mark as the center, 5G communication as the core, satellite communication as the supplement, through AIS, Beidou positioning and other equipment , to conduct real-time day and night monitoring of the operation of ships and navigation marks in the surrounding waters.

Figuer1   Application planning of 700MHZ multi-function beacon

On December 4, 2021 and December 28, 2021 respectively, our office installed one 5G CPE (mobile signal access equipment that receives mobile signals and forwards them with wireless WIFI signals), one integrated fusion gateway and three miniaturized ultra-low-power dome surveillance cameras and other equipment on the xiazhimen light board, complete equipment installation and commissioning tasks, and transmit all kinds of information of navigation mark supervision to Ningbo Navigation Aid Office through 5G network.

Figure2    Lightship 5G CPE access to  the Liuheng Laoyingzui base station 20 nautical miles away after on-site testing

Figure3    Mobile phone speed records in nearby waters

Figure4    Light Vessel  Surveillance Camera Live Image

After a series of upgrades and renovations, the Xiazhimen light vessel has realized a variety of functions, real-time perception of the hydro-meteorology of the deep water channel waters outside the Xiazhimen gate,

transmitted the collected hydro-meteorological data and image data to the processing center using 700MHz 5G technology and shared with users such as ship driving, maritime supervision, port scheduling, etc., so that users can grasp the waterway hydro-meteorological data in real time, guide the safe navigation of ships, and the supervisor can monitor and schedule passing ships more efficiently, improve the time limit for ships entering and leaving the port.

# References

- N/A

# Proposed measures to be taken by the committee

The Committee is invited to note the information provided in the paper.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain