Annex  4

Technical characteristics of VDES-satellite dowlink
in the maritime mobile band

v1.0

26 Feb 2015

# Structure of the VDES

# The following downlink transmission types are defined:

- Single packet signalling and short messaging

- Multi-packet data transfer with ARQ

- Multicast multi-packet data transfer without ARQ

# OSI Layer

# The OSI model used in this Annex is shown in Figure 1.

# Physical layer

  - Link Parameters for the satellite to ship link (LEO satellite)

    - Satellite powerflux density limit on earth surface

The maximum satellite EIRP can be derived from Power Flux spectral Density (PFD) given in Table 1.

Table 1

Proposed power spectral and flux density (PFD) mask

θ°=earth-satellite elevation angle

PFDθ° (dBW/(m2*4 kHz))=-149+0.16*θ°                         0°≤θ<45°;-142+0.53*θ°-45°      45°≤θ<60°;-134+0.1*θ°-60°         60°≤θ≤90°.

Table 2 shows the theoretical maximum satellite EIRP as a function of elevation angles. This EIRP is for linear polarization, for circular polarization the maximum EIRP is 3 dB higher.

Table 2. Satellite maximum EIRP vs. elevation angle.

| Elevation angle | Powerflux density limit | Satellite range | Maximum satellite EIRP |
| --- | --- | --- | --- |
| (degrees) | (dBW/m2/4 kHz) | (km) | (dBW in 25 kHz) |
| 0 | -149,0 | 2831 | -1,0 |
| 10 | -147,4 | 1932 | -2,7 |
| 20 | -145,8 | 1392 | -4,0 |
| 30 | -144,2 | 1075 | -4,6 |
| 40 | -142,6 | 882 | -4,7 |
| 50 | -139,4 | 761 | -2,8 |
| 60 | -134,0 | 683 | 1,6 |
| 70 | -133,0 | 635 | 2,0 |
| 80 | -132,0 | 608 | 2,6 |
| 90 | -131,0 | 600 | 3,5 |

    - Baseline transmit antenna gain and EIRP vs elevation

    - There are a limited number of VHF antennas providing circular polarization suitable for small satellites. Most of the satellite coverage area and visibility time will be at low elevation angles. The baseline linkbudgets are optimised for 0 degrees elevation angle using a satellite antenna pointed at the horizon. A three element crossed Yagi antenna gain as a function of ship elevation is given in Table 3.

    - The transmit power is -9,4 dBW in 25 kHz to match the PSD limit, and the EIRP vs elevation angle is also given in Table 3.

Table 3. Nominal satellite EIRP vs. elevation using a three element Yagi satellite antenna pointed at the horizon, compliant with the PFDL in Table 1.

|  | Yagi antenna |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ship elevation angle | Nadir offset angle | Boresight offset angle | Satellite antenna gain | Satellite EIRP circular polarization | Satellite range | Calculated PFD linear polarization | PFDL Table 1 | PFDL margin |
| degrees | deg. | degrees | dBi | dBW | km | dBW/m2/4 kHz | dBW/m2/4 kHz | dB |
| 0 | 66,1 | 0 | 8 | -1,42 | 2830 | -152,41 | -149,0 | 3,4 |
| 10 | 64,2 | 1,9 | 8 | -1,42 | 1932 | -149,09 | -147,4 | 1,7 |
| 20 | 59,2 | 6,9 | 8 | -1,42 | 1392 | -146,24 | -145,8 | 0,4 |
| 30 | 52,3 | 13,8 | 7,8 | -1,62 | 1075 | -144,20 | -144,2 | 0,0 |
| 40 | 44,4 | 21,7 | 6,9 | -2,52 | 882 | -143,38 | -142,6 | 0,8 |
| 50 | 36 | 30,1 | 5,5 | -3,92 | 761 | -143,50 | -139,4 | 4,2 |
| 60 | 27,2 | 38,9 | 3,6 | -5,82 | 683 | -144,46 | -134,0 | 10,5 |
| 70 | 18,2 | 47,9 | 0,7 | -8,72 | 635 | -146,73 | -133,0 | 13,7 |
| 80 | 9,1 | 57 | -2,2 | -11,62 | 608 | -149,25 | -132,0 | 17,3 |
| 90 | 0 | 66,1 | -5,5 | -14,92 | 600 | -152,43 | -131,0 | 21,4 |

    - * 3 dB polarization loss assumed

    - Range  (min and max)

    - The orbit height determines the satellite range. For 600 km orbit the maximum range is 2830 km. The minimum range is the orbit height.

    - For timing purposes a maximum range of 3000 km shall be used.

    - The minimum range shall be 400 km.

    - The path delay will therefore vary from 10 ms to 1,3 ms, a variation of 8,7 ms.

    - Receive antenna gain

    - Existing ship antennas shall be used for VDES. The maximum antenna gain for these range from 2 to 10 dBi. Representative antenna patterns are shown in Figure 2. (Source: Derived from www.comrod.com)

    - Figure 2. Ship antenna gain vs. elevation angle.

    - For linkbudget purposes a 3 dBi ship antenna is used.

    - Receive noise floor

    - The noise floor is a function of many sources such as cabling losses, LNA noise figure  and local noise sources. Measurements show that a typical noise level is -116 dBm/25 kHz. (Source: Reference xxxx)

    - Link C/N0

    - The nominal signal level and C/N0 vs. elevation for a 25 kHz channel is given in Table 4. The signal level and C/N0 increase 3 dB for 50 kHz channels and 6 dB for 100 kHz channels.

    - Table 4. Nominal link budget vs. ship elevation angle for 25 kHz channel.

| Ship elevation angle | Satellite EIRP in circular polariz. | Range | Pathloss | Polarization and propagation losses | Ship antenna gain | Antenna signal level | Noise level in 25 kHz | C/N0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| deg | dBW | km | dB | dB | dBi | dBm | dBm | dBHz |
| 0 | -1,4 | 2830 | 144,6 | 6 | 3 | -119,0 | -116 | 41,0 |
| 10 | -1,4 | 1932 | 141,2 | 6 | 3 | -115,7 | -116 | 44,3 |
| 20 | -1,4 | 1392 | 138,4 | 6 | 2,5 | -113,3 | -116 | 46,7 |
| 30 | -1,6 | 1075 | 136,2 | 6 | 1 | -112,8 | -116 | 47,2 |
| 40 | -2,5 | 882 | 134,4 | 6 | 0 | -113,0 | -116 | 47,0 |
| 50 | -3,9 | 761 | 133,2 | 6 | -1,5 | -114,6 | -116 | 45,4 |
| 60 | -5,8 | 683 | 132,2 | 6 | -3 | -117,0 | -116 | 42,9 |
| 70 | -8,7 | 635 | 131,6 | 6 | -4 | -120,3 | -116 | 39,7 |
| 80 | -11,6 | 608 | 131,2 | 6 | -10 | -128,8 | -116 | 31,2 |
| 90 | -14,9 | 600 | 131,1 | 6 | -20 | -142,0 | -116 | 18,0 |

    - Interference

    - Received signal to noise plus Interference level

  - Framing Structure

  - The VDES frame structure is identical and synchronized in time on the earths surface to UTC (as AIS) and is shown in Figure 3. Frame 0 starts at 00:00:00 UTC, and there are 1440 frames in a day.

  - The minimum satellite propagation delay is 1,33.. ms and the satellite frames are transmitted 1,33 ms before UTC minutes. Ships at maximum range (0 degrees elevation) will receive the frames 8,66.. ms late.

  - Figure 3. VDES frame structure.

    - Packet size

    - The slot duration is 26,66…ms as shown in Figure 4.

    - Single slot packets and the last packet in a sequence of packets use a guardtime of 8,33… ms, and the packet duration is 18,0 ms for these packets.

    - Packets may use an integer fraction of a 26,66.. ms slot (subslot), again 8,33 ms guardtime is used.

    - Figure 4. VDES satelite packet structure

    - Ramp-up time

    - The ramp up time from 10% to 90% of the power shall occur in less than 100 uS.

    - Unique word

    - The Unique Word is defined in Table 5. It is modulated with BPSK at a symbol rate of 10 ksps. Bit 0 is transmitted first. Duration 1,1 ms.

    - Table 5. Unique Word

| Bit 0 | Bit 1 | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 | Bit 9 | Bit 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 |

    - Packet format

    - The packet format defines the packet duration (26,7/N, 18, 26,7 … ms), the modulation type (BPSK, QPSK, 8PSK, 16APSK…), the channel bandwidth (symbol rate), the FEC type (3GPP), FEC rate(1/3, ½…7/8) and interleaver type.

    - The packet format field sizes are shown in Table 6.

    - Table 6a. Packet format fields

| Field no | Content | Size in bits |
| --- | --- | --- |
| 1 | Packet duration | 3 |
| 2 | Modulation | 4 |
| 3 | Symbol rate (channel bandwidth) | 3 |
| 4 | FEC type | 2 |
| 5 | FEC rate | 3 |
| 6 | Interleaver type | 2 |
|  | Subtotal | 17 |
| 7 | BCH check bits | 7 |
|  | Total | 24 (2,4 ms) |

    - Table 6b. Packet formats (6 bits + FEC=0,9 ms)

| Format no | Packet duration (ms) | Modulation | Symbol rate (ksps) | FEC type | FEC rate | Inter-leaver |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 4,4 | QPSK | 10 | 3GPP | 7/8 | 1 |
| 1 | 18 | QPSK | 10 |  | 3/4 | 1 |
| 2 | 26,7 | QPSK | 10 |  | 3/4 | 1 |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |
| 21 |  |  |  |  |  |  |
| 22 |  |  |  |  |  |  |
| 23 |  |  |  |  |  |  |
| 24 |  |  |  |  |  |  |
| 25 |  |  |  |  |  |  |
| 26 |  |  |  |  |  |  |
| 27 |  |  |  |  |  |  |
| 28 |  |  |  |  |  |  |
| 29 |  |  |  |  |  |  |
| 30 |  |  |  |  |  |  |
| 31 |  |  |  |  |  |  |

    - Scrambling

  - Modulation

    - Channel Bandwidth

    - The channel bandwidths shall be 25, 50 or 100 kHz

    - Symbol rate and spectral mask

      - The symbol rates shall depends on the bandwidth and are given in Table 7. Doppler of up to +/- 4 kHz have been taken into account.

      - Table 7. Symbol rates and filtering masks

| Bandwidth | 25 kHz | 50 kHz | 100 kHz |
| --- | --- | --- | --- |
| Symbol rate | 10 ksps | 22,5 ksps | 47,5 ksps |
| Filtering mask | Figure 4 | Figure 5 | Figure 6 |

      - Figure 4. Spectral mask for 25 kHz channel

      - Figure 5. Spectral mask for 50 kHz channel

      - Figure 6. Spectral mask for 100 kHz channel

    - Modulation

    - The modulation schemes are given in Table 8.

| Bits/symbol |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | GFSK |  |  |  |
| 1 | GMSK |  |  |  |
| 2 | PI/4 DQPSK |  |  |  |
| 2 | QPSK |  |  |  |
| 3 | 8PSK |  |  |  |
| 4 | 16PSK |  |  |  |
| 4 | 16APSK |  |  |  |
| 4 | 4x16QAM |  |  |  |
| 5 | 32APSK |  |  |  |
| 5 |  |  |  |  |
| 6 | 64 APSK |  |  |  |

    - Bit-to-symbol mapping

    - Emission Assumptions

    - Pilot tone characteristics

    - Duration: 0,4 ms TBC

    - Interval:   4,4 ms (26,7/6) TBC

    - Data chunk duration

    - 4 ms TBC

    - Carrier Frequency error

    - 5 ppm TBC+/- 4 kHz Doppler

    - Symbol timing accuracy

    - ~20 ppm TBC

    - Transmitter Timing Jitter

    - <5% rms TBC

    - Slot Tranmission Accuracy at satellite

    - +/- 100 micro sec TBC

  - Forward error correction

    - Data packet FEC Turbo for example  3 GPP, CCSDS

    - Performance measure, Packet error ratio (1e-1, 1e-2 with ARQ, 1e-3)

      - Channel definition (e.g. AWGN)

      - Signal to noise ratios thresholds per MODCODS

      - The C/N0 values are given in Table 9. The packet error rates on a AWGN channel shall be <10%.

      - Table 9. MODCODS

| C/N0 (dBHz) | Modulation | Channel bandwidth | Symbols per packet | FEC type | FEC rate |
| --- | --- | --- | --- | --- | --- |
| 37 |  |  |  |  |  |
| 40 |  |  |  |  |  |
| 43 |  |  |  |  |  |
| 46 |  |  |  |  |  |
| 49 |  |  |  |  |  |
| 52 |  |  |  |  |  |
| 55 |  |  |  |  |  |
| 58 |  |  |  |  |  |
| 61 |  |  |  |  |  |

  - Interleaving (multi-packet)

# Link layer

  - First packet of multipacket format

Field 1: Packet type: 8 bits

Field 2: MMSI: 20 bits

Field 3: Ship subadress: 8 bits

Field 4: Transaction ID: 16 bits

Field 5: Message length in bytes: 32 bits

Field 6: Message chunk-1: Depends on format

Field 7: CRC: 32 bits

  - Continuation packet format

Field 1: Packet type: 8 bits

Field 2: Transaction ID: 16 bits

Field 3: Packet number: 16 bits

Field 3: Message chunk-N: Depends on format

Field 4: CRC: 32 bits

  - End packet format

Field 1: Packet type: 8 bits

Field 2: Transaction ID: 16 bits

Field 3: Packet number: 16 bits

Field 4: Message chunk-N: Depends on format

Field 5: Padding: Depending on message length

Field 6: ACK slot/channel: 16 bits

Field 7: CRC: 32 bits

  - Single packet format

Field 1: Packet type: 8 bits

Field 2: MMSI: 20 bits

Field 3: Ship subadress: 8 bits

Field 4: Transaction ID: 16 bits

Field 5: Message length in bytes: 16 bits

Field 6: Message chunk-1: Depends on format

Field 7: Padding: Depends on message length

Field 8: ACK slot/channel: 16 bits

Field 9: CRC: 32 bits

    - Cyclic redundancy  check

32 bit ITU polynom: 0x04C11DB7

x32 + x26 +  x23 + x22 + x16 + x12 + x11 + x10 + x8 + x7 + x5 + x4 + x2 + x + 1

Initial state: 0xFFFF

# MAC layer

  - Multicast without ARQ protocols

Network status/bulletin board incl. orbit data

Other network information (other systems)

Network configuration (incl. access control)

Group calls

  - Multicast with ARQ protocols

  - Short messages to ship

  - Ship polling

  - Long messages/file transfer to ships

  - Frequency sharing

Lower 100 kHz shared satellite- terrestrial(e.g. timeslots, location, carrier sense…)

Upper 50 kHz exclusive to satellite

- Protocol timing diagrams

# Detailed packet content

# Table x. Detailed packet content

| Packet type | Description | Field 1 |  | Field N |
| --- | --- | --- | --- | --- |
| 0 |  |  |  |  |
| 256 |  |  |  |  |

# Transport layer