| Radiocommunication Study Groups |  |
| --- | --- |
| Received: 17 October 2014 | Document 5B/695-E |
|  | 22 October 2014 |
|  | English only |
| Canada |  |
| Study Regarding Review of VDES channel Plans Conducted by Canadian Coast Guard relating to WRC-15 Agenda ITem 1.16 |  |

ENAV16-11.16

Introduction

The goal of WRC-15 agenda item 1.16 is to consider potential new automatic identification system (AIS) technology applications as well as new applications for enhanced maritime radiocommunication.

Due to the safety of navigation considerations associated with AIS, it is necessary to ensure that any new applications do not reduce the capacity of current AIS operations.

The 2012 World Radiocommunication Conference (WRC-12) approved agenda item 1.16 to consider regulatory provisions and spectrum allocations to enable possible new Automatic Identification System (AIS) technology applications and possible new applications to improve maritime radiocommunication in accordance with Resolution 360 (WRC-12).  Resolution 360 (WRC-12) also resolves to consider allocations and appropriate regulatory measures for new applications being developed for the maritime mobile service. This was done to accommodate proposed uses such as VHF data exchange (VDE).

Three channeling plans for the VHF data exchange system (VDES) have been considered at Working Party 5B (WP 5B) and evaluated in Working document towards a preliminary draft new Report ITU-R M.[VDES-SELECT].  The channel plans are similar in that they use the same group of VHF maritime Appendix 18 channels, and they are different in the way the channels are used.

Discussion

In a recent study commissioned by the Canadian Coast Guard, the three proposed channel plans to improve maritime radiocommunication (commonly referred to as VHF data exchange or VDE) were examined from a different perspective.

Some objectives of the report were:

- Quantify the implications of the frequency plans (A, B and C) proposed in November 2013 to Working Party 5B (WP 5B).

- Draft the optimal topologies for the ship, coastal and satellite radio equipment.

- Calculate the margins the topologies would theoretically yield for the main radiofrequency risks such as adjacent channel interference, intermodulation interference and desensitization of receiver by transmitter noise.

- Identify which of the risks can be mitigated by interference cancellation systems, and quantify the theoretical improvement by such systems. Determine if and how an interference cancellation system can affect other radio performance parameters.

This study indicated, in part:

- With Plans A and B, the VDE1 upper leg frequencies are too close to those of AIS to give uncompromising options for mitigating receiver desensitization issues to a level comparable to an AIS system alone. This would result in reduced radio coverage unless AIS and VDE systems are coordinated or are not co-sited.

- Unless the AIS and VDE systems are coordinated, the three frequency plans would imply significant challenges with the design of satellite radio subsystems as there are risks of adjacent channel interference and receiver desensitization.

- Plan C implementation would yield a limited net data throughput compared Plan A or B.

- Allowing ship to ship communications on a duplex channel that is also allocated for ship-shore communications and allocating one same duplex channel for all coastal stations are three features that would generate co-channel interference.

The full study is included as an embedded file to this document (available in English only).

Proposal

It is proposed that the results of this study, and the identified compatibility issues, be considered in future work on VDES recommendations and/or reports.

______________

____________________