ENAV16-11.17

Use Cases / User Requirements

Table of Contents

1 Use Case	4

1.1 SAR Communications	4

1.1.1 Distress	4

Ship to ship relay	4

Ship sends a distress	4

Shore to ship relay	4

ship to shore relay	4

loss of vessel / investigation	4

1.1.2 SAR operations	5

1.1.3 Tele-medical	5

1.2 MSI [Safety Information]	5

1.2.1 Weather – other - JCG - from Jan 2014 workshop	5

1.2.2 Ice maps	6

1.2.3 Notices to Mariners	6

1.2.4 SBAS corrections - polar regions	6

1.3 Ship reporting	6

1.3.1 submit arrival notice	7

1.3.2 submit updated information	7

1.3.3 Initial report	7

1.3.4 Ship reporting systems	7

1.3.5 Event reporting	7

1.3.6 clearance	7

1.3.7 Other reports	8

1.4 VTS	8

1.4.1 Waterway monitoring	8

1.4.2 Information Service	8

1.4.3 Navigation Assistance	8

1.4.4 Traffic Organisation Services	8

1.5 update charts and  publications	8

1.5.1 Updates linked to ships route - is there an update required?	9

1.5.2 updates linked to a ships route - provide the update	9

1.5.3 Required publications onboard	9

1.6 route exchange	9

1.6.1 ship to ship	9

1.6.2 Ship to shore	9

1.6.3 Shore to ship	9

1.6.4 Other	9

1.7 Logistic services	10

2 Requirements	10

2.1 machine to machine communications	10

2.1.1 standardised formats for data exchange	10

2.1.2 Different reporting frequencies	10

2.1.3 Different 'lifespan' for information	10

Caching of data	10

2.1.4 share information between participants in the system	10

2.1.5 Different reporting area requirements	11

2.1.6 alerting	11

2.1.7 Support mesh network	11

2.1.8 indication of how to access information	11

2.2 forward information to a specific ship	11

2.3 Forward information to a group of ships	11

2.4 send information to a geographically defined area	11

2.5 use of VHF voice and VDES terrestrial at the same time	11

2.6 Information on request	11

2.6.1 one-off	11

2.6.2 to cover off a route	11

2.7 each VDES terminal is identified (unique)	12

2.8 Information to be verified - authentication of information	12

2.9 authentication of the sender and receiver	12

2.10 integrity of data transmitted	12

2.11 data sent in a manner that multiple receptions can increase reliability	12

2.12 data sent in a short time frame	12

2.13 scalability	12

2.14 confidentiality of information	12

2.15 identify the priority of the message	12

2.15.1 network to respect the priority of the message	12

2.15.2 Transfer of large files of lower priority without degrading transfer of higher priority, smaller files.	12

2.16 Immediacy of information	12

2.17 Determine most appropriate means to transfer data	12

2.17.1 System identifies least cost routing	13

2.17.2 User control over routing	13

2.18 a means to enable charging	13

2.18.1 charging based on the transport method for the data	13

2.18.2 charging based on the actual data	13

3 Out of scope / other discussion items	13

3.1 metadata	13

3.2 transfer of personal information?	13

3.3 scientific use of data	13

3.4 commercial use	13

Use Case

Focus at Feb 2015 was the discussion for 'non-commercial' and related vessels.

Commercial communications were not included.

Specific use case for 'commercial' entities - include the other 'users'.

SAR Communications

Distress

Initial - indication that a vessel is in distress.

Using agreed Distress alert capabilities

Ship to ship relay

Ship sends a distress

Shore to ship relay

ship to shore relay

through terrestrial or satellite means

Automated relay can be done using VDES.

The system decides the fastest / most effect means to transfer information.

loss of vessel / investigation

As a vessel sinks the data from the VDR is forwarded - save searching for the 'black box' after the case.

'imminent collision' (or after collision) - information from the AIS is provided to identify all the other vessels in that area.

SAR operations

On scene communications / incident management

Send search pattern / coordinates / common operating image for the search coordination.

Could be used in a standard format

From RCC / coordination to ship

Return capability from the searchers - identify where search happened.

RCC to multiple responders / assets.

'machine to machine'

General requirement - standardised formats for data exchange

[formats being designed for aspects of this work]

[relevant work - S-100 - framework / data dictionary - S- 100 doesn't indicate the transfer network]]

Core requirement - machine to machine communications.

Digital information in graphical display - capacity in VDES (size of data?)  Concept of data dictionary to minimise the data to be sent (send a code for an image)

Tele-medical

A seafarer is injured or sick onboard, there is a need to communicate with a doctor ashore to determine the severity / possible need for medivac.

Note - the actual discussion with doctor could be by voice, but data on the condition of the patient, images of the injury could be provided by VDES.  A data library could be used to identify the specifics of injury.

MSI [Safety Information]

Maritime Safety Information

Who - broadcasts?

Authorities (WWRNS/ IHO / WMO)

What?

Lock operating / limitations / levels (sill clearance)

IMO (Navarea) defined information

- reference Maritime Service Portfolio (MSPs) (all 16 elements of MSP) (check e-nav website)

Note - include formal definition of MSI (from IMO) MSC1/Circ.1287 Ammendment to Res A.705(17) / Promulgation of MSI (covers warnings, met, SAR and other urgent safety information)

Comsar Cir.15 (9 March 1998) IMO/IHO WMO manual.

- information that is defined in S- 100

- information to a specific area (geogrpahically)

Weather – other - JCG - from Jan 2014 workshop

Weather (weather warnings / weather observations)

Safety information

Weather observations from ships to shore

Ice maps

- icemaps - ships required to hve updated ice maps every 6 hours.  Ice maps are a certain size (at the moment) - in future - shore says this is the last I have / then the 'updated' amount is provided.  Very short access time for the satellite.  (transmit delta rather than full product - a registry that can be used to minimize).

- typical ice map 8-10 kbs

Ice map uses earth observation

Requirements

Prior to leaving - ship downloads current ice map

As vessel proceeds get updates to ice map every 6 hours.

Updates are made from data from earth observation satellites / distributed by the authority. Option can also be made to 'purchase'.  Information provided is the 'delta' to the last distributed map version.  There could be a need for the full map.

vessel needs to see the satellite.

Version - can advise how many were missed.

Alternatives - send full map / send delta / vessel receives full map / vessel receives delta / vessel doesn't receive map at all.

Id to determine source of data / verify the authoritative information.

Data sent in a way so that information received / multiple transmissions can enable full data delivery (i.e. a chart isn't fully received the first time, but additional data sent then complements and provides the full map)

[get example of data files]

Notices to Mariners

Updates on specific aspects - for example, new cable laid, change in buoy location.

Ship trasnsitting a route / indication of pollution ahead / information transmitted prior to arriving to allow vessel to determine appropriate steps to take.

SBAS corrections - polar regions

use of VDES to broadcast SBAS corrections in polar regions

IMO Resolution 915 (to be confirmed) enabling navigation to meet the IMO requirements for precision of navigation

Ship reporting

Note Fal.5/Circ.36 - single window - not duplicating work, but providing a communications capability to support single window / electronic form reporting.

IMO SOLAS V/11 (ship reporting)

Note FAL indicates that it does not provide a required format.

In Europe directive that makes it mandatory for member states to set up a single window to accept mandatory reports - Directive from EU. (formal arrangement / be aware of sensitivities)

Discussions at IMO on electronic forms / reporting.  (MARPOL)

Note on Norway single window - requirements identified, but some ships can be exempt - for example, position report requirements / ships with AIS can be exempt from this report.

submit arrival notice

96 hr arrival notice to port

Content of the message - pre- format / provide updated information.

Ship is required to provide 96 hr pre-entry report to a port.  The information is based on set template to be filled in.  Bulk of the template can be pre-filled (base ship information).  Only changed material to be provided (the delta).  Less effort for the mariner.  Report when received needs to be understood (standardised format for data).

Standardised - meta data format that is transmitted from shore to ship indicating the required information to be provided.  (note many items will be similar, but some requirements will be specific to the port. )

-could be transmit on schedule / transmit in response to a request / could be addressed to a specific ship.

submit updated information

through the voyage, the ship sends only 'changed' information as required.

Initial report

Full data report provided to agend prior to departure.

shore to shore transmission of the data.

Discussion regarding the Norwegian machine to to machine single window.

Ship reporting systems

LRIT,

AMVER

MASTREP

JASREP

NOREP

etc....

Ship travelling in an area where there is a reporting system.  The reporting occurs automatically based on the ship position (AIS).

Event reporting

Danger Messages’ (as in SOLAS V/32) which can be transmitted by ships (encountering tropical cyclones, ice, derelicts and other dangers to navigation).

SOLAS V/5 - Met services and warnings.

MARPOL

clearance

Clearance provided to a vessel on receipt of a report (arrival notice / change of information notice).

From shore to ship only.

Other reports

VOS - voluntary observation reporting system  (note that VDES could be set to do automatically - using onboard weather sensors).

crowd source bathymetry (tie into depth sounder)

Remote Condition Monitoring (RCM) - exception monitoring.

VTS

high volume position reporting

Make up a scenario that covers Ins / Nas / Tos

Alarm aspect... (similar to the radio concept)

VTS SOLAS V/12

Reg A.857(20)

VTS definition - to be a VTS there is a need to maintain a traffic image / interact with traffic.

Waterway monitoring

Not all vessels have AIS / report to VTS.

VDES could provide a means to interact with smaller craft / craft not reporting to VTS.

Information Service

Vessel in the area requests information (specific) and the information is forwarded from the VTS.

- 'bird's eye' view of the traffic

- indication of no-go areas / changes in the waterway status / diving operations / channel closures.

Navigation Assistance

When a vessel requires navigational assistance - promulgate specific data / image information.

example - scenario - exiting a corridor / vessels (FV) losing situational awareness - communications required in second level.

Traffic Organisation Services

Organising traffic through a lock or a 'one-way' area

update charts and  publications

Limit to documents as relevant to SOLAS V/27 for the intended voyage.

Nautical charts and Nautical publications.

Chart updates

list of radio signals

List of Lights

Notices to Mariners

etc.

Differentiate between what all ships have (general requirements) vs what specific ships have.

Is there a requirement to 'pay' for the updates (chart update services).

May require a larger file transfer - but this would be a lower priority.  requirement to enable the large transfer, but with short, small, higher priority data.

Updates linked to ships route - is there an update required?

Ship transiting to a specified location, has on board the latest publications and charts as at time of sailing, but would be interested to know if there is an update.

Request related to the need to find out IF there is an update. But not ask for the actual update.

Related to the ships route.

updates linked to a ships route - provide the update

If there is a small amount of data, or bandwidth is available, then the data could be provided.

This could be linked to a version number - as the vessel goes into an area the version onboard can be compared to the latest version, and the latest version is then provided.

Related to the ships route.

Note - the extra bandwidth could be available on a regional basis to provide additional data through the updates.

Required publications onboard

A vessel has onboard the required publications as initially identified.

As a vessel enters an area information on required publications for that area is provided (could be the delta - if there is a change in the required publications).

route exchange

Routing to minimise costs - slow steaming / cost efficient routing.

ship to ship

Vessel transmit route / changes in their route

other vessels receive the route and take this into consideration for safety of navigation / situation awareness and vessel domain management / maintain separation.

Ship to shore

Shore stations receive the routes and can use for traffic planning / targeted information transmission (i.e. information provision as appropriate to the route - weather, MSI, etc)

Shore to ship

shore stations can forward a recommended route to a vessel for consideration / confirmation - avoid weather issues; pilot boarding; VTS routing for traffic management; could be linked to SAR planning (creeping line / expanding square / waypoints)

Shore monitoring of route - if vessel is not maintaining its route alerting would occur.

Other

UKCM

Logistic services

[Maritime Service Portfolio]

Request for services

- Pilotage services

- berthing assignment

- anchorage

- tugs

- logistics (supplies; etc)

Requirements

machine to machine communications

Automated reporting - could be transmit on schedule / transmit in response to a request / could be addressed to a specific ship.

standardised formats for data exchange

Defined in S-100

Different reporting frequencies

Some information provided 'on request'

Some information provided on schedules

Some information provided as information is changed

Different 'lifespan' for information

Some information may be 'cached', to be retained

Some information time-sensitive / aging of information then deleted.

Validity of information - for example a container floating, would be moving, how long would the information be valid (can use standard timing as per existing standards / note there could be other changes).

why waste bandwidth for data that is no longer valid.

Specific discussion - requirement if VDR data is to be uploaded - where to?  Caching to a nearby vessel / store and forward.

Caching of data

share information between participants in the system

If information is missing, 'opportunity based' information exchange.  (lower level priority for transmission)

[use case - vessel is leaving a port and meets a vessel going to the same port - changed information provided]

concept of an 'almanac' or catalogue of information.

Authorised concept - verification that the information can be exchange (i.e. if a chart update is available, but the other vessel hasn't paid for that update, then the update doesn't get sent).

Different reporting area requirements

Information linked to an area of transit

type of communication will be linked to the area of requirement - terrestrial VDE coastal / satellite VDE ocean

link to the geographic area

alerting

when an issue is identified an alert can be sent to the ship.

examples - VTS identified alert / weather alert / change in waterway condition alert.

Example - japan CG automatic alerting with AIS - vessel passes a line and alert - AIS automatically sends a message to the ship.

Support mesh network

Peer group network management

indication of how to access information

information in data packets sent and displayed in predetermined manner -

i.e. - displayed by e-mail; ecdis;

forward information to a specific ship

Forward information to a group of ships

Information to all ships of the same flag, for example.

ships in a specific area

type of ships (FV)

send information to a geographically defined area

can be a predefined area (Navarea) / also an ability to define an area 'on the fly' (pollution incident / stay clear)

advise for vessels that are proceeding to that area - information about aspects on your route

ability to 'request' information that may come into play on a route.

use of VHF voice and VDES terrestrial at the same time

user case on the tele-medical

need to develop system so that the data packets can be sent at the same time as using VHF voice.  (small packets of data)

Discussion - VDES able to sent e-mail to an e-mail

Information on request

one-off

i.e is there enough water over the sill of the lock?

to cover off a route

poll information on your route

have information provided during the voyage on a route

each VDES terminal is identified (unique)

identified in a way to support the different sub groups required.

Information to be verified - authentication of information

of the transmission - from authority, from a ship, etc)

Note - discussion on possible way to 'crash' the system - a hacker sends a request for a significant amount of data.  Need to ensure the request for information is authentic / from a 'trusted' agent.

authentication of the sender and receiver

- confirmation that it IS the ship (security of transmission - authentication) / that it IS the authority

integrity of data transmitted

Information integrity is maintained. Note the information will be as supplied by ship / authority  -  there could be bad information provided, but the system must keep the message integrity during transmission.

data sent in a manner that multiple receptions can increase reliability

data sent in a short time frame

example of ice map - when a satellite passes, which may be only a few minutes.

Ice map - estimate 10 kb

scalability

addressing not only SOLAS vessels

Smaller vessels / Military vessels / fishing vessels

confidentiality of information

Information may be specific for a ship or a group of ships (i.e. chart updates - may not be available to all ships, but could be available to a ship that has 'authority' to receive the data - has paid).

identify the priority of the message

A need to identify the priority of the message within the message.

network to respect the priority of the message

Transfer of large files of lower priority without degrading transfer of higher priority, smaller files.

Immediacy of information

Information provided on a 'just in time / just as needed' basis.

Determine most appropriate means to transfer data

Terrestrial / satellite

based on the priority of the data to be transferred.

based on location of vessel

based on data packet size to be transferred

system looks at the means of transferring data available and picks the most appropriate

If a cost may incur due to the data transfer method chosen, the user is alerted to this fact

System identifies least cost routing

Noting the charging requirements / system looks at the options for routing and identifies the least cost option as a 'default' option.

User control over routing

The user can determine an alternate routing, which may not be the least cost routing (but may be faster).

a means to enable charging

some service will be 'free' of charge to the user

the user will have a need for a product or a service, some of which will have a cost.

Discussion noted the phone system - mobile phone charges. exchange information - which user has used what data.

charging based on the transport method for the data

data is transferred by infrastructure / therefore there may be a cost for the transport mode (terrestrial / satellite).

Could include the cost of the infrastructure (who puts the infrastructure in place / is there a need to / how to recover costs)

Note existing maritime costing - Accounting Authority (ITU system exists)

charging based on the actual data

Some data provided 'free' (i.e distress, urgency, safety);

other data related to 'commercial' aspects charged (logistics)

Content provider may be required to pay for the use of the infrastructure

Out of scope / other discussion items

metadata

Discussion on the ability to have metadata identified / transmitted through VDES - commercial implications.

Determined commercial implications are out of scope for this discussion.

transfer of personal information?

discussion on the possible role of 'personal' data transmission through VDES (communication with home)

scientific use of data

access to data over time

data analytics

commercial use

mining companies access data / routes of vessels