Annex 2

Technical characteristics of the Application Specific Messages (ASM)               for the VDES in the VHF maritime mobile band

# 1	Structure of the application specific messages

For the VDES, to mitigate AIS VDL loading effects, ASM should conform to the data structure specified in Recommendation ITU-R M.1371 and may use the two channels designated for ASM in Table 1 (ASM 1 and ASM 2) instead of AIS 1 and AIS 2. Transmission method should be according to Section ‎0This Annex describes the characteristics of the TDMA access schemes which include random access TDMS (RATDMA), incremental TDMA (ITDMA), fixed access TDMA (FATDMA) and carrier-sense TDMA (CSTDMA) techniques.

# 1.1	Application specific messages layer module

This recommendation covers layers 1 to 4 (physical layer, link layer, network layer and transport layer) of the open system interconnection (OSI) model.

Figure 4 illustrates the layer model of the ASM station (physical layer to transport layer) and the layers of the applications (session layer to applications layer):

Figure 4

| Application Layer |  |  |  |  |
| --- | --- | --- | --- | --- |
| Presentation Layer |  |  |  |  |
| Session Layer |  |  |  |  |
| Transport Layer |  |  |  |  |
| Network Layer AISM1 AISM2 |  |  |  |  |
| Link Management Entity (LME) Layer |  |  | Link Management Entity (LME) Layer |  |
| Data Link Service (DLS) Layer |  |  | Data Link Service (DLS) Layer |  |
| Medium Access Control (MAC) Layer |  |  | Medium Access Control (MAC) Layer |  |
| Physical Layer |  |  | Physical Layer |  |
| RX ASM1 | TX ASM1/ASM2 |  |  | RX ASM 2 |
| Rx: Receiver Tx: Transmitter |  |  |  |  |

## 1.2	Responsibilities of layers for preparing ASM data for transmission

### 1.2.1	Transport layer

The transport layer is responsible for converting data into transmission packets of correct size and sequencing of data packets.

### 1.2.2	Network layer

The network layer is responsible for the management of priority assignments of messages, distribution of transmission packets between channels, and data link congestion resolution.

### 1.2.3	Link layer

The link layer is divided into three sub-layers with the following tasks:

#### 1.2.3.1	Link management entity

Assemble ASM message bits, see Annex xxx.

Order ASM message bits into 8-bit bytes for assembly of transmission packet, see § xxx

#### 1.2.3.2	Data link services

Calculate frame check sequence (FCS) for ASM message bits, TBD

Append FCS to ASM message to complete creation of transmission packet contents TBD

Apply bit stuffing process to transmission packet contents, TBD.

Complete assembly of transmission packet, see § xxx.

#### 1.2.3.3	Media access control

Provides a method for granting access to the data transfer to the VHF data link (VDL). The method used is a TDMA scheme using a common time reference.

### 1.2.4	Physical layer

Non return to zero inverted (NRZI) encode assembled transmission packet see § xxx.

Convert digital NRZI coded transmission packet to [appropriate modulation scheme signal to modulate transmitter, see § xxx.

# 2	Physical layer

## 2.1	Parameters

### 2.1.1	General

The physical layer is responsible for the transfer of a bit-stream from an originator, out on to the data link. The performance requirements for the physical layer are summarized in Tables xxx to xxx.

For transmit output power see also § xxx.

The low setting and the high setting for each parameter is independent of the other parameters.

TABLE 3

| Symbol | Parameter name | Units | Low setting | High setting |
| --- | --- | --- | --- | --- |
| PH.CHS | Channel spacing (encoded according to RR Appendix 18 with footnotes)(1) | kHz | 25 | 25 |
| PH.ASM1 | ASM 1 (2027)(1) | MHz | 161.950 | 161.950 |
| PH.ASM2 | ASM 2 (2028)(1) | MHz | 162.000 | 162.000 |
| PH.BR | Bit rate | kbit/s | TBD | 28.8 |
| PH.TS | Training sequence | Bits | TBD | TBD |
| PH.TXBT | Transmit BT product |  | TBD | TBD |
| PH.RXBT | Receive BT product |  | TBD | TBD |
| PH.MI | Modulation index |  | TBD | TBD |
| PH.TXP | Transmit output power | W | 1 | 12.5 |
| (1) See Recommendation ITU-R M.1084, Annex 4. |  |  |  |  |

### 2.1.2	Constants

TABLE 4

| Symbol | Parameter name | Value |
| --- | --- | --- |
| PH.DE | Data encoding | TBD |
| PH.FEC | Forward error correction | TBD |
| PH.IL | Interleaving | TBD |
| PH.BS | Bit scrambling | TBD |
| PH.MOD | Modulation | TBD |

### 2.1.3	Transmission media

Data transmissions are made in the VHF maritime mobile band. Data transmissions should use ASM 1 and ASM 2 channels.

### 2.1.4	Multi-channel operation

The ASM should be capable of receiving on two parallel channels and transmitting on two independent channels in accordance with § xxx. Two separate TDMA receiving processes should be used to simultaneously receive on two independent frequency channels. One TDMA transmitter should be used to alternate TDMA transmissions on two independent frequency channels.

## 2.2	Transceiver characteristics

The transceiver should perform in accordance with the characteristics set forth herein.

TABLE 5

Minimum required time division multiple access transmitter characteristics

| Transmitter parameters | Requirements |
| --- | --- |
| Carrier power error | TBD |
| Carrier frequency error | TBD |
| Slotted modulation mask | ∆fc < ±10 kHz: 0 dBc ±10 kHz < ∆fc < ±25 kHz: below the straight line between −25 dBc at ±10 kHz and –70 dBc at ±25 kHz ±25 kHz < ∆fc < ±62.5 kHz: –70 dBc |
| Transmitter test sequence and modulation accuracy | TBD |
| Transmitter output power versus time | TBD |
| Spurious emissions | TBD |
| Intermodulation attenuation (base station only) | TBD |

TABLE 6

Definitions of timing for Figure xxx

| Reference |  |  | Bits | Time (ms) | Definition |
| --- | --- | --- | --- | --- | --- |
| T0 |  |  | TBD | 0 | Start of transmission slot. Power should NOT exceed –50 dB of Pss before T0 |
| TA |  |  | TBD | 0-0.625 | Power exceeds –50 dB of Pss |
| TB |  | TB1 | TBD | 0.625 | Power should be within +1.5 or –3 dB of Pss |
|  |  | TB2 | TBD | 0.833 | Power should be within +1.5 or –1 dB of Pss (start of training sequence) |
| TE (includes 1 stuffing bit) |  |  | TBD | 24.271 | Power should remain within +1.5 or –1 dB of Pss during the period TB2 to TE |
| TF (includes 1 stuffing bit) |  |  | TBD | 25.104 | Power should be –50 dB of Pss and stay below this |
| TG |  | TBD | 26.667 | Start of next transmission time period |

TABLE 7

Minimum required time division multiple access receiver characteristics(1)

| Receiver parameters | Requirements |
| --- | --- |
| Sensitivity | 20% PER @ –107 dBm |
| Error behaviour at high input levels | 1% PER @ –77 dBm 1% PER @ –7 dBm |
| Adjacent channel selectivity | 20% PER @ 70 dB |
| Co-channel selectivity | 20% PER @ 19 dB |
| Spurious response rejection | 20% PER @ 70 dB |
| Intermodulation response rejection | 20% PER @ 74 dB |
| Spurious emissions | –57 dBm (9 kHz to 1 GHz) –47 dBm (1 GHz to 4 GHz) |
| Blocking | 20% PER @ 86 dB |

## 2.3	Modulation scheme

The modulation scheme is PI/4 Differential Quadrature Phase-Shift Keying (DQPSK).

### 2.3.1	PI/4 DQPSK

2.3.1.1	The encoded data should be PI/4 DQPSK coded before modulating the transmitter.

2.3.1.2	The modulator BT-product used for transmission of data should be TBD maximum (highest nominal value).

2.3.1.3	The demodulator used for receiving of data should be designed for a BT-product of maximum TBD (highest nominal value).

### 2.3.2	Phase modulation

The coded data should modulate the VHF transmitter. The modulation index should be TBD.

### 2.3.3	`Frequency stability

The frequency stability of the VHF radio transmitter/receiver should be ± TBD Hz or better.

## 2.4	Data transmission bit rate

The transmission bit rate should be max 28.8 kbit/s  50 ppm.

## 2.5	Training sequence

Data transmission should begin with a TBD -bit demodulator training sequence (preamble) consisting of one segment synchronization. This segment should consist of alternating zeros and ones (0101....). This sequence may begin with a 1 or a 0 since TBD encoding is used.

## 2.6	Data encoding

The TBD waveform is used for data encoding

## 2.7	Forward error correction

Forward error correction is TBD

## 2.8	Interleaving

Interleaving is TBD

## 2.9	Bit scrambling

Bit scrambling is TBD

## 2.10	Data link sensing

Data link occupancy and data detection are entirely controlled by the link layer.

## 2.11	Transmitter transient response

The attack, settling and decay characteristics of the RF transmitter should comply with the mask shown in Fig. x and defined in Table x.

Figure 2

Transmitter output envelope versus time

### 2.11.1	Switching time

The channel switching time should be less than 25 ms (see Fig. x).

The time taken to switch from transmit to receive conditions, and vice versa, should not exceed the transmit attack or release time. It should be possible to receive a message from the slot directly after or before own transmission.

The equipment should not be able to transmit during channel switching operation.

The equipment is not required to transmit on the other ASM channel or AIS channels in the adjacent time slot.

## 2.12	Transmitter power

The power level is determined by the link management entity (LME) of the link layer.

2.12.1	Provision should be made for two levels of nominal power (high power, low power) as required by some applications. The default operation of the ASM station should be on the high nominal power level. Changes to the power level should only be by assignment by the approved channel management means (see § xxx).

2.12.2	The nominal levels for the two power settings should be [1] W and [12.5] W, Tolerance should be within 1.5 dB.

## Shutdown procedure

2.13.1	An automatic transmitter hardware shutdown procedure and indication should be provided in case a transmitter continues to transmit for more than 2 s. This shutdown procedure should be independent of software control.

## 2.14	Safety precautions

The ASM installation, when operating, should not be damaged by the effects of open circuited or short circuited antenna terminals.

# 3	Link layer

The link layer specifies how data is packaged in order to apply error detection and correction to the data transfer. The link layer is divided into three (3) sub-layers.

## 3.1	Sub-layer 1: medium access control

The medium access control (MAC) sub layer provides a method for granting access to the data transfer medium, i.e. the VHF data link. The method used is a TDMA scheme using a common time reference.

### 3.1.1	TDMA synchronization

TDMA synchronization is achieved using an algorithm based on a synchronization state as described below. The sync state flag within ITDMA communication state, indicates the synchronization state of a station.

The TDMA receiving process should not be synchronized to slot boundaries.

Synchronization other than UTC direct may be provide by the AIS system.

If there is no synchronization available the unit stops transmitting VDE and ASM messages.

#### 3.1.1.1	Coordinated universal time direct

A station, which has direct access to coordinated universal time (UTC) timing with the required accuracy should indicate this by setting its synchronization state to UTC direct.

#### 3.1.1.2	Coordinated universal time indirect

A station, which is unable to get direct access to UTC, but has access to the AIS system, may get its synchronization from the AIS system.  It should then change its synchronization state to indicate the type of synchronization which is being provided by the AIS system.

### 3.1.2	Time division

The system uses the concept of a frame. A frame equals one (1) minute and is divided into 4 500 slots.  Access to the data link is, by default, given at the start of a slot. The frame start and stop coincide with the UTC minute, when UTC is unavailable the AIS system may provide the frame synchronization.

### 3.1.3	Slot phase and frame synchronization

Slot phase synchronization and frame synchronization is done by using information from UTC or from the AIS System.

#### 3.1.3.1	Slot phase synchronization

Slot phase synchronization is the method whereby the slot boundary is synchronized with a high level of synchronization stability, thereby ensuring no message boundary overlapping or corruption of messages.

#### 3.1.3.2	Frame synchronization

Frame synchronization is the method whereby the current slot number for the frame is known.

##### 3.1.3.2.1    	Coordinated universal time available

A station, which has direct access to UTC, should continuously re-synchronize its transmissions based on UTC source. A station, which has indirect access to UTC should continuously resynchronize its transmissions based on those UTC sources (see § xxx).

##### 3.1.3.2.2	Coordinated universal time not available

When the station determines that its own internal slot number is equal to the semaphore slot number, it is already in frame synchronization and it should continuously slot phase synchronize.

##### 3.1.3.2.3	Synchronization sources

The primary source for synchronization should be the internal UTC source (UTC direct). If this source should be unavailable the synchronization should be derived from remote AIS transmissions (refer to ITU-R M.1371)

### 3.1.4	Slot identification

Each slot is identified by its index (0-4 499). Slot zero (0) should be defined as the start of the frame.

### 3.1.5	Slot access

The transmitter should begin transmission by turning on the RF power at slot start.

The transmitter should be turned off after the last bit of the transmission packet has left the transmitting unit. This event must occur within the slots allocated for own transmission. The default length of a transmission occupies one (1) slot. The slot access is performed as shown in Fig. 5:

Figure 5

### 3.1.6	Slot state

Each slot can be in one of the following states:

–	Free: meaning that the slot is unused within the receiving range of the own station. Externally allocated slots that have not been used during the preceding three frames are also Free slots. This slot may be considered as a candidate slot for use by own station (see § xxx).

–	Internal allocation available: meaning that the slot is allocated by own station and can be used for transmission of ASM messages.

–	Internal allocation unavailable: meaning that the slot is allocated by own station for the purpose of AIS or VDE transmissions and cannot be a candidate for slot reuse.

–	External allocation: meaning that the slot is allocated for transmission by another station and cannot be a candidate for slot reuse.

–	Available: meaning that the slot is externally allocated by a station and is a possible candidate for slot reuse (see § xxx).

–	Unavailable: meaning that the slot is externally allocated by a station and cannot be a candidate for slot reuse (see § xxx).

## 3.2	Sub layer 2: data link service

The data link service (DLS) sub layer provides methods for:

–	data link activation and release;

–	data transfer; or

–	error detection, correction and control.

### 3.2.1	Data link activation and release

Based on the MAC sub layer the DLS will listen, activate or release the data link. Activation and release should be in accordance with § xxx. A slot, marked as free or externally allocated, indicates that own equipment should be in receive mode and listen for other data link users. This should also be the case with slots, marked as available and not to be used by own station for transmission (see § xxx)

### 3.2.2	Data transfer

Data transfer should use a bit-oriented protocol which is based on the high-level data link control (HDLC) as specified by ISO/IEC 13239:2002 – Definition of packet structure. Information packets (I-Packets) should be used with the exception that the control field is omitted (see Fig. 6).

#### 3.2.2.1	Bit stuffing

The bit stream of the data portion and the FCS, see Fig. xxx, § xxx and § xxx, should be subject to bit stuffing. On the transmitting side, this means that if five (5) consecutive ones (1’s) are found in the output bit stream, a zero should be inserted after the five (5) consecutive ones (1’s). This applies to all bits between the HDLC flags (start flag and end flag, see Fig. xxx). On the receiving side, the first zero after five (5) consecutive ones (1’s) should be removed.

#### 3.2.2.2	Packet format

Data is transferred using a transmission packet as shown in Fig. xxx:

Figure 6

The packet should be sent from left to right. This structure is identical to the general HDLC structure, except for the training sequence. The training sequence should be used in order to synchronize the VHF receiver and is discussed in § xxx. The total length of the default packet is 256 bits. This is equivalent to one (1) slot.

#### 3.2.2.3	Training sequence

The training sequence should be a bit pattern consisting of alternating 0’s and 1’s (010101010...). Twenty-four bits of preamble are transmitted prior to sending the flag. This bit pattern is modified due to the NRZI mode used by the communication circuit (see Fig. x).

Figure 7

The preamble should not be subject to bit stuffing.

#### 3.2.2.4	Start flag

The start flag should be 8 bits long and consists of a standard HDLC flag. It is used in order to detect the start of a transmission packet. The start flag consists of a bit pattern, 8 bits long: 01111110 (7Eh). The flag should not be subject to bit stuffing, although it consists of 6 bits of consecutive ones (1’s).

#### 3.2.2.5	Data

The data portion is 168 bits long in the default transmission packet. The content of data is undefined at the DLS. Transmission of data, which occupy more than 168 bits, is described in § xxx.

#### 3.2.2.6	Frame check sequence

The FCS uses the cyclic redundancy check (CRC) 16-bit polynomial to calculate the checksum as defined in ISO/IEC 13239:2002. The CRC bits should be pre-set to one (1) at the beginning of a CRC calculation. Only the data portion should be included in the CRC calculation (see Fig. 7).

#### 3.2.2.7	End flag

The end flag is identical to the start flag as described in § xxx.

#### 3.2.2.8	Buffer

The buffer is normally 24 bits long and should be used as follows:

–	bit stuffing:	4 bits (normally, for all messages except safety related messages and binary messages)

–	distance delay:	14 bits

–	synchronization jitter:	6 bits

##### 3.2.2.8.1	Bit stuffing

A statistical analysis of all possible bit combinations in the data field of the fixed length messages shows that 76% of combinations use 3 bits or less, for bit stuffing. Adding the logically possible bit combinations shows, that 4 bits are sufficient for these messages. Where variable length messages are used, additional bit stuffing could be required. For the case where additional bit stuffing is required, see § xxx and Table xx.

##### 3.2.2.8.2	Distance delay

A buffer value of 14 bits is reserved for distance delay. This is equivalent to 235.9 nautical miles (NM). This distance delay provides protection for a propagation range of over 120 NM.

##### 3.2.2.8.3	Synchronization jitter

For synchronization jitter refer to Recommendation ITU-R M.1371 Annex 2 (Class A)

#### 3.2.2.9	Summary of the default transmission packet

The data packet is summarized as shown in Table 12:

TABLE 12

| Ramp up | 8 bits | T0 to TTS in Fig. 8 |
| --- | --- | --- |
| Training sequence | 24 bits | Necessary for synchronization |
| Start flag | 8 bits | In accordance with HDLC (7Eh) |
| Data | 168 bits | Default |
| CRC | 16 bits | In accordance with HDLC |
| End flag | 8 bits | In accordance with HDLC (7Eh) |
| Buffer | 24 bits | Bit stuffing distance delays, repeater delay and jitter |
| Total | 256 bits |  |

#### 3.2.2.10	Transmission timing

Figure 8 shows the timing events of the default transmission packet (one slot). At the situation where the ramp down of the RF power overshoots into the next slot, there should be no modulation of the RF after the termination of transmission. This prevents undesired interference, due to false locking of receiver modems, with the succeeding transmission in the next slot.

#### 3.2.2.11	Long transmission packets

A station may occupy at maximum ten consecutive slots for one (1) continuous transmission. Only a single application of the overhead (ramp up, training sequence, flags, FCS, buffer) is required for a long transmission packet. The length of a long transmission packet should not be longer than necessary to transfer the data; i.e. the ASM should not add filler.

### 3.2.3	Error detection and control

Error detection and control should be handled using the CRC polynomial as described in § xxx. CRC errors should result in no further action by the ASM.

### 3.2.4	Forward Error correction

Forward Error correction should be applied. TBD

### 3.3	Sub layer 3 – link management entity

The LME controls the operation of the DLS, MAC and the physical layer.

### 3.3.1	Access to the data link

There should be different access schemes for controlling access to the data transfer medium. The application and mode of operation determine the access scheme to be used. The access schemes are ITDMA, RATDMA, CSTDMA and FATDMA.

### 3.3.1.1	Cooperation on the data link

The access schemes operate continuously, and in parallel, on the same physical data link. They all conform to the rules set up by the TDMA (see § xxx). The ASM and VDE systems must give priority to the AIS system when accessing the physical data link.

### 3.3.1.2	Candidate slots

Slots, used for transmission, are selected from candidate slots in the selection interval (SI) (see Fig. xxx). The selection process uses received data. There should always be at minimum four candidate slots to choose from unless the number of candidate slots is otherwise restricted due to loss of position information (see § xxx). When selecting candidates for messages longer than one (1) slot (see § xxx) a candidate slot should be the first slot in a consecutive block of free or available slots. When no candidate slot is available, the transmission should be re-scheduled. The candidate slots are primarily selected from free slots (see § xxx). When required, available slots are included in the candidate slot set. When selecting a slot from the candidates, any candidate has the same probability of being chosen, regardless of its slot state (see § xxx). If the station cannot find any candidate slots at all, because all slots in the SI are restricted from slot reuse (see § xxx), the station should not reserve a slot and re-schedule the transmission.

Example:

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E | E | F | F | F | F | F | E |

A three-slot-message is to be sent. Only slot Nos. 2, 3 and 4 should be considered candidates.

figure 8

Transmission timing

When selecting among candidate slots for transmission in one channel, the slot usage of other channels should be considered. If the candidate slot in the other channel is used by another station, the use of the slot should follow the same rules as for slot reuse (see § xxx). If a slot in either channel is occupied by or allocated by other base station or mobile station, that slot should be reused only in accordance with § xxx.

The slots of another station, whose navigational status is not set to “at anchor” or “moored” and has not been received for 3 min, should be used as candidate slots for intentional slot reuse.

The own station is unable to transmit on an adjacent slot on the two parallel channels because of the necessary switching time (see § 2.11.1). Thus, the two adjacent slots on either side of a slot that is being used by the own station on one channel should not be considered as candidate slots on the other channel.

The purpose of intentionally reusing slots and maintaining a minimum of four candidate slots within the same probability of being used for transmission is to provide high probability of access to the link. To further provide high probability of access, time-out characteristics are applied to the use of the slots so that slots will continuously become available for new use.

Figure 9 illustrates the process of selecting among candidate slots for transmission on the link.

figure 9

### 3.3.2	Modes of operation

There should be three modes of operation. The default mode should be autonomous and may be switched to/from other modes.

### 3.3.2.1	Autonomous and continuous

A station operating autonomously should determine its own schedule for transmission. The station should automatically resolve scheduling conflicts with other stations.

### 3.3.2.2	Assigned

A station operating in the assigned mode takes into account the transmission schedule of the assigning message when determining when it should transmit.

### 3.3.2.3	Polled

A station operating in polled mode should automatically respond to interrogation messages. Operation in the polled mode should not conflict with operation in the other two modes. The response should be transmitted on the channel where the interrogation message was received.

### 3.3.3	Initialization

At power on, a station should monitor the TDMA channels for one (1) min to determine channel activity, other participating member IDs, current slot assignments and reported positions of other users, and possible existence of shore stations. During this time period, a dynamic directory of all stations operating in the system should be established. A frame map should be constructed, which reflects TDMA channel activity. After one (1) min has elapsed, the station may enter the operational mode and start to transmit according to its own schedule.

### 3.3.4	Channel access schemes

The access schemes, as defined below, should coexist and operate simultaneously on the TDMA channel.

### 3.3.4.1	Incremental time division multiple access

The ITDMA access scheme allows a station to pre-announce additional transmission slots.

### 3.3.4.1.1	Incremental time division multiple access algorithm

A station can begin its ITDMA transmission by allocating a new, unannounced slot, using RATDMA or CSTDMA.

Prior to transmitting in the first ITDMA slot, the station may randomly selects the next following ITDMA slot and calculates the relative offset to that location. This offset should be inserted into the ITDMA communication state. Receiving stations will be able to mark the slot, indicated by this offset, as externally allocated (see § xxx and § xxx). The communication state is transmitted as a part of the ITDMA transmission.  The process of allocating slots continues as long as required. In the last ITDMA slot, the relative offset is set to zero.

### 3.3.4.1.2	Incremental time division multiple access parameters

The parameters of Table 13 control ITDMA scheduling:

TABLE 13

| Symbol | Name | Description | Minimum | Maximum |
| --- | --- | --- | --- | --- |
| LME.ITINC | Slot increment | The slot increment is used to allocate a slot ahead in the frame. It is a relative offset from the current transmission slot. If it is set to zero, no more ITDMA allocations should be done | 0 | 8 191 |
| LME.ITSL | Number of slots | Indicates the number of consecutive slots, which are allocated, starting at the slot increment | 1 | 5 |
| LME.ITKP | Keep flag | This flag should be set to TRUE when the present slot(s) should be reserved in the next frame also. The keep flag is set to FALSE when the allocated slot should be freed immediately after transmission | False = 0 | True = 1 |

### 3.3.4.2	Random access time division multiple access

RATDMA is used when a station needs to allocate a slot, which has not been pre-announced. This is generally done as the first transmission slot for a message group, or for messages that cannot be pre-announced by a previous transmission.

### 3.3.4.2.1	Random access time division multiple access algorithm

The RATDMA access scheme should use a probability persistent (p-persistent) algorithm as described in this paragraph (see Table 14).

An AIS station should avoid using RATDMA. A scheduled message should primarily be used to announce a future transmission to avoid RATDMA transmissions.

Messages, which use the RATDMA access scheme, are stored in a priority first-in first-out (FIFO). When a candidate slot (see § xxx) is detected, the station randomly select a probability value (LME.RTP1) between 0 and 100. This value should be compared with the current probability for transmission (LME.RTP2). If LME.RTP1 is equal to, or less than LME.RTP2, transmission should occur in the candidate slot. If not, LME.RTP2 should be incremented with a probability increment (LME.RTPI) and the station should wait for the next candidate slot in the frame.

The SI for RATDMA should be 150 time slots, which is equivalent to 4 s. The candidate slot set should be chosen within the SI, so that the transmission occurs within 4 s.

Each time that a candidate slot is entered, the p-persistent algorithm is applied. If the algorithm determines that a transmission shall be inhibited, then the parameter LME.RTCSC is decremented by one and LME.RTA is incremented by one.

LME.RTCSC can also be decremented as a result of another station allocating a slot in the candidate set. If LME.RTCSC  LME.RTA  4 then the candidate set shall be complemented with a new slot within the range of the current slot and LME.RTES following the slot selection criteria.

### 3.3.4.2.2	Random access time division multiple access parameters

The parameters of Table 14 control RATDMA scheduling:

TABLE 14

| Symbol | Name | Description | Minimum | Maximum |
| --- | --- | --- | --- | --- |
| LME.RTCSC | Candidate slot counter | The number of slots currently available in the candidate set. NOTE 1 – The initial value is always 4 or more (see § 3.3.1.2). However, during the cycle of the p-persistent algorithm the value may be reduced below 4 | 1 | 150 |
| LME.RTES | End slot | Defined as the slot number of the last slot in the initial SI, which is 150 slots ahead | 0 | 2 249 |
| LME.RTPRI | Priority | The priority that the transmission has when queuing messages. The priority is highest when LME.RTPRI is lowest. Safety related messages should have highest service priority (refer to § 4.2.3) | 1 | 0 |
| LME.RTPS | Start probability | Each time a new message is due for transmission, LME.RTP2 should be set equal to LME.RTPS. LME.RTPS shall be equal to 100/LME.RTCSC. NOTE 2 – LME.RTCSC is set to 4 or more initially. Therefore LME.RTPS has a maximum value of 25 (100/4) | 0 | 25 |
| LME.RTP1 | Derived probability | Calculated probability for transmission in the next candidate slot. It should be less than or equal to LME.RTP2 for transmission to occur, and it should be randomly selected for each transmission attempt | 0 | 100 |
| LME.RTP2 | Current probability | The current probability that a transmission will occur in the next candidate slot | LME.RTPS | 100 |
| LME.RTA | Number of attempts | Initial value set to 0. This value is incremented by one each time the p-persistent algorithm determines that a transmission shall not occur | 0 | 149 |
| LME.RTPI | Probability increment | Each time the algorithm determines that transmission should not occur, LME.RTP2 should be incremented with LME.RTPI. LME.RTPI shall be equal to (100 − LME.RTP2)/LME.RTCSC | 1 | 25 |

### 3.3.4.3	Fixed access time division multiple access

FATDMA should be used by base stations only. FATDMA allocated slots should be used for repetitive messages. For base stations use of FATDMA refer to § xxx and § xxx.

### 3.3.4.3.1	Fixed access time division multiple access algorithm

Access to the data link should be achieved with reference to frame start. Each allocation should be pre-configured by the competent authority and not changed for the duration of the operation of the station, or until re-configured. Except where the time-out value is otherwise determined, receivers of data link management message (Message 20) should set a slot time-out value in order to determine when the FATDMA slot will become free. The slot time-out should be reset with each reception of the message.

FATDMA reservations should consist of a base station report (message 4) in conjunction with a data link management message with the same base station ID (MMSI). FATDMA reservations apply within a range of 120 nautical miles from the reserving base station. AIS stations (except when using FATDMA) should not use FATDMA reserved slots within this range. A data link management message (Message 20) without a base station report (Message 4) should be ignored. Base stations may reuse FATDMA reserved slots within this range for their own FATDMA transmissions but may not reuse FATDMA reserved slots for RATDMA transmissions.

FATDMA reservations do not apply beyond 120 nautical miles from the reserving base station. All stations may consider these slots as available.

### 3.3.4.3.2	Fixed access time division multiple access parameters

The parameters of Table 15 control FATDMA scheduling:

TABLE 15

| Symbol | Name | Description | Minimum | Maximum |
| --- | --- | --- | --- | --- |
| LME.FTST | Start slot | The first slot (referenced to frame start) to be used by the station | 0 | 2 249 |
| LME.FTI | Increment | Increment to next block of allocated slots. An increment of zero indicates that the station transmits one time per frame, in the start slot | 0 | 1 125 |
| LME.FTBS | Block size | Default block size. Determines the default number of consecutive slots which are to be reserved at each increment | 1 | 5 |

#### 4.3.1.2	Carrier sense TDMA access method

Carrier Within a time window of 1 146 µs starting at 833 µs and ending at 1 979 µs after the start of the time period intended for transmission (T0) the AIS Class B “CS” should detect if that time period is used (CS detection window).

NOTE 1 – Signals within the first 8 bits (833 µs) of the time period are excluded from the decision (to allow for propagation delays and ramp down periods of other units).

The Class B “CS” AIS should not transmit on any time period in which, during the CS detection window, a signal level greater than the “CS detection threshold” (§ 4.3.1.3) is detected.

The transmission of a CSTDMA packet should commence 20 bits (TA = 2 083 µs + T0) after the nominal start of the time period (see Fig. 35).

Figure 35

Carrier sense timing

#### 4.3.1.3	Carrier sense detection threshold

The carrier sense (CS) detection threshold should be determined over a rolling 60 s interval on each Rx channel separately. The threshold should be determined by measuring the minimum energy level (representing the background noise) plus an offset of 10 dB. The minimum CS detection threshold should be –107 dBm and background noise should be tracked for a range of at least 30 dB (which results in a maximum threshold level of –7 dBm).

#### 4.3.1.4	VHF data link access

The transmitter should begin transmission by turning on the RF power immediately after the duration of the carrier sense window (TA).

The transmitter should be turned off after the last bit of the transmission packet has left the transmitting unit (nominal transmission end TE assuming no bit stuffing).

The access to the medium is performed as shown in Fig. 36 and Table 37:

Figure 36

Power versus time mask

TABLE 37

Definition of timings for Figure 36

| Reference |  | Bits | Time (ms) | Definition |
| --- | --- | --- | --- | --- |
| T0 to TA |  | 0 | 0 | Start of candidate transmission time period Power should not exceed –50 dB of Pss |
| TA to TB |  | 20 | 2 083 | Begin of upramping |
| TB | TB1 | 23 | 2 396 | Power should reach within +1.5 or –3 dB of Pss |
|  | TB2 | 25 | 2 604 | Power should reach within +1.5 or –1 dB of Pss |
| TE (plus 1 stuffing bit) |  | 248 | 25 833 | Power should still remain within +1.5 or –1 dB of Pss |
| TF (plus 1 stuffing bit) |  | 251 | 26 146 | Power should reach –50 dB of steady state RF output power (Pss) and stay below this |

There should be no modulation of the RF after the termination of transmission (TE) until the power has reached zero and next time period begins (TG).

### 3.3.4.4	Initialization phase (after switched on)

The initialization phase is described using the flowchart shown in Fig. 11.

figure 11

Ready for transmission

##### Monitor VHF data link

At power on, a station should monitor the TDMA channel for one (1) min interval to determine channel activity, other participating member IDs, current slot assignments and reported positions of other users, and possible existence of base stations. During this time period, a dynamic directory of all members operating in the system should be established. A frame map should be constructed, which reflects TDMA channel activity.

#### Network access and entry of a new data stream

- Initial network entry using RATDMA, CSTDMA

- Entry of new data stream using previously allocations by ITDMA

- For periodic transmissions using ITDMA schedule

- Single message transmission using RATDMA, CSTDMA or scheduled ITDMA

- Dynamic allocation of additional slots using existing ITDMA

- Priority of transmissions

- Limits how many slots

### Assigned operation

A controlling station may

- Assign slots to a mobile station

- Exclude slots for autonomous use by other stations

- Limit the amount of slots for a specific station or in area

- The controlling station needs to support authentication

##### Entering assigned mode

##### Operating in the assigned mode

##### Returning to autonomous and continuous mode

### Message structure

Messages, which are part of the access schemes, should have the following structure shown in Fig. 16 inside the data portion of a data packet: TBD

Figure 16

Each message is described using a table with parameter fields listed from top to bottom. Each parameter field is defined with the most significant bit first.

Parameter fields containing sub-fields (e.g. communication state) are defined in separate tables with sub-fields listed top to bottom, with the most significant bit first within each sub-field.

Character strings are presented left to right most significant bit first. All unused characters should be represented by the @ symbol, and they should be placed at the end of the string.

When data is output on the VHF data link it should be grouped in bytes of 8 bits from top to bottom of the table associated with each message in accordance with ISO/IEC 13239:2002. Each byte should be output with least significant bit first. During the output process, data should be subject to bit-stuffing (see § 3.2.2) and NRZI coding (see § 2.6).

Unused bits in the last byte should be set to zero in order to preserve byte boundary.

Generic example for a message table:

TABLE 17

| Parameter | Symbol | Number of bits | Description |
| --- | --- | --- | --- |
| P1 | T | 6 | Parameter 1 |
| P2 | D | 1 | Parameter 2 |
| P3 | I | 1 | Parameter 3 |
| P4 | M | 27 | Parameter 4 |
| P5 | N | 2 | Parameter 5 |
| Unused | 0 | 3 | Unused bits |

Logical view of data as described in § 3.3.7:

| Bit order | M----L-- | M------- | -------- | -------- | --LML000 |
| --- | --- | --- | --- | --- | --- |
| Symbol | TTTTTTDI | MMMMMMMM | MMMMMMMM | MMMMMMMM | MMMNN000 |
| Byte order | 1 | 2 | 3 | 4 | 5 |

Output order to VHF data link (bit-stuffing is disregarded in the example):

| Bit order | --L----M | -------M | -------- | -------- | 000LML-- |
| --- | --- | --- | --- | --- | --- |
| Symbol | IDTTTTTT | MMMMMMMM | MMMMMMMM | MMMMMMMM | 000NNMMM |
| Byte order | 1 | 2 | 3 | 4 | 5 |

#### Message identification

The message ID should be 6 bits long and should range between 0 and 63. The message ID should identify the message type.

#### Incremental time division multiple access message structure

The ITDMA message structure supplies the necessary information in order to operate in accordance with § xxx. The message structure is shown in Fig. 18:

figure 18

##### User identification

The user ID should be the MMSI (see § 3, Annex 1). The MMSI is 30 bits long. The first 9 digits (most significant digits) should be used only. The tenth digit as stated in Recommendation ITU-R M.1080 may be used.

- Unique identifier, e.g. MAC address

##### Incremental time division multiple access communication state

The communication state provides the following functions:

–	it contains information used by the slot allocation algorithm in the ITDMA concept;

–	it also indicates the synchronization state.

The ITDMA communication state is structured as shown in Table 20:

TABLE 20

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Sync state | 2 | 0 UTC direct (see § 3.1.1.1) 1 UTC indirect (see § 3.1.1.2) 2 Station is synchronized to a base station (base direct) (see § 3.1.1.3) 3 Station is synchronized to another station based on the highest number of received stations or to another mobile station, which is directly synchronized to a base station (see § 3.1.1.3 and § 3.1.1.4) |
| Slot increment | 13 | Offset to next slot to be used, or zero (0) if no more transmissions |
| Number of slots | 3 | Number of consecutive slots to allocate. 0  1 slot, 1  2 slots, 2  3 slots, 3  4 slots, 4  5 slots, 5 = 1 slot; offset = slot increment + 8 192, 6 = 2 slots; offset = slot increment + 8 192, 7 = 3 slots; offset = slot increment + 8 192. Use of 5 to 7 removes the need for RATDMA broadcast for scheduled transmissions up to 6 min intervals |
| Keep flag | 1 | Set to TRUE  1 if the slot remains allocated for one additional frame (see Table 13) |

The ITDMA communication state should apply only to the slot in the channel where the relevant transmission occurs. ASM 1 and ASM 2 are independent channels.

#### 3.3.7.4	Random access time division multiple access message structure

The RATDMA access scheme may use message structures determined by message ID and may thus lack a uniform structure.

A message with a communication state may be transmitted using RATDMA in the following situations:

–	When initially entering the network (refer to § xxx).

–	When repeating a message. (?)

3.3.7.4.1	The communication state when initially entering the network should be set in accordance with § xxx and § xx.

3.3.7.4.2	The communication state when repeating a message should be set in accordance with § 4.6.3.

#### 3.3.7.5	Fixed access time division multiple access message structure

The FATDMA access scheme may use message structures determined by message ID and may thus lack a uniform structure.

A message with a communication state may be transmitted using FATDMA, e.g. when repeated. In this situation, the communication state should be set in accordance with § 4.6.3 (see also § 3.16, Annex 8).

____________________

1 Nautical mile = 1 852 metres

235.9 Nautical miles = 436 886.8 metres; 120 Nautical miles = 222 240 metres

The following example is compliant with the requirement:

Sample the RF signal strength at a rate >1 kHz, average the samples over a sliding 20 ms period and over a 4 s interval determine the minimum period value. Maintain a history of 15 such intervals. The minimum of all 15 intervals is the background level. Add a fixed 10 dB offset to give the CS detection threshold.