# IALA ENAV Working Group 3 – Telecommunication

The IALA eNAV working group 3 telecommunication met at an intersessional meeting in St. Germaine-en-Laye the week of February 23, 2015.  The agenda called for progress on the following objectives:

- Advance work on the PDNR for VDES;

- Advance work on User Requirements to drive Design Requirements;

- Provide ITU feedback as required;

The input documents can be found at:

ftp: ftp.e-navigation.nl

username: ftpuser

password: @C0penhagen

Under the Directory:    Feb 2015 WG3 Comms intersessional/INPUT

The output documents can be found at:

ftp: ftp.e-navigation.nl

username: ftpuser

password: @C0penhagen

Under the Directory:    Feb 2015 WG3 Comms intersessional/OUTPUT

The group met in plenary to discuss each input document and the following presentations were made:

- JRC/IHI on the ASM protocol coordination;

- DLR on the terrestrial and satellite aspects of VDES;

- JCG on the report for the last Tokyo workshop where a draft IMO Performance Standard for VDES was initiated;

- SAAB on the current progress of Swedish testbeds;

- Vice-Chair Stefan Bober on behalf of Canada on VDES usage in Canada;

- Furuno on Output Spectrum and Adjacent Channel Power of Transmitter for ASM and VDE channel;

After the presentations and review of the input documents the group agreed to break into two subgroups:

- WG1:  Advance work on the PDNR for VDES – chaired by Dr. Nader Alagha from ESA ;

- WG2:  Advance work on User Requirements to drive Design Requirements – chaired by Ms. Jillian Carson Jackson from AMSA;

The ITU feedback will be addressed during eNAV16.

The chair and vice-chair would like to thank all of the attendees and their sponsoring organizations.  This work is extremely important for the future of maritime comms.   Further, the work is very time sensitive in order to meet the requirements of ITU and ultimately IMO.

## Planning/Actions

IALA eNAV working group 3 telecommunication will continue to advance the work using telecoms, email lists, etc. and will hold another intersessional at IALA headquarters the week of May 11, 2015.    We will provide our working documents into the next eNAV meeting.   At that time we will inform the eNAV Committee of our planned changes to the PDNR and ask permission to send our PDNR directly to council (meeting May 26th) for approval and submission to ITU WP5B by the beginning of June.

Action 1:  WG1 chair will provide a draft document of all developed changes and the Chair and Vice-chair will incorporate those changes into the official ITU PDNR and submit to eNAV16 as an official input document;  Due to Chair/VChair – April 10, 2015;

Action 2:  WG2 chair will provide a working document for input into eNAV16;

Action 3:  All WG members will review the VDES Channel select document and update as required for input into eNAV16;

# Report from WG1 – PDNR for VDES

## Scope:

The scope of this working sub-group was to review the PDNR document submitted to ITU in October 2014 and update according to the new inputs and findings in the group.

## Starting point:

The sub-group decided to restructure the PDNR document in order to clarify the content of each Annex, as a technical specification as opposed to an informative paper.

The group agreed on the following structure:

- Body of the recommendation (scope, references, glossary, tec.) remains the same as before

- Annex 1: Operational characteristics of a VHF data exchange system  in the VHF maritime mobile band

- Annex 2: Technical characteristics of the Application Specific Messages (ASM) for the VDES in the VHF maritime mobile band

- Annex 3: Technical characteristics of VDE-TER

- Annex 4: Technical characteristics of VDE-SAT Downlink

- Annex 5: Technical characteristics of VDE-SAT Uplink

- Annex 6: Spectrum Sharing Strategies for the VDE terrestrial and VDE satellite services

The group agreed to re-use, as much as possible, the content of the previous version of the PDNR as submitted to ITU after eNav16. Changes to the original text would be highlighted in track-change mode.

## Summary of Technical discussions

Input contributions corresponding to each Annex of the PDNR document were discussed. For each new annex (Annex 2-5), an extended table of content was proposed and discussed.

### Annex 2: ASM

Inputs from representatives of the USCG and JRC were discussed.

### FEC Selection

DLR presented results of Forward Error Code Selection for different block sized. In order to maintain a coding FEC scheme, it was suggested and agreed to use a common coding scheme for ASM, VDE-TER and VDE-SAT.

The Turbo Coding scheme form the existing standards (such as 3GPP, and CCSDS) was proposed.

- 3GPP2 C.S0024-200-C, “Physical Layer for cdma2000 High Rate Packet Data Air Interface Specification,” v2, December 2011, Section 1.3.1.3.4.2.

- Bit interleaver: CCSDS, “TM Synchronization and Channel Coding”. Blue Book. Issue 2. August 2011.

For this proposed coding scheme, the information block size can be from 40 bits up to 12 288 bits (with granularity of 1 byte). Code rates can be 1/5, 1/4, 1/3, 1/2, 2/3.

### Annex 3: VDE-TER

An extended table of Content was provided and discussed.

Discussions regarding the frame structure, the unique sequence (preamble selection) and adaptive coding and modulation signalling were presented.

Furuno Electric Co. presented their work on Digital Pre-distortion techniques and its applicability to VDE transmitters to reduce spectral regrowth. The contribution was considered relevant to the design of the transceiver and provide effective way to counteract the impact of non-linearity. However, there is no need to add this contribution to the PDNR document.

### Annex 4: VDE-SAT

Input paper from ESA on technical consideration on VDE-SAT downlink was presented.

A table of content for Annex 4 was presented.

## Planning/Action Items

In preparation of each Annex, The work on each Annex will be coordinated by:

Annex 1: Stefan Bober, Peggy Browning

Annex 2: Johnny Schultz

Annex 3: Johan Lindborg

Annex 4: Hans Haugli/Nader Alagha

Annex 5: Hans Haugli/Nader Alagha

Annex 6: Nader Alagha

Regular teleconferences will be planned to review the working documents and inputs provided by the group members. Those members who are interested to participate and contribute to each Annex, should contact the work coordinators listed above.

# Report from WG2 – User Requirements

## Scope

The User Requirements working group developed specific use case narratives, which then led to the development of user requirements.  The work structure focused on a review of the template user requirements document as presented at Plenary, and identified guidance for a brainstorm session to further the work.

## Summary of discussion

The group prepared three working documents:

- the outcome of the brainstorm session (mindmap / word document);

- the main user requirement (word) document;

- an excel table that provides more detailed information on the use cases and user requirements.

It was agreed to keep the three documents as working documents within ENAV WG3.  To further the development of the main user requirement working document, the following agreed to address specific use case scenario developments.  Input from all members will be welcomed in the development of the document.

It was noted that the user requirements document, when completed to an initial draft level, should be reviewed by other working groups of ENAV as well as other IALA committees – for example, the VTS Committee and ARM Committee.

Work on the Use Case and the User Requirements (Comms WG-SG-Feb 2015-UseRequirements-WD-02) will continue intersessionally.  The work will be progressed by the identified primary reviewer, however all members are welcome to provide input. Input is to be provided to the Primary Reviewer, and forwarded to J Carson-Jackson by 10 April 2015.

| Section | Primary Reviewer |
| --- | --- |
| Use Cases |  |
| 3.1.1 SAR communications (UC-1) | Jean-Charles Cornillou |
| 3.1.2 Maritime Safety Information (UC-2) | Brian Tetreault (coordinate) |
| 3.1.2.1 Weather warnings (UC-2.1) | Lars Loge (Jillian) |
| 3.1.2.2. Ice Maps (UC-2.2) | LarsLoge |
| 3.1.2.3 Notices to Mariners (UC-2.3) | Brian Tetreault |
| 3.1.2.4 SBAS Corrections (Polar) (UC-2.4) | Lars Loge |
| 3.1.3 Ship Reporting (UC-3) | Robert Tremlett |
| 3.1.4 Vessel Traffic Services (UC-4) | Jillian Carson-Jackson |
| 3.1.5 Update Charts and Publications (UC-5) | Jillian Carson-Jackson |
| 3.1.6 Route Exchange (UC-6) | Jeffrey van Gils |
| 3.1.7 Logistics (UC-7) | Brian Tetreault |
| Requirements (from brainstorm / put into excel) |  |
| 3.2 | Stefan Pielmeier |

In discussion it was noted that there are other user requirement documents either developed or in development that could be of relevance.  These include the IHO work on S-100 and the development of MSP.  It was also noted that IEC 61174 Maritime navigation and radiocommunication equipment and systems – Electronic chart display and information system (ECDIS) – Operational and performance requirements, methods of testing and required test results may include route exchange in ECDIS.  It was agreed that these documents would be valuable as reference documents for the work on the user requirements of VDES.

## Planning/Action Items

Members to review the use cases and requirements as per the above table and provide input to J Carson-Jackson by 10 April, 2015.

J Carson-Jackson to compile input received from the review of the use cases and requirements and provide a revised draft document by 20 April, 2015.

The Chair of the ENAV WG 3 (Communications) forward the working documents (WD01, WD02, WD03) for continuing development ENAV16.

# eNAV committee requirements progress summary

| Task | Start Session | Planned End Session | Revised End Session | Progress Indicator |  |  | Status Overview |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Green | Yellow | Red |  |
| TD#1 – Operations |  |  |  |  |  |  |  |
| 1.4.1 Develop VDES Message Structures | 17 | 21 |  | ☒ | ☐ | ☐ | Commence ENAV20 |
| 1.4.2 Develop e-NAV Message Structure | 17 | 21 |  | ☒ | ☐ | ☐ | Commence ENAV20 |
| 2.1.1 Update the Marine Radio Communication Plan | 20 | 21 |  | ☒ | ☐ | ☐ | Commence ENAV20 |
| 2.1.2 Develop Recommendation on VDES | 15 | 21 |  | ☒ | ☐ | ☐ | On-going |
| 2.2.1 Update Recommendations and Guidelines for AIS | 15 | 21 |  | ☒ | ☐ | ☐ | On-going |
| 2.3.1 Manage Application Specific Message (ASM) catalogue | 16/20 | 17/21 |  | ☒ | ☐ | ☐ | Commence ENAV16 and 20 |
| 2.4.1 Monitor Developments with GMDSS Modernization | 15 | 21 |  | ☒ | ☐ | ☐ | On-going |

* On target, behind target but under control behind target needs action

# Output Papers

Intersessional output papers will be referred to the next eNAV for further progress.

| Number |  | Title | Status |
| --- | --- | --- | --- |
| Output 1 |  | Last Official PDNR submitted to ITU_R12-WP5B-C-0761!N08!MSW-E.docx | Working document |
| Output 2 |  | VDES channel select document_R12-WP5B-C-0761!N13!MSW-E17-20-2015.docx | Working document |
| Output 3 |  | WD_Annex 3 VDE TER v0.1.docx | Working document |
| Output 4 |  | WD_VDES Satellite downlink v1 Haugli.docx | Working document |
| Output 5 |  | IALA-1082_AIS_Overview_Ed2_WD15022.docx | Working document |
| Output 6 |  | R12-WP5B-C-0695!!MSW-E(1) Canada study regarding VDES channel plan.docx | Working document |
| Output 7 |  | WD_Comms WG-SG-Feb-2015-UserRequirements-WD01-Use Case Studies – Brainstorm.docx | Working document |
| Output 8 |  | WD_Comms WG-WG-Feb-2015-UserRequirements-WD03-rev1-Use Case-Requirements.docx | Working document |
| Output 9 |  | WD_Comms WG-WG-Feb-2015-UserRequirements-WD02-rev1-Use Case-Requirements.xlsx | Working document |
| Output 10 |  | Tokyo_workshop_report(editorial changed).pdf | For review at eNav16 plenary |
| Output 11 |  | DRAFT INF PAPER on VDES for MSC95.doc | For review at eNav16 plenary |
| Output 12 |  | PDNR-M_VDES Annex 2.docx | Working document |

# Attendees

| Name | Organization | Email | Subgroup |
| --- | --- | --- | --- |
| Hideki Noguchi | Japan Coast Guard | hideki.noguchi@gmail.com | User Rqmts |
| Arunas Macikunas | exactEarth | arunas.macikunas@exactearth.com | PDNR |
| Johnny Schultz | USCG/Odyssey | johnny.schultz@odysseyconsult.com | PDNR |
| Takuo Kashiwa | Furuno Electric Co. | takuo.kashiwa@furuno.co.jp | PDNR |
| Hiroyasu Nakagawa | Furuno Electric Co. | hiroyasu.nakagawa@furuno.co.jp | PDNR |
| Johan Lindborg | SAAB Transponder Tech AB | johan.lindborg@saabgroup.com | PDNR |
| Ernest Batty | IMIS Global LTD | ernie.b@IMISGLOBAL.com | None |
| Brian Tetreault | US Army Corp of Engineers | brian.j.tetreault@usace.army.mil | User Rqmts |
| Jillian Carson Jackson | AMSA | jac@amsa.gov.au | User Rqmts – Chair |
| Stefan Pielmeier | Cobham, DK | stefan.pielmeier.cobham.com | User Rqmts |
| Yoshio Miyadera | Japan Radio Co., LTD | miyadera.yoshio@jrc.co.jp | PDNR |
| Atsushi Karaki | IHI Co., LTD | atsushi_karaki@ihi.co.jp | PDNR |
| Per Chr. Berntsen | Kongsberg Seatex | pcb@kongsberg.com | PDNR |
| Mark Johnson | USCG/Shine Micro | classbais.gmail.com | PDNR |
| Kaisu Heikonen | FinnishTransport Agency | kaisu.heikonen@fta.fi | User Rqmts |
| Christian Rissone | ANFR/ITU | christian.rissone@anfr.fr | PDNR |
| Jean-Charles Cornillou | Cerema/French Maritime Administration | jean-charles.cornillou@cerema.fr | User Rqmts |
| Lars Loge | Space Norway/Norwegian Coastal Administration | lars.loge@statsat.no | User Rqmts |
| Hans Haugli | Space Norway/Norwegian Coastal Administration | hans.christian.haugli@spacenorway.no | PDNR |
| Krzysztof Bronk | National Institute of Telecommunic | k.bronk@itl.waw.pl | PDNR |
| Takamasa Yauchi | OKI Consulting Solutions Co., LTD | yauchi427@oki.com | PDNR |
| Robert Tremlett | AMS | robert.tremlett@ne.com | User Rqmts |
| Jeffrey van Gils | RWS | jeffrey.van.gils@rws.no | User Rqmts |
| Nader Alagha | ESA | nader.alagha@esa.int | PDNR – Chair |
| Gianluigi Liva | DLR | gianluigi.liva@dlr.de | PDNR |
| Ronald Raulefs | DLR | ronald.raulefs@dlr.de | PDNR |
| Jean-Jacques Valette | CLS | jvalette@cls.fr | User Rqmts |
| Pierre Debusschere | CLS | pdebusschere@cls.fr | User Rqmts |
| Stefan.Bober | WSV | stefan.bober@wsv.bund.de | Vice-chair |
| Peggy Browning | exactEarth | peggy.browning@exactearth.com | Chair |