ENAV 16	Input

Agenda item	11

Task Number

Author(s)	Yoshio MIYADERA (JRC)

# The Result of CPM15-2 related to WRC-15 AI 1.16

# Introduction

The second session of the Conference Preparatory Meeting for WRC-15 (CPM15-2) was held at Geneva from 23 March to 2 April 2015, under the chairmanship of Mr. Mr. Aboubakar ZOURMBA (Cameroon). The Chairman of SWG 3b (AI 1.15+1.16-Maritime), Mr. Christian RISSONE (France), was also present.

The SWG 3b considered 12 input contributions addressing AI 1.16. After detailed discussion, the methods to satisfy the agenda item have been modified as follows in the draft CPM text. The output is attached as Annex.

Issue A - ASM
Method A3 has been added.

Issue B - Terrestrial VDE
Editorial changed.

Issue C - Satellite part of VDES
Method C1 was divided into C1-A and C1-B

Issue D - Regional VDE
No changed.

# over view of each method

## Issue A - ASM

| Method | ASM channel (effective implementation date) (Referenced ITU-R Rec.) | Transmission from ship on voice CHs 2078, 2019, 2079 and 2020 (To prevent blocking of the reception of the channels AISs and ASMs) |
| --- | --- | --- |
| A1 | 2027, 2028 (1 Jan. 2019) (Rec. ITU-R M.[VDES]) | prohibition |
| A2 | 87, 88 (1 Jan. 2019) (-) | limiting the output power to 1 W |
| A3 | 2027, 2028 (- ?) (Rec. ITU-R M.[VDES]) | Depend on the Administrations (including prohibition) |

## Issue B - Terrestrial VDE

| Method | Terrestrial VDE channel (effective implementation date) (Referenced ITU-R Rec.) |
| --- | --- |
| B1 | 24, 84, 25 and 85 (1 Jan. 2019) (Rec. ITU-R M.[VDES]) |
| B2 | 24, 84, 25, 85, 26 and 86 ([1 Jan. 2019]) (Rec. ITU-R M.1842) |

## Issue C - Satellite part of VDES

| Method | SAT-VDE channel (uplink) Priority (effective date) (Referenced ITU-R Rec.) | SAT-VDE channel (downlink) Priority pfd mask (effective date) (Referenced ITU-R Rec.) | ASM satellite detection Priority (effective date) (Referenced ITU-R Rec.) |
| --- | --- | --- | --- |
| C1-A | 1024, 1084, 1025, 1085, 1026, 1086, 2027 and 2028 MMSS (Earth-to-spase), Secondary (1 Jan. 2019) (Rec. ITU-R M.[VDES]) | 2024, 2084, 2025, 2085, 2026 and 2086 MMSS (space-to-Earth), Secondary RR Article 5 (1 Jan. 2019) (Rec. ITU-R M.[VDES]) | 2027, 2028 MMSS (Earth-to-spase), Secondary (1 Jan. 2019) (Rec. ITU-R M.[VDES]) |
| C1-B | 1024, 1084, 1025, 1085, 1026, 1086, 2027 and 2028 MMSS (Earth-to-spase), Secondary (1 Jan. 2019) (Rec. ITU-R M.[VDES]) | 2024, 2084, 2025, 2085, 2026 and 2086 MMSS (space-to-Earth), Primary Annex 1 to RR Appendix 5 (1 Jan. 2019) (Rec. ITU-R M.[VDES]) | 2027, 2028 MMSS (Earth-to-spase), Secondary (1 Jan. 2019) (Rec. ITU-R M.[VDES]) |
| C2 | 148-149 MHz MSS(Earth-to-space), Primary (existing) (-) | 137-138 MHz MSS(space-to-Earth) Secondary RR Article 9 (existing) (-) | 148-149 MHz MSS(Earth-to-space), Primaary (existing) (-) |

## Issue D - Regional VDE

| Method | Regional VDE channel (Terrestrial) (effective implementation date) (Referenced ITU-R Rec.) |
| --- | --- |
| D | 80, 21, 81, 22, 82, 23 and 83 (1 Jan. 2017) (Rec. ITU-R M.1842) |

# annex

Modification to Chapter 3 of draft CPM Report - Agenda item 1.16 (Document CPM15-2/225).

______________