ENAV 16	Input

Agenda item	11

Task Number

Author(s)	Yoshio MIYADERA (JRC)

# Additional substance for PDNR ITU-R M.[VDES] Annex 2

# 4	Optional message for long-range application

Long-range application may be provided by VDES equipment as an optional application. It also may provide by dedicated equipment for long-range application.

## 4.1	General

The medium access control (MAC) sub layer provides a method for granting access to the data transfer medium, i.e. the VHF data link. The method used is a TDMA scheme using a common time reference.

# 4.1	Long-range applications by broadcast

Long-range ASM receiving systems may receive long-range ASM broadcast messages, provided these messages are appropriately structured and transmitted to suit the receiving systems.

## 4.2	Packet bit structure for the automatic identification system long-range broadcast message

Long-range ASM receiving systems require suitable guard period and buffering in order to preserve the integrity of the ASM message in the ASM slot boundaries. Table ** shows an example of modified packet bit structure that is designed to support reception of ASM messages by satellites with orbital altitudes up to 600 km.

TABLE **

Modified packet bit structure for long-range ASM message reception

| Slot composition | Bits | Notes |
| --- | --- | --- |
| Guard period | 100 | Propagation time delay difference = 144 bits Propagation time delay difference of the other ASM messages = 28 bits Ramp up = 16 bits Note - Propagation time delay difference may be changed in accordance with the target satellite orbital altitude |
| Ramp up | 16 | Standard |
| Training sequence | 24 | Standard |
| Data field | 170 | Data field is 384 bits for other single-slot ASM messages. This field is shortened by 194 bits to support the long-range receiving system Message ID = 6 bits User ID = 30 bits Spare = 6 bits Binary data = 128 bits Note – Binary data length may be changed in accordance with the target satellite orbital altitude |
| CRC | 32 | Standard |
| End flag | 8 | Standard |
| Long-range ASM receiving system buffer | 162 | Bit stuffing = 8 bits Synch jitter (mobile station) = 6 bits Synch jitter (mobile/satellite) = 2 bits Propagation time delay difference = 144 bits Spare = 2 bits Note - Propagation time delay difference may be changed in accordance with the target satellite orbital altitude |
| Total | 512 | Standard |

## 4.2.1	Long-range automatic identification system broadcast message

The long-range ASM broadcast message data field is shown in Table **.

This message should be transmitted by shipborne mobile station.

TABLE **

Long-range ASM broadcast message data field

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for this message; always ** |
| User ID | 30 | MMSI number |
| Binary data | 138 |  |
| Spare | 16 | Set to zero, to preserve byte boundaries |
| Total number of bits | 190 |  |

## 4.3	Transmission method for the long-range automatic identification system broadcast message

The long-range ASM broadcast message may be transmitted using an arbitrary ASM channel at the current power setting as a transmit-only function.

### 4.3.1	Access scheme

There should be different access schemes for controlling access to the data transfer medium. The application and mode of operation determine the access scheme to be used.

The access schemes for the long-range ASM are ITDMA, RATDMA and CSTDMA. The ITDMA and RATDMA should be used in accordance with § 3.3.

### 4.3.2	Carrier sense time division multiple access

The CSTDMA access scheme may be selected by the long-range ASM message application as an optional.

#### 4.3.2.1	Carrier sense detection method

Within a time window of 1 146 µs starting at 833 µs and ending at 1 979 µs after the start of the time period intended for transmission (T0) ASM equipment using CSTDMA should detect if that time period is used (CS detection window).

NOTE 1 – Signals within the first 16 bits (833 µs) of the time period are excluded from the decision (to allow for propagation delays and ramp down periods of other units).

ASM equipment using CSTDMA should not transmit on any time period in which, during the CS detection window, a signal level greater than the “CS detection threshold” (§ 4.3.2.2) is detected.

The transmission of a CSTDMA packet should commence 90 bits (4 688 µs + T0) after the nominal start of the time period (see Fig. 35).

Figure **

Carrier sense timing

#### 4.3.2.2	Carrier sense detection threshold

The carrier sense (CS) detection threshold should be determined over a rolling 60 s interval on each Rx channel separately. The threshold should be determined by measuring the minimum energy level (representing the background noise) plus an offset of 10 dB. The minimum CS detection threshold should be –107 dBm and background noise should be tracked for a range of at least 30 dB (which results in a maximum threshold level of –7 dBm).

#### 4.3.2.3	Multi-channel operation

If ASM uses only for long-range application with only CSTDMA access scheme. The ASM should be capable of receiving on two independent channels and transmitting on two independent channels. At least one TDMA receiving process should be used to receive on two independent frequency channels. One TDMA transmitter should be used to alternate TDMA transmissions on two independent frequency channels.

### 4.4	Transmitting the long-range broadcast message

The long-range ASM broadcast message should be transmitted only on ASM channels and not on the AIS channels (channels 75, 76, AIS 1, AIS 2 or regional channels).

____________________

The following example is compliant with the requirement:

Sample the RF signal strength at a rate >1 kHz, average the samples over a sliding 20 ms period and over a 4 s interval determine the minimum period value. Maintain a history of 15 such intervals. The minimum of all 15 intervals is the background level. Add a fixed 10 dB offset to give the CS detection threshold.