Design considerations for VDES

This document is a compilation of ideas expressed at WG3+4 intersessional meeting in Sait Germain en Laye 14.03.31-14.04.04   and amended at ENAV 15, 16th October 2014

Assuming Channel Plan A

1.) VDES Requirements:

- Shall protect AIS integrity for collision avoidance and identification purposes;

  - Consider scheduled AIS transmissions

  - Consider slot selection rules of AIS  to avoid the loss of near targets on AIS

  - Similar service area as AIS (power 12.5 W, sensitivity)

- The duty cycle of any given transmission shall respect AIS and voice comms;

  - Limit length of ASM and VDE transmissions

  - Limit amount of ASM and VDE transmissions per minute (mobile, base station),
maybe adapted dynamically  depending on channel load

- The transition period between having a VDES service and capabilities available on ships  shall be carefully considered;

  - Existing AIS equipment may not receive on ASM and VDE

  - interference with current VHF voice communications

There will be a transition period where ASMs will be on AIS and/or ASM;

2.) ASM Requirements

ASM Channels shall, to the extent that is practical, support all application specific message (ASM) formats:

- Shall protect AIS integrity for collision avoidance and identification purposes;

- Shall protect AIS integrity for shore side monitoring and control purposes (cargo type, etc.);

- The duty cycle of any given ASM transmission must respect AIS and voice comms, and if practical VDE;

- The ASM modulation may be variant on a per slot basis; (one modulation scheme for ASM only)

- The ASM shall use an efficient modulation scheme, not necessarily GMSK;

- The ASM shall provide a robust link, e.g. FEC;

- The ASM shall support differentiated priority of message transmissions as described in the GMDSS;

- All ASM transmissions must have a unique identity (MMSI, MAC, etc.);

- The ASM should maximize, to the extent that is practical, satellite reception; (e.g. FEC)

- The ASM channels do not need to duplicate transmissions on both channels, two independent data channels;

- It shall be possible to manage the capacity and use of the ASM channels from shore side;

  - Set limits for amount of ASM messages per ASM channel per unit,

  - Different limits for different ASM 1 and ASM 2

  - Allow for reservation of slots

- The ASM link shall support a maximum control range of TBD with considerations for authentication;

- The ASM link may support at a minimum line of sight communications;

- Synchronisation: either UTC direct or derived from received AIS messages.

3.) VDERequirements :

- The VDE shall support differentiated priority of message transmissions as described in the GMDSS;

- The VDE shall provide a robust link, e.g. FEC;

- The VDE medium access schemes and waveform may include multiple modulation and encoding schemes as dictated by the link requirements;

- The VDE shore side may support full duplex operation;

- The VDE shall support semi duplex operation;

- The VDE shall support a satellite sharing scheme (freq. and/or time);

- The VDE transmissions shall have a unique identity;

- The duty cycle of any given VDE transmission shall respect AIS, ASM, and voice comms;

- It shall be possible to manage the capacity and use of the VDE channels  from shore side;

- The VDE link shall support a maximum control range of TBD with considerations for authentication;

- The VDE link may support  a minimum terrestrial  service range within line of sight at a lower quality of service;