ENAV15-3.3.1

| Radiocommunication Study Groups | ENAV16-11.7 Formerly ENAV15-14.2.42 |
| --- | --- |
| Source: Annex 24 to document 5B/475 Received: | Document 5B/ -E |
|  | 2014 |
|  | English only |
| International Association of Marine Aids to Navigation and Lighthouse Authorities |  |
| Liaison Note to ITU-R WP 5B |  |
| Regarding A WORKING DOCUMENT TOWARD A PRELIMINARY DRAFT NEW RECOMMENDATION ITU-R M.[VDES] |  |

# 1	Background

IALA thanks ITU-R WP 5B for the opportunity to contribute to studies and the work for WRC-15 agenda item 1.16. This liaison note provides a contribution on the Working Document Toward a Preliminary Draft New Recommendation ITU-R M. [VDES].

# 2	Discussion

IALA has further developed the Working Document Toward a Preliminary Draft New Recommendation ITU-R M. [VDES]. Due to the complexity of the changes, IALA is submitting the changes in its entirety as a new document to replace the existing ITU-R WP5B Annex 24 to document 5B/475.

# 3	Action requested

IALA plans to further develop the Working Document Toward a Preliminary Draft New Recommendation ITU-R M. [VDES] and submit a contribution to the May 2015 ITU-R WP5B meeting.

IALA requests ITU-R WP 5B to consider this information in the future development of the Working Document Toward a Preliminary Draft New Recommendation ITU-R M. [VDES] and provide comments to IALA to continue work on the VDES Recommendation.

______________

| Radiocommunication Study Groups |  |
| --- | --- |
| Ref: Document 5B/475 Annex 24 (9 January 2014) Document 5B/636 Annex 5 (30 June 2014) Document 5B/636 Annex 28 (9 July 2014) Document 5B/636 Annex 29 (9 July 2014) Document 5B/636 Annex 30 (9 July 2014) |  |
| Working Document Toward a Preliminary Draft New Recommendation ITU-R M.[VDES]* |  |
| Technical characteristics for a VHF data exchange system in the VHF maritime mobile band |  |

Scope

This Recommendation provides the technical characteristics of a VHF data exchange system (VDES) which integrates the functions of VHF data exchange (VDE), application specific messages (ASM) and the automatic identification system (AIS) in the VHF maritime mobile band (156.025-162.025 MHz).

Keywords:

[TBD]

Glossary:

ACPR 	Adjacent channel power ratio

AIS	Automatic identification system

AMSL	Above mean sea level

ASM 	application specific messages

CIRM 	Comité International Radio Maritime

DA 	Doherty Amplifier

DPD 	Digital Pre-Distortion

DSC	digital selective calling

ET 	Envelope Tracking

FATDMA	fixed access time-division multiple access

FEC 	Forward error correction

HAAT	Height above average terrain

IALA 	International Association of Marine Aids to Navigation and Lighthouse Authorities

ICAO 	International Civil Aviation Organization

IEC	International Electrotechnical Commission

IMO 	International Maritime Organization

ITDMA	incremental time division multiple access

LEO 	low-earth orbiting

MEO 	medium-earth orbiting

MMSI	maritime mobile service identity

OFDM 	orthogonal frequency division multiple access

PAPR 	peak to average power ratio

PFD 	power flux-density

QoS 	Quality of Service

RR 	radio regulations

SOLAS	safety of life at sea convention

SOTDMA 	self-organized time division multiple access

VDES 	VHF data exchange system

VDL	AIS VHF data link

VTS 	vessel traffic services

The ITU Radiocommunication Assembly,

considering

a)	that the International Maritime Organization (IMO) has a continuing requirement for a universal shipborne automatic identification system (AIS);

b)	that the use of a universal shipborne AIS allows efficient exchange of navigational data between ships and between ships and shore stations, thereby improving safety of navigation;

c)	VDES should use appropriate access schemes that ensure the protection of AIS while making efficient use of the spectrum and accommodate all users;

d)	that while AIS is used primarily for surveillance and safety of navigation purposes in ship to ship use, ship reporting and vessel traffic services (VTS) applications, a growing need for other maritime safety related communications has developed;

e)	that the VHF data exchange system (VDES) shall give priority to AIS, and also accommodate future expansion in the number of users and diversification of data communications applications, including vessels which are not subject to IMO AIS carriage requirements, aids to navigation and search and rescue;

f)	that the VDES has data communications capacity and technical characteristics that support the harmonized collection, integration, exchange, presentation and analysis of marine information onboard and ashore by electronic means to enhance berth to berth navigation and related services for safety and security at sea and protection of the marine environment;

recognizing

that the implementation of VDES must ensure that the functions of digital selective calling (DSC), AIS and voice distress, safety and calling communication (Channel 16), are not impaired;

noting

that the report ITU-R M.[VDES-SELECT] describes the use cases and requirement for VDES,

recommends

1	that VDES should be designed in accordance with the operational characteristics given in Annex 1 and the technical characteristics and examples given in the following Annexes;

2	that applications of the VDES which make use of application specific messages (ASM) designed for AIS, as defined in Recommendation ITU-R M.1371 should also take into account the international application identifier branch, as specified in IMO SN Circ. 289, maintained and published by IMO;

3	that the design and installation of VDES should also consider relevant technical requirements, recommendations and guidelines published by IMO, IEC and IALA.

Annex 1

Operational characteristics of a VHF data exchange system (VDES)
in the VHF maritime mobile band

# General

  - The system should give its highest priority to the automatic identification system (AIS) position reporting and safety related information.

  - The system installation should be capable of receiving and processing the digital messages and interrogating calls specified by this Recommendation.

  - The system should be capable of transmitting additional safety information on request.

  - The system installation should be able to operate continuously while under way, moored or at anchor.

  - The system should use for the terrestrial links time-division multiple access (TDMA) techniques, access schemes and data transmission methods in a synchronized manner as specified in the Annexes.

  - The system should be capable of various modes of operation, including the autonomous, assigned and polled modes.

  - The system should provide flexibility for the users in order to prioritize some applications and consequently adapting some parameters of the transmission (robustness or capacity) while minimizing system complexity.

  - The system should address the use cases identified in the report ITU-R M.[VDES-SELECT].

# VHF data exchange system functions and frequency usage

VDES functions and frequency usage are illustrated pictorially in Figure 1.

Figure 1

VDES functions and frequency usage

NOTE – SAT Up is receive-only by satellite.

## VHF data exchange system channel usage in accordance with RR Appendix 18

### VHF data exchange system data exchange between terrestrial stations

- AIS 1 (channel 2087 ) and AIS 2 (channel 2088 ) are AIS channels, in accordance with Recommendation ITU-R M.1371;

- ASM 1 (channel 2027) and ASM 2 (channel 2028) are the channels used for application specific messages (ASM);

- VDE1-A lower legs (channels 1024, 1084, 1025, 1085) are ship-to-shore VDE;

- VDE1-B upper legs (channels 2024, 2084, 2025, 2085) are shore-to-ship and ship-to-ship VDE.

### VHF data exchange system data exchange between satellites and terrestrial stations

- AIS 1 (channel 2087) and AIS 2 (channel 2088) are terrestrial AIS channels that are also used as uplinks for receiving AIS messages by satellite;

- Long Range AIS using channel 75 and channel 76 are specified channels to be used as uplinks for receiving AIS messages by satellite. SAT Up1 (channel 2027) and SAT Up 2 (channel 2028) are used for receiving ASM by satellite;

- SAT Up3 (channels 1024, 1084, 1025, 1085,1026 and 1086) are used for ship-satellite VDE uplinks;

- SAT Downlink (channels 2024, 2084, 2025, 2085, 2026 and 2086) are used for satellite-ship VDE downlinks.

### Technical characteristics

#### Shipborne VHF data exchange system receivers are protected

As in AIS, shipborne VDES receivers are on the upper legs of RR Appendix 18, 4.6 MHz above the lower legs, which facilitates protection by filtering from receiver blocking by ships VHF radios.

#### SAT Downlink

The satellite downlink complies with the power flux-density (PFD) mask described in Table 6 (section 10) to minimize interference to terrestrial services and to maximize reception by ship VDES stations.

#### VDE1 uses both legs of the duplex channels

Channel capacity is utilized for the duplex channels in VDE1 by using the lower legs (VDE1-A) for ship to shore and the upper legs (VDE1-B) for shore-to-ship and ship-to-ship digital messaging.

Table 1 describes the RR Appendix 18 channels used for the various applications of VDES.

Table 1

RR Appendix 18 channels for VDES applications: AIS, ASM, VDE

| RR Appendix 18 channel number |  | Transmitting frequencies (MHz) |  |
| --- | --- | --- | --- |
|  |  | Ship stations (ship-to-shore) (long range AIS) Ship stations (ship-to-satellite) | Coast stations Ship stations (ship-to-ship) Satellite-to-ship |
| AIS 1 |  | 161.975 | 161.975 |
| AIS 2 |  | 162.025 | 162.025 |
| 75 (long range AIS) |  | 156.775 (ships are Tx only) | N/A |
| 76 (long range AIS) |  | 156.825 (ships are Tx only) | N/A |
| 2027 (ASM 1) |  | 161.950 (2027) | 161.950 (2027) |
| 2028 (ASM 2) |  | 162.000 (2028) | 162.000 (2028) |
| 24/84/25/85 (VDE 1) 24 84 25 85 | 24/84/25/85/26/86 (Ship-satellite, satellite-ship) 24 84 25 85 26 86 | 100/150 kHz channel (24/84/25/85, lower legs (VDE1-A) merged) Ship-to-shore (24/84/25/85/26/86,) Ship-to-satellite | 100/150 kHz channel (24/84/25/85, upper legs (VDE1-B) merged) Ship-to-ship, Shore-to-ship (24/84/25/85/26/86,) Satellite-to-ship |
|  |  | 157.200 (1024) | 161.800 (2024) |
|  |  | 157.225 (1084) | 161.825 (2084) |
|  |  | 157.250 (1025) | 161.850 (2025) |
|  |  | 157.275 (1085) | 161.875 (2085) |
|  |  | 157.300 (1026) | 161.900 (2026) |
|  |  | 157.325 (1086) | 161.925 (2086) |

Figure 2

VDES functions and frequency usage engineer’s perspective

# Identification

Identification and location of all active maritime stations is provided automatically.  All VDES stations should be uniquely identified.   For the purpose of identification, the appropriate numerical identifier, for example maritime mobile service identity (MMSI), could be used, as defined in the latest version of Recommendation ITU-R M.585. Recommendation ITU-R M.1080 should not be applied with respect to the 10th digit (least significant digit). Automatic identification system

AIS is a part of VDES. AIS should have the highest priority in the VDES, and all other functions should be organized such that the AIS is not adversely affected.

## Automatic identification system VHF data link non-controlling stations

### Automatic identification system shipborne station

The AIS part of the shipborne VDES should conform to requirements for Class A shipborne mobile equipment using SOTDMA technology as described in Recommendation ITU-R M.1371, except that the channel switching aspect of channel management should not be used.

## Automatic identification system VHF data link controlling stations

### Automatic identification system shore base station

The AIS part of the VDES shore base station should conform to the requirements for AIS base stations as described in Recommendation ITU-R M.1371, except that the channel switching aspect of channel management should not be used in conjunction with VDES. AIS should have the highest priority of all functions in the VDES shore base station, and all other functions should be organized such that the AIS is not adversely affected.

# Application specific messages

For the VDES, to mitigate AIS VDL loading effects, application specific messages (ASM) should conform to the data structure specified in Recommendation ITU-R M.1371 and may use the two channels designated for ASM in Table 1 (ASM 1 and ASM 2) instead of AIS 1 and AIS 2. Transmission method should be according to Section 7.1.

# Protocol layer overview

The VDES architecture should utilize the OSI layers 1 to 4 (physical layer, link layer, network layer, transport layer) as illustrated in Figure 3.

Figure 3

OSI layers 1-4.

|  | Layer 4: |  | Transport |  | ITU-R M.1371 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Layer 3: |  | Network |  |  |  |
|  | Layer 2: |  | Data Link |  |  |  |
|  | Layer 1: |  | Physical |  |  |  |
|  | Channels |  | VDE | ASM | AIS |  |

## Note: This sections describes the ASM function only, that needs to be coordinated with the AIS the VDE TER and VDE SAT functions.

## Responsibilities of layers for preparing ASM data for transmission

### Transport layer

The transport layer is responsible for converting data into transmission packets of correct size and sequencing of data packets.

### Network layer

The network layer is responsible for the management of priority assignments of messages, distribution of transmission packets between channels, and data link congestion resolution.

### Link layer

The link layer is divided into three sub-layers with the following tasks:

#### Link management entity

Assemble ASM message bits, see Annex 8.

Order ASM message bits into 8-bit bytes for assembly of transmission packet, see § xxx

#### Data link services

Calculate frame check sequence (FCS) for ASM message bits, TBD

Append FCS to ASM message to complete creation of transmission packet contents TBD

Apply bit stuffing process to transmission packet contents, TBD.

Complete assembly of transmission packet, see § xxx.

#### Media access control

Provides a method for granting access to the data transfer to the VHF data link (VDL). The method used is a TDMA scheme using a common time reference.

### Physical layer

Non return to zero inverted (NRZI) encode assembled transmission packet see § xxx.

Convert digital NRZI coded transmission packet to [appropriate modulation scheme signal to modulate transmitter, see § xxx.

# Physical layer

## Parameters

### General

The physical layer is responsible for the transfer of a bit-stream from an originator, out on to the data link. The performance requirements for the physical layer are summarized in Tables xxx to xxx.

For transmit output power see also § xxx.

The low setting and the high setting for each parameter is independent of the other parameters.

TABLE 3

| Symbol | Parameter name | Units | Low setting | High setting |
| --- | --- | --- | --- | --- |
| PH.CHS | Channel spacing (encoded according to RR Appendix 18 with footnotes)(1) | kHz | 25 | 25 |
| PH.ASM1 | ASM 1 (2027)(1) | MHz | 161.950 | 161.950 |
| PH.ASM2 | ASM 2 (2028)(1) | MHz | 162.000 | 162.000 |
| PH.BR | Bit rate | kbit/s | TBD | 28.8 |
| PH.TS | Training sequence | Bits | TBD | TBD |
| PH.TXBT | Transmit BT product |  | TBD | TBD |
| PH.RXBT | Receive BT product |  | TBD | TBD |
| PH.MI | Modulation index |  | TBD | TBD |
| PH.TXP | Transmit output power | W | 1 | 12.5 |
| (1) See Recommendation ITU-R M.1084, Annex 4. |  |  |  |  |

### Constants

TABLE 4

| Symbol | Parameter name | Value |
| --- | --- | --- |
| PH.DE | Data encoding | TBD |
| PH.FEC | Forward error correction | TBD |
| PH.IL | Interleaving | TBD |
| PH.BS | Bit scrambling | TBD |
| PH.MOD | Modulation | TBD |

### Transmission media

Data transmissions are made in the VHF maritime mobile band. Data transmissions should use ASM 1 and ASM 2 channels.

### Multi-channel operation

The ASM should be capable of receiving on two parallel channels and transmitting on two independent channels in accordance with § xxx. Two separate TDMA receiving processes should be used to simultaneously receive on two independent frequency channels. One TDMA transmitter should be used to alternate TDMA transmissions on two independent frequency channels.

## Transceiver characteristics

The transceiver should perform in accordance with the characteristics set forth herein.

TABLE 5

Minimum required time division multiple access transmitter characteristics

| Transmitter parameters | Requirements |
| --- | --- |
| Carrier power error | TBD |
| Carrier frequency error | TBD |
| Slotted modulation mask | ∆fc < ±10 kHz: 0 dBc ±10 kHz < ∆fc < ±25 kHz: below the straight line between −25 dBc at ±10 kHz and –70 dBc at ±25 kHz ±25 kHz < ∆fc < ±62.5 kHz: –70 dBc |
| Transmitter test sequence and modulation accuracy | TBD |
| Transmitter output power versus time | TBD |
| Spurious emissions | TBD |
| Intermodulation attenuation (base station only) | TBD |

TABLE 6

Definitions of timing for Figure xxx

| Reference |  |  | Bits | Time (ms) | Definition |
| --- | --- | --- | --- | --- | --- |
| T0 |  |  | TBD | 0 | Start of transmission slot. Power should NOT exceed –50 dB of Pss before T0 |
| TA |  |  | TBD | 0-0.625 | Power exceeds –50 dB of Pss |
| TB |  | TB1 | TBD | 0.625 | Power should be within +1.5 or –3 dB of Pss |
|  |  | TB2 | TBD | 0.833 | Power should be within +1.5 or –1 dB of Pss (start of training sequence) |
| TE (includes 1 stuffing bit) |  |  | TBD | 24.271 | Power should remain within +1.5 or –1 dB of Pss during the period TB2 to TE |
| TF (includes 1 stuffing bit) |  |  | TBD | 25.104 | Power should be –50 dB of Pss and stay below this |
| TG |  | TBD | 26.667 | Start of next transmission time period |

TABLE 7

Minimum required time division multiple access receiver characteristics(1)

| Receiver parameters | Requirements |
| --- | --- |
| Sensitivity | 20% PER @ –107 dBm |
| Error behaviour at high input levels | 1% PER @ –77 dBm 1% PER @ –7 dBm |
| Adjacent channel selectivity | 20% PER @ 70 dB |
| Co-channel selectivity | 20% PER @ 19 dB |
| Spurious response rejection | 20% PER @ 70 dB |
| Intermodulation response rejection | 20% PER @ 74 dB |
| Spurious emissions | –57 dBm (9 kHz to 1 GHz) –47 dBm (1 GHz to 4 GHz) |
| Blocking | 20% PER @ 86 dB |

## Modulation scheme

The modulation scheme is [PI/4 DQPSK] TBD.

### [PI/4 DQPSK] TBD

2.3.1.1	The encoded data should be [PI/4 DQPSK] TBD coded before modulating the transmitter.

2.3.1.2	The modulator BT-product used for transmission of data should be TBD maximum (highest nominal value).

2.3.1.3	The demodulator used for receiving of data should be designed for a BT-product of maximum TBD (highest nominal value).

### Phase modulation

The coded data should modulate the VHF transmitter. The modulation index should be TBD.

### Frequency stability

The frequency stability of the VHF radio transmitter/receiver should be ± TBD Hz or better.

## Data transmission bit rate

The transmission bit rate should be max 28.8 kbit/s  50 ppm.

## Training sequence

Data transmission should begin with a TBD -bit demodulator training sequence (preamble) consisting of one segment synchronization. This segment should consist of alternating zeros and ones (0101....). This sequence may begin with a 1 or a 0 since TBD encoding is used.

## Data encoding

The TBD waveform is used for data encoding. The waveform is specified as giving a change in the level when a zero (0) is encountered in the bit stream.

## Forward error correction

Forward error correction is TBD

## Interleaving

Interleaving is TBD

## Bit scrambling

Bit scrambling is TBD

## Data link sensing

Data link occupancy and data detection are entirely controlled by the link layer.

## Transmitter transient response

The attack, settling and decay characteristics of the RF transmitter should comply with the mask shown in Fig. x and defined in Table x.

Figure 2

Transmitter output envelope versus time

### Switching time

The channel switching time should be less than 25 ms (see Fig. 8).

The time taken to switch from transmit to receive conditions, and vice versa, should not exceed the transmit attack or release time. It should be possible to receive a message from the slot directly after or before own transmission.

The equipment should not be able to transmit during channel switching operation.

The equipment is not required to transmit on the other ASM channel in the adjacent time slot.

## Transmitter power

The power level is determined by the link management entity (LME) of the link layer.

2.12.1	Provision should be made for two levels of nominal power (high power, low power) as required by some applications. The default operation of the AIS station should be on the high nominal power level. Changes to the power level should only be by assignment by the approved channel management means (see § xxx).

2.12.2	The nominal levels for the two power settings should be 1 W and 12.5 W or 1 W, Tolerance should be within 1.5 dB.

## Shutdown procedure

2.13.1	An automatic transmitter hardware shutdown procedure and indication should be provided in case a transmitter continues to transmit for more than TBD s. This shutdown procedure should be independent of software control.

## Safety precautions

The ASM installation, when operating, should not be damaged by the effects of open circuited or short circuited antenna terminals.

# Link layer

The link layer specifies how data is packaged in order to apply error detection and correction to the data transfer. The link layer is divided into three (3) sub-layers.

## 3.1	Sub-layer 1: medium access control

The medium access control (MAC) sub layer provides a method for granting access to the data transfer medium, i.e. the VHF data link. The method used is a TDMA scheme using a common time reference.

### TDMA synchronization

TDMA synchronization is achieved using an algorithm based on a synchronization state as described below. The sync state flag within ITDMA communication state, indicates the synchronization state of a station (see Figs 3 and 4).

The TDMA receiving process should not be synchronized to slot boundaries.

Synchronization other than UTC direct could be done using received AIS messages.

TBD

- use AIS for indirect synchronization

- if no synchronization is available the unit stops transmitting.

#### Coordinated universal time direct

A station, which has direct access to coordinated universal time (UTC) timing with the required accuracy should indicate this by setting its synchronization state to UTC direct.

#### Coordinated universal time indirect

A station, which is unable to get direct access to UTC, but can receive other stations that indicate UTC direct, should synchronize to those stations. It should then change its synchronization state to UTC indirect. Only one level of UTC indirect synchronization is allowed.

#### Synchronized to base station (direct or indirect)

Mobile stations, which are unable to attain direct or indirect UTC synchronization, but are able to receive transmissions from base stations, should synchronize to the base station which indicates the highest number of received stations, provided that two reports have been received from that station in the last 40 s. Once base station synchronization has been established, this synchronization shall be discontinued if fewer than two reports are received from the selected base station in the last 40 s. When the parameter slot time-out of the SOTDMA communication state has one of the values three (3), five (5), or seven (7), the number of received stations should be contained within the SOTDMA communication state-submessage. The station which is thus synchronized to a base station should then change its synchronization state to “base station” to reflect this. A station that has Sync. state = 3 (see § 3.1.3.4.3) shall synchronize to a station that has Sync. state = 2 (see § 3.1.3.4.3) if no base station or station with UTC direct is available. Only one level of indirect access to the base station is allowed.

When a station is receiving several other base stations which indicate the same number of received stations, synchronization should be based on the station with the lowest MMSI.

#### Number of received stations

A station, which is unable to attain UTC direct or UTC indirect synchronization and is also unable to receive transmissions from a base station, should synchronize to the station indicating the highest number of other stations received during the last nine frames, provided that two reports have been received from that station in the last 40 s. This station should then change its synchronization state to “Number of received stations” (see § 3.3.7.2.2 for SOTDMA communication state and § 3.3.7.3.2 for ITDMA communication state). When a station is receiving several other stations, which indicate the same number of received stations, synchronization should be based on the station with the lowest MMSI. That station becomes the semaphore on which synchronization should be performed.

### Time division

The system uses the concept of a frame. A frame equals one (1) min and is divided into 2 250 slots (TBD: half slots?). Access to the data link is, by default, given at the start of a slot. The frame start and stop coincide with the UTC minute, when UTC is available. When UTC is unavailable the procedure, described below should apply.

### Slot phase and frame synchronization

### Slot phase synchronization and frame synchronization is done by using information from remote AIS transmissions as described in ITU-R M.1371.

### TBD

#### 3.1.3.1	Slot phase synchronization

Slot phase synchronization is the method whereby one station uses the messages from other AIS stations or AIS base stations to re-synchronize itself, thereby maintaining a high level of synchronization stability, and ensuring no message boundary overlapping or corruption of messages.

#### 3.1.3.2	Frame synchronization

Frame synchronization is the method whereby one station uses the current slot number of another AIS station or AIS base station, adopting the received slot number as its own current slot number.

##### Coordinated universal time available

A station, which has direct access to UTC, should continuously re-synchronize its transmissions based on UTC source. A station, which has indirect access to UTC should continuously resynchronize its transmissions based on those UTC sources (see § xxx).

##### Coordinated universal time not available

When the station determines that its own internal slot number is equal to the semaphore slot number, it is already in frame synchronization and it should continuously slot phase synchronize.

##### Synchronization sources

The primary source for synchronization should be the internal UTC source (UTC direct). If this source should be unavailable the synchronization should be derived from remote AIS transmissions (refer to ITU-R M.1371)

### Slot identification

Each slot is identified by its index (0-2249 or TBD). Slot zero (0) should be defined as the start of the frame.

### Slot access

The transmitter should begin transmission by turning on the RF power at slot start.

The transmitter should be turned off after the last bit of the transmission packet has left the transmitting unit. This event must occur within the slots allocated for own transmission. The default length of a transmission occupies one (1) slot. The slot access is performed as shown in Fig. 5:

Figure 5

### 3.1.6	Slot state

Each slot can be in one of the following states:

–	Free: meaning that the slot is unused within the receiving range of the own station. Externally allocated slots that have not been used during the preceding three frames are also Free slots. This slot may be considered as a candidate slot for use by own station (see § xxx).

–	Internal allocation: meaning that the slot is allocated by own station and can be used for transmission.

–	External allocation: meaning that the slot is allocated for transmission by another station using the functions AIS, ASM or VDE.

–	Available: meaning that the slot is externally allocated by a station and is a possible candidate for slot reuse (see § xxx).

–	Unavailable: meaning that the slot is externally allocated by a station and cannot be a candidate for slot reuse (see § xxx).

## Sub layer 2: data link service

The data link service (DLS) sub layer provides methods for:

–	data link activation and release;

–	data transfer; or

–	error detection and control.

### Data link activation and release

Based on the MAC sub layer the DLS will listen, activate or release the data link. Activation and release should be in accordance with § xxx. A slot, marked as free or externally allocated, indicates that own equipment should be in receive mode and listen for other data link users. This should also be the case with slots, marked as available and not to be used by own station for transmission (see § XXX)

### Data transfer

Data transfer should use a bit-oriented protocol which is based on the high-level data link control (HDLC) as specified by ISO/IEC 13239:2002 – Definition of packet structure. Information packets (I-Packets) should be used with the exception that the control field is omitted (see Fig. 6).

#### Bit stuffing

The bit stream of the data portion and the FCS, see Fig. 6, § 3.2.2.5 and § 3.2.2.6, should be subject to bit stuffing. On the transmitting side, this means that if five (5) consecutive ones (1’s) are found in the output bit stream, a zero should be inserted after the five (5) consecutive ones (1’s). This applies to all bits between the HDLC flags (start flag and end flag, see Fig. 6). On the receiving side, the first zero after five (5) consecutive ones (1’s) should be removed.

#### Packet format

Data is transferred using a transmission packet as shown in Fig. 6:

Figure 6

The packet should be sent from left to right. This structure is identical to the general HDLC structure, except for the training sequence. The training sequence should be used in order to synchronize the VHF receiver and is discussed in § 3.2.2.3. The total length of the default packet is 256 bits. This is equivalent to one (1) slot.

#### Training sequence

The training sequence should be a bit pattern consisting of alternating 0’s and 1’s (010101010...). Twenty-four bits of preamble are transmitted prior to sending the flag. This bit pattern is modified due to the NRZI mode used by the communication circuit (see Fig. 7).

Figure 7

The preamble should not be subject to bit stuffing.

#### Start flag

The start flag should be 8 bits long and consists of a standard HDLC flag. It is used in order to detect the start of a transmission packet. The start flag consists of a bit pattern, 8 bits long: 01111110 (7Eh). The flag should not be subject to bit stuffing, although it consists of 6 bits of consecutive ones (1’s).

#### Data

The data portion is 168 bits long in the default transmission packet. The content of data is undefined at the DLS. Transmission of data, which occupy more than 168 bits, is described in § 3.2.2.11.

#### Frame check sequence

The FCS uses the cyclic redundancy check (CRC) 16-bit polynomial to calculate the checksum as defined in ISO/IEC 13239:2002. The CRC bits should be pre-set to one (1) at the beginning of a CRC calculation. Only the data portion should be included in the CRC calculation (see Fig. 7).

#### End flag

The end flag is identical to the start flag as described in § 3.2.2.4.

#### Buffer

The buffer is normally 24 bits long and should be used as follows:

–	bit stuffing:	4 bits (normally, for all messages except safety related messages and binary messages)

–	distance delay:	14 bits

–	synchronization jitter:	6 bits

##### Bit stuffing

A statistical analysis of all possible bit combinations in the data field of the fixed length messages shows that 76% of combinations use 3 bits or less, for bit stuffing. Adding the logically possible bit combinations shows, that 4 bits are sufficient for these messages. Where variable length messages are used, additional bit stuffing could be required. For the case where additional bit stuffing is required, see § 5.2 and Table 21.

##### Distance delay

A buffer value of 14 bits is reserved for distance delay. This is equivalent to 235.9 nautical miles (NM). This distance delay provides protection for a propagation range of over 120 NM.

##### Synchronization jitter

For synchronization jitter refer to Recommendation ITU-R M.1371 Annex 2 (Class A)

#### 3.2.2.9	Summary of the default transmission packet

The data packet is summarized as shown in Table 12:

TABLE 12

| Ramp up | 8 bits | T0 to TTS in Fig. 8 |
| --- | --- | --- |
| Training sequence | 24 bits | Necessary for synchronization |
| Start flag | 8 bits | In accordance with HDLC (7Eh) |
| Data | 168 bits | Default |
| CRC | 16 bits | In accordance with HDLC |
| End flag | 8 bits | In accordance with HDLC (7Eh) |
| Buffer | 24 bits | Bit stuffing distance delays, repeater delay and jitter |
| Total | 256 bits |  |

#### Transmission timing

Figure 8 shows the timing events of the default transmission packet (one slot). At the situation where the ramp down of the RF power overshoots into the next slot, there should be no modulation of the RF after the termination of transmission. This prevents undesired interference, due to false locking of receiver modems, with the succeeding transmission in the next slot.

#### Long transmission packets

A station may occupy at maximum five consecutive slots for one (1) continuous transmission. Only a single application of the overhead (ramp up, training sequence, flags, FCS, buffer) is required for a long transmission packet. The length of a long transmission packet should not be longer than necessary to transfer the data; i.e. the AIS should not add filler.

### Error detection and control

Error detection and control should be handled using the CRC polynomial as described in § 3.2.2.6. CRC errors should result in no further action by the AIS.

### Forward Error correction

Forward Error correction should be applied. TBD

## Sub layer 3 – link management entity

The LME controls the operation of the DLS, MAC and the physical layer.

### Access to the data link

There should be different access schemes for controlling access to the data transfer medium. The application and mode of operation determine the access scheme to be used. The access schemes are ITDMA, RATDMA, CSTDMA and FATDMA.

#### Cooperation on the data link

The access schemes operate continuously, and in parallel, on the same physical data link. They all conform to the rules set up by the TDMA (see § 3.1).

#### Candidate slots

Slots, used for transmission, are selected from candidate slots in the selection interval (SI) (see Fig. 10). The selection process uses received data. There should always be at minimum four candidate slots to choose from unless the number of candidate slots is otherwise restricted due to loss of position information (see § 4.4.1). For Class A mobile AIS stations when selecting candidates for messages longer than one (1) slot (see § 3.2.2.11) a candidate slot should be the first slot in a consecutive block of free or available slots. For Class B “SO” mobile AIS stations the candidate slots for Messages 6, 8, 12 and 14 should be free. When no candidate slot is available, the use of the current slot is allowed. The candidate slots are primarily selected from free slots (see § 3.1.6). When required, available slots are included in the candidate slot set. When selecting a slot from the candidates, any candidate has the same probability of being chosen, regardless of its slot state (see § 3.1.6). If the station cannot find any candidate slots at all, because all slots in the SI are restricted from slot reuse (see § 4.4.1), the station should not reserve a slot in the SI until there is at least one candidate slot.

Example:

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E | E | F | F | F | F | F | E |

A three-slot-message is to be sent. Only slot Nos. 2, 3 and 4 should be considered candidates.

figure 8

Transmission timing

When selecting among candidate slots for transmission in one channel, the slot usage of other channels should be considered. If the candidate slot in the other channel is used by another station, the use of the slot should follow the same rules as for slot reuse (see § 4.4.1). If a slot in either channel is occupied by or allocated by other base station or mobile station, that slot should be reused only in accordance with § 4.4.1.

The slots of another station, whose navigational status is not set to “at anchor” or “moored” and has not been received for 3 min, should be used as candidate slots for intentional slot reuse.

The own station is unable to transmit on an adjacent slot on the two parallel channels because of the necessary switching time (see § 2.11.1). Thus, the two adjacent slots on either side of a slot that is being used by the own station on one channel should not be considered as candidate slots on the other channel.

The purpose of intentionally reusing slots and maintaining a minimum of four candidate slots within the same probability of being used for transmission is to provide high probability of access to the link. To further provide high probability of access, time-out characteristics are applied to the use of the slots so that slots will continuously become available for new use.

Figure 9 illustrates the process of selecting among candidate slots for transmission on the link.

figure 9

### Modes of operation

There should be three modes of operation. The default mode should be autonomous and may be switched to/from other modes. For a simplex repeater there should only be two modes of operation: autonomous and assigned, but no polled mode.

#### Autonomous and continuous

A station operating autonomously should determine its own schedule for transmission. The station should automatically resolve scheduling conflicts with other stations.

#### Assigned

A station operating in the assigned mode takes into account the transmission schedule of the assigning message when determining when it should transmit.

#### Polled

A station operating in polled mode should automatically respond to interrogation messages. Operation in the polled mode should not conflict with operation in the other two modes. The response should be transmitted on the channel where the interrogation message was received.

### Initialization

At power on, a station should monitor the TDMA channels for one (1) min to determine channel activity, other participating member IDs, current slot assignments and reported positions of other users, and possible existence of shore stations. During this time period, a dynamic directory of all stations operating in the system should be established. A frame map should be constructed, which reflects TDMA channel activity. After one (1) min has elapsed, the station should enter the operational mode and start to transmit according to its own schedule.

### Channel access schemes

The access schemes, as defined below, should coexist and operate simultaneously on the TDMA channel.

#### Incremental time division multiple access

The ITDMA access scheme allows a station to pre-announce transmission slots , with one exception. This allows a station to pre-announce its allocations for autonomous and continuous operation.

ITDMA should be used for periodic data transmissions and for “multi part transmissions”.

##### Incremental time division multiple access algorithm

A station can begin its ITDMA transmission by either substituting a previously allocated slot or, by allocating a new, unannounced slot, using RATDMA or CSTDMA.

Prior to transmitting in the first ITDMA slot, the station randomly selects the next following ITDMA slot and calculates the relative offset to that location. This offset should be inserted into the ITDMA communication state. Receiving stations will be able to mark the slot, indicated by this offset, as externally allocated (see § 3.3.7.3.2 and § 3.1.5). The communication state is transmitted as a part of the ITDMA transmission. During network entry, the station also indicates that the ITDMA slots should be reserved for one additional frame. The process of allocating slots continues as long as required. In the last ITDMA slot, the relative offset is set to zero.

##### Incremental time division multiple access parameters

The parameters of Table 13 control ITDMA scheduling:

TABLE 13

| Symbol | Name | Description | Minimum | Maximum |
| --- | --- | --- | --- | --- |
| LME.ITINC | Slot increment | The slot increment is used to allocate a slot ahead in the frame. It is a relative offset from the current transmission slot. If it is set to zero, no more ITDMA allocations should be done | 0 | 8 191 |
| LME.ITSL | Number of slots | Indicates the number of consecutive slots, which are allocated, starting at the slot increment | 1 | 5 |
| LME.ITKP | Keep flag | This flag should be set to TRUE when the present slot(s) should be reserved in the next frame also. The keep flag is set to FALSE when the allocated slot should be freed immediately after transmission | False = 0 | True = 1 |

#### Random access time division multiple access

RATDMA is used when a station needs to allocate a slot, which has not been pre-announced. This is generally done for the first transmission slot during data link network entry, or for messages of a non-repeatable character.

##### Random access time division multiple access algorithm

The RATDMA access scheme should use a probability persistent (p-persistent) algorithm as described in this paragraph (see Table 14).

An AIS station should avoid using RATDMA. A scheduled message should primarily be used to announce a future transmission to avoid RATDMA transmissions.

Messages, which use the RATDMA access scheme, are stored in a priority first-in first-out (FIFO). When a candidate slot (see § 3.3.1.2) is detected, the station randomly select a probability value (LME.RTP1) between 0 and 100. This value should be compared with the current probability for transmission (LME.RTP2). If LME.RTP1 is equal to, or less than LME.RTP2, transmission should occur in the candidate slot. If not, LME.RTP2 should be incremented with a probability increment (LME.RTPI) and the station should wait for the next candidate slot in the frame.

The SI for RATDMA should be 150 time slots, which is equivalent to 4 s. The candidate slot set should be chosen within the SI, so that the transmission occurs within 4 s.

Each time that a candidate slot is entered, the p-persistent algorithm is applied. If the algorithm determines that a transmission shall be inhibited, then the parameter LME.RTCSC is decremented by one and LME.RTA is incremented by one.

LME.RTCSC can also be decremented as a result of another station allocating a slot in the candidate set. If LME.RTCSC  LME.RTA  4 then the candidate set shall be complemented with a new slot within the range of the current slot and LME.RTES following the slot selection criteria.

##### Random access time division multiple access parameters

The parameters of Table 14 control RATDMA scheduling:

TABLE 14

| Symbol | Name | Description | Minimum | Maximum |
| --- | --- | --- | --- | --- |
| LME.RTCSC | Candidate slot counter | The number of slots currently available in the candidate set. NOTE 1 – The initial value is always 4 or more (see § 3.3.1.2). However, during the cycle of the p-persistent algorithm the value may be reduced below 4 | 1 | 150 |
| LME.RTES | End slot | Defined as the slot number of the last slot in the initial SI, which is 150 slots ahead | 0 | 2 249 |
| LME.RTPRI | Priority | The priority that the transmission has when queuing messages. The priority is highest when LME.RTPRI is lowest. Safety related messages should have highest service priority (refer to § 4.2.3) | 1 | 0 |
| LME.RTPS | Start probability | Each time a new message is due for transmission, LME.RTP2 should be set equal to LME.RTPS. LME.RTPS shall be equal to 100/LME.RTCSC. NOTE 2 – LME.RTCSC is set to 4 or more initially. Therefore LME.RTPS has a maximum value of 25 (100/4) | 0 | 25 |
| LME.RTP1 | Derived probability | Calculated probability for transmission in the next candidate slot. It should be less than or equal to LME.RTP2 for transmission to occur, and it should be randomly selected for each transmission attempt | 0 | 100 |
| LME.RTP2 | Current probability | The current probability that a transmission will occur in the next candidate slot | LME.RTPS | 100 |
| LME.RTA | Number of attempts | Initial value set to 0. This value is incremented by one each time the p-persistent algorithm determines that a transmission shall not occur | 0 | 149 |
| LME.RTPI | Probability increment | Each time the algorithm determines that transmission should not occur, LME.RTP2 should be incremented with LME.RTPI. LME.RTPI shall be equal to (100 − LME.RTP2)/LME.RTCSC | 1 | 25 |

#### Fixed access time division multiple access

FATDMA should be used by base stations only. FATDMA allocated slots should be used for repetitive messages. For base stations use of FATDMA refer to § 4.5 and § 4.6.

##### Fixed access time division multiple access algorithm

Access to the data link should be achieved with reference to frame start. Each allocation should be pre-configured by the competent authority and not changed for the duration of the operation of the station, or until re-configured. Except where the time-out value is otherwise determined, receivers of data link management message (Message 20) should set a slot time-out value in order to determine when the FATDMA slot will become free. The slot time-out should be reset with each reception of the message.

FATDMA reservations should consist of a base station report (message 4) in conjunction with a data link management message with the same base station ID (MMSI). FATDMA reservations apply within a range of 120 nautical miles from the reserving base station. AIS stations (except when using FATDMA) should not use FATDMA reserved slots within this range. A data link management message (Message 20) without a base station report (Message 4) should be ignored. Base stations may reuse FATDMA reserved slots within this range for their own FATDMA transmissions but may not reuse FATDMA reserved slots for RATDMA transmissions.

FATDMA reservations do not apply beyond 120 nautical miles from the reserving base station. All stations may consider these slots as available.

##### Fixed access time division multiple access parameters

The parameters of Table 15 control FATDMA scheduling:

TABLE 15

| Symbol | Name | Description | Minimum | Maximum |
| --- | --- | --- | --- | --- |
| LME.FTST | Start slot | The first slot (referenced to frame start) to be used by the station | 0 | 2 249 |
| LME.FTI | Increment | Increment to next block of allocated slots. An increment of zero indicates that the station transmits one time per frame, in the start slot | 0 | 1 125 |
| LME.FTBS | Block size | Default block size. Determines the default number of consecutive slots which are to be reserved at each increment | 1 | 5 |

#### Initialization phase (after switched on)

The initialization phase is described using the flowchart shown in Fig. 11.

figure 11

Ready for transmission

#### 4.3.1.2	Carrier sense detection method

Within a time window of 1 146 µs starting at 833 µs and ending at 1 979 µs after the start of the time period intended for transmission (T0) the AIS Class B “CS” should detect if that time period is used (CS detection window).

NOTE 1 – Signals within the first 8 bits (833 µs) of the time period are excluded from the decision (to allow for propagation delays and ramp down periods of other units).

The Class B “CS” AIS should not transmit on any time period in which, during the CS detection window, a signal level greater than the “CS detection threshold” (§ 4.3.1.3) is detected.

The transmission of a CSTDMA packet should commence 20 bits (TA = 2 083 µs + T0) after the nominal start of the time period (see Fig. 35).

Figure 35

Carrier sense timing

#### 4.3.1.3	Carrier sense detection threshold

The carrier sense (CS) detection threshold should be determined over a rolling 60 s interval on each Rx channel separately. The threshold should be determined by measuring the minimum energy level (representing the background noise) plus an offset of 10 dB. The minimum CS detection threshold should be –107 dBm and background noise should be tracked for a range of at least 30 dB (which results in a maximum threshold level of –7 dBm).

#### 4.3.1.4	VHF data link access

The transmitter should begin transmission by turning on the RF power immediately after the duration of the carrier sense window (TA).

The transmitter should be turned off after the last bit of the transmission packet has left the transmitting unit (nominal transmission end TE assuming no bit stuffing).

The access to the medium is performed as shown in Fig. 36 and Table 37:

Figure 36

Power versus time mask

TABLE 37

Definition of timings for Figure 36

| Reference |  | Bits | Time (ms) | Definition |
| --- | --- | --- | --- | --- |
| T0 to TA |  | 0 | 0 | Start of candidate transmission time period Power should not exceed –50 dB of Pss |
| TA to TB |  | 20 | 2 083 | Begin of upramping |
| TB | TB1 | 23 | 2 396 | Power should reach within +1.5 or –3 dB of Pss |
|  | TB2 | 25 | 2 604 | Power should reach within +1.5 or –1 dB of Pss |
| TE (plus 1 stuffing bit) |  | 248 | 25 833 | Power should still remain within +1.5 or –1 dB of Pss |
| TF (plus 1 stuffing bit) |  | 251 | 26 146 | Power should reach –50 dB of steady state RF output power (Pss) and stay below this |

There should be no modulation of the RF after the termination of transmission (TE) until the power has reached zero and next time period begins (TG).

#### 4.3.1.5	VHF data link state

The VDL state is based on the result of the carrier sense detection (see § 4.3.1.2) for a time period. A VDL time period can be in one of the following states:

–	FREE: time period is available and has not been identified as used in reference to § 4.3.1.2.

–	USED: VDL has been identified as used in reference to § 4.3.1.2.

–	UNAVAILABLE: time periods should be indicated as “UNAVAILABLE” if they are reserved by base stations using Message 20 regardless of their range.

Time periods indicated as “UNAVAILABLE” should not be considered as a candidate time period for use by own station and may be used again after a time-out. The time-out should be 3 min if not specified or as specified in Message 20.

##### Monitor VHF data link

At power on, a station should monitor the TDMA channel for one (1) min interval to determine channel activity, other participating member IDs, current slot assignments and reported positions of other users, and possible existence of base stations. During this time period, a dynamic directory of all members operating in the system should be established. A frame map should be constructed, which reflects TDMA channel activity.

#### Network access and entry of a new data stream

- Initial network entry using RATDMA, CSTDMA

- Entry of new data stream using previously allocations by ITDMA

- For periodic transmissions using ITDMA schedule

- Single message transmission using RATDMA, CSTDMA or scheduled ITDMA

- Dynamic allocation of additional slots using existing ITDMA

- Priority of transmissions

- Limits how many slots

### Assigned operation

A controlling station may

- Assign slots to a mobile station

- Exclude slots for autonomous use by other stations

- Limit the amount of slots for a specific station or in area

- The controlling station needs to support authentication

##### Entering assigned mode

##### Operating in the assigned mode

##### Returning to autonomous and continuous mode

### Message structure

Messages, which are part of the access schemes, should have the following structure shown in Fig. 16 inside the data portion of a data packet: TBD

Figure 16

Each message is described using a table with parameter fields listed from top to bottom. Each parameter field is defined with the most significant bit first.

Parameter fields containing sub-fields (e.g. communication state) are defined in separate tables with sub-fields listed top to bottom, with the most significant bit first within each sub-field.

Character strings are presented left to right most significant bit first. All unused characters should be represented by the @ symbol, and they should be placed at the end of the string.

When data is output on the VHF data link it should be grouped in bytes of 8 bits from top to bottom of the table associated with each message in accordance with ISO/IEC 13239:2002. Each byte should be output with least significant bit first. During the output process, data should be subject to bit-stuffing (see § 3.2.2) and NRZI coding (see § 2.6).

Unused bits in the last byte should be set to zero in order to preserve byte boundary.

Generic example for a message table:

TABLE 17

| Parameter | Symbol | Number of bits | Description |
| --- | --- | --- | --- |
| P1 | T | 6 | Parameter 1 |
| P2 | D | 1 | Parameter 2 |
| P3 | I | 1 | Parameter 3 |
| P4 | M | 27 | Parameter 4 |
| P5 | N | 2 | Parameter 5 |
| Unused | 0 | 3 | Unused bits |

Logical view of data as described in § 3.3.7:

| Bit order | M----L-- | M------- | -------- | -------- | --LML000 |
| --- | --- | --- | --- | --- | --- |
| Symbol | TTTTTTDI | MMMMMMMM | MMMMMMMM | MMMMMMMM | MMMNN000 |
| Byte order | 1 | 2 | 3 | 4 | 5 |

Output order to VHF data link (bit-stuffing is disregarded in the example):

| Bit order | --L----M | -------M | -------- | -------- | 000LML-- |
| --- | --- | --- | --- | --- | --- |
| Symbol | IDTTTTTT | MMMMMMMM | MMMMMMMM | MMMMMMMM | 000NNMMM |
| Byte order | 1 | 2 | 3 | 4 | 5 |

#### Message identification

The message ID should be 6 bits long and should range between 0 and 63. The message ID should identify the message type.

#### Incremental time division multiple access message structure

The ITDMA message structure supplies the necessary information in order to operate in accordance with § 3.3.4.1. The message structure is shown in Fig. 18:

figure 18

##### User identification

The user ID should be the MMSI (see § 3, Annex 1). The MMSI is 30 bits long. The first 9 digits (most significant digits) should be used only. The tenth digit as stated in Recommendation ITU-R M.1080 may be used.

- Unique identifier, e.g. MAC address

##### Incremental time division multiple access communication state

The communication state provides the following functions:

–	it contains information used by the slot allocation algorithm in the ITDMA concept;

–	it also indicates the synchronization state.

The ITDMA communication state is structured as shown in Table 20:

TABLE 20

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Sync state | 2 | 0 UTC direct (see § 3.1.1.1) 1 UTC indirect (see § 3.1.1.2) 2 Station is synchronized to a base station (base direct) (see § 3.1.1.3) 3 Station is synchronized to another station based on the highest number of received stations or to another mobile station, which is directly synchronized to a base station (see § 3.1.1.3 and § 3.1.1.4) |
| Slot increment | 13 | Offset to next slot to be used, or zero (0) if no more transmissions |
| Number of slots | 3 | Number of consecutive slots to allocate. 0  1 slot, 1  2 slots, 2  3 slots, 3  4 slots, 4  5 slots, 5 = 1 slot; offset = slot increment + 8 192, 6 = 2 slots; offset = slot increment + 8 192, 7 = 3 slots; offset = slot increment + 8 192. Use of 5 to 7 removes the need for RATDMA broadcast for scheduled transmissions up to 6 min intervals |
| Keep flag | 1 | Set to TRUE  1 if the slot remains allocated for one additional frame (see Table 13) |

The ITDMA communication state should apply only to the slot in the channel where the relevant transmission occurs. ASM 1 and ASM 2 are independent channels

#### 3.3.7.4	Random access time division multiple access message structure

The RATDMA access scheme may use message structures determined by message ID and may thus lack a uniform structure.

A message with a communication state may be transmitted using RATDMA in the following situations:

–	When initially entering the network (refer to § 3.3.4.1.1).

–	When repeating a message. (?)

3.3.7.4.1	The communication state when initially entering the network should be set in accordance with § 3.3.4.1.1 and § 3.3.7.3.2.

3.3.7.4.2	The communication state when repeating a message should be set in accordance with § 4.6.3.

#### 3.3.7.5	Fixed access time division multiple access message structure

The FATDMA access scheme may use message structures determined by message ID and may thus lack a uniform structure.

A message with a communication state may be transmitted using FATDMA, e.g. when repeated. In this situation, the communication state should be set in accordance with § 4.6.3 (see also § 3.16, Annex 8).

# 4	Network layer

The network layer should be used for:

–	establishing and maintaining channel connections;

–	management of priority assignments of messages;

–	distribution of transmission packets between channels;

–	data link congestion resolution.

## 4.1	Multi-channel operation and channel management

In order to satisfy the requirements for multi-channel operation (see § 2.1.4), the following should apply.

### 4.1.1	Operating frequency channels

Two frequencies have been designated in RR Appendix 18 for AASM use worldwide, on the high seas and in all other areas. The two designated frequencies are:

–	ASM 1 (Channel 2027, 161.950 MHz);

–	ASM 2 (Channel 2028, 162.000 MHz);

### 4.1.2	Normal default mode of multi-channel operation

The normal default mode of operation should be two-channel receiving and two-channel transmitting, for shipborne mobile stations, where the ASM simultaneously receives on ASM 1 and ASM 2 in parallel.

Channel access is performed independently on each of the two parallel channels.

Transmissions of own station following slot allocation announcements of own station, responses of own station to interrogations, responses of own station to requests, and acknowledgements of own station should be transmitted on the same channel as the initial message received.

For addressed messages, transmissions should utilize the channel in which messages from the addressed station were last received.

For non-periodic messages other than those referenced above, the transmissions of messages, regardless of message type, should alternate between ASM 1 and ASM 2, unless the channel is specified.

### 4.1.4	Regional operating areas

Regional operating areas should be designated by a Mercator projection rectangle with two reference points (WGS-84). The first reference point should be the geographical coordinate address of the north-eastern corner (to the nearest tenth of a minute) and the second reference point should be the geographical coordinate address of the south-western corner (to the nearest tenth of a minute) of the rectangle.

When a station is subject to the regional boundaries, it should set its operating behavior as assigned. When a station is not subject to the regional boundaries, the station should utilize the default settings.

If regional operating areas are used, the areas should be defined in such a way that these areas will be covered completely by transmissions of assignments from at least one base station.

### 4.1.5	Transitional mode operations near regional boundaries

The AIS device should automatically switch to the two-channel transitional operating mode when it is located within five nautical miles, or the transitional zone size (see Table 75, Annex 8), of a regional boundary. In this mode the AIS device should transmit and receive on the primary AIS frequency specified for the occupied region; it should also transmit and receive on the primary AIS frequency of the nearest adjacent region. Only one transmitter is required. Additionally, for multi-channel operations as specified in § 4.1.2, except when the reporting interval has been assigned by Message 16, when operating in this mode, the reporting interval should be doubled and shared between the two channels (alternate transmission mode). When the AIS is entering the transitional mode, it should continue to utilize the current channels for transmitting for a full one-minute frame while switching one of the receivers to the new channel. The TDMA access rules should be applied to vacating slots on the current channel and accessing slots on the new channel. This transitional behaviour is necessary only when the channels are changing.

Regional boundaries should be established by the competent authority in such a way that this two-channel transitional operating mode can be implemented as simply and safely as possible. For example, care should be taken to avoid having more than three adjacent regions at any regional boundary intersection. In this context the high seas area should be considered to be a region where default operating settings apply. The mobile AIS station should ignore any channel management command, when there are three different regional operating settings with adjacent regional operating areas, their corners within eight nautical miles to each other.

Regions should be as large as possible. For practical purposes, in order to provide safe transitions between regions, these should be no smaller than 20 NM but not larger than 200 NM on any boundary side. Examples of acceptable and unacceptable regional boundary definitions are illustrated in Figs 19 and 20.

| figure 19 | figure 20 |
| --- | --- |

### 4.1.7	Resumption of operation after power on

After power on, a mobile station should resume operation using the default settings, unless the own position is within any of the stored regions.

In this case, the mobile station should operate using the stored operating settings of that identified region.

TBD: the ASM unit must have own position to execute area assignments.

### 4.1.8	Priority of assignment commands and clearing of stored regional operating settings

The most current and applicable commands received should override previous assignment commands in accordance with the following rules:

The mobile ASM station should constantly check, if the nearest boundary of the regional operating area of any stored regional operating setting is more than 500 nautical miles away from the current position of own station, or if any stored regional operating setting was older than 24 hours. Any stored regional operating setting which fulfils any one of these conditions should be erased from the memory.

The regional operating settings set should be handled as a whole, i.e. a change requested for any parameter of the regional operating settings should be interpreted as a new regional operating setting.

The mobile ASM station should not accept, i.e. ignore, any new regional operating setting which includes a regional operating area, which does not conform to the rules for regional operating areas laid out in § 4.1.5.

If the regional operating area of the new, accepted regional operating setting overlaps in part or in total or matches the regional operating areas of one or more older regional operating settings, this or these older regional operating settings should be erased from the memory. The regional operating area of the new, accepted regional operating setting may be neighbouring tightly and may thus have the same boundaries as older regional operating settings. This should not lead to the erasure of the older regional operating settings.

Subsequently, the mobile AIS station should store a new, accepted regional operating setting in one free memory location of the eight memories for regional operating settings. If there is no free memory location, the most distant regional operating setting should be replaced by the new, accepted one. If the AIS station does not have position it should delete the area most distant from the position provided in the channel management command.

No means other than defined herein should be allowed to clear any or all of the stored regional operating settings. In particular, it should not be possible to solely clear any or all of the stored regional operating settings by a manual input or by an input via the Presentation Interface without inputting a new regional operating setting.

## 4.2	Distribution of transmission packets

### 4.2.1	The user directory

The user directory is internal to the AIS, and it is used to facilitate slot selection and synchronization. It is also used to select the proper channel for the transmission of an addressed message.

### 4.2.2	Routing of transmission packets

The following tasks are fulfilled with regard to packet routing:

––	A priority is assigned to messages if message queuing is necessary.

### 4.2.3	Management of priority assignments for messages

There are 4 (four) levels of message priority, namely:

Priority 1 (highest priority): Critical link management messages including position report messages in order to ensure the viability of the link.

Priority 2 (highest service priority): Safety related messages. These messages should be transmitted with a minimum of delay.

Priority 3: Assignment, interrogation and responses to interrogation messages.

Priority 4 (lowest priority): All other messages.

For details, refer to Table 46, Annex 8.

The above priorities are assigned to the relevant type of messages, thereby providing a mechanism for sequencing specific messages in order of priority. The messages are serviced in order of priority. This applies to both messages received and messages to be transmitted. Messages with the same priority are dealt with in an FIFO order.

## 4.4	Data link congestion resolution

When the data link is loaded to such a level that the transmission of safety information is jeopardized, one of the following methods should be used to resolve the congestion.

### 4.4.1	Intentional slot reuse by the own station

A station should reuse time slots only in accordance with this paragraph and only when own position is available.

When selecting new slots for transmission, the station should select from its candidate slot set (see § 3.3.1.2) within the desired SI. When the candidate slot set has less than 4 slots, the station should intentionally reuse available slots, in order to make the candidate slot set equal to 4 slots. Slots may not be intentionally reused from stations that indicate no position available. This may result in fewer than 4 candidate slots. The intentionally reused slots should be taken from the most distant station(s) within the SI. Slots allocated or used by base stations should not be used unless the base station is located over 120 NM from the own station. When a distant station has been subject to intentional slot reuse, that station should be excluded from further intentional slot reuse during a time period equal to one frame.

Slot reuse provides candidate slots for random selection. This process attempts to increase the candidate slot set to a maximum of four. When the candidate slot set has reached four, the candidate slot selection process is complete. If four slots have not been identified after all the rules have been applied, this process may report less than four slots. Candidate slots for reuse should be selected using the following priorities beginning with Rule 1 (also see the Slot selection rules flow diagram – Fig. 22).

Add to the Free slot set (if any) all slots that are:

Rule 1:	Free (see § 3.1.6) on selection channel and Available(1) (see § 3.1.6) on the other channel.

Rule 2:	Available(1) on selection channel and Free on the other channel.

Rule 3:	Available(1) on both channels.

Rule 4:	Free on selection channel and Unavailable(2) on the other channel.

Rule 5:	Available(1) on selection channel and Unavailable(2) on the other channel.

(1)	Available – Mobile Station (SOTDMA or ITDMA), or Base Station reserved slot (FATDMA or Message 4) beyond 120 NM.

(2) 	Unavailable – Base Station reserved slot (FATDMA or Message 4) within 120 NM, or a Mobile Station reporting without position information.

Figure 21 is an example applying these rules.

figure 21

It is intended to reuse one slot within the SI of frequency channel A. The current status of the use of the slots within the SI on both frequency channels A and B is given as follows:

F:	Free

I:	Internally allocated (allocated by own station, not in use)

E:	Externally allocated (allocated by another station near own station)

B:	Allocated by a base station within 120 NM of own station

T:	Another station under way that has not been received for 3 min or more

D:	Allocated by the most distant station(s)

X:	Should not be used.

The slot for intentional slot reuse should then be selected by the following priority (indicated by the number of the slot combination as given in Fig. 21):

Highest Selection Priority:	No. 1
	No. 2
	No. 5
	No. 6
	No. 3
	No. 4
	No. 7
Lowest Selection Priority	No. 8

Combinations 9, 10, 11 and 12 should not be used.

Rationale for not using slot combinations:

No. 9	Adjacent slot rule
No. 10	Opposite channel rule
No. 11	Adjacent slot rule
No. 12	Base station rule.

Figure 22

Slot selection rules flow diagram

### 4.4.2	Use of assignment for congestion resolution

A base station may assign Rr to all mobile stations except Class A shipborne mobile AIS stations to resolve congestion and can thus protect the viability of the VDL. To resolve congestion for Class A shipborne mobile AIS stations, the base station may use slot assignments to redirect slots used by the Class A shipborne mobile AIS station to FATDMA reserved slots.

## 4.5	Base station operation

A Base station accomplishes the following tasks:

–	provides transmission slot assignments);

–	provides assignment of amount of slots available to mobile station(s) ;

–	optionally provides GNSS corrections via the VDL(e.g. similar to AIS Message 17);

## 4.6	Meshed  network operation

TBD

## 4.7	Handling of errors related to packet sequencing and groups of packets

It should be possible to group transmission packets, which are addressed to another station (refer to addressed binary and addressed safety related messages) based on sequence number. Addressed packets should be assigned a sequence number by the transmitting station. The sequence number of a received packet should be forwarded together with the packet to the transport layer. Also, when errors related to packet sequencing and groups of packets are detected (see § 3.2.3), they should be handled by the transport layer as described in § 5.3.1.

# 5	Transport layer

The transport layer is responsible for:

–	converting data into transmission packets of correct size;

–	sequencing of data packets;

–	interfacing protocol to upper layers.

The interface between the transport layer and higher layers should be performed by the presentation interface.

## 5.1	Definition of transmission packet

A transmission packet is an internal representation of some information which can ultimately be communicated to external systems. The transmission packet is dimensioned so that it conforms to the rules of data transfer.

## 5.2	Conversion of data into transmission packets

### 5.2.1	Conversion to transmission packets

The transport layer should convert data, received from the presentation interface, into transmission packets.

- Sequencing of transmission packets

- Length od data packets

- Amount of data packets per minute

TABLE 21

| Number of slots | Maximum data bits | Stuffing bits | Total buffer bits |
| --- | --- | --- | --- |
| 1 | 136 | 36 | 56 |
| 2 | 360 | 68 | 88 |
| 3 | 584 | 100 | 120 |
| 4 | 808 | 132 | 152 |
| 5 | 1 032 | 164 | 184 |

## 5.3	Transmission packets

5.3.1	Addressed Messages

Addressed messages should have a destination user ID. The source station should anticipate an acknowledgement message. If an acknowledgement is not received the station  should retry the transmission. The station should wait 4 s before attempting retries. When a transmission is retried, the retransmit flag should be set to retransmitted. The number of retries should be 3, but it could be configurable between 0 and 3 retries by an external application via the presentation interface. When set to a different value by an external application, the number of retries should default to 3 retries after 8 min. The overall result of the data transfer should be forwarded to above layers. The acknowledgement should be between transport layers in two stations.

Each data transfer packet on the presentation interface should have a unique packet identifier consisting of the message type (binary or safety related messages), the source-ID, the destination-ID, and a sequence number.

The sequence number should be assigned in the appropriate presentation interface message which is input to the station.

The destination station should return the same sequence number in its acknowledgement message on the presentation interface.

The source station should not reuse a sequence number until it has been acknowledged or time-out has occurred.

The acknowledgement should be put first in the data transfer queue both on the presentation interface and on the VDL.

These acknowledgements are applicable only to the VDL. Other means must be employed for acknowledging applications.

See Fig. 23 and Annex 6.

figure 23

### 5.3.2	Broadcast messages

A broadcast message lacks a destination identifier ID. Therefore receiving stations should not acknowledge a broadcast message.

### 5.3.3	Conversion to presentation interface messages

Each received transmission packet should be converted to a corresponding presentation interface message and presented in the order they were received regardless of message category. Applications utilizing the presentation interface should be responsible for their own sequencing numbering scheme, as required. For a mobile station, addressed messages should not be output to the presentation interface, if destination User ID (destination MMSI) is different to the ID of own station (own MMSI).

### Sequencing of transmission packets

This Annex describes the method by which information should be exchanged between stations’ application layers (Application A and Application B) over the VDL through the PI.

The originating application assigns a sequence number to each transmission packet, using the addressed message. The sequence numbers can be 0, 1, 2 or 3. This number together with message type and destination gives the transmission a unique transaction identifier. This transaction identifier is communicated to the receiving application.

figure 26

Step 1:	Application A delivers 4 addressed messages addressed to B with sequence numbers 0, 1, 2 and 3 via PI.

figure 27

Step 2:	VDL A receives addressed messages and puts them in the transmit queue.

figure 28

Step 3:	VDL A transmits the messages to VDL B, which only receives messages with sequence numbers 0 and 3.

figure 29

Step 4:	DL B returns VDL-ACK messages with sequence numbers 0 and 3 to VOL A.

figure 30

Step 5:	VDL B delivers addressed messages with sequence numbers 0 and 3 to Application B.

figure 31

Step 6:	VDL A returns PI-ACK (OK) to Application A with sequence numbers 0 and 3.

figure 32

Step 7:	VDL A times out on sequence numbers 1 and 2 and retransmits the addressed messages to VDL B.

figure 33

Step 8:	VDL B successfully receives Message 2 and returns a VDL-ACK with sequence number 2.

Step 9:	VDL B delivers ABM (addressed binary message) message with sequence number 2 to Application B.

Step 10:		VDL A delivers PI-ACK (OK) with sequence number 2 to Application A.

figure 34

Step 11:		VDL A retransmits message, with sequence number 1, but does not receive a VDL-ACK from VDL B. It does this two times and is unsuccessful in delivering message.

Step 12:		VDL A, upon failing the transmit transaction of message with sequence number 1, delivers a PI-ACK (FAIL) to Application A.

## 5.4	Presentation interface protocol

Data, which is to be transmitted by the AIS device, should be input via the presentation interface; data, which is received by the AIS device, should be output through the presentation interface. The formats and protocol used for this data stream are defined by IEC 61162. S 100 and data structure from WG 1.

Application specific messages (ITU-R M.1371)

# 1	General

AIS messages where the data content is defined by the application are application specific messages. Examples of this are the binary Messages 6, 8, 25 and 26. The data content does not affect the operation of the AIS. AIS is a means for transferring the data content between stations. A functional message’s data structure consists of an application identifier (AI) followed by the application data.

## 1.1	Binary messages

A binary message consists of three parts:

–	Standard AIS framework (message ID, repeat indicator, source ID, and, for addressed binary messages, a destination ID)

–	16-bit application identifier (AI = DAC + FI), consisting of:

–	10-bit designated area code (DAC) – based on the MID;

–	6-bit function identifier (FI) – allows for 64 unique application specific messages.

–	Data content (variable length up to a given maximum).

## 1.2	Definition of application identifiers

The application identifier uniquely identifies the message and its contents. The application identifier is a 16-bit number used to identify the meaning of the bits making up the data content. The use of application identifiers is defined in § 2.

The DAC is a 10-bit number. DAC assignments are:

–	international (DAC = 1-9), maintained by international agreement for global use;

–	regional (DAC ≥ 10), maintained by the regional authorities affected;

–	test (DAC = 0), used for test purposes.

It is recommended that DAC 2-9 be used to identify subsequent versions of international specific messages and that the administrator of application specific messages base the DAC selection on the maritime identification digit (MID) of the administrator’s country or region. It is the intention that any application specific message can be utilized worldwide. The choice of the DAC does not limit the area where the message can be used.

The FI is a 6-bit number assigned to uniquely identify the data content structure within an application under a DAC assignment. Each DAC can support up to 64 applications.

–	The definition of the technical characteristics, as defined in Annexes 2, 3 and 4, of any AIS station covers layers 1 to 4 of the OSI model, only (see § 1, Annex 2).

–	The layers 5 (session layer), 6 (presentation layer) and 7 (application layer, that includes the human-machine-interface) should be in accordance with the definitions and guidelines given in this Annex in order to avoid application conflicts.

## 1.3	Definition of function messages

Each unique combination of application identifier (AI) and application data forms a functional message. The coding and decoding of the data content of a binary message is based on a table identified by the AI value. Tables identified by an International AI (IAI) value should be maintained and published by the international authority responsible for defining international function messages (IFM). Maintenance and publication of regional AI tables (RAI), defining regional function messages (RFM) should be the responsibility of national or regional authorities.

Table 24 identifies up to ten IFM designed to provide support for any implementation of broadcast and addressed binary messages (system applications). These are defined and maintained by ITU.

# 2	Binary data structure

This chapter provides general guidance for developing the structure of the data content for broadcast and addressed binary messages.

## 2.1	Application identifier

Addressed and broadcast binary messages should contain a 16-bit application identifier, structured as follows:

TABLE 23

| Bit | Description |
| --- | --- |
| 15-6 | Designated area code (DAC). This code is based on the maritime identification digits (MID). Exceptions are 0 (test) and 1 (international). Although the length is 10 bits, the DAC codes equal to or above 1 000 are reserved for future use |
| 5-0 | Function identifier. The meaning should be determined by the authority which is responsible for the area given in the designated area code |

Whereas the application identifier allows for regional applications, the application identifier should have the following special values for international compatibility.

### 2.1.1	Test application identifier

The test application identifier (DAC = 0) with any function identifier (0 to 63) should be used for testing purposes. The function identifier is arbitrary.

### 2.1.2	International application identifier

The international application identifier (DAC = 1) should be used for international applications of global relevance. Specific international applications are identified by a unique function identifier (see Table 24).

TABLE 24

| Application identifier (decimal) |  | Application identifier (binary) |  | Description |
| --- | --- | --- | --- | --- |
| DAC | Function identifier | DAC | Function identifier |  |
| 001 | 00 | 0000 0000 01 | 00 0000 | IFM 0 = Text telegram 6-bit ASCII (§ 5.1) |
| 001 | 01 | 0000 0000 01 | 00 0001 | Discontinued |
| 001 | 02 | 0000 0000 01 | 00 0010 | IFM 2 = Interrogation on specific IFM (§ 5.2) |
| 001 | 03 | 0000 0000 01 | 00 0011 | IFM 3 = Capability interrogation (§ 5.3) |
| 001 | 04 | 0000 0000 01 | 00 0100 | IFM 4 = Capability interrogation reply (§ 5.4) |
| 001 | 05 | 0000 0000 01 | 00 0101 | IFM 5 = Application acknowledgement to an addressed binary message (§ 5.5) |
| 001 | 06 to 09 | 0000 0000 01 | – | Reserved for future system applications |
| 001 | 10 to 63 | 0000 0000 01 | – | Reserved for international operational applications |

NOTE 1 – The DAC codes 1000 to 1023 are reserved for future use.

# 3	Guidelines for creating functional messages

Slot use by functional messages should take into account the system level impact on the VHF data link loading.

## 3.1	International functional messages

The following should be considered when creating international functional messages:

–	published international functional messages (see IMO and ITU documents);

–	legacy and compatibility issues with current, superseded, or obsolete message structures;

–	period of time needed to formally introduce a new functionality;

–	each functional message should have a unique identifier (AI);

–	limited number of available international functional identifiers.

## 3.2	Regional functional messages

The following should be considered when creating regional functional messages:

–	published regional and international functional messages;

–	legacy and compatibility issues with current, superseded or obsolete message structures (e.g. 3-bit FI version indicator);

–	period of time needed and cost to formally introduce a new functionality;

–	each functional message should have a unique identifier (AI);

–	limited number of functional identifiers allocated for local, regional, national, or multinational use;

–	requirements for encrypted messages.

# 4	Guidelines for drafting functional messages

When developing functional messages, the following should be considered:

–	a message for test and evaluation purposes to ensure integrity in an operational system;

–	rules given in § 3.3.7, Annex 2 (Message structure), and § 3, Annex 8 (Message descriptions);

–	values for not available, normal, or malfunctioning should be defined for every data field, as appropriate;

–	default values should be defined for each data field.

When position information is included, in addition to latitude and longitude, and if applicable, it should comprise data fields in the following order (see AIS Messages 1 and 5):

–	position accuracy;

–	longitude;

–	latitude;

–	precision;

–	type of electronic position fixing device;

–	time stamp.

When transmitting time and/or date information, other than time stamp for position information, this information should be as defined as follows (see AIS Message 4):

–	UTC year: 	1-9999; 0 = UTC year not available = default (14 bits)

–	UTC month:	1-12; 0 = UTC month not available = default (4 bits)

–	UTC day:	1-31; 0 = UTC day not available = default (5 bits)

–	UTC hour:	0-23; 24 = UTC hour not available = default (5 bits)

–	UTC minute:	0-59; 60 = UTC minute not available = default (6 bits)

–	UTC second:	0-59; 60 = UTC second not available = default (6 bits).

When transmitting information on direction of movement, this should be defined as direction of movement over ground (see AIS Message 1).

All data fields of the functional messages should observe byte boundaries. If needed to align with byte boundaries, spares should be inserted.

Applications should minimize slot use, taking into account buffering and bit-stuffing, see Annex 2 at the appropriate definition of the binary message.

# 5	Definitions of system related international function messages

## 5.1	International function message 0: Text using 6-bit ASCII

IFM 0 is used by applications that use AIS stations to transfer 6-bit ASCII text between applications. The text can be sent with binary Message 6, 8, 25 or 26. The parameter, “acknowledge required flag”, should be set to 0 when broadcast with Message 8, 25 or 26.

When long text strings are sub-divided, an 11-bit “text sequence number” is used. The text sequence number is used by the originating application to sub-divide the text and by the recipient application to re-assemble the text. The text sequence numbers for each sub-division should be selected to be contiguous and always increasing (110, 111, 112, …). If multiple texts are being transferred, the text sequence numbers should be chosen to associate correctly the sub-divided text with the correct text strings.

TABLE 25

International function message 0 using Message 6, addressed binary message

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 6; always 6 |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. See § 4.6.1, Annex 2; 0-3; 0 = default; 3 = do not repeat any more |
| Source ID | 30 | MMSI number of source station |
| Sequence number | 2 | 0-3; see § 5.3.1, Annex 2 |
| Destination ID | 30 | MMSI number of destination station |
| Retransmit flag | 1 | Retransmit flag should be set upon retransmission: 0 = no retransmission = default; 1 = retransmitted |
| Spare | 1 | Not used. Should be zero |
| DAC | 10 | International DAC = 110 = 00000000012 |
| FI | 6 | Function identifier = 010 = 0000002 |
| Acknowledge required flag | 1 | 1 = reply is required, optional for addressed binary messages and not used for binary broadcast messages 0 = reply is not required, optional for an addressed binary message and required for binary broadcast messages |
| Text sequence number | 11 | Sequence number to be incremented by the application. All zeros indicates that sequence numbers are not being used |
| Text string | 6-906 | 6-bit ASCII as defined in Table 47, Annex 8. When using this IFM, the number of slots used for transmission should be minimized taking into account Table 29. For Message 6 the maximum is 906. |
| Spare bits | Max 6 | Not used for data and should be set to zero. The number of bits should be either 0, 2, 4, or 6 to maintain byte boundaries. NOTE 1 – When a 6-bit spare is needed to satisfy the 8-bit byte boundary rule, the 6-bit spare will be interpreted as a valid 6-bit character (all zeros is the “@” character). This is the case when the number of characters is: 1, 5, 9, 13,17, 21, 25, etc. |
| Total number of application data bits | 112-1 008 | For Message 6 the maximum is 920. |

TABLE 26

International function message 0 using Message 8, broadcast binary message

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 8; always 8 |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. See § 4.6.1, Annex 2; 0-3; 0 = default; 3 = do not repeat any more |
| Source ID | 30 | MMSI number of source station |
| Spare | 2 | Not used. Should be zero |
| DAC | 10 | International DAC = 110 = 00000000012 |
| FI | 6 | Function identifier = 010 = 0000002 |
| Acknowledge required flag | 1 | 1 = reply is required, optional for addressed binary messages and not used for binary broadcast messages 0 = reply is not required, optional for an addressed binary message and required for binary broadcast messages |
| Text sequence number | 11 | Sequence number to be incremented by the application. All zeros indicates that sequence numbers are not being used |
| Text string | 6-936 | 6-bit ASCII as defined in Table 47, Annex 8. When using this IFM, the number of slots used for transmission should be minimized taking into account Table 29. For Message 8 the maximum is 936. |
| Spare bits | Max 6 | Not used for data and should be set to zero. The number of bits should be either 0, 2, 4, or 6 to maintain byte boundaries. NOTE 1 – When a 6-bit spare is needed to satisfy the 8-bit byte boundary rule, the 6-bit spare will be interpreted as a valid 6-bit character (all zeros is the “@” character). This is the case when the number of characters is: 1, 5, 9, 13,17, 21, 25, etc. |
| Total number of application data bits | 80-1 008 |  |

TABLE 27

International function message 0 using Message 25, broadcast or addressed binary message

| Parameter | Number of bits | Description |  |
| --- | --- | --- | --- |
| Message ID | 6 | Identifier for Message 25; always 25 |  |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. See § 4.6.1, Annex 2; 0-3; 0 = default; 3 = do not repeat any more |  |
| Source ID | 30 | MMSI number of source station |  |
| Destination indicator | 1 | 0 = Broadcast (no Destination ID field used) 1 = Addressed (Destination ID uses 30 data bits for MMSI) |  |
| Binary data flag | 1 | Always 1 |  |
| Destination ID | 0/30 | Destination ID if used. | If Destination indicator = 0 (Broadcast), no data bits are needed for Destination ID. If Destination indicator = 1, 30 bits are used for Destination ID and spare bits for byte alignment. |
| Spare | 0/2 | Spare (if Destination ID used) |  |
| DAC | 10 | International DAC = 110 = 00000000012 |  |
| FI | 6 | Function identifier = 010 = 0000002 |  |
| Text sequence number | 11 | Sequence number to be incremented by the application. All zeros indicates that sequence numbers are not being used. |  |
| Text string | 6-66/6-96 | 6-bit ASCII as defined in Table 47, Annex 8. When using this IFM, the number of slots used for transmission should be 1 taking into account Table 29. For Message 25 the maximum is 66 for Addressed or 96 for Broadcast. |  |
| Spare bits | Max 7 | Not used for data and should be set to zero. The number of bits should be either 1, 3, 5 or 7 to maintain byte boundaries. NOTE 1 – When a 7-bit spare is needed to satisfy the 8-bit byte boundary rule, the 6-bit spare will be interpreted as a valid 6-bit character (all zeros is the “@” character). This is the case when the number of characters is: 1, 5, 9 and 13. |  |
| Total number of application data bits | 112-168/ 80-168 | 112-168 bits for Addressed, or 80-168 bits for Broadcast. |  |

TABLE 28

International function message 0 using Message 26, broadcast or addressed binary message

| Parameter | Number of bits | Description |  |
| --- | --- | --- | --- |
| Message ID | 6 | Identifier for Message 26; always 26 |  |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. See § 4.6.1, Annex 2; 0-3; 0 = default; 3 = do not repeat any more |  |
| Source ID | 30 | MMSI number of source station |  |
| Destination indicator | 1 | 0 = Broadcast (no Destination ID field used) 1 = Addressed (Destination ID uses 30 data bits for MMSI) |  |
| Binary data flag | 1 | Always 1 |  |
| Destination ID | 0/30 | Destination ID if used. | If Destination indicator = 0 (Broadcast), no data bits are needed for Destination ID. If Destination indicator = 1, 30 bits are used for Destination ID and spare bits for byte alignment. |
| Spare | 0/2 | Spare (if Destination ID used) |  |
| DAC | 10 | International DAC = 110 = 00000000012 |  |
| FI | 6 | Function identifier = 010 = 0000002 |  |
| Text sequence number | 11 | Sequence number to be incremented by the application. All zeros indicates that sequence numbers are not being used |  |
| Text string | 6-942/972 | 6-bit ASCII as defined in Table 47, Annex 8. When using this IFM, the number of slots used for transmission should be minimized taking into account Table 29. For Message 26 the maximum is 942 for Addressed or 972 for Broadcast. |  |
| Spare bits | Max 7 | Not used for data and should be set to zero. The number of bits should be either 1, 3, 5 or 7 to maintain byte boundaries. NOTE 1 – When a 7-bit spare is needed to satisfy the 8-bit byte boundary rule, the 6-bit spare will be interpreted as a valid 6-bit character (all zeros is the “@” character). This is the case when the number of characters is: 3, 7, 11, 15, 19, 23, 27, etc. |  |
| Communication state selector | 1 | 0 = SOTDMA communication state follows 1 = ITDMA communication state follows |  |
| Communication state | 19 | SOTDMA communication state (see § 3.3.7.2.1, Annex 2), if communication state selector flag is set to 0, or ITDMA communication state (§ 3.3.7.3.2, Annex 2), if communication state selector flag is set to 1 |  |
| Total number of application data bits | 128-1 064/ 96-1 064 | 128-1 064 bits for Addressed, or 96-1064 bits for Broadcast. |  |

Table 29 gives an estimate of the maximum number of 6-bit-ASCII characters that can be in the application data field of the binary data parameter of Messages 6, 8, 25 and 26. The number of slots used will be affected by the bit stuffing process.

TABLE 29

| Estimated number of slots | Maximum number of 6-bit ASCII characters based upon typical bit stuffing |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Addressed binary Message 6 | Broadcast binary Message 8 | Message 25 |  | Message 26 |  |
|  |  |  | Addressed binary | Broadcast binary | Addressed binary | Broadcast binary |
| 1 | 6 | 11 | 6 | 11 | 2 | 7 |
| 2 | 43 | 48 | − | − | 40 | 45 |
| 3 | 80 | 86 | − | − | 77 | 82 |
| 4 | 118 | 123 | − | − | 114 | 120 |
| 5 | 151 | 156 | − | − | 150 | 163 |
| NOTE 1 – The 5-slot value accounts for the worst case bit stuffing condition. |  |  |  |  |  |  |

## 5.2	International function message 2: Interrogation for a specific functional message

IFM 2 should be used by an application to interrogate (using Message 6) another application for the specified functional message.

The application responding to this interrogation should use an addressed binary message to reply.

TABLE 30

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 6; always 6 |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. See § 4.6.1, Annex 2; 0-3; 0 = default; 3 = do not repeat any more |
| Source ID | 30 | MMSI number of source station |
| Sequence number | 2 | 0-3; see § 5.3.1, Annex 2 |
| Destination ID | 30 | MMSI number of destination station |
| Retransmit flag | 1 | Retransmit flag should be set upon retransmission: 0 = no retransmission = default; 1 = retransmitted |
| Spare | 1 | Not used. Should be zero |
| DAC | 10 | International DAC = 110 = 00000000012 |
| FI | 6 | Function identifier = 210 = 0000102 |
| Requested DAC code | 10 | IAI, RAI or test |
| Requested FI code | 6 | See appropriate FI reference document(s) |
| Spare bits | 64 | Not used, should be set to zero, reserved for future use |
| Total number of bits | 168 | The resulting Message 6 occupies 1 slot. |

## 5.3	International function message 3: Capability interrogation

IFM 3 should be used by an application to interrogate (using Message 6) another application for the availability of application identifiers for the specified DAC. The request is made separately for each DAC.

IFM 3 can only be used as the data content of an addressed binary message.

TABLE 31

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 6; always 6 |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. See § 4.6.1, Annex 2; 0-3; 0 = default; 3 = do not repeat any more |
| Source ID | 30 | MMSI number of source station |
| Sequence number | 2 | 0-3; see § 5.3.1, Annex 2 |
| Destination ID | 30 | MMSI number of destination station |
| Retransmit flag | 1 | Retransmit flag should be set upon retransmission: 0 = no retransmission = default; 1 = retransmitted |
| Spare | 1 | Not used. Should be zero |
| DAC | 10 | International DAC = 110 = 00000000012 |
| FI | 6 | Function identifier = 310 = 0000112 |
| Requested DAC code | 10 | IAI, RAI or test |
| Spare bits | 70 | Not used, should be set to zero, reserved for future use |
| Total number of bits | 168 | The resulting Message 6 occupies 1 slot |

## 5.4	International function message 4: Capability reply

IFM 4 should be used by an application to reply (using Message 6) to a capability interrogation (IFM 3) function message. The reply contains the availability status of the application for each function identifier for the specified DAC.

The application should use an addressed binary message to reply to the interrogating application.

TABLE 32

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 6; always 6 |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. See § 4.6.1, Annex 2; 0-3; 0 = default; 3 = do not repeat any more |
| Source ID | 30 | MMSI number of source station |
| Sequence number | 2 | 0-3; see § 5.3.1, Annex 2 |
| Destination ID | 30 | MMSI number of destination station |
| Retransmit flag | 1 | Retransmit flag should be set upon retransmission: 0 = no retransmission = default; 1 = retransmitted |
| Spare | 1 | Not used. Should be zero |
| DAC | 10 | International DAC = 110 = 00000000012 |
| FI | 6 | Function identifier = 410 = 0001002 |

TABLE 32 (end)

| Parameter | Number of bits | Description |
| --- | --- | --- |
| DAC code | 10 | IAI, RAI or test |
| FI availability | 128 | FI capability table, pair of two consecutive bits should be used for every FI, in the order FI 0, FI 1, … FI 63. First bit of pair: 0 = FI not available (default) 1 = FI available. Second bit of the pair: reserved for future use; should be set to zero |
| Spare | 126 | Not used, should be set to zero, reserved for future use |
| Total number of bits | 352 | The resulting Message 6 occupies 2 slots |

## 5.5	International function message 5: Application acknowledgement to an addressed binary message

When requested, IFM 5 should be used by an application to confirm the reception of an addressed binary message. An application should never acknowledge a binary broadcast message.

If the interrogating application does not receive an IFM 5, when requested, then the application should assume that addressed AIS station does not have an application attached to its Presentation Interface (PI).

If there is any application at the AIS station, it should not respond if the “Acknowledge Required Flag” is set to 0.

TABLE 33

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 6; always 6 |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. See § 4.6.1, Annex 2; 0-3; 0 = default; 3 = do not repeat any more |
| Source ID | 30 | MMSI number of source station |
| Sequence number | 2 | 0-3; see § 5.3.1, Annex 2 |
| Destination ID | 30 | MMSI number of destination station |
| Retransmit flag | 1 | Retransmit flag should be set upon retransmission: 0 = no retransmission = default; 1 = retransmitted |
| Spare | 1 | Not used. Should be zero |
| DAC | 10 | International DAC = 110 = 00000000012 |
| FI | 6 | Function identifier = 510 = 0001012 |
| DAC code of received functional messsage | 10 | Recommended to be spare |
| FI code of received functional message | 6 |  |
| Text sequence number | 11 | Sequence number in the message being acknowledged as properly received 0 = default (no sequence number) 1-2 047 = sequence number of received functional messsage |
| AI available | 1 | 0 = received but AI not available 1 = AI available |

TABLE 33 (end)

| Parameter | Number of bits | Description |
| --- | --- | --- |
| AI response | 3 | 0 = unable to respond 1 = reception acknowledged 2 = response to follow 3 = able to respond but currently inhibited 4-7 = spare for future use |
| Spare bits | 49 | Not used, should be set to zero, reserved for future use |
| Total number of bits | 168 | The resulting Message 6 occupies 1 slot |

## 3.23	Message 25: Single slot binary message

This message is primarily intended short infrequent data transmissions. The single slot binary message can contain up to 128 data-bits depending on the coding method used for the contents, and the destination indication of broadcast or addressed. The length should not exceed one slot. See application identifiers in § 2.1, Annex 5.

This message should not be acknowledged by either Message 7 or 13.

TABLE 80

| Parameter | Number of bits | Description |  |
| --- | --- | --- | --- |
| Message ID | 6 | Identifier for Message 25; always 25 |  |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. Refer to § 4.6.1, Annex 2; 0-3; default = 0; 3 = do not repeat any more |  |
| Source ID | 30 | MMSI number of source station |  |
| Destination indicator | 1 | 0 = Broadcast (no Destination ID field used) 1 = Addressed (Destination ID uses 30 data bits for MMSI) |  |
| Binary data flag | 1 | 0 = unstructured binary data (no Application Identifier bits used) 1 = binary data coded as defined by using the 16-bit Application identifier |  |
| Destination ID | 0/30 | Destination ID (if used) | If Destination indicator = 0 (Broadcast); no data bits are needed for the Destination ID. If Destination indicator = 1; 30 bits are used for Destination ID and spare bits for byte alignment. |
| Spare | 0/2 | Spare (if Destination ID used) |  |

TABLE 80 (end)

| Parameter | Number of bits | Description |  |  |
| --- | --- | --- | --- | --- |
| Binary data | Broadcast Maximum 128 Addressed Maximum 96 | Application identifier (if used) | 16 bits | Should be as described in § 2.1, Annex 5 |
|  |  | Application binary data | Broadcast Maximum 112 bits Addressed Maximum 80 bits | Application specific data |
| Maximum number of bits | Maximum 168 | Occupies up to 1 slot subject to the length of sub-field message content Class B “CS” mobile AIS stations should not transmit |  |  |

Table 81 gives the maximum number of binary data-bits for settings of destination indicator and coding method flags, such that, the message does not exceed one slot.

TABLE 81

| Destination indicator | Coding method | Binary data (maximum bits) |
| --- | --- | --- |
| 0 | 0 | 128 |
| 0 | 1 | 112 |
| 1 | 0 | 96 |
| 1 | 1 | 80 |

## 3.24	Message 26: Multiple slot binary message with communications state

This message is primarily intended for scheduled binary data transmissions by applying either the SOTDMA or ITDMA access scheme. This multiple slot binary message can contain up to 1 004 data-bits (using 5 slots) depending on the coding method used for the contents, and the destination indication of broadcast or addressed. See application identifiers in § 2.1, Annex 5.

This message should not be acknowledged by either Message 7 or 13.

TABLE 82

| Parameter | Number of bits | Description |  |  |
| --- | --- | --- | --- | --- |
| Message ID | 6 | Identifier for Message 26; always 26 |  |  |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. Refer to § 4.6.1, Annex 2; 0-3; default = 0; 3 = do not repeat any more |  |  |
| Source ID | 30 | MMSI number of source station |  |  |
| Destination indicator | 1 | 0 = Broadcast (no Destination ID field used) 1 = Addressed (Destination ID uses 30 data bits for MMSI) |  |  |
| Binary data flag | 1 | 0 = unstructured binary data (no Application Identifier bits used) 1 = binary data coded as defined by using the 16-bit Application identifier |  |  |
| Destination ID | 0/30 | Destination ID (if used) |  | If Destination indicator = 0 (Broadcast); no data bits are needed for the Destination ID. If Destination indicator = 1; 30 bits are used for the Destination ID and 2 spare bits for byte alignment. |
| Spare bits | 0/2 | Spare (if Destination ID used) |  |  |
| Binary data | Broadcast Maximum 104 | Application identifier (if used) | 16 bits | Should be as described in § 2.1, Annex 5 |  |
|  | Addressed Maximum 72 | Application binary data | Broadcast Maximum 88 bits Addressed Maximum 56 bits | Application specific data |  |
| Binary data added by 2nd slot | 224 | Allows for 32 bits of bit-stuffing |  |  |  |
| Binary data added by 3rd slot | 224 | Allows for 32 bits of bit-stuffing |  |  |  |
| Binary data added by 4th slot | 224 | Allows for 32 bits of bit-stuffing |  |  |  |
| Binary data added by 5th slot | 224 | Allows for 32 bits of bit-stuffing |  |  |  |
| Spare | 4 | Needed for byte alignment |  |  |  |
| Communication state selector flag | 1 | 0 = SOTDMA communication state follows 1 = ITDMA communication state follows |  |  |  |
| Communication state | 19 | SOTDMA communication state (see § 3.3.7.2.1, Annex 2), if communication state selector flag is set to 0, or ITDMA communication state (§ 3.3.7.3.2, Annex 2), if communication state selector flag is set to 1 |  |  |  |
| Maximum number of bits | Maximum 1 064 | Occupies up to 3 slots, or up to 5 slots when able to use FATDMA reservations. For Class B “SO” mobile AIS stations the length of the message should not exceed 3 slots. Class B “CS” mobile AIS stations should not transmit |  |  |  |

Table 83 gives the maximum number of binary data-bits for settings of destination indicator and coding method flags, such that, the message does not exceed the indicated number of slots.

TABLE 83

| Destination indicator | Binary data flag | Binary data (maximum bits) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 1-slot | 2-slot | 3-slot | 4-slot | 5-slot |
| 0 | 0 | 104 | 328 | 552 | 776 | 1000 |
| 0 | 1 | 88 | 312 | 536 | 760 | 984 |
| 1 | 0 | 72 | 296 | 520 | 744 | 968 |
| 1 | 1 | 56 | 280 | 504 | 728 | 952 |

## 3.14	Message 16: Assigned mode command

Assignment should be transmitted by a base station when operating as a controlling entity. Other stations can be assigned a transmission schedule, other than the currently used one. If a station is assigned a schedule, it will also enter assigned mode.

Two stations can be assigned simultaneously.

When receiving an assignment schedule, the station should tag it with a time-out, randomly selected between 4 and 8 min after the first transmission.

When a Class A shipborne mobile AIS station receives an assignment it should revert to either the assigned reporting rate or the resulting reporting rate (when slot assignment is used) or the autonomously derived reporting rate (see § 4.3.1, Annex 2), whatever is higher. The Class A shipborne mobile AIS station should indicate that it is in assigned mode (by using the appropriate messages), even if it reverts to a higher autonomously derived reporting rate.

NOTE 1 – The assigning station should monitor the mobile station’s transmissions in order to determine when the mobile station will time-out.

For bounds of assignment settings see Table 16, Annex 2.

Transmissions of Message 16 by base stations using assignment of transmission slots should consider directing transmissions to slots which have previously been reserved by the base station by FATDMA (Message 20).

If continued assignment is required, the new assignment should be transmitted before the start of the last frame of the previous assignment.

TABLE 67

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 16. Always 16 |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. See § 4.6.1, Annex 2; 0-3; 0 = default; 3 = do not repeat any more |
| Source ID | 30 | MMSI of assigning station |
| Spare | 2 | Spare. Should be set to zero. Reserved for future use |
| Destination ID A | 30 | MMSI number. Destination identifier A |
| Offset A | 12 | Offset from current slot to first assigned slot(1) |
| Increment A | 10 | Increment to next assigned slot(1) |
| Destination ID B | 30 | MMSI number. Destination identifier B. Should be omitted if there is assignment to station A, only |
| Offset B | 12 | Offset from current slot to first assigned slot. Should be omitted if there is assignment to station A, only(1) |

TABLE 67 (end)

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Increment B | 10 | Increment to next assigned slot(1). Should be omitted, if there is assignment to station A, only |
| Spare | Maximum 4 | Spare. Not used. Should be set to zero. The number of spare bits, which should be 0 or 4, should be adjusted in order to observe byte boundaries. Reserved for future use |
| Number of bits | 96 or 144 | Should be 96 or 144 bits |
| (1) To assign a reporting rate for a station, the parameter increment should be set to zero. The parameter offset should then be interpreted as the number of reports in a time interval of 10 min. |  |  |

When number of reports per 10 min are assigned, only multiples of 20 between 20 and 600 should be used. If a mobile station received a value which is not a multiple of 20 but below 600, it should use the next higher multiple of 20. If a mobile station receives a value greater than 600 it should use 600.

When slot increments are assigned, one of the following increment parameter settings should be used:

0 = see above
1 = 1 125 slots
2 = 375 slots
3 = 225 slots
4 = 125 slots
5 = 75 slots
6 = 45 slots 
7 = undefined.

If a station receives the value 7, the station should disregard this assignment. Class B mobile AIS stations should not be assigned a reporting interval of less than 2 s.

## 3.18	Message 20: Data link management message

This message should be used by base station(s) to pre-announce the fixed allocation schedule (FATDMA) for one or more base station(s) and it should be repeated as often as required. This way the system can provide a high level of integrity for base station(s). This is especially important in regions where several base stations are located adjacent to each other and mobile station(s) move between these different regions. These reserved slots cannot be autonomously allocated by mobile stations.

The mobile station, within 120 nautical miles should then reserve the slots for transmission by the base station(s) until time-out occurs. The base station should refresh the time-out value with each transmission of Message 20 in order to allow mobile stations to terminate their reservation for the use of the slots by the base stations (refer to § 3.3.1.2, Annex 2).

The parameters: offset number, number of slots, time-out, and increment should be treated as a unit, meaning that if one parameter is defined all other parameters should be defined within that unit. The parameter offset number should denote the offset from the slot in which Message 20 was received to the first slot to be reserved. The parameter number of slots should denote the number of consecutive slots to be reserved starting with the first reserved slot. This defines a reservation block.

This reservation block should not exceed 5 slots. The parameter increment should denote the number of slots between the starting slot of each reservation block. An increment of zero indicates one reservation block per frame. The values recommended for increment are as follows: 2, 3, 5, 6, 9, 10, 15, 18, 25, 30, 45, 50, 75, 90, 125, 150, 225, 250, 375, 450, 750, or 1125. Use of one of these values guarantees symmetric slot reservations throughout each frame. This message applies only to the frequency channel in which it is transmitted.

If interrogated and no data link management information available, only offset number 1, number of slots 1, time-out 1, and increment 1 should be sent. These fields should all be set to zero.

TABLE 72

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 20; always 20 |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. See § 4.6.1, Annex 2; 0-3; 0 = default; 3 = do not repeat any more |
| Source station ID | 30 | MMSI number of base station |
| Spare | 2 | Not used. Should be set to zero. Reserved for future use |
| Offset number 1 | 12 | Reserved offset number; 0 = not available(1) |
| Number of slots 1 | 4 | Number of reserved consecutive slots: 1-15; 0 = not available(1) |
| Time-out 1 | 3 | Time-out value in minutes; 0 = not available(1) |
| Increment 1 | 11 | Increment to repeat reservation block 1; 0 = one reservation block per frame(1) |
| Offset number 2 | 12 | Reserved offset number (optional) |
| Number of slots 2 | 4 | Number of reserved consecutive slots: 1-15; optional |

TABLE 72 (end)

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Time-out 2 | 3 | Time-out value in minutes (optional) |
| Increment 2 | 11 | Increment to repeat reservation block 2 (optional) |
| Offset number 3 | 12 | Reserved offset number (optional) |
| Number of slots 3 | 4 | Number of reserved consecutive slots: 1-15; optional |
| Time-out 3 | 3 | Time-out value in minutes (optional) |
| Increment 3 | 11 | Increment to repeat reservation block 3 (optional) |
| Offset number 4 | 12 | Reserved offset number (optional) |
| Number of slots 4 | 4 | Number of reserved consecutive slots: 1-15; optional |
| Time-out 4 | 3 | Time-out value in minutes (optional) |
| Increment 4 | 11 | Increment to repeat reservation block 4 (optional) |
| Spare | Maximum 6 | Not used. Should be set to zero. The number of spare bits which may be 0, 2, 4 or 6 should be adjusted in order to observe byte boundaries. Reserved for future use |
| Number of bits | 72-160 |  |
| (1) If interrogated and no data link management information is available, only Offset number 1, number of slots 1, time-out 1, and increment 1 should be sent. These fields should all be set to zero. |  |  |

## 3.21	Message 23: Group assignment command

The Group assignment command is transmitted by a base station when operating as a controlling entity(see § 4.3.3.3.2 Annex 7 and § 3.20). This message should be applied to a mobile station within the defined region and as selected by “Ship and Cargo Type” or “Station type”. The receiving station should consider all selector fields concurrently. It controls the following operating parameters of a mobile station:

–	transmit/receive mode;

–	reporting interval;

–	the duration of a quiet time.

Station type 10 should be used to define the base station coverage area for control of Message 27 transmissions by Class A and Class B “SO” mobile stations. When station type is 10 only the fields latitude, longitude are used, all other fields should be ignored. This information will be relevant until three minutes after the last reception of controlling Message 4 from the same base station (same MMSI).

TABLE 76

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 23; always 23 |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. 0-3; default = 0; 3 = do not repeat any more |
| Source ID | 30 | MMSI of assigning station |
| Spare | 2 | Spare. Should be set to zero |
| Longitude 1 | 18 | Longitude of area to which the group assignment applies; upper right corner (north-east); in 1/10 min (180, East = positive, West = negative) |
| Latitude 1 | 17 | Latitude of area to which the group assignment applies; upper right corner (north-east); in 1/10 min (90, North = positive, South = negative) |
| Longitude 2 | 18 | Longitude of area to which the group assignment applies; lower left corner (south-west); in 1/10 min (180, East = positive, West = negative) |
| Latitude 2 | 17 | Latitude of area to which the group assignment applies; lower left corner (south-west); in 1/10 min (90, North = positive, South = negative) |
| Station type | 4 | 0 = all types of mobiles (default); 1 = Class A mobile stations only; 2 = all types of Class B mobile stations; 3 = SAR airborne mobile station; 4 = Class B “SO” mobile stations only; 5 = Class B “CS” shipborne mobile station only; 6 = inland waterways; 7 to 9 = regional use; 10 = base station coverage area (see Message 4 and Message 27); 11 to 15 = for future use |
| Type of ship and cargo type | 8 | 0 = all types (default) 1...99 see Table 53 100...199 reserved for regional use 200...255 reserved for future use |
| Spare | 22 | Not used. Should be set to zero. Reserved for future use |
| Tx/Rx mode | 2 | This parameter commands the respective stations to one of the following modes: 0 = TxA/TxB, RxA/RxB (default); 1 = TxA, RxA/RxB, 2 = TxB, RxA/RxB, 3 = reserved for future use |
| Reporting interval | 4 | This parameter commands the respective stations to the reporting interval given in Table 77 |
| Quiet time | 4 | 0 = default = no quiet time commanded; 1-15 = quiet time of 1 to 15 min |
| Spare | 6 | Not used. Should be set to zero. Reserved for future use |
| Number of bits | 160 | Occupies one-time period |

TABLE 77

Reporting interval settings for use with message 23

| Reporting interval field setting | Reporting interval for Message 23 |
| --- | --- |
| 0 | As given by the autonomous mode |
| 1 | 10 min |
| 2 | 6 min |
| 3 | 3 min |
| 4 | 1 min |
| 5 | 30 s |
| 6 | 15 s |
| 7 | 10 s |
| 8 | 5 s |
| 9 | Next shorter reporting interval (only applicable if in autonomous mode) |
| 10 | Next longer reporting interval (only applicable if in autonomous mode) |
| 11 | 2 s (not applicable to the Class B “CS” and Class B “SO”) |
| 12-15 | Reserved for future use |
| NOTE 1 – When the dual channel transmission is suspended by Tx/Rx mode command 1 or 2, the required reporting interval should be maintained using the remaining transmission channel. |  |

# Technical considerations for VDES Access Schemes

This section provides technical considerations in designing access schemes for VDE terrestrial, VDE Satellite and the interaction between these VDES components.

From Figure 1, the satellite downlink shares the spectrum with the terrestrial ship-ship and shore-ship links, and the satellite uplink shares the spectrum with the terrestrial ship-shore link. Thus, access schemes should be considered to mitigate potential conflicts between the links.

## TDMA access scheme for the VDE terrestrial service

The VDES terrestrial service is comprised of ASM, VDE ship-shore, VDE shore-ship and VDE ship-ship. An example shipborne VDES transceiver implementation is illustrated in Figure 4 below. Note that in this implementation example all receivers, including the AIS receivers, are protected from blocking from the shipborne VHF radio by the band-pass filter that attenuates signals from the lower side of the Appendix 18 band. The AIS receiver blocking issue, along with the fact that the AIS can share the same antenna with the other VDES functions, is incentive for manufacturers to consider this implementation for their VDES system designs.

Figure 4

Example VDES transceiver implementation

### TDMA access scheme for the VDES ASM channels

Note that Recommendation ITU-R M.1371-5 specifies the access schemes for the AIS Messages, including ITDMA, on the AIS channels, and it specifies the structure for ASM with various contents. VDES takes ASM to another level by providing dedicated ASM channels to relieve congestion on the AIS channels. Under VDES, the access scheme for using the ASM channels could be initially by CSTDMA (Carrier-Sense TDMA) for the first transmission in a frame, followed by ITDMA for subsequent transmissions in that frame. This scheme mitigates simultaneous transmissions by ships and/or shore stations on the ASM channels. An ASM transmission should not exceed five contiguous slots.

### TDMA access scheme for the VDES ship-shore link

The TDMA access scheme for the VDE1-A ship-shore link could be by reservation through ITDMA (Incremental TDMA) from an ASM (Application Specific Message) on either one of the ASM channels, as described in 6.2.2.2. A VDE1-A ship-shore transmission should not exceed five contiguous slots.

### TDMA access scheme for the VDES ship-ship link

The TDMA access scheme for the VDE1-B ship-ship link could be the same as for the ASM channels, i.e., initially by CSTDMA (Carrier-Sense TDMA) for the first transmission in a frame, followed by ITDMA for subsequent transmissions in that frame. This scheme mitigates simultaneous ship-ship transmissions. A VDE1-B ship-ship transmission should not exceed five contiguous slots.

### TDMA access scheme for the VDES shore-ship link

The TDMA access scheme for the VDE1-B shore-ship link could be the same as for the VDE1 ship-shore link, i.e., by reservation through ITDMA (Incremental TDMA) from an ASM (Application Specific Message) on either one of the ASM channels. This is necessary because the shore station has a very wide coverage area compared to ships, and it needs to have priority access to the VDE1 channel in its coverage area. A VDE1-B shore-ship transmission should not exceed five contiguous slots.

## Sharing options for the VDE terrestrial and VDE satellite services

### VDE Terrestrial links on the upper legs (VDE1-B) and VDE Satellite Downlink

Table 3 provides the power flux-density (PFD) at the Earth’s surface from the satellite downlink at various elevation angles from 00 to 900.  Although the PFD mask is selected to minimize interference to the land mobile service and to maximize reception by ship VDES stations, there is a potential effect of raising the noise floor for reception of the terrestrial VDES links during satellite VDE downlink transmissions when the satellite is the field of view.

Issues to be considered for the sharing the VDE1-B frequencies and the VDE-SAT Downlink are:

- When shipborne VDES transceivers are simplex they cannot receive while transmitting.

- VDE-SAT downlink transmission levels, by raising the noise floor, will potentially have an impact on reception of ship-to-ship and shore-to-ship VDES.

- Ship-to-ship and shore-to-ship VDES transmissions, depending on the distance, by co-channel interference, will potentially interfere with reception of the VDE-SAT downlink

#### Frequency division multiple access (FDMA)

Frequency division multiple access is accomplished by using only the upper 50 kHz for the VDE-SAT downlink, i.e., only the two channels 2026 and 2086.  The frequency division multiple access would mitigate the last two issues stated above. Compared to other techniques proposed below, the FDMA would be the most straightforward to implement. However it would result in a reduction of the bandwidth to 1/3, and cause the VDE-SAT downlink transmissions to last three times longer for the same payload, and it would not mitigate the first issue stated above.

#### Time division multiple access (TDMA)

Time division multiple access approach for shore-ship/ship-ship and VDE SAT downlink services would allow the full use of the spectrum assigned to each service in a time sharing manner. Time sharing can mitigate all the three of the issues stated in Section 6 above. However, it would impose some design challenges for the VDE-SAT components and compromise the throughput of the VDE-SAT Downlink.

The AIS-based TDMA slot structure (2250 slots/minute/frame) and access schemes (ITDMA, CSTDMA and FATDMA) that are used for VDES are defined in Recommendation ITU-R M.1371-5. This TDMA organization scheme protects the integrity of the AIS and is used to organize and synchronize the ASM and VDE transmissions.

#### Full Frequency reuse (superposition)

In this approach, the terrestrial and satellite components are allowed to simultaneously use channels 2024, 2084, 2025 and 2085.  The VDE-SAT downlink will additionally use channels 2026 and 2086. The VDE-SAT downlink could continuously broadcast to maximize the data dissemination to a large number of ships in its field of view. This would allow for more efficient implementation of the VDE-SAT receivers. The interference caused by the VDE-SAT downlink on the VDE terrestrial could, in principle, be compensated for by the use of more protected coding scheme in the terrestrial link, only during the satellite passage.

For a most likely scenario of Low Earth Orbit satellites with a polar orbit, the impact of satellite interference could be limited to only less than 15 minutes per day per satellite for geographical locations with latitudes within ±50 degrees, as shown in Figure 5.

Figure 5

Time duration where signal level exceeds -117dBm as a function of geographical position

### VDE Terrestrial (VDE1-A) and VDE-SAT Uplink

Due to the large field of view, a passing satellite would receive a number of colliding messages from different VDE-terrestrial links (ship-to-shore) simultaneously that would interfere with ship to satellite links (channel 1024, 1084, 1025, 1085).  The following multiple access schemes can be envisaged to mitigate/minimize the impact of VDE terrestrial link on VDE satellite uplink.

#### Frequency Division Multiple Access (FDMA)

The frequency division multi-access scheme separates the satellite channels into two groups: Channels 1024, 1084, 1025 and 1085 that are subject to terrestrial interference are considered as a single or multi-carrier satellite uplink channel(s). Highly robust waveforms would be selected for these channels to allow for interference mitigations caused by VDE terrestrial.

The second group of carriers are considered to occupy Channel 1026 and 1086 where no VDE terrestrial transmission is present.

#### Time Division Multiple Access (TDMA)

VDE-SAT uplink follows the same frame structure as VDE terrestrial occupying VDE1-A channels. There are pre-assigned time slots dedicated to satellite transmission preventing interference from any VDE terrestrial link.

Alternatively, noting that Recommendation ITU-R M.1371-5 specifies the access schemes for the AIS Messages, including ITDMA, on the AIS channels, and it specifies the structure for ASM with various contents. VDES takes ASM to another level by providing dedicated ASM channels to relieve congestion on the AIS channels. Under VDES, the access scheme for using the ASM channels could be initially by CSTDMA (Carrier-Sense TDMA) for the first transmission in a frame, followed by ITDMA for subsequent transmissions in that frame. This scheme mitigates simultaneous transmissions by ships and/or shore stations on the ASM channels.

#### Full Frequency reuse

The terrestrial and satellite components are allowed to simultaneously use channels 1024, 1084, 1025 and 1085.  The VDE-SAT uplink would use properly designed waveforms occupying the VDE-SAT uplink channels to minimize the impact of interference caused by the VDE terrestrial transmissions.

# Transmission waveforms for VDES Terrestrial Links

ITU-approved waveforms for spectrum-efficient data transmission in the VHF maritime band are described in Recommendation ITU-R M.1842-1. These waveforms have been demonstrated in the land-mobile service and in maritime trials, to provide robust data service and to mitigate multipath degradation at extended propagation ranges in intense electromagnetic environments. Table 2 below provides a comparison of performance between the current AIS standard, Recommendation ITU-R M.1371, and the new applications introduced for the terrestrial VDES links, ASM and VDE. Note that the spectrum efficiency for the AIS is much lower than for VDES, but the AIS modulation has superior co-channel rejection which provides better range discrimination and improved safety of navigation for ships. Each modulation type is intended to best fit its designated application (AIS, ASM and VDE).

Propagation range predictions for the terrestrial links are provided in Annex 3 in accordance with the ITU propagation standard Recommendation ITU-R P.1546-4.

Table 2

ITU-standard transmission waveforms for AIS, ASM and VDE terrestrial links

|  | 25 kHz Channels for AIS | 25 kHz Channels for ASM | 100 kHz Channels for VDE |
| --- | --- | --- | --- |
| ITU Standard | ITU-R M.1371-5 | ITU-R M.1842-1 Annex 1 | ITU-R M.1842-1 Annex 4 *** |
| Digital Modulation | GMSK, single carrier | π/4 DQPSK, single carrier | 16-QAM, 32 multi-carriers, 2.7 kHz spacing |
| Data Rate (raw)* | 9.6 kbps (1X) | 28.8 kbps (3X) | 307.2 kbps (32X) |
| Sensitivity** | -107dBm (min) -112dBm (typical) | -107dBm (min) -112dBm (typical) | -98dBm (ships) -103dBm (base stations) |
| Co-channel rejection (CCR)** | 10dB | 19dB | 19dB |
| AIS Message types | 1, 2, 3, 5, 18, 19, 27 … | 6, 7, 8, 12, 13, 14, 25, 26 … and ASM | VDE messages |
| Rationale | Optimum choice (better CCR) for position reports in a ship-to-ship navigation safety environment. | Provides higher (3X) data transmission than AIS. Inferior CCR (+9dB) and range discrimination compared to AIS. | Provides much higher (32X) data transmission than AIS. Inferior CCR (+9dB) and range discrimination compared to AIS. |

* These figures are raw, over the air, bit transmission rates.  The data rates are less, subject to coding, packet structure and FEC

** These figures are based on published standards.  For AIS, the standard is IEC 61993-2 and for VDE the standard is ETSI EN 300 392-2 version 3.4.1, which refers to a land mobile application TETRA.

*** For greater robustness where needed, ITU-R M.1842-1 Annex 1 may be used.

## Transmission waveform for the 25 kHz ASM channels

Transmission of ASM on 25 kHz channels should be by π/4 DQPSK single-carrier modulation as described in Rec. ITU-R M.1842-1 Annex 1. Forward error correction (FEC) is applied due to the fact that the ASM messages are not repeated as are AIS position reports (which do not have FEC). The waveform is recommended because it has high sensitivity, 70 dB adjacent channel power ratio (ACPR) and 28.8 kbps data rate.

- It is generated by phase modulation with an inter-symbol rotation of π/4 radians. This produces an amplitude envelope with very moderate peak to average power ratio (PAPR);

- It has excellent characteristics for detection by satellites as required by the channel plan.

## Transmission waveform for the 100 kHz VDE channels

Transmission of VDE on 100 kHz channels should be by 16-QAM, 32 multi-carriers, with 2.7 kHz spacing and 307.2 kbps data rate as described in Rec. ITU-R M.1842-1 Annex 4.    This multi-carrier scheme is not OFDM (orthogonal frequency division multiple access) since the carrier spacing is 2.7 kHz which provides more inter-carrier margin than OFDM which would require 2.4 kHz spacing. This waveform is comprised of 32 multi-carriers. Each carrier is modulated by 16-QAM to generate 4-bit symbols at 2400 symbols/second (2400 symbols/sec/carrier X 4 bits/symbol = 9600 bits/sec/carrier).

The long symbol duration (2400 symbols/sec = 416.7 µs/symbol) is designed to mitigate multi-path inter-symbol interference, since (ref: Document 5B/636 Annex 28) reflections in a 100 kHz maritime channel environment have been found to be contained primarily within the first 10.4 µs.  It is noted that further reflections were beyond this, some as far as 50 µs.  By comparison, note that AIS uses GMSK to generate 2-bit symbols at 4800 symbols/second (9600 bits/second) and that its excellent propagation characteristics have been proven in practice.

The modulation, coding and scrambling techniques described in EN 300 392-2 v.3.4.1 are combined to reduce the amplitude envelope PAPR (PAPR ≤ 10dB) to mitigate the RF power transmitter design difficulty. Both analog, e.g. Doherty Amplifier (DA), and digital, e.g. Envelope Tracking (ET) and Digital Pre-Distortion (DPD), design techniques for RF power amplifiers are available to provide better than 50% efficiency with this waveform. By comparison, the AIS power amplifiers used by ships and base stations are also approximately 50% efficient. A technical report describing these techniques and others for modern high efficiency power amplifiers with actual test results can be found at:

http://www.microwavejournal.com/articles/21965-modern-high-efficiency-amplifier-design-envelope-tracking-doherty-and-outphasing

Note that the analog design approach using Doherty Amplifiers provides efficiency over 50% and the original patent for this technology has expired. Solid state Doherty Amplifiers are currently in service in cellular terrestrial infrastructures which produce the range of power levels needed for shipborne VDES transceivers (12.5 Watts) and VDES base stations (50 Watts).

# Antenna options for VDES terrestrial stations

Commercially available antenna options for the VDES terrestrial stations are characterized in Figure 6 below. Since the shipborne antenna is required to receive the VDES satellite downlink at high elevation angles, the 0dBd (2.1dBi) option is selected. To achieve optimum satellite reception, this antenna should be mounted as high as possible, preferably on an extension pole, on the ship to minimize obstructions to the antenna’s view of the horizon. For the terrestrial VDES base station, the 6dBd (8dBi) option is selected. These two antennas are used in the propagation range predictions in Annex 2.

Figure 7 presents a mask for the receiving antenna gain as a function of elevation that would allow the received signal from satellite to be at constant power level at the receiver input for a wide range of elevation angles, taking into account the PFD constraints imposed on the VDE-SAT downlink (ref. Table 3 of Annex 1). Although this mask may not represent the antenna pattern associated with a commercially available antenna, it could serve as a guide for designing an antenna to enhance the satellite reception. The same mask is also applicable to the design of shipborne antenna for VDE terrestrial link due its high directivity in the horizontal direction. Annex 3 provides further rationale for the selection of this mask.

Figure 6

Antenna options for shipborne VDES stations

Figure 7

Mask for ‘Ideal’ Antenna

Gain in dBi

Elevation in degrees

# Presentation interface protocol

For VDES transceivers:

- Data to be transmitted by the VDES device should be input via the presentation interface;

- Data received by the VDES device should be output through the presentation interface.

- The formats and protocol used for data streams should be in accordance with IEC 61162.

# VHF data exchange by satellite

VHF data exchange by satellite should use the channels designated for satellite in Table 1 and should be in accordance with this Recommendation. This is further described below.

## General

### VHF data exchange system satellite component

The VHF data exchange VDE satellite component is an effective means to extend the VDES to areas outside of coastal VHF coverage. Hereafter, the satellite component is referred to as the VDE-SAT.

Satellite communications is able to deliver information in a broadcast, multicast or unicast mode to a large number of ships, i.e. efficiently addressing many ships using only minimal radio spectrum resources.

The VDE-SAT provides a communication channel that is complementary to the terrestrial components of the VDES system (i.e. coordinated with terrestrial VHF data exchange (VDE), application specific messages (ASM) and AIS functionalities and their supporting systems).

### Applications

Continuous exchanges with the maritime community will provide further insight into the priorities, Quality of Service (QoS), security, integrity and other requirements of future VDES services.

There is a large population of smaller size ships – which have no satellite communication equipment on board, but do have regular VHF/AIS reception equipment – that could benefit from the services mentioned above. This would be of particular benefit for vessel populations in areas with limited shore based infrastructure.

Using low-cost satellite reception technology, VDE-SAT can address a large population of ships and offer services for non-SOLAS vessels, fishing vessel, recreational users, life rafts, and even individuals in distress.

## Overall architecture, operational characteristics and assumptions

### Architecture

The VHF data exchange system architecture is shown in Figure 8 below. The VDE-SAT is composed of one or more satellites transmitting and receiving in the maritime VHF bands – this is the space segment.

Due to the frequencies used, it is likely that VDE-SAT will consist of low-earth orbiting (LEO) or medium-earth orbiting (MEO) satellites. VDE-SAT could also consist of hosted payload on spacecraft in such orbits.

The VDE-SAT user terminals may be integrated in ship-borne VDES equipment. This is called the user segment. These terminals could be integrated in the terrestrial VDE equipment along with ASM and AIS functionalities. Also VDE-SAT receive-only terminals can be considered: these would provide a very cost-effective means to disseminate maritime information to smaller ships outside terrestrial VHF coverage, for example in areas with limited shore based infrastructure.

There will be a ground segment which consists of one of more ground stations that will send and receive maritime information to/from ships for further processing or dissemination, via the space segment.  Communication between the coastal VDE station, maritime information provider, VDE-SAT ground station and feeder link is not part of the VDES architecture.

Figure 8

VHF data exchange-satellite component architecture

### Operational characteristics

The VDE-SAT should complement the VDE terrestrial in areas in which no terrestrial VDE coverage is available, i.e. at the high-seas.

The VDE-SAT should provide a downlink capability (i.e. allow to send information from a ground station to one or more ships). Note that VDE-SAT will likely use its specific unicast, multicast or broadcast capability which is inherent in a satellite downlink.

The VDE-SAT should provide an uplink capability (i.e. allow a ship to send information to the satellite, for further relaying to a ground station).

As VDE-SAT will be based on LEO or MEO satellite(s), provisions will need to be taken for the discontinuous contact that ships will have with individual satellites. Furthermore, if there are multiple VDE-SAT satellites or payload footprints that overlap, some coordination between them may be required.

It is proposed that VDE-SAT supports priority, pre-emption and precedence for different services; this could be mapped into different downlinks.

## Technical characteristics

### VHF data exchange-satellite channels and spectrum

The VDE-SAT downlink should be used for data downlink from the satellite to vessels in a broadcast, multicast or unicast manner. The VDE-SAT should also provide data uplink from vessels to satellites using one or several multiple-access schemes. The VHF data exchange system via satellite uses the channel allocation shown in Figure 9

Figure 9

VHF data exchange system channel allocation

#### SAT Downlink

The satellite downlink frequency spectrum consists of six 25 kHz channels (2024 to 2086). These channels may be bundled into one 150 kHz channel to reduce the guard band (needed due to the frequency Doppler shift of incoming signals from LEO satellites), increase the throughput, and more importantly, improve the power efficiency of the satellite power amplifier (avoiding multi-carrier transmission which typically requires a larger output back-off). (refer to section 6)

Due to the PFD limit imposed on the VDE-SAT downlink (as part of sharing the frequencies with land mobile), a certain level of redundancy (in the form of frame repetition, forward error correction or higher layer redundancy) is implemented in the VDE-SAT protocol in order to mitigate the error and enhance the data detection probability.

The VDE-SAT downlink signal also includes repeated known symbols (e.g. pilots, preamble, post-amble) to facilitate signal detection and synchronization as well as possible interference mitigation and channel estimation. In order to avoid unwanted in-band spectral lines, the data symbols are scrambled with a known sequence. The example in section 10.4.2 concludes that a downlink data rate of 240 kbps is possible.

The signal level generated by the satellite should be kept below the PFD mask limit (referred to the earth’s surface) specified in Table 3 below. Note that this is based on coordination with terrestrial VHF services and that the PFD level refers to the vertical component of radiation normal to the earth’s surface.

Table 3

Power flux-density (PFD) mask

θ°=earth-satellite elevation angle

PFDθ° (dBW/(m2*4 kHz))=-149+0.16*θ°                         0°≤θ<45°;-142+0.53*θ°-45°      45°≤θ<60°;-134+0.1*θ°-60°         60°≤θ≤90°.

This PFD mask is to ensure that there is no harmful interference caused by the satellite downlink on non-maritime terrestrial services sharing the same frequency (ensuring in-band carrier-to-interference requirements of terrestrial service receivers).

#### SAT 3 Uplink

The frequency spectrum corresponding to 6 lower VDE channels (starting from Channel 1024) are used for satellite data uplink. Compared to the AIS channels, and long range AIS, these 6 channels provide a significant data uplink capability via satellite.

The access scheme protocol for data uplink via satellite is designed to take into account the entire satellite field of view and to maximize the probability of message detections by avoiding message collisions.

### Rationale of channel allocation for VHF data exchange-satellite

The frequency plan for the entire VDES, as depicted in Figure 9 above, facilitates a realistic implementation of the proposed system in co-existence with, and complementing, the current AIS. The following points regarding the frequency plan are highlighted:

- The requirements for VDES concentrate the reception frequencies on board of the ship to a limited range of 250 kHz at the upper maritime VHF band. This provides an efficient implementation of VDES on-board receivers by narrowing the input filter bandwidth, reducing potential impairments due to other activities within the maritime VHF band.

- The VDE-SAT downlink shares the same frequency range as the terrestrial VDE and AIS. This allows sharing the same antenna as well as the receiver front-end design.

- Satellite and shore reception frequencies of shipborne VDE signals occupy the lower end of the VHF maritime band. This allows for a complementary service close to the shore and at the high sea while sharing the same spectrum. The frequency separation between the upper and lower spectra (with 4.6 MHz separation) provides an acceptable level of isolation between VDES receiving chain and the VDE ship-borne transmitters.

- The frequency separation between the uplink and downlink allows hosting VDE-SAT transmitter and receiver on the same satellite which allows for a more cost-effective satellite mission concepts (i.e. reduce number of satellites, improved efficiency and possible interactivity).

## Example VDES satellite implementation

The following example VDES satellite implementation fits the PFD angular mask and supports the requirements of this Recommendation.

### Determine the VDES satellite orbital characteristics

The following VDES satellite implementation is considered. The satellite orbital characteristics that are needed to support this application are determined as follows.

#### Determine the satellite’s orbit

The example VDES satellite employs a polar orbit at a height of 550 km above the surface of the earth. The velocity, acceleration and orbital period of the satellite are determined, given: Mearth = 5.98 x 1024 kg, Rearth = 6.37 x 106 m.

The satellite’s orbit and the known and unknown parameters are shown in Figure 10 below.

Figure 10

Satellite orbital characteristics

| Given/Known: R = Rearth + height = 6.92 x 106 m Mearth = 5.98x1024 kg G = 6.673 x 10-11 N m2/kg2; Unknown: v = ??? a = ??? T = ???. |  |
| --- | --- |

The radius of a satellite's orbit can be determined from the earth's radius and the height of the satellite above the earth. As shown in Figure 10, the radius of orbit for a satellite is equal to the sum of the earth's radius and the height above the earth. These two quantities are added to yield the orbital radius. The 550 km altitude is first converted to 0.550 x 106 m and then added to the radius of the earth.

Determine the velocity of the satellite

v = SQRT [ (G•MCentral ) / R ]

v = SQRT [ (6.673 x 10-11 N m2/kg2) • (5.98 x 1024 kg) / (6.92 x 106 m) ]

v = 7.594 x 103 m/s

Determine the acceleration of the satellite

| a = (G • Mcentral)/R2 |  |
| --- | --- |

a = (6.673 x 10-11 N m2/kg2) • (5.98 x 1024 kg) / (6.92 x 106 m)2

a = 8.333 m/s2

Determine the orbital period of the satellite

T = SQRT [(4 • pi2 • R3) / (G*Mcentral)]

T = SQRT [(4 • (3.1415)2 • (6.92 x 106 m)3) / (6.673 x 10-11 N m2/kg2) • (5.98x1024 kg) ]

T = 5725.7 s = 1.59 hrs

### VDES satellite antenna and downlink characteristics

A directional vertically polarized Yagi-Uda antenna is used for communicating with ships’ vertical antennas and also for conformance with the PFD angular mask.

#### Determine the earth’s rotation at the equator between each satellite orbit:

The period of the earth Te  is approximately 24 hours (86.4 x 103 s), the radius of the earth Re is 6.37 x 106 m and the circumference of the earth (distance around the equator) is Cearth  = 2 • (3.1415) • (6.37 x 106 m) = 40.0239 x 106 m. Therefore, in each pass of the satellite, the earth will have rotated at the equator by ROTequator = Cearth • T / Te   = 40.0239 x 106 m • 5725.7 s /86.4 x 103 s = 2.6524 x 106 m = 2652.4 km.

#### Determine the slant distance to the earth’s horizon:

The slant distance Ds from the satellite to the earth’s horizon is Ds = SQRT [(R2 – Re2] = SQRT [(6.92 x 106 m)2 – (6.37 x 106 m)2] = 2.7036 x 106 m = 2703.6 km.

Determine the slant downward tilt angle to the earth’s horizon:

#### The satellite’s downward tilt angle to the earth’s horizon is:

θd = 900 - sin-1 (Re / R) = 900 - sin-1(6.37 x 106 m / 6.92 x 106 m) = 900 - 670  = 23 degrees.

#### Determine the width of the antenna coverage path:

The example VDES satellite antenna pattern is shown in Figure 11 below. The beamwidth (+/- 3 dB) of the antenna is 80 degrees. The width of the satellite antenna’s coverage path is:

Wc = 2(Ds cos (900-θa/2))

Wc = 2 • 2.7036 x 106 m • cos (900-800/2) = 3.4757 x 106 m = 3475.6 km.

Note from 4.2.1 that since ROTequator = 2652.4 km, this antenna beamwidth (θa = 800) is sufficiently wide for contiguous earth coverage by one satellite every 24 hours. This vertically-polarized Yagi-Uda antenna is pointed in the forward direction with an optimized downward tilt angle to provide the vertical component of radiation for reception by ships’ vertical dipole antennas.

Figure 11

Example VDES satellite antenna pattern

#### Determine the maximum Doppler frequency shift (fd) between the satellite and ships in the satellite’s antenna coverage area

The maximum Doppler frequency shift (fd) between the satellite and a ship will occur when the relative velocity between them is a maximum, i.e., when the ship is situated on the satellite’s earth horizon. Note that the coverage for this satellite is only in the forward direction and that the satellite’s antenna pattern will cover ships in the range of 23 degrees (earth’s horizon) downward from the satellite’s velocity vector. Therefore, the maximum Doppler shift is fd (max) = fVDES (v/c) • cos θd = 162 x 106 (7.594 x 103)/(3 x 108) • cos 230 = 3775 Hz. The satellite transmitter frequency should be reduced by half of fd (max) to provide a range of +/- 1887.5 Hz in the coverage area.

Determine the optimum downward tilt angle for the satellite VDES antenna for coverage of ships in the forward direction

From the VDES satellite antenna characteristics in Figure 11 above, note that the response is flat to approximately 120. This supports an additional downward tilt of 120 below the horizon of 230 for an optimized total downward tilt angle of 35 degrees below the line that is tangent to the satellite’s orbital path. This provides a sufficient vertical radiation component for ships in the coverage area.

#### Consideration of the angular power flux-density(PFD) mask limits for transmission by the VDES satellite

The PFD angular mask (the maximum allowable PFD in dB(W/(m2*4kHz)) as a function of the elevation angle from the earth), is shown in Table 3 of Section 10.3.1.1. Note that the PFD mask at 00 (horizon) is -149 dB(W/(m2*4kHz)), at 450 elevation is -142 dB(W/(m2*4kHz)), at 600 elevation is -134 dB(W/(m2*4kHz)) and at 900 (overhead) is -131 dB(W/(m2*4kHz)). Note also that since the PFD mask level refers to the vertical component of radiation normal to the earth’s surface, the polarization loss (≈ 3 dB @ 450 elevation angle) based on the angular relationship between the vertical axis of the satellite antenna and the earth’s surface should be considered in the determination of the satellite VDES transmitter power.

#### Determine the PFD levels at elevations of 00, 100, 300, 600 and 900 when the PFD level at 450 elevation is set to -142 dB(W/(m2*4kHz))

This section confirms that the elevation angle of 450 is the CPA (closest point of approach) between the PFD mask and the actual radiated VDES space-earth downlink signal.

Calculations of the slant ranges and elevation angles note from the previous calculations that the slant range from the satellite earth horizon is 2703.6 km. The results of these calculations are shown in Table 2 below. Note that the “orbital angle” (the angle of rotation of the satellite’s orbit above the earth) is used as a reference for geometric calculations (angles and distances) and for time-keeping (elapsed time from the horizon to the point of rotation).

The slant ranges from the satellite to an earth station are determined from the law of cosines (c = SQRT (a2 +b2 +c2 -2ab cos (C)), where c = slant range, a = Re + h, b = Re and C = the satellite orbital angle. The calculations start with C = 230 (the angle to the horizon) and proceed to C = 00 (the directly above/below position), shown in Table 4.

To find the elevation angles, reference angles are determined from the inverse law of cosines (C = cos-1 ((a2 + b2 + c2)/(2ab)) where C = the reference angle between the slant range (line of observation) and the earth radius (line from the earth station to the center of the earth), a = slant range, b = earth radius and c = Re + h. The elevation angles for the earth stations are determined by subtracting 900 from the reference angles, also shown in Table 4 below.

#### Determine reference levels based on the 450 elevation angle

From Table 3, the slant range to the satellite at 450 elevation is 748.3 km and the PFD at 450 elevation is set to the mask limit of -142 dB(W/(m2*4kHz)). Since the relative angle of the satellite antenna (down-tilted by 350) in that direction is approximately (450 – 350) = 100, the gain of the satellite antenna in that direction, from Figure 7, is 8 dB. These values were used as the set point values (the 0 dB reference levels) to calculate the PFD levels for the other elevation angles.

#### Determine the PFD level for the elevation angle of 00

The slant range at 00 (horizon) is 2703.6 km, the satellite relative angle to the horizon is -230, the satellite antenna relative angle with a 350 down-tilt is (350-230) = 120 and the gain, from Figure 7, is 8 dB. Since the relative range loss is (20 log (748.3/2703.6) = -11.2 dB, the PFD at 00 is 11.2 dB below the 450 level (-142 – 11.2) = -153.2 dB(W/(m2*4kHz)) which is (-149 - (-153.2)) = 4.2 dB below the 00 mask limit.

#### Determine the PFD level for the elevation angle of 100

The slant range at 100 elevation is 1818.4 km, the satellite relative angle to the horizon is -230, the satellite antenna relative angle with a 350 down-tilt is (350-230-100) = 20 the gain, from Figure 7, is 8 dB (the same as the reference), the relative range loss is 20 log (748.3/1818.4) = -7.7 dB and thus the PFD at 100 is (-142 – 7.7) = -149.7 dB(W/(m2*4kHz)) which is 2.3 dB below the 100 mask limit of -147.4 dB(W/(m2*4kHz)).

#### Determine the PFD level for the elevation angle of 300

The slant range at 300 elevation is 993.5 km, the satellite relative angle to the horizon is -230, the satellite antenna relative angle with a 350 down-tilt is (350-300) = 50 the gain, from Figure 7, is 8 dB (the same as the reference), the relative range loss is 20 log (748.3/993.5) = -2.5 dB and thus the PFD at 300 is (-142 – 2.5) = -144.5 dB(W/(m2*4kHz)) which is 0.3 dB below the 100 mask limit of -144.2 dB(W/(m2*4kHz)).

#### Determine the PFD level for the elevation angle of 600

The slant range at 600 elevation is 632.7 km, the satellite relative angle to the horizon is -230, the satellite antenna relative angle with a 350 down-tilt is (350-600) = -180 the gain, from Figure 7, is 7.5 dB (0.5 dB below the reference), the relative range is 20 log (748.3/632.7) = +1.5 dB (1.5 dB above the reference) and thus the PFD at 600 is (-142 - 0.5 + 1.5) = -141.0 dB(W/(m2*4kHz)) which is 7.0 dB below the 600 mask limit of -134.0 dB(W/(m2*4kHz)).

#### Determine the PFD level for the elevation angle of 900

The slant range at 900 (overhead) is the satellite altitude of 550 km, the gain of the satellite antenna in that direction, from Figure 7, with a down-tilt of 35 degrees is the gain at (350-900) = -55 degrees is 2 dB (6 dB below the reference), the relative range factor is 20 log (748.3/550) = +2.7 dB (2.7 dB above the reference) and thus the PFD at 900 is (-142 -6 + 2.7) = -145.3 dB(W/(m2*4kHz)) which is 14.3 dB below the 900 mask limit of -131 dB(W/(m2*4kHz)).

The PFD values for elevation angles from 00 to 900 are shown in Table 4 below.

Table 4

PFD for various elevation angels

| Orbital angle (degrees) | Elapsed time from horizon (sec) | Slant range (km) | Reference angle (degrees) | Elevation angle (degrees) | PFD (actual/mask/margin, in dB(W/(m2*4kHz))) |
| --- | --- | --- | --- | --- | --- |
| 23 | 0 | 2703.6 | 90 | 0 | -153.2/-149/4.2 |
| 22 | 15.9 | 2592.7 | 90.5 | 0.5 | -152.8/-148.9/3.9 |
| 21 | 31.8 | 2481.6 | 91.0 | 1.0 | -152.4/-148.8/3.6 |
| 20 | 47.7 | 2370.5 | 93.2 | 3.2 | -152/-148.5/3.5 |
| 19 | 63.6 | 2259.6 | 94.4 | 4.4 | -151.6/-148.3/3.3 |
| 18 | 79.5 | 2148.8 | 95.6 | 5.6 | -151.2/-148.1/3.1 |
| 17 | 95.4 | 2038.3 | 97.0 | 7.0 | -150.7/-147.9/2.8 |
| 16 | 111.3 | 1928.1 | 98.4 | 8.4 | -150.2/-147.7/2.5 |
| 15 | 127.2 | 1818.4 | 100.0 | 10.0 | -149.7/-147.4/2.3 |
| 14 | 143.1 | 1709.2 | 101.6 | 11.6 | -149.2/-147.1/2.1 |
| 13 | 159.0 | 1600.6 | 103.5 | 13.5 | -148.6/-146.8/1.8 |
| 12 | 175.0 | 1493.0 | 105.5 | 15.5 | -148/-146.5/1.5 |
| 11 | 190.9 | 1386.5 | 107.8 | 17.8 | -147.4/-146.1/1.3 |
| 10 | 206.8 | 1281.4 | 110.3 | 20.3 | -146.7/-145.8/0.9 |
| 9 | 222.7 | 1178.1 | 113.2 | 23.2 | -145.9/-145.3/0.6 |
| 8 | 238.6 | 1077.3 | 116.6 | 26.6 | -145.2/-144.7/0.5 |
| 7.145 | 252.2 | 993.5 | 120.0 | 30.0 | -144.5/-144.2/0.3 |
| 7 | 254.5 | 979.6 | 120.6 | 30.6 | -144.3/-144.1/0.2 |
| 6 | 270.4 | 886.3 | 125.3 | 35.3 | -143.5/143.35/0.15 |
| 5 | 286.3 | 798.7 | 131.0 | 41.0 | -142.5/-142.4/0.1 |
| 4.38 | 296.1 | 748.3 | 135.0 | 45.0 | -142/-142/0 (reference) |
| 4 | 302.2 | 719.2 | 137.8 | 47.8 | -141.7/-140.5/1.2 |
| 3 | 318.1 | 650.6 | 146.2 | 56.2 | -141.5/-136.1/5.4 |
| 2.7 | 322.9 | 632.7 | 150.0 | 60.0 | -141/-134/7 |
| 2 | 334.0 | 596.8 | 156.1 | 66.1 | -141.8/-133.4/8.4 |
| 1 | 349.9 | 562.1 | 167.6 | 77.6 | -143.1/-132.2/10.9 |
| 0 | 365.8 | 550.0 | 180 | 90 | -145.3/-131/14.3 |

Notes for Table 4:

- When the PFD level is set to the mask limit of -142dB(W/(m2*4kHz))) at 450 elevation angle, the PFD levels at all other elevation angles are below the mask.

- The maximum PFD level is -141dB(W/(m2*4kHz)) at 600 elevation angle, which is 7 dB below the mask limit level of -134dB(W/(m2*4kHz)).

#### Consider the shipborne VDES antenna and receiver characteristics

The shipborne antenna and receiver characteristics are considered, along with the satellite radiated PFD levels, to determine the performance of the example VDES satellite downlink.

#### Specify the shipborne VDES antenna characteristics

The available shipborne antenna options are comprised of stacked vertical dipole elements of various lengths and gain values, were previously shown in Figure 6 in section 8. This analysis considers the 0 dBd antenna because it has the best performance for the elevation angles required for satellite detection.

#### Determine the shipborne VDES receiver characteristics

The shipborne VDES receiver characteristics and the coordination levels for the terrestrial service are considered, and the set of metrics in Table 5 below are used to determine a reference value of C/N (carrier-to-noise ratio) for the example shipborne VDES receiver.

Table 5

Metrics for considering ITU-R coordination levels and calculating C/N in a VDES Receiver

Power received (referred to the Rx antenna) by a shipboard VHF receiver (reference 25 kHz channel):

Power received (linear formula): Pr = GE²c²/480π²f², where

G = gain of a half-wavelength (λ/2) dipole antenna = 1.64 E = field strength = 4 x10exp-6 volts/ meter (4 µV/m = +12dBu) c = speed of light in free space = 3 x 10exp8 meters/second f = VDES downlink frequency = 161.9x10exp6 (161.9 MHz) λ = 1.852 meters (at 161.9 MHz) Pr = 19.02x10exp-15 watts = -137.2dBW = -107.2dBm

The logarithmic formula can also be used to calculate Pr (dBm): Pr (dBm) = 42.8 - 20logF + 20logE + G, where

G = antenna gain in dBi = 2.1dBi (2.1dB over isotropic) F = frequency in MHz = 161.9 Pr (dBm) = 42.8 – 44.1 – 108 + 2.1 = -107.2dBm (-137.2dBW) PFD = dB(E) -153.72 = 12-153.72 = -141.72dB(W/(m2*4kHz)) from a vertically-polarized source Ae = effective area for a dipole antenna = 0.13λ2 = 0.446m2 Pr (25 kHz channel) = PFD + 10logAe + 10log(25/4) = -141.7 -3.5 +8 = -137.2dBW = -107.2dBm Power received by a shipboard VDES receiver (reference 150 kHz channel): Noise floor in a 150 kHz bandwidth: kTB = 10log((1.38x10exp-23)(290)(150x10exp3))= -152.2dBW Rx carrier power (reference) in a 150 kHz bandwidth: C = 10log((19.02x10exp-15)(150/25))= -129.4dBW Applying adjustments for cable loss (2dB) and Rx noise figure (4dB), the C/N calculation follows: C/N (150 kHz bandwidth): C/Nref = (-129.4-2) – (-152.2 +4) = 16.8dB (Rx 0dBd antenna, 00 elevation) NOTE: These calculations serve to confirm the applicability of the metrics and reference levels.

#### Determine the values of C/N vs. elevation angle for the shipborne VDES receiver

Based on the C/N reference level (C/Nref) from Table 5, determine the C/N for the PFD values and elevation angles in Table 4, taking into account the shipborne antenna angular gain values for the 0 dBd antenna in Figure 6. For this antenna, Ga = 2.1 dBi at 00 elevation angle.

C/N = C/Nref - (-142 – PFD – (2.1- Ga)), where Ga = shipborne antenna gain at the elevation angle.

- At 00 elevation angle, C/N = 16.8 – (-142 – (-153.2) - (2.1-2.1)) = 5.6 dB.

- At 100 elevation angle, C/N = 16.8 – (-142 – (-149.7) - (2.1-1.9)) = 8.9 dB.

- At 300 elevation angle, C/N = 16.8 – (-142 - (-144.5) - (2.1-(-0.3)) = 11.9 dB.

- At 450 elevation angle, C/N = 16.8 – (-142 – (-142) - (2.1-(-3.5)) = 11.2 dB.

- At 600 elevation angle, C/N = 16.8 – (-142 – (-141) - (2.1 – (-7.6)) = 8.1 dB.

- At 900 elevation angle, C/N = 16.8 – (-142 - (-145.3) - (2.1-(-11.6)) = -0.2 dB.

The C/N values for elevation angles from 00 to 900 are shown in Table 6 below.

Table 6

C/N and PFD for various elevation angels

| Orbital angle (degrees) | Elapsed time from horizon (sec) | Slant range (km) | Elevation angle (degrees) | PFD (actual/mask/margin, in dB(W/(m2*4kHz))) | C/N ship receiver (dB) |
| --- | --- | --- | --- | --- | --- |
| 23 | 0 | 2703.6 | 0 | -153.2/-149/4.2 | 5.6 |
| 22 | 15.9 | 2592.7 | 0.5 | -152.8/-148.9/3.9 | 6 |
| 21 | 31.8 | 2481.6 | 1.0 | -152.4/-148.8/3.6 | 6.4 |
| 20 | 47.7 | 2370.5 | 3.2 | -152/-148.5/3.5 | 6.8 |
| 19 | 63.6 | 2259.6 | 4.4 | -151.6/-148.3/3.3 | 7.2 |
| 18 | 79.5 | 2148.8 | 5.6 | -151.2/-148.1/3.1 | 7.6 |
| 17 | 95.4 | 2038.3 | 7.0 | -150.7/-147.9/2.8 | 8 |
| 16 | 111.3 | 1928.1 | 8.4 | -150.2/-147.7/2.5 | 8.5 |
| 15 | 127.2 | 1818.4 | 10.0 | -149.7/-147.4/2.3 | 8.9 |
| 14 | 143.1 | 1709.2 | 11.6 | -149.2/-147.1/2.1 | 9.4 |
| 13 | 159.0 | 1600.6 | 13.5 | -148.6/-146.8/1.8 | 9.7 |
| 12 | 175.0 | 1493.0 | 15.5 | -148/-146.5/1.5 | 10.2 |
| 11 | 190.9 | 1386.5 | 17.8 | -147.4/-146.1/1.3 | 10.8 |
| 10 | 206.8 | 1281.4 | 20.3 | -146.7/-145.8/0.9 | 10.9 |
| 9 | 222.7 | 1178.1 | 23.2 | -145.9/-145.3/0.6 | 11.5 |
| 8 | 238.6 | 1077.3 | 26.6 | -145.2/-144.7/0.5 | 11.8 |
| 7.145 | 252.2 | 993.5 | 30.0 | -144.5/-144.2/0.3 | 11.9 |
| 7 | 254.5 | 979.6 | 30.6 | -144.3/-144.1/0.2 | 11.9 |
| 6 | 270.4 | 886.3 | 35.3 | -143.5/143.35/0.15 | 11.9 |
| 5 | 286.3 | 798.7 | 41.0 | -142.5/-142.4/0.1 | 11.7 |
| 4.38 | 296.1 | 748.3 | 45.0 | -142/-142/0 (reference) | 11.2 |
| 4 | 302.2 | 719.2 | 47.8 | -141.7/-140.5/1.2 | 11.0 |
| 3 | 318.1 | 650.6 | 56.2 | -141.5/-136.1/5.4 | 8.6 |
| 2.7 | 322.9 | 632.7 | 60.0 | -141/-134/7 | 8.1 |
| 2 | 334.0 | 596.8 | 66.1 | -141.8/-133.4/8.4 | 4.4 |
| 1 | 349.9 | 562.1 | 77.6 | -143.1/-132.2/10.9 | 2.4 |
| 0 | 365.8 | 550.0 | 90 | -145.3/-131/14.3 | -0.2 |

#### Determine the data rate for elevation angles 00 to 600 using the DVB-S standards

The DVB-S (digital video broadcast by satellite) standards are designed to provide the maximum utilization of the available bandwidth in a low-to-moderate C/N ratio. The spectral efficiencies for DVB-S2X and DVB-S2 are shown in Figure 12 below.

DVB-S2X is based on the well-established DVB-S2 specification. It uses the proven and powerful LDPC Forward Error Correction (FEC) scheme in combination with BCH FEC as outer code and introduces the following additional elements:

• Smaller roll-off options of 5% and 10% (plus 20%, 25% and 35% in DVB-S2)

• A finer gradation and extension of number of modulation and coding modes

• New constellation options for linear and non-linear channels

• Additional scrambling options for critical co-channel interference situations

• Channel bonding of up to 3 channels

• Very Low SNR operation support down to -10 dB SNR

• Super-frame option

Figure 12

Performance of DVB-S2X and DVB-S2

#### Performance conclusion

From Figure 12 above, it is concluded that the DVB-S2X standard transmission applied to the VDES satellite downlink provides spectral efficiency of 1.6bps/Hz and a date-rate of 240 kbps in a 150 kHz bandwidth for C/N ≥ 5 dB, which, from Table 6, includes elevation angles from 00 to 600.

# Summary of operational capability and performance

This Recommendation provides the following operational capability and performance:

- Protection of AIS

- Relief of AIS VDL congestion

- Raw ASM data transfer at 28.8 kbps

- Raw VDE data transfer ship-to-ship, ship-to-shore and shore-to-ship at 307.2 kbps

- Raw VDE satellite data transfer up to 240 kbps

- VDE satellite downlink that satisfies the PFD mask requirements

- VDE shore-to-ship and ship-to-shore service to 85km (46NM)

- Channel access and sharing schemes that organize the links and mitigate conflicts

- Full VDES satellite and terrestrial functionality from a single shipborne antenna

ANNEX 2

Propagation range predictions for VDES terrestrial links

# Introduction

This is an informative annex.  The excellent propagation characteristics of AIS are well established and appreciated. It is expected that the ASM will have similar performance to AIS. The propagation range predictions for the 100 kHz VDE ship-to-shore and shore-to-ship links follow below.

# Ship-to-shore application

## Basis for the coverage assessment

This coverage assessment is based on Recommendation ITU-R P.1546-4 (assuming no ducting), taking into account the antenna height and the seawater propagation path:

Height of antenna (Base Station):	75 meters (see graph for various heights)

Transmitter power for ships:		12.5 Watts

Tx ships antenna gain:		2dBi (0dBd)

Rx shore antenna gain:		8dBi (6dBd)

Pr:					-103dBm (VDE shore station sensitivity)

## Purpose for use of the Recommendation ITU-R P.1546-4 propagation curve

Recommendation ITU-R P.1546-4 prescribes the use of the propagation curves ( §3 from Annex 5 and Figure 4 (Figures 13 and 14 of this Annex) from Annex 1, see below), assuming no ducting and a smooth earth/sea surface. This analysis may be used as a reference point for field test measurements that usually include some ducting, depending on weather, atmospheric conditions, and other factors.

## Determination of transmitting/base antenna height, h1

Recommendation ITU-R P.1546-4 specifies (§3 of Annex 5) the transmitting/base antenna height, h1, to be used in calculation depending on the type and length of the path. For sea paths h1 is the height of the antenna above mean sea level (AMSL); for land paths h1 is the height above average terrain (HAAT).

## Determination of the minimum field strength (sensitivity threshold) at the VDE Base receiving site

For ship-to-shore:

Power received (linear formula): Pr = Gr Er²c²/480π²f²

Rearranged: Er = √ (480π²f² Pr /Gr c2), where

Er = field strength in volts/meter

Gr = gain of receiving antenna = 6.3 = 8dBi

c = speed of light in free space = 3 x 108 meters/second

f = VDE ship-to-shore frequency = 1.57 x 108 (157 MHz)

Pr = 5x10-14 watts = -133dBW = -103dBm

Thus,

Er = 3.21 x 10-6 = 3.21 µV/m = +10.1dB µV/m

The logarithmic formula can also be used to calculate Pr (dBm):

Pr (dBm) = 42.8 - 20logF + 20logE + G, where

G = antenna gain in dBi = 8dBi

F = frequency in MHz = 157

Pr (dBm) = 42.8 – 43.9 – 109.9 + 8 = -103dBm (-133dBW)

## Determine the range to the +10.1dBu (-103dBm) coverage limit for a seawater propagation path

Calculate the effective radiated power:

Ps = Pt + G

Pt = 10 log 12.5 – 30 = -19dBk (19dB below 1 kW)

G = 2dBi = +0dBd (0dB over a dipole)

Thus Ps = -19 +0 = -19dBk ERP

Fe = F – Ps  (vertical scale reference for the propagation graph in Figure 4 of Recommendation ITU-R P.1546-4, Figure 13 of this Annex)

F = +10.1dBu

Ps = -19dBk

Thus Fe = 10.1 – (-19) = +29.1dB

## Determine the seaward ship-to-shore coverage range from Figure 13:

The +10.1dBu (-103dBm) range is 85km, which is 46NM (use h1 = 75m).

## Determine the RSSI values for various other ranges

The reference point: RSSI = -103dBm at a range of 85km (46NM) is determined above. For other ranges, the RSSI value is determined from the propagation curve (Figure 13) for the assumed antenna height of 75m. RSSI values in 10dB increments above the sensitivity threshold are shown in Table 7 below.

Table 7

VDE base station RSSI value vs. distance ship-to-shore

| -103dBm | 85km (46NM) |
| --- | --- |
| -93dBm | 60km |
| -83dBm | 40km |
| -73dBm | 25km |
| -63dBm | 15km |
| -53dBm | 8km |
| -43dBm | 4.5km |

Figure 13

Figure 1

+29.1 dB reference -103dBm

85km (46NM) ship-to-shore Coverage range for -103dBm shore antenna height = 75m

# Shore-to-Ship Application

## Basis for the coverage assessment

Referring to section 2 above, consider the reverse direction, shore-to-ship, signal levels at the ship receiving site, the shore transmitter power of 50 Watts and the shore-to-ship frequency of 162 MHz:

Height of antenna (VDES Base Station):	75 meters (see graph for various heights)

Transmitter power of VDES on shore:	50 Watts (at base of shore antenna)

Tx shore antenna gain:			8dBi (6dBd)

Rx ships antenna gain:			2dBi (0dBd)

Pr:						-98dBm (VDE ship station sensitivity)

### Determination of the minimum field strength (sensitivity threshold) at the VDE ship receiving site

For shore-to-ship:

Power received (linear formula): Pr = Gr Er²c²/480π²f²

Rearranged: Er = √ (480π²f² Pr /Gr c2), where

Er = field strength in volts/meter

Gr = gain of receiving antenna = 1.62 = 2.1dBi

c = speed of light in free space = 3 x 108 meters/second

f = VDE shore-to-ship frequency = 1.62 x 108 (162 MHz)

Pr = 1.58x10-13 watts = -128dBW = -98dBm

Thus,

Er = 11.61 x 10-6 = 11.61 µV/m = +21.3dB µV/m

The logarithmic formula can also be used to calculate Pr (dBm):

Pr (dBm) = 42.8 - 20logF + 20logE + G, where

G = antenna gain in dBi = 2.1dBi

F = frequency in MHz = 162

Pr (dBm) = 42.8 – 44.1 – 98.7 + 2.1 = -98dBm (-128dBW)

### Determine the range to the +21.3dBu (-98dBm) coverage limit for a seawater propagation path

Calculate the effective radiated power:

Ps = Pt + G

Pt = 10 log 50 – 30 = -13dBk (13dB below 1 kW)

G = 8dBi = +6dBd (6dB over a dipole)

Thus Ps = -13 +6 = -7dBk ERP

Fe = F – Ps  (vertical scale reference for the propagation graph in Figure 4 of Recommendation ITU-R P.1546-4, Figure 2 of this Annex)

F = +21.3dBu

Ps = -7dBk

Thus Fe = 21.3 – (-7) = +28.3dB

Note that since this value of Fe is within 1dB of the value calculated in Section 2.5 because the reduced sensitivity of the ship station is compensated by the higher power and antenna gain of the shore base station.

### Determine the seaward shore-to-ship coverage range from Figure14:

The +28.3dBu (-98dBm) range is 85km, which is 46NM (use h1 = 75m). This is the same as the ship-to-shore coverage range, an ideal balanced two-way coverage, which confirms the proposed choices of antennas and transmitter power values for the shipborne and shore VDES stations.

### Determine the RSSI values for various other ranges

The reference point: RSSI = -98dBm at a range of 85km (46NM) is determined in 2.6 above. For other ranges, the RSSI value is determined from the propagation curve (Figure 14) for the assumed antenna height of 75m. RSSI values in 10dB steps above and below the -98dBm threshold sensitivity for the shipborne VDE receiver are shown in Table 8 below.

Table 8

VDE ship station RSSI value vs. distance shore-to-ship

| -118dBm | 170km |
| --- | --- |
| -108dBm | 130km |
| -98dBm | 85km (46NM) |
| -88dBm | 60km |
| -78dBm | 40km |

Figure 14

Figure 2

85km (46NM) shore-to-ship   Coverage range for -98dBm shore antenna height = 75m

+28.3 dB reference -107dBm

Annex 3

Example of VDE-SAT Downlink  Implementation and Analysis

- Introduction

This is an informative annex providing an example of implementing the VDE-SAT downlink component and presenting performance results.

- VDE Satellite Orbital Characteristics

The spacecraft flies in a circular orbit of 600km and 68° inclination compliant with orbital debris regulations and safe de-orbiting of the spacecraft after its lifetime. The satellite counts with attitude control mechanisms to guarantee a stable antenna pointing in the nadir direction (i.e. satellite to Earth).

Under these assumptions Figure 15 shows the elevation (left axis) of the spacecraft as a function of time as seen by a ground terminal during an overhead pass. The right axis of the same figure depicts the signal delay.

Figure 15.

Satellite Elevation and delay for the selected orbit as a function of time

Elevation in degrees

Delay in seconds

Time

Figure 15 shows that the satellite is just over 4 minutes above 30° elevation, thus 9 minutes under 30° elevation from acquisition-of-signal (AOS) to loss-of-signal (LOS) for a pass duration of about 13 minutes. The roundtrip delay varies from 19 ms at AOS down to 4ms at Zenith (i.e. 90° elevation). During that pass the Doppler shift varies from -3.73kHz to +3.73kHz and the Doppler rate reaches 47Hz/s at Zenith.

Figure 16

Pass elevation scheme for selected orbit over 24 hours.

Figure 16 illustrates the satellite elevation as a function of time, as seen by a ground terminal at a fixed location in a 24 hour period. As shown the contact periods are short and low. Depending on the latitude, the duration and the number of contact periods will vary. (distance is provided in km)

Figure 17

Satellite Field of View

Figure 17 presents the satellite field of view. A wide geographical area is covered by the satellite field of view at any given point of the orbit. For this area, the average instantaneous ship count is 22000 respectively as shown in Figure 18. The ship count is based on combined received terrestrial and satellite data for AIS class A.

Figure 18

Field of view case for ship instantaneous number

Figure 18 is indicative of the AIS received by terrestrial stations is displayed in blue while AIS received by satellite is displayed in red.

  - VDE Satellite Downlink Characteristics

The power flux density mask to be respected is (as also presented in Table 3 of Annex 1).

PFDθ° (dBW/(m2*4 kHz))=-149+0.16*θ°                         0°≤θ<45°;-142+0.53*θ°-45°      45°≤θ<60°;-134+0.1*θ°-60°         60°≤θ≤90°.

which translates to:

Figure 19

Elevation in degrees

Power flux density mask.

Power in dBm

Figure 19 depicts the PFD mask in dBm as a function of elevation in a reference bandwidth of 4 kHz and in 100 kHz bandwidth.

The corresponding e.i.r.p. mask seen by the satellite corresponds to a transformed version of the PFD mask dictated by the Earth-satellite geometry.  Figure 20 shows the e.i.r.p. mask which is symmetric around the nadir direction (90° angle in the figure).

Figure 20

Satellite EIRP mask.

Elevation in degrees

Power in dB

Assuming a circularly polarized downlink signal from the satellite meeting the e.i.r.p. mask in Figure 20, then the PFD in 100 kHz seen in an overhead pass by a ground terminal is shown as a violet curve in Figure 21.  In this figure the signal power of a nearby ship (shown in yellow) is also presented as a benchmark reference.  The green line represents the realization of an antenna on the satellite compliant with the e.i.r.p. mask.

Figure 21

Rx carrier input for a 0 dB gain antenna. Iso and compensated sat TX antenna + nearby ship (for the sake of completeness, not relevant for this example)

  - VDE Satellite Receiver Characteristics

On the receiver side, the ship’s system temperature is considered to be between 630°K (noise figure of 3dB and 2dB of cable loss) and 1500°K. Variations can occur, but it is not expected that the system temperature falls below roughly 900°K in a standard installation. The system temperature accounts for the noise source integrated in the antenna patterns. Some onboard ‘industrial’ noise is yet to be added, but will be ignored for the remainder of the document.

  - ‘Ideal’ Receiving antenna

For the sake of completeness, the receiver antenna mask that would allow the received signal to be at constant power level at the receiver input is calculated and shown as a function of elevation angle in Figure 22.

Figure 22

"Ideal" RX antenna mask, Zenith is 90°

Gain in dBi

Elevation in degrees

Figure 23

Received carrier power for a receiver with an “ideal” antenna.

Elevation in degrees

Gain in dBi

Signal level in dBm

Figure 23 shows the received signal power in dBm at the input of a receiver with the “ideal” receiving antenna as a function of elevation. The link analysis is computed using professional commercial software tools for satellite communications that account for the signal propagation impairments. The software tool however, does not account for possible loss of power strength at very low elevation (<1°). The power loss could be as high as 6 dB due to reflecting surface of seawater, mainly in circular or horizontal polarizations.  It is worth noting that the signal power at the receiver input is around -101dBm, and this is 3dB lower than the ITU-R M.1842 recommended sensitivity for 16-QAM for ship stations.

Figure 24

Eb/N0 compensated patterns for ‘ideal’ antenna

Eb/N0 in dB

Figure 24 shows the corresponding Eb/N0 observed for the 100 kHz carrier in an overhead pass for the ‘ideal’ antenna.

  - Realistic Receiving antenna

Four different antennas are considered:

- The 0dBd ITU-R 1336 antenna pattern and vertical polarization (antenna 1)

- A  1.25 λ vertical antenna  (commercially available antenna, computed pattern when mounted on the top of the bridge a 200 m long tanker), vertical polarization (antenna 2).

- A satellite dedicated Turnstile, with Right Hand Circular Polarization (RHCP) (antenna 3)

- A hemispherical 0dBi gain antenna, vertical polarization (antenna 4).

Using professional software tools for satellite communications, simulations have been carried out to determine the carrier power level at the receiver input and to determine the Eb/N0 in the following cases:

-	Overhead pass

-	Side pass

-	Very low pass

Results corresponding to each scenario are reported in the following sections.

### Overhead pass

Figure 25

Overhead pass, Carrier level at receiver input

Antenna 1

Antenna 2

Antenna 3

Antenna 4

Figure 26

Overhead pass, Eb/N0 at demodulator input

Antenna 1

Antenna 2

Antenna 3

Antenna 4

### Side pass

Consider a 16° elevation pass, the signal power and corresponding signal quality measured in Eb/N0 are presented in the following figures. Due to the variation of the signal strength at the receiver over time (due to the change of elevation and distance), the signal may fall below the detection threshold.

The use of highly robust waveform (as a combination of modulation, coding and frame structure) can potentially improve the performance at the expense of reduced throughput.

Figure 27

Carrier level at receiver input, Side pass

Antenna 1

Antenna 2

Antenna 3

Antenna 4

Figure 28

Eb/N0 at demodulator input, side pass

Antenna 1

Antenna 2

Antenna 3

Antenna 4

### Very Low side pass

Results for a very low side pass (below 5° elevation) are presented in figures below.

Figure 29

Carrier input at receiver input, very low side pass

Antenna 1

Antenna 2

Antenna 3

Antenna 4

Figure 30

Eb/N0 at demodulator input, very low elevation side pass.

Antenna 1

Antenna 2

Antenna 3

Antenna 4

  - Waveform choice

As shown in previous sections, for a realistic antenna the signal to noise ratio at the input of the receiver can vary considerably as a function of elevation angle. The choice of the waveform modulation, coding and frame structure has a significant impact on the link throughput and its availability.

The decision on continuous versus intermittent transmission of the signal will impact the acquisition, tracking and the overall performance (bit rate, probability of error, etc.) of the VDE satellite broadcasting. At the system level, a time slot-based transmission (time division) may increase the complexity of the satellite-terrestrial system interactions and reduce the overall efficiency. However, the coexistence of VDE broadcasting and terrestrial shore-to-ship or ship-to-ship may also impact the detection performance of the terrestrial signal.

The choice of modulation scheme has an impact on the efficiency of the power amplifier on board of the satellite. The use of (quasi-) constant envelope reduces the peak to average power ratio and allows the transmitter to operate at a more power efficient mode with less signal distortion.

In order to facilitate synchronisation and signal detection at the receiver, the use of known symbols (as pilot or preamble) is essential as part of the air interface definition.

The use of data sequence randomisation (scrambling) facilitates the synchronisation and mitigates spectral abnormality.

A system capability to allow more than one coding rate (and modulation scheme) may provide more flexibility in the system dimensioning and service availability.

There are a number of existing open standards with air interface specifications, such as Digital Video Broadcasting via satellite DVB-S2x , DVB-SH and DVB-RCS2,  that offer mature technical solutions as a starting point for such design trade-offs. The performance characteristics of DVB-RCS2 waveforms are reported in Table 1. Figure 31 presents the spectral efficiency (information bits/symbol) as a function of Es/N0 for these waveforms.

Note - DVB-RCS2 reference: ETSI TS 101 545-1 V1.2.1 (2014-04) available at: http://www.etsi.org/deliver/etsi_ts/101500_101599/10154501/01.02.01_60/ts_10154501v010201p.pdf

Table 9

Waveform Efficiency in AWGN Channel

| Frame Size (symbols) | Guard (symbols) | Payload (bits) | Efficiency (Bits/Symbol) | Es/N0 @ PER=10-5 |
| --- | --- | --- | --- | --- |
| 266 | 4 | 408 | 1.51 | 7.3 |
| 266 | 4 | 440 | 1.63 | 8.71 |
| 266 | 4 | 496 | 1.84 | 10.04 |
| 266 | 4 | 552 | 2.04 | 11.59 |
| 266 | 4 | 672 | 2.49 | 11.73 |
| 266 | 4 | 744 | 2.76 | 13.18 |
| 536 | 4 | 304 | 0.56 | 0.22 |
| 536 | 4 | 472 | 0.87 | 2.34 |
| 536 | 4 | 680 | 1.26 | 4.29 |
| 536 | 4 | 768 | 1.42 | 5.36 |
| 536 | 4 | 864 | 1.60 | 6.68 |
| 536 | 4 | 920 | 1.70 | 8.08 |
| 536 | 4 | 1040 | 1.93 | 9.31 |
| 536 | 4 | 1152 | 2.13 | 10.85 |
| 536 | 4 | 1400 | 2.59 | 11.17 |
| 536 | 4 | 1552 | 2.87 | 12.56 |
| 1616 | 4 | 984 | 0.61 | -0.51 |
| 1616 | 4 | 1504 | 0.93 | 1.71 |
| 1616 | 4 | 2112 | 1.30 | 3.69 |
| 1616 | 4 | 2384 | 1.47 | 4.73 |
| 1616 | 4 | 2664 | 1.64 | 5.94 |
| 1616 | 4 | 2840 | 1.75 | 7.49 |
| 1616 | 4 | 3200 | 1.98 | 8.77 |
| 1616 | 4 | 3552 | 2.19 | 10.23 |
| 1616 | 4 | 4312 | 2.66 | 10.72 |
| 1616 | 4 | 4792 | 2.96 | 12.04 |
| 3236 | 4 | 984 | 0.30 | -3.52 |
| 3236 | 4 | 1504 | 0.46 | -1.3 |

Figure 31

Spectral efficiency of DVB-RCS2 waveform

____________________

*	This Recommendation should be brought to the attention of the International Maritime Organization (IMO), the International Civil Aviation Organization (ICAO), the International Association of Marine Aids to Navigation and Lighthouse Authorities (IALA), the International Electrotechnical Commission (IEC) and the Comité International Radio Maritime (CIRM).

1 Nautical mile = 1 852 metres

235.9 Nautical miles = 436 886.8 metres; 120 Nautical miles = 222 240 metres

The following example is compliant with the requirement:

Sample the RF signal strength at a rate >1 kHz, average the samples over a sliding 20 ms period and over a 4 s interval determine the minimum period value. Maintain a history of 15 such intervals. The minimum of all 15 intervals is the background level. Add a fixed 10 dB offset to give the CS detection threshold.

1 Nautical mile = 1 852 metres

20 Nautical miles = 37 040 metres; 200 Nautical miles = 370 400 metres

1 Nautical mile = 1 852 metres

500 Nautical miles = 926 000 metres

1 Nautical mile = 1 852 metres

120 Nautical miles = 222 240 metres

A base station report (Message 4) in conjunction with a data link management message (Message 20) with the same base station ID (MMSI) must be received by the mobile station so that it can determine its distance from the transmitting base station.