ENAV16	Input paper

Agenda item	11

Task Number	2.1.2

Author(s) / Submitter(s)	European GNSS Agency (GSA)

# Proposal for using EGNOS as a DGNSS corrections source for broadcast over AIS / VDES

# Summary

1.1	Purpose

This paper proposes Satellite Based Augmentation Systems (SBAS) as a potential source of DGNSS corrections for broadcast over AIS and its future extension VHF Data Exchange System (VDES).

1.2	Background

DGNSS is an essential component of the e-Navigation system. It has the potential to meet most accuracy requirements within e-Navigation, and to provide the integrity information which is lacking to GNSS while being vital for safety applications.

The IALA Beacon DGNSS system has coverage limitations which could be overcome or mitigated by broadcasting corrections over different transmission means such as AIS/VDES.  DGNSS corrections currently are computed locally at a reference station (RS), with fully redundant equipment.

SBAS can provide an alternative source for such corrections that can be accessed either through the SBAS SIS or through the Internet. Furthermore SBAS corrections are immune to possible local (RS) disturbances. They consequently have the potential to become at least the secondary source of corrections for any DGNSS reference station and to be part of a more robust, truly redundant design.

2	ACTION REQUESTED

The Committee is invited to consider the information provided in the Annex, related to a Call for interest for maritime/river authorities in a pilot project, for its distribution within IALA members.

ANNEX

CALL FOR INTEREST FOR MARITIME / RIVER AUTHORITIES

PILOT PROJECT: EGNOS over AIS/VDES

The European GNSS Agency (GSA) is launching this call to know your interest in participating to potential demonstration actions detailed in the proposal below, for the use of EGNOS over AIS/VDES to complement your IALA DGNSS infrastructure.

Background

GSA is fostering the use of EGNOS in maritime as a means to complement the current IALA DGNSS infrastructure deployed in your country.

EGNOS (European Geostationary Navigation Overlay Service) is the European satellite-based augmentation service (SBAS) that provides corrections to GPS (e.g. ionosphere delay, clocks errors) increasing the accuracy of the position and informing about its reliability (i.e. so-called “integrity” information). EGNOS corrections can be received from Geostationary satellites (i.e. signal in space) or from internet using the EGNOS Data Access Service (EDAS).

So far, several studies (EGNOS Tran, Galewat, Maruse, Margal, etc) have proved that EGNOS corrections can be also broadcasted via the AIS shore-based stations. Re-transmission of EGNOS corrections received from internet (EDAS) using AIS stations (and in the future VDES) is foreseen to be used in areas where EGNOS Geo Satellites may not be visible (e.g. shadowed inland waterways, High latitudes in Arctic circle) and for users that do not have an EGNOS receiver on-board.

Do not use the system!

If HPE>HAL,

DGNSS corrections

Pilot project

GSA is analysing the possibility to launch a pilot project to show the added value of transmitting EGNOS corrections using EDAS at the AIS shore stations. In particular, it will represent a back-up to the current IALA DGNSS infrastructure, improving as well the accuracy and enabling a local integrity check.

The objective of the project is to show that the use of EDAS over AIS (and VDES in the future) is a cost-effective solution implemented at ground infrastructure level that brings to the mariners a complementary service to increase their safety at sea without the need of any investment from their side in new receivers in the short time. Technical and Cost-Benefit Analysis will be prepared for Maritime Authorities/End Users to show the benefits of EGNOS.

The outcomes of the pilot project will be used to support the definition of VDES (VHF Data Exchange System) at IALA eNav Committee, in such a way that it can be used as a means of transmission for EGNOS data.

Request

Maritime and River Authorities interested in collaborating in such a pilot project by offering an AIS station for the tests are requested to send an email to market@gsa.europa.eu before 15 May 2015.

AIS: Automatic Identification System

VDES: VHF Data Exchange System which represents an evolution of the current AIS.