ENAV16	Input paper

Agenda item	13

Task Number	3.1.5

Author(s) / Submitter(s)	European GNSS Agency

# Proposal for using EGNOS as a DGNSS corrections source for broadcast over radio beacons.

# Summary

1.1	Purpose

This paper proposes Satellite Based Augmentation Systems (SBAS) as a potential source of DGNSS corrections for broadcast over the IALA Beacon DGNSS system.

1.2	Background

IALA recommendation R-135 on Future DGNSS includes among others the following statements:

- Members should consider re-engineering their DGNSS to ensure required levels of service.

- The results of investigations of alternative development routes for IALA DGNSS e.g. software RSIM, VRS and SBAS integration should be considered […]

- The merit of pre-broadcast integrity should be assessed […]

One key concept in IALA proposals for a World Wide Radio Navigation Plan (WWRNP) in support of e-Navigation is the possibility of separating the generation of correction data from the means of transmission.

EGNOS can currently provide a cost-effective alternative source for L1 DGPS corrections that can be accessed either through the SIS or through the Internet (EDAS service). Furthermore EGNOS planned evolutions will extend this capability to Galileo + GPS, L1 + L5 corrections at minimal or no cost for the DGNSS stations, depending on the system architecture.

2	ACTION REQUESTED

The Committee is invited to consider the information provided in the Annex, related to a Call for interest for maritime/river authorities in a pilot project, for its distribution within IALA members.

CALL FOR INTEREST FOR MARITIME / RIVER AUTHORITIES

PILOT PROJECT: EGNOS over IALA Beacons - An alternative source of corrections for DGNSS

The European GNSS Agency (GSA) is launching this call to know your interest in participating to potential demonstration actions detailed in the proposal below, for the use of EGNOS corrections over Maritime Radiobeacon to complement or back-up your IALA DGNSS infrastructure.

## Background

GSA is fostering the use of EGNOS in maritime as a means to complement the current IALA DGNSS infrastructure deployed in your country.

EGNOS (European Geostationary Navigation Overlay Service) is the European satellite-based augmentation service (SBAS) that provides corrections to GPS (e.g. ionosphere delay, clocks errors) increasing the accuracy of the position and informing about its reliability (i.e. so-called “integrity” information). EGNOS corrections can be received from Geostationary satellites (i.e. signal in space) or from internet using the EGNOS Data Access Service (EDAS).

Several studies have proven that EGNOS corrections can be distributed via other communication links than the SIS. Indeed, the EDAS service itself is an example of such alternate method. 
Using EGNOS corrections received from the SIS or from internet (EDAS) is foreseen to be a cost effective alternative / back-up to generating locally these corrections in a Reference Station, thus providing a truly independent second source and possibly allowing more robust and redundant design for IALA DGNSS stations.

## Pilot project

GSA is analysing the possibility to launch a pilot project to show the added value of transmitting EGNOS corrections using EDAS at the IALA DGNSS stations. In particular, it will represent a back-up to the current dual RS/IM architecture and an important contribution to the pre-broadcast integrity monitoring thanks to EGNOS “built in” integrity.

The objective of the project is to show that the use of EDAS is a cost-effective alternative to the current reference station design implemented in IALA DGNSS stations, fully compatible with the performance requirements of this system. Furthermore, by offering an independent second source of corrections and of integrity information, it paves the way to a more robust / redundant design of a DGNSS station. Technical and Cost-Benefit Analysis will be prepared for Maritime Authorities to show the benefits of EGNOS.

The results and outcomes of the pilot project will be presented at IALA meetings to support IALA R-135 recommended strategy on the future of DGNSS.

## Request

Maritime Authorities interested in collaborating in such a pilot project by offering a DGNSS station for the tests are requested to send an email to market@gsa.europa.eu before 15 May 2015.