IALA Recommendation R-121

On

the Performance and Monitoring of

DGNSS Services in the Frequency Band

283.5 – 325 kHz

Draft Edition 2.0

October 2014

(Previous Edition 1.0 – June 2001)

AISM  Association Internationale de Signalisation Maritime   IALA

International Association of Marine Aids to

Navigation and Lighthouse Authorities

10, rue des Gaudines

78100 Saint Germain en Laye, France

Telephone: +33 1 34 51 70 01  Fax:  +33 1 34 51 82 05

e-mail:  contact@iala-aism.org       Internet:  www.iala-aism.org

# Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |
| Sept. 2004 | 5. Integrity, 11. Validation, New Appendix 1 "DGNSS Broadcast Site Settings | Provide better guidance on these important areas to administrations setting up and operating DGNSS services. |
| Oct. 2014 | New structure Renewing of content | Options for DGNSS reengineering Service based architecture Improved explanation of requirements |

IALA Recommendation on the Performance and Monitoring of a DGNSS Service in the frequency band 283.5 - 325 kHz

THE COUNCIL

NOTING the function of the IALA with respect to Safety of Navigation, the efficiency of maritime transport and the protection of the environment,

NOTING ALSO IMO resolutions A.915(22) on Maritime Policy for the Future Global Navigation Satellite System (GNSS), and A.1046 (27) on World Wide Radionavigation System,

NOTING FURTHER ITU-R Recommendation M.823-3 on the Technical characteristics of differential transmissions for Global Navigation Satellite Systems (GNSS) from maritime radio DGNSS sites in the frequency band 285-325 kHz (283.5-315 kHz in Region 1),

RECOGNISING the need to ensure that Differential GNSS (DGNSS) services in the frequency band 283.5 kHz – 325 kHz are operated in accordance with certain minimum standards that take into account relevant ITU-R Recommendations and IMO Resolutions,

RECOGNISING ALSO that the minimum standards should include the signal format, reference datum, availability, continuity, integrity, accuracy, signal monitoring, range and coverage, status reporting, validation, and the publication of information about the system,

HAVING CONSIDERED the proposals made by the e-Navigation Committee:

RECOMMENDS that Members and other appropriate Authorities providing, or intending to provide, DGNSS services in the frequency band 283.5 – 325 kHz, according to the following principles:

# Provide the service in accordance with ITU-R Recommendation M.823-3

# Provide integrity information for GNSS

# Provide the service with a level of redundancy to achieve performance requirements IMO A.1046 (27)

# Provide means of verifying the performance of the service

# Provide mariners with information about the service for example

# description of the service

# achieved service performance

# service disruptions

# geographical service area

# Adopt the design and implementation principles set out in the relevant IALA Guideline(s)

# .