International Association of Marine Aids to Navigation and Lighthouse Authorities

AISM Association Internationale de Signalisation Maritime     IALA

10, rue des Gaudines

78100 Saint Germain en Laye, France

Telephone: +33 1 34 51 70 01    Fax: +33 1 34 51 82 05

e-mail: contact@iala-aism.org       Internet:  www.iala-aism.org

IALA Guideline

[No ###]

On the

Technical Specification
of a

Generic Service Engineering Model

[Edition 1]

[2015]

Initial Edition

# Document Revisions (Title style)

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

# Table of Contents

Document Revisions (Title style)	2

Table of Contents	3

Index of Tables	4

Index of Figures	4

1	Introduction	5

2	OVERVIEW ON THE Guideline STRUCTURE	6

3	The Place of the technical service in the Service-oriented Architecture of the CSSA and its benefits	7

4	Fundamental service design considerations	8

4.1	The notion of the Basic Services (BS)	8

4.2	Service Data Model - the CMDS Interface of a service – or: data a service delivers and how it is being modelled	12

4.3	The parts of a service and the models describing their relationships	13

4.4	The influence of geographical topology – the Service Distribution Model	16

5	The layered structure of a service - Structure Model	17

5.1	Services with and without a Physical Layer	17

5.2	Generic composition of a layer	17

5.3	The Logical Layer and the Logical Shore Station	17

5.4	The Physical Layer and the Remote Fixed Facility	17

5.5	The Service Management Layer	18

5.6	The complete generic Structure Model	18

6	Resulting service setup and its component’s tasks	20

6.1	Resulting service setup	20

6.2	Resulting tasks of the functional components of own service	22

8	Overview of the appendices of the generic service engineering model	26

8.1	Deliverables of a service to the CSS	26

8.2	Parts of the generic service model related to the layout and setup of own service	26

8.3	Parts of the generic service model related to components of a service	27

8.4	Parts of the generic service model related to installation and life-cycle management issues	29

# Index of Tables

Table 1	List of standard appendices for generic service model	6

Table 2	Degree of impact of a functional component on relevant quality and service management aspects	25

# Index of Figures

Figure 1	External Basic Services of a Data Collection and Transfer Service (by example)  as comprehensive capability statement of that service	9

Figure 2	Examples of  External Basic Services and their Categories E-1 to E-3  (for a comprehensive description compare Appendix 1)	11

Figure 3	Parts and individual models of own service and their interactions with external entities	15

Figure 4	Generic geographic distribution setup of a CSS and the place of different services	16

Figure 5	Layered structure of own service with Physical Layer – Structure model	19

Figure 6	Example of a service setup (One-Node Service Distribution Configuration within CSS context)	21

Figure 7	Overview over tasks of the functional components of own service	23

Figure 8	Sample mapping of External BS (Cat. E-1) to component functionality in the case of service with at Physical Layer	24

Figure 9	Sample mapping of the use cases contained in External BS (Cat. E-1) to component functionality in the case of service without at Physical Layer	24

# Introduction

The main building block of the Common Shore-based System Architecture (CSSA) is the so-called technical service. This guideline provides a common generic engineering model template for the technical services of the CSSA.

The common generic engineering model template views the technical service from a variety of relevant angles, which complement each other. To present this volume of material in a reader-friendly, orderly and consistent manner, this guideline consists of this main part and a variety of Appendices. This main part introduces the different perspectives and links the different Appendices.

This engineering model, comprising the different relevant perspectives complementing each other, is meant to be a ‘best practice’ template for technical services. Within any one instance or implementation of a Common Shore-based System (CSS), it is desirable that all technical services described adhere to this engineering model.

As a well-developed example of the application of this generic engineering model to a specific technology, the AIS Service description has been developed in parallel (refer to IALA document on shore-based AIS Service). The AIS Service provides an example of a technical service with a Physical Layer and also exhibits a very large technology specific complexity. Hence, this generic engineering model was validated by using a ‘difficult’ application.

For other technologies which exhibit a lesser degree of complexity in functional and architectural terms, this generic engineering model may be tailored as required while maintaining its core concepts and tenets.

Important notes:

For the purpose of this generic engineering service model, the term ‘technical service’ is abbreviated to ‘service.’ Once a specific service is addressed, it is capitalized, e.g. in ‘Basic Service’ or ‘AIS Service.’

Also, when considering the ‘service under consideration,’ this is abbreviated to ‘own service’ (as opposed to the ‘requesting service’ or the ‘requested service’).

It is advisable to read this main part at least once before considering any Appendix to gain an overview.

Also, it is advisable to re-visit this main part several times after consulting Appendices.

References and abbreviations are collected in Appendix 0.

# OVERVIEW ON THE Guideline STRUCTURE

For practical reasons this guideline is sub-divided into a main part (i.e. this part) and a number of Appendices. The overall list of Appendices is given in Table 1.

Main Part = this document [introduce status according to workplan]

Appendix 0 – References, Abbreviations, Glossary of Terms

This Appendix 0 contains all references, abbreviations and terms from throughout this guideline.

Deliverables of a service to the Common Shore-based System (CSS):

Appendix 1 – 	Basic Service Categories definitions and Minimum Set of Required Basic Services

Appendix 2 – Service Data Model – the CMDS interface of a service

Appendices related to the layout and setup of a service:

Appendix 3 – 	Distribution model

Appendix 4 – 	Interaction and data storage model

Appendix 5 – 	Interfacing model

Appendix 6 – 	Internal Time Latency model

Appendix 7 – 	Internal Reliability model

Appendix 8 – 	Test model

Appendices related to functional components of a service

Appendix 9 – 	generic functional description of the Logical Shore Station

Appendix 10.1 – generic functional description of the Physical Layer as a whole

Appendix 10.2 – generic functional description of the Remote Fixed Facilities

Appendix 10.3 – generic functional description of the Physical Link Terminal Equipment

Appendix 10.4 – generic functional description of the Physical Link Couplers

Appendix 11 – generic functional description of the Service Management

Appendices related to installation and life-cycle management issues

Appendix 12 – Site co-location issues and on-site infrastructure considerations

Appendix 13 – Recommendation regarding efficient operation and maintenance of a service

Appendix 14 – Runtime configuration management of a service

List of standard appendices for generic service model

Since the present guideline is meant to be a template to inform IALA documents on individual, i.e. technology-specific services (such as AIS Service or Radar Service), the order of appearance and the numbering scheme was developed to be used in an identical fashion throughout the IALA documents on individual, i.e. technology-specific services.

This allows for ease of reference to this guideline while providing some flexibility regarding technology-specific aspects related to installation and life-cycle management. Additional appendices on e.g. technology-specific runtime configurations may be added as additional Appendices as needed. Hence, designers of technology-specific service documentations are strongly encouraged to adhere to this setup of Appendices.

# The Place of the technical service in the Service-oriented Architecture of the CSSA and its benefits

The layout of the CSSA reflects the user-requirement driven, top-down approach, to IMO’s e-Navigation concept. The functionality and data delivery required from the technical Common Shore-based System (CSS) by the various users are fulfilled by arranging the necessary functionality and data storage in a Service-Oriented Architecture (SOA).

In a SOA a ‘service’ is the main building block and its composition is developed starting from the service-delivery to the requesting entity, which may be the (human) user or another technical service or system. So, the very name of a ‘service’ reflects the user-requirement driven approach stipulated by IMO.

IALA members are tasked with the deployment and continuous operation of the user-required functionality and data delivery of their shore-based system. Cost efficiency of deployment as well as minimisation of expenditure over the life-cycle of the system need to be taken into account, too. These aspects are summarized by the term ‘Life Cycle Management,’ and they have a large impact on the engineering design of the services in addition to the user requirements.

Analyses have shown that there are also certain structural and functional commonalities between different services, which must be exploited taking into account the cost efficiency requirements. These commonalities have led to the development of the present generic engineering model for the services.

The present generic engineering model was developed based on in-depth understanding of a variety of different technologies. Now that the generic engineering model exists, it may be applied to any other technology contained in the CSSA and hence, it serves as a generic engineering model template.

The benefits of this approach are:

A pool of well-defined generic solutions exists, and these solutions may easily be applied to different technologies;

This generic engineering model template also serves as a ‘checklist’ for the international community when addressing the deployment of any new technology for the shore side;

A common understanding between engineers for the services is achieved internationally; this is not only one of the objectives of IMO’s e-Navigation concept but also serves as a common ground between IALA National members and industry;

This generic engineering model template provides a powerful tool for management decisions within IALA national members, i.e. Aids-to-Navigation authorities;

This generic engineering model template promotes the concept of modularity and will allow mixing and matching of best in class components.

# Fundamental service design considerations

The service is embedded in its CSS. Therefore, there are features of the service under consideration, i.e. the ‘own service,’ which are of interest to the system at large, in particular to any of the  ‘requesting services,’ and which should therefore be known or be ‘visible’ to the system at large.

This requirement applies to the discrete subsets of the overall functionality of own service and to the data elements exchanged between own service and the system. Since these elements are visible to the system at large, they will be described first (i.e. in this chapter).

Conversely, there are features of own service which are of interest within the boundaries of own service and for the technical personnel operating, maintaining and further developing own service. These features, which are the majority of the generic service model, should be ‘encapsulated’ within own service in order not to unnecessarily burden the system at large. These features will be discussed in this chapter afterwards.

The boundary of own service thus demarcates the difference between ‘external,’ i.e. visible to system at large and ‘internal,’ i.e. encapsulated within own service. This terminology will be used throughout this document.

## The notion of the Basic Services (BS)

Definition: A Basic Service (BS) is a discrete subset of the overall functionality of a service. It represents the functional delivery of a defined set of data objects or a defined set of functionality performed by that service. Basic implies that it is one consistent fundamental functionality of own service. The notion of the Basic Services is essentially driven by a requirement-fulfilment (i.e. client-server) relationship from the shore’s point of view, eventually driven by the shore-based user’s point of view.

Every service of the CSS comprises a minimum set of Basic Services.

A Basic Service is always represented by a name and a mnemonic meaningful in the context of that service.

### The definition of External Basic Services

Any service is part of the CSS. Hence what capabilities of the specific service under consideration contributes to the CSS would be of primary interest to any other service. These capabilities can be defined as External Basic Services (E-BS) of own service. They are called ‘external’ because data and/or the effects of some functionality cross the boundary of own service.

External Basic Services are all justified by at least one desired interaction between own service and some entity outside the boundary of own service. These entities are other services within the CSS which request data and/or functionality, and are hence called ‘requesting service.’ In the case of the User Interaction Service and the Gateway Service, both of which interface the CSS to shore-based users (humans; external systems), these users could also be that entity requesting External Basic Services.

All External Basic Services of own service taken together provide the summary capability statement of that service from the perspective of the CSS or – in the case of the User Interaction Service and the Gateway Service – by the shore-based user. They thus contribute to the reduction of complexity by encapsulation (compare Figure 1).

CSS

Point of view

from ashore

Encapsulation of complexity 
for shore-based e-Nav applications

Data
Collection and Data Transfer Service

Physical Link

Appli-cation

Appli-cation

Appli-cation

External Basic Services of the service as its functional interface to the CSS

External Basic Services of a Data Collection and Transfer Service (by example) 
as comprehensive capability statement of that service

For every service a list of its External Basic Services is given.

For each of the External Basic Services one functional description needs to be developed. The full description of the External Basic Service consists of

its name,

a mnemonic,

its External Basic Service Category indication (see below),

the data objects involved

a standardized table describing the functions of the interacting components of own service,

a figure, which maps it to the appropriate components of own service, and

additional functional detail description as appropriate.

The encoding format of the data objects will be detailed in the appropriate Basic Service definition.

External Basic Services may request the support of other services of the CSS, i.e. making them ‘requested services’ and turn own service into a ‘requesting service.’

For a description of the minimum set of External Basic Services refer to Appendix 1.

### Basic Service Categories

The definition of BS Categories is useful for the following reasons:

BS Categories reflect the different net data flow mechanism within own:

There are different data paths and data processing requirements for receiving and transmitting within own service with associated consequences for design and operation.

Although apparently the same data object being treated in different BS Categories, the meaning of that data object depends on the net data flow direction, i.e. its context. That means, that although the data object looks the same and there is a similarity in appearance and therefore even identity of description, the data objects are different due to their role in the data flow from an ontological point of view.

To illustrate this, an example from the AIS Service is given: Data received from AtoN AIS stations by ATON_DAT (received) means something different than transmitting AtoN messages via the AIS Service by ATON_DAT (transmit), although the data object description looks identical. In the first case there is a physically existing AtoN AIS Station involved, while in the second case, there is a synthetic or even virtual AtoN involved.

BS Categories best reflect the data orientation paradigm (as opposed to the technological orientation), which is a fundamental feature of the e-Navigation concept.

BS Categories reduce the complexity of own service by employing functional similarities to the largest extent possible.

BS Categories, once defined, provide a fixed reference point throughout the description of own service for statements like: ‘For BS Category x the following applies: ...’.

BS Categories increase description efficiency of the functionality of own service substantially, i.e. even allow for a minimum description.

BS Categories make the description of own service robust to amendments of the precise data object content.

BS Categories simplify the maintenance of the documentation of own service at large because additional data contents can be added easily to the appropriate BS Category without the need to change much of the otherwise BS category-related description: The amendment to the BS Category description will have an immediate effect on all individual BS of the same BS Category.

### Example of External Basic Services and their BS Categories to illustrate top level interaction between services of CSS

Figure 2 is an illustration of the concepts of Basic Services and of Basic Service Category. This figure shows some of the External Basic Services, as categorized into External BS Categories E-1, E-2, E-3, as an example. The full list is given in Appendix 1. This example thus also serves as an illustration for the top-level interaction of own service with the other services of the CSS.

E-BS Category E-1 for service with (left) / without (right) a Physical Layer:

E-BS Category E-2 for with (left) / without (right) a Physical Layer:

E-BS Category E-3 with (left) / without (right) a Physical Layer:

Examples of  External Basic Services and their Categories E-1 to E-3 
(for a comprehensive description compare Appendix 1)

- Figure 2 shows the net data flow, i.e. the flow of the desired/required data through or from or into own service to/from requesting service, as requested by this:

It is important to note the boundaries of ‘own CSS’ and ‘own service’ (indicated by the boxes) for proper understanding of the interaction of the entities: ‘Own service’ is always within ‘own CSS’ because it is a part of ‘own CSS’; Physical Link(s) are outside of both ‘own CSS’ and ‘own service’ etc.

The net data flow is construed here in an abstract fashion without considering any data encoding or interface protocol issues at this point;

There is always a requesting service to own service; in the case of the User Interaction Service, the human user assumes role of the requesting service, in the case of the Gateway Service an external shore-based system may assume the role of the requesting service.

Sometimes own service also requests the support of other services of the CSS, in particular in the case if own service does not have a Physical Layer, in which case own service is assuming the role of a requesting service towards this/these other service(s).

### Internal Basic Services and their categories

There are also Basic Services, which are not justified or requested by an external requesting service of the CSS or – in the case of the User Interaction Service or the Gateway Service – by a shore-based external user. These Basic Services are called Internal Basic Services (I-BS): They are supportive in nature, but are essentially required.

Examples of Internal Basic Services are recording of service data (pre-requisite to any replay), service initialisation, service termination and component management of own service.

Internal Basic Services may not be directly visible outside own service on its Physical Link (in the case of a service with a Physical Layer); Internal Basic Services are not directly visible for the CSS, although their effect may be.

Frequently the more sophisticated functionality related to the specific technology of own service is often an Internal Basic Service. Internal Basic Services often interact with the Technical Operation Personnel responsible for own service, while this functionality is not requested by the CSS, i.e. while there is no requesting service.

The Internal Basic Services are also categorised into Internal Basic Services Categories in a similar fashion to the External Basic Services.

An Internal Basic Service may request the support of other services of the CSS, i.e. making them requested services and turn own service into a requesting service.

For a description of the Internal Basic Services which every service needs please refer to the description of the minimum set of Basic Services in Appendix 1.

## Service Data Model - the CMDS Interface of a service – or: data a service delivers and how it is being modelled

The above Basic Services define the functionality provided by own service to the CSS in an abstract manner. They are operations performed on certain data objects, or: they are f(x), to express it in mathematical terms. The data objects treated by the Basic Services are the operands, i.e. the ‘x’, of these functions. To provide a complete picture of what own service delivers to the CSS, it is necessary to consider those data objects, their structure and mutual relationship. Essentially, it is the data, which is only relevant for other services. The first major purpose of the Service Data Model, therefore, is to describe what data objects own service delivers to requesting services, in an appropriate data modelling fashion.

This ‘appropriate data modelling fashion’ is given by the Common Maritime Data Structure (CMDS). The CMDS features prominently In the overarching architecture IMO has defined for e-Navigation. In a nutshell, the CMDS is an overarching data model for the maritime domain, both shipboard and shore-based. It is to be based on the IHO S-100 framework. This framework sets up, amongst other things, a framework of Registers in the IHO GI Registry. Some of the Registers defined therein are the Feature Concept Dictionary Register, the Portrayal Register, and the Product Register. The Service Data Model, as its second major purpose, assigns the references of its own data objects to those data objects already defined at the CMDS / IHO GI Registry.

A third major purpose of the Service Data Model is to specify the relationship between own service technical description and the notion of ‘products’ in the context of CMDS / IHO GI Registry. This is necessary when considering own service exclusively from its data delivery point of view (although this does not change its physical setup, as described in other appendices of the generic service model at all).

There are other purposes of the Service Data Model also.

For a more detailed description of above facets of the Service Data Model as the interface of own service to the CMDS refer to Appendix 2.

Although the Service Data Model does describe the contribution of own service to the CSS Data Model at large, in data object terms, it does not contain encoding details as to how the data objects are exchanged at the Machine-to-Machine (M2M) or Human-Machine interfaces (HMI). However, specifications for encoding at application level (i.e. regarding the Basic Services) as well as statements regarding the specifics of M2M and HMI interfaces of a service are required. This is stated in the generic service model in the Interfacing Model (Appendix 5) which firstly identifies the M2M and HMI interfaces of own service as ontological entities. Secondly, the Interfacing Model provides a portfolio of appropriate and compatible encoding techniques to be used with data objects of own service, which in turn reference another generic and framework IALA guideline on M2M interfacing techniques within the CSSA.

## The parts of a service and the models describing their relationships

The previous section was concerned with what a service delivers to the CSS in terms of functionality and in terms of data. After having thus covered the deliverables of a service to the CSS, it is now necessary to turn towards its generic architecture.

### Components of a service: service components and service-owned infrastructure

The material parts of a service are called components of the service, which are further sub-divided into service components and service-owned infrastructure (components).

The service components may be functional software modules or technology-specific hardware components (such as technology-specific base stations, antennas and the like). The service components carry the functionality and/or the data delivery to create the Basic Services, while the service-owned infrastructure components (such as computers and Local Area Networks) are involved in that process but only as a tool and should remain ‘invisible’ in the process.

This distinction of service components and their respective functional description from the service-owned infrastructure and their respective requirement statement allows to decouple the more steadily changing or even lasting definition of the actually desired functionality from the rapid technology change at the service-owned infrastructure (compare for example the short-lived computer and IT network generations). Thus this distinction effectively and efficiently supports life-cycle management.

The service components are described in this guideline as far as generic features are concerned. The technology-specific functionalities of the service components are described in the technology-specific description of the individual services, which are informed by this IALA Guideline. For example, this means that the AIS technology specifics of AIS are described in the IALA Recommendation on the AIS Service as informed by this recommendation.

### Modelling the internal structure of a service and its relationships

There are immaterial parts of own service as well, such as various immaterial ‘models’ (compare Figure 3 overleaf). Their purpose is mainly to show the interactions between the material components, which were introduced in the previous section, from different perspectives.

There are interactions between both material and immaterial parts of own service with external entities (association indicated by lines in Figure 3). ‘External’ means that those entities lie outside the boundary of own service. They may be either entities external to own service or may even be external to the CSS. These interactions are taken into account in the appropriate description.

The possible interactions with external entities depend on the relative position of a service within the CSS, as indicated by the footnotes in Figure 3. From this the distinction between services with and without Physical Layer is also derived, which will be used throughout this document. Compare IALA Guideline on the Technical Specification of a Common Shore-based System Architecture (CSSA) system layout for a list of the different services in this regard. The different service group names and some of the specific service names will be used throughout this guideline.

The description of the models of own service forms the largest part of this guideline. There is a section dedicated to each of those models, in many cases more details are also captured in appropriate Appendices. Hence, Figure 3 will serve as a reference diagram throughout this guideline.

Parts and individual models of own service and their interactions with external entities

## The influence of geographical topology – the Service Distribution Model

It is necessary to consider the influence of geographical topology on the design of own service. This very important influence is captured in the Service Distribution Model.

There are functional similarities regarding the distribution of components of the different services of the CSSA, and there are also certain criteria to select a specific Service Distribution Configuration. The Service Distribution Model of own service describes some standard distribution configurations which would apply to most administrations and also provides technology-specific rationales and criteria for a geographical distribution of the components of a service.

In most cases, due to coverage area requirements, own service spans a larger part of an administration’s coast or even the whole of its coast(s). The actual topology of the coastlines may vary from country to country. The actual number of sites needed depends on a variety of aspects. In some cases, own service may shrink to an ‘one spot service’. Figure 4 provides a generic geographic distribution setup of a CSS.

Generic geographic distribution setup of a CSS and the place of different services

The Service Distribution Model is both functional and generic. It needs to be applied (‘tailored’) to the actual coastal topology by every administration. To that end guidance is provided on how to arrive at a proper individual distribution model taking into account the following aspects:

coverage requirements and coverage planning;

coastal topology (concave, convex, line, one spot, ...);

required functionality setup of own service, i.e. required Basic Services;

distribution of receive / transmit functionalities (if appropriate);

maintenance and life-cycle considerations (rationalisation);

data transfer network considerations;

infrastructure requirements.

The full development of the different generic geographic distribution setups are contained in the Appendix 3 (Distribution model).

# The layered structure of a service - Structure Model

The Structure Model describes the layered structure of own service and thereby defines in a general way its specific functionality.

## Services with and without a Physical Layer

Each and every service with a Physical Layer consists of three main functional layers as follows:

Logical Layer

Physical Layer: comprises all Remote Fixed Facility (RFF) of own service

Service Management (SM) Layer

For all services without a Physical Layer, the structure is simplified as follows:

Logical Layer

Service Management (SM) Layer

- The statement regarding the Physical Layer in the following sections do not apply to the services without a Physical Layer. However, there is a lot of commonality regarding the other aspects of the Structure Model.

In the CSS, all services without a Physical Layers are requesting services, i.e. at least one requested service within the CSS steps into the place of the (missing) Physical Layer in this case.

## Generic composition of a layer

Each layer comprises:

service component(s), which provides the required functionality in terms of service-specific data processing;

supporting components and resources, which are exclusively used by own service, such as computers and local networking devices, i.e. the so called service-owned infrastructure;

(remote) Human Machine Interfaces (HMI) to allow for access to Technical Operation Personnel.

Each layer is supported by on-site infrastructure, such as energy supply, which may be shared with other services co-located on the same site.

## The Logical Layer and the Logical Shore Station

The Logical Layer is the layer which contains all functionality of own service, which treats system-requested net data (as opposed to e.g. management and/or maintenance data) and which is not tied to a specific location at the waterway (as opposed to the Physical Layer). Hence, by definition, the Logical Layer resides at a Node site (compare Distribution model). The Logical Layer comprises the functional component Logical Shore Station (LSS) and other supportive service-owned components. The LSS is the functional component of own service, where the actual net data treatment is done. Hence, by definition, the LSS resides at a Node site. In most cases it is a software module residing in a computer (= service-owned infrastructure component).

## The Physical Layer and the Remote Fixed Facility

The Physical Layer directly interacts with its Physical Link. The Physical Link could be a radio link or a visual link or an acoustic link, depending on the actual technology of own service. The Physical Link as such does not belong to own service, although own service may influence it by its usage and/or management. Also, the Physical Link is outside the CSS in most cases.

For the description of the Remote Fixed Facility (RFF), the Physical Layer can be further subdivided into three layers as follows:

Physical Shore Station (PSS) Layer

Physical Link Terminal Equipment (PLTE) Layer

Physical Link Couplers (PLC) Layer

For reasons of space/direction diversity in coverage of a RFF, the lowest two layers are often sub-divided into Sectors, while their stacked or hierarchical relationship remains. A Sector comprises always PLTE and PLC together which are dedicated to a certain space direction regarding coverage. Conceptually wise, omnidirectional coverage ranges can be construed as a One-Sector RFF. Well known examples for the above concept of sectorisation are e.g. radio services using directional antennas or fixed visual aids with sector lights.

The Physical Layer of own service may serve different Physical Links or different physical link technologies simultaneously, which would lead to another subdivision of the two lowest Layers of the Physical Layer together: PLTE ‘x’ together with PLC ‘x’ would together and in conjunction serve the Physical Link ‘x’. Similarly, the PLTE ‘y’ together with the PCL ‘y’ would together and in conjunction serve Physical Link ‘y’. A well-known example for a service with a Physical Layer concurrently serving different Physical Links is Floating Visual Aids Service: Buoys as the RFFs of that service often operate on various Physical Links simultaneously, namely on the visual link (colour and lights), the acoustic link (fog horn), the radar link (radar reflector and RACON), and on the AIS VHF Data Link (AIS AtoN station).

All different Physical Links served and/or all sectors of a RFF are co-ordinated by the Physical Shore Station (PSS) alone and concurrently, and all data delivered to/from all different Physical Links served and/or to/from all sectors are processed by the PSS concurrently.  Hence, the PSS Layer is not subject to the sectorisation, but it needs to take into account different Physical Links, if present. Also, in order to avoid ambiguity and discrepancies, the Service Management Layer (see below) interacts directly with the PSS alone, and not with any component within the Physical Link Terminal Equipment and the Physical Link Couplers and within any sector.

## The Service Management Layer

The Service Management (SM) Layer is the layer which contains all functionality of own service to configure it and manage its components at run-time.

Configuration commands may be received from the requesting service of the CSS and/or from the Technical Operation Personnel.

The SM Layer is represented at the Nodes and at the remote sites.

Service components within the SM Layer are a Service Management Master, which contains all centralized management functionality and data storage and which resides at a Node, and Service Management Agents at a Node, hence SM-Node, and at a RFF, hence SM-RFF.

## The complete generic Structure Model

With the above structural elements of a service, the complete generic Structure Model can be represented as shown in Figure 5 overleaf.

* Note: This symbolic representation does not imply any inference as to the amount of required personnel for the technical operation of the service. This symbolic representation aims at indicating that human interaction with the largely automated service is required as the last resort - and in some cases possibly on a regular basis - on all layers of the service.

Boundary of CSS

Technical Operation Personnel (master control)

HMI

Logical Layer -

Logical Shore Station (LSS)

HMI

To/from CSS

Net Data

Status

Service
Management Layer

HMI

Physical Shore Station (PSS) Layer

HMI

Physical Link Terminal Equipment (PLTE) Layer

HMI

Physical Link Couplers (PLC) Layer

Physical Link 
(= medium)

Technical Operation Personnel 
(there are tasks for each layer in principle)

Configuration

Physical
Layer /
Remote

Fixed
Facility

(RFF)

Layered structure of own service with Physical Layer – Structure model

# Resulting service setup and its component’s tasks

## Resulting service setup

So far, the most fundamental aspects for own service have been introduced such as its deliverables to the CSS, as well as its functional parts, the influence of geographical distribution, and its layered structure. These elements have been introduced largely independent of each other. This chapter aims at tying these elements together.

Figure 6 shows a typical setup of a service with Physical Layer (for services without Physical Layer, simply drop the statements related to the Physical Layer and replace its role by the contribution of all of the requested services of the CSS). It is one possible translation of the more abstract layered structure depiction of own service in Figure 5 into one example setup using specific physical components on specific functional sites.

Figure 6 shows an One-Node Service Configuration setup. Figure 6 shows the functional components, the functional site designations, where these components reside, and also gives some indication regarding the interfacing of own service within the CSS.  Figure 6  shows some important features of own service as follows:

The setup of the Remote Fixed Facility (RFF), i.e. the components internal to it, will be described in Appendix 10.1 in more detail. Regarding Figure 6 it is sufficient to note, that there is PLTE (symbolised by antenna on roof top of RFF), that allows own service to access its Physical Link(s). The depiction of the RFF as a house also gives an indication of the relevance of on-site infrastructure consideration.

A Node site with the Logical Shore Stations is shown, and how they are connected to the components of the other services of the CSS at the same Node site. As an (typical) example, particularly the User Interaction Service of the CSS is shown. The ‘other services’ may be any other service of the CSS.

The components of the Service Management (SM) that reside at the Node site are shown. (It should be noted, that components of the SM also reside at the RFF; see Appendices 10.1 and 11).

Also the support for Technical Operation Personnel by Human Machine Interface(s) (HMI) provided by own service is shown.

Note the concept of the functional sites in combination with the premises concept.

All components at the [premise |location] N where the Node site, the Technical Operation Personnel site and the User site reside in the example are connected with LAN functional links / LAN technology components. WAN feeder links will be used, when it comes to connecting the components of own service at the Node site with its Remote Shore Stations on the Remote sites.

Cautionary note: Figure 6 should not be construed as normative or the only recommended way to set up own service. Figure 6 does claim however to be correct and consistent with the normative statements in this guideline in regard to what it shows.

Cautionary note: While Figure 6 describes one example of a workable service setup, it should be noted, that the available features and the available options in setting up own service can only be precisely captured by the more abstract descriptions provided in this guideline.

Example of a service setup (One-Node Service Distribution Configuration within CSS context)

## Resulting tasks of the functional components of own service

### Tasks of the functional components in the context of their interaction

The functional components can now (more) precisely be defined by their individual set of tasks in the context of their interaction with each other and with entities external to own service. The Figure 7 shows the different tasks of the different functional components of own service in context.

Please note

the different task orientation of functional components, i.e.

the orientation of the Logical Layer towards the requesting / requested services of the CSS and their requirements;

the orientation of the Physical Layer towards the Physical Link(s) served/employed by own service and its specific physical requirements;

the need to have own-service-wide data storage which allows own-service-wide visibility and access of relevant data objects;

the configuration and control capabilities of the Service Management (SM).

that the instances are now implied, namely

that the Physical Layer consists of all Remote Fixed Facilities (RFF) of own service as deployed by an individual administration;

that the Logical Layer consists of all Logical Shore Stations (LSS) set up as individual software processes of own service as deployed by an individual administration;

that there are necessary functionalities, such as own-service-wide data storage, which are invisible to the net data flow between the Physical Link(s) and the requesting services, and vice versa. These functionalities are supported by infrastructure components owned by own service (i.e. service-owned infrastructure), namely by distributed data bases, local area networking, replicator devices, etc.

To specify the precise features and attributes of those functional components, such as data processing capabilities, interaction capabilities, interfacing requirements, timing constraints, and reliability figures, the appropriate service models need to be considered.

These models have discussed the own service on class level. In order to set up an individual service by an individual administration, it is necessary to eventually arrive at specific components which materialize at specific sites, i.e. at the instance level. This is where the Distribution model fits in.

All these different aspects need to be merged within the description of the functional components. While this will be done in the appropriate Appendices, describing the particular functional components, a short introduction is provided in the following sections.

Figure 7 will be used and explained in several Appendices which elaborate certain aspects.

Overview over tasks of the functional components of own service

### Mapping of service capabilities to component functionalities and component requirements

To finally arrive at the required functionalities and at the requirements for the components specifically, it is necessary to map the Basic Services (both external and internal) to the relevant individual components internal to own service. Mapping of Basic Services results in use cases being identified for each service component as their component functionalities.

In accordance with the layered structure of own service the three main components to be considered for the mapping are Logical Shore Station (LSS), Physical Shore Station (PSS), and Service Management (SM).

In the case of services without Physical Layer there is no PSS. In the case of services with Physical Layer the Physical Link Terminal Equipment (PLTE), the Physical Link Couplers (PLC), and also the Physical Link as such are omitted from the consideration at this point since they are transparent in regard to the net data flow. They will re-appear when considering secondary issues of own service, such as component management proper or management of the Physical Link.

The Basic Service Categories, as defined in Appendix 1, are used. As an example for the mapping in the case of a service with a Physical Layer, Figure 8 shows the mapping of an External Basic Service (Category E-1) to the component functionalities of the above components.

Sample mapping of External BS (Cat. E-1) to component functionality in the case of service with at Physical Layer

As an example for the mapping in the case of a service without a Physical Layer, the Figure 9 is given.

Sample mapping of the use cases contained in External BS (Cat. E-1) to component functionality in the case of service without at Physical Layer

The required component functionalities of own service are collected in Appendices 9, 10 and 11 of this document which deal with the functional descriptions of the respective components.

### Type approval considerations for PSS, LSS, and SM software modules

There may be required certain quality levels of service offered by a competent authority. Ultimately these requirements can only be achieved and validated by passing the procedure of type approval for the functional components involved. This section elaborates on the type approval aspect using the above introduction of the functionality descriptions for the PSS, the LSS, and the SM.

When there is type approval required, the following notions should be taken into account:

Type approval for software modules should only apply to the service components of the PSS, LSS, and SM software modules. Ideally these specific functionalities would be plug-in software modules. Type approval would not address or include service-owned infrastructure components, such as computers, LAN components etc.

The description of the service components of PSS, LSS, and SM in Appendices 9, 10, and 11 would be precise in terms of functionality while not being prescriptive regarding implementation by a manufacturer. Thus, the functional components PSS, LSS, and SM would be modelled as ‘black boxes,’ rendering open interface definitions.

The associated degree of impact of a service component’s functionality could guide the selection of functionality which should be made subject to type approval and in what regard (compare Table 2).

Degree of impact of a functional component on relevant quality and service management aspects

| Impact of | Degree of impact |  |
| --- | --- | --- |
|  | HIGHER | LOWER |
| run-time configurations of Physical Link(s) (1) | SM (in particular SM-Master functionality) (due to CSS-wide influence) | PSS (limited to local influence) |
| Quality of data delivery to/from CSS | LSS + PSS | SM |

(1) for services with Physical Layer only, and only when Physical Layer has capability to influence Physical Link(s) as such.

# Overview of the appendices of the generic service engineering model

The previous chapters provided a solid introduction and overview. The individual parts of the generic service model need to be introduced in more detail to arrive at the precision necessary for specific use by IALA, IALA membership and the supply industry for planning, procurement, deployment and operation of any specific service. There is an Appendix dedicated to one of the following parts each (compare List of Appendices in Table 1). The following introduction also serves as an overview content definition for the Appendices.

The generic service engineering model is described by the following parts, which complement each other:

## Deliverables of a service to the CSS

### Basic Services Categories and Minimum Set of Basic Services

Introduces the normative definitions of the categories of both External and Internal Basic Service Categories and introduces the Minimum Set of Basic Services, i.e. a comprehensive normative definition of Basic Services generally required for each service of the CSS. Refer to above section 4.1 and Appendix 1 for further discussion.

### Service Data Model – the CMDS interface of service – or: data that a service delivers and how it is being modelled

Introduces abstract descriptions of the data provided by own service at its Basic Services level to the CSS. In the Service Data Model data objects of own service together with their properties or attributes are defined by extensively referencing existing entries in the appropriate Registers of the S-100 based IHO GI Registry, in particular the Feature Concept Dictionary Register. Thus, the Service Data Model provides the CMDS interface of own service – here: in a generic fashion.

By giving the relationships of the data objects used by and in own service (which are therefore referenced to IHO GI Registry), the context or the semantics of the data objects used by and in own service is given – hence: Service Data Model. This is the starting point for any ‘product’ definition for own service.

Refer to section 4.2 above and to Appendix 2 for further discussion.

## Parts of the generic service model related to the layout and setup of own service

### Distribution model

The Distribution model deals with issues arising from geographical distributed service layouts, including generic aspects of coverage planning as one major factor if appropriate. The Distribution model is given in Appendix 3.

### Interaction and Data Storage model

The Interaction model introduces the dynamics of the interaction of the components within own service (data flow dynamics, such as push or pull concepts) and deals with associated data storage issues on a generic level.

The Data Storage Model deals with storage entities of own service. The need for data storage is due to the geographical distribution of own service components. The Data Storage Model deals with aspects like visibility of data objects, age of data objects, and rule base for data processing in own service, and consequences of geographically distributed data storage, such as the need for replication.

The Interaction Model and Data Storage models together are given in Appendix 4.

### Interfacing model

The Interfacing model of own service identifies the M2M interfaces of own service to other services of the CSS and the M2M interfaces between the components of own service, i.e. within own service. Also the HMI interfaces are identified. All identified interfaces are briefly introduced as to their required functionality.

For the details of the M2M interfaces, the Interfacing model references another IALA framework guideline on M2M interfaces within the CSS. There, for the M2M interfaces the following aspects are taken into account:

the application encoding methods; a portfolio of encoding techniques is recommended to suit the data objects of the Data Model;

the protocol stacks recommended for data transfer internal and external of own service.

- All application encoding methods and protocol stacks recommended are in keeping with the open system philosophy of the CSSA.

- The Interfacing Model is given in Appendix 5.

### Internal Time Latency model

The Internal Time Latency Model of own service describes the time latency limits regarding the Basic Services (by external and internal BS Category). In particular the resulting permissible latency of data delivery of own service operating within the CSS is discussed. This is relevant, for example, when correlating time-critical data from different Data Collection and Data Transfer Services within the Value Added Data Processing Services of the CSS. From the timing requirements of the Basic Services, timing requirements for components of own service and their mutual interconnection are derived. The Internal Time Latency Model is given in Appendix 6.

### Internal Reliability model

The Internal Reliability Model of own service analyses the internal reliability aspects of own service and describes how to measure internal reliability during run-time. The Internal Reliability Model is based on the Structure Model, the Distribution Model and the Interaction Model. The Internal Reliability model is given in Appendix 7.

### Test model

The Test Model for own service:

describes what are good test strategies for own service as a whole for Technical Operation Personnel, and

provides guidance how to ascertain the proper function of own service after cold or warm start.

The Test Model does not generally deal with testing of individual components. Details are given in Appendix 8.

## Parts of the generic service model related to components of a service

### Generic functional description of the Logical Shore Station (LSS)

The generic functional description of the LSS describes the functionality needed on the Logical Layer to provide the External and Internal Basic Services, employing the concept of the Basic Services Category. In particular, this part contains the contribution of the LSS to the Minimum Set of Basic Services. For the description of an individual service, the technology-specific functionalities of the LSS of this individual service is contained in Appendix 9.

### Generic functional description of the Physical Layer as a whole – the Remote Fixed Facilities (RFF)

The generic functional description of the Physical Layer as a whole describes how the different Sub-Layers of the Physical Layer interact to achieve the contribution of the Physical Layer to the External and Internal Basic Services. Also the additional components of own service and their interactions at the Remote site are introduced, thus rendering a comprehensive description of the Remote Fixed Facilities of own service at a Remote site. See Appendix 10.1.

### Generic functional description of the Physical Shore Station (PSS)

The Physical Shore Station (PSS) performs the following main tasks:

Pre-processing of data in one or both directions (receive and/or transmit), depending on configuration

Control of the Physical Link Terminal Equipment Layer of the RFF depending on the configuration pre-set of the Service Management; this includes also control of the Sectors, if defined.

The PSS essentially is a software process running as application software on a computer as a physical entity or as an integral firmware of a dedicated unit, such as a smart switch.

Details are described in Appendix 10.2.

### Generic functional description of the Physical Link Terminal Equipment (PLTE)

The Physical Link Terminal Equipment (PLTE) comprises all devices and/or functionality needed to – regarding ‘receiving’ – transform a signal received from the Physical Link Couplers into a data object as appropriate for the specific technology of own service. The data object is forwarded to the Physical Shore Station (PSS).

Regarding ‘transmission,’ a data object received from the PSS is transformed into an appropriate signal which is forwarded to the Physical Link Couplers for eventual transmission on the Physical Link. Examples of PLTE are radio transceiver devices in the case of radio based services or light generating/receiving devices in the case of visual services. Details are described in Appendix 10.3.

### Generic functional description of the Physical Link Couplers

Regarding ‘transmission,’ Physical Link Couplers (PLC) are all devices and/or functionality needed to effectively couple a signal generated by the PLTE into the appropriate Physical Link(s) and/or – regarding ‘receiving’ – to extract signal(s) present in the Physical Link(s) and forward it/them to the PLTE. Examples of PLCs are radio antennas for radio wave signals or optical lenses for light signals. Details are described in Appendix 10.4.

### Generic functional description of the Service Management

The Service Management (SM) of a competent authority performs the following tasks:

It acts as a management entity for the whole of own service;

It manages the Internal Basic Services, and

It is the last resort regarding faulty behaviour.

Specifically, the SM:

Invokes, initialises, configures and terminates all the instances of LSS and PSS at run-time;

Determines the functional connections between PSS and their associated LSS for them to use during run-time;

Determines the functional connections between the LSS and the requesting services’ logical interface associated with them, i.e. this top level acts as a ‘switch-board’ for the data exchange relationships between the different processes;

Provides the HMI for the Technical Operation Personnel to monitor the current status of own service and potentially of the appropriate Physical Link(s) and configure them accordingly.

For most Service Distribution Configurations the SM consists of several distributed agents and of one centralized master functionality, all of which are functional sub-components of the SM and are generally software processes. There are the following functional sub-components:

SM-Agent at the Remote Site, which represents the ASM at the RFF of own service,

SM-Agent at the Node site, representing the SM to the LSS,

SM-Master (regularly only one SM master per service).

Details are described in Appendix 11.

## Parts of the generic service model related to installation and life-cycle management issues

### On-site infrastructure considerations and site co-location issues

These two aspects are related:

All the components of own service, i.e. the service components proper and the service-owned infrastructure components, have requirements regarding the on-site infrastructure, such as housing, protection against a variety of external influences, energy supply, source for a time base, and so forth.

Also, at one Remote Site of the CSS usually several different services of the CSS are present. For example, there are Remote Sites typically populated by a variety of radio services, such as AIS, VHF voice communication, and GMDSS DSC Services. Sharing on-site infrastructure may be an economic necessity in most cases. This results in site co-location issues.

Refer to Appendix 12.

### Recommendation regarding efficient operation and maintenance of a service

In this Appendix all generic aspects regarding the efficient operation by Technical Operation Personnel as well as the efficient maintenance and further development by Technical Maintenance/Development Personnel are discussed. Thus own service is tied into the well-established conceptual framework of technical operation and life-cycle management. Means to maintain and improve efficiency of service delivery over the full life-cycle are described generically in Appendix 13.

### Runtime configuration management of a service

This Appendix addresses generic runtime configuration of the service by the Technical Operation Personnel. In this Recommendation the conceptual framework is provided, which is used by the individual descriptions of the technology-specific individual services. Refer to Appendix 14.

The shore-based AIS Service was one very important technology in this regard (and it still serves as application example of this generic service model to a complex technology).

For further details on the service interfaces refer to Appendix 5.

Note: ‘Service components’ and ‘components of a service’ is not the same, since the latter comprises the former plus the service-owned infrastructure components.

It is planned to describe the service-owned components in some other IALA document as well as the on-site infrastructure.

Refer to Distribution model in Appendix 3 for further details

Refer again to the Distribution model, Appendix 3, for further explanation regarding the sites.

For further detail refer to the Interfacing Model, Appendix 5.

For details regarding the setup and installation of an RFF also in infrastructure terms refer to the Installation guidelines for RFF, Appendix 12.

The term ‘class’ is used as defined by the Object-Oriented Paradigm.