Input paper:	     ENAV16-9.23

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	□  ENG		□  PAP					□  Input

X  ENAV	□  VTS							X  Information

Agenda item			9

Technical Domain / Task Number	E-NAV / Task 5.1.19

Author(s) / Submitter(s)			Jens K. Jensen, Danish Maritime Authority, Denmark

Mikael Lind, Victoria Swedish ICT AB, Sweden

Kwangil LEE, ETRI, Republic of Korea,

Jin Hyoung Park, KRISO, Republic of Korea

Per Setterberg, Swedish Maritime Administration, Sweden

# How the Maritime Cloud supports e-navigation

# Summary

This document describes how the infrastructural requirements for e-navigation are met by the Maritime Cloud concept, such as: A global Identity concept covering ships as well as shore stakeholders, mechanisms to support authentication, information integrity validation, encryption for confidentiality, discovery of standardized services for e-navigation and messaging interoperability across different communication links for seamless roaming.

The Maritime Cloud is the term used to describe the concept of an infrastructure that support authorized, seamless information transfer, as requested by the IMO e-navigation strategy, and as derived from testbeds focused on e-navigation. The Maritime Cloud concept is similar to the maritime infrastructure framework, described in ENAV16-9.24, adding those elements, that are necessary to support the e-navigation domain.

A limited testbed version of the Maritime Cloud concept exists, which has so far demonstrated interoperable information exchange between systems developed by different e-navigation testbed projects in Northern Europe and Korea. A more complete instance is planned to be established during the period 2015-2018 by the EfficienSea 2 project, with the intention of providing large scale validation of the Maritime Cloud concept itself, as well as concrete services enabled by it, laying the foundation for establishing a model for governance, operation and evolutionary standardization of services based on a maritime infrastructure framework.

## Purpose of the document

In response to e-Nav15 Action Item 37, and Task 5.1.19 of the e-navigation committee, this paper describes the Maritime Cloud concept, as a foundation for e-navigation, elaborating on how the core services will support those solutions prioritized by IMO.

## Related documents

- ENAV15_16_1_report_enav15 – report of IALA e-NAV15

- ENAV16-9.24: Jensen J.K., Lind M., Park J.H., Lee K., Setterberg P. (2015) A maritime infrastructure framework

- ENAV16-9.22: Lind M., Jensen J. K., Lee K., Park J.H., Setterberg P. (2015) A Federated Approach to Distributed Service Interaction in Sea Traffic Management

# Background

The IMO strategy for e-navigation requested “A communication infrastructure providing authorized seamless information transfer on board ships, between ships, between ship and shore and between shore authorities and other parties with many related benefits.” (MSC 85/26/Add.1, Annex 20 + 21).

Based on the experience of several e-navigation test bed projects in Europe (EfficienSea, MonaLisa, and ACCSEAS) as well as projects in Korea and Japan, the concept of the Maritime Cloud has been developed into an open source functional prototype.

In the years 2015-2018 a project called EfficienSea 2 will initialize a realization of the Core Services of the Maritime Cloud, in order to perform large scale validation of the infrastructure concept itself, as well as validation of concrete services in the Baltic Sea Region. During that period, world wide collaboration, and commitment of other stakeholders and federations outside e-navigation, will be sought, to enable evolution of a sustainable community around the Maritime Cloud.

# Discussion

This section will discuss functional requirements derived from the e-navigation strategy process related to the infrastructure, look at the basic function of the Maritime Cloud, and how the core services of the Maritime Cloud meet these requirements.

## Prioritized solutions for e-navigation – the derived functional requirements

The IMO e-navigation Strategy Implementation Plan points out 5 prioritized solutions:

- Improved, harmonized and user-friendly bridge design

- Means for standardized and automated reporting

- Improved reliability, resilience and integrity of bridge equipment and navigation information

- Integration and presentation of available information in graphical displays received via communication equipment

- Improved communication of VTS service portfolio

While the Maritime Cloud concept only has little, indirect impact on improved, harmonized and user-friendly bridge design, the last four solutions provide insight into the relevant requirements for a supporting communication infrastructure.

### Means for Standardized and automated reporting

Timely arrival of information is a vital component in international transport. While much work in the IMO Facilitation Committee has gone towards developing EDI standards for reporting, todays information flow is in a transition between paper documents and electronic information transfer.

While most of the FAL reporting has little to do with the act of navigation, it is a related administrative task posing a risk to distract navigators from paying the required attention to the navigational situation.

Three primary challenges exist:

- Standardization of reporting services, including data structures  and data messaging formats for FAL reporting, technical interfacing protocols and conversation sequences for automated reporting systems

- A globally applicable identity concept, covering ships, ports and other organizations involved in automated reporting services, including support for ‘empowered employees’ authorized to sign reports on behalf of organizations.

- Supporting mechanisms for encrypted information transfer and verification of Identity and Integrity of provided reporting information (electronic equivalents of ships stamp and captains signature), to ensure confidentiality and non-repudiation of reported information.

- A transition mechanism enabling dynamic discovery of which Ports or National Single Window solutions support standardized (or other) types of reporting services.

### Improved reliability, resilience and integrity of bridge equipment and navigation information

The Maritime Cloud will have no influence on reliability, resilience and integrity of bridge equipment, but where navigation information is provided from outside theship, mechanisms for validating authenticity and integrity are important.

For instance, an Electronic Nautical Chart or and update to an ENC should be treated with caution. Does it originate from a trustworthy source – or could it originate from a source with insufficient or no Quality Assurance procedures in place, or a deliberate intention of misguidance?

When a NAVWARN is broadcast, it may influence decisions of navigators, and users require trust in the authenticity and integrity of the information. Currently, the perceived trust may depend solely on the channel through which information was received. If received via e-mail, or AIS safety related text, the exact same text may be perceived as less trustworthy, than if received via NAVTEX, although no more secure.

In terms of providing resilience and fault tolerance, a mechanism for assuring message delivery

### Integration and presentation of available information in graphical displays received via communication equipment

Enabling integration of information - such as MSI or route information of other ships - received via communication equipment into presentation on graphical displays, requires standardized data structures and presentation libraries. Furthermore, the ability to verify authenticity and integrity influence the ability to indicate the trustworthiness of information received.

AIS AtoN (electronic Aids to Navigation) can be considered an early example of provision of electronic information services dedicated to be presented on ships navigation related graphical displays. Introduction of AIS AtoN services have proven to suffer from the lack of ability to evolve the service (data structures and presentation libraries), based on the feedback from users. Service providers further lack the ability to verify whether the equipment of the intended users actually support such a service, or whether provision of the service is futile.

### Improved communication of VTS service portfolio

VTS services potentially constitute a vast multitude of different services in different areas. Services related to weather forecast or real time oceanographic information, traffic conditions or special restrictions may exist in many forms, and their accessibility may depend on communication capabilities, and the equipment capabilities of onboard ships.

Improved communication of the VTS service portfolio will thus be closely associated with the ability to discover and use services, based on the capabilities of on board equipment and the services available.

Navigational Assistance Service or Traffic Organization Service involves a high degree of ship/shore interaction, requiring a high level of trust in authenticity and integrity of the information exchanged.

VTS functions are frequently associated with Ship Reporting Systems (SRS), collecting information from ships to achieve a complete domain awareness picture. These reporting systems often rely on manual processes, partially due to the lack of standardized data structures and protocols for automated SRS reporting service.

Collection of the primarily similar information from ships that pass through several VTS areas may be at risk of distracting navigators, and could be automated, provided that an access point for the standardized SRS service and the information requirements are published for each SRS, leaving passage of a SRS to a less distracting exercise, where the VTS operators can focus on exercising the ‘policeman effect’, getting an impression of the alertness of the OOW.

Further the improved communication of the VTS service portfolio requires availability of data links available to both VTS and ships, or communication services enabling “seamless roaming” across different data links, supporting a secure communication channel).

### General domain specific considerations

A general observation of the legacy of the maritime domain reveals that universal availability of interoperable digital communication capabilities that can facilitate automated information exchange with systems on ships bridges is a challenge to shipping. While the GMDSS guarantees that distress, urgency and safety related information can be communicated via dedicated and interoperable systems globally, these systems are primarily based on voice communication or transfer of text messages to be read and understood by humans, rather than designed for automated transfer of machine readable data structures for system integration.

While multiple communication systems are available, which may provide internet connectivity to ships on commercial terms, availability may vary greatly and bandwidth comes at a significant cost. Maintaining seamless access to information services while roaming across multiple datalinks with different characteristics, is not trivial.

When introducing external digital information transfer into safety critical systems such as navigation related systems on ships bridges, cyber security becomes an issue. While internet connectivity is great for enabling information transfer, appropriate firewalling and other security measures must be applied to prevent hackers, viruses or other cyber threats from affecting maritime safety.

## Overview of how the Maritime Cloud works

Figure 1 - The core elements of the Maritime Cloud: Maritime Identity Registry, Maritime Service Portfolio Registry and Maritime Messaging Service

Identity registration and vetting

An Actor (a shipping company, a ship, a VTS center, a port, an agent, a service provider, a leisure boat or an empowered employee of a company) may be registered in the Maritime Identity Registry.

A Maritime Identity may be Vetted by peers, by Organizations which the Actor is empowered by, or by Authorities (which again may be vetted by federations or high level organizations). Vetting will indicate the level of trust, and lock the key identifying parameters of a Maritime Identity, but the Actor may maintain own basic contact information. Public information related to a Vetted Identity may be published in a digital publication called the e-navigation ALMANAC.

Service Registration and vetting

Based on a standard for Service Specification, any provider of an information Service may register a Service and it’s endpoint (point of access), based on a Service Specification.

An MSI provider may register different MSI services for an area. A port may register a webpage with port information, or a reporting service based on FAL standardized reports. A VTS center may register a multitude of VTS services. An Agent may register a proprietary web service or a ship may register a route exchange or text messaging capability. Services may be Vetted in similar fashions as identities, and published in the ALMANAC.

Figure 2 - How e-navigation fits into the Maritime Cloud (infrastructure framework) concept

Seamless roaming facilitated by the Maritime Messaging Service

Ships may roam seamlessly between communication links, using the Maritime Messaging Service (MMS) as a supplementary, global interoperability service. It may be used for exchanging information with a shorebased stakeholder or confidential messages with another ship, where other peer-to-peer links are insufficient, regardless of distance and availability of identical communication links. Using the ALMANAC, ships may discover available services in a distant port, or different levels of local as well as NAVAREA MSI services and poll for updates. The MSI service provider may utilize the MMS to perform a MSI ‘geocast’ (broadcast to an area, independent of datalink) to ships in the area that subscribe to MSI services. MSI received via standardized services will be presentable on graphical display systems.

The vetted identities and related public/private key infrastructure – even available offline via the ALMANAC - can be used for managing access control to a proprietary system, authentication of identity and encryption of data transfer.

## How the Maritime Cloud support the requirements of e-navigation

### Identity management – the Maritime Identity Registry

All maritime stakeholders may communicate with each other through maritime cloud. For the communication, each stakeholder shall be identified uniquely and globally. There are two issues related with Maritime Identity. First is how to assign the identifying keys uniquely for all maritime stakeholders. The second is how to verify and validate the identity of the stakeholders.

Identification keys like ships IMO number, name, callsign, MMSI number, phone number or an e-mail address may offer a limited level of confidence in whom information is being exchanged with, but relies on trust in that the other party of a conversation is who he says he is. However, none of these identifying keys are universal to all stakeholders – not all ships have an e-mail address, Ports don’t have IMO numbers, most stakeholders ashore do not have MMSI numbers.

In order to support different stakeholders with different communication capabilities, a uniform identity concept is needed, where identities are not related to the choice of data link, and can be vetted by relevant federations or authorities to achieve a ‘chain of trust’.

Current identifying keys like ships IMO number, name, callsign, MMSI number, phone number cannot by themselves be used for secure authentication in a conversation, which will satisfy any trustworthy level of access control.

The introduction of AIS – the Automatic Identification System - was primarily aimed at aiding the identification of nearby ships. The introduction of AIS has however proven, how easy it is - intentionally or unintentionally - to provide a ‘false’ identity in a fully cooperative system.

Combined with mechanisms supporting verification of authenticity and integrity, as well as confidentiality, this will enable authorized and secure exchange of, or access to, sensitive information. These also can be provided when maintaining authenticity of identities of stakeholders between heterogeneous data links.

The Maritime Cloud will provide different levels of support for authentication and encryption, as obviously not all e-navigation services will require strong encryption or widespread use of digital signatures.

Public services such as MSI promulgation would not require authentication to access, but the authenticity of content might need to be verifiable. In some situation, more comprehensive multi-level mechanisms might be needed, for instance delivering authentication similar to ‘ships stamp’ and ‘captains signature’ for non-repudiation in a digital FAL reporting scheme.

These mechanism to will support e-navigation requirements by enabling verification of identity, validation of authenticity and integrity, confidentiality, non-repudiation, indication of reliability and integrity of navigation information. This may be achieved by maritime public key infrastructure (Maritime PKI). PKI is implemented in LRIT, but is applicable only to LRIT. In the Maritime Cloud, it shall be applied to accommodate all different kinds of stakeholders. So, general and scalable PKI infrastructure for maritime domain shall be provided in Maritime Cloud.

### Service portfolio management – The Maritime Service Portfolio Registry

The Maritime Service Portfolio Registry contains service specifications and lists provisioned service instances. This enables service providers, consumers, and regulatory authorities to share a common view. The service registry does not provide maritime information but a specification of services and the information they carry, plus the technical interface to obtain it. The service registry enables the “provider” to “publish” its service instances so that “consumers” are able to “discover” them and obtain interface information required to use a service. Furthermore, the service registry may have descriptions on how to “automatically and digitally” use the services by providing the users with information on essential procedures and requirements of the services.

This way, the Maritime Service Portfolio Registry provides a universal place for publication of standardized services, as well as a transition mechanism enabling dynamic discovery of ports, VTS or other stakeholders providing an endpoint for an automated Ship Reporting System.

A ‘service’ may span from non-digital operational services (a VHF working channel and coverage area or phone number of a VTS) to a technical digital interface definition of a service enabling automated transfer of information in an automated Ship Reporting System using standardized machine readable data structures, enabling system integration.

In other words, the ‘service’ concept supports publication and discovery of existing systems and procedures as well as future e-navigation solutions.

Figure 3 - Service development, deployment, discovery and use

Solutions envisaged in the IMO e-navigation Strategy Implementation Plan are not the only use cases that could benefit from the existence of such a service infrastructure.  For instance several stakeholders within the port environment could publish services targeted other shore based stakeholders, or within other modes of transport.

Highly standardized e-navigation services are not the only usecases that could benefit from the service registry. The registry will allow standardized as well as proprietary or regional services to coexist, meeting regional needs or market demands. This will further support an evolutionary standardization – where proof-of-concept, harmonization and adoption of services preceed publication as standard, rather than service design by standardization committee, before technical implementation. This process will allow user evaluation and feedback to be part of the standardizations process. Once a service specification has evolved into such maturity that it has been adopted by a relevant federation of interested stakeholders – such as an SRS service specification based on FAL forms recommended by IMO or a service specification adhering to an IALA or IHO recommendation - a service specification may become part of relevant standards related to that federation. In turn, the relevant governing federations may perform vetting of service providers adherence to relevant quality standards, enabling trust in the reliability and quality of a given service instance.

### The ALMANAC

A dynamically updated, downloadable, digital publication called ‘the ALMANAC’ will contain registered Identities, published contact information and service instances, plus public keys for encryption. This way central framework functions like service discovery and initiation of secure peer-to-peer communication between registered parties can be available without requiring online access to the central registries.

While all Actors should be able to register their identity or service in the registries, exposing a service to be discovered through online access to the registries, qualitative entry barriers may be applied before identities or services are published in the e-navigation Almanac. This could be similar to how APP’s in the Appstore need to be certified to conform to certain testing criteria, ensuring a certain level of quality. This mechanism would ensure that the (e-navigation) Almanac will reflect only those services – and service providers – that conform to relevant international standards and have been approved for e-navigation.

### Maritime Messaging Service (MMS)

The MMS is a carrier agnostic messaging hub, designed to allow seamless roaming. It enables global interoperable connectivity across varying data links with varying technical characteristics and limited bandwidth. Bandwidth efficiency and fault tolerance in an information transfer session are the prime design criteria for those services designed for provision via the MMS.  It will be used as an option for transporting standard information ‘Containers’ (Common Maritime Message), designed to take lessons learned from AIS ASM on encoding, software quality and the need for an evolutionary standardization process into account. The MMS is not intended to carry ALL future maritime communication, but to provide an enabler for seamless roaming, and for initiating a secure, digital conversation, regardless of choice of communication provider. The MMS may utilize the internet as a carrier, but gateways to other communication links may facilitate seamless roaming across alternative data links.

The MMS will enable a geo-aware protocol for ‘geocasting’ – i.e. actors can ‘listen’ (subscribe) to - a service in an area, similar to how a radio broadcast has a coverage area where subscribers may listen in – but in this case independent of each actors communication channel. This will provide a basis for interoperability regardless of communication capability.

Figure 4 - Geocasting - a logical broadcast independent on communication channel

### ‘Maritime Cloud Client Component’ or ‘Maritime Cloud Service Agent’

Previous testbed projects have developed draft reference implementations of ‘Maritime Cloud Client Components’ or ‘- Service Agents’. They are software components which may be introduced in an environment of existing systems or build into new systems, facilitating the integration and utilization of the core elements (Identity and Service registries and MMS) in the Maritime Cloud, and provide a . The role of this kind of interface component is to encapsulate the complexities of using the central framework to lookup identity information, discover services and roam seamlessly while communicating with services published via the MMS. Furthermore, this component is intended to be the component, which holds the Maritime Identity (including private and publics keys for encryption and digital signatures) of a given actor.

Such reference implementations may be further evolved to fit direct integration of existing systems, or simply enable rapid development and testing of own integration software components. Further supporting an evolutionary standardization approach, such reference implementations of interfacing components should be maintained in sync with the evolution of the core services of the Maritime Cloud, and may provide stable local interfaces to an evolving service infrastructure.

The role or functions of the ‘Marititime Cloud Client Component (or Service Agent) is closely related to ongoing standardization activities in IEC related to onboard systems, namely the Draft IEC 61162-460 (related to the safety and security of Ethernet interconnections) and Draft IEC 92940 (Maritime Integrated Communication System). The relevant relationship with these standards will have to be defined. Where the Draft IEC 61162-460 requires all connections to non-controlled networks to be through encrypted VPN tunnels, the Almanac function will enable lookup of public keys for initiating an encrypted connection with other stakeholders registered in the Maritime Cloud.

In principle, one Integrated Communication System device could include all functions needed to interface all communication links, including an IEC61162-460 gateway interfacing internet connections regardless of links (Satellite broadband, 3G/4G cellular, Wifi,…)

Figure 5 - The role of the Maritime Cloud Client Component (or Service Agent)

Seamless roaming example - Text messaging

A simple text message service is a good example to illustrate seamless roaming. Today, ships (or shore based stakeholders with access to an AIS Base Station) may communicate peer to peer – but only when utilizing compatible links. The Maritime Cloud concept will enable development of a local ‘Text chat’ service component, which may roam between different links, to transfer a text message, depending on availability and cost. If the addressed stakeholder is in AIS range, AIS may be used for the transfer.  If not, and one ship has an INMARSAT terminal, and another ship has a VSAT, but both support the Text Chat Service, one ship can send a message through the INMARSAT to a MMS Gateway, which is transported via the MMS and internet through another ships VSAT, and appear on the other ships text communication display. In addition, seamless data exchange between different onboard/onshore systems would be also available with seamless roaming.

Figure 6 - The simplest example of seamless roaming: Test messaging

### Example: Three variants of an MSI service

Existing Maritime Safety Information (MSI) service providers, such as a local Maritime Authority or the NAVTEX coordinator of a NAVAREA, may often provide MSI in different formats, depending on the priority and local or regional nature of the MSI.

NAVTEX will be used for important, regional MSI, local warnings may be distributed through other means such as webpages or voice VHF. The MMS will enable a complementary ‘Geocasting’ capability, enabling the distribution of MSI through multiple data links at the choice of the ship – or future data links such as VDES - and the Maritime Cloud Client Component may host a local MSI service component, able to receive NAVTEX messages as well as MSI in S-100 based data structures, perform doublet filtering, and make the MSI available to multiple workstations on a ship.

Figure 7 - How an existing MSI service may be complemented

# Initial realization of the Maritime Cloud

The European Commission has granted 10 million Euro for a three year project called ‘EfficienSea 2’, collecting 32 partners including IALA, CIRM, BIMCO, authorities, academia and significant industry partners. The project aims to develop the initial core services based on the requirements derived for e-navigation and establish an instance of the Maritime Cloud

It is the ambition within that project to demonstrate scalability and interoperability – also with other initiatives, such as other regional projects, which have been proposed. In cooperation with e-navigation testbeds worldwide, an instance of the Maritime Cloud, enabling development, demonstration and large scale validation of services developed for e-navigation and e-maritime in the Baltic Sea Region and the Arctic. It is the ambition that a fully operational instance of this infrastructure shall evolve out of this initiative, provided that a sustainable community with appropriate governance can be established around it.

Figure 8 - the world wide collaboration between testbeds, facilitated by a common service infrastructure for demonstration, will evolve into an operational infrastructure for e-navigation

Planning of workshops (and potentially a founding conference) to define a sustainable governing community and business model for the infrastructure, based on the emerging international collaboration on e-navigation testbeds, will be initiated as part of the EfficienSea 2 project.

Other projects are expected to be initiated within the next few years, which will cooperate on establishing or utilizing the infrastructure, and perform large scale validation of service provision based on this concept. But the potential value of the service infrastructure will not be fully released, until an operational environment with proper governing structures and evolutionary processes to guarantee its long term sustainability have been established.

# Conclusions

- To support the e-navigation strategy for global maritime transport, the Maritime Cloud offers a Maritime Identity concept for all participating stakeholders in a common framework.

- Information services for e-navigation will be published in a dynamic registry, available for discovery by relevant stakeholders.

- Access to lookup services of the Identity and Service registries may be available online or as an offline (cached) ALMANAC.

- E-navigation Services must be based on published, standardized service specifications.

- Interoperability between heterogeneous data links, which means independence on a single communication carrier, is facilitated by the Maritime Messaging Service.

- Proprietary and regional services should be supported by the same infrastructure, and an evolutionary process for standardization should be applied, for promoting proof of concept and harmonization before a service specification reaches the level of standard.

- An opportunity for placing the Maritime Cloud Client Component (or Service Agent) into the ships bridge architecture exists in the ongoing standardization processes for IEC 61162-460 and IEC 92940.

- While a window of opportunity for initializing the establishment of the Maritime Cloud exists, a governing structure, business model and operational environment with related evolutionary processes for cooperation amongst many different projects and stakeholders will have to be established for the Maritime Cloud (or maritime infrastructure framework) to ensure long term sustainability.

# References

https://dma-enav.atlassian.net/wiki/display/MCCT/Overview

DRAFT IEC 61162-460 Ed.1 (CDV stage): Maritime Navigation and Radiocommunication Equipment and Systems – Digital Interfaces – Part 460: Multiple talkers and multiple listeners – Ethernet interconnection – Safety and Security.

DRAFT IEC 92940 (CD stage): Maritime Navigation and Radiocommunication Equipment and Systems – Integrated Communication System (ICS) – Operational and performance requirements, methods of testing and required test results

# Action requested of the Committee

The Committee is requested to:

Take note of the information provided

Discuss any appropriate implications in the Harmonization WG