International Association of Marine Aids to Navigation and Lighthouse Authorities

AISM Association Internationale de Signalisation Maritime     IALA

10, rue des Gaudines

78100 Saint Germain en Laye, France

Telephone: +33 1 34 51 70 01    Fax: +33 1 34 51 82 05

e-mail: contact@iala-aism.org       Internet:  www.iala-aism.org

IALA Guideline on

Machine-to-Machine Interface within Common Shore-based System (CSS)

[Working towards] Edition No. 1

[2015]

[Initial Version]

Appendix Revisions

Revisions to this Appendix are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

# Guideline on

Machine-to-Machine Interface within

Common Shore-based System (CSS)

This guideline contains interfaces between machines, systems, and services in Common Shore-based system architecture. This also security some issues to provide safe and secure interfaces among them machine-to-machine interface.

# Scope

This Machine-to-Machine interface should be provided between “Data collection and Data transfer Services” and “Value-added Data Processing Services”, “User Interaction Services” and “Gateway Services”.

# Terms and definitions

## Service

Each technical service provides a functional (M2M) interface to any other technical service of their own CSS, i.e. on their ‘domestic’ side. This interface provides the respective functions of a technical service as a set of Basic Services specific to that technical service.

It should be noted, that data exchange between different technical services is exclusively done by so-called Service Couplers and only in so-called Node sites, e.g. using LAN technologies at the Nodes. This has at least two important benefits: The wide-area networking topologies of the different technical services, which may be different from each other, are encapsulated within the service. Also, LANs allow for maximum and highly cost-efficient performance of data exchange at the Node sites.

A service performs a specific function within the CSSA. It can be thought of as a black box that receives inputs, processes them and transmits outputs. The inputs and outputs can be analog or digital data streams. A service typically processes inputs using an application program. This program runs on an operating system and may use a database of some kind. The operating system runs on a processor (CPU) which could be embedded in a device. A service has a number of characteristics:

### Identity

It has a unique identifier. For authentication purposes this identifier may be an authorization certificate (X.509. RFC 5755). To allow encryption of inputs and outputs a service may also have a Public Key Certificate. The system administrator is responsible for assigning certificates and renewing them periodically.

### Vendor(s)

This includes vendor information for the device that hosts the service, for the operating system as well as for the application program. The vendor information should include contact information. It also could include information about the installer, the warranty and details about the support contract (if any)

### Application program

This describes what the program does and provides the URL to the manual. It also lists the version of the currently used application program. Additional information may be the log of who performed the upgrade from the previous version, what the previous version was and a time stamp of completion of the upgrade.

### Operating System

This is the make, model and version of the operating system. Additional information may be the log of who performed the upgrade from the previous version, what the previous version was, and a time stamp of the completion of the upgrade.

### Hardware

This is the make, model and part number of the hardware. It may include the URL for the manual, when the hardware was installed, who installed it and a log of modifications that were made to the hardware. It may include a log of inspections and the findings.

### Description

This is an XML file that describes the service in detail. It includes information on how its output is encoded and detailed information about each of the data streams it generates. It also specifies the access rights for the various roles that subscribers can have (i.e. “Public”, “Basic”or “Admin”). The XML file could include the S-100 Product Specification for the service. It also describes the results of the latest Built-In-Integrity Test (BIIT) or other health indication (i.e. heartbeat), if available. It furthermore should provide a description of the accuracy of the values that are included in the data stream(s) that the service generates.

### Summary Description

The summary describes the nature of the service at a high level. The summary description should use the Simple Service Discovery Protocol (SSDP). It is an advertisement for the service that is broadcast over the network via UDP and is used by potential subscribers to select subscriptions and then request them.

### Access Control List

This list specifies the ID’s of the authorized service subscribers. It is maintained by the system administrator. Each entry also specifies the role that the system administrator assigned to the subscriber. (i.e. “Public”, “Basic” or “Admin”)

## Actuators

Services may include actuators that allow remote control for instance of CCTV cameras or simple on/off actuators to control fog horns.

## Digital Data Stream

A data stream consists of digital information. Services generate outputs as a digital data stream on the CSS local area network. In most cases the inputs to services are data streams that come to the technical service via its connection to the CSS LAN. Data streams can be broadcast on the network but more typically they are distributed by one technical service to specific other technical services who subscribe to the data stream for further processing.

It main characteristics are

### Origin

### Its origin or source is typically a service within the CSSA

### Destination

Its destination or sink is typically a service that consumes the data stream

### Volume

The volume of a data stream of is characterized by its average throughput in Kilo Bits Per Second (KBPS) along with its peak volume.

### Encoding

A data stream consists of digital information that is typically encoded to compress its volume and still allow it to convey all information. Typical encoding methods include IEC 61162-x including AIS basic messages, AIS Specific Messages and other encoding schemas that are governed by an internationally recognized standard. Encoding schemas may also include any protocols that are specific to a service but that are not governed by an international standard.

### Encryption

A data stream may be encrypted for security reasons. A typical encryption schema is Transport Layer Security (TLS).

## Subscription

A subscription governs the relationship between the service that generates a data stream and the service that consumes it.

This distribution of data streams can be thought of as subscriptions. Some technical services subscribe to the data stream “service” that others publish. In Information Technology terms this is called a “publish and subscribe model”. Its characteristics are:

### Identification of the publisher of the datastream

### Identification of the subscriber (i.e. the subscribing service)

### Identification of the datastream itself

### Subscription details

#### Expiration Date (if any)

#### Cost (if any)

#### Other Subscription Contract details

### Subscription Set Up

Subscriptions have to be set-up. It involves:

#### ACL

Part of the set-up is to populate the ACL of the publisher (i.e. the service that generates the data stream). The ACL needs to include the authentication information of all subscribers (i.e. other services that are allowed to subscribe to the service). It also requires assignment of a role (Public, Basic or Admin) of each subscriber and a specification of the access rights associated with each role.

#### QoS

Another part of the set-up is to allocate bandwith on the CSS LAN so that the data stream has a dedicated “pipeline” that ensures that the data stream delivery to the subscriber runs smoothly with minimal delays. In IT terms such delays are called latency. Setting up such a pipeline is called Quality of Service or simply QoS. If the technical service that publishes the data stream fails and starts generating a large data stream (of garbage information) then the pipeline won’t big enough to handle that volume but at least it won’t affect other data streams that run through the CSS LAN. In IT terms this is called flow control.

## ACL

## Role

The system administrator assigns a role to each authenticated subscriber (subscribing service) of a service that generates a data stream. The roles typically are:

### Publuc

This role often means “Read Only” access but could be “No Access”

### Basic

This role typically allows the subscriber access to data and actuators that are part of the regular operation of the service (i.e. operating a remote CCTV Camera)

### Admin

This role typically is reserved for system administrators, maintenance and technical personnel. It typically allows them to start/stop a service, to configure the service and even to upgrade the application software or the operating system.

## Discovery

## SSDP

## Technical Service Description

## QoS

## TLS

## Gateway Service

- The Gateway Service (GWY) – another individual technical service – specialises in data exchange shore-to-shore. It interfaces mainly to external systems of ‘third parties.’ External systems provide data upon request by own system and/or receive access to relevant data from own system, upon legitimate request. The Gateway Service can also interface different shore-based systems locally, regionally, and globally.

# High level Goals and requirements

## General

There are requirements which are almost universally applicable and are described here as a reference. Some of these may not be applicable to M2M interfaces. However, all of these must be considered.

These terms are in common use so they are not defined here.

Scalability

Interoperability

Flexibility

Modularity

Reliability / continuity / availability

Latency

Maintainability / Life-cycle-friendliness / future-proofing

Security

Integrity

Survivability / robustness

Safety

Seamlessness

Verifiability / validation

Usability

Extensibility

Inclusivity

Consistency

## Interface Design Principles

To achieve the above goals the following fundamental principles for the CSSA Technical Service Interface Standard should be used:

- User-requirement driven design: Only clearly and consistently specified user requirements should be addressed by the interface standard.

- Introduction of the OEP for all technical aspects.

- Employment of the principles of modularity and encapsulation, while preserving a holistic view of the system’s intended functionality.

- Application of a uniform model for all technical services provided by the system, regardless of technology, thus exploiting commonality especially at their interfaces.

- Implementation of functional specifications, based on international standards to the largest extent possible: Strive to procure technical solutions based on functional specifications.

- Adherence to open system architecture and focus on open and standardized interfaces between components and services: Strive to avoid proprietary interfaces.

- Employment of remote access techniques where feasible in order to allow for minimal number of technical operation and maintenance centres: Components without remote access capabilities should be avoided (access capabilities should allow for open interfacing, too).

- Implementation of life-cycle management in order to prevent “quick-fix-solutions” with associated long-term costs: Full life-cycle impact of technical proposals should be required to obtain their approval.

- Documentation of each and every functional aspect in a uniform, comprehensive documentation system: This is a pre-requisite for any quality management system.

- Separation of technical operational and maintenance personnel and their roles on one hand (they are not authorized to change functionality and/or structure of component) and system development and optimization personnel and their roles on the other hand (they are authorized to change service functionality and/or structure).

- Take into account regulatory constraints when creating the system architecture, but there may be consequential requirements to amend existing or create new regulations based on the development of that system architecture.

## Migration to a n e-navigation compliant CSSA

IMO has stated that e-Navigation implementation will take place in phases. The first phase will be to integrate existing technology and systems.  For IALA members this means that the current technical environment will still be relevant as IALA members should strive to move towards a ‘shore-based system architecture harmonized for e-Navigation’. However, in the usual courses of upgrading, recapitalization, changing user requirements and new regulations, the IALA member will eventually become ‘fit for e-Navigation’.

## Unbundling

- Developing a standard for the interfaces between technical services is expected to result in unbundling services that are currently sold by vendors as a bundle. The services included in such bundles typically use proprietary interfaces and often cause IALA National Members to be locked into these vendors for maintenance and upgrades. Vendors can and often do charge a premium for maintenance and upgrades. Unbundling the services is expected to improve the position of IALA National Members when they negotiate maintenance, support and upgrade contracts with vendors. It also is expected to allow IALA National Members to Mix-n-Match the best in class technical services rather than be forced to use bundled services that may include individual technical services that do not meet their requirements.

Developing a standard for the interfaces between services is also expected to create an ‘innovation friendly eco-system’ for technical services and their vendors:

It would expand the market for individual technical services. This is expected to increase the supply, i.e. to increase the number of solutions available to IALA National Members to select from for each technical service. This, in turn, is expected to reduce their cost and simultaneously improve their quality.

Technical service developers would be able to rely on well-proven, reliable and already available other technical services (i.e. Data Collection Services, Data Transfer Services, Value Added Data Processing Services and Application Services) and thus avoid having to develop their own (proprietary) ones. This, too, is expected to significantly reduce the cost of developing technical services and improve their quality.

## Future proofing the CSS

Technology, user requirements and the regulatory environment will change over time therefore the CSS architecture needs to be flexible enough to accommodate such changes and allow smooth migration to new or upgraded technical services. The life cycle costs associated with maintaining and upgrading a CSS needs to be minimized.

Technical Services need to interact with other technical services and yet be autonomous to make the CSS future proof by accommodating upgrades of individual technical services without having to change all the other technical services it interacts with. In that sense technical services need to act as building blocks for the CSS as a whole but allow for replacing or upgrading the blocks without or with minimum system downtime.

Introducing a new or upgraded technical service into the CSS needs to be controlled from a security point of view by specifying which other services are allowed to subscribe to it, however, actual configuration of the interface between two services needs to be seamless and automatic.

## Remote Maintenance

To contain life cycle costs and to improve system availability (i.e. “up-time”), remote maintenance and administration of a CSS service should be possible. This should include remote configuration and remote upgrading of operating and application software.

## Use Existing Standards

The technical service interface standard should be based, as much as possible, on publicly available, proven and widely used international standards. The advantages of using existing interface standards are:

- Re-use of interface software components that are proven and maintained by professionals in the industry rather than re-inventing the wheel. Re-use of components can be expected to reduce the cost and improve the quality of the interface.

- Ready availability of test tools to verify that a particular vendor’s implementation of the standard interface complies with the standard.

- Ability to take advantage of newer versions of the standard that adapts it to changes in technology (i.e. Internet of Things or IoT).

## Interface Standard Certification

The M2M Technical Service Interface Standard should support concepts such as certification in general, IT security certification in accordance with ISO 27000 series, and also the IMO Member State Audit Scheme (IMSAS).

## Ease of adding standard M2M interface to existing services

It should be possible to change the interface between existing services. The standard should use resources that existing services can be expected have available:

### Support for an Internet Protocol (IP) based network connection (IPv4 or IPv6)

### Support for Hypertext Transfer protocol (HTTP)

# Selection of available m2m interface standard

## Available M2M Interface Standards

The following publicly available, proven and widely used existing international standards are available:

### UPnP+

UPnP+ protocols provide readily available implementations for inter-vendor operability between devices, systems and services from different vendorsThe UPnP Forum published a number of “Device Control Protocols” (DCP’s) and Services.

The most relevant ones are listed below. Note that “Device Protection” and “Quality of Service” provide practical publicly available implementation of a significant number of IEC 61162-460 Secure LAN requirements (i.e. authentication, encryption, access control, flow control, etc.)

UPnP+ protocols are secure, extensible and future proof. They allow for seamless zero configuration establishment of M2M communications. They are modular allowing mixing and matching of best in class devices, systems and services thus avoiding vendor lock-in.

UPnP+ protocols can be used to remotely trouble shoot and fix issues without requiring down-time of the network and they are congruent with evolving standards for the Maritime Cloud Messaging System, S-100 XML, IEC 61162-460 Secure LAN and for the Internet of Things (IoT).

The UPnP+ standard uses “Devices” instead of publishers/services that generate a data stream and “Control Points” instead of subscribing services that consume data streams.

#### Basic Device Control Protocol

A minimal Basic Device (one that does not add any services or embedded devices) is discoverable using UPnP discovery and may provide identifying information and a Presentation URL using the normal Device Description mechanism.

The Basic Device type also may be useful as the root device type for a product that incorporates standard elements in ways not anticipated by the standard types

The detailed specification is published at www.upnp.org at Basic Device Control Protocol

#### Sensor Management

This specification defines a general-purpose device that can be used to instantiate a bridge or gateway device that is connected to Sensors and Actuators. This device implements a bridge between the UPnP network and Sensors/Actuators endpoints. It exposes services to check the status, and send and receive data from Sensors. Sensors/Actuators may be connected using a variety of transport protocols including Bluetooth, ZigBee and IEEE 11073. The device defined herein provides the following capabilities:

##### Access and control of multiple Sensors and Actuators

##### Expose configuration information from Sensors and Actuators.

Figure 1 — SensorManagement Device Functional Diagram

The shaded boxes represent UPnP services that are contained by the SensorManagement

device. The un-shaded boxes represent device specific modules that are vendor specific.

A SensorManagement device allows a control point to read from and write data to Sensors or Actuators. SensorManagement devices can be used in conjunction with one or more UPnP devices to store sensor’s data using a DataStore service. The process starts when a control point discovers a SensorManagement device. The control point can read and write data directly to SensorManagement device using SensorTransportGeneric service. The control point can also connect a SensorManagement device to an external storage device using SensorTransportGeneric::ConnectSensor() and SensorTransportGeneric DisconnectSensor() actions. The data transfer is performed directly by the SensorMangement device and the other device. The data transfer happens independently from the control point. The control point uses UPnP to setup the data transfer, but the transfer is performed using HTTP based transfer protocol. Therefore, the main operations performed by SensorManagement devices are:

- Discovery, configuration and status reading of sensors and actuators.

- Reading and writing of data to sensors or actuators using SensorTransportGeneric service.

- Connecting of SensorManagement to external storage devices, where these devices can be UPnP devices or any other type of device supporting HTTP transport.

- Provide allowed access control based on privacy and security settings.

The detailed specification is published at www.upnp.org at Sensor Management

#### Device Protection

The DeviceProtection service enables Devices to provide privacy and restrict access to sensitive operations to authorized Control Points. When a Control Point wishes to establish a protected connection with a device, it discovers the HTTPS base URL (as described in Section 2.3.1 in the linked document) and initiates a TLS connection to that URL. The Device will provide its Device certificate during the handshake and also request the Control Point’s certificate. Retrieval of device and service description documents and any SOAP actions invoked over this connection will be encrypted and authenticated by the TLS channel. If the Device has no prior knowledge of the Control Point’s certificate (i.e., the hash of the certificate does not correspond to any of the <CP><ID> values in the Device’s ACL), the connection, then the Role associated with the connection will be Public.

Trust in the certificates is established through pair-wise secure introduction or by introduction by an authorized control point. The DeviceProtection access control model is role-based, where Devices support a set of name-based Roles that are mapped internally in accordance with DCP and manufacturer policy onto UPnP SOAP actions. Control Points are assigned Roles in each Device’s Access Control List.

If a Control Point discovers a Device that supports DeviceProtection, the first question to answer is whether the Control Point needs to use any actions that require authentication and authorization. If not, then the Control Point does not need to use DeviceProtection at all. It can simply operate as if it were a legacy Control Point, using only those actions that the Device makes available to the role Public and that are accessible through the normal control URL. However, if the Control Point needs to use protected actions, then it MUST:

##### Have a digital certificate

##### Have the Role(s) it needs to perform protected actions on the Device (note that the required Role may also be dependent upon argument values)

##### Establish a TLS connection with the Device

##### Invoke the desired actions

The detailed specification is published at www.upnp.org at DeviceProtection

#### Configuration Management

The ConfigurationManagement Service (CMS) defines a generic UPnP service, hosted by an UPnP Parent Device, which allows a control point to manage the configuration in terms of Parameters supported by the device and their actual values.

The term Parent Device is frequently used thorough this document. It refers to UPnP device/service sub- tree whose root is the UPnP device that contains the ConfigurationManagement service instance.  UPnP actions or other operations on a Parent Device SHOULD apply to all levels of this sub-tree, but SHOULD NOT apply to an embedded device that itself contains a ConfigurationManagement service instance.

Parameters may describe configuration features of the Parent Device or may be related to its status information. CMS defines the concept of Data Model as the set of Parameters provided by a Parent Device for being managed by CMS actions.

CMS can be used as an UPnP service in any UPnP Device, whether the UPnP DM ManageableDevice or a UPnP Device defined by another UPnP Working Committee. Refer to [DEVICE] for details of the possible deployment scenarios.

The linked document specifies two related concepts:

- Generic actions for managing Parent Device configuration.

- A basic set of configuration parameters that a Parent Device can support. In case the Parent Device is a ManageableDevice (see also [DEVICE]), then such configuration parameters are mandatory and referred as Common Objects; additional or alternative configuration parameters can be defined by other UPnP DCPs and optionally supported, by other organizations’data model definitions or by vendor specific extensions.

This service-type enables the following functions:

- Reading the actual configuration and status of a Parent Device using CMS (i.e. “reading” parameters), in terms of available data model parameters with their values.

- Changing the actual configuration of a Parent Device using CMS (i.e. “writing” parameters), by setting new values of parameters and creating or deleting object instances (i.e. rows in parameter tables).

- A warning mechanism to allow control points to be informed as some relevant changes occur in the data model parameter values (e.g.: a critical fault, a warning, a significant event, ...), in order to take immediate actions if needed. This mechanism uses the eventing by providing extra information within the event message (parameters and values).

These CMS operations can be protected by an OPTIONAL Security Feature based on  DeviceProtection:1 [DPS]. Actions that do not return sensitive information, change the device configuration, or affect normal device operation can always be invoked by all control points.  If the Security Feature is supported, other actions can only be invoked if the control point is appropriately authorized.

The detailed specification is published at www.upnp.org at ConfigurationManagement

#### Software Management

Many devices in the home network (for example, TV, Camera, Media Server etc.) have embedded software or firmware that needs to be managed. The SoftwareManagement service is intended to provide control points with the ability to manage this embedded software or firmware.

The SoftwareManagement service enables a control point to:

- Install or Update software entities on a targeted Execution Environment.

- Uninstall software entities on a targeted Execution Environment.

- Start software entities on a targeted Execution Environment.

- Stop software entities on a targeted Execution Environment.

- Update device firmware.

The SoftwareManagement service does not:

- Mandate the protocol used for downloading the software entities

- Provide details of any particular Execution Environment

These operations can be protected by an OPTIONAL Security feature based on DeviceProtection:1 [DPS]. Actions that do not return sensitive information, change the device configuration, or affect normal device operation can always be invoked by all control points.  If the Security feature is supported, other actions can only be invoked if the control point is appropriately authorized.

The detailed specification is published at www.upnp.org at SoftwareManagement

#### Quality of Service (QoS)

The UPnP-QoS specifications are designed with a network in mind that consists of a single IP subnet. Therefore routing between devices in the network is out of scope for UPnP-QoS.

The UPnP-QoS specifications support QoS setup for all traffic streams and UPnP-AV traffic streams in particular. The UPnP-QoS specifications also support QoS management on the LAN for traffic streams originating from or terminating in a WAN.

UPnP-QoS defines three services. These are the QosPolicyHolder Service [QPH:3], the QosManager Service [QM:3] and the QosDevice Service [QD:3]. All three services are shown in Figure 1. In this figure, the numbers in parenthesis indicate the order in which the UPnP-QoS actions are invoked.

Fundamentally, UPnP-QoS manages the QoS for a traffic stream that flows between a source and a sink device. A traffic stream is viewed as a uni-directional flow from a source device to a sink device, possibly passing through intermediate devices.

In the interaction described in this section, a Control Point application is assumed to have the knowledge of source, sink and content characteristics to be streamed, along with the content’s Traffic Specification (TSPEC). The Control Point constructs a TrafficDescriptor structure and requests a QosManager Service on the UPnP network to setup QoS for a traffic stream (step 1). The Control Point in the QosManager Service (from hereon referred to as QoS Manager) requests the QosPolicyHolder Service (step 2) to provide appropriate policy for the traffic stream described by the TrafficDescriptor structure (step 3). Based on this policy, the QoS Manager configures the QosDevice Service(s) for establishing the QoS for the new traffic stream (step 4).

Figure 1 UPnP-QoS Architecture Overview

The detailed specification is published at www.upnp.org at QoSV3

#### Certification

Device Control Protocols are royalty free, practical and proven implementations. To certify a device, system or service to be compliant with UPnP Forum standards the Forum makes a test tool available to vendors that are “Implementer-Level” Member of the Forum. The annual dues are about US$5,000.

To certify a device a vendor must follow this six-step process:

- Become a UPnP Forum Implementer Member. Membership requires execution of the Implementer Membership Addendum and payment of US$5000 annual membership dues.

- Access the certification test tool via the Members Only area.

- Run the test tool at your own facility or at a 3rd party test house.

- Submit the test logs, along with Web-based device registration form, to the Test Reviewer.
Note: Your company must submit a device registration form for each device. Visit the Members Only area for additional information.

- Once you have submitted the test logs and device registration forms, the Test Reviewer reviews the test logs and contacts you with testing results.
Note: The review may require up to five (5) working days; however, two working days is our goal.

- If the device passes, your company receives a certificate of conformity for that particular device. The certificate authorizes your company to use the certification mark on the certified device and in associated marketing material, according to the Implementer Membership Addendum and the Logo Usage Guidelines document and FAQ, which are accessible via the Members Only area).
If the device fails, your company is notified of the failure and reason, no certificate is issued, and your company may not market the device as UPnP-certified. Your company is welcome to alter the device and retest.

## Evaluation of available M2M interface standards

### Selection Criteria

#### Publicly available

#### Based on Internet Protocols (IPv4, IPv6 and HTTP)

#### Independent of Hardware Processor (CPU) and Operating System

#### Supports both Local and Wide Area Networks (LAN & WAN)

#### Authentication of subscribing services

#### Assignment of authorization levels through Roles

#### Allow encryption using Security Certificates (TLS)

#### Advertising and Discovery of services using SSDP

#### Detailed Service Description using XML

#### Auto configuration after Authentication and Role Assignment

#### Latency and Flow Control using QoS

#### Ability to process audio/visual data streams

#### Remote Maintenance (Configuration and Software Management)

#### Certification

#### Congruent with S-100

#### Congruent with the Maritime Cloud

#### Congruent with future standards such as Internet of Things (IoT)

# Overview

Machine-to-Machine interface is defined as physical interface, network interface, service interface, maritime data exchange interface, service specific interface, operation and management and security. The relationship between OSI 7 layer and M2M interface is shown in Figure 1.

# Physical Interface

Physical interface represents physical connections for the communication between two different machines such as serial lines, Ethernet,…, etc.

## Network type

- wired

- wireless

## Topology Model

- Star Model : 1:1 physical connection between device, systems and service.

- Ring Model :

- Bus model : Shared a communication infrastructure

# Network interface

Network interface provides an mechanism to transport data over the networks.

## Addressing

- TBD

## Intra-network interface

Network interface represents an network protocol interface to communicate with each services. This includes TCP/IP protocols, IEC 61162-1/NMEA 0183, or IEC 61162-3/NMEA 2000.

- switch

- Router

- Gateway service

- TBD

## Gateway (WAN)

When a network is connected with WAN networks including Internet, it is required to provide a gateway to external CSS components.

## Data delivery

- TBD

# service interface

Service interface is an interface to communicate and to exchange information among services. There are several ways to exchange the information. For the service exchange, the followings shall be provided.

- Naming

- Service Discovery

- Service Management

# Maritime data exchange interface

This section covers the how to exchange Common Maritime Data Structure within a CSSA architecture.

- NMEA sentences/PGNs

- ENAV-61162

- Transportation of CMDS including encapsulation with S-10x

# User specific data interface

Each service may have its own data interface for proprietary purpose.

- TBD

# Operation and Management interface

Each interface maintains its own operation and management function. The interface includes the following

- TBD

# cyber Security

Each interface requires additional security features for the secure and confidential information exchange. Each interface may require the different levels of security. This section will cover the security issues for the interfaces.

- TBD

# other issues

The other related issues for the Machine-to-Machine Interfaces are following:

- TBD

An introduction to the concept of Nodes and the data distribution model can be found in the corresponding generic service engineering model guideline.