VTS Committee

17th Session								VTS17/WG1/WP2

September 13, 2002

### Proposed Amendments to Chapter 2.2 of the IALA VTS Manual

# CATEGORISATION OF VTS

2.2.1 General

A Formal Risk Assessment provides the basis for establishing the need for a VTS and is the fundamental process whereby hazards to the safety of navigation may be identified, the risks estimated and then evaluated. Formal Risk Assessment increasingly forms the basis upon which a Safety Management System is derived. As it is upon this Safety Management System that the scope and functionality of the VTS is determined, it follows that it is fundamental in defining the category of the VTS.

    - Categorisation

When considering the development and implementation of VTS, the VTS Authority will need to decide on the level of service that they provide, the availability of the service and the training standards of the staff that provide the service. Level and availability of service go to make up the Categorisation of the VTS.

There are two potential beneficiaries of  “categorising” VTS, namely:

- The mariner, who needs to know what services and activities a specific VTS is able to provide and perform and under what circumstances;

- The VTS Authority and/or the Competent Authority, in deciding what VTS may be required to mitigate identified risks, and thereby what level and availability of VTS should be procured.

The categorisation of a VTS, designed to benefit the mariner, is expressed in terms of a coding mechanism (see paragraph 2.2.4 below), which is based on similar regimes already in place in the aviation and maritime worlds. The code is designed to offer essential information, on the level and availability of the service, to the mariner in a simple, easy to read format. It is a compilation of VTS elements that are the core requirements for categorisation, as identified by IALA. The use of the code enables sub-areas or sectors within a VTS area to be categorised differently, if necessary.

2.2.3	Promulgation of Information

When developing a VTS and having determined the level and availability of service, consideration should be given to the means of promulgation of this information. In order to assist users of the IALA/IAPH/IMPA World VTS Guide, VTS authorities are able to declare the details of the services they provide in a format compatible with the coding mechanism. On submission of the requisite information it is entered into and made available to mariners through the Guide. In order to ease identification of the VTS entry, within the Guide, it is prefixed by VTS. The VTS Authority and/or Competent Authority should arrange for this information to be published in other appropriate nautical publications.

2.2.4	The VTS Categorisation Coding Mechanism

| - Service Availability: |  |  |
| --- | --- | --- |
| X = 24hours | Y- Other, for example |  |
|  | ## Code number | ## Explanation |
|  | 1 | 24 hours except holidays |
|  | 2 | Between 0900 and 1700 daily |
|  | 3 | 24 hours weekdays only |
|  | 4 | All others (between the hours specified) |

An entry of Y2 indicating a service that is provided only between 0900 and 1700 daily.

| - Traffic Image Generation: |  |  |
| --- | --- | --- |
| Indicated by "T"; and any other facilities that currently apply: |  |  |
|  | ## Code number | ## Explanation |
|  | 1 | Automatic Identification Systems (AIS) |
|  | 2 | Radar |
|  | 3 | Closed Circuit Television (CCTV) |
|  | 4 | Visual |
|  | 5 | Radio direction finder (RDF) |
|  | 6 | VHF radio ONLY |

An entry of T125 indicating that the service provided incorporates the following facilities: AIS, RADAR, and RDF.

| - VTS Levels of Service |  |
| --- | --- |
| Information Service | Indicated by INS |
| Navigation Assistance Service | Indicated by NAS |
| Traffic Organisation Service | Indicated by TOS |

- Availability to allied services

A service that includes communication with other VTS or allied services is indicated by AS

- Example of the code

VTS/Y2/T125/INS+TOS/AS

This example describes a VTS that is available from 0900 to 1700 daily; that builds its traffic image using AIS, radar and RDF, that provides information and traffic organization services, and has communications with other VTSs or allied services.

2.3 SERVICE AND OPERATING OPTIONS [IALA Secretariat/ Editor of the VTS Manual to insert existing section 2.2 of the VTS Manual and update the existing text to ensure consistency with IMO Resolution A.857(20), Guidelines for Vessel Traffic Services]

  - CERTIFICATION OF VTS

The responsibility of determining the level and standard of VTS required to mitigate identified hazards lies with the VTS authority and/or the Competent Authority. They are accountable for the standards they set, the resources they allocate and for the effectiveness of VTS they choose to adopt. It follows that appropriate and adequate operational and administrative procedures should be developed in order to provide the service effectively.  The Certification of a VTS is concerned with assuring that the operational and administration procedures used by a VTS Authority provide the services declared in its Categorisation.  Certification can be achieved by an appropriate auditing and accreditation process.

It is a national responsibility to determine whether the Competent Authority has taken action to ensure that the VTS authority/centre has adequate procedures and practices in place to meet the declared standards.