Input paper:   ENG17-3.1.1.15

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	X  ENG		□  PAP					☑  Input

□  DTEC	□  VTS							□  Information

Agenda item 			n.n

Technical Domain / Task Number 2	…………………………………

Author(s) / Submitter(s)			………CHINA MSA………

# Proposal on the Revision of Partial Chapter of IALA G1077

# on Maintenance of Aids to Navigation

# Summary

In December 2019, The Council approved Recommendation R1018 on Design and Operation in the Provision of Marine Aids to Navigation (Ed.1.0) at its 70th session. With the approval of R1018, the Guideline 1077on Maintenance of Aids to Navigation(Hereinafter referred to as the Guideline) needs to be revised accordingly. This paper provides some suggestions on the revision of partial chapter of the Guideline.

## Purpose of the document

1.1.1The main purpose of this document is to provide a reference for supplementing and improving the maintenance introduction of non-steel buoys in the Guideline, so as to ensure the overall objectivity of the maintenance introduction of non-steel buoys in the Guideline.

1.1.2Radar transponders are all-weather navigational aids installed in important locations such as bridges, light posts, and lighthouses to locate ships. Maintaining a good radar transponder can promote the economic development of port shipping, improve industry management level, and have significant economic and social benefits. The second purpose of the proposal is to supplement the maintenance related content of the radar transponder.

## Related documents

IALA R1018 Responsible Design, Operation and Maintenance in the Provision of Marine Aids to Navigation

IALA G1077 Maintenance of Aids to Navigation

IALA R1001 The IALA Maritime Buoyage System

IALA R101 Recommendation Marine Radar Beacons (racons)

IALA G1010 On Racon Range Performance

# Background

2.1As the maintenance introduction of polyethylene non-steel buoys described in Appendix A2.2 of the Guideline  mainly describes the maintenance of low-density polyethylene buoys. However, ultra high molecular weight polyethylene buoys are more widely used in practice. Choosing to repair the damaged  surface on buoys by spraying new plastic with flame can only be applied to low-density polyethylene buoys. And the modular structure mentioned in this chapter, which is in good condition during repair, can same only be applied to low-density polyethylene buoys. Obviously, this conflicts with the maintenance methods of ultra high molecular weight polyethylene buoys, thus the lack of maintenance of ultra high molecular weight polyethylene buoys  leads to a blank in Guideline.

2.2Appendix D of the "Guide" mainly provides maintenance instructions for lanterns and their external structures, ordinary light fixtures, sound emitting devices, etc., but lacks maintenance instructions for commonly used radar transponders.

# Discussion

3.1 There are mainly two kinds of polyethylene used in AtoNs, one is linear low density polyethylene (LLDPE) and the other is ultra high molecular weight polyethylene (UHMWPE). Although they are all polyethylene, the material properties differ greatly because of the completely different branched chain structures. LLDPE has the advantages of good toughness, strong rigidity, heat resistance, cold resistance and so on. The material melts immediately after heating, which is convenient for later repair. However, its impact resistance and its deep-sea stability are poor; UHMWPE has excellent impact resistance, wear resistance, weather resistance, stress cracking resistance, adhesion resistance, self lubrication and other advantages. However, due to almost no branched chain, UHMWPE is incompatible with each other even after heating, and cannot be repaired in small area. The second part of the Guidelines states in the guiding principles for maintenance strategies that the interrelationship with requirements and design must be taken into consideration. Where the costs of maintaining systems are excessive, it may be appropriate to revisit the original designs and user requirements. There are deficiencies in the current Guidelines for the maintenance of UHMWPE buoys. Users may encounter difficulties when using the Guidelines for buoy maintenance, which affects the maintenance effectiveness of the buoys.

Recommendation 1018 requires that implement systematic procedures for the design, maintenance, and safe and sustainable operation of their marine aids to navigation and associated equipment to meet or exceed the required performance; The revision of G1077 under R1018 is conducive to more standardized maintenance of AtoN and achieving the performance required for UHMWPE buoys.

3.2The maintenance requirements for radar transponders mainly include: (1) Appearance: all components should not have obvious dents, scratches, cracks, deformation, and pollution, metal components should not have rust requirements, and the surface coating should be uniform, without bubbles, cracks, scratches, detachment, and wear, and fasteners should not be loose. (2) Installation method: It should be installed vertically, and the installation height should ensure that its geographical range matches the designed range of action, and the principle is that there are no obstructions in the direction of action. The installation position should avoid vibration. (3) Voltage stabilizing equipment: The power supply part of the transponder should be equipped with voltage stabilizing equipment, and its rated power should be greater than 50% of the requirements of the transponder; The output voltage should be stable at the standard value required by the transponder, and the fluctuation should be less than 5%. (4) Daily management: The management agency should arrange inspections twice a month, starting from a range of 1-5 nautical miles from the transponder using marine radar and ending at the maximum operating distance specified by the transponder, to scan and detect the corresponding transponder. The scanning time after searching for signals should not be less than five minutes, and relevant forms should be filled out.

At present, IALA has developed an upcoming working plan for 2023-2027, which also mentions updating information related to G1077 Maintenance of Aids to Navigation to assist in developing maintenance strategies.

# Application of UHMWPE buoys

4.1 UHMWPE buoys are suitable for various scenarios and are currently used in both inland waterways and sea. The vast majority of them are used in the ocean, accounting for about 80%, while inland waterways account for about 20%. UHMWPE buoys can produce 7 kinds of buoys according to the standards of the IALA Maritime Buoyage System. Since 2020, about 2000 UHMWPE buoys  have been purchased and managed by domestic and foreign maritime traffic management departments, including lateral buoyage , cardinal buoyage , isolated danger buoyage, safe water buoyage, special buoyage, and emergency wreck buoyage, In addition, there are approximately 1000 UHMWPE buoys that belong to the procurement and management of non transportation or private sectors, such as fisheries, meteorology, marine environment, etc.

UHMWPE buoys (lateral buoyage)

UHMWPE buoys (cardinal buoyage and special buoyage)

4.2 In addition, according to the classification standard of other mark in the IALA Maritime Buoyage System, light beacons and large navigational buoys made of UHMWPE material are widely purchased and applied domestically and internationally. The light beacons made of UHMWPE can not only resist class 12 typhoon, but also resist fading and have maintenance-free resistance, and can be modular designed with simple handling and installation. It also has obvious advantages in installation on islands, reefs, and other areas with inconvenient transportation. There are many scenarios for the application of UHMWPE light beacons, At present, domestic and foreign maritime traffic management departments have purchased and managed UHMWPE light beacons, which are applied to islands, dark sand and other areas. There are about 1000 large light beacons with a height exceeding 8 meters, and about 2000 UHMWPE light beacons are purchased for use in scenarios such as bridges, docks, and breakwaters. In addition, more than 5000 UHMWPE light beacons are purchased and managed in offshore wind power, offshore oil and gas, nuclear power construction engineering, and other fields. In some areas, steel light beacons have been completely replaced.

UHMWPE light beacons

# Proposal

Considering that the IALA Committee has developed an upcoming working plan for 2023-2027, which also mentions updating information related to G1077 Maintenance of Aids to Navigation to assist in developing maintenance strategies. Therefore, it is recommended that the ENG committee refer to the content discussed in the third part and the application in the fourth part above to add content on the maintenance of UHMWPE buoys in Appendix A2.2 of the Guideline, and enrich and improve the guidance under Recommendation 1018, in order to ensure the comprehensive objectivity of the maintenance introduction of non-steel buoys in the Guideline. Add relevant content on radar transponder maintenance in Appendix D, and improve the types of navigation aids in the Guide.

# Action requested of the Committee

The ENG Committee is invited to consider the proposal in part 5 and take action as appropriate.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain